3GPP SA WG2 Meeting #172	 S2-2510403

# Dallas, USA,17-21 November 2025					             				    (was S2-250xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1687 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on supporting of IMS DC in roaming scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the reply LS from SA3 (S2-2509874) on LI requirements on IMS Data Channel, following statement is clarified by SA3: Feedback from SA3: The S8HR/N9HR are direct interfaces between a roaming UE and its HPLMN and LI Requirements on the S8HR/N9HR imply that these interfaces shall not be confidentiality protected. It is possible to VPLMN have access to a copy of the DC content without confidentiality protection by using DTLS1.2 in DC protocol stack and setting NULL cipher for DTLS1.2. However, it’s not recommended in security point of view because of following reasons: - NULL cipher is not possible for DTLS1.3 and is not recommended in DTLS1.2. - DTLS1.3 is recommended since Rel19. - The downgrade security configuration is applied to all UEs of the HPLMN to avoid LI being detectable. - Regulation such as EU Cyber Resilience Act (CRA) mandates that data in transit and storage by default should be encrypted using state-of -the-art mechanisms In summary, in the context of S8HR/N9HR-based IMS roaming, SA3 was unable to identify a solution that meets LI requirements for IMS DC in the VPLMN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that home-routed roaming scenarios of IMS DC is not supported when LI requirements on IMS DC are enforced in the VPLMN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lawful Interception for P2P data channel is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

# AC.1	General

This annex describes IMS architecture enhancements to support data channel services.

Service based interfaces applicable to data channel services are specified in clauses AA.2.4 and AA.2.5.

The architecture in this annex supports separation of signalling function and media function supporting data channel services. A network function for data channel management signalling, Data Channel Signalling Function (DCSF), is specified. The network function to handle data channel media can be provided by introducing a new network function called Media Function (MF), which interacts with the IMS AS via the service-based interface DC2. MF provides media capabilities in support of IMS DC and Augmented Reality.

NOTE 1:	In this Release of the specification the IMS AS supporting data channel services is collocated with the TAS.

The IMS architecture supporting data channel services also supports exposure of IMS DC. The architecture and procedures of IMS DC event exposure are specified in Annex AD. The architecture and procedures of IMS DC capability exposure are specified in Annex AG.

In this Release of the specification IMS Data Channel is supported in the case of non-roaming scenarios.

NOTE 2:	Home-routed roaming scenarios of IMS DC is not supported when LI requiments on IMS DC need to be enforced in the VPLMN.

Multiplexing of multiple IMS Data Channel streams with different endpoints onto a single SCTP association is not supported in this Release of the specification.

IMS data channel interaction with supplementary services is supported as specified in TS 24.186 [105].

# >>>>END OF CHANGES<<<<