3GPP TSG-WG SA2 Meeting #172	S2-2510412

Dallas, US, 17th – 21st Nov, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4: Conclusion proposal

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Conclusion on KI#4 on authorization is proposed in this document.

# 1. Introduction

Based on the discussions and analyses in S2-2508235, the architectural conclusion of KI#1 in S2-2509659, and the agreed paper of KI#4 in S2-2509826, this paper provides the conclusion on KI#4 on Sensing Data and the Associated Information Collection and Transport.

On top of S2-2509826, the following aspects are further clarified:

-	Sensing Function may have different Endpoints for control messages and for Sensing Data;

-	Adding NOTEs for clarifying the split deployment of SCF and SPF;

-	Adding the explanation on reusing the sensing information for different sensing service request.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

### 7.1.4	Agreed Principles for KI#4 Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Sensing Function may have different Endpoints for control messages and for Sensing Data.

NOTE 1:	In case of the separate SCF and SPF deployment, the endpoints for control messages and for Sensing Data associates to SCF and SPF, respectively.

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

NOTE:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple gNBs by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

NOTE 3: 	In case of the SF splitting into SCF and SPF, the SPF receives the Sensing Data from the Sensing Entities and generated the Sensing Results.

-	If the Sensing Function determines it already sends the sensing service requirement for a different sensing service request to the same Sensing Entity, i.e., gNB, the Sensing Function may skip sending the sensing service requirement, or update the existing sensing service requirement.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

Principles in clause 7.1.4 are agreed as the conclusion.

# * * * * End of changes * * * *