3GPP TSG-WG SA2 Meeting #172	S2-2510435

Dallas, USA, 17th – 21th November, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI #2, New Sol] Enhance IMS AS service to support audio, video capability in addition with DC capability

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract: This contribution is intended to provide a new solution of Key Issue#2: Enhancements to capability exposure framework for eMMTel services.

1. Introduction/Discussion

This pCR proposes a new solution to the Key Issue#2: Enhancements to capability exposure framework for eMMTel services.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-56.

# * * * * First change * * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

# * * * * Second change (all new text) * * * *

6.X	Solution #X: Enhance IMS AS service to support audio, video capability in addition with DC capability

6.X.0	High level solution Principles

Release 19 NG_RTC_Ph2 has specified IMS data channel capability exposure procedures and the related services to enables the AF/AS to request the IMS AS via the NEF to create or release an IMS session with DC media or modify the DC media in a specific IMS session. But in some cases, the NF/AF needs to create an IMS session with audio/video media and DC media at the same time.

OMA has specified a RESTful Network API for Audio Call, but it cannot support DC media. If a 3rd party AF/AS wants to create an IMS session with audio and DC media, it has to invoke the OMA API to create a IMS session with audio firstly and then it invokes Nnef_ImsSessionManagement API specified by 3GPP to add DC media in the established IMS session. It means that the 3rd party AF/AS need to utilize two sets of APIs with different styles (OMA style and 3GPP style) and it cannot use both audio and DC media at alerting phase.

To ease the development of the 3rd party AF/AS and enable that audio and DC media can be used at alerting phase, both Nnef_ImsSessionManagement, Nimsas_ImsSessionManagement and Nmf_MediaResourceManagement service specified in Release 19 need to be enhanced to support audio and video media.

6.X.1	Description

As a NB-IoT GEO satellite may serve multiple countries/areas, when the UE moves between different multiple countries/areas, different emergency numbers should be provided to UE by MME.

-	If the NB-IoT GEO satellite serves for multiple PLMNs of different countries/areas, the MME can provide correct emergency numbers associated with the selected PLMN after location verification.

-	If the NB-IoT GEO satellite serves for a single PLMN (e.g., with MCC 901) covering different countries/areas, the MME may need to know the country/area UE located and provide or update the emergency numbers to UE with UE movement between countries/areas.

If the network supports emergency service, the MME should also provide an Emergency Service Support indicator to UE at successful Attach and TAU procedures.

6.X.2	Service definition

6.X.2.1	Nnef_ImsSessionManagement service

6.X.2.1.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nnef_ImsSessionManagement API in the following clauses.

6.X.2.1.2	Nnef_ImsSessionManagement_Create service operation

Service operation name: Nnef_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

Inputs, Required: Calling ID, Called ID, Media information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel, Audio or Video.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information;

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If PA2 or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC;

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: None.

6.X.2.1.3	Nnef_ImsSessionManagement_Update service operation

Service operation name: Nnef_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the data channel media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel, Audio or Video).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters:

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	App Binding Information;

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address of the application layer is indicated.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted

-	if the Operation Type indicates adding or updating the media, the MDC1 media endpoint address.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1 of TS 23.228 [55].

Outputs, Required: Result indication.

Outputs, Optional: None.

6.X.2.2	Nimsas_ImsSessionManagement Service

6.X.2.2.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nimsas_ImsSessionManagement API in the following clauses.

6.X.2.2.2	Nimsas_ImsSessionManagement_Create service operation

Service operation name: Nimsas_ImsSessionManagement_Create

Description: This service enables the consumer NF to create an IMS session with the requested media (e.g. standalone data channel media).

NOTE 1:	Only data channel media is supported in this Release.

Inputs, Required: Calling ID, Called ID, Media Information set.

Calling ID: It is the public identity of calling IMS Subscriber or the identity of the consumer NF, e.g. DC AS.

Called ID: It is the public identity of called IMS Subscriber.

Media Information set: Includes a set of media information indicating how the media used in the created session. Each media information contains:

-	Media Correlation ID: It identifies the media component to be created. It is allocated by the consumer NF.

-	Media Type: It indicates the media type used in the created session, e.g. Data Channel, Audio or Video.

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID(s): the public identity(ies) of the IMS Subscriber the DC is targeted;

NOTE 2:	For P2P ADC, the Calling ID and Called ID of the session are used as the Target IDs.

-	If P2A or P2A2P is selected, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional:  Session ID allocated by IMS AS.

6.X.2.2.3	Nimsas_ImsSessionManagement_Update service operation

Service operation name: Nimsas_ImsSessionManagement_Update

Description: This service operation enables the consumer NF to modify the media in a specific IMS session, such as adding or removing bootstrap data channel(s) or application data channel(s), modify media parameters for the targeted data channel(s).

Inputs, Required: Session ID, Media operation set.

Session ID: It is the identity of session that this request targeted to.

Media operation set: Includes a set of media operations commands indicating how to modify the media used in the specific session. Each media operation command contains:

-	Media Correlation ID: It identifies the media component to be created.

-	Operation Type: It indicates adding, updating or removing the corresponding media.

-	Media Type: It indicates the type of operated media (e.g. Data Channel, Audio or Video).

-	Media Parameter set: It indicates the related media parameters. For different media, it contains different parameters.

-	When the Media Type is Audio or Video, it includes:

-	Codecs and Formats;

-	When the Media Type is Data Channel, it includes:

-	DC Type: indicates the Data Channel Type (ADC or BDC);

-	When DC Type is ADC:

-	ADC Type: the ADC type (P2A, P2P, P2A2P);

-	Application Binding Information; and

-	Target ID: the public identity of the IMS Subscriber the DC is targeted;

-	If P2A or P2A2P is selected, if the Operation Type indicates adding or updating the media, the MDC2 media endpoint address(es) of the application layer is included.

-	When DC Type is BDC:

-	Target ID: the public identity of the IMS Subscriber the DC is targeted.

Inputs, Optional: Notification Target Address and Notification Correlation ID.

If the request includes a Notification Target Address, the request creates an implicit subscription to be notified for the events in Table AA.2.4.4.5-1.

Outputs, Required: Result indication.

Outputs, Optional: None.

6.X.2.3	Nmf_MediaResourceManagement (MRM) service

6.X.2.3.1	General

The Media Type and Media Parameter set parameter in the following clauses need to be extended to support audio and video media. To be clear, revision mode is used to update the service definition of Nmf_MediaResourceManagement API in the following clauses.

6.X.2.3.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. DCSF, to be applied on the media stream by the MF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value 'null' by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The Media ID value is set by the consumer.

-	associated Media ID: This parameter only exists when a multiplexed media is de-multiplexed to multiple media and is used to associate different media streams which are de-multiplexed from a multiplexed media component. This parameter is set to the value of the media ID of the original multiplexed media.

NOTE 1:	Different Termination IDs associated with the same Media ID or associated media ID corresponding to the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and ports allocated at the remote UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	Media Function Specifications: Specification of the media function to be applied to the media stream as requested by the application layer, which includes:

-	Media Function Type: The value is set based on the type of the media function required, e.g. "DC media function type, Audio, Video or AR media function type.

-	Media Function Description: Application specific media function description specifying how the media function shall handle and process the media stream. Data content depends on type of media function.

If the Media Function Type is "Audio" or "Video", then the Media Function Description incudes:

-	Codecs and Formats;

If the Media Function Type is "DC Media Function", then the Media Function Description incudes:

-	Media proxy configuration (HTTP or UDP) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL per Stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Data Channel SCTP Port: This attribute identifies the SCTP port for the Data Channel.

-	Security Setup: This attribute identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity

NOTE 3:	Some of the above attributes are optional depending on the scenario and proxy configuration.

If the Media Function Type is "AR Media Function", then the Media Function Description includes:

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE 4:	The detail of Media Processing Specification is defined by SA4 and needs to be further aligned.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource URL: URL that allocated by DC AS for MF to retrieve the avatar representation.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric with MF rendering, network centric with DC AS rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 5:	Avatar Media Specification are optional depending on the scenario.

Inputs, Optional: None.

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information. For each Media Termination Descriptor; the Termination ID and the list of media stream descriptors corresponding to the input list of media stream descriptor:

Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer. This is identical to the Media ID used in the input.

-	associated Media ID: identical to the associated Media ID received from the consumer.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the DCSF by the consumer NF. Data content depends on type of media function. If the Media type is "DC media function", the Media Application Function Output Description includes:

-	MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

### 6.X.3	Impacts on Services, Entities and Interfaces

IMS AS:

-	Enhance Nimsas_ImsSessionManagement API to support audio and video media when creating, updating and removing an IMS session.

NEF:

-	Enhance Nnef_ImsSessionManagement API to support audio and video media when creating, updating and removing an IMS session.

MF:

-	Enhance Nmf_MediaResourceManagement API to support creating, updating and removing audio and video media.

# * * * * End of changes * * * *