SA WG2 Meeting #172	S2-2510472

17-21 Nov, 2025, Dallas, USA

Source:	CSCN

Title:	[General aspects] Architecture Requirements for 6G

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:This paper proposes Architectural Requirements.

Discussion

The SA2#171 Meeting discussed the Architectural Requirements for 6G. However, only a partial consensus was reached due to time constraints. The discussion was postponed to this meeting.

Beyond the requirements that reached a consensus in the previous meeting, the dynamic NF deployment should also be taken into account. When facing more complex network environments, the existing architecture may still present challenges. For example, although the SBA improves deployment flexibility, its signaling paths remain relatively fixed. It may limit the overall flexibility and scalability of the network in highly dynamic environments. 6G provides a series of new services, such as NTN, ISAC and Immersive Communication. To support these services, the deployment model of NFs must evolve accordingly.

To adapt to the NTN scenario, the deployment and orchestration of NFs must exhibit high physical dynamism. This means that NFs need to be able to seamlessly switch and migrate between different physical domains dynamically based on user mobility and network conditions.

The ISAC brings new types of network functions, such as NFs for processing and interpreting sensing data. To reduce data backhaul latency and achieve localized processing, sensing related NFs must be tightly integrated with RAN, and even dynamically deployed on the same edge node.

Immersive Communication imposes extreme requirements on network performance. To meet these nearly stringent latency requirements, any centralized data processing model will be infeasible. This directly requires that NFs must be capable of being dynamically and granularly deployed to the network edge.

Dynamic NF deployment is a requirement that enable operator to deploy NFs more freely to address various services (e.g. satellite communications, ISAC, XR), which means the 6G services require CN has the capability that NFs can be added or removed freely and the connection can be established quickly. Therefore, the 6G system should consider dynamic NF deployment to meet the needs of these services.

Proposal

It is proposed to agree the following changes into TR 23.801-01.

**** First Change (all text new) ****

4.2	Architectural Requirements

The following architectural requirements are considered:

// Option 1

1.	The 6G System supports roaming.

2.	The 6G System considers the following aspects: cloud nativeness, sustainability and energy efficiency, robustness and resiliency, dynamic NF deployment.

3.	The 6G System interfaces specified based on study conclusion aim to support multi-vendor interoperable interfaces.

4.	The 6G System aims to minimize inter dependencies between NFs.

12.	The 6G System aims to be predictable for all services offered by 6G System (e.g. when a user wants connectivity, the user gets connectivity).

// Option 2

1.	The 6G System shall support roaming.

2.	The 6G System should consider the following aspects: cloud nativeness, sustainability and energy efficiency, robustness and resiliency, dynamic NF deployment.

3.	The 6G System interfaces specified based on study conclusion shall support multi-vendor interoperable interfaces.

4.	The 6G System should minimize inter dependencies between NFs.

12.	The 6G System shall be predictable for all services offered by 6G System (e.g. when a user wants connectivity, the user gets connectivity).

**** End of Changes ****