SA WG2 Meeting #172	S2-2510487

17 - 21 Nov, 2025, Dallas, US                              (revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Update for KI#2 UC#1 Interim Agreements and Conclusions

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements and conclusion for AIML_CN KI#2 use case 1.

- Discussion

Interim agreements have been made for KI#2 use case#1 with some open issues left. This contribution proposes to further discuss the open issue blow and reach conclusion:

-  For UC1, the input from AF, possibly via NEF, is FFS.

Since the AF is able to collect information of normal traffic sent and received by itself (source entity of the traffic, i.e. AS/EAS). If the AF can generate and provide the traffic characteristics of normal traffic to NWDAF, it will useful for NWDAF to determine whether a traffic is abnormal based on the comparison with normal traffic characteristics.

Hence, it is proposed to capture the principles in interim agreement and the input from AF in conclusion.

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** First change ***

7	Interim agreements

7.2	Topics for further consideration

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

The following topics are for further consideration for KI#2:

-	The need and details of type/identification of traffic pattern is FFS.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

NOTE:	How UPF subscribes to the new analytics is FFS.

*** Second change ***

8	Conclusions

Following conclusions for key issue#2 use case#1 has been made:

- A new analytic shall be defined to support abnormal traffic analysis.

- Following inputs shall be supported:

- Table 8-1: input data for abnormal traffic analytic

| Information | Source | Description |
| --- | --- | --- |
| Source of traffic | UPF/SMF/AF | Source entity of the traffic identified by IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc. |
| traffic flow filters | UPF/SMF/AF | Traffic filter identifies the traffic flow (e.g. IP 5-tuple, IP 3-tuple). |
| Identification of corresponding UEs | UPF/SMF/AF | Identifier of the UEs receive/send the traffic. |
| Information of traffic characteristics | UPF/SMF | Traffic characteristics of the traffic that NWDAF can use to determine whether the traffic is abnormal (e.g. traffic lifetime, data rate, maximum burst size, burst periodicity, measurement duration, UPF interface info, SSL/TLS fingerprint, Syn-Ack signalling info). |
| Traffic characteristics of normal traffic | AF | Traffic characteristics of previous normal traffic corresponding to the same source entity of the traffic. NWDAF can use this information to compare with the input traffic flow and traffic characteristic to identify whether the traffic characteristic is different from normal traffic. |

- Table 8-2: output data for abnormal traffic statistics/predictions

| Information | Description |
| --- | --- |
| Source of abnormal traffic | Source entity of the abnormal traffic identified by IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc. |
| Filters of the abnormal traffic | Traffic filter identifies the abnormal traffic flow (e.g. IP 5-tuple, IP 3-tuple). |
| Identification of the affected entity | Identifiers/addresses of UPF(s), UE(s), PDU session(s) affected by the abnormal traffic. |
| Type of abnormal traffic | Type of abnormal traffic, e.g. due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst |
| Traffic characteristics of the abnormal traffic. | Volume, rate or burst size of abnormal traffic. |

The consumer of NWDAF abnormal traffic analytic may take the following actions:

- SMF as consumer:

  - UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

  - Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations

- PCF as consumer:

  - Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

*** End of the changes ***