3GPP TSG-SA WG2 Meeting #172	S2-2510538

Dallas, USA, November 17 – 21, 2025

Source:	Samsung

Title:	Interim agreement for KI#2

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This is a proposal for the agreed principles for KI#2.

1. Discussion

This is a proposal for the agreed principles for KI#2.

One of the proposal that is captured in the interim agreement was:

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

However, given that considering energy consumption information from OAM is dynamically changing, the proposal to consider energy consumption information from OAM would lead to frequent signalling to the UE, resulting in consumption of additional energy.

On the other hand, we see a value of considering the Energy Saving Indicator in the subscription information, which is not frequently changing one, when generating UE policy (i.e., URSP rules). This approach would not require additional signalling from UE while achieving energy saving in the network.

Thus, we propose to update the agreed principles for KI#2 accordingly.

2. Proposal

It is proposed to adopt the following changes to TR 23.700-67.

* * * Start of 1st Change  * * * *

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

The following principles are agreed for normative work on KI#2:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information has dependency with KI#1 and will be considered if the support of renewable energy aspect related to KI#1 is decided druing the normative work

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	AM policy control.

-	SM policy control.

NOTE 3:	Based on operator policy, it should have the balance between energy saving and user experience.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related subscription information (i.e., Energy Saving Indicator).

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

Editors note:	Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 4:	Existing AF services can be updated but no new AF services will be introduced for the AF request.

* * * End of Change * * * *