SA WG2 Meeting #S2-172	S2-2510545

17 - 21 November, 2025, Dallas, USA	(Revision of S2-2508909)

# Title:	[Draft] Reply LS on Clarification on 5G VN group

# Response to:	S2-2509925/C4-253576

# Release:	Rel-19

# Work Item:	GMEC, FS_PAIDC_UPF

Source:	SA2

To:	CT4

Cc:	CT3

Contact Person: 	Georgios Gkellas (georgios.gkellas@nokia.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks for the LS from CT4 and would like to provide the following feedback.

Q1: Can a UE, that is not member (temporary/permanently) of any 5G VN group, have an SM subscription (in DNN configuration) with an S-NSSAI/DNN that is associated with a 5G VN group?

SA2 has discussed Q1 and confirms that a UE that is not member of a 5G VN group cannot have in DNN configuration the S-NSSAI/DNN that is associated with the 5G VN group. As described in TS 23.501 clause 5.29.2 a DNN and S-NSSAI is associated with a 5G VN group.

Q2: Assuming an enterprise deployment with many UEs subscribed to an enterprise specific S-NSSAI/DNN, is it allowed to provision only a subset of these UEs into one 5G VN group with the same aforementioned S-NSSAI/DNN?

SA2 has discussed Q2 and confirms that it is not allowed to provision a subset of the UEs belonging to an enterprise deployment with a specific S-NSSAI/DNN into a 5G VN group with the same S-NSSAI/DNN of the enterprise. Either all UEs of the enterprise shall be provisioned to a 5G VN group with the S-NSSAI/DNN of the enterprise or if a subset of the enterprise UEs belongs to a 5G VN group then this 5G VN group shall be associated with a different S-NSSAI/DNN than the one of the enterprise.

Q3: If the answer to Q1 and Q2 is no, before the subset of UEs (of Q2) is added to a 5G VN group, this subset of UEs have no S-NSSAI/DNN subscription for the 5G VN group, does it imply that this subset of UEs needs to be re-configured with the S-NSSAI/DNN associated with 5G VN group when they are added to the 5G VN group by the AF/NEF invoking Nudm_pp service(such that these UEs start using the S-NSSAI/DNN associated with the 5G VN Group when they establish PDU sessions)?

SA2 has discussed Q3 and confirms that 5G VN group management, including group membership definition, can be configured by a network administrator or can be managed dynamically by the AF/NEF invoking the Nudm_ParameterProvision service. The subset of the UEs belonging to the 5G VN group has to be reconfigured.

Q4: If the answer to Q3 is yes, how are the UEs re-configured?

SA2 has discussed Q4 and confirms that the procedure described in TS 23.502 clause 4.16.12.2 applies. PCF is notified by the UDR about the 5G VN group and eventually a UE Configuration Update procedure is leveraged as in step 7 of the “UE Policy Association Modification Initiated by the PCF” (TS 23.502, clause 4.16.12.2).

2. Actions:

To CT4

ACTION: 	SA2 kindly asks CT4 to take the answers above into consideration.

3. Dates of Next SA2 Meetings:

SA2#173		09th – 13th February 2026 		 Goa, India

SA2#174		13th – 17st April 2026 			 EU