SA WG2 Meeting #172	S2-2510590

17 - 21 November, 2025, Dallas, USA                                                   revision of S2-2509795

Source:	Samsung

Title:	[WT#6 Computing] WT update

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the WT update and KI description on 6G computing in TR 23.801-01.

1. Discussion

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

This paper proposes the WT update and KI description on 6G computing on top of S2-2509795, the baseline paper postponed in SA2#172, with the consideration of the following aspects:

Further Clarifications Required

Further clarification on the “support of computing for UE, core network and application server” is needed. The current wording “computing for X” is not clear. The possible clarification on this part can be given as follows:

- Computing for UE: It can be interpreted as follows. The app client in the UE can delegate its (some part of) workflow to its corresponding server. When broken down into finer detail, this implies the followings: (i) the app client itself is designed or implemented to support such flexible operation, (ii) a given app server (corresponding to service client in the UE) will be hosted on a given service hosting environment with a reserved resource such as memory, CPU, GPU, etc. (iii) the core network can support, for a given UE, discovery of the app server and connection establishment can be controlled by the core network, (iv) the service hosting environment can be managed via 3GPP-defined entity.

- Computing for application server: The app server outside the core network (maybe in the DN) can delegate its (some part of) workflow to its corresponding server. When broken down into finer detail, this implies the followings: (i) the app server itself is designed or implemented to support such flexible operation, (ii) a given app server (corresponding to the server which need support of computing) will be hosted on a given service hosting environment with a reserved resource such as memory, CPU, GPU, etc. (iii) the core network can support, for a given app server, discovery of the app server and connection establishment can be controlled by the core network, (iv) the service hosting environment can be managed via 3GPP-defined entity

- Computing for core network: It is not clear what type of and how computing related support/service can be offered to core network functions.

Further clarification on the computing service is also required. As per discussion in SA1, TR 22.870 describes the terminology “6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party”. From SA2 architecture perspective, the following aspects should be clarified:

Location of Service Hosting Environment (SHE): There would be different understanding of the location of SHE, e.g., within the CN, Data Network, or both.

- If located in the CN, the computing service provider would be limited to the MNO. The current 5G system cannot support such architecture.

- If located in the DN, it is very similar to edge computing service. MNO or any 3rd party can be a computing service provider. 5G system already provides various functionalities, which are related to 5G edge computing features.

Clarification on computing resource: In SA1 discussion in 22.870, some examples were described as follows: data centre, edge node, edge computing server, something distributed across cloud server/network/device edge (e.g., CPE), xPU, etc. Considering these examples, the following two types of computing resource can be considered: (i) node or server can be referred as computing resource, or (ii) finer level of virtual resource of hosting environment can be a type of resource, e.g., xPU, storage. The related computing resource has been specified in SA5 and SA6 as below:

| SA6 specified resource information for EAS (TS 23.558) |  | SA5 specified virtual resource attribute (TS 28.538) |  |
| --- | --- | --- | --- |
| Available Compute | The maximum compute resource available for the AC. | virtualResource | This parameter defines the virtual resource requirements of an EAS. |
| Available Graphical Compute | The maximum graphical compute resource available for the AC. | virtualMemory | It indicates the minimum virtual memory size requirements for EAS in megabytes. (see clause 7.1.9.3.2.2 in ETSI NFV IFA-011 [7]). |
| Available Memory | The maximum memory resource available for the AC. | virtualDisk | It indicates the minimum virtual disk storage requirement for the EAS (see clause 7.1.9.4.3.2 in ETSI NFV IFA-011 [7]). |
| Available Storage | The maximum storage resource available for the AC. | virtualCPU | It indicates the virtual CPU requirement for the EAS (see clause 7.1.9.2.3.2 in ETSI NFV IFA-011 [7]). |

Relevance with 5G edge computing: For a computing service to subscriber (via UE), it should be clarified what is the difference from the edge computing. 5G system provides the EAS discovery and UP path configuration for a given EAS. Considering the above examples of computing resource, if computing resource includes edge node or edge computing server and it is located in the DN, then the functionalities of 5G edge computing can be reused or enhanced. If computing resource is something within the core network or belongs to the core network, then 5G edge computing features cannot be re-used as it is.

Further clarification on “coordination between UE, core network and application server” is required. “Coordination” needs clarification. Interaction among UE, CN, and application server would be mainly based on the following functionalities provided by the CN to UE and application server: e.g., discovery of the computing resource on operator’s SHE and authorization for accessing the computing resource on operator’s SHE toward UE or AF. If not clarified, this pCR proposes to delete the term, coordination.

Considering the above discussion, for clarity in task definition, this pCR proposes to categorizing 6G computing service into conventional edge computing service and a new operator-provided computing service.

|  | Edge computing as 6G computing service | Operator service as 6G computing service |
| --- | --- | --- |
| Location of SHE | Data Network (outside to core network) | Core Network (or operator’s network) |
| Computing resource / consumer of resource | Edge Application Server / Application in the UE | FFS (Application in the UE and/or 3rd party application server) |
| Service Path between UE/AF and SHE | Over UP (for between UE and SHE); No path required for between AF and SHE other than network capability exposure | FFS (UP or new dedicated plane for between UE/AF and SHE) |
| Controllability of computing resource | Provider of hosting environment (MNO or 3rd party) | MNO |
| Target service (main purpose) | Low latency service | Service requiring heavy computation; Service implemented with a specific app logic of separating and delegating its workflow to other (mainly providing sufficient resource for computation, may not be for low latency service) |

**** First Change (on top of the postponed baseline pCR S2-2509795)****

Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing service for UE and/or application server via 6G system

In order to support computing, the following aspects need to be studied:

WT#6.1:  Definition of the computing service provided by 6G CN and its corresponding computing resource:

  - Whether and how to define the computing service.

  - Whether and how to define the computing resource (e.g., computing resource type and/or status, location of the computing resource, etc.).

NOTE 1:  The term of Computing Service in TR22.870 can be used as starting point.

WT#6.2: Enablement/authorization of computing service for UE or AF.

  - Whether and how to enable UE/AF to request computing service.

  - Whether and how to authorize UE/AF request for computing service.

WT#6.3: Coordination of communication and computing, service continuity and QoS aspects:

  - Whether and how to coordinate communication and computing resource (e.g., within or outside the core network).

  - Whether and how to identify, collect/monitor, provision the computing resource related information.

  - Whether and how to expose the information for computing resource (hosted and managed by MNO) and/or network metrics.

NOTE X: 	What kind of network metrics needs to be exposed with computing resource information will be identified based on the clarified computing service (outcome of WT#6.1 discussion).

  - Whether and how to improve the service experience (e.g., by defining new metrics or enhancing the existing metrics for QoS).

  - Whether and how to support service continuity for computing service upon change of computing site and/or user plane function (e.g., due to UE mobility, computing load balancing, AF influence, etc.).

WT#6.4: Discovery and (re-)selection of compute site(s)/Application Server(s) for UE/AF requested service:

  - Whether and how to discover and (re-)select computing site(s)/Application Server(s) for UE/AF requested service.

WT#6.5: Whether and how to utilise and/or enhance Edge Computing mechanisms specified in 5G

NOTE 2:  Which of above aspects should fall into SA5 and/or SA6 scope will be further identified in study phase, coordination with SA5 and SA6 may be needed.

NOTE 3:	Potential coordination with other WTs (e.g., WT#1.1 on service enablement and UE-CN interaction, WT#1.2 on QoS, exposure framework and UP aspects, etc.) may be needed.

NOTE 4:	Computing site can be potentially considered as the computing resources in a specific location of network topology. The computing site can offer computing services by allocating its computing resources to run application workloads upon demand of a computing service consumer. Further detailed clarification on computing site and its relation to service hosting environment will be discussed during study phase as part of WT#6.1.

**** End of Changes ****