SA WG2 Meeting #172																					S2-2510596

Dallas, U.S., Nov. 17th – 21st, 2025	(was S2-2509823)

Source:	OPPO

Title:	[WT#1.2, Policy] Update on Policy Framework architecture in 6G

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides slight update on the WT#1.2 Policy based on the postponed paper S2-2509823 at SA2#171.

# 1. Discussion

Wording issue in bullet #b of bullet #1.

The 6G roaming architecture should be defined under WT#1.1, the corresponding NOTE 9 should be clarified to reflect the exact number of WT.

**** First Change (new text) ****

# Annex A.X. WT#1.2 [Policy] Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

To support the Policy Framework in 6GS, the policy and charging control Framework defined in 5G for policies (i.e., SM Policy, UE Policy, AM Policy) is considered as starting point for discussion, the following aspects are to be studied:

- Whether and how to simplify the policy control framework for 6G, including:

a)   whether and how to reduce the number of services or interactions to efficiently provide policies to the different policy enforcement points.

b)  whether and how to support the UE to request UE Policies from the 6G CN.

2. 	 Whether and how to consider user preferences during UE policy evaluation at UE.

- Whether and how to improve the External Parameter Provisioning aspects of policy control for 6G.

NOTE 2: General Authorization of the external parameter provisioning is handled in the WT#1.2 Network Exposure.

- Whether and how to improve events notification about policies to the AF (e.g. to notify the AF that policy and charging control changed).

- Whether and what parameters the UE can provide as input for PCC decision, e.g. related to QoS control to the 6G CN, in case AF input for QoS is not available.

NOTE 3:  The mechanism for the UE to provide the input for PCC decisions to the 6G CN is studied in WT.1#2 Network exposure, WT#1.1 control signalling for 6G System and the WT1.2 QoS. The AF input defined in 5G can be treated as starting point for discussion.

NOTE 4: The UE can decide if and when to initiate the request to the network.

NOTE 5: The UE is not assumed to know whether AF input for QoS is available.

- Whether, how and what to support for policy and charging control in the different roaming scenarios in 6G.

NOTE 6:		This WT takes energy efficiency and consumption aspects into account for policy decisions.

NOTE 7:  	This WT has IWK aspects that are scoped under WT#2.

NOTE 8: 		This WT can study potential enhancements to the policy control framework for 6G services based on the input identified and provided by other WTs.

NOTE 9:		This WT depends on the overall 6G roaming architecture based on the output of WT#1.1.

NOTE 10: 	This WT will consider the output of WT1.2 slicing on improvements of the application traffic mapping to network slice(s)/user plane connection(s).

*** End of Changes ****