3GPP TSG-SA WG2 Meeting #172	S2-2510605

Dallas, USA, 17 - 21 November, 2025

Source:	CATT. OPPO

Title:	23.700-19: Discussion and Interim conclusion for KI#2

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This paper discusses the text-based or binary-based IMS signalling of KI#2, and proposes Interim conclusion for KI#2.

1 Summary of the solutions on KI#2

| Sol# | Optimization Approach | Encoding | Architecture Aspects |
| --- | --- | --- | --- |
| #4 | SCC-AS functionality for IMS Session Setup; Use I1 with fixed header-length of 7 bytes | Binary-based (I1) | SCC-AS as a B2BUA; leverage IMS centralised services architecture; |
| #12 | Context reuse, no precondition, signalling compression | Text-based (SIP) | P-CSCF as B2BUA |
| #13 | No precondition call | Text-based (SIP) | AS/MT SCC AS as B2BUA |
| #14 | Simplified call flow; Handle Precondition in Supported/ Require header | Text-based (SIP) | P-CSCF as B2BUA |
| #15 | SIP/SDP simplification | Text-based (SIP) | P-CSCF as B2BUA |
| #16 | Pre-configure SDP during SIP registration | Text-based (SIP) | None |
| #17 | SIP message compression/decompression; proactive media transcoding | Text-based (SIP) | AS as B2BUA, MF/MRF for transcoding |
| #18 | I1 is extended to include voice resource information | Binary-based (extended I1) | AS translate between extended I1 and SIP |
| #19 | Binary encoded Gm* ; registration carries call info | Binary-based (Gm*) | P-CSCF as SIP User Agent |
| #20 | Binary encoding of SIP over UNI based on I1 framework | Binary-based (I1-like) | UE proxy function in P-CSCF |

2 Discussion

2.1 Common optimization across solutions

Text-based optimizations (#12–#17) retain SIP compatibility and can coexist within current IMS framework.

Binary-based mechanisms (#18–#20) introduce new protocol encodings and translation entities.

Text-based SIP optimization (Solutions #12–#17):

-	Smaller SIP messages size (e.g., necessary SIP headers only, SDP pre-configuration).

-	Fewer SIP messages number (e.g., no preconditions).

-	Message compression (e.g., SigComp).

-	B2BUA roles in network (e.g., in P-CSCF / AS).

-	Optional reuse of context from registration.

Binary signalling approaches (Solutions #18–#20):

-	Replace SIP on the Gm interface with a binary‑based protocol (e.g., I1-like or Gm*), enabling optimized IMS call setup over NB‑IoT( GEO) .

-	Provide interworking functionality (e.g., P-CSCF, AS) to translate between the binary protocol and SIP .

2.2 Comparison of text-based and binary-based

| Aspect | Text-based SIP (#12–#17) | Binary-based solution (#18–#20) |
| --- | --- | --- |
| Protocol | SIP protocol fully defined in RFC 3261/4566 | New encoding, and I1-like or other protocol (not fully defined) |
| Impact to Network | Low impact. SIP and B2BUA has been well defined. | High impact, i.e. support newly defined IWK function or functionalities in IMS for the required protocol translation |
| Impact to UE | Mostly reuse Text/SIP | UE needs to support new protocol |
| Solution principle | Reduce message size with widely used compression method (e.g. SigComp) and reduce numbers of message exchange to fulfill the required call setup time | Replace SIP with a highly compacted Binary encoding to minimize message overhead to fulfill the required call setup time |
| Compatibility | Near fully backward compatible | Requires new protocol (e.g. I1 extension), new capability negotiation and function discovery |
| Maturity | Text/SIP has been widely deployed | Binary is not large-scale commercialized |
| Fullfill required call setup time (<30 s) | Yes | Yes |

3 Proposal

Based on the above observations, it is proposed to agree the interim conclusion as follows:

* First change (all New)*

8.1	Interim conclusions for Key Issue #2

To support KI#2 of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC, it is proposed to use Text-based SIP procedures together with B2BUA as the solution to fulfill the required call setup time.

Note 1: Which network function (e.g., P-CSCF, AS) to support B2BUA will be determined in normative phase.

Note 2: The specific method to compress the SIP message, e.g. using SigComp or defining necessary header fields, will be determined in normative phase.

* End of change (all New)*