SA WG2 Meeting #172	S2-2510619

17 – 21 November, 2025, Dallas, US	(revision of S2-250xxxx)

Source:	vivo

Title:	KI#2: update of the agreed principles

Document for:	Agreement

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update the principles for KI#2 for FS_EnergySys_Ph2.

1.	Introduction

In solution 7, it proposes to use the energy saving QoS profiles at RAN due to energy saving. The PCF generates the Energy Saving QoS profile(s) and the priority list for the QoS flow based on UE subscription information, AF's request and operator policy. Then, the PCF sends the energy saving QoS profiles and the priority list to the NG-RAN. The NG-RAN selects the Energy Saving QoS profile only if NG-RAN decides to downgrade the QoS parameters due to energy saving.

In this solution, the NG-RAN can choose the energy saving QoS profile due to specific energy saving status or energy constrained conditions at RAN. This solution takes the energy conditions at RAN into account for the decisions on applying the energy saving QoS profile. Considering the RAN node energy consumption is the main part of the whole system’s energy consumption, this solution could help the RAN node to save the energy due to specific energy information or status within the RAN. So it is proposed to support this solution during the normative phase.

Proposal: It is proposed to agree the Energy Saving QoS profile during the normative phase.

2. Text Proposal

It is proposed to add the following changes to the agreed principles for KI#2 in TR 23.700-67.

# * * * * First change * * * *

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#2. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

The interim agreements on principles for KI#2 are as follows:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information can be considered based on SA5 feedback on supporting renewable energy.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	SM policy control.

NOTE 3:	Based on operator policy, it should have the balance between energy saving and user experience.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

Editor's note:	Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 4:	Existing AF services can be updated but no new AF services will be introduced for the AF request.

- Based on UE subscription information and AF request, Energy Saving QoS profile(s) may be generated and forwarded to the NG-RAN for the purpose of network energy saving. Due to specific energy saving status or energy constrained conditions at NG-RAN, the NG-RAN may apply and match the Energy Saving QoS profile provided by the core network and notify the core network.

NOTE 5:	The QoS parameters within the Energy Saving QoS profile shall be based on the UE subscription and user consent.

Editor's note:	Whether to provide the Energy Saving QoS profile(s) to the NG RAN for the purpose of network energy saving depends on the feedback from RAN WGs.

# * * * * End of change * * * *