SA WG2 Meeting #172	S2-2510643

17 - 21 November, 2025, Dallas, USA                                                    was (S2-2509184)

Source:			Nokia, Turkcell, TurkTelekom

Title:			[WT#4] WT for Integrated Sensing And Communication

Document for:			Agreement

Agenda Item:			20.6.4

Work Item / Release:			FS_6G_ARC / Rel-20

Abstract of the contribution: WT revision proposal on ISAC for 6G based on outcome of SA2 171.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

At SA2 171 meeting, S2-2509793 paper was postponed with aggregable content and with some content requiring further clarification. Below is updated version which adds additional clarification related to the scope and also core NFs as consumers.

**** First Change ****

# Annex A.X. WT#4 Scope

WT#4: Study the Integrated Sensing and Communication over 3GPP access, assuming RAN node and UE operating as Sensing Entities in different sensing modes (aligned with TSG RAN) and multiple sources of sensing data.

The following aspects will be studied:

a)	Architecture and functional support for ISAC, including:

a.1)	Service authorization and revocation.

a.2)	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

a.3)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

a.4)	Collection, transport and storage of sensing related data.

a.5)	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How Core NFs make use of sensing results is not part of this WT.

b)	Whether and how to support sensing in consideration of the mobility of the UE as sensing entity.

c)	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of WT#4 includes the study of sensing modes (aligned with TSG RAN) and their selection, where sensing entities involve RAN node and UE.

NOTE 4:	WT#4 can involve coordination with WT#5.

NOTE 5:	Coordination with RAN WG for RAN aspects, SA3 for privacy and user consent aspects, and SA5 for OAM aspects will be needed.

NOTE 6:	The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

**** Second Change (all text new) ****

# 5.X. Key Issues

### 5.X.4

WT#4: Study the integration of Sensing and Communication over 3GPP access, assuming RAN node and UE operating as Sensing Entities in different sensing modes (aligned with TSG RAN) and multiple sources of sensing data.

The following aspects will be studied:

a)	Architecture and functional support for ISAC, including:

a.1)	Service authorization and revocation.

a.2)	Discovery and (re-)selection of Sensing function and Sensing Entities (i.e. RAN nodes, UEs), including sensing modes selection.

a.3)	Configuration parameters and policy provisioning to support sensing service, e.g., to the selected Sensing Entities, Sensing function, etc.

a.4)	Collection, transport and storage of sensing related data.

a.5)	Exposure framework of sensing result to the Sensing service consumer (i.e. AFs, UEs, Core NFs).

NOTE 1:	How Core NFs make use of sensing results is not part of this WT.

b)	Whether and how to support sensing in consideration of the mobility of the UE as sensing entity.

c)	Whether and how to support providing sensing data, which is not obtained from 3GPP radio signals, to the Sensing Function for sensing result generation.

NOTE 2:	This includes sensing data obtained using technologies that are different from 3GPP defined ISAC mechanisms, such as sensing based on IEEE 802.11, camera, sonar, lidar, etc.

NOTE 3:	The scope of WT#4 includes the study of sensing modes (aligned with TSG RAN) and their selection, where sensing entities involve RAN node and UE.

NOTE 4:	WT#4 can involve coordination with WT#5.

NOTE 5:	Coordination with RAN WG for RAN aspects, SA3 for privacy and user consent aspects, and SA5 for OAM aspects will be needed.

NOTE 6:	The scope of WT#4 will be aligned with RAN for 6G in Q1 2026. A checkpoint is set for Q1 2026 to revisit WT/KI descriptions and determine if solutions can be discussed in Q2 2026.

**** End of Changes ****