SA WG2 Meeting #172	S2-2510649

17 – 21 Nov, 2025, Dallas, US

Source:	Huawei, Hisilicon

Title:	DC3/DC4 Enhancement to Support Implicit Subscription and Notification for P2A and P2A2P DC establishment

Document for:	Agreement

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / R20

Abstract of the contribution: This contribution is intended to provide a new solution of Key Issue 3: Supporting dynamic configuration of MDC2 endpoints

# 1. Discussion

This pCR proposes a new the solution to the working task about the DC3/DC4 enhancement in Next Generation Real Time Communication Phase 3.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

## 6.x	Solution #x: DC3/DC4 Enhancement to Support Implicit Subscription and Notification for P2A and P2A2P DC establishment

### 6.x.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints. This solution enhances the current P2A and P2A2P ADC establishment procedure defined in TS 23.228 [2] Annex AC.7.2.2 and AC.7.2.3.

### 6.x.1	Description

This solution does not require the DC AS to subscribe to the call events from DCSF. Instead, the DCSF is implicated subscribed to and will notify the DC AS about the ADC establishment request event when receiving the notification from IMS AS. The DC AS will send the its MDC2 endpoint in the response if needed.

This solution also enhances and unifies the timing of notifying DC AS in the current P2A and P2A2P ADC establishment procedure defined in TS 23.228 [2] Annex AC.7.2.2 and AC.7.2.3.

### 6.x.2	Service definition

#### 6.x.2.1	General

The following table shows the DCSF Services defined.

Table 6.x.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_EventExposure | Notify | Subscribe/Notify | NEF, DC AS |

#### 6.x.2.2	Ndcsf_EventExposure

##### 6.x.2.2.1	General

Service description: This service enables the consumer to be notified when the given event is detected.

##### 6.x.2.2.2	Ndcsf_EventExposure_Notify

Service operation name: Ndcsf_EventExposure_Notify

Description: This service operation enables the DCSF to notify the comsumer about the subscribed event.

Inputs, Required: Session ID, Event ID

Three Event ID is currently defined: application data channel establishment request, application data channel establishment success, application data channel establishment failure.

Inputs, Optional: application binding information, MF-side MDC2 address, indication of requesting DC AS-side MDC2 address.

Outputs, Required: Result.

Outputs, Optional: DC AS-side MDC2 address.

NOTE:	The DC AS-side MDC2 address is required if the indication of requesting DC AS-side MDC2 address is present in the input.

### 6.x.3	Procedure

#### 6.x.3.1	P2A Application Data Channel Establishment Procedure

Figure 6.x.3.1-1 depicts the P2A application data channel establishment procedure.

Figure 6.x.3.1-1: P2A ADC establishment procedure.

1.	Steps 0-3 of TS 23.228 [2] Annex AC.7.2.2 applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the implicit subscription.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received. The DCSF needs to indicates that it is requesting an MDC2 endpoint address from the DC AS side.

4.	The DC AS sends 200 OK indicating it receives the notification. The DC AS should include its MDC2 endpoint address in the response.

5.	Steps 11-20 of TS 23.228 [2] Annex AC.7.2.2 applies.

6.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS.

7.	The DC AS sends 200 OK indicating it receives the notification.

8.	Steps 21-27 of TS 23.228 [2] Annex AC.7.2.2 applies.

#### 6.x.3.2	P2A2P Application Data Channel Establishment Procedure

Figure 6.x.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.x.3.2-1: P2A2P Application Data Channel Establishment Procedure.

1.	Steps 0-3 of TS 23.228 [2] Annex AC.7.2.3 applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the implicit subscription.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received. The DCSF needs to indicates that it is requesting an MDC2 endpoint address from the DC AS side.

4.	The DC AS sends 200 OK indicating it receives the notification. The DC AS should include its MDC2 endpoint address in the response.

5.	Steps 6-8 of TS 23.228 [2] Annex AC.7.2.3 applies. The DCSF shall send all the MDC2 media related information received from the DC AS to the MF. In this case, the MF receives two sets of MDC2 endpoints.

6.	Steps 10-18 of TS 23.228 [2] Annex AC.7.2.3 applies. The DCSF will instruct the MF via IMS AS to update the media related information after receiving SDP answer from the terminating side.

7.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS. In the P2A2P scenario, two sets of endpoint information shall be sent to the DC AS.

8.	The DC AS sends 200 OK indicating it receives the notification.

9.	Steps 19-26 of TS 23.228 [2] Annex AC.7.2.3 applies.

### 6.x.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	receiving notification from the DCSF which may include MDC2 related media information.

-	sending DC AS side MDC2 related media information to DCSF in the notification response.

DCSF:

-	notifying the session events to the DC AS.

-	sending MF side MDC2 related media information to DC AS.

-	receiving DC AS side MDC2 related media information from DC AS.

# * * * * End of changes * * *