3GPP SA WG2 Meeting #172	S2-2510685

Dallas, USA, 17 - 21 November 2025	(revision of S2-2509816)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6424 | rev | 5 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indirect Network Sharing for disaster condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodaone, earlier revisions were (China Unicom[, China Telecom, Samsung, ZTE, Ericsson,] Huawei[, Nokia, Vodafone]) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_NetShare_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2#171: The Indirect Network Sharing can be used for the UE of the participating operator with the disaster condition to reduce the service interruption. In the case that the RAN of participating operator may be broken down during the disaster condition, the shared RAN of the hosting operator can broadcast the PLMN ID representing the participating operator so that the UE of participating operator can access the network via Indirect Network Sharing. In order to support the Indirect Network Sharing for disaster condition, the NFs of participating operator network (i.e. UDM, AUSF, SMF) should be aware of which UE(s) are in disaster condition via the message sent from the hosting operator network instead of in the normal Indirect Network Sharing case. This CR proposes to introduce and describe the Indirect Network Sharing for disaster condition. SA2#172: This CR proposes to solve th EN and add the description for the case when the disaster condition is no longer applicable. ** The disaster roaming could be provided by multiple hosting operators. If these multiple hosting operators broadcast the same PLMN ID for the UEs from the participating operator, then UEs from the participating operator will move between these operators with cell reselection (i.e. not PLMN reselection) and (as the different hosting PLMNs are unlikely to have an inter PLMN, inter-AMF interface) the UEs will have their Tracking Area Updates rejected leading to a new Attach procedure. This will cause massive signalling load and can risk causing failure of the hosting operators! This shall be avoided. Hence if there are multiple hosting operators, then it will be essential that they broadcast different PLMN IDs towards the UEs of the participating operator. Note: If the participating PLMN has a two digit MNC (e.g. MNC=XY), then the hosting operators could broadcast three digit MNCs such as MNC=XY1 and XY2, etc. ** Once the disaster condition has ended, it is very important that UEs are returned to the participating operator’s own cells in a manner that ensures the signalling load on participating operator’s core network does not cause another failure of that participating network. The MINT phase 1 specifications handled this situation with the following text in section 5.40.5 of TS 23.501: A PLMN providing Disaster Roaming: /…/ - Shall organise the return of the Disaster Roaming UEs in a manner that does not cause overload (e.g. of signalling) in the PLMN that previously had the Disaster Condition. ,. |  |  |  |  |  |  |  |  |
| Summary of change: |  | SA2#171: Introduce the Indirect Network Sharing for disaster condition and describe the related function enhancement. SA2#172: Add the description to consider the case that more than one hosting operators can provide the Indirect Network Sharing for Disaster Condition provided that they broadcast different PLMN IDs towards the UEs of the participating operator. Add text to say that when the Disaster Condition is no longer applicable, the hosting operators shall ensure that they do not cause signalling overload in the participating operator’s network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Indirect Network Sharing for disaster condition cannot be supported. Indirect Network Sharing with multiple Hosting Operators can lead to failure of the hosting networks. Incomplete description of INS for Disaster Condition. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.x (new), 5.40.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The TS changes in this CR compared to rev 4 are highlighted in green. |  |  |  |  |  |  |  |  |

FIRST CHANGE (All the text is new)

### 5.18.x	Indirect Network Sharing for Disaster Condition

Subject to operator policy and national/regional regulations, the Indirect Network Sharing can be used by UEs of the PLMN with the Disaster Condition to minimize the service interruption.

NOTE 1:  In the case of Indirect Network Sharing for Disaster Condition, the UE is not aware of disaster condition and the UE behaviour is same as the normal case of Indirect Network Sharing as specified in clause 5.18.3.

NOTE 2:	How the hosting PLMN is made aware that another PLMN (as the participating operator network) is a PLMN with disaster condition and how the hosting PLMN is made aware of the area where the Disaster Condition applies is out of scope of 3GPP.

When the Disaster Condition arises, the AMF of the hosting operator’s network receives the NAS Registration Request message sent by the UE of the participating operator. The AMF identifies that the UE is registering to the network via the Indirect Network Sharing for Disaster Condition by checking that the UE is from a participating operator that can use Indirect Network Sharing for Disaster Condition and that the UE is registering to the network for the Indirect Network Sharing and the UE is in the area of Disaster Condition. If the above conditions are satisfied, comparing to the normal Indirect Network Sharing case, the AMF also provides the Disaster Roaming service indication to AUSF and UDM during the registration procedure.  In addition, the AMF also provides this indication to V-SMF during the PDU session related procedure and the V-SMF provides it to the H-SMF. The NFs of the participating operator network (e.g. AUSF, UDM and SMF) are able to identify which UE(s) are using the Indirect Network Sharing for Disaster Condition based on the Disaster Roaming service indication and the PLMN ID of the hosting operator.

Indirect Network Sharing for Disaster Condition can be provided by multiple hosting operators, however, in this case they shall not broadcast the same PLMN IDs towards the UEs of for the participating operator.

NOTE: 2a:	The use of different PLMN IDs is essential to ensure that the UE does not perform cell reselection between different AMFs of different hosting PLMNs (as this would lead to significant signalling load, e.g. mobility registration rejection followed by initial registration). Any mobility between hosting PLMNs would be performed by PLMN reselection.

NOTE 2b: If the participating PLMN, is using a two digit MNC, e.g. MNC=XY, then the hosting operators might be able to use 3 digit PLMN IDs, e.g., MNCs XY1 and XY2 for the UEs of the participating operator.

NOTE 3:  The Disaster Roaming service indication is specified in the clause 5.40.4.

NOTE 4:  Per-PLMN unified access control can be used to reduce load in the hosting operator’s network and allow the access priority for the specific UEs during the disaster condition.

When the Disaster Condition is no longer applicable, the hosting RAN can stop broadcasting of the PLMN ID representing the participating operator, the UE of the participating operator performs PLMN selection as described in TS 23.122 [17] and TS 24.501 [47] and returns to the PLMN previously with Disaster Condition. However, the hosting operator shall organise the return of the Disaster Roaming UEs in a manner that does not cause overload (e.g. of signalling) in the PLMN that previously had the Disaster Condition.

### NOTE 7:  In the above case, considering the service continuity, the existing mechanism of RFSP index (as specified in clause 5.3.4.3) can be used to redirect active mode UEs of the participating operator to home network previously with Disaster Condition.

SECOND CHANGE

### 5.40.1	General

Subject to operator policy and national/regional regulations, 5GS provides Disaster Roaming service for the UEs from PLMN(s) with Disaster Condition. The UE shall attempt Disaster Roaming only if:

-	there is no available PLMN which is allowable (see TS 23.122 [17]);

-	the UE is not in RM-REGISTERED and CM-CONNECTED state over non-3GPP access connected to 5GCN;

-	the UE cannot get service over non-3GPP access through ePDG;

-	the UE supports Disaster Roaming service;

-	the UE has been configured by the HPLMN with an indication of whether Disaster roaming is enabled in the UE set to "disaster roaming is enabled in the UE" as specified in clause 5.40.2; and

-	a PLMN without Disaster Condition is able to accept Disaster Inbound Roamers from the PLMN with Disaster Condition.

In this Release of the specification, the Disaster Condition only applies to NG-RAN nodes, which means the rest of the network functions except one or more NG-RAN nodes of the PLMN with Disaster Condition can be assumed to be operational.

NOTE:	In this Release of the specification, 5GS also provides the Indirect Network Sharing for Disaster Condition to minimize the service interruption. The detailed operation is specified in the clause 5.18.x.

End of CHANGES