3GPP TSG-SA WG2#172	S2-2510762

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm (Moderator)

Title:	Summary and way forward for the moderated discussion on 5G feature support by the 6G system

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper provides a summary and way forward for the moderated discussion on 5G feature support by the 6G system.

# 1.	Background

SA2#170 endorsed a process to determine 5G features to be supported by the 6G system [1]. It consists of the following steps:

- First step: companies requested to provide feedback on 5G features table (S2-2507876) (example feedback: missing 5G features, references) until SA2#171: - Email discussion triggered by the moderator on September 2nd (Tuesday) - Feedback from companies requested by September 19th, 2025 (Friday) 23:59 UTC. - Final document targeted for moderated discussion submitted for approval at SA2#171. - Second step: Companies to provide feedback on support of 5G features for first release of 6G specification (Rel-20/Rel-21 normative work) based on the table agreed in the first step (see slide 4) - Moderated discussion process timeline October 20th to 31st, 2025.

Step 1 and 2 have been completed. The company feedback of the second step has been merged into one Excel spreadsheet (see attachment).

# 2.	Discussion

For each feature, one of the following answers were possible, in line with the endorsed process [1]:

Yes – we want this feature to be supported for first release of 6G specification (Rel-20 study/Rel-21 normative work). This implies that the corresponding 5G feature x will be supported, but how to support it is up to 6G System Study. It also does not preclude adopting the 5G feature x with minimal or no changes for first release of 6G specification. - No – we do not want this feature to be supported for first release of 6G specification (Rel-20 study/Rel-21 normative work). - Neutral – not decided yet or do not have an opinion. - Not to be decided yet

It is worth noting that some companies who indicated "No" for some features clarified that a "No" meant that they did not need the feature and not necessarily that they did not want the feature to be specified. In other words, not all "No" votes imply an objection to including a 5G feature also for the first release of the 6G system.

It can be seen from the attached Excel sheet that the 5G features can be broadly organized in three categories:

-	(Almost) unanimous support: Many "Yes" votes and no or very few "No" or "Not to be decided" votes;

-	(Almost) no support: Many "No"/ votes and no or only very few "Yes" votes;

-	Disparate views: Many "No" votes but also many "Yes" or /"Not to be decided" votes.

# 3.	Proposal

For features with (almost) unanimous support:

-	Conclude that those features are to be supported by the first release of the 6G system.

-	Proposed process:

-	At SA2#172, include 5G features with (almost) unanimous support in a new Annex of TR 23.801-01 and add an architectural requirement into clause 4.2 (Architectural Requirements) that those feature shall be supported also by the first release of the 6G system.

-	At 	SA2#173, allow companies to propose updates to existing key issues or propose a new key issue (where deemed necessary) to study how to support those features in the 6G system.

-	Since some of these features have RAN dependency, send an LS to RAN2 and RAN3 to ask for feedback.

For features with (almost) no support:

-	Conclude that those features are not part of the first release of the 6G system.

For features with disparate views:

-	Postpone conclusion for those features to SA2#175.

-	Proposed process:

-	Agree a milestone for SA2#175 to decide for 5G features with disparate views whether to support those features in the first release of the 6G system.

The attached document shows the proposed classification for every 5G feature.

# 4.	References

[1]	Nokia, Qualcomm, Verizon, AT&T, Orange: Process for adopting 5G features for 6G day-1. S2-2508114.

# 2.	Text proposal

It is proposed to agree the following changes to TR 23.801-01:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[X]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2"

# >>>>NEXT CHANGE<<<<

## 4.2	Architectural Requirements

Editor's note:	This clause defines the architectural requirements that serve as the foundation for the study.

The 6G system shall support the 5G features listed in Annex X.

NOTE X:	How to support these features is up to the study. It is not precluded to adopt 5G features with minimal or no changes.

# >>>>NEXT CHANGE<<<<

# Annex X: 5G features to be supported by the first release of 6G

Table X-1: 5G features to be supported by the first release of 6G

|  | Feature name for 5GS | Reference to existing 5G feature description | Release |  |  |  |  | Impacts |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 15 | 16 | 17 | 18 | 19 | UE | UE | (R)AN | CN |
|  |  |  |  |  |  |  |  | (AS) | (NAS) |  |  |
| 1 | Specific services |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Application Triggering Services | TS 23.501 [2] clause 4.4.5 | X |  |  |  |  |  | X |  | X |
| 2 | Location services |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Location service For all services, location service architecture, MT-LR, MO-LR, and deferred MT-LR, LMF change procedures, privacy support, broadcast of location assistance data, etc. | TS 23.273 [X] clause 6 |  | X |  |  |  | X | X | X | X |
| 3 | 3GPP access specific aspects |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Paging strategy handling | TS 23.501 [2] clause 5.4.3 | X |  |  |  |  |  |  |  | X |
| 3.2 | UE radio capability information storage in the AMF | TS 23.501 [2] clause 5.4.4.1 | X |  |  |  |  |  |  | X | X |
| 3.3 | UE radio capability signalling optimisation (RACS) | TS 23.501 [2] clause 5.4.4.1a |  | X |  |  |  | X | X | X | X |
| 3.4 | UE Radio Capability Match Request | TS 23.501 [2] clause 5.4.4.2a | X |  |  |  |  |  |  | X | X |
| 3.5 | Paging assistance information | TS 23.501 [2] clause 5.4.4.3 | X |  |  |  |  |  |  | X | X |
| 3.6 | Core Network assistance information for RAN optimization | TS 23.501 [2] clause 5.4.6 | X |  |  |  |  |  |  | X | X |
| 3.7 | NG-RAN location reporting | TS 23.501 [2] clause 5.4.7 | X |  |  |  |  |  |  | X | X |
| 3.8 | LP-WUS with Paging Subgrouping Assistance | TS 23.501 [2] clause 5.4.12a |  |  |  |  | X | X | X | X | X |
| 3.9 | MICO mode | TS 23.501 [2] clause 5.4.1.3 |  | X |  |  |  |  | X |  | X |
| 4 | Session management related features |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session | TS 23.501 [2] clause 5.6.6 | X |  |  |  |  |  | X |  | X |
| 4.2 | Application Function influence on traffic routing | TS 23.501 [2] clause 5.6.7 | X |  |  |  |  |  |  |  | X |
| 4.3 | Selective activation and deactivation of UP connection of existing PDU Session | TS 23.501 [2] clause 5.6.8 | X |  |  |  |  | X | X | X | X |
| 4.4 | Session and Service Continuity (SSC) modes | TS 23.501 [2] clause 5.6.9 | X |  |  |  |  |  | X |  | X |
| 4.5 | Support of Ethernet PDU Session type | TS 23.501 [2] clause 5.6.10.2 | X |  |  |  |  |  | X |  | X |
| 4.6 | Support of Unstructured PDU Session type | TS 23.501 [2] clause 5.6.10.3 | X |  |  |  |  |  | X |  | X |
| 4.7 | UE presence in Area of Interest reporting usage by SMF | TS 23.501 [2] clause 5.6.11 | X |  |  |  |  |  |  |  |  |
| 4.8 | Use of Network Instance | TS 23.501 [2] clause 5.6.12 | X |  |  |  |  |  |  |  | X |
| 4.9 | Always-on PDU session | TS 23.501 [2] clause 5.6.13 | X |  |  |  |  |  | X |  | X |
| 4.10 | Support of Framed Routing | TS 23.501 [2] clause 5.6.14 | X |  |  |  |  |  |  |  | X |
| 4.11 | PDU Session User Plane Security | TS 23.501 [2] clause 5.10.3 |  | X |  |  |  | X | X | X | X |
| 4.12 | DNN replacement | TS 23.501 [2] clause 5.6.1 |  | X |  |  |  |  |  |  | X |
| 5 | Control Plane Load Control, Congestion and Overload Control |  |  |  |  |  |  |  |  |  |  |
| 5.1 | AMF control of overload | TS 23.501 [2] clause 5.19.5 | X |  |  |  |  |  | X |  | X |
| 5.2 | SMF control of overload | TS 23.501 [2] clause 5.19.6 | X |  |  |  |  |  | X |  | X |
| 5.3 | NAS level congestion control | TS 23.501 [2] clause 5.19.7 | X |  |  |  |  |  | X |  |  |
| 6 | Other features |  |  |  |  |  |  |  |  |  |  |
| 6.1 | 3GPP PS Data Off | TS 23.501 [2] clause 5.24 | X |  |  |  |  |  | X |  | X |
| 6.2 | Support of Tracing: Signalling Based Activation/Deactivation of Tracing | TS 23.501 [2] clause 5.25 | X |  |  |  |  |  |  | X | X |
| 6.3 | Architecture to support User Plane Information Exposure via a service-based interface | TS 23.501 [2] clause 4.2.16 |  |  |  | X |  |  |  |  | X |
| 6.4 | IPv6 Prefix Delegation via DHCPv6 | TS 23.501 [2] clause 5.8.2.2.4 |  |  |  | X |  |  |  |  | X |
| 6.5 | Enhancement of Application Detection Event Exposure | TS 23.503 [4] clause 6.1.3.18 |  |  |  | X |  |  |  |  | X |
| 6.6 | Support of roaming value-added services (Subscription-based routing to a particular core network, etc.) | TS 23.501 [2] clause 5.48 |  |  |  |  | X |  |  |  | X |
| 6.7 | MSISDN verification operation support to Nnef_UEId Service | TS 23.501 [2] clause 5.20 |  |  |  |  | X |  |  |  | X |
| 6.8 | Support of QoS monitoring capability configuration and transfer within 5GC | TS 23.501 [2] clause 5.45 |  |  |  |  | X |  |  |  | X |
| 6.9 | NF discovery and selection by target PLMN | TS 23.501 [2] clause 6.3.1 |  |  |  |  | X |  |  |  | X |
| 7 | Support for Ultra Reliable Low Latency Communication |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Per QoS Flow per UE QoS Monitoring | TS 23.501 [2] clause 5.33.3.2 |  | X |  |  |  | X |  | X | X |
| 8 | Support of deployments topologies with specific SMF Service Areas |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Support of deployments topologies with specific SMF Service Areas | TS 23.501 [2] clause 5.34 |  | X |  |  |  |  |  |  | X |
| 9 | Advanced Interactive Services (AIS) / eXtended Reality (XR) |  |  |  |  |  |  |  |  |  |  |
| 9.1 | AIS-related 5QIs | TS 23.501 [2] clause 5.37 |  |  | X |  |  |  | X | X | X |
| 9.2 | Policy control enhancements to support multi-modal services | TS 23.501 [2] clause 5.37.2 |  |  |  | X |  |  |  |  | X |
| 9.3 | L4S | TS 23.501 [2] clause 5.37.3 |  |  |  | X |  |  |  | X | X |
| 9.4 | Network Exposure of 5GS information | TS 23.501 [2] clause 5.37.4 |  |  |  | X |  |  |  |  | X |
| 9.5 | PDU Set based Handling | TS 23.501 [2] clause 5.37.5 |  |  |  | X |  | X | X | X | X |
| 9.6 | UL/DL policy control based on round-trip latency requirement | TS 23.501 [2] clause 5.37.6 |  |  |  | X |  |  |  | X | X |
| 9.7 | 5GS Packet Delay Variation monitoring and reporting | TS 23.501 [2] clause 5.37.7 |  |  |  | X |  |  |  | X | X |
| 9.8 | Periodicity and N6 Jitter Information associated with Periodicity | TS 23.501 [2] clause 5.37.8.2 |  |  |  | X |  |  |  | X | X |
| 9.9 | End of Data Burst Indication | TS 23.501 [2] clause 5.37.8.3 |  |  |  | X |  |  |  | X | X |
| 9.10 | Multi-modal Service ID to NG-RAN | TS 23.501 [2] clause 5.37.2 |  |  |  |  | X |  |  | X | X |
| 9.11 | PDU Set Information Marking Support | TS 23.501 [2] clause 5.37.5 |  |  |  |  | X |  |  |  | X |
| 9.12 | Support of transferring media related information over N6 | TS 23.501 [2] clause 5.37.9 |  |  |  |  | X |  |  |  | X |
| 9.13 | Supporting dynamically changing traffic characteristics via the User Plane | TS 23.501 [2] clause 5.37.10 |  |  |  |  | X |  |  | X | X |
| 9.14 | Packet filters for (S)RTP packet headers | TS 23.501 [2] clause 5.7.6.2 |  |  |  |  | X |  | X |  | X |
| 9.15 | Expose UL and DL data rate limitation information | TS 23.501 [2] clause 5.37.4 |  |  |  |  | X |  |  |  |  |
| 9.16 | Traffic identification and differentiated QoS for multiplexed media flows | TS 23.501 [2] clause 5.37.11 |  |  |  |  | X |  | X |  | X |
| 10 | Support of Energy Efficiency and Energy Saving |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Support of Energy Information Function (EIF) and related energy consumption information collection, calculation and exposure | TS 23.501 [2] clause 5.51.2 |  |  |  |  | X |  |  |  | X |
| 10.2 | Energy Saving indicator in UE Subscription | TS 23.501 [2] clause 5.51.5 |  |  |  |  | X |  |  |  | X |
| 10.3 | Policy control based on energy related information | TS 23.501 [2] clause 5.51.4, 5.51.6 |  |  |  |  | X |  |  |  | X |
| 10.4 | BDT based on network energy related information | TS 23.501 [2] clause 5.51.4 |  |  |  |  | X |  |  |  | X |

# >>>>END OF CHANGES<<<<