3GPP TSG-SA WG2#172	S2-2510766

Dallas, USA, 17-21 November 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	FS_Sensing_ARC: Additional principles for KI#4 on Sensing Data and the Associated Information Collection and Transport

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_Arc / Rel-20

Abstract of the contribution: This contribution proposed additional principles for the KI#4 on Sensing Data and the Associated Information Collection and Transport.

# 1.	Discussion

In SA2#171, several general principles were agreed for the KI#3 for FS_Sensing_Arc on Sensing Data and the Associated Information Collection and Transport. This contribution proposed the additional principles based on the architecture discussion in KI#1.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-14:

# >>>>BEGINNING OF CHANGES<<<<

### 7.1.4	Agreed Principles for KI#4: Sensing Data and the Associated Information Collection and Transport

The following principles are agreed for KI#4:

-	Based on the received sensing service requirements from the Sensing Function, the Sensing Entity (gNB) can determine how to perform the relevant sensing measurement to collect the Sensing data.

-	Sensing Entity can support one time, periodical Sensing Data to the Sensing Function.

Editor's note:	The type (one time, periodical, etc.) and content of Sensing Data that will be provided by gNB needs to be coordinated with RAN WGs in the normative phase.

-	Sensing data can be collected from one or multiple Sensing Entities, i.e. gNBs, by Sensing Function for the same sensing service request.

-	The Sensing Function can generate sensing results based on the information provided by the Sensing Entity (gNB) node(s).

-	The Sensing Entity sends the sensing data to the Sensing Function via a direct connection, under the instruction of the Sensing Function.

-	The sensing data transport is agonistic to the format of the sensing data.

NOTE 1: The sensing protocols defined by RAN WGs handle the different sensing data formats/measurement levels.

NOTE 2:	The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

# >>>>END OF CHANGES<<<<