3GPP TSG SA WG2 Meeting #172	S2-2510780

17 - 21 November, 2025, Dallas, USA

Source:	Google

# Title:	KI#2, New Sol: Enabling Emergency Response Centre callback for Roaming UEs

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution to support local routing of MT SMS from ERC to UE in roaming scenarios with SMS over NAS.

1. Introduction

SA2#170 has approved the assumptions and Key Issues for the study on Short Message Service to Emergency Response Centre.

Under KI#2, SA2 agreed to study on how to support SMS delivery to Emergency Response Centre via NAS:

- how to support Emergency Response Centre callback for sending short messages using regular SMS delivery service.

This contribution addresses KI#2, specifically focusing on roaming scenarios.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-65.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X |  | X |  |

# * * * * Second change * * * *

## 6.X	Solution #x: Enabling local routing of MT SMS from Emergency Response Centre for Roaming UEs

### 6.X.1	High level principles

This solution addresses KI# 2.

The following high-level principles are applied to this solution:

-	The essential principle of this solution is for the VPLMN to allocate a temporary local MSISDN (i.e. phone number) to the roaming UE. The UE uses this MSISDN in the SMS payload, so that the ERC and SMSC treat the MO SMS message as originating from a local MSISDN (i.e., a MSISDN allotted by the VPLMN) and the ERC uses this MSISDN in SMS payload for MT SMS callback to roaming UE.

-	Each SMSF, which supports the SMS2EC feature, is configured with a distinct set of local MSISDNs to support roaming UEs.

-	The UDM is pre-configured with the mapping between the local MSISDNs used for roaming UEs and the SMSF that the local MSISDN is allotted to.

-	The SMSF temporarily assigns one of the local MSISDNs under its control to the roaming UE during the Registration procedure.

-	When a roaming UE intends to send an MO SMS message to the ERC, it utilizes the temporary local MSISDN and SMSC address of the VPLMN, obtained during the Registration procedure, to create the SMS payload.

-	For MT SMS callback, the ERC uses the temporary local MSISDN of the UE as the destination (in TP-DA field) which was received in the MO SMS payload. The SMSC uses the routing information stored in the UDM to route the MT SMS message to the appropriate SMSF.

-	No impact on the SMSC or ERC is anticipated.

-	The core network functions (AMF/SMSF) do not modify the SMS payload (i.e. SMS-TPDU payload) during transit.

The above principles are applied to EPS with MME taking the role of SMSF for SMS over SGd, MSC taking the role of SMSF for SMS over SGs, and HLR/HSS taking the role of UDM in the VPLMN.

### 6.X.2	Description

This solution proposes to enable ERC SMS callback functionality for inbound roaming UEs through local routing within the VPLMN.

The solution requires the following enhancements to the existing protocols and network functions:

- Selection of SMSF:

If the network and UE support SMS2EC, the AMF selects the SMSF for SMS delivery between the UE and local ERC in the VPLMN. The SMSF selection is based on AMF configuration or by discovering the SMSF via NRF if no configuration exists.

- Temporary local MSISDN allocation:

The temporarily local MSISDNs are assigned to UEs for future use when routing MT SMS from an ERC to a roaming UE. Each SMSF in the PLMN, which supports the SMS2EC feature, is allocated a portion of these local MSISDNs, and the UDM is configured with the mapping of local MSISDNs to each of these SMSFs.

On receipt of an SMS service activation for a roaming UE during the Registration procedure, the SMSF of VPLMN allocates temporarily local MSISDN to the UE from the pool of local MSISDN under its control. This temporary local MSISDN is then sent to the AMF, which forwards the number to the UE in the Registration Accept message. In addition, the SMSC address of the VPLMN is also sent to the UE.

The SMSF of VPLMN releases the assigned temporary local MSISDN to the roaming UE when it receives the SMS service deactivation request or the UE detaches from the network.

- MO SMS to ERC within VPLMN:

When the UE sends a MO SMS to the ERC, it utilizes the temporary local MSISDN and SMSC address of the VPLMN, obtained during the Registration procedure, to create SMS payload.

- MT SMS callback from ERC routing within the VPLMN:

The MT SMS originating from the local ERC uses the temporarily assigned local MSISDN of the roaming UE received in the MO SMS, as its destination (in TP-DA field of SMS payload). No new functionality is required since the ERC treats the received MO SMS as a regular SMS originating from a local MSISDN.

The SMSC of the VPLMN retrieves the UE routing information from the UDM, utilizing the temporarily assigned local MSISDN of the roaming UE received in the MT SMS from the ERC, to identify and facilitate delivery of MT SMS further to the appropriate SMSF. No new functionality in required at the SMSC.

The SMSF then forward the MT SMS to the AMF for delivery to the roaming UE. The AMF then looks up the destination (based on the mapping between temporary local MSISDN and UE identity) and delivers the MT SMS to the UE.

The above description also applies to EPS with MME taking the role of SMSF for SMS over SGd, MSC taking the role of SMSF for SMS over SGs, and HLR/HSS taking the role of UDM in the VPLMN.

### 6.X.3	Procedures

This clause describes procedures to facilitate local routing of MT SMS from ERC to UE in roaming scenarios via the SMSF of the VPLMN.

#### 6.X.3.1	ERC MT SMS callback routing within the VPLMN – in 5GS

Figure 6.X.3.1-1 illustrates a high level MT SMS procedure from an ERC to UE utilizing SMS over NAS, to be used by roaming UEs.

Figure 6.X.3.1-1: MT SMS procedure from ERC to UE with temporarily assigned local MSISDN as destination

Precondition:	The MO SMS from UE containing a temporarily assigned local MSISDN of the user is delivered to the ERC (Considering the solutions for KI#2 for MO procedures in 3GPP TR 23.700-65) with additional modifications described in this step.

The UE receives a temporary local MSISDN and SMSC address of the VPLMN during the Registration procedure.

The UE generates an SMS-SUBMIT TPDU for MO SMS, containing the local emergency number within the TP-DA field and sends it within a NAS message, as defined in step 1 to step 2a of clause 4.13.3.3 of TS 3GPP 23.502. The UE includes the temporary local MSISDN and the SMSC address in this MO SMS instead of its HPLMN-assigned MSISDN and SMSC address. The SMSF forwards this MO SMS to the SMSC for final delivery to the appropriate local ERC.

1-3.	The ERC replies to the UE's initial MO SMS by sending a MT SMS callback destined for the UE's temporarily assigned local MSISDN. The local SMSC receives the MT SMS callback and identifies the SMSF address by querying the UDM of the VPLMN using the UE’s temporarily assigned local. The MT SMS is then delivered to the appropriate SMSF. The MT SMS interaction between the SMSC/SMS-GMSC/UDM follows the procedure as defined in step 1 to step 3 of clause 4.13.3.6 of TS 3GPP 23.502.

4.	The SMSF forwards the MT SMS to the AMF. Steps 4a-7 of clause 4.13.3.6 of TS 3GPP 23.502 are performed to deliver the MT SMS to the UE and to forward the delivery report to the ERC respectively. The AMF uses the mapping between the temporary local MSISDN and UE identity while invoking the Namf_Communication_N1N2MessageTransfer service operation in step 5a-5g of clause 4.13.3.6 of TS 3GPP 23.502.

#### 6.X.3.2	Deregistration procedure for local MSISDN deallocation – in 5GS

The UE shall perform the procedure as described in the clause 4.13.3.2 of TS 3GPP 23.502 with AMF invoking Nsmsf_SMService_Deactivate service operation to trigger the release of UE Context including the mapping information of temporarily assigned local MSISDN to UE identity and release the temporarily assigned local MSISDN.

### 6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

- Support the use of temporary local MSISDN and network signalled SMSC of the VPLMN during roaming.

AMF:

- Support to select an SMSF that supports local routing for ERC SMS callback for roaming UEs.

- Support the use of a temporary MSISDN mapped to the UE identity.

SMSF:

- Manage the use of temporary MSISDNs assigned to the SMSF.

UDM:

- The UDM is pre-configured with the mapping of local MSISDNs to SMSF in the serving PLMN.

The above impacts also apply to EPS with MME taking the role of SMSF for SMS over SGd, MSC taking the role of SMSF for SMS over SGs, and HLR/HSS taking the role of UDM in the VPLMN.

# * * * * End of change * * * *