3GPP TSG-WG SA2 Meeting #172	S2-2510806

Dallas, USA, 17th Nov – 21st Nov, 2025	(revision of S2-xxxxxx)

Source:	T-Mobile USA

Title:	Proposed Text for WT 1.4 Voice Services in 6G

Document for:	Approval

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC / R20

Abstract of the contribution: Proposes text for WT 1.4 “Essential Services” for the KI’s related to 6G Voice and Emergency Voice.

# Discussion

One of the remaining controversial issues for WT 1.4 “Essential Services” is related to “fallback”.  T-Mobile USA believes that the inclusion of fallback services for 5G ultimately delayed introduction of native 5G services, since when a smartphone received a call it would fallback to 4G and hence loose 5G service, and thus any 5G feature operating would be interrupted.  Hence in the 6G study T-Mobile would prefer to only support native 6G voice.

T-Mobile does recognise that in some cases not all capabilities to support voice may not always be present in the 6GS, and hence some companies desire a mechanism to support voice services.  In the past this was done using fallback, a solution developed for 5G based on a solution to support circuit switched systems for the transition to 4G.  It is evident that many new features and capabilities were introduced in the 4G and 5G era, and thus T-Mobile suggests that a simple adoption of the fallback solution is not ideal, when other solutions using capabilities developed in the 4G and 5G era’s may be a better fit to meet T-Mobiles goal of native 6G voice, while allowing for companies that prefer to re-use 5G voice capabilities in their initial 6G deployments.

Hence T-Mobile is open to studying 5G voice interactions with 6G in a more general manner than just repeating a solution from a previous era transition.

T-Mobile USA would also like to study what is the preferred means to handle Diameter bases IMS interfaces in the 6G era.  Should we continue to support specifications of new functionality using Diameter based interfaces or should new functionality be only supported using SBI equivalents.

It is unclear to T-Mobile what if any co-ordination with SA4 will be required at this stage since no new IMS services are being described in this WT.  Additionally, since the majority of the interworking and Migration will be handled by WT#2, it is unclear if NOTE 4a is necessary at this time.  Coordination will occur using the normal manner if impacts or feedback is required from any group during the study.

# Proposal

It is proposed to use the following text for the KI related to 6G Voice and Emergency Voice in TR 23.801.

# 5.P. Key Issue #P: Voice Services for 6G

This Key Issue will investigate how to support voice services for the 6G system (referring to the mechanisms used in  5GS to support voice services as the starting point for discussion) – the following aspects will be studied:

1.	How a UE supporting 6G RAN selects and connects to a network for voice service.

2.	Whether and how to update the IMS specification to support an IPCAN in the 6GS.

3.	How the 6GS supports IMS based voice over the 6G access network

4.	How to support the voice continuity between 6G RAT and selected 3GPP  RATs or between 6G RAT and selected non-3GPP Accesses.

5.	Whether and how to support for emergency voice services if not natively provided by 6GS.

6.		Whether and How to continue to specify Diameter based interfaces for IMS for new services in the 6G era.

NOTE 1:	Any NTN specific aspects for voice will be studied as part of Key Issue related to WT#7

NOTE 2:	Interworking, migration and service continuity aspects need to be coordinated with WT#2.

NOTE 3:	Support of voice services via 6G NTN need to be coordinated with WT#7.

NOTE 4: The existing IMS architecture is used as a starting point for the study.

# 5.Q. Key Issue #Q: Emergency Voice Services for 6G

This key issue will investigate how to support emergency voice services over the 6G system using the capabilities defined for voice service in Key issue P.

Additionally, this key issue will study:

1.	The functionality required in the UE and the 6G system to enable emergency voice service for 6GS.

2.	The functionality required in the UE and the 6G system to establish emergency session via 6GS for the emergency voice service.

3.	How to identify emergency services in 6GS in case of non UE detectable emergency call, such that the required priority for an emergency call is applied.

4.	How to support a UE in Limited-Service state so that it can establish emergency session via 6GS.

5.	Whether and how to support for emergency voice services if not natively provided by 6GS.