3GPP TSG-SA WG2 Meeting #171	S2-2510844

Dallas, USA, 17 – 21 November 2025

Source:	Ericsson

Title:	KI#3, New Solution: SMS over IP delivery to Emergency Response Centre with emergency numbers

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution provides a solution for SMS over IP delivery to Emergency Response Centre KI#3.

# 1.	 Proposal

It is proposed to update TR 23.700-65 with below changes:

* * * Start of change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 22.101: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; Service aspects; Service principles".

[7]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[8]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[9]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[10]	ATIS J-STD-110: "Joint ATIS/TIA Implementation Guideline for J-STD-110, Joint ATIS/TIA Native SMS/MMS Text to 9-1-1 Requirements and Architecture Specification".

[11]		GSMA NG.119: "Emergency Communication".

[12]	3GPP TS 23.221: "Architectural requirements".

[13]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[14]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[15]	3GPP TS 23.540: "5G System: Technical realization of Service Based Short Message Service; Stage 2".

[16]	3GPP TS 29.338: "Diameter based protocols to support Short Message Service (SMS) capable Mobile Management Entities (MMEs)".

[17]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[18]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[19]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[2x]	3GPP TS 24.229: "IP multimedia call control protocol based on SIP and SDP; stage 3".

* * * Next change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| #x | x |  | x |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

* * * Next change  (all new)* * * *

## 6.X	Solution #X: SMS over IP delivery to Emergency Response Centre based on emergency call numbers

### 6.X.0	High level principles

This solution aims to address KI#3 and KI#1 with the following key principles:

-	The UE receives the support of SMS delivery to Emergency Response Centre indication during the IMS registration in case of non-roaming scenario, or during emergency IMS registration from VPLMN in case of roaming.

-	The UE uses emergency PDU session to support the SMS over IP delivery to Emergency Response Centre (ERC) and performs emergency IMS registration prior to the SMS over IP delivery to Emergency Response Centre.

-	The IP-SM-GW in the serving PLMN is responsible to deliver the SMS to the SMS service centre based on the emergency number inserted in RP-DA

-	The serving SMS service centre is responsible to forward the SMS to the local Emergency Response Centre.

### 6.X.1	Description

In this proposal, it is assumed that emergency PDU session is used to deliver the SMS over IP to Emergency Response Centre/PSAP. It is also assumed that UE is aware of the supporting capability of the network based on the solutions provided for KI#1 (e.g. UE is informed that SMS over IP delivery to Emergency Response Centre is supported by the network during IMS registration) for the non-roaming case. For the roaming case, the UE realizes the support SMS over IP delivery to Emergency Response Centre during the emergency IMS registration.

The main change is IP-SM-GW deliver the SMS to the SMS service centre based on the RP-DA field.

### 6.X.2	Emergency IMS registration procedure

#### 6.x.2.1	Emergency PDU session/PDN connection establishment in 5GS/EPS

In order to use the SMS over IP delivery to Emergency Response Centre, the UE triggers Emergency PDU Session /PDN connection establishment procedure as specified in clause 4.3.2 of TW 23.502 [3] and clause 5.10.2 of TS 23.401 [5].

#### 6.x.2.1	Emergency IMS registration in non-roaming scenario

In order to use the SMS over IP delivery to Emergency Response Centre in non-roaming scenario, for UE-detectable SMS delivery to Emergency Response Centre, the UE performs emergency IMS registration first as described in clause 7.2 of TS 23.167 [9].

#### 6.x.2.1	Emergency IMS registration in roaming scenario

In order to use the SMS over IP delivery to Emergency Response Centre in roaming scenario, for UE-detectable SMS delivery to Emergency Response Centre, the UE performs emergency IMS registration as described in Annex K.3 of TS 23.167 [9] with following additions:

Step 8: When P-CSCF responds with 420 or 403, the P-CSCF also includes an indication if SMS over IP delivery to Emergency Response Centre is supported based local policy configuration.

When network supports SMS over IP delivery to Emergency Response Centre (ERC), and the Emergency IMS registration is triggered by UE due to the need of SMS over IP delivery to ERC, then UE continues step 9-12 or step 13-15.

NOTE: for the SMS over IP delivery to ERC in step 11-12 and step 13-15 are further detailed in clause 6.x.3 below

### 6.X.3	MO SMS over IP delivery procedures

The following procedures illustrate the MO SMS over IP delivery to PSAP with reference to TS 23.167 [9].

#### 6.x.3.1	MO SMS over IP delivery to Emergency Response Centre using Emergency PDU session in 5GS

Figure 6.3: Successful encapsulated Short Message origination procedure

1.	If the UE decides to send SMS over IP, then it sends a SIP MESSAGE containing the SMS to PSAP to the P-CSCF in the serving network containing an R-URI set to SOS URN or an emergency number. The UE may include also the UE location information in the SIP message. The UE include emergency number in RP-DA.

2.	The P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling SMS over IP delivery to PSAP.

3.	E-CSCF optionally queries the LRF to get user location information and determines a suitable PSAP address.

NOTE 1: 	the optional LRF involvement is useful in markets where the PSAP to handle the emergency is determined based on the user location (e.g. cell -id)

4.	E-CSCF sends the SIP MESSAGE to the IP-SM-GW . The MESSAGE message may optionally include the  PSAP address.

5.	The IP-SM-GW receives SIP message and optionally the PSAP address from E-CSCF and sends the SMS payload to SMS-C including the PSAP address information (if received).

Editor’s Note: the way to deliver the PSAP address to the SMS-C is FFS.

6.	SMSC may acknowledge to the IP-SM GW.

7.	SMSC sends the SMS to local PSAP. Optionally, SMSC can use  the PSAP address received in step 5 in the routing process.

8.  The successful response code Accepted is sent from IP-SM-GW to E-CSCF -> P-CSCF -> UE.

#### 6.x.3.2	MO SMS over IP delivery to Emergency Response Centre using Emergency PDN connection in EPS

The SMS over IP delivery to Emergency Response Centre is similar to 5GS procedure with the change that the Emergency PDN connection is established to provide the IP tunnel.

### 6.X.4	Non-detectable SMS to PSAP handling

Like UE non-detectable voice calls, the UE would use the IMS PDN connection/PDU session for such cases. The network can be based on configuration, reject such attempts if the destination number is recognized as an emergency centre.

While for normal voice calls this is a function of HPLMN P-CSCF but to avoid performance penalties for P-CSCF having to scan all SIP MESSAGE messages, it is proposed that such function is performed by HPLMN IPSMGW. The IPSMGW is proposed to perform similar procedures as specified for P-CSCF in TS 24.229 [2x], §5.2.10.5 and the UE to follow similar procedures as specified in TS 24.229 [2x], §5.1.2A.1.1.

This would apply to both roaming and non-roaming scenarios.

### 6.X.5	Impacts to Services, Entities and Interfaces

UE:

-	Support the handling of SMS over IP delivery to Emergency Response Centre capability during IMS registration procedure, or during emergency IMS registration procedure.

-	Formulates the RP-DA in the RP-DATA with serving PLMN emergency number and formulate the SIP message containing the SMS to PSAP.

P/S/E-CSCF:

-	Support SMS over IP delivery to Emergency Response Centre capability handling during IMS  registration/emergency IMS registration and delivery of SMS to PSAP via IP-SM-GW.

IP-SM-GW:

-	Support of SMS delivery to SMS service centre of the serving PLMN based on emergency number with PSAP address.

-	Support the handling of rejection in case of non-detectable SMS delivery to PSAP (e.g. e.g. returning a response with 380, the same way as P-CSCF handling the non-detectable emergency call).

* * * End of change * * * *