3GPP TSG-WG SA2 Meeting #172	S2-250xxxx

Dallas, TX, USA, 17 – 21 November 2025                                                       (revision of S2-2509221)

Source:	Apple

# Title:	KI#3, New Sol: Short Message to Emergency Response Centre via SMS over IP

# Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#3.

# 1. Discussion

This contribution proposes a new solution for KI#3: Short Message to Emergency Response Centre via SMS over IP.

# 2. Text Proposal

It is proposed to add this solution to TR 23.700-65.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| x |  |  | x |

# * * * * Second change * * * *

## 6.X	Solution #x: Short Message to Emergency Response Center via SMS over IP

### 6.X.0	High level principles

This solution addresses KI#3.

This solution applies when the UE detects the SM to Emergency Response Centre based on the used emergency numbers.

The Short Message to Emergency Response Centre uses an emergency number or SOS URN as destination address. A key requirement for this service is that the Short Message (SM) to Emergency Response Centre is routed to a PSAP in the country where the UE is located.

The serving PLMN sends an indication towards the UE during IP-CAN registration, e.g., during the 5GS Registration procedure or during E-UTRAN Attach/TAU procedure, to indicate support for SM to Emergency Response Centre over IMS.

When the UE does not receive the indication that SM to Emergency Response Centre over IMS is supported in the serving PLMN, it attempts to send the SM to Emergency Response Centre over NAS, if configured in the UE.

NOTE X: If SM to Emergency Response Centre is not supported in the serving network, it is up to UE implementation how to proceed. The UE can try to choose another network to send the SM to Emergency Response Centre. The UE can also try to establish an emergency call or indicate to the user that SM to Emergency Response Centre is not possible.

### 6.X.1	Description

The UE sends a SIP MESSAGE request to the P-CSCF containing a Request-URI set to SOS URN or an emergency number. The SIP MESSAGE contains the SM to the Emergency Response Centre.

The P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling the SM to the Emergency Response Centre.

The E-CSCF includes the PSAP address in the Request-URI in the SIP MESSAGE request and forwards it to the IP-SM-GW configured in the E-CSCF.

The IP-SM-GW selects a local SMSC and forwards the SM to the Emergency Response Centre via MAP-MO to the SMSC including the PSAP address.

The SMSC selects an appropriate Emergency Response Centre and sends the SM to the Emergency Response Centre.

### 6.X.2	Procedures

#### 6.X.2.1	Short Message routing to Emergency Response Centre via SMS over IP

The serving PLMN sends an indication towards the UE during IP-CAN registration, e.g., during the 5GS Registration procedure or during E-UTRAN Attach/TAU procedure, to indicate support for SM to Emergency Response Centre over IMS. If the serving network has provided this indication, then UE performs this procedure to send an SM to an Emergency Response Centre:

Figure 1: Procedure for Short Message to Emergency Response Centre via SMS over IP

1.	The UE sends a SIP MESSAGE request to the P-CSCF containing a Request-URI set to SOS URN or an emergency number. The SIP MESSAGE contains the SM to the Emergency Response Centre.

2.	The P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling the SM to the Emergency Response Centre.

3.	The E-CSCF optionally queries the LRF to get user location information and selects a proper PSAP.

4.	The E-CSCF includes the PSAP address in the Request-URI in the SIP MESSAGE request and forwards it to the IP-SM-GW configured in the E-CSCF.

5.	The IP-SM-GW selects a local SMSC and forwards the SM to the Emergency Response Centre via MAP-MO to the SMSC including the PSAP address.

6.	The SMSC sends the SM to the Emergency Response Centre to the PSAP.

  - The successful response code 202 Accepted is sent from the IP-SM-GW to E-CSCF -> P-CSCF -> UE.

### 6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

- The UE needs to support processing an indication whether serving PLMN supports the SM to Emergency Response Centre over IMS.

P-CSCF:

- The P-CSCF needs to select an E-CSCF and route the SIP MESSAGE request.

E-CSCF:

- The E-CSCF needs to includes the PSAP address in the SIP MESSAGE request carrying the SM to the Emergency Response Centre and forward the SIP MESSAGE request to a configured IP-SM-GW.

IP-SM-GW:

- IP-SM-GW needs to select a local SMSC and forward the SM to the Emergency Response Centre via MAP-MO to the SMSC including the PSAP address.

# * * * * End of change * * * *