3GPP TSG-WG SA2 Meeting #172	S2-2510813

Dallas, TX, USA, 17 – 21 November 2025                                                       (revision of S2-25xxxxx)

Source:	Apple

# Title:	KI#2: Interim Conclusions

# Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes interim conclusions for KI#2.

# 1. Discussion

This contribution proposes interim conclusions for KI#2: Support of SMS Delivery to Emergency Response Centre via NAS.

# 2. Text Proposal

It is proposed to add this solution to TR 23.700-65.

# * * * * First change * * * *

# 8	Conclusions

Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities.

## 8.x	Interim conclusions for KI#2

Editor's note: This is not an exhuastive list containing all the conclusions necessary to conclude KI#2.

-	For sending a SM to Emergency Response Centre, the UE generates the SMS-SUBMIT TPDU with the local emergency number included in the RP-DA field.

-	The MME/SMSF routes the SM to Emergency Response Centre to the serving network’s SMSC based on the RP-DA field which includes the emergency number. The MME/SMSF replaces the RP-DA field with a local SMSC address.

-	The local SMSC identifies the type of emergency service based on the emergency number and routes the SM to the corresponding Emergency Response Centre.

-	When the UE does not receive the indication that SM to Emergency Response Centre over NAS is supported in the serving PLMN, it attempts to send the SM to Emergency Response Centre over IMS, if configured in the UE and supported by the network.

-	When the UE does not receive the indication that neither SM to Emergency Response Centre over NAS, nor over IMS is supported in the serving PLMN, the UE can attempt to select another PLMN in order to send a SM to Emergency Response Centre.

# * * * * End of change * * * *