3GPP TSG-WG SA2 Meeting #172	S2-2510816

Dallas, USA, 17th Nov – 21st Nov, 2025	(revision of S2-2510801)

Source:	Rapporteur (T-Mobile USA)

Title:	Cleanup of Solution Descriptions

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / R20

Abstract of the contribution: Some solution descriptions agreed at SA2 #171 were difficult to implement into TR 23.700-56, while some notes were added MCC requested further edits to better align with drafting guidelines for TR documents.  This pCR attempts to update the Solution descriptions to align with these suggestions.

# Discussion

Following SA2 #171 the agreed solutions were implemented as per the approved p-CR’s (with a few added notes for readability), however MCC informed the rapporteur that several sections were now misaligned with TR drafting rules, and suggested that changes should be made to align with the drafting rules.  This document attempts to update some solutions to better align with the drafting rules.

# Proposal

The following changes to TR 23.700-56 are proposed:

*** Start of Changes ***

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #5 | X |  |  |
| #6 | X |  |  |
| #7 |  | X |  |
| #8 |  | X |  |
| #9 |  | X |  |
| 10 |  |  | X |
| 11 |  |  | X |
| 12 |  |  | X |
| 13 |  |  | X |
| 14 |  |  | X |

*** Second Change ***

## 6.9	Solution #9: Capability exposure of IMS DC events supported by DCSF

### 6.9.0	High-level solution Principles

This solution addresses Key Issue #2: Capability exposure of IMS DC events supported by DCSF.

The main principle of this solution for support capability exposure of IMS DC events supported by DCSF as follows:

-	The trusted AF or untrusted AF via NEF shall discover and select the DCSF.

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of supported events with DCSF and get notified when DCSF detects the events.

Editor's note:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach is FFS.

### 6.9.1	Description

This solution proposes how the trusted AF and untrusted AF via NEF discovers the DCSF and subscribes to get notification for the supported events by DCSF.

Discovery of DCSF:

The trusted AF can directly discover the DCSF with the help of HSS and in case of untrusted AF, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. Once HSS comes to know the DCSF address, it notifies the NEF/AF.

Subscription and Notification with DCSF:

Upon successful discovery, AF/NEF shall subscribe with DCSF for the supported events and get notified when DCSF detects the event.

Table 6.9.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Successful Application download to UE | Detected when an Application Data Channel is established. DCSF detects that UE has been successfully downloaded one specific application | DCSF |

NOTE:	This solution identifies changes to existing specifications, these changes are shown using RED text.

### 6.9.2	Service definition

#### 6.9.2.1	General

The following table shows the required service operation.

##### 6.9.2.1.1	HSS Service

The following table shows the services exposed by an SBI capable HSS.

Figure 6.9.2.1.1 Updated Table AA.2.1.1-1 (from TS 23.228 [2]): IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| ImsSubscriber Data | Subscribe | Subscribe/Notify | S-CSCF, AS |
| Management (_ImsSDM) | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
|  | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS |
| Ims UE Context | Authorize | Request/Response | I-CSCF |
| Management (_ImsUECM) | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, AF |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, AF |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, AF |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

6.9.2.1.1.0	Nhss_ImsUECM_Registration service operation (modified)

Clause AA.2.1.2.1 of TS 23.228 [2] is modified to:

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id if the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

6.9.2.1.1.1	Nhss_ImsUECM_DcsfInfoGet service operation

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.1.2	Nhss_ImsUECM_DcsfUnsubscribe

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.1.3	Nhss_ImsUECM_DcsfNotify

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.9.2.1.2	DCSF Service

6.9.2.1.2.1	Ndcsf_SessionEvent

6.9.2.1.2.1.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

6.9.2.1.2.1.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event ID is currently defined: successful download of specific application by UE.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application ID

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

6.9.2.1.2.1.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.9.2.1.2.1.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, DC Application ID.

Inputs, Optional: Calling party and called party ID.

Outputs, Required: Result.

### 6.9.3	Procedure

#### 6.9.3.1	DCSF instance registration in HSS

Figure 6.9.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.9.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

Editor's note:	It is unclear which clause is being referenced here.

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE:	Figure 6.9.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.9.3.2	DCSF address discovery by AF

Figure 6.9.3.2-1 depicts a signalling flow diagram for discovering DCSF address by AF.

.

Figure 6.9.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as per Fig1 solution.

2.	AF discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5.	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to AF.

Editor's note:	It is FFS how to address the scenario where the IMPU which is not registered or deregistered but queried by DC AS.

#### 6.9.3.3	Subscription for IMS DC events supported by DCSF

Figure 6.9.3.3-1depicts a signalling flow diagram for subscriptions for IMS DC events supported by DCSF.

Figure 6.9.3.2-1: DCSF address discovery by DC AS

1.	AF subscribes with NEF using Nnef_ImsEE_Subscribe request.

2.	NEF provides Nnef_ImsEE_Subscribe Response.

3.	Based on the requested events, NEF decides that the request need to be sent to DCSF. It is assumed that NEF has discovered DCSF based on clause 6.9.2. NEF then sends the Ndcsf_ImsEE_Subscribe request with IMPU and Application ID, event ID as input.

4.	DCSF provides Ndcsf_ImsEE_Subscribe Response.

5.	DCSF detected that the application which was subscribed by NEF has been downloaded by the UE.

4.	DCSF sends Ndcsf_ImsEE_Notify to NEF.

7.	NEF sends Nnef_ImsEE_Notify to AF.

### 6.9.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

AF/NEF:

-	subscribing with DCSF for specific application download by UE event and get notified when the event is detected by DCSF.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for specific successful application download event from NEF or AF and notifying when UE successfully downloads the application.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

*** Third Change ***

## 6.10	Solution #10: DC3/DC4 Enhancement to Support Subscription, Notification and Instruction for P2A and P2A2P DC establishment.

### 6.10.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints. This solution enhances the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.10.1	Description

This solution requires the DC AS to subscribe to the call events from DCSF so that DC AS can give media instruction after receiving the notification from DCSF. This solution defines two services to enable DC AS to subscribe to the DCSF and to instruct DCSF.

This solution also enhances and unifies the timing of notifying DC AS in the current P2A and P2A2P ADC establishment procedure defined in clauses AC.7.2.2 and AC.7.2.3 of TS 23.228 [2].

### 6.10.2	Service definition

#### 6.10.2.1	General

The following table shows the DCSF Services defined.

Table 6.10.2-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
|  | Subscribe | Subscribe/Notify | NEF, DC AS |
| Ndcsf_EventExposure | Unsubscribe | Subscribe/Notify | NEF, DC AS |
|  | Notify | Subscribe/Notify | NEF, DC AS |
| Ndcsf_MediaControl | MediaInstuction | Request/Response | NEF, DC AS |

#### 6.10.2.2	Ndcsf_EventExposure

##### 6.10.2.2.1	General

Service description: This service enables the consumer to subscribe and to be notified when the given event is detected.

##### 6.10.2.2.2	Ndcsf_EventExposure_Subscribe

Service operation name: Ndcsf_EventExposure_Subscribe

Description: This service operation enables consumers to subscribe to specific events.

Inputs, Required: Notification Target address, Event ID

Three Event ID is currently defined: application data channel establishment request, application data channel establishment success, application data channel establishment failure.

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the notification will be sent to the consumer for all users.

When no application binding information is given, the notification will be sent to the consumer for all IMS DC applications.

Outputs, Required: Result indication, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

Editor's note:	Whether the explicit subscription is needed is FFS.

Editor's note:	Whether the service operation type is Subscribe/Notify (and provided by DCSF) or Request/Response (and provided by DC AS) is FFS.

##### 6.10.2.2.3	Ndcsf_EventExposure_Unsubscribe

Service operation name: Ndcsf_EventExposure_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the subscription.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

##### 6.10.2.2.4	Ndcsf_EventExposure_Notify

Service operation name: Ndcsf_EventExposure_Notify

Description: This service operation enables the DCSF to notify the consumer about the subscribed event.

Inputs, Required: Session ID, Event ID

Inputs, Optional: User IDs of calling party and called party, application binding information.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.10.2.3	Ndcsf_MediaControl

##### 6.10.2.3.1	General

Service description: This service enables the consumer to give media instruction to DCSF. In this release, the instruction is only used to establish P2A and P2A2P data channel.

##### 6.10.2.3.2	Ndcsf_MediaControl_MediaInstruction

Service operation name: Ndcsf_MediaControl_MediaInstruction

Description: This service operation enables the consumer to provide instruction to the DCSF.

Inputs, Required: Session ID, Media information of MDC2 interface.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

### 6.10.3	Procedure

#### 6.10.3.1	Service Subscription Procedure

Figure 6.10.3.1-1: service subscription procedure

1.	DC AS sends Ndcsf_EventExposure_Subscribe to DCSF to subscribe to specific ADC establishment event.

2.	DCSF sends 200 OK.

Editor's note:	How the DC AS find the right DCSF is FFS.

#### 6.10.3.2	P2A Application Data Channel Establishment Procedure

Figure 6.10.3.2-1 depicts the P2A application data channel establishment procedure.

Figure 6.10.3.2-1: P2A ADC establishment procedure

1.	Steps 0-3 of clause AC.7.2.2 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator's configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 11-20 of clause AC.7.2.2 of TS 23.228 [2] applies.

8.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS.

9.	The DC AS sends 200 OK indicating it receives the notification.

10.	Steps 21-27 of clause AC.7.2.2 of TS 23.228 [2] applies.

#### 6.10.3.3	P2A2P Application Data Channel Establishment Procedure

Figure 6.10.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.10.3.3-1: P2A2P Application Data Channel Establishment Procedure.

1.	Steps 0-3 of clause AC.7.2.3 of TS 23.228 [2] applies.

2.	The DCSF receives notification from IMS AS and detects this is an application data channel establishment request. The DCSF decides to notify DC AS based on the subscription from DC AS.

3.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that an application data channel establishment request is received.

4.	The DC AS sends 200 OK indicating it receives the notification.

5.	The DC AS detects that this is a P2A2P ADC so it allocates two sets of MDC2 endpoints. The DC AS sends Ndcsf_MediaControl_MediaInstruction to instruct the DCSF about the MDC2 related media information.

6.	The DCSF sends 200 OK indicating it receives the instruction.

NOTE:	It is up to operator's configuration and implementation that how the DCSF knows that the DC AS will send instruction to the DCSF. A possible implementation is that the DCSF will continue the following steps after a fix timer is expired.

7.	Steps 6-8 of clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF shall send all the MDC2 media related information received from the DC AS to the MF. In this case, the MF receives two sets of MDC2 endpoints.

8.	Steps 10-18 of clause AC.7.2.3 of TS 23.228 [2] applies. The DCSF will instruct the MF via IMS AS to update the media related information after receiving SDP answer from the terminating side.

9.	The DCSF sends Ndcsf_EventExposure_Notify to the DC AS. This step is to notify the DC AS that the application data channel establishment success event. The DCSF will also forward MDC2 related information from the MF to the DC AS. In the P2A2P scenario, two sets of endpoint information shall be sent to the DC AS.

10.	The DC AS sends 200 OK indicating it receives the notification.

11.	Steps 19-26 of clause AC.7.2.3 of TS 23.228 [2] applies.

### 6.10.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	sending subscription request to the DCSF.

-	receiving notification from the DCSF which may include MDC2 related media information.

-	sending DC AS side MDC2 related media information to DCSF.

DCSF:

-	receiving subscription from DC AS.

-	notifying the session events to the DC AS.

-	sending MF side MDC2 related media information to DC AS.

-	receiving DC AS side MDC2 related media information from DC AS.

*** Forth Change ***

## 6.13	Solution #13: DC3/DC4 enhancements with new media management

### 6.13.1	Description

This solution addresses KI#3.

This solution specifies the DCAS services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following new service function is defined: Ndcas_Media_Management. the associated service parameters and corresponding procedures are also defined.

The DC AS provisions the service triggering (triggering information) criteria for establishing P2A/P2A2P ADC at the DCSF using the Nimsas_ImsParameterProvision_Create service operation (see clause AA2.4.6 of TS 23.228 [2], v19.0.0). Such provisioning can be explicitly initiated by the DC AS or implicitly configured per DC AS in the IMS AS and/or in the service document in HSS (Repository data). Triggering can be user specific or user unrelated (i.e. PSI) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications. Upon receipt of a P2A or P2A2P ADC establishment request meeting the triggering filter conditions, the IMS AS invokes Nimsas_SessionEventControl_Notify to notify the DCSF of a media change request event and the availability of an P2A/P2A2P ADC. This notification includes the SDP information for the P2A/P2A2P ADC representation on the Mb interface. Based on the notification received, the DCSF will request anchoring of media in the MF for the originating MF-side of the MDC2 interface. The DCSF will then provide the originating MF-side of the MDC2 to the DCAS and request the DCAS-side for the MDC2 via the Ndcas_Media_Management_Create service operation. The DCSF will provide the received media information from the DCAS to IMS AS over the Nimsas_mediaControl service with the requested MDC2 configuration based on the information received from the DC AS.

Note that the proposal utilizes the principle that configuration/trigger can be sent to any (not all) IMS AS, according to operator choice and the data will be then stored in HSS over N71 as repository data (using the same mechanisms as defined in clause AF.4 of TS 23.228 [2]) and can subsequently be used by any IMS AS serving specific subscriber or non-subscriber specific triggers.

The use of IMS Nimsas_SessionEventControl_Notify was purposefully done in order to reuse (thus no need for new interfaces in DCSF, etc.). The provisioning paradigm used is a request-resp interface and it is not connected to IMS events (as covered in KI#2) ensuring that all IMS ASs can avail of the data even if not always applicable only to a specific subscriber. These triggers are static in nature and are similar to iFC triggers.

This enhancement to the DC3/DC4 interface is crucial for ensuring a time sensitive and 3GPP SBA principle compliant consistent handling of MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signalling using the Dcas_Media_Management_Update service operation.

The current P2A/P2A2P data channel procedures lack a 3GPP SBA service self-containment complaint mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information. The DCSF availing of the DC AS offered / provided service, Dcas_Media_Management ensures that it is the resource control entity when interworking with DC AS.

Furthermore, this solution ensures that DC AS can be selected by DCSF (for establishing P2A/P2A2P ADC) on a per session basis without creating long lived dependencies between the entities.

NOTE:	This solution identifies changes to existing specifications, these changes are shown using RED text.

### 6.13.2	Service definition

#### 6.13.2.1	General

The following table shows the DC AS Services and DC AS Service Operations.

Table 6.13.2-1: NF services provided by the DC AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcas_Media_Management | Create | Request/Response | DCSF |
|  | Update | Request/Response | DCSF |
|  | Delete | Request/Response | DCSF |

Editor's note:	Whether the service operation type is Subscribe/Notify (and provided by DCSF) or Request/Response (and provided by DC AS) is FFS.

#### 6.13.2.2	Ndcas_Media_Management

##### 6.13.2.2.1	General

Service description: This service enables the consumer (DCSF) to create, update or terminate a media resource controlled by the producer (DCAS).

##### 6.13.2.2.2	Ndcas_Media_Management_Create

Service operation name: Ndcas_Media_Management_Create

Description: This service operation enables the consumer to create a media resource (e.g. ADC (P2A, P2A2P) or other media resources like audio/Video. The information is used by the DCAS to create a resource and establish a media resource in an IMS session.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode, trigger Information.

Inputs, Optional:

Outputs, Required: Result. The result contains the following data elements: session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent operations.

Outputs, Optional: None.

##### 6.13.2.2.3	Ndcas_Media_Management_Update

Service operation name: Ndcas_Media_Management_Update

Description: This service operation enables the consumer to update a media resource information used by the DCAS.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), destination URL

Inputs, Optional: certificate, fingerprint of the UE, trigger Information

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.13.2.2.4	Ndcas_Media_Management_Delete

Service operation name: Ndcas_Media_Management_Delete

Description: This service operation enables the consumer to provide information used by the DCAS for media termination in an IMS session

Inputs, Required: Session ID, mediaInformation, destination URL

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.13.2.3	Nimsas_ImsParameterProvision Service API

##### 6.13.2.3.1 	Type: ImsPpDataEntry

Table 6.13.2.3.1-1 New Parameters for: Table 6.5.6.2.2-1: Definition of type ImsPpDataEntry, TS 29.175 [9]

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rcdServerInformation | RcdServerInfo | O | 0..1 | The RCD server information provisioned to the IMS AS. |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke for example dcas media management |

Editor’s note:	The TriggerInfo data type needs to be defined as common data and is for FFS.

NOTE:	An additional data type is required (existing defined operation only contains RCD related info) to trigger the service to invoke DCSF (and locate the correct DCAS) for this session.

#### 6.13.2.4	Nimsas_SessionEventControl Service API

##### 6.13.2.4.1	Type: SessionEventNotification

The parameters that are new are: afInstance, triggerinformation

Table 6.13.2.4.1-1 New Parameters for: Table 6.1.6.2.2-1: Definition of type SessionEventNotification TS 29.175 [9]

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationEvent | NotificationEvent | M | 1 | The NotificationEvent corresponds to a report on one subscribed event. |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the event relates to. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| sessionInfo | SessionInfo | C | 0..1 | The sessionInfo is the IMS session related information, e.g. the calling and called identity. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "MEDIA_CHANGE_REQUEST", "EXTERNAL_SESSION_CREATE", "SESSION_ESTABLISHMENT_SUCCESS" (NOTE1). |
| mediaInfoList | map(MediaInfo) | C | 1..N | Media info list includes a list of media related information, e.g. media identity, media type and media specifications. The mediaId attribute within the MediaInfo data type is the key of the map. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "SESSION_ESTABLISHMENT_PROGRESS", "SESSION_ESTABLISHMENT_ALERTING", "SESSION_ESTABLISHMENT_SUCCESS", "MEDIA_CHANGE_REQUEST", "MEDIA_CHANGE_SUCCESS", "MEDIA_CHANGE_FAILURE", or "MEDIA_CHANGE_CANCEL". |
| psDataOff | PsDataOffStatus | C | 0..1 | It indicates the active/inactive status of PS Data Off for the served UE. It shall be present if the attribute eventType of the notificationEvent is set to one of the following: - "SESSION_ESTABLISHMENT_REQUEST", when IMS session establishment is invoked; - "PS_DATA_OFF_STATUS_CHANGE", when the status change of PS Data Off happens without any other changes to the IMS session; It may be present in other cases if the change of PS Data Off status happens together with other events (e.g. media changes). |
| mediaInfoSet | array(MediaInfoExternal) | C | 0..1 | Media information set includes a set of media information. It shall be contained when the attribute eventType of the notificationEvent is one of the "EXTERNAL_SESSION_UPDATE" "EXTERNAL_SESSION_CREATE" |
| afInstance | string | O | 0..1 | The string identifying the served DCAS service instance |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke dcas media management |
| NOTE1: The sessionInfo shall be present when the attribute eventType of the notificationEvent is set to "SESSION_ESTABLISHMENT_SUCCESS" and the originating IMS AS received the redirected-to party identity via the History-Info header field in the 200 OK responses to the initial INVITE request. |  |  |  |  |

### 6.13.3	Procedure

#### 6.13.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.13.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.13.3.1-1: P2A ADC establishment with a trusted DC AS

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

NOTE:	The IMS Parameter Provisioning flow is simplified for illustration purposes. The IMS Parameter Provisioning flow is defined in clause AF.4 of TS 23.228 [2] and uses an IMS AS instance that does not need to be the one assigned to the specific IMS user. Since the provisioned parameters are stored as repository data in HSS, the IMS AS assigned to the IMS user will retrieve the DCAS provided service triggers from HSS.

Editor's note:	How the IMS Parameter Provisioning procedure can support provisioning of service triggers for any UE is FFS.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11.	UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in step 1. IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.		DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id/DCAS instance, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE.

13.		DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-29.	IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel established.

#### 6.13.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure

Figure 6.13.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.13.3.2-1: P2A2P ADC establishment with a trusted DC AS.

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

NOTE:	The IMS Parameter Provisioning flow is simplified for illustration purposes. The IMS Parameter Provisioning flow is defined in TS 23.228 [2] Annex AF.4 and uses an IMS AS instance that does not need to be the one assigned to the specific IMS user. Since the provisioned parameters are stored as repository data in HSS, the IMS AS assigned to the IMS user will retrieve the DCAS provided service triggers from HSS.

Editor's note:	How the IMS Parameter Provisioning procedure can support provisioning of service triggers for anyUE is FFS.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11.	UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in step 1. IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.	DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id / DC AS ID, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message and the certificate fingerprint of UE.

13.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-24.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation.

25.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF.

26-33.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel established.

### 6.13.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	provisioning service triggers for the UE-initiated P2A/P2A2P ADC establishment;

-	receiving MF-side information for MDC2 configuration;

-	receiving the certificate fingerprint of UE for end-to-end security between UE and DC AS;

-	sending DC AS-side information for MDC2 configuration;

-	sending the certificate fingerprint of DC AS for end-to-end security between UE and DC AS.

DCSF:

-	receiving subscription of the event UE-initiated P2A/P2A2P ADC establishment;

-	notifying about the event of UE-initiated P2A/P2A2P ADC establishment;

-	sending MF-side information for MDC2 configuration;

-	sending the certificate fingerprint of UE for end-to-end security between UE and DC AS;

-	sending request to create DC AS-side for the MDC2;

-	receiving DC AS-side information for MDC2 configuration;

-	receiving the certificate fingerprint of DC AS for end-to-end security between UE and DC AS;

-	requesting IMS AS for MDC2 configuration.

IMS AS:

-	receiving provisioning of trigger information from the DCAS;

-	receiving MDC2 configuration request from DCSF;

-	instructing MF for MDC2 configuration.

MF:

-	receiving MDC2 configuration instructions from IMS AS;

-	performing MDC2 configuration.

*** Fifth Change ***

## 6.14	Solution #14: Dynamic MDC2 endpoint configuration IMS Data Channel

### 6.14.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The main principle of this solution for support of dynamic MDC2 endpoint configuration for IMS Data Channel during UE initiated P2A and P2A2P session as follows

-	The DC AS shall discover and select the DCSF handling the UE for P2A or P2A2P session

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of P2A or P2A2P events with DCSF and get notified when DCSF detects the events.

-	After receiving notification, If DC AS accepts P2A application data channel establishment request, DC AS sends an MDC2 reserved media resource in the newly proposed Ndcsf_MediaControl service operation message.

-	DCSF after receiving MDC2 endpoint information from DC AS, invoke IMS AS to provide the MDC2 endpoint address of DC AS and request for MDC2 endpoint from MF side.

-	DCSF provides the MF side MDC2 endpoint address to DC AS in response to Ndcsf_MediaControl service operation.

Editor's note:	This solution involves DCSF during registration of IMPU while it was invoked during session earlier. The implications of this approach is FFS.

### 6.14.1	Description

This solution proposes how the DC AS discovers the DCSF and subscribes to get notification for the UE initiated P2A and P2A2P session and then shares the MDC2 endpoint addresses. This will overcome the existing gaps of the P2A and P2A2P session which does not provide any flexibility in dynamically exchanging MDC2 endpoint addresses between DC AS and MF.

NOTE:	This solution identifies changes to existing specifications, these changes are shown using RED text.

Discovery of DCSF by DC AS:

The trusted DC AS can directly discover the DCSF with the help of HSS and in case untrusted DC AS, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. Once HSS comes to know the DCSF address, it notifies the NEF/ DC AS.

Subscription with DCSF by DC AS:

Upon successful discovery, DC AS shall subscribe with DCSF for the UE initiated P2A or P2A2P session establishment events.

Providing MDC2 endpoint address by DC AS:

Upon notified by DCSF about establishment of P2A or P2A2P call, DC AS provides its MDC2 endpoint address with DCSF.

Table 6.14.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| P2A pr P2A2P Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel offer takes DC Application Server as target endpoint and requires to anchor in the MF | DCSF |

### 6.14.2	Service definition

#### 6.14.2.1	General

The following tables shows the required service updates in TS 23.502 [8] and 23.228 [2].

##### 6.14.2.2	NEF Service

The following shows the required NEF service updates in TS 23.502 [8].

##### 6.14.2.2.1	Service Summary table Update

The NEF services for IMS Event exposure is already defined in table 5.2.6.1-1 of TS 23.502 [8].

Table 6.14.2.2.1-1 New Service for: Table 5.2.6.1-1: NF Services provided by the NEF (TS 23.502[8])

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_ImsEventExposure | Subscribe | Subscribe/Notify | AF, DC AS |
|  | Unsubscribe | Subscribe/Notify | AF, DC AS |
|  | Notify | Subscribe/Notify | AF, DC AS |

##### 6.14.2.2.2	Update to Nnef_ImsEventExposure (ImsEE) service (5.2.6.39.2 TS 23.502[8])

The following describes the updated service operations for the IMS Event Exposure service:

6.14.2.2.2.1	Update to Nnef_ImsEE_Subscribe service operation

Service operation name: Nnef_ImsEE_Subscribe

Description: The NF consumer (for e.g. AF or DC AS) subscribes for a specific IMS event.

Inputs, Required: Event-ID(s) representing the event of interest, Event Reporting Information as defined in Table 4.15.1-1.

Inputs, Optional: List one or more IMS Public Subscriber identity, Event Filters, Notification Target address, Notification Correlation Information, Expiry time, Subscription Correlation ID (in the case of modification of the event subscription), AF Identifier.

IMS Public subscriber identity entry is only present for subscriber specific IMS events.

NOTE 1:	The semantics of the Event ID and Event Filters are transparent to the NEF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: If applicable to the event type, the current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1).

NOTE:	If the current status related to the subscribed event type is available and included in the response, it is not considered as an event report (i.e. it is not considered a one-time notification on event detection). That is, it is not deducted from the maximum number of reports if included in the request within the Event Reporting Information.

##### 6.14.2.3	HSS Service

The following shows the required HSS service updates in TS 23.228 [2].

##### 6.14.2.3.1	Service Summary table Update

The HSS services for IMS Services provided by an SBI capable HSS are already defined in table AA.2.1.1-1 of TS 23.228 [2].

Table 6.14.2.3.1-1 Update to Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS (from TS 23.228[2])

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
| Management (_ImsUECM) | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfoGet | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, DC AS |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

##### 6.14.2.3.2	Update to Nhss_ImsUECM_Registration service operation

The following describes the Nhss_ImsUECM_Registration service operation from clause AA.2.1.2.1 of TS 23.228 [2], and the updates for this solution.

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id if the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

6.14.2.3.3	Nhss_ImsUECM_DcsfInfoGet service operation

The following describes a new service for IMS UE Context Management used for this solution:

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF.

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.3.4	Nhss_ImsUECM_DcsfUnsubscribe

The following describes a new service for IMS UE Context Management used for this solution:

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.3.5	Nhss_ImsUECM_DcsfNotify

The following describes a new service for IMS UE Context Management used for this solution:

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.4	DCSF Service

6.14.2.4.0	General

The following table shows the new DCSF Services and new DCSF Service Operations added to table AA.2.4.1-1 of TS 23.228 [2].

Table 6.14.2.4.0-1: Updated Table AA.2.4.1-1: NF services provided by the DCSF from TS 23.228[2]

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_ADCControl_MDC2 | Create | Request/Response | NEF, DC AS |
|  | Update | Request/Response | NEF, DC AS |
|  | Delete | Request/Response | NEF, DC AS |

6.14.2.4.1	Ndcsf_ADCControl_MDC2 service operation

Service operation name: Ndcsf_ADCControl_MDC2

6.14.2.4.2	Service Description

This service enables the consumer to create, update or terminate an ADC.

6.14.2.4.3	Ndcsf_ADCControl_MDC2Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.4.4	Ndcsf_ADCControl_MDC2Update

Service operation name: Ndcsf_ADCControl_Update

Description: This service operation enables the consumer to update information used by the DCSF for ADC updating in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

6.14.2.4.5	Ndcsf_ADCControl_MDC2Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.14.2.5	Ndcsf_SessionEvent

##### 6.14.2.5.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

##### 6.14.2.5.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event Id is currently defined: establishment of a MDC2 channel is needed

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.14.2.5.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.14.2.5.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the Notification), DC application ID, certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: Calling party and called party IDs, application ID, securitySetup if UE initiates the DTLS negotiation.

Outputs, Required: Result.

### 6.14.3	Procedure

#### 6.14.3.1	DCSF instance registration in HSS

Figure 6.14.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.14.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

Editor's note:	This reference is unclear

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS AS performs third party Registration with the DCSF instance assigned to the registering UE. This is over http and not SIP.

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE:	Figure 6.14.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.14.3.2	DCSF address discovery by DC AS

Figure 6.14.3.2-1depicts a signalling flow diagram for discovering DCSF address by DC AS.

.

Figure 6.14.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as for the Figure 6.14.3.1-1 solution.

Editor's note:	Add correct clause reference

2.	DCAS discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS.

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5.	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to DC AS.

Editor's note:		It is FFS how to address the scenario where the IMPU which is not registered or deregistered but queried by DC AS.

The P2A and P2A2P call flow is to point to "solution x" from S2-2509275 during pCR implementation.

Editor's note:	Update to add reference text to solution 11.

### 6.14.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS/NEF:

-	subscribing the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving notification about the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving MF-side MDC2 endpoint information in the notify message.

-	sending DC AS-side MDC2 endpoint information in a new service operation message.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for UE-initiated P2A/P2A2P ADC establishment event from NEF or DC AS and sending MF-side MDC2 endpoint information in notification message.

-	receiving DC AS-side MDC2 endpoint information in a new service operation message.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

*** End of Changes ***