SA WG2 Meeting #172	S2-2510822

17 – 21 Nov 2025, Dallas, USA	(was 0062)

Source:	Nokia

Title:	KI#2: Update solution #8 for DC application downloading events

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution provides an update of Sol#8 in TR 23.700-56.

# 1. Discussion

This pCR proposes a solution #8 update to the KI#2 in Next Generation Real Time Communication Phase 3.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

## 6.8	Solution #8: Subscription and notification of DC application downloading events

### 6.8.0	High-level solution principles

This solution targets KI2.

This solution enables an AF (a DC AS) to subscribe to and being notified about events related to the downloading of DC applications. Four types of events are supported: these events can be related to specific or any UE(s), as well as independent or dependent on application(s) (see clause 6.8.1). Subscription requests  optionally include filter information to control the granularity and frequency of notifications.  inducing usage of NWDAF to enforce statistics or predictions on the notifications related with application download The AF (DC AS) can use the corresponding notifications to assess the popularity of applications (especially P2P applications).

Once the IMS AS has received a subscription request from the AF as defined in Annex AD.3 of TS 23.228 [2] and when a BDC is established, the IMS AS, based on the type of the DC application downloading events (see clause 6.8.1), provides the subscription to the selected DCSF handling the BDC (this assumes the definition of an IMS AS event requiring the IMS AS is to forward to the selected DCSF). The solution does not assume a 1-1 mapping between IMS AS and DCSF. The selected DCSF is responsible for detecting these events and reports the relevant information directly to the AF/DC AS (or to the NEF).

### 6.8.1	Types of DC application downloading events

To support enhanced exposure capabilities, the IMS DC exposure framework provides mechanism for a DC AS to subscribe to events related to the downloading of DC applications. Four types of DC application downloading events are supported:

-	Type 1: Subscriber-specific and DC application-specific event: The subscription request relates to one or more UEs (identified by their IMPUs) and one or more DC application identifiers. A notification shall be triggered only when one of the UE(s) downloads one of the DC applications.

-	Type 2: Subscriber-specific and DC application-agnostic event: The subscription request relates to one or more UEs, without indicating any specific DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when one of the specified UE(s) download any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

-	Type 3: Subscriber-agnostic and DC application-specific event: The subscription request relates to one or more DC application identifiers, without specifying any UE. A notification shall be triggered when a UE downloads the specified DC application(s).

-	Type 4: Subscriber-agnostic and DC application-agnostic event: The subscription request shall not specify any UE or DC application identifier. The DC AS identifier shall be included in the subscription request. A notification shall be triggered when a UE downloads any DC application associated with the subscribing DC AS. Notifications for DC applications associated with other DC ASs shall not be provided.

NOTE 1:	It is assumed that the DCSF can determine that a DC application is associated with a DC AS either via configuration or via proper application naming convention.

Subscription requests optionally include filter information to enable the DC AS to specify criteria for event reporting, such as periodic notifications or reporting of event statics to prevent excessive signalling for individual events. The network may negotiate the event filter parameters with the DC AS to ensure alignment with network capabilities, resource availability and operational policies.

The event subscription provided to the DCSF contains the AF ID of the DC AS allowing the DSCF to control whether to send a notification related to a given application to the DC AS.

NOTE 2:	This is to ensure that a DC AS can receive notifications only about "its own applications".

### 6.8.2	Procedure of DC application downloading event subscription and notification

Figure 6.8.2-1 depicts a signalling flow diagram for DC application downloading event subscription and notification.

Figure 6.8.2-1: Procedure of DC application downloading event subscription and notification

1.	The AF (DC AS) sends a subscription request (see clause 6.8.1) to the DC application download event(s) to IMS AS via the NEF. For Type 1 or Type 2 event targeting specific UE(s), clause AD.3.2 of TS 23.228 [2] is followed. For Type 3 or Type 4 event targeting any UE, clause AD.3.3 of TS 23.228 [2] is followed.

The IMS AS instance(s) locally store the subscription request (either globally for subscription targeting any UE or in association with the IMS registration for subscription targeting an individual UE). The IMS AS needs to understand the event type and the targeted UE(s) but does not need to understand other information about this event in the subscription request.

NOTE 1:	This corresponds to a new IMS AS event requiring the IMS AS to forward the corresponding event subscription to the selected DCSF.

2.	The UE requests BDC establishment as defined in clause AC.7.1 of TS 23.228 [2]. During this process, the IMS AS selects a DCSF.

3.	The IMS AS on behalf of the NEF/DC AS forwards the event subscription request received in step 1 to the selected DCSF for: when the IMS AS notifies the DCSF of the session establishment event by sending Nimsas_SessionEventControl_Notify (SessionEstablishmentRequestEvent, Session ID, Calling ID, Called ID, etc.) it provides the DC AS / NEF subscription request to the DCSF.

4.	The DCSF responds to the IMS AS, indicating successful subscription.

5.	The UE downloads a DC application via the established BDC.

6.	The DCSF checks whether the downloaded DC application is associated with the DC AS having issued the subscription request and whether the Event is authorized for this UE. If it does not, the event shall not be notified.

NOTE 2:	This is to ensure that a DC AS can receive notifications only about "its own applications". To be able to detect whether the downloaded DC application is associated with the DC AS having issued the subscription request either the DSCF can be configured with the mapping between DC applications and the AF ID associated with the DC AS or the application Id (negotiated with the DC AS) contains a reference to this AF ID,

7-8.	The DCSF notifies the NEF of the DC application downloading event. The NEF responds to the notification.

9-10.	The NEF notifies the DC AS of the DC application downloading event. The DC AS responds to the notification.

### 6.8.3	Procedure for reporting DC application usage event analytics

Figure 6.8.3-1 depicts a signalling flow diagram for reporting of DC application usage event analytics.

The AF / DC AS requests for analytics of DC application usage (e.g. download) events providing criteria for analytics reporting, e.g., reporting the number of DC app A downloading within the next 10 days or per period of 10 .

If no analytics filter is provided, the procedure in clause 6.8.2 applies

NOTE 1: 	Analytics of DC application downloading events allows providing AF / DC AS with information about DC application usage without burdening AF / DC AS each time there is an application download.

Figure 6.8.3-1: Procedure for analytics related with DC application usage event subscription

1.	The AF (DC AS) sends a subscription request (Nnef_AnalyticsExposure_Subscribe) for the DC application download event(s) (see clause 6.8.1) to the NEF, including a Notification Target Address (+ Notification Correlation ID) for this subscription request. The difference with similar step in clause 6.8.2 is that filter information specifying criteria for analytics event reporting, is provided as part of the subscription request. Target of Analytic Reporting may indicate an individual UE or any UE.

2.	NEF creates a Notification Target Address (+ Notification Correlation ID) for the received subscription request. The NEF selects a NWDAF (that supports the newly defined analytics related with DC application usage event subscription) and sends the subscription request to the NWDAF using Nnwdaf_AnalyticsSubscription_Subscribe including the Notification Target Address (+ Notification Correlation ID).

3.	NWDAF creates a NWDAF Notification Target Address (+ Notification Correlation ID) for receiving the DC application downloading related event report. NWDAF responds to the NEF including the NWDAF Notification Target Address (+ Notification Correlation ID).

4.	NEF subscribes to IMS network for the event described in the subscription request and includes the NWDAF Notification Target Address (+ Notification Correlation ID) received in step 3 in the subscription request. The way of subscription follows Annex AD.3.2 and AD.3.3 of TS 23.228 [2], as described in step 1 of clause 6.8.2. This subscription is on behalf of the NWDAF.

5.	Event detection by DCSF as defined step 2-6 in clause 6.8.2.

6-7.	The DCSF reports the detected event to the NWDAF using the NWDAF Notification Target Address (+ Notification Correlation ID) received in step 4. The NWDAF responds to the report.

8-9.	NWDAF aggregates the events based on the Notification Correlation ID contained in the messages from step 6 and generates the analytics result. NWDAF notifies NEF about the analytics result using Notification Target Address (+ Notification Correlation ID) received in step 2 via Nnwdaf_AnalyticsSubscription_Notify. The NEF responds to the notification.

10-11.	The NEF determines the DC AS based on the Notification Correlation ID received in step 1 and notifies the DC AS about the analytics result via Nnef_AnalyticsExposure_Notify. The DC AS responds to the notification.

As an alternative described in Figure 6.8.3-2, the NWDAF may directly subscribes to the IMS network

NOTE 2: 	Only One of alternative will be specified: either the alternative described in Figure 6.8.3-1 or the alternative described in Figure 6.8.3-2 where the NWDAF directly subscribes to the IMS network

Figure 6.8.3-2: Procedure for analytics related with DC application usage event subscription (NWDAF directly subscribes to the IMS network  )

- Steps 1 and 2 are the same as in figure 6.8.3-1

- Step 3: the NWDAF subscribes to the IMS network for the event described in the subscription request and includes the NWDAF Notification Target Address (+ Notification Correlation ID). The way of subscription follows Annex AD.3.2 and AD.3.3 of TS 23.228 [2], but where the NWDAF initiates the subscription instead of the NEF

- Step 4: the NWDAF answers to the subscription request and the answer is provided by the NEF to the DC AS

- The rest of the steps are as in figure 6.8.3-1

### 6.8.4	Impacts on services, entities and interfaces

The variant with DC application usage event analytics assumes a deployment where the IMS network can access to a NWDAF; this means the definition of a new Reference point in 23.228.

DC AS:

-	Requesting new events on DC application downloading.

-	Providing analytics filter information about the DC application downloading event for procedure of DC application downloading event analytics in clause 6.8.3.

NEF:

-	Definition of new event about DC application downloading.

IMS AS:

-	Forwarding of new event subscription request to DCSF based on event type and target UE(s).

-	Definition of an IMS AS event requiring the IMS AS to forward the corresponding subscription to the selected DCSF.

DSCF:

-	Receiving new event subscription request.

-	Checking the association between DC application and DC AS.

-	Notifying of the event to the consumer (DC AS, or to NWDAF for DC application downloading event analytics).

NOTE:	For DC application downloading event analytics, the DCSF interfaces NWDAF when sending notifications to NWDAF but based on SBA principles, the DCSF does not need to be aware which entity it is sending a notification to.

NWDAF (DC application usage event analytics):

- Generating analytics about DC application downloading events based on the filter information and the received event reports.

- (if Figure 6.8.3-2 applies) subscription following Annex AD.3.2 and AD.3.3 of TS 23.228 [2], where the NWDAF initiates the subscription instead of the NEF

# * * * * End of changes * * *