3GPP SA WG2 Meeting #172	 S2-2510879

# Dallas, USA,17-21 November 2025					             				    (was S2-2510401)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1686 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the usage scenarios of DC Application Proxy for LI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SA2 has an agreement that MF acting as "DC Application Proxy" can be used to fulfil Lawful Interception requirements for the UE initiated P2P application data channels. However, under the current specification in clause AC.6, the "DC Application Proxy" is only applicable for the network initiated P2P application data channel. It is proposed that the use of "DC Application Proxy" be considered for scenarios involving Lawful Interception requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that "DC Application Proxy" mode is used for network-initiated data channels or used according to local NW configuration. Clarify the supported proxy modes of MF in P2P application data channel establishment. Add DC Application Proxy mode in the Media proxy configuration of DC media description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | National regulations for P2P data channel is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.6, AC.7.2.1, AA.2.4.3.2, AC.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

# AC.6	Data Channel Media Setup

MF provides media anchoring when needed for the IMS Bootstrap Data Channel and the IMS Application Data Channel. To that effect, they support the following capabilities:

-	HTTP Proxy: In this configuration, MF supports terminating DTLS with HTTP traffic being transparent to MF. When acting as HTTP Proxy, MF reserves media resources on both the UE side and on the MDC1/MDC2 side allowing to send HTTP traffic to the DC Application Server. This mode is deployed both for Bootstrap Data Channel and HTTP Application Data Channels. Figure AC.6-1 illustrates the protocol stack for the HTTP proxy configuration mode for the Bootstrap Data Channel case.

When used with HTTP Application Data Channel, the downloaded Data Channel application will in this case communicate with the DC Application Server, using basic HTTP, within the same bootstrap SCTP association as in which this application was downloaded. UDP, TCP and SCTP should be supported as the transport layer protocols for MDC2.

Figure AC.6-1: MF acting as "HTTP Proxy"

-	UDP Proxy: In this configuration, MF transparently proxies HTTP traffic to its target. When acting as UDP Proxy, the media resources reserved by MF contain MF connection information (e.g. IP addresses and port numbers). This mode is deployed only for Application Data Channels. Figure AC.6-2 illustrates the protocol stack for the UDP proxy configuration mode for the Application Data Channel case, providing a Person2Application/Application2Person/Person2Person Data Channel Application.

Figure AC.6-2: MF acting as "UDP Proxy"

-	DC Application Proxy: In this configuration, MF transparently proxies DC application traffic between two DCMTSI UEs. When acting as DC Application Proxy, the MF terminates UDP/DTLS/SCTP towards both peer UEs. This mode is deployed only for P2P Application Data Channels, e.g., used for network initiated data channel, or used according to local network configuration. Figure AC.6-3 illustrates the protocol stack for the DC Application proxy configuration mode for the P2P Application Data Channel case,

Figure AC.6-3: MF acting as "DC Application Proxy"

# >>>> NEXT CHANGES<<<<

### AC.7.2.1	Person to Person (P2P) Application Data Channel Setup

Figure AC.7.2.1-1 depicts a signalling flow diagram for establishing an application data channel in a person-to-person use case. In this scenario, the MF is not used to anchor the Application data channel.

In the call flow the UEs have already established an IMS audio session and the originating UE is updating the IMS audio/video session to an IMS data channel session.

Figure AC.7.2.1-1: Person-to-Person Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.	IMS session and bootstrap data channel have been established as described in Figure AC.7.1-1. Each UE (UE#1 and UE#2) may have bootstrap data channels established with both originating MF and terminating MF. Selected data channel application has been downloaded to UE#1 and possibly UE#2.

1.	UE#1 initiates a Person-to-Person Application Data Channel for the selected application. The UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the bootstrap data channel information, as well as the requested application data channel and the associated DC application binding information, according to clause AC.5 and TS 26.114 [76].

NOTE 1:	The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	The IMS AS validates user subscription data to determine whether the media change request event should be notified to the DCSF.

3.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (MediaChangeRequest Event, Calling ID, Called ID, Session ID, Event Direction, Media Info List) of the media change request event.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

5.	The DCSF determines that the added application data channel media descriptor of the SDP offer takes UE#2 as target endpoint and does not require anchoring on the local MF.

NOTE 2:	If MF acting as as "UDP Proxy" or "DC Application Proxy" needs to anchor application data channel, DCSF would have used the Nimsas_MediaControl service operation to instruct IMS AS to allocate data channel media resources of the MF.

6.	DCSF responds to the notification received in step 3.

7-8.	IMS AS sends the re-INVITE to the originating S-CSCF and then to the terminating network side and UE#2.

9-11.	UE#2 and terminating network returns a 200 OK response with SDP answer for application data channel to originating network. UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

NOTE 3:	It is assumed that the data channel application is downloaded to the UE#2 via the established bootstrap data channel between UE#2 and the MF/DCSF when UE#1 has downloaded the application from DCSF. In a case the UE#2 does not have it available when UE#2 receives the SDP offer for the application DC, the UE#2 can attempt to download it again.

NOTE 4:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

12.	IMS AS notifies the DCSF of the successful data channel modification.

13.	DCSF responds to the notification received in step 12.

14-15.	The IMS AS sends 200 OK response to the originating S-CSCF and P-CSCF.

16.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

17.	P-CSCF returns the 200 OK response to UE#1.

18.	UE#1 sends ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established. In this example, it is not anchored in MF.

If the DCSF receives a notification for a media change request event for an application request that is initiated from the remote UE, if the local policy does not allow the remote UE to use an application, the DCSF may use the Nimsas_MediaControl_MediaInstruction service operation to instruct IMS AS to reject the offered media.

NOTE 5:	It is possible that the remote UE has cached the application during a previous IMS DC session.

# >>>> NEXT CHANGES<<<<

#### AA.2.4.3.2	Nimsas_MediaControl_MediaInstruction service operation

Service operation name: Nimsas_MediaControl_MediaInstruction

Description: This operation provides instructions to the IMS AS for all media flows a consumer wants to control based on its policies for the received IMS session event and that may require resource reservation in media resource e.g. MF.

For the case when a specific media flow needs to be terminated in MF media resource (i.e. termination of a Data Channel media descriptor offered by one of the IMS subscribers) or originated by the MF (i.e. origination of a data channel media descriptor to be offered towards one of the IMS subscribers), the consumer must provide a complete MF media specification including information required by MF to know how to terminate or originate the media flow.

Inputs, Required: Session ID, Media instruction set

Session ID specifies the IMS session for which the MediaInstruction operation applies.

Media instruction set includes a set of instructions for each media flow to control. Each instruction includes:

-	media ID: used by the producer (IMS AS) and the consumer for referencing purposes. The consumer reuses the media ID it received from the IMS AS for referencing the same media or, if this media is originally a data channel multiplexing media and is instructed to be de-multiplexed, one of the de-multiplexed media component. This field shall be null for instructions related to originating new media or for data channel media components de-multiplexed from a data channel multiplexing media which do not reuse the media ID of that data channel multiplexing media.

NOTE 1:	When a data channel multiplexing media is de-multiplexed to multiple data channel media components, which data channel media component reuses the original media ID of the data channel multiplexing media is decided by the consumer by implementation.

-	associated Media ID: This parameter only exists when a multiplexed media is de-multiplexed to multiple media and is used to associate different media streams which are de-multiplexed from a multiplexed media component. This parameter is set to the value of the media ID of the original multiplexed media.

-	Media resource capability: Identify the Media Resource capabilities the Media instruction is intended for (e.g. DC, AR).

-	Media instruction: includes instructions to the producer (IMS AS) for handling the media. The following instructions are supported:

-	"TerminateMedia": Terminate the offered media descriptor of the UE in the mediaResource, i.e. this media descriptor will not be exposed to the other UE.

-	"OriginateMedia": Originate and offer a media descriptor from the mediaResource to the UE. The media ID representing the new media flow will be provided by the IMS AS in the response.

-	"TerminateAndOriginateMedia": Terminate the offered media flow in the mediaResource from the UE and originate the same media flow from the mediaResource to the other UE, i.e. the offered media descriptor of the originating UE will be replaced by the mediaResource provided media descriptor, which is sent towards the other UE.

-	"UpdateMedia": Update a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia"

-	"DeleteMedia": Delete a media flow of the mediaResource previously allocated by the instructions "TerminateMedia", "OriginateMedia" and "TerminateAndOriginateMedia".

-	"RejectMedia": Remove an offered media flow, i.e. the offered media descriptor will be removed both from the offer sent to the remote UE and from the answer returned to the initiator of the offer.

-	DC Media Specification(s): Description of additional media specification information needed for data channel media stream from application layer. If multiple application data channels are multiplexed in the same m line in the SDP, multiple DC media specifications for all multiplexed data channels are included. Each media specification includes:

-	Media proxy configuration (HTTP, UDP or DC Application Proxy) applicable to the media flow.

-	MDC1/MDC2 media endpoint address of the application layer.

-	Replacement HTTP URL per stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface. This attribute is used only in BDC establishment.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	DC Interworking indication indicating the DC media is for DC interworking via DC AS when procedure in clause AC.7.9.3 applies.

-	Data Channel Port.

-	Data Channel SCTP Port.

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies how the media stream should be processed.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource UE ID: IMPU identifies the UE that owns the avatar representation to be retrieved by the MF.

-	Resource URL: URL allocated by DC AS for MF to retrieve the avatar representation.

-	Requester UE ID: IMPU identifies the UE that is requesting the avatar representation to be retrieved by the MF.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric with MF rendering, network centric with DC AS rendering.

-	Media Processing Specification: It specifies how the media stream associated with the Resource URL should be processed. It is only used in network centric rendering mode.

NOTE 2:	DC Media Specification, AR Media Specification and Avatar Media Specification are optional and they are used depending on the scenario.

Inputs, Conditional: None.

Inputs, Optional: None.

Outputs, Required: operation result indication.

Outputs, Optional: media resource information set:

This set includes entries corresponding to each instruction in the received media instruction set. Each entry in the set includes the media ID (same as received or new for new or de-multiplexed media) and includes the media type according to SDP. Each entry may optionally contain Avatar Media Specification, DC Media Specification and/or AR Media Specification. The DC Media Specification shall exist only with media type "DC" and contains additional media resource information, e.g. allocated media address for MDC2 interface when an offer must be provided to Data Channel Application Server. The AR Media Specification contains media processing instruction for MF on how to process the media. The Avatar Media Specification contains media processing instruction for MF on how to perform network rendering for avatar communication.

# >>>> NEXT CHANGES<<<<

### AA.2.5.2	Nmf_MediaResourceManagement (MRM) service

#### AA.2.5.2.1	General

Service description: This service enables the consumer to create, update and delete media resources related to IMS Data Channel.

The media resource represents a media context including one or multiple media terminations. A media termination includes media resources for one or multiple media streams on the Mb interface.

Each media resource context is identified by a Media Resource Context ID.

Each media termination is identified by a Termination ID.

Each media stream is identified by a Media ID and specified by a media descriptor.

NOTE:	Termination is a concept widely used in media protocols like H.248. When a media stream, e.g. application data channel, pass through the MF, there is one termination for the input stream and one termination for the output stream. In this case, the two terminations can use the same media ID.

Figure AA.2.5.2.1-1illustrates the above. The Figure shows a media Context whose ID is AX and two media terminations whose IDs are AB and XY. Media Termination AB anchors two media streams; 1 and 3. Media termination XY anchors media streams 2 and 3.

Figure AA.2.5.2.1-1: Media Context Resource Example

If the MF support data channel multiplexing as specified in clause AC.7.10, the MF correlates the multiplexed media streams between terminations as follows:

-	If the multiplexed media streams are kept unchanged between terminations in the MF, the MF correlates the media streams in different terminations associated with the same Media ID for data forwarding.

-	If the media streams are multiplexed in one termination and demultiplexed in the other termination, the MF correlates the media streams in different terminations via the same associated Media ID as specified in clause AA.2.5.2.2 for data forwarding.

#### AA.2.5.2.2	Nmf_MRM_Create service operation

Service operation name: Nmf_MRM_Create

Description: The consumer NF requests the MF to create a media context including one or multiple media terminations and reserve media resources for anchoring one or multiple media streams of Mb interface in each termination. The consumer NF may also include media application function specification information requested by the application layer, e.g. DCSF, to be applied on the media stream by the MF.

Inputs, Required: List of Media Termination Descriptors.

Each Media Termination Descriptor of the list includes:

-	Termination ID: A unique identity of the termination within the media context resource. The identity is allocated by the producer and must be set to value 'null' by the consumer when adding a termination to a context.

-	List of Media Stream Descriptors belonging to the Termination.

Each Media Stream Descriptor of the list includes:

-	Media ID: A unique identity of the media stream within the media context instance. The Media ID value is set by the consumer.

-	associated Media ID: This parameter only exists when a multiplexed media is de-multiplexed to multiple media and is used to associate different media streams which are de-multiplexed from a multiplexed media component. This parameter is set to the value of the media ID of the original multiplexed media.

NOTE 1:	Different Termination IDs associated with the same Media ID or associated media ID corresponding to the same Media ID represent the media stream flow for data forwarding.

-	Remote Mb Specification: Media Specification specifying SDP parameters of a remote media endpoint e.g. media stream IP address and ports allocated at the remote UE.

NOTE 2:	In the case of reserving termination media resource before receiving media negotiation information from the UE, Remote Mb Specification should be set to Null.

-	Media Function Specifications: Specification of the media function to be applied to the media stream as requested by the application layer, which includes:

-	Media Function Type: The value is set based on the type of the media function required, e.g. "DC media function type or AR media function type.

-	Media Function Description: Application specific media function description specifying how the media function shall handle and process the media stream. Data content depends on type of media function.

If the Media Function Type is "DC Media Function", then the Media Function Description incudes:

-	Media proxy configuration (HTTP, UDP or DC Application Proxy) applicable to the media flow.

-	Remote MDC1/MDC2 media endpoint address.

-	Replacement HTTP URL per Stream ID allocated by the application layer representing the application list (e.g. graphical user interface) provided to the IMS subscriber via the MDC1 interface. This attribute is only used during BDC.

-	Data Channel Mapping and Configuration information when originating/terminating data channel media flows on the Mb interface.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Data Channel SCTP Port: This attribute identifies the SCTP port for the Data Channel.

-	Security Setup: This attribute identifies the security setup of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity

NOTE 3:	Some of the above attributes are optional depending on the scenario and proxy configuration.

If the Media Function Type is "AR Media Function", then the Media Function Description includes:

-	AR Media Specification: Description of additional media specification information needed for AR communication services from application layer, which includes:

-	Media Processing Specification: It specifies the how the media stream should be processed.

NOTE 4:	The detail of Media Processing Specification is defined by SA4 and needs to be further aligned.

-	Avatar Media Specification: Description of additional media specification information needed for avatar communication services from application layer, which includes:

-	Resource UE ID: IMPU identifies the UE that owns the avatar representation to be retrieved by the MF.

-	Resource URL: URL allocated by DC AS for MF to retrieve the avatar representation.

-	Requester UE ID: IMPU identifies the UE that is requesting the avatar representation to be retrieved by the MF.

-	Rendering mode: identifies the mode of rendering avatar media, i.e. UE centric, network centric with MF rendering, network centric with DC AS rendering.

-	Media Processing Specification: It specifies how the media stream should be processed.

NOTE 5:	AR Media Specification and Avatar Media Specification are optional. They are used depending on the scenario.

Inputs, Optional: None.

Outputs, Required: Result indication for the entire list, Media Resource Context ID, List of allocated Media Termination Descriptor Resource information. For each Media Termination Descriptor; the Termination ID and the list of media stream descriptors corresponding to the input list of media stream descriptor:

Media Resource Context ID uniquely identifies the media resources created by the MF. This identity shall be used as reference when updating or deleting the created media resource.

List of allocated Media Termination Descriptor Resource information includes one or multiple Media Termination Descriptor Resources allocated. The list will include same number of terminations, in the same order as provided in the request. Each Media Termination Descriptor Resource includes:

-	Termination ID: Unique identity allocated by the consumer. The identity shall be used when updating/deleting the Media Termination.

-	List of Media Stream Descriptor Resource information elements allocated within the media termination. The list will include same number of media streams, in the same order as provided in the request.

Media Stream Descriptor Resource information includes:

-	Media ID: Unique identity of the media stream within the media context instance allocated by the consumer. This is identical to the Media ID used in the input.

-	associated Media ID: identical to the associated Media ID received from the consumer.

-	Local Mb Specification: Media specification specifying SDP parameters representing the media endpoint of the provider e.g. media stream IP address and port.

-	Media Application Function Output Description: Optional media application function description including information required by the application layer. The Media Application Function Output Description shall be relayed to the DCSF by the consumer NF. Data content depends on type of media function. If the Media type is "DC media function", the Media Application Function Output Description includes:

-	MDC1/MDC2 media point information (IP address/port number, etc.) which shall be relayed to the DCSF by the consumer NF.

Outputs, Optional: None.

# >>>>END OF CHANGES<<<<