3GPP TSG-WG SA2 Meeting #172	S2-2510892

# Dallas, US, 17th Nov – 21st Nov, 2025		(revision of S2-2510556)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6480 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Report serving satellite identifier to SMF during Service Request for all satellite types |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the UE-satellite-UE communication in IMS, if the terminating side UE is in CM_IDLE state, the Service Request procedure will be performed by the UE. In the Service Request procedure, the AMF should also report the identifier of the UE’s serving satellite to the SMF, so that the terminating side P-CSCF can obtain the satellite identifier from the PCF via the SMF to determinate whether to perform UE-satellite-UE communication. Currently it is not clear the service satellite identifier is provided for UE-satellite-UE communication as part of the PDU Session procedures within the Service Request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update functional description of UE-satellite-UE communication in IMS to cover AMF reporting satellite identifier via Service Request procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The reporting of the satellite identifier via the Service Request procedure is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.14.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.4.14.2	Setup of UE-Satellite-UE communication with IMS user plane components on-board satellites

The AMF reports the identifier of the satellite serving the UE to the SMF as part of the PDU Session establishment and update procedures as described in clauses 4.2.3, 4.3.2 and 4.3.3 of TS 23.502 [3].

NOTE 1:	The AMF can determine that the UE is accessing gNB onboard a LEO or MEO satellite and can also determine the satellite identifier based on local configuration, e.g. Global RAN Node IDs associated with the LEO or MEO satellite.

Upon receiving a request from the P-CSCF for accessing network information report (e.g. as a result of the P-CSCF processing an IMS call setup), the PCF subscribes to Access Network Information report from the SMF for the IMS PDU Session, as described in clause 6.1.3 of TS 23.503 [45]. The Access Network Information report contains the identifier of the satellite serving the UE as reported by the AMF.

NOTE 2:	How the serving satellite identifier is used by the IMS to determine whether the UE-Satellite-UE communication can be activated is specified in TS 23.228 [15].

.

# * * * * End of changes * * * *