3GPP TSG-SA WG2#172	S2-2510905

Dallas, USA, November 17 – November 21, 2025	(was S2-2510510)

Source:	Qualcomm Incorporated

Title:	[KI#3, New Solution] SMS to Emergency Centre over IP

Document for:	Discussion/Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC

Abstract of the contribution: the paper proposes a new solution to be documented in the TR for the support of SMS over IP.

# 1.	Discussion

## 1.1	Work Task and Key Issue

Key Issue #3 “SMS over IP Routing to Appropriate PSAP” has been agreed:

-	how and when to trigger location determination and location data sharing when sending short messages using SMS over IP for routing to Emergency Response Centre, considering both 3GPP and non-3GPP access scenarios;

-	 how to route short messages using SMS over IP to Emergency Response Centre serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location);

-	how to support PSAP callback capabilities for sending and routing regular short messages using SMS over IP.

The key issue covers SMS delivery via IP both in EPS and 5GS.

The key issue covers both roaming and non-roaming scenarios.

## 1.2	Further Considerations

### 1.2.1	“Callback” for SMS to emergency center

SMS to Emergency Service Centre is not considered an emergency service. It has been agreed that messages back to the UE are sent as regular SMS, with no specific treatment. This may enable a fast solution that caters for low volume of such messages to be sent and only in very rare situations, but realistically we need to consider the following:

-	in case of emergency, the “conversation” between the user and the PSAP operator will require multiple messages back and forth. An emergency call already requires multiple questions to be answered before any actions are taking, and we can easily expect that multiple round of messages will be required in order to determine and confirm the type of incident and the user location.

-	Public safety dispatch can be very expensive to the user and to the system, and would often need to be exactly the right type (e.g. an ambulance and not a police car or fire engine) and sent ASAP to exactly the right location. No PSAP operator would realistically instigate PS dispatch based on just ONE incoming SMS without at least first trying to contact the user and get more information. Therefore, what we propose to consider is that in case of SMS

Even if SMS to Emergency Centre may be only a “small fraction of cases”, ensuring a reliable, flexible solution that enables a full dialogue is important for emergency services even if it is only 1% or less.

### 1.2.2	Dirt and easy solution or future proof solution

The SA1 requirements in 22.101 do not prevent a good solution. For example, they do not say that a PSAP SM response must be routed as a regular SM message or that a Rel-20 solution must be based on existing Rel-19 SM solutions. Those are proposed SA2 assumptions.  Here is what SA1 says in 22.101:

-    EPS and 5GS shall route a Short Message using the emergency numbers to a local emergency response centre (i.e. PSAP), according to the local regulations.

-    The Short Message shall be routed to the emergency response centre serving the location where the subscriber is located and replies routed back to the subscriber in accordance with national regulations.

-    The 3GPP system shall support a Short Message to be routed to different emergency response centres, depending on the type of emergency, e.g., as specified in 10.1.

-    The Short Message Service to emergency response centre shall be able to use the same identification methods as for emergency calls, e.g., as specified in 10.1.1.

-    The Short Message Service to emergency response centre shall be supported also for roaming UEs, where roaming subscribers’ SMS can be routed to a local PSAP, according to the local regulations of the VPLMN.

There is a risk we are taking: if the FCC in the US or the EC in Europe mandates that SM replies must be routed within the visited country for roamers and/or must have low delay and high reliability, or if they mandate that it must be possible to route all SM messages to the same PSAP operator or to a PSAP operator to which an emergency call was previously setup, some of the assumptions we have been making so far will become non-compliant and so probably will be any solutions that are based on the assumptions.

We recommend defining assumptions that are not any narrower than SA1 requirements, and allowing for solutions that are more future proof.

Moreover, there are efforts to standardize and promote solutions (e.g. in ATIS J-STD-110) supporting IP connectivity that Message Session Relay Protocol (MSRP) for text and other media types and enabling PSAPs to receive multimedia messages (i.e. text, video, and voice calls) when the technology and time is right for each individual PSAP. Though such solutions are not yet a requirement, there is a clear interest by regulators and operators to move towards such solutions, thus enabling in Rel. 20 a solution for SMS2EC over IP that can be easily evolved to support MSRP would benefit the industry with respect to adopting a pure SMS solution.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-65. The text in the third change is all new.

# >>>>BEGINNING OF CHANGES<<<<

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |  |
| X |  |  | x |  |

# >>>>NEXT CHANGE<<<<

## 6.X	Solution #X: Support of SMS to Emergency Centre over IP

### 6.X.0	High-level solution principles

This solution applies to KI#3.

This section describes support for the SMS to Emergency Centre over IP.

SMS to Emergency Centre over IP in this context is an SMS over IMS using an emergency number as destination address. A key requirement for this service is that the SMS to Emergency Centre must be routed to a PSAP in the country where the UE initiating the SMS is located.

The following principles apply to the solution:

-	applies to UE detectable emergency numbers

-	based on the concept of establishing a session between the UE and the PSAP

-	for roaming UE the solution is based on PDU session connectivity in the VPLMN and using the VPLMN for routing to the correct PSAP. The traditional IMS PDU session routed via the HPLMN is not used

-	the UE discovers whether the serving PLMN supports SMS to Emergency Centre over IMS during the initial attach and IMS registration to the serving PLMN

-	the SMS to Emergency Centre over IMS is routed via a PDU session in local breakout and via IMS in the VPLMN

-	the E-CSCF in the VPLMN is upgraded to handle SMS to Emergency Centre as if it were and SCSF and route the SMS to Emergency Centre to an appropriate PSAP

-	the SMS to Emergency Centre is not routed via an SMS-SC, in order to enable a future proof solution that can be expanded to additional media

-	the solution has UE impacts in order to enable a complete service for SMS to Emergency Centre over IMS that allows for a “dialogue” between the UE and the PSAP. Moreover, this allow for future extensions to enable additional media.

-	The UE that supports SMS to Emergency Centre over IP detects an SMS to Emergency Centre based on used emergency numbers. Provisioning of emergency numbers and associated types as specified in TS 23.167 [z] clause 4.1 bullets 2a, and 2b) apply.

-	Serving network can support SMS to Emergency Centre over IP for own subscribers and inbound roamers.

-	PSAP are upgraded to support SIP sessions.

### 6.X.1	Description

#### 6.X.1.1	General

The proposed solution for SMS to Emergency Centre over IP enables reliability by using SIP end to end between the UE and the PSAP and enabling an actual “dialogue” between the UE and the PSAP. In case of potential subsequent voice emergency calls, the same PSAP would most probably be selected thus allowing for a better end user service. The solution provides high reliability and security thanks to the use of IMS spoofing protection.

NOTE 1:	The solution can be also be expanded to additional services to Emergency Centre (e.g. using MSRP for real-time text, voice, video).

NOTE 2:	The solution assumes that PSAP supports IMS and SMS over IP.

Most PSAPs are already migrating to SIP/IP, thus the solution can be future proof.

The solution can be defined completely within 3GPP and does not require IETF change, since use of SDP without SIP media is an existing IETF option (in RFCs 3264 and 4566).

### 6.X.2	Procedures

Figure 6.X.2.1: Registration Procedure

1.	The UE attaches or registers with the serving PLMN if subscribed for access and can be authenticated. The UE also performs IMS Registration according to existing mechanisms.

2.	The UE detects that a user has instigated a short message (SM) to an emergency number. The UE is aware whether the serving PLMN and local PSAP support “SMS to Emergency Centre over IMS” (SMS2EC over IMS) from step 1.

3.	The UE may follow one of the following three options:

a.	the UE attaches/registers with an SMS2EC-specific DNN configured via roaming agreement for the Serving PLMN that is routed in local breakout

NOTE 1:	the SM2EC-specific DNN is configured in the UE and is in the UE subscription.

b.	based on bi-lateral agreement between HPLMN and Serving PLMN, the UE uses a specific DNN/S-NSSAI pair in 5GS or APN in EPS that is routed in local breakout

NOTE 2: the SM2EC-specific DNN/S-NSSAI pair is configured in the UE and is in the UE subscription.

c.	the UE performs an emergency attach or emergency registration

4.	The UE establishes an PDN connection or PDU session as per step 3.

5.	The UE attempts to perform an IMS Emergency Registration if step 1 occurred or if authentication succeeded in step 3.

6.	The UE establishes an IMS emergency call with no SIP media to a PSAP.

6a. As part of step 6, the UE sends a SIP INVITE containing the emergency number detected at step 2 or an SOS URN corresponding to this number to a P-CSCF in the IMS which forwards the SIP INVITE to an E-CSCF. The SIP INVITE contains a Session Description Protocol (SDP) description for IETF compatibility but not necessarily SIP media (see NOTE 1) if the intention is to be used only for SMS.

NOTE 1:	The solution enables the UE to optionally include other media e.g. MSRP or voice/video in the SIP "offer".

6b. The E-CSCF routes the SIP INVITE towards a PSAP that supports SMS over IMS based on a location or current serving cell for the UE and possibly with the assistance of an LRF. As an option, the emergency SM can be included in the SIP INVITE in which case steps 7 and 8 are not performed. The SIP INVITE without SIP media (and with an emergency SM if included) can be an implicit indication that the UE is able to send and receive emergency SMS messages.

6c. The PSAP may recognize from the SIP message received at step 6b that the UE is able to send and receive emergency SMS messages. The PSAP returns a SIP 200 OK to the UE via the E-CSCF and P-CSCF that also contain an SDP description but no SIP media. The SIP 200 OK without SIP media can be an implicit indication that the PSAP is able to receive and send only emergency SMS messages.

NOTE 2:	If the UE has optionally included other media in step 6a and the PSAP supports such media, the PSAP may accept them in the SIP "answer".

7.	The UE sends the SMS message to the PSAP in a SIP MESSAGE in association with the SIP dialogue established at step 6.

8.	The PSAP returns a SIP 200 OK to acknowledge receipt of the SMS MESSAGE and in association with the SIP dialogue established at step 6.

9.	The PSAP optionally returns an SMS reply to the UE in a SIP MESSAGE in association with the SIP dialogue.

10. The UE returns a SIP 200 OK to acknowledge receipt of the SMS MESSAGE and in association with the SIP dialogue.

11. The UE and PSAP may exchange further SMS messages using SIP MESSAGEs as in steps 7 to 10.

NOTE 3:	if additional media were offered by the UE and accepted by the PSAP, the UE and the PSAP may exchange such media in addition or instead of the SMS exchange of steps 7 to 10.

12a. Following a timeout after sending or receipt of the last SMS message, the UE sends a SIP BYE to the PSAP to release the SIP dialogue established at step 6.

12b. Alternatively, the PSAP can send a SIP BYE at any time to release the SIP dialogue.

### 6.X.3	Impacts on services, entities and interfaces

UE:

-	Identify the need to send SMS to Emergency Centre over IP based on the use of emergency numbers.

-	Use the appropriate connectivity as described in step 3 of the procedure

E-CSCF

- Support handling of SMS to Emergency Centre

- Support identification of the appropriate PSAP to route the SMS to Emergency Centre via the establishment of a SIP session

PSAP

- Support of IMS and SMS over IP

# >>>>END OF CHANGES<<<<