3GPP TSG SA WG2 Meeting #172	S2-2510912

# 17-21 November, 2025, Dallas , US 			 (Revision of S2-2510355, 08871, 6998, 5207)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1651 | rev | 5 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 3GPP PS Data Off Exempted Services for DC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-07-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current specification does not cover the scenario of IMS AS removing the DC media when IMS DC interworking applies. When IMS DC is removed by the UE, or the PS data off is activated to trigger the IMS AS to remove IMS DC media, the media component associated with the IMS DC, e.g. video media component, which is established by the DC AS for IMS DC interworking as specified in AC.7.9, should also be removed, since it is not needed anymore. In this case the description in clause AC.7.6 is not correct. It is proposed to add texts to clarify the termination of IMS DC media for this scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add texts clarifying that media component associated with a DC removed due to PS data off activation shall also be removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of PS data off for IMS DC is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## AC.7.6	Termination of IMS Data Channel

When a bootstrap data channel is established successfully, the bootstrap data channel shall be terminated in one of the following cases:

-	when the IMS session is released;

-	when conditions to terminate bootstrap DC due to interaction with supplementary services as specified in TS 24.186 [105] are met.

The application data channel shall be terminated along with the release of the call or be terminated by closing this application separately. In the latter case, the UE triggers a SDP negotiation to release the application data channel.

The termination of application data channel shall not have any impact on the audio or video within the IMS sessions, unless the terminated application data channel is correlated to audio or video media due to IMS DC interworking as specified in clause AC.7.9, in which case, the correlated audio or video media are also terminated.

# * * * * End of changes * * * *