SA WG2 Meeting #S2-172	S2-2510959

17 - 21 November, 2025, Dallas, USA	(revision of S2-2510956)

Source: 	vivo, OPPO, ZTE, ETRI, LG Electronics, NTT DOCOMO

Title: 	KI#1 interim conclusion update

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes to update interim agreement for KI#1: Transfer of data over UP for UE data collection.

1	Discussion

Some interim principles are proposed for KI#1: Transfer of data over UP for UE data collection.

2. Proposal

It is proposed to add the following changes to TR 23.700-04.

Start of Change

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

1. UP path termination entity

-	An 5GC NF acts as the UP path termination entity to support the transfer of standardized data over UP for UE data. The UP path termination entity will further transfer the received data to the UMTES. (see Editor’s Note [2][15] in clause 7.2.1)

NOTE 1: The UMTES is the AF/OTT server for UE-side model training.

2. Discovery of UP path termination entity

-	(the UMTES discovers UP termination entity) The UP path termination entity may be either locally configured at the UMTES, or the UP path termination entity may be discovered by the UMTES via NRF (if UMTES is trusted) or by NEF (the UMTES is untrusted). (see Editor’s Note [3] in clause 7.2.1)

The following options allow the UE to learn the UP path termination entity, either a or b, (see Editor’s Note [5] in clause 7.2.1)

a)	the UE has the UP path termination entity address/FQDN locally configured (see Editor’s Note [6] in clause 7.2.1)

b)	the 5G core network sends UP Path termination entity address (i.e. IP address/FQDN) to the UE in a N1 container via NAS. (see Editor’s Note [4] in clause 7.2.1)

3. UP path establishment

-	The UE establishes a new PDU Session, or uses an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, by reusing existing mechanism (i.e. URSP).

4. Data transfer procedure

- 	UMTES sends the data transfer request to UP path termination entity including the information related to candidate UE(s) for data transfer (e.g. UE list, TAC(s), geographical area), and the requested data id. (see Editor’s Note [16] in clause 7.2.1)

-	A request for data transfer with requested data id is sent to the target UE(s). (see Editor’s Note [9][10][11] [18] in clause 7.2.1)

NOTE 2: The list of target UEs is determined taking into account at least the candidate UE(s) provided by the UMTES and user consent

-	The UE reports the requested data to the UP path termination entity via the established UP path.

-	The UP path termination entity can send requested data id to gNB via AMF, which may assist the gNB to decide data collection configuration to UE. (see Editor’s Note [1][14][17] in clause 7.2.1)

NOTE 3: whether and what information to assist data collection configuration will be defined by RAN.

5.	Traffic Differentiation with following two alternatives:

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of the collected standardized data from the UE regular traffic per PDU Session level.

-	A QoS Flow is used to differentiate the transfer of the collected standardized data from the UE regular traffic per QoS flow level.

6.	Controllability

-	The UP path termination entity may initiate / stop data transfer procedure (see Editor’s Note [12][19] in clause 7.2.1)

-	The UE may accept / reject data transfer from the UP path termination entity. The UE may stop data transferring based on internal determination. (see Editor’s Note [13] in clause 7.2.1)

7. Visibility:

-	The UP path termination entity verifies/matches the requested data to be transferred and the data that is being reported. (see Editor’s Note [20] in clause 7.2.1)

8. User consent:

-	User consent checking with UDM for data transfer is needed. (see Editor’s Note [7][8] in clause 7.2.1)

Next Change

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

The following ENs are needed to be resolved to make the Option-2 UP solution to work following the agreed principles:

[1]	Editor’s Note: 	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

[2] Editor’s Note: Which 5GC NF will be defined as the UP path termination entity is FFS.

[3] Editor’s Note: Whether the UP path termination entity can be discovered by other network function is FFS.

[4] Editor’s Note: it’s FFS which 5GC NF sends the address of UP path termination entity to the UE

[5] Editor’s Note: how to select which option (2a or 2b or co-exist) to use is FFS.

[6] Editor’s Note: If there are multiple UP path termination entities, whether and how to ensure that the UE and the UMTES connect to the same UP Path termination entity is FFS.

[7] Editor’s Note: Where and how to perform user consent check is FFS.

[8] Editor’s Note: Whether using the existing or new user consent purpose is FFS. And coordination with SA3 is required.

[9] Editor’s Note: whether, where and how to select the target UE by CN is FFS.

[10] Editor’s Note: Which entity sends the request to the target UE(s) is FFS.

[11] Editor’s Note: How to send the data transfer request to the UE, i.e. via NAS signalling or UP is FFS.

[12] Editor’s Note: How the UP path termination entity initiate / stop data transfer procedure, i.e. via CP or UP is FFS.

[13] Editor’s Note: Whether the UE may request data transfer is FFS.

[14] Editor’s Note: Whether the information (e.g. requested data) is sent with the data transfer request or within a separate request is FFS.

[15] Editor’s NOTE: the data transfer from UP Path termination entity to the UMTES over UP or new interface is FFS.

[16] Editor’s Note: How the mapping between the requested data id and the data to be transferred is determined is FFS.

[17] Editor’s Note: whether the request data id is sent to the gNB depends on RAN’s feedback.

[18] Editor’s Note: whether and how to select the UE by RAN is FFS.

[19] Editor’s Note: whether the UP path termination entity send to gNB via AMF an indication that data transfer is allowed for a UE is FFS.

[20] Editor’s Note: whether and how to verify/match the requested data to be transferred and the data that is being reported in standardized manner is FFS and requires RAN’s feedback on visibility requirements.

End of Changes