SA WG2 Meeting #172	S2-2510157

17 - 21 November, 2025, Dallas, USA	(revision of S2-2510157)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5612 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on EIF unsubscription from SMF Event ID Usage data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The text on the triggers for the unsubscrition from the SMF Event ID Usage data are not properly defined as they may seem to point to a unsubscription for periodi reporting which actually should not be the case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The end of reporting period is replaced by the Expiration time of the subscription and the the end of time window for reporting. Steps 8 and 9 are updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 4.29.2	Energy Consumption information collection

Figure 4.29.2-1 shows the procedure for EIF to collect the Energy consumption related information.

Figure 4.29.2-1: Energy Consumption information collection

1.	The Consumer NF (i.e. trusted AF or 5GC NF) subscribes to the energy consumption event with the EIF by using Neif_EventExposure_Subscribe service for the required granularities as described in clause 5.51.2 of TS 23.501 [2] and other necessary information as defined in clause 5.2.28.

If the Consumer is untrusted AF, the subscription is sent via NEF. If the event subscription is authorized by the NEF, then the NEF sends the event subscription to the EIF via Neif_EventExposure_Subscribe.

2.	The EIF invokes Nudm_UECM_Get service operation to retrieve the SMF ID for the required granularity.

3.	The EIF subscribes to be notified using Nudm_SDM_Subscribe regarding the establishment of the PDU Session(s) of the UE.

4.	For the SMF ID retrieved from step 2 or for the new SMF ID retrieved from step 7, the EIF invokes the Nsmf_EventExposure_Subscribe request to the SMF to subscribe to Event ID Usage data per gNB and (I-)UPF, defined in clause 5.2.8.3.1, to collect the information as defined in clause 5.51.2 of TS 23.501 [2].

If the I-SMF is involved in the PDU session, the SMF of PDU session requests to I-SMF to report the ULI and I-UPF ID.

5a.	The SMF instructs the PSA UPF to report the data volume at the required granularity by using the N4 procedures defined in TS 29.244 [69] to collect Usage Reports with periodic reporting with period defined by the SMF in the Usage Reporting Rule to be the time interval T defined in clause 5.51.2.2.1 of TS 23.501 [2]. The SMF may also request, for a configured Usage Reporting Rule, to report the current value of the measurements on demand by requesting immediate reporting when a new gNB ID and/or UPF-ID of a I-UPF is received at the SMF for the UE PDU session as defined in clause 5.51.2.2.2 of TS 23.501 [2].

5b.	The PSA UPF provides the data volume of the required granularity to the SMF as configured by step 5a.

6.	The SMF periodically provides the information collected for energy consumption calculation at the end of each time interval T (defined in clause 5.51.2 of TS 23.501 [2]) to EIF starting from the time that the EIF subscribes by invoking Nsmf_EventExposure_Notify service operation for the required granularity. Upon the PDU session release, the SMF terminates the subscriptions and notifies the EIF the information it has collected since the last reporting. If there are no UP resources established on the 3GPP access for the PDU session of the UE, the SMF behaves as described in clause 5.51.2 of TS 23.501 [2].

7.	The UDM notifies the EIF via Nudm_SDM_Notification including the PDU Session ID and SMF ID when PDU Session(s) of the UE are established at any time. Steps 4, 5 and 6 are repeated for the new PDU Session if it matches the required granularity.

8.	EIF unsubscribes from the UDM subscriptions of step 3 above upon the expiration of the EIF event exposure subscription (associated with  the Expiration Time provided as output of the Neif_EventExposure_Subscribe service operation to the EIF consumer at subscription time - see clause 5.2.28.2.2), or when the time window for reporting requested by the subscriber has elapsed and no further reporting was requested by the EIF consumer, or when the Consumer NF of the EIF Event Exposure service unsubscribes.

9.	The EIF,when the EIF Event Exposure subscription ends (as per triggers identified in step 8) also unsubscribes from SMF Event ID Usage data by sending Nsmf_EventExposure_UnSubscribe request, and the SMF notifies the EIF the information it has collected since the last reporting.

10.	If the related Node-level energy consumption and Node-level data volume information is not available in the EIF, the EIF obtains the Node-level energy consumption and Node-level data volume information from OAM as defined in clause 5.51.2.2.3 of TS 23.501 [2].

11.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2] and then reports the energy consumption information of the required granularity to the Consumer NF via Neif_EventExposure_Notify.

End of changes