3GPP TSG-SA2 Meeting #172	S2-2510973

# Dallas, TX, USA, 17th Nov 2025 - 21th Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6372 | rev | 2 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EIF discovery and selection to avoid duplicated OAM interaction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the handling of the energy information for the same UE, different EIFs can be selected, resulting in duplicatied signalling to/from OAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Consumer NF discovers/selects EIF based on SUPI as an input (or GPSI as an input for NF consumer where only GPSI is available, e.g., NEF) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicated information retrieval from OAM |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.27	EIF discovery and selection

The NF consumers shall utilise the NRF services to discover EIF instance(s) unless the EIF selection information is locally configured in NF consumers. The NF consumer selects an EIF instance based on the available EIF instances obtained from the NRF or locally configured information in the NF consumer.

The following factors may be considered by the NF consumer for EIF selection:

-	Location (see locality in clause 6.1.6.2.2 of TS 29.510 [58]).s

- 	SUPI: The NF consumer selects a EIF instance based on the results of a discovery procedure with NRF using the UE's SUPI as input.

-	GPSI: The NF consumers which manage network signalling not based on SUPI (e.g. the NEF) select a EIF instance based on the results of a discovery procedure with NRF using the UE's GPSI as input.

NOTE: When multiple EIF is deployed, an EIF instance is assigned with a range of SUPI (and a range of GPSI) and the same EIF is selected for a UE by using SUPI or GPSI of the UE to select the EIF.

# * * * * End of changes * * * *