3GPP TSG-SA2 Meeting #172	S2-25010975

# Dallas, USA, November 17 – 21, 2025		(revision of S2-2510077, S2-2508659)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0092 | rev | 2 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of Unsupported Command |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, OPPO, Lenovo, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CT1 sent LS (C1-256897) to SA2 regarding the unsupported AIoT commands. In the LS, CT1 agreed on the procedure for an AIoT device to inform the AIOTF of the receipt of an unsupported or unknown message. And CT1 believes on the network should avoid sending unsupported commands. In the agreed CR (C1-256647), the AIoT device send a STATUS message with cause code #97 if the message type is not defined or not implemented. CT1 also highlighted: NOTE: If the read command procedure or the write command procedure was aborted due to reception of the AIOT STATUS message with the cause #97 "message type non-existent or not implemented", the network can take this into account to avoid initiating further command procedures of the same command type towards the AIoT device, in order to save unnecessary signalling wasting radio resources. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce the scenario that the AIoT device sends AIOT STATUS if the command is not supported and the AIOTF can use it to avoid send unsupported command to the device. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misalignment with stage 3 - Unspecified behavior if the AIoT device replies to the AIOTF the command is not supported. - Waste of resources and energy if the AIOTF cannot utilize NAS failure message provided by the AIoT device. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2.2.1, 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * Start of Changes * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

[10]	3GPP TS 38.413: "NG Application Protocol (NGAP)".

[11]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

[12]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[X]	3GPP TS 24.369: "Ambient IoT Non-Access-Stratum (AIoT NAS) protocol for 5G System (5GS); Stage 3".

# * * * Next Change * * *

#### 5.2.2.1	Overview

Three types of Command service operations are supported: Read, Write and Disable.

An AF uses the Command service Read operation to retrieve information from AIoT Device(s), and the AIOTF uses the commands described in clause 5.2.2.2 towards the AIoT Device for the operation.

An AF uses the Command service Write operation to write information to AIoT Device(s), and the AIOTF uses the commands described in clause 5.2.2.2 towards the AIoT Device for the operation.

An AF uses the Command service Disable operation to permanently disable the capability of AIoT Device(s) to transmit RF signals, and the AIOTF uses the commands described in clause 5.2.2.3 towards the AIoT Device for the operation.

If the AIoT Device receives a NAS Command Request that the AIoT Device does not support or has not implemented, the AIoT Device shall return an AIOT STATUS message with an appropriate cause code as defined in TS 24.369 [X]. Otherwise, the AIoT Device performs the Command service operation according to the received NAS Command Request.

# * * * Next Change * * *

### 6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this AIoT service operation request, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	Step 2 of the Inventory Procedure specified in clause 6.2.2 is performed for External Target Area information processing and AIOTF selection with the following clarifications:

-	If AIOTF selection fails, the NEF rejects the Nnef_AIoT_Command request and step 6 of this procedure is performed instead.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters], [location information requested]) message to the selected AIOTF(s).

4.	The AIOTF receives the Naiotf_AIoT_Command Request and checks the parameters included in the request. The AIOTF performs Reader Selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AIoT service operation request and the Correlation ID is used for the AIOTF to correlate the service operation responses received from NG-RAN to the request. The AIOTF creates the AIoT Session for the AF service operation request, which is identified by the Correlation ID.

The AIOTF performs Reader Selection, see clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT service operation request with an appropriate cause code.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the AIoT service operation request.

The AIOTF performs AF authorization for AIoT service operation request as described in clause 5.6.

The AIOTF may perform AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

NOTE 1:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the AIoT data.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

If the NAS Command Response indicates the NAS Command is not supported by the AIoT Device, the AIOTF reports the error result in step 12 and step 13.

NOTE 2: The AIOTF can take this into account to avoid initiating further command procedures of the same command type towards the AIoT Device.

12.	The AIOTF reports the result of the Naiotf_AIoT_Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]). If multiple AIOTFs are involved in the procedure, the NEF may receive Naiotf_AIoT_Command Notify messages from multiple AIOTFs.

Based on operator policy, if the location information is requested by the AF and if the location of the reader is configured, the AIOTF uses the Reader ID reported from NG-RAN during inventory in step 7 to determine the AIoT Device Location.

When the last report is sent, the AIOTF ends the AIoT Session.

13.	The NEF informs the AF of the result of the Nnef_AIoT_Command request by sending the Nnef_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), AIoT data and optionally location of each AIoT Device), AF ID, [Last Report Indication]).

# * * * End of Changes* * *