3GPP TSG-SA WG2 Meeting #172	S2-2511014

Dallas, USA, November 17 - 21, 2025	(revision of S2-2510097, S2-25010083)

Source:	Ericsson, OPPO

Title:	Interim Agreements for KI#2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: The contribution merges the contributions of companies and proposes interim agreement principles about DO-A Data transfer aspect of KI#2.

1. Introduction

As proposed by Rapporteur, this paper is to be the baseline for DO-A data transfer aspects in KI#2 Interim Agreements. It tries to summarize the DO-A Delivery aspects from following contributions:

| S2-2510083 | 23.700-30: Interim Agreements for KI#2 | Ericsson |
| --- | --- | --- |
| S2-2510147 | 23.700-30: KI#2 Interim Agreements on Device registration and DO-A transmission | Lenovo |
| S2-2510150 | 23.700-30: [KI#2] Interim agreements | Sony |
| S2-2510244 | 23.700-30: KI2 Interim Conclusion on DO-A Data Routing to AF | ZTE |
| S2-2510319 | 23.700-30: Interim Agreements for KI#2 | CATT |
| S2-2510327 | 23.700-30: Draft interim Agreements for KI#2 | Tejas Network Limited |
| S2-2510408 | 23.700-30: KI#2 Interim agreement | LG Electronics |
| S2-2510468 | 23.700-30: KI#2 Interim Agreements | Samsung |
| S2-2510652 | 23.700-30: Interim Agreement for KI#2 | OPPO |
| S2-2510667 | 23.700-30: Key Issue 2 Interim Conclusion Proposals | Huawei, HiSilicon |
| S2-2510712 | 23.700-30: Interim Agreement for KI#2 | Vivo |

The following aspects are considered as common aspects, which are to be covered by the baseline paper for Registration aspect in KI#2 Interim Agreements:

- Temporary ID and AIOTF ID/AIOTF information, which are used for message routing purpose form NG-RAN to the AIOTF

- AIOTF relocation when the NAS message is sent to a new AIOTF

Trigger of the DO-A delivery

- S2-2510667 and S2-2510147 mention that DO-A data transfer can be triggered by local configuration or device implementation. Regarding local configuration directly. S2-2510667 also supports the local configuration can be pre-condifured or configuration by the AF using AIoT command procedure, with an EN about the potential enhancement of Rel-19 command procedure enhancement.

- S2-25010150 mentions that the configuration of application-level DO-A transmission triggers/policies to the AIoT Device is not in scope of 3GPP.

- S2-25010327 also mentions periodic/ event triggered data collection from AIoT devices can be supported via device pre-configuration or AF configuration.

- S2-25010652 indicates that how the AIoT device triggers the DOA Data Delivery is implementation issue and not standardized.

- S2-2510319 indicates that DO-A capable AIoT Devices send DO-A data to the AIOTF autonomously or by responding to a request from AIOTF.

[Proposal-1]: The DO-A capable Devices initiates DO-A data transfer, triggered by the conditions which can be stored in local configuration or device implementation. When the condition is met, the AIoT Device sends out DO-A data. The local configuration can be configured by the AF using the AIoT command procedure. An EN can be left for further enhancement of AIoT command procedure, and a NOTE can be left about the the triggers/policies in the local configuration is out the scope of 3GPP.

Regarding the DO-A data delivery by responding to a request from AIOTF,  it can be seen as a reuse of Rel-19 AIoT command mechnaism. It is recommended not to be included in the Interim Agreement. For the improvements on periodic data collection, further study is required.

DO-A Delivery from AIoT Device to AIOTF

- All pCRs support the AIoT Devices sends DO-A data to the AIOTF via NAS message.

- Regarding the security protection, S2-2510150, S2-2510319, S2-2510408, S2-2510468, S2-2510667 support the security protection of the NAS message and the dependency on SA3.

- Regarding whether the NAS message needs to be acknowledged, S2-2510319 supports the acknowledge. S2-2510083 suggests depending on whether it is needed as an acknwolege in MAC layer. S2-25010652 supports the DO-A ACK, but the delivery mechanism will be different depending on whether the radio resource has been released or not. S2-2510667 introduces an indicator to NG-RAN about the follow-on NAS message is expected, which hints the optional follow-on NAS message(s)

- Regarding the data from the AIoT device, besides the DO-A data, S2-2510083, S2-2510319, S2-2510468, S2-2510667 propose temporary ID. S2-2510150 suggests AIOTF ID. S2-2510147 recommnds temporary ID and optional AF info, AIOTF info. S2-2510652 suggests target AF information if the data needs to be delivered to multiple AFs.

[Proposal-2]: It is agreed the AIoT Devices send DO-A data to the AIOTF via AIoT NAS message which is security protected. Whether NAS ACK response is needed is for further study and also depends on RAN2. Other information provided by the AIoT Devices is FFS.

Message routing from AIOTF to AF

- S2-2510083, S2-2510150, S2-2510244, S2-2510319, S2-25010408, S2-2510667 support the AIOTF retrieves the target AF information from ADM.

- S2-2510147 suggests either based on the AF information indicated by the AIoT Device, or by retrieving the target AF information from the ADM.

- S2-2510652 recommends that if the AIoT Device is allowed to send the DOA Data to multiple AFs, the target AF information is required in the NAS message when the AIoT Device triggers the DOA Data. If the target AF information is not included and the AIoT Device is only allowed to send the DOA Data to a specific AF, the AIOTF can retrieve the AF information from the ADM.

- Regarding the information to be sent to the AF, S2-2510083 and S2-2510319 supports DO-A data and AIoT Device Permanent ID.

[Proposal-3]: It can be regarded as the baseline solution that the AIOTF retrieves target AF information from ADM. After the AIOTF receives the AIoT NAS message, the AIOTF retrieves the target address from the AIoT Device Profile Data in ADM if not available locally.

Whether the message routing based on AF information from AIoT Devices need further study, as it is unclear on how the solution works if NEF is expected to be involved (i.e., the AF is outside trusted domain) and it is less efficient to let AIoT Device provide AF information over the air.

Regarding the information to the AF, based on the analysis, the AIoT Device Permanent ID is essential.

AF subscribe/configure the target AF information

- S2-2510083, S2-2510150, S2-2510244, S2-2510408 support AF subscribe to the AIOTF for the DO-A Delivery from AIoT devices. S2-2510652 seems also on this direction.

- S2-2510147 proposes the OAM configuration in ADM for the target AF information

- S2-2510667 suggests letting AF configure the target AF information to ADM via the NEF.

[Proposal-4]: From “subscribe/notification” SBI design principle perspective, it is recommended to let subscribe option be the baseline solution.

For OAM option, usually, it is always possible to configure AIoT Device Profile Data via OAM. But it is not clear on how the configuration works if NEF needs to be involved (i.e., AF is outside trusted domain).

For the configuration option, the AIOTF needs to notify the DO-A data to the NEF, but there is neither implicit nor explicit subscription from the NEF to the AIOTF, which is less clean from “subscribe/notification” SBI design perspective. And the AF authorization is performed in the ADM which does not align with the AF authorizaiton solution in Rel-19. It requires cross data-set operation (i.e., the ADM authorize the configuration data from the NEF which needs to access AF authorization data and the updates is on AIoT Device Profile Data).

It is recommended that OAM option and configuration option are FFS.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-30:

* * * Start of Change (All texts are new) * * * *

### 7.1.2	Agreed Principles for KI#2

Following principles have been agreed for DO-A data transfer aspects in KI#2:

- DO-A capable AIoT Devices initiates DO-A data transfer, triggered by the conditions which can be stored in local configuration or device implementation. The local configuration can be configured by the AF using the AIoT command procedure.

Editor’s note:    It is FFS on the potential enhancement of Rel-19 command procedure to support configuration by the AF to the AIoT Device.

Editor’s note:    It is FFS on the potential enhancement of periodic data collection.

NOTE 1:	The conditions in the local configuration are on application-level and out of scope of 3GPP.

- DO-A capable AIoT Devices send DO-A data to the AIOTF via AIoT NAS message. The AIoT NAS message is security protected.

NOTE 2:     The details of security protection of AIoT NAS message will be determined by SA3 WG.

Editor’s note:		Whether a NAS ACK response is needed is FFS, and it also depends on RAN2 to determine whether a NAS ACK response message is needed as an acknowledgement in MAC layer.

Editor’s note:    Other information provided by AIoT Devices is FFS, including temporary ID, AIOTF ID, AF information and follow-up message indication.

- After the AIOTF receives the AIoT NAS message, the AIOTF retrieves the target address from the AIoT Device Profile Data in ADM if not available locally.

- The AIOTF notifies the DO-A data and the AIoT Device Permanent ID to the AF(s), optionally via the NEF.

Editor’s note:		It is FFS on whether the AIOTF routes the DO-A data based on the AF information from AIoT Devices.

Editor’s note:		It is FFS on whether the AIoT Device Profile Data stores multiple target addresses.

- The AF subscribes to the AIOTF for the DO-A data delivery of AIoT Devices directly or via the NEF, with target address provided.

Editor’s note:		Whether subscription is used to update AIoT Device Profile Data with target address is to be determined.

- The network performs AF authorization for the subscribe request from the AF. If authorized, the AIoT Device Profile Data in ADM for the target address is updated.

Editor’s note:		It is FFS on the OAM option and the configuration solution.

* * * End of Change * * * *