SA WG2 Meeting #172	S2-2511021

17 – 21 November 2025, Dallas, US	(revision of S2-2511004 of S2-2509910)

Source:	LG Electronics, FutureWei, NTT DOCOMO, Samsung, Xiaomi, CATT, Tejas Networks Limited, Ericsson, Lenovo, vivo, CEWiT, Apple, OPPO

Title:	KI#3: Conclusions and Interim Agreements

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes conclusions and interim agreements for KI#3.

1. Discussion

As a result of SA2#171, the following Interim Agreements for KI#3 were added to TR 23.700-14.

7.1.3 Agreed Principles for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection The following principles are agreed in this release to address the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection: - The Sensing Function selection is based on NEF's local configuration or NRF query. - The Sensing Function registers itself to the NRF, with its NF profile. - Sensing Function is selected based on at least its supported sensing area. - Sensing Function selects one or more Sensing Entities, i.e. gNBs, based on the sensing service request. - Sensing Function selects the Sensing Entities, i.e. gNBs, based on information about the Sensing Entities, e.g. its supported sensing area. NOTE: The architecture aspect of this KI's interim conclusions will be aligned with KI#1 conclusions.

At this meeting, KI#3 should be concluded.

2. Proposal

It is proposed to agree the following changes into TR 23.700-14.

***********************************************************************************************

LGE as a pen holder provides the merged Interim Agreements for KI#3 based on the following 16 tdocs.

| Tdoc# | Subject | Source | Clause(s) |
| --- | --- | --- | --- |
| S2-2509910 | 23.700-14: KI#3: Conclusions and Interim Agreements | LG Electronics | Clauses 7 & 8 |
| S2-2509955 | 23.700-14: Proposal on Interim agreements update for KI#3 | OPPO | Clauses 7 & 8 |
| S2-2509991 | 23.700-14: Proposed Conclusions for KI#3 | CEWiT | Clause 8 |
| S2-2510037 | 23.700-14: FS_Sensing_ARC -- update on Interim agreements for KI#3 | China Mobile Com. Corporation | Clause 7 |
| S2-2510070 | 23.700-14: Proposal for KI#3 conclusion | InterDigital Communications | Clauses 7 & 8 |
| S2-2510134 | 23.700-14: Update the agreed principles and add conclusion for KI#3 | CATT | Clauses 7 & 8 |
| S2-2510197 | 23.700-14: KI#3 Conclusion proposal | ZTE | Clauses 7 & 8 |
| S2-2510389 | 23.700-14: Conclusions on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection | vivo | Clause 8 |
| S2-2510411 | 23.700-14: KI#3: conclusion update proposal | Huawei, HiSilicon | Clauses 7 & 8 |
| S2-2510531 | 23.700-14: KI#3: Update on Interim Agreements | TOYOTA MOTOR CORPORATION, BUPT | Clause 7 |
| S2-2510539 | 23.700-14: KI#3 Interim Agreement Update | Sony | Clauses 7 & 8 |
| S2-2510571 | 23.700-14: Update: Interim agreements for FS_Sensing_ARC KI#3 | NTT DOCOMO | Clause 7 |
| S2-2510679 | 23.700-14: Interim agreement update and final conclusion for KI#3 | Samsung | Clauses 7 & 8 |
| S2-2510697 | 23.700-14: KI#3: Update to the Interim Agreements | Lenovo | Clause 7 |
| S2-2510719 | 23.700-14: Conclusion for KI#3 Sensing Entity and Sensing Function Discovery and (Re-)Selection | Beijing Xiaomi Electronics | Clause 8 |
| S2-2510764 | 23.700-14: FS_Sensing_ARC: Additional principles for KI#3 on Sensing Entity and Sensing Function Discovery and (Re-)Selection | Qualcomm Incorporated | Clause 7 |

***********************************************************************************************

* * * * Start of 1st Change * * * *

!! All New Texts !!

8.3	Conclusion on KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection

The following principles are agreed in this Release to conclude the key issue on Sensing Entity and Sensing Function Discovery and (Re-)Selection for normative work:

1.	The AF and NEF perform discovery and selection of Sensing Function (SF).

2.	The Sensing Function selection is based on local configuration or NRF query.

3.	Sensing Function re-selection is not supported in this release.

4.	The Sensing Function registers itself to the NRF, with its NF profile that contains the following, e.g.:

-	NF Type.

-	Supported sensing area of Sensing Function.

5.	Sensing Function is selected based on at least supported sensing area of SF.

6.	The Sensing Function selects one or more Sensing Entity(ies) (i.e. gNB(s)), based on the sensing service request (e.g., target sensing service area), supported sensing area of Sensing Entity.

7.	The Sensing Function may re-select Sensing Entity(ies) (i.e. gNB(s)), based on the sensing service request (e.g., target sensing service area), supported sensing area of Sensing Entity.

NOTE 1:	How SF can obtain the information of SEs (e.g. via OAM, via signalling) will be determined during the normative phase and require coordination with RAN WGs.

NOTE 2:	The details of the information/capability of SE and the information/capability used for SE (re-)selection will be determined during the normative phase and require coordination with RAN WGs.

* * * * End of Changes * * * *