3GPP TSG-WG SA2 Meeting #172	S2-25011048

Dallas, US, 17th Nov – 21st Nov, 2025	(revision of S2-2511047)

Source:	Huawei, HiSilicon, Ericsson, Samsung, FutureWei, Nokia, LG Electronics, China Mobile, NEC, ZTE, CATT, Tejas Networks, NTT DOCOMO, Lenovo, vivo

Title:	Key Issue 2 Interim Conclusion Proposals

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: this pCR proposes interim conclusion for key issue 2.

- Introduction

There are 19 solutions for key issue 2 are documented in TR 23.700-30, v0.2.0. It is observed the following two DO-A related features are widely proposed in the TR solutions:

- DO-A capable Device registration

- DO-A data transfer

The pCR focused on the aspect of DO-A capable Device registration, and proposed interim conclusions based on the following papers submitted to SA2#172.

| S2-2510083 | 23.700-30: Interim Agreements for KI#2 | Ericsson |
| --- | --- | --- |
| S2-2510147 | 23.700-30: KI#2 Interim Agreements on Device registration and DO-A transmission | Lenovo |
| S2-2510150 | 23.700-30: [KI#2] Interim agreements | Sony |
| S2-2510319 | 23.700-30: Interim Agreements for KI#2 | CATT |
| S2-2510327 | 23.700-30: Draft interim Agreements for KI#2 | Tejas Network Limited |
| S2-2510408 | 23.700-30: KI#2 Interim agreement | LG Electronics |
| S2-2510468 | 23.700-30: KI#2 Interim Agreements | Samsung |
| S2-2510652 | 23.700-30: Interim Agreement for KI#2 | OPPO |
| S2-2510667 | 23.700-30: Key Issue 2 Interim Conclusion Proposals | Huawei, HiSilicon |
| S2-2510712 | 23.700-30: Interim Agreement for KI#2 | Vivo |

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30, v0.2.0.

# * * * * First change * * * *

7.1.Y	Agreed Principles for KI#2

7.1.Y.1	DO-A capable Device Registration for topology 1

Initial registration

A.	Initial registration is supported and used by the DO-A capable AIoT Device to inform the network of its presence and get authenticated/authorized by the network. After successful registration, the AIoT Device and the network establish a context which is used, for example, in the subsequent DO-A data transfer.

Editor’s note:	It will be revisited whether or not to support initial registration in SA2#173.

B.	The AIoT Device determines to perform the initial registration based on its implementation e.g., power on.

C.	Following information is sent by the AIoT Device for initial registration:

1.	An AIoT Device identifier, it is:

-	If available, the AIoT temporary ID information,

-	otherwise, a form of the permanent AIoT Device identifier.

2.	AIoT NAS Registration Request message

3.	AIOTF routing information in AS, if available.

NOTE XX: Whether the AIoT Device identifier and AIOTF routing information are combined or are separate will be coordinated with SA3.

NOTE YY: The form of the permanent AIoT Device identifier will be coordinated with SA3.

D.

NG-RAN selects an AIOTF based on the AIOTF routing information, if provided by the AIoT Device, or local configuration.

E.	NG-RAN sends at least the NAS registration request message from the AIoT Device to the selected AIOTF. If Indirect Connectivity is used between NG-RAN and the AIOTF, NG-RAN sends the selected AIOTF ID to AMF which can then route the information to the selected AIOTF.

F.

If the initial registration request from the AIoT Device is accepted, the serving AIOTF may allocate an updated AIoT temporary ID information. The AIOTF sends an AIoT NAS Registration Accept to the AIoT Device, including the updated AIoT temporary ID information and AIOTF routing information.

The AIoT Device acknowledges the AIOTF by sending a Registration Complete message.

G.	The AIOTF considers the AIoT Device as registered when it has an AIoT Device context for the AIoT Device. The AIOTF stores its AIOTF ID in the AIoT Device profile data in the ADM as serving AIOTF for the AIoT Device.

H.	The ADM informs the old AIOTF (if available) that it is no longer the serving AIOTF for the AIoT Device, and the old AIOTF removes the AIoT Device context locally.

Mobility registration

Editor’s note:	whether and how to support mobility registration is FFS.

Periodic registration

Editor’s note:	whether and how to support periodic registration is FFS.

# * * * * End of changes * * * *