3GPP TSG-SA2 Meeting #172	S2-2511050

17 - 21 November, 2025, Dallas, USA                             (was S2-2511031)

Source:	Nokia, Qualcomm Incorporated

Title:	Conclusions proposal for KI#2

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes to proceed with some conclusions for normative work for KI#2

# 1.	Introduction

At SA2#171 some principles were agreed. This paper aims at documenting a set of conclusions for KI#2 as a basis for normative WID drafting at SA2#172.

# 2. Conclusion

It is proposed that the following text is agreed for TR 23.700-67

Start of changes (all new text)

## 8.2	Conclusions for KI#2

As a result of the study documented in this TR, the following are to be specified as part of rel-20 normative specifications:

-	The PCF may subscribe to notifications of energy related information from the EIF and OAM.

NOTE 1:	Details of the notification reporting control (e.g. how to prevent too frequent notifications) will be decided during normative phase.

NOTE 2:	Renewable energy information can be considered based on SA5 feedback on supporting renewable energy.

-	The PCF may make the following policy control decisions for network energy saving based on subscription information, AF request (e.g. preferred or expected network energy saving behaviours such as QoS adjustment) and/or operator policy, and/or taking into account energy related information from the EIF/OAM:

-	SM policy control.

NOTE 3:	Based on operator policy, it should have the balance between energy saving and user experience.

-	PDTQ policy negotiation with the AF, e.g. adjusting the time window and/or QoS for data transfer.

-	Optionally UE policy control, i.e. generating URSP rules with the existing parameters based on energy related information from the OAM.

-	Reusing the existing AQP framework, Alternative QoS Profiles are provided to the NG-RAN additionally indicating to the NG-RAN (e.g., with an indication per Alternative QoS profile or per QoS profile or per UE) that the NG-RAN can adjust the QoS level for the UE based on NG-RAN energy related conditions or local policy.

NOTE X: Whether to consider normative work on AQP for energy saving is dependent on the RAN WG inputs.

NOTE Y: Whether the indication provided to the NG-RAN is per Alternative QoS profile or per QoS profile or per UE is dependent on the RAN WG inputs.

-	The AF may subscribe to be notified for the result of energy saving decisions made by PCF along with the request for preferred or expected energy saving behaviours.

-	The PCF may notify the SMF when making policy change decisions due to network energy saving. The SMF, in turn, may provide the reason for the service adjustment to the CHF.

Editor's note: Whether to provide the reason (i.e. network energy saving) for the service adjustment to the CHF depends on the feedback from SA WG5.

NOTE 4:	Existing AF services can be updated but no new AF services will be introduced for the AF request.

End of changes