SA WG2 Meeting #172	S2-2511152

17 - 21 November, 2025, Dallas, USA

# Title:	[DRAFT] Reply LS on Multiple Notification targets for NF Event Exposure services

# Response to:	C3-254606, S2-2509861, LS on Multiple Notification targets for NF Event Exposure services

# Release:	Rel-17

# Work Item:	eNA_Ph2

# Source:	SA2

# To:	CT3

# Cc:	CT4

# Contact Person:	Xiaoyan Duan

# duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1	Overall description

SA2 thanks CT3 for the LS on multiple notification targets for NF Event Exposure services.

SA2 discussed the question from CT3 and would like to provide the following answer:

Answer: When the DCCF receives a subscription with multiple notification target addresses, the DCCF shall not send the multiple notification target addresses in the subscription request towards the data source. Instead, the DCCF includes its own address as the notification target address in the subscription request towards the data source, and sends the notifications to the multiple notification target addresses by itself after receiving the notification from the data source. This has been specified in clause 5A.3.1 and 6.2.6.3.2 of TS 23.288.

Excerpts from clause 5A.3.1 and 6.2.6.3.2 of TS 23.288 are cited as below for your reference, with relevant texts highlighted:

5A.3.1	Data Delivery via the DCCF or NWDAF

Figure 5A.3.1-1: Data Delivery via DCCF

Data Delivery via DCCF is shown in Figure 5A.3.1-1. Each Event Notification received from a Data Source NF is sent to the DCCF which propagates it to all Data Consumers / Notification Endpoints specified by the Data Consumers or determined by the DCCF. Each Data Consumer may specify in its request to the DCCF multiple notification endpoints, which may include the requesting Data Consumer, an ADRF or other NFs. The DCCF may also select an ADRF or other notification endpoint based on configuration. The DCCF supports formatting and processing for each Consumer / notification endpoint so notifications comply with the data requests received from each Consumer NF.

Upon the DCCF determining the status of data collection for a data request (see clause 5A.2):

-	If the requested data is not already being collected from a Data Source, the DCCF sends a new subscription/request towards the Data Source with the notification target specified as the DCCF.

-	If the requested data is partially covered by existing subscriptions with the Data Source, the DCCF sends to the existing Data Source a request to modify the subscription and/or creates new subscription(s) to new Data Source for the newly requested data which cannot be provided by the current Data Source.

-	If the requested data is already being collected from the Data Source, the DCCF determines that no subscriptions to the Data Source need to be created or modified.

[…]

6.2.6.3.2	Data Collection via DCCF

[…]

1.	The data consumer subscribes to data via the DCCF by invoking the Ndccf_DataManagement_Subscribe (Service_Operation, Data Specification, Formatting Instructions, Processing Instructions, NF (or NF-Set) ID, ADRF Information) service operation as specified in clause 8.2.2. The data consumer may specify one or more notification endpoints. …

[…]

5.	…If the data requested at step 1 are not already available or not being collected yet, the DCCF subscribes to data from the NF using the Nnf_EventExposure_Subscribe service operation as specified in clause 5A.2 and clause 6.2.2.2, with DCCF indicated as Notification Target Address. The DCCF adds the data consumer to the list of data consumers that are subscribed for these data.

[…]

7.	The DCCF uses Ndccf_DataManagement_Notify to send the data to all notification endpoints indicated in step 1. …

[…]

2.	Actions

To CT3:

ACTION: SA2 kindly asks CT3 to take the above information into account.

3.	Date of Next SA2 Meetings

SA2#173	09 – 13 February, 2026	Goa, IN

SA2#174	13 – 17 April, 2026			Malta, MT