SA WG2 Meeting #S2-172	S2-2511154

17 - 21 November, 2025, Dallas, USA

Title:	Reply to LS on a new measurement related to the maintainability of a QoS flow

Response to:	Reply to LS on a new measurement related to the maintainability of a QoS flow (S2-2509876 / S5-254627)

Release:	Release 19

Work Item:	eNA_Ph3, TEI19

Source:	SA2

To:	SA5

Contact person:	Thomas Belling

thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.288 CR 1535

# 1	Overall description

SA2 thanks SA5 for their LS.

SA2 informs SA5 that SA2 agreed the attached CR to improve the QoS sustainability analytics with the new measurement related to the maintainability of a QoS flow introduced by SA5 as input data.

SA5 asked SA2 to assess whether the measurements specified by SA5 address SA2’s requirements, and to provide any feedback. In response, SA2 suggests some requirements for the new QoS flow Maintainability KPI and request SA5 to assess if those requirements are already met and otherwise to consider possible updates to the KPI to meet those requirements:

SA2 wants to clarify that the new KPI only applies to GBR flows, and that for the purpose of the new KPI a QoS flow is downgraded when the requested QoS can no longer be provided and a related notification according to TS 23.501, Clause 5.7.2.4 is sent; this applies for QoS flows with and without Alternative QoS Profiles (AQP) according to TS 23.501, Clause 5.7.1.2a.

SA2 would like to suggest that possible changes between AQPs (if there are multiple AQPs for a QoS flow) as well as notifications that the requested QoS is again met (e.g. due to change from an AQP to the requested QoS profile) are not considered for the QoS flow Maintainability KPI; i.e. only notifications that the requested QoS cannot be met (e.g. due to changes from the requested QoS profile to some AQP) should be considered in the QoS flow Maintainability KPI.

# 2	Actions

To SA5

ACTION: 	SA2 kindly asks SA5 to assess if the requirement that only downgrading events from the requested QoS profile are considered is already met by the QoS flow Maintainability KPI and otherwise to consider possible updates to the KPI to meet those requirements.

# 3		Dates of next TSG CT WG 3 meetings

SA2#173                                 February 9th – 13th, 2026                                 Goa, India

SA2#174                                 April 13th – 17th, 2026                                      Malta, EU