SA WG2 Meeting #172	S2-2511157

17 - 21 November 2025, Dallas, USA	(revision of S2-2511125)

Source:	Futurewei, Nokia, Ericsson, NEC

Title:	[WT#1.2, Enhanced User Plane] Scope and Key Issue for Enhanced User Plane

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Enhanced User plane revision with updates to work task scope and key issue.

- Justification:

This document merges the following WT1.2 User Plane Architecture contributions:

| S2-2509918 | SK Telecom, Deutsche Telekom, Verizon, Boost Mobile, Rakuten Mobile, China Mobile |
| --- | --- |
| S2-2509953 | CATT |
| S2-2510047 | Futurewei, Nokia, Ericsson, NEC |
| S2-2510104 | China Mobile |
| S2-2510115 | Vivo |
| S2-2510172 | Mediatek Inc. |
| S2-2510328 | CATT, China Mobile, China Telecom |
| S2-2510431 | Huawei, Hisilicon |
| S2-2510502 | ETRI |
| S2-2510635 | ZTE |
| S2-2510696 | Samsung |
| S2-2510735 | Ofinno |
| S2-2510748 | Xiaomi |
| S2-2510758 | China Telecom Corporation Ltd. |

Pen-holder recommendations:

- S2-2509918 provides the details of requirements for enhancing reliability, scalability for UPF.

- S2-2509953 and S2-2510735: propose to change user plane function to user plane “node”. Function is more accurate for UP architecture requirements (i.e., not necessarily an instance).

- S2-2510104 proposes the following updates: 
- study whether N4 may be extended to be an SBI. This is already covered in bullet 1.
- new bullet 4 is already covered in NOTE z.
- Transport technology is not in the scope of 6G user plane architecture.

- S2-2510115 proposes a new clause and NOTE. Network header compression in CN is not needed, and small packet transmission efficiency is handed by transport network underlays. Signalling optimization would be mainly in WT1.1 NAS. Signalling optimization in UP is covered in bullet 2.

- S2-2510172 notes that “data network” may refer to local or external, and external data network is out of scope. Added NOTE c to clarify.

- S2-2510328 proposes addition to bullet 1 - NOTE a: Generic aspects of SBA are handled in WT1.2 SBA framework. SBA aspects of CP-UP functional split are handled in this WT; and reference to SRv6 in bullet 2. 
Bullet 1 already does not restrict discussion of SBA (or PFCP). Bullet 2 does not restrict discussion of user plane architectural aspects to any specific protocol (GTP-U, SRv6 or QUIC).

- S2-2510431 and S2-2510696 propose further details to enumerate aspects of bullet 2, mainly in the KI.
Key aspects proposed are captured in the WT. Following recommendations to keep WT and KI the same.

- S2-2510502 proposes to align format with TR 23.801-01. Revised heading/subsection to A.1.2.X as indicated and now aligns with TR 23.801-01.
NOTE z is moved to the end of the set of requirements as suggested.
Bullet 1 rewording is like baseline after CC#2 and therefore no change is made.

- S2-2510635. Change in bullet 2 is like proposal in S2-2510047 – added. Change to bullet 1 is already captured in ‘enhance CP-UP functional split’. Clarified that bullet 3 is for user plane function and have agreement that it is OK to keep/study.

- S2-2510748 – with the revision from CC#2 to scope bullet 3 to user plane function aspects (while bullet 2 is on user plane flexibility), overlap is naturally avoided (NOTE M’s intention is covered).
NOTE N on QoS – there is no dependency to consider at this stage (while solutions are not yet designed).

- S2-2510758 proposal to add “traffic handling efficiency” for ULCL/multiple anchors is covered in bullet 2 (it is one of the original motivating use cases).

**** First Change ****

## A.1.2	Work Task 1.2

### A.1.2.X	User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

- Whether and how to enhance user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility with consideration of user plane function capability and path performance between access network and data network.

NOTE b: This work task may require coordination with RAN WG for user plane interface.

- Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE z: The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

**** Second Change ****

# 5.X. Key Issue #X: User Plane Architecture

In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied taking the 5GS user plane framework as a starting point for discussion:

- Whether and how to enhance CP-UP functional split and interaction for better multi-vendor interoperability.

- Whether and how to enhance user plane flexibility (by user plane function (re)selection) for different service requirements, session continuity and mobility with consideration of user plane function capability and path performance between access network and data network.

NOTE b: This work task may require coordination with RAN WG for user plane interface.

- Whether and how to enhance resilience, scalability, and high availability of user plane function.

NOTE z: The outcome of this study may include architectural requirements based on which stage 3 working group can work on the user plane protocols. The stage 3 protocol aspects, including how to design and select user plane protocols, are not in the scope of stage 2.

**** End of Changes ****