3GPP TSG-SA WG2 Meeting #172	S2-2511158

17–21 Nov, 2025, Dallas, USA	was S2-2510698

Source:	Samsung (Penholder)

Title:	[WT#1.2, Localized Service Access] WT Scope and Key Issue for Support of Localized Service Access in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: (Penholder version) this contribution proposes the WT scope and Key Issue for supporting localized service access.

# Justification

Summary from discussion in Pre-SA2#172 CC#1 and potential way forward:

- WT scope seems to be too broad for few companies. After various offline discussion, the main justification for studying alternate solutions by proposing companies was to have some “kind of local control” on subscription for the UEs accessing localized services

- Bullet 1 is now confined to only the case for PNI-NPN, (i.e. Localized service access provided with the support of PLMN)

- Added a new Note 1c to clarify the solutions, which use PNI-NPN, should have no UE dependency. This was proposed by Qualcomm because the justification from proposing companies to study new solutions was the concerns regarding 5G solutions (e.g. SNPN or PNI-NPN with CAG) having a lot of UE dependency.

- Note 3a was further clarified; the purpose not interpret that the “localized” services (which may be confined to only a specific area) need to have service continuity.

- Bullet 2 is split into 2 cases now: one for SNPN and one for PNI-NPN/part of PLMN

Following contributions submitted to SA2#172 are considered for the merged proposal

| Localized Service Access |  |  |  |
| --- | --- | --- | --- |
| S2-2509944 | 23.801-01: WT#1.2_Localized Service Access | CATT | Adds bullet on Discovery and NW selection |
| S2-2510054 | 23.801-01: [WT#1.2, Localized Service Access] Support of Localized Service Access in 6G. | China Mobile | Adds Note on selection and Access control being out of scope : covered by Note 1b use a different wording for Bullet 1 related to local control. Removed interface and interaction procedures in bullet 3. |
| S2-2510118 | 23.801-01: [WT#1.2, Localized Service Access] Support of Localized Service Access | vivo | Only change is adding a new bullet to reduce UE impacts : Already covered by Note 1b and 1c |
| S2-2510373 | 23.801-01: [WT#1.2, Localized Service Access] Support of Localized Service Access in 6G | China Telecom | Adds Note on selection and Access control being out of scope : covered by Note 1b use a different wording for Bullet 1 related to local control. |
| S2-2510429 | 23.801-01: [WT#1.2, Localized Service] WT & KI for Localized Service Access | Huawei, HiSilicon | Adds bullet on NW access control: Specifies that minimal dependency on UE and RAN is needed for network access control. use a different wording for Bullet 1 related to local control. |
| S2-2510536 | 23.801-01: [WT#1.2, Localized Service Access] Support of Localized Service Access in 6G. | Nokia | Removes everything except bullet 1. Naturally implies no NW selection, NO access control, no local control of subscription |
| S2-2510636 | 23.801-01: [WT#1.2, Localized Service Access] Localized Service Access | ZTE | Removes note on Access NW selection being out of scope |
| S2-2510698 | 23.801-01: [WT#1.2, Localized Service Access] WT Scope and Key Issue for Support of Localized Service Access in 6G | Samsung | NOTE 1B and 1c tries to specific that NW selection and Access control are out of scope, and less UE impacts is preferred compared to regular PLMN UEs |

Discussion points and way forward proposals:

- Whether to use the “via PNI-NPN” or “via PLMN” or “via part of PLMN”

PNI-NPN -> 			Yes: 		No

Via PLMN ->   		Yes: 		No

Via part of PLMN ->   Yes: 		No

  - CAG was the major issue from the companies as they did not wanted to re-use CAG. The definition of PNI-NPN is quite general in 23.501 and does not imply any CAG specific procedures that are need to be used.

  - Some companies think that since some new services may not be available using PDU Sessions as such (and thus may not be tied to Slice/DNN), thus using PNI-NPN may not be wise.

- Regarding Note 1b: Whether to have Network Selection and Network access control in the scope of this WT.

  - Proposal 2: New mechanisms for NW selection/Access control are not to be studied. But re-use of mechanisms from 5G could still be proposed to explain the end-to-end solution.

- Regarding Note 1c: Whether to have Network Selection and Network access control in the scope of this WT.

Option 1: No UE dependency at all, over what is needed for PLMN access

Option 2: No additional UE dependency i.e., UE dependency for localized service access is allowed if something from 5G is re-used.

Option 3: Remove any restriction to the solutions and decide during Evaluation phase.

  - Proposal 3: Similar to proposal 2, for UE dependency for the case of PNI-NPN/part of PLMN; additional UE impacts are not expected (over what is needed for general PLMN access); but solution can still propose to re-use any mechanisms that were specified in 5G even if those have UE dependencies.

- Determine the exact wording for the Sub-bullet 1.1 related to local management aspects. Some proposals:

Option 1: use a generic terminology to just refer to the SA1 requirements.

e.g. For the case when localized service access is provided via PNI-NPN/part of the PLMN, study whether and how to support new service requirements to be specified by SA1

Option 2: continue working/refining on the wording proposed in the CC: some proposals:

  - Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the PNI-NPN/part of PLMN providing localized service access .

  - Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the PNI-NPN/part of PLMN providing localized service access.

  - Whether and how to enable local usage and management for the part of subscription (that is needed for localized service access) in the PNI-NPN/part of PLMN providing localized service access.

  - Whether and how to manage and use the UE subscription by the related NFs present locally in the PNI-NPN/part of PLMN providing localized service access .

  - Whether and how to manage and use the UE subscription by the related NFs present locally in the PNI-NPN/part of PLMN providing localized service access.

**** Option 1****

# Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, which consists of the following:

NOTE 1b:	New mechanisms for Network selection and access control that are specific for localized service access are not to be studied in this work task. Solution can propose to re-use the related mechanisms specified in 5G, or assume that mechanisms used for general PLMN access can be re-used.

NOTE 1c: For the case when Localized service access is provided via PNI-NPN/part of PLMN, solutions are expected not to introduce any additional UE functionality that is specific for localized service access. Solutions can still propose to use any mechanisms specified in 5G (e.g. CAG) that may be specific for localized service access which require UE dependency beyond what is needed for general PLMN access.

- For the case when localized service access is provided via PNI-NPN/part of PLMN, study how to perform authorization and authentication of UE for localized service access including

  - Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present in the PNI-NPN/part of PLMN providing localized service access.

NOTE 2: Authentication aspects need to be coordinated with SA3

- Whether and how to support UE mobility and/or service continuity between PLMN network and SNPN, or between the SNPN, including interface and interaction procedures, if any, between PLMN network and SNPN, or between the SNPNs.

- Whether and how to support UE mobility and/or service continuity between PLMN network and PNI-NPN/part of that PLMN(i.e. PNI-NPN/part of PLMN belonging to the same PLMN) providing localized service access, or between the PNI-NPNs/parts of a PLMN (i.e. PNI-NPNs/part of PLMN belonging to same PLMN) providing localized service access, including interface and interaction procedures, if any, between PLMN network and PNI-NPN/part of that PLMN” providing localized service access, or between the PNI-NPN/parts of a PLMN” providing localized service access.

NOTE 3a: For service continuity, only those services are considered such that the same service (e.g. internet connectivity) can be provided or accessed by both the initial/source network and the latter/target network.

NOTE 3b: Localized service access can be provided by a part of a PLMN as well.

**** Option 2: separate out into 2 scenarios ****

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, which consists of the following:

- For the case when localized service access is provided via PLMN, study how to perform authorization and authentication of UE for localized service access including

  - Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the network providing localized service access.

NOTE 1b:	Network selection and access control are not to be studied in this work task.

NOTE 1c: For the case when Localized service access is provided via PLMN, solutions are expected not to introduce additional UE functionality “that is specific for localized service access” (i.e. anything that requires UE dependency beyond what is needed for general PLMN access).

NOTE 2: Authentication aspects need to be coordinated with SA3

Items that require consensus whether to have in scope or not

For bullet 1

- Whether and how to support UE mobility and/or service continuity between PLMN network and “part of that PLMN” providing localized service access, or between the “parts of a PLMN” providing localized service access, including interface and interaction procedures, if any, between PLMN network and “part of that PLMN” providing localized service access, or between the “parts of a PLMN” providing localized service access.

NOTE 3aX: For service continuity, only those services are considered such that the same service (e.g. internet connectivity) can be provided or accessed by both the initial/source network and the latter/target network.

- Study whether and how to adopt 5G NPN (SNPN and PNI-NPN) for localized service access in 6G and additionally study the following:

2.1 Whether and how to support UE mobility and/or service continuity between PLMN network and SNPN, or between the SNPN, including interface and interaction procedures, if any, between PLMN network and SNPN, or between the SNPNs.

NOTE 3a: For service continuity, only those services are considered such that the same service (e.g. internet connectivity) can be provided or accessed by both the initial/source network and the latter/target network.

**** Second Change ****

# 5.X. Key Issue #X: Localized Service Access

To support the localized service access in 6G, the following are studied:

NOTE 1: Support for access to Localized Services as specified in 5G (refer to TS 23.501) is the starting point for the support of 6G use cases and requirements related to localized service access.

NOTE 1b:	Network selection and access control are out of scope of this work task.

NOTE 1c: For the case when Localized service access is provided via PNI-NPN, solutions are expected not to introduce additional UE functionality that is specific for localized service access to avoid dependency on specific UE feature support.

NOTE 1d: PNI-NPN does not imply that CAG feature is required to be used.

- For the case when localized service access is provided via PNI-NPN, study how to perform authorization and authentication of UE for localized service access including

  - Whether, how and which aspects (e.g. provisioning, updating, retrieval) for the part of subscription (that is needed for localized service access) can be performed/managed via the NFs present locally in the network providing localized service access.

- Whether and how to support UE mobility and/or service continuity between PLMN network and network providing localized service access, or between the networks providing localized service access, including interface and interaction procedures, if any, between PLMN network and networks providing localized service access, or between the networks providing localized service access.

NOTE 3a: For service continuity, only those services are considered such that the same service (e.g. internet connectivity) can be provided or accessed by both the initial/source network and the latter/target network.

NOTE 3b: Localized service access can be provided by a part of a PLMN as well.

**** End of Changes ****