3GPP SA WG2 Meeting #172	S2-2511169

Dallas, USA, 17 - 21 November 2025	(revision of S2-2510159)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6424 | rev | 6 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indirect Network Sharing for disaster condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom, China Telecom, Samsung, ZTE, Ericsson, Huawei, Nokia, Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_NetShare_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The Indirect Network Sharing can be used for the UE of the participating operator with the disaster condition to reduce the service interruption. In the case that the RAN of participating operator may be broken down during the disaster condition, the RAN of the hosting operator can broadcast the PLMN ID representing the participating operator so that the UE of participating operator can access the network via Indirect Network Sharing. In order to support the Indirect Network Sharing for disaster condition, the NFs of participating operator network (i.e. UDM, AUSF, SMF) should be aware of which UE(s) are in disaster condition via the message sent from the hosting operator network instead of in the normal Indirect Network Sharing case. This CR proposes to introduce and describe the Indirect Network Sharing for disaster condition. The INS for disaster condition could be provided by multiple hosting operators. If the inter-AMFs interface between the different hosting operators is not established, the RANs of the different hosting operators shall broadcast the different PLMN IDs representing the participating operator toward the UEs of the participating operator, otherwise the UEs will have their Tracking Area Updates rejected leading to a new Attach procedure. This will cause massive signalling load and can risk causing failure of the hosting operators! This shall be avoided. Therefore, this CR proposes the description when the multiple hosting operators can provide the INS for disaster condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the Indirect Network Sharing for Disaster Condition and describe the related function enhancement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Indirect Network Sharing for disaster condition cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.x (new), 5.40.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE (All the text is new)

5.18.x	Indirect Network Sharing for Disaster Condition

Subject to operator policy and national/regional regulations, the Indirect Network Sharing can be used by UEs of the PLMN with the Disaster Condition to minimize the service interruption.

NOTE 1:  In the case of Indirect Network Sharing for Disaster Condition, the UE is not aware of disaster condition and the UE behaviour is same as the normal case of Indirect Network Sharing as specified in clause 5.18.3.

NOTE 2:	How the hosting PLMN is made aware that another PLMN (as the participating operator network) is a PLMN with Disaster Condition and how the hosting PLMN is made aware of the area where the Disaster Condition applies is out of scope of 3GPP.

When the Disaster Condition arises, the AMF of the hosting operator’s network receives the NAS Registration Request message sent by the UE of the participating operator. The AMF identifies that the UE is registering to the network via the Indirect Network Sharing for Disaster Condition by checking that the UE is from a participating operator that can use Indirect Network Sharing for Disaster Condition and that the UE is registering to the network for the Indirect Network Sharing and the UE is in the area of Disaster Condition. If the above conditions are satisfied, comparing to the normal Indirect Network Sharing case, the AMF also provides the Disaster Roaming service indication to AUSF and UDM during the registration procedure.  In addition, the AMF also provides this indication to V-SMF during the PDU session related procedure and the V-SMF provides it to the H-SMF. The NFs of the participating operator network (e.g. AUSF, UDM and SMF) are able to identify which UE(s) are using the Indirect Network Sharing for Disaster Condition based on the Disaster Roaming service indication and the PLMN ID of the hosting operator.

NOTE 3:  The Disaster Roaming service indication is specified in the clause 5.40.4.

NOTE 4:  Per-PLMN unified access control can be used to reduce load in the hosting operator’s network and allow the access priority for the specific UEs during the Disaster Condition.

In the case that more than one hosting operators can provide the Indirect Network Sharing for Disaster Condition for the participating operator in the disaster area, in order to reduce the signalling load, the different hosting operators shall ensure their RANs broadcast different PLMN IDs all representing the participating operator toward the UEs of the participating operator.

NOTE 5:  In the above case, even if inter-AMFs interface between the different hosting operators can be established, it is recommended that the different hosting operators enable their RANs to broadcast different PLMN IDs representing the participating operator toward the UEs of the participating operator.

NOTE 6:	How to achieve to broadcast different PLMN IDs representing the participating operator is up to national/regional regulations and operator policy. For instance, if the participating PLMN is using a two digit MNC, e.g. MNC=XY, then the RANs of the hosting operators might be able to broadcast 3 digit PLMN IDs, e.g., MNCs XY1 and XY2 for the UEs of the participating operator.

When the Disaster Condition is no longer applicable, the RAN of the hosting operator can stop broadcasting of the PLMN ID representing the participating operator, the UE of the participating operator performs PLMN selection as described in TS 23.122 [17] and TS 24.501 [47] and returns to the PLMN previously with Disaster Condition. The hosting operator shall organize the return of the UEs of the participating operator in a manner that does not cause overload (e.g. of signalling) in the PLMN that previously had the Disaster Condition.

NOTE 7:  In the above case, the Network-initiated Deregistration (as specified in the clause 4.2.2.3.3 of TS 23.502 [3]) can be performed to the UEs of the participating operator.

NOTE 8:  In the above case, the AN release procedure (as specified in clause 4.2.6 of TS 23.502 [3]), considering optionally the setting of  RFSP index (as specified in clause 5.3.4.3), can be used to redirect active mode UEs of the participating operator to home network previously with Disaster Condition.

SECOND CHANGE

5.40.1	General

Subject to operator policy and national/regional regulations, 5GS provides Disaster Roaming service for the UEs from PLMN(s) with Disaster Condition. The UE shall attempt Disaster Roaming only if:

-	there is no available PLMN which is allowable (see TS 23.122 [17]);

-	the UE is not in RM-REGISTERED and CM-CONNECTED state over non-3GPP access connected to 5GCN;

-	the UE cannot get service over non-3GPP access through ePDG;

-	the UE supports Disaster Roaming service;

-	the UE has been configured by the HPLMN with an indication of whether Disaster roaming is enabled in the UE set to "disaster roaming is enabled in the UE" as specified in clause 5.40.2; and

-	a PLMN without Disaster Condition is able to accept Disaster Inbound Roamers from the PLMN with Disaster Condition.

In this Release of the specification, the Disaster Condition only applies to NG-RAN nodes, which means the rest of the network functions except one or more NG-RAN nodes of the PLMN with Disaster Condition can be assumed to be operational.

NOTE:	In this Release of the specification, 5GS also provides the Indirect Network Sharing for Disaster Condition to minimize the service interruption. The detailed operation is specified in the clause 5.18.x.

End of CHANGES