3GPP TSG-SA2 Meeting #172	S2-2511211

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0388 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS broadcast support for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.247 contains the term “NR NTN Intended Service Area” and states that related encoding and procedures are defined in TS 38.413 and TS 38.331. However, those specs use the terminology “MBS Intended Service Area List”: #### TS 38.413 defines #### 9.3.1.209 MBS Service Area Information This IE contains MBS service area information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. MBS Service Area Cell List; 0..<maxnoofCellsforMBS>. >NR CGI; M; 9.3.1.7. MBS Service Area TAI List; 0..<maxnoofTAIforMBS>. >TAI; M; 9.3.3.11. MBS Intended Service Area List; 0..<maxnoofIntendedAreasforMBS>. >Intended Service Area Coordinates; M; 9.3.1.273. #### 9.3.1.273 Intended Service Area Coordinates This IE provides information on the Intended Service Area Coordinates for MBS Broadcast Service. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Intended Service Area Coordinates; M; OCTET STRING; Includes the areaCoordinates contained in the IntendedServiceAreaInfo IE as defined in TS 38.331 [18]. #### TS 38.331 defines #### – SIBXX SIBXX contains the information of the intended service area(s) of an MBS broadcast service(s) in an NTN cell. SIBXX information element -- ASN1START -- TAG-SIBXX-START SIBXX-r19 ::= SEQUENCE { intendedServiceAreaList-r19 IntendedServiceAreaList-r19 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } IntendedServiceAreaList-r19 ::= SEQUENCE (SIZE (1..maxNrofMBS-Area-r19)) OF IntendedServiceAreaInfo-r19 IntendedServiceAreaInfo-r19 ::= SEQUENCE { intendedServiceAreaId-r19 MBS-IntendedAreaID-r19, areaCoordinates-r19 CHOICE { polygonArea-r19 OCTET STRING, circleArea-r19 SEQUENCE { center-r19 ReferenceLocation-r17, radius-r19 INTEGER(0..65535) } } } MBS-IntendedAreaID-r19 ::= INTEGER (1..maxNrofMBS-Area-r19) -- TAG-SIBXX-STOP -- ASN1STOP SIBXX field descriptions. intendedServiceAreaList Contains a list of intended service areas, each associated to one or more MBS broadcast services provided in an NTN cell.. polygonArea Parameter type Polygon defined in TS 37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.. |
| Summary of change: |  | Clarify that “NR NTN Intended Service Area” denotes the MBS Intended Service Area List as defined in TS 38.413 and 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misallignment with RAN terminology may cause confusion. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 3.1	Terms

For the purposes of the present document, the terms and definitions defined in TR 21.905 [1] and the following apply:

5GC Individual MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions, hence for each such UE one PDU session is required to be associated with a Multicast MBS Session.

5GC Shared MBS traffic delivery: 5G CN receives a single copy of MBS data packets and delivers a single copy of those MBS data packets to a RAN node.

Area Session Identifier: A unique identifier within an MBS Session used for an MBS session with location dependent content. When present, the Area Session ID, together with the TMGI, is used to uniquely identify the data flow of an MBS Session in a specific MBS service area.

Associated PDU Session: A PDU Session associated to a multicast MBS session that is used for 5GC Individual MBS traffic delivery method and for signalling related to a user's participation in a multicast MBS session such as join and leave requests.

Associated QoS Flow: A unicast QoS Flow that belongs to the associated PDU Session and is used for 5GC Individual MBS traffic delivery method. The associated QoS Flow is mapped from a multicast QoS Flow in a multicast MBS session.

Broadcast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to all UEs in a geographical area (i.e. all UEs in the broadcast coverage area are authorized to receive the data).

NOTE 1:	For the broadcast communication service, the content provider and network may not be aware whether the authorized UEs are actually receiving the data being delivered.

Broadcast MBS session: An MBS session to deliver the broadcast communication service. A broadcast MBS session is characterised by the content to send and the geographical area where to distribute it.

Broadcast service area: The area within which data of one or multiple Broadcast MBS session(s) are sent.

MBS QoS Flow: The finest granularity for QoS forwarding treatment for MBS data. Providing different QoS forwarding treatment requires separate MBS QoS Flows in 5GS supporting MBS.

MBS Service Announcement: Mechanism to allow users to be informed about the available MBS services.

MBS session: A multicast MBS session or a broadcast MBS session.

MBS service area: The area within which data of one Multicast or Broadcast MBS session may be sent. For location dependent MBS, for each MBS service area, an Area Session ID, which is unique per MBS Session ID, is allocated and the same location dependent content data for an MBS session is delivered to the UE(s) within an MBS service area.

MB-SMF service area: The area toward which MBS data received by any of the MB-UPF(s) controlled by the MB-SMF can be delivered. For MBS supporting NG-RAN node(s) within MB-SMF service area, MBS data are delivered via 5GC Shared MBS traffic delivery. For MBS non-supporting NG-RAN node(s) within MB-SMF service area, Multicast MBS session data are delivered via 5GC Individual MBS traffic delivery, while Broadcast MBS session data are not delivered.

Multicast communication service: A 5GS communication service in which the same service and the same specific content data are provided simultaneously to a dedicated set of UEs (i.e. not all UEs in the coverage of the MBS service area are authorized to receive the data).

NOTE 2:	For multicast communication service, the content provider and network can be aware whether the authorized UEs are actually receiving the data being delivered.

Multicast MBS session: An MBS session to deliver the multicast communication service. A multicast MBS session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a geographical area where to distribute it.

NR NTN Intended Service Area: The term “NR NTN Intended Service Area” used in the present specification denotes the “MBS Intended Service Area List” as defined in TS 38.413 [15] and TS 38.331 [28], and as also used in TS 38.300 [9].