SA WG2 Meeting #172	S2-2511293

17 - 21 November, 2025, Dallas, USA

# Title:	Reply LS on Multiple Notification targets for NF Event Exposure services

# Response to:	C3-254606, S2-2509861, LS on Multiple Notification targets for NF Event Exposure services

# Release:	Rel-17

# Work Item:	eNA_Ph2

# Source:	SA2

# To:	CT3

# Cc:	CT4

# Contact Person:	Xiaoyan Duan

# duanxiaoyan@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1	Overall description

SA2 thanks CT3 for the LS on multiple notification targets for NF Event Exposure services.

SA2 discussed the question from CT3 and would like to provide the following answer:

Answer: When the DCCF receives a subscription with multiple notification target addresses, the DCCF does not send the multiple notification target addresses in the subscription request towards the data source. Instead, the DCCF includes its own address as the notification target address in the subscription request towards the data source, and sends the notifications to the multiple notification target addresses by itself after receiving the notification from the data source. This has been specified in clause 5A.3.1 and 6.2.6.3.2 of TS 23.288.

2.	Actions

To CT3:

ACTION: SA2 kindly asks CT3 to take the above information into account.

3.	Date of Next SA2 Meetings

SA2#173	09 – 13 February, 2026	Goa, IN

SA2#174	13 – 17 April, 2026			Malta, MT