3GPP TSG-WG SA2#173	S2-2601416

# Goa, India, 9 – 13 February, 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5637 | rev | 3 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PFCP per Node procedure for SMF to trigger mitigation actions towards UPF on non-PDU session level for abnormal use plane traffic handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_Ph2-ARC |  |  |  |  |  | Date: |  |  | 2026-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SMF provisioning of mitigation actions towards UPF on non-PDU Session level are required for abnormal user plane traffic handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposal to define mitigation actions on node level |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not possible to configure actions on UPF level when abnormal traffic is detected |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

#### 4.4.3.2	N4 Association Update Procedure

The N4 Association Update procedure shall be used to modify an existing N4 association between the SMF and the UPF. It may be initiated by the UPF or by the SMF to update the supported features or available resources of the UP function.

N4 Association Update procedure can be used by the SMF to update the provisioning of External Clock Drift Report as specified in clause 4.4.3.1.

N4 Association Update procedure may be used by the SMF to provide or update the provisioning of measurement assistance information of N6 delay for the connection between the UPF and the endpoint in the DN (e.g. EAS) as specified in clause 5.8.2.23 of TS 23.501 [2].

The N4 Association Update procedure may be used by the SMF to provide or update mitigation actions on a per node level basis, which applies to the case when the SMF subscribes/requests the AnalyticsID for any UE and NWDAF sends the notification report(s) that include the Abnormal traffic pattern ID(s) and the (if available) application ID(s) and/or packet filter(s) of the source of the traffic causing the exception(s). Based on analytics output received from NWDAF, SMF decides to trigger mitigation actions such as:

-	Drop: Block traffic matching Application ID/IP Packet Filter Set.

-	Throttle: Apply bandwidth limitation (e.g. to a pre-configured value in Mbps) to the traffic matching Application ID/IP Packet Filter Set.

-	Inhibit Downlink Data Reporting (DLDR): When UE is in IDLE mode and receives (DL) traffic matching Application ID/IP Packet Filter Set, UPF does not trigger N4 DLDR notification towards SMF, thus avoiding waking up UE unnecessarily (due to abnormal traffic).

Figure 4.4.3.2-1: SMF initiated N4 association update procedure

Figure 4.4.3.2-2: UPF initiated N4 association update procedure

**** End of Changes ****