SA WG2 Meeting #172	S2-250XXXX

17 - 21 Nov, 2025, Dallas, USA                                  is revision of 9986

Source:	China Mobile, ZTE, SK Telecom, Rakuten Mobile, Nokia，CATT, vivo, Samsung, AT&T, Deutsche Telekom, Verizon

Title:	Update for KI#2 Interim Agreements and Conclusions for UC1

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update the interim agreements for AIML_CN KI#2.

- Discussion

In TR 23.700-04 V0.3.1 (2025-09), some principles are agreed for KI#2. However, there are still several topics for further consideration depicted as 7.2.2 in TR 23.700-04.

The following topics are for further consideration for KI#2 for UC1:

-	The need and details of type/identification of traffic pattern is FFS.

-	Whether new UPF or SMF events are needed or not for NWDAF data collection.

-	How to reduce the reporting load of input data sources and control plane signalling overhead? For example by configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

-	Consumer actions are FFS.

-  For UC1, the input from AF, possibily via NEF, is FFS.

-	For UC1, it is FFS whether mitigation actions applicable to any UE can be provisioned by the SMF into the UPF on N4 level, a new PFCP per Node procedure is defined.

NOTE : How UPF subscribes to the new analytics is FFS.

This proposal discusses these topics and concludes the aspects for KI #2 for UC1.

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** First change ***

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.2 gets resolved and a principle is agreed.

7.1.2.1	General

No impact on RAN and UE operation.

To reduce the reporting load of input data sources (e.g. UPF), the analytics consumer may include the Analytics Filter Information (as described in clause 6.1.3 of TS 23.288) for input data filtering to UPF.

7.1.2.2	Agreed Principles for Use Case #1

-	The service consumer of the analytics service may be SMF, PCF, OAM, UPF, and AF. The consumer may take the output Analytics into account.

-	To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and predictions of:

-	Type of abnormal traffic, e.g. abnormal traffic due to DDoS, abnormal data packets patterns, unexpected traffic volume or burst.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), UE(s), PDU session(s),

-	Identifiers/ information of the source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	To derive the output analytics, the following input data may be collected from the UPF or SMF, to support the NWDAF-based analytics:

- From UPF/SMF

-	Information to identify the traffic pattern that can contain:

-	Source of traffic (e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.)

-	traffic flow filters

-	identification of corresponding UEs

- 	Information of traffic characteristics.

- The SMF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501)

-	Configuring UPF to block or shape abnormal traffic, to enforce bandwidth limitations

- UPF as consumer of NWDAF analytics may for instance take the following actions:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations)

-	adjust packet processing resources,

- NOTE 1: UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level.The PCF as consumer of NWDAF analytics may for instance take the following actions:

- 	Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.- 	As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

NOTE 2: This can be done either by detection of abnormal traffic at UPF with reporting to SMF that results in related N4 rules or directly detection and enforcement of N4 rules at the UPF.

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in clause 7.1.2) and can either be then marked as not pursued or postponed to a future release.

*** Second change ***

8	Conclusions

Regarding KI #2, the following principles are concluded for UC1.

- The service consumer of the analytics service may be SMF, PCF, and UPF and AF. The consumer may take the output Analytics into account.

NOTE 1: Whether OAM can be the Analytics consumer will be determined in the normative phase, coordination with SA5 is needed.

- To provide analytics to the consumers, the NWDAF may provide the output analytics, including the statistics and/or predictions of:

-  Identification of the traffic/traffic flow (e.g. traffic flow filters).

-  Whether the traffic flow(s) identified by the traffic flow filters or the traffic source is abnormal or not.

-	Type of abnormal traffic, including abnormal data packets patterns (e.g. malformed, unexpected traffic volume, burst, etc.), malicious (e.g. DDoS) or suspicious.

-	volume, rate or burst size of abnormal traffic,

-	Identifiers/addresses of affected UPF(s), interface if applicable, UE(s), PDU session(s),

-	Identifiers/ information of the traffic source, e.g. IP packet filter(s), IP Protocol (TCP, UDP, etc.), application ID, etc.

-	Time window (e.g. start, duration).

-	UP Traffic Pattern information containing pattern type (e.g. small‑packet burst, malformed, unknown, duplicate, fragmented packets, etc) and overload type (e.g. CPU/resource saturated, interface queue full/N6 congestion) when such data are available from analytics.

- To derive the output analytics, the following input data may be collected from the UPF or SMF or AF, to support the NWDAF-based analytics (If individual UEs are targeted, the NWDAF subscribes at the SMF to obtain related information from the UPF)

-	traffic flow information (e.g. Source and/or, destination IP address and port, Protocol Type, URL list)

-	identifiers of corresponding UEs

- 	Information of traffic characteristics from SMF or UPF, including measured UL/DL data volumes, measured UL/DL data rates, QFI

-	Information of UP pattern, e.g. pattern type (including malformed, unknown, duplicate, fragmented, overload, etc.)

NOTE 1: It is to be determined in the normative phase whether the Application ID is used to identify the traffic pattern.

- Traffic characteristics of normal traffic and abnormal traffic from external server/AF, and the abnormal type for the abnormal traffic , which can be used by the NWDAF to determine whether the input traffic is abnormal traffic by comparing the characteristics of input traffic with the characteristics of normal traffic or abnormal traffic.

The event subscription can Target any UE or specific UEs and provides information about targeted UP pattern data and thresholds for the occurrence/volume/burst The UPF only reports when corresponding traffic patterns are detected.

The reporting load of input data sources and control plane signalling overhead may be reduced via configuring appropriate Analytics Filter Information, combining notification to NWDAF at UPF for multiple events, instructions from NWDAF to UPF for pre-processing in the UPF, etc.

NOTE 2:  Related UPF and SMF events will be determined in the normative phase. Whether the "User Data Usage Measures" event or another existing event is extended or new event(s) are defined will be determined in the normative phase

- The SMF as consumer of NWDAF analytics may determine the mitigation actions based on local operator policies for example :

-	UPF reselection to distribute the load across UPF instances (as defined in clause 6.3.3 of TS 23.501[X] and clause 4.3.5 of TS 23.502 [X])

-	Configuring UPF to enforce the downlink traffic suppression (e.g., selective packet dropping or rate limitations) or shape abnormal traffic, to enforce bandwidth limitations

The mitigation actions per observed traffic anomaly and associated traffic identification reported by NWDAF to SMF are provisioned into SMF. Then, the SMF provides them to the UPF, as stated in NOTE 5, these are not actions on PDU session level, as such a new PFCP per Node procedure will be defined. Besides, the OAM can also provide node level mitigation action to UPF.

NOTE 4: Coordination with SA5 is needed regarding the OAM providing the node level mitigation action.

- The UPF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-	Downlink traffic suppression (e.g., selective packet dropping or rate limitations) and optionally notify the action to SMF

-  Adjust packet processing resources,

NOTE 5: UPF will only receive Analytics with reduced Output (i.e. type of abnormal traffic and traffic descriptor). UPF has operator-configured policies corresponding to the types of abnormal traffic, that can be activated for the received traffic descriptor. This does not apply on PDU-session level.

Note 6: Consistently with the Architecture Assumptions and High Level Principles in TR 23.700 04 v1.1.0 to minimize Control Plane NF load, either operator-deployment may be used: (i) UPF as the NWDAF consumer, or (ii) SMF subscribing on behalf of the UPF per TS 23.501 clause 7.1.2. For UC#1, option (i) is preferred to enable per UPF/interface scope and rapid local mitigation while minimizing CP NF load. In all cases, UPF actions are local and non-Session/QoS, and SMF/PCF retain authority.

As stated in the architecture assumptions the UPF can detect abnormal traffic. When the abnormal traffic is detected the UPF can take actions to improve user plane performance using existing PDRs, and associated N4 rules such as QER or FAR, this is independent on the use of analytics.

- The PCF as consumer of NWDAF analytics may for instance take the following actions upon the detection of the abnormal traffic:

-  Policies creation or update and provisioning to SMF, e.g. executing traffic gating or shaping, enforcing bandwidth parameters (e.g. rate limiting to 5Mbps) or adjusting QoS parameters.

- The AF (e.g. IoT server) as consumer of NWDAF analytics and data source could take action to correct abnormal traffic

*** End of the changes ***