SA WG2 Meeting #173	S2-2601386

# Goa, India, 09-13 February 2026                                  was S2-2601119 was S2-2600056

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 1593 | rev | 2 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.503 – TEI20_ DAMP – Deferred AM Policies in the 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Oracle, Verizon, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20_DAMP |  |  |  |  |  | Date: |  |  | 2026-1-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR updates 23.503 along the lines of TEI20 DAMP WID (Deferred AM Policies in the 5GC), as follows: - Add for AM policy the mechanism of policy activation/deactivation in a deferred mode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implementing in 23.503 AM policy activation/deactivation in a deferred mode. Rev1 does not change the content. It is only to allow submitting the CR under the proper AI (20.15). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AM will lack the ability to use the mechanism of policy activation/deactivation in a deferred mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.1.X(New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

#### 6.1.2.1.X	Policy activation/deactivation in a deferred mode

The interaction between AMF and PCF related to access and mobility policy may also be performed in a deferred mode. An access and mobility policy may have either a single deferred activation time, or a single deferred deactivation time(s) or both.

An AM policy with only deferred activation time shall be inactive until that time. An AM policy with only deferred deactivation time shall be active until that time. When the policy activation time occurs prior to the policy deactivation time, the policy is inactive until the activation and remains active until the deactivation time occurs. When the policy deactivation time occurs prior to the policy activation time, the policy is initially active until the deactivation time, then remains inactive until the activation time, and then becomes active again. An inactive AM policy, that has not been activated yet, is still considered to be installed, and may be removed by the PCF. When modifying a dynamic AM policy by setting, modifying or clearing its deferred activation and/or deactivation time or by changing any other attribute of an AM policy having a deferred activation and/or deactivation time, the PCF shall provide all information of that AM policy.

An AM policy (e.g., UE-AMBR) may be currently set to a value X. It may also be associated with a deferred activation or/and activation time with a different value, Y. Once activation time has reached, the AM policy is automatically set to a value of Y. Once deactivation time has reached, the AM policy automatically resumes its previous value X.

A group of AM policies may share the same deferred activation or/and activation time.

AM policies of the following categories may not be associated with a deferred activation or/and activation time:

-	SMF selection management

-	Charging related information

-     Slice replacement management

* * * * Start of 2nd Change * * * *

## 6.5	Access and mobility related policy information

To enable the enforcement in the 5GC system of the access and mobility policy decisions made by the PCF for the control as described in clause 6.1.2.1, the 5GC system may provide the Access and mobility related policy information from the PCF to the AMF.

Table 6.5-1 lists the AMF access and mobility related policy information.

Table 6.5-1: Access and mobility related policy information

| Information name | Description | Category | PCF permitted to modify in a UE context in the AMF | Scope |
| --- | --- | --- | --- | --- |
| Aggregate maximum bit rate | This part defines the aggregate maximum bit rate |  |  |  |
| UE-AMBR | This defines the UE-AMBR value that applies for a UE | Conditional (NOTE 5) | Yes | UE context |
| List of UE-Slice-MBR | This defines the List of UE-Slice-MBR (UL/DL) that each applies to the network slice of the UE. | Conditional (NOTE 8) | Yes | UE context |
| Service Area Restrictions | This part defines the service area restrictions |  |  |  |
| List of allowed TAIs. | List of allowed TAIs (NOTE 3) (NOTE 4). | Conditional (NOTE 1) | Yes | UE context |
| List of non-allowed TAIs. | List of non-allowed TAIs (NOTE 3). | Conditional (NOTE 1) | Yes | UE context |
| Maximum number of allowed TAIs | The maximum number of allowed TAIs. (NOTE 4) | Conditional (NOTE 1) | Yes | UE context |
| RFSP Index | This part defines the RFSP index related information |  |  |  |
| RFSP Index for Allowed NSSAI | Defines the RFSP Index associated with Allowed NSSAI that applies for a UE | Conditional (NOTE 2) | Yes | UE context |
| RFSP Index for Target NSSAI | Defines the RFSP Index associated with Target NSSAI that applies for a UE | Conditional (NOTE 2) | Yes | UE context |
| RFSP Index in Use Validity Time | Defines the time by which the RFSP Index will be used in MME after 5GS to EPS mobility. | Conditional (NOTE 2, NOTE 11) | Yes | UE context |
| 5G access stratum time distribution | This part defines the 5G access stratum time distribution |  |  |  |
| 5G access stratum time distribution indication | Defines if 5G access stratum time distribution via Uu reference point is enabled or disabled | Conditional (NOTE 9) | Yes | UE context |
| Uu interface time synchronization error budget | Indicates the Uu Time Synchronization error budget for 5G access stratum time distribution | Conditional (NOTE 10) | Yes | UE context |
| Clock quality detail level | Defines which clock quality information (clock quality metrics or acceptable/not acceptable indication) to report to the UE as defined in clause 5.27.1.12 of TS 23.501 [2] | Conditional (NOTE 9) | Yes | UE context |
| Clock quality acceptance criteria | Indicates the acceptable criteria as defined in clause 5.27.1.12 of TS 23.501 [2] | Conditional (NOTE 9) | Yes | UE context |
| SMF selection management | This part defines the SMF selection management instructions |  |  |  |
| DNN replacement of unsupported DNNs | Defines if a UE requested unsupported DNN is requested for replacement by PCF | Conditional (NOTE 6) | Yes | UE context |
| List of S-NSSAIs | Defines the list of S-NSSAIs containing DNN candidates for replacement by PCF | Conditional (NOTE 6) (NOTE 7) | Yes | UE context |
| Per S-NSSAI: List of DNNs | Defines UE requested DNN candidates for replacement by PCF | Conditional (NOTE 6) | Yes | UE context |
| Slice replacement management | Defines slice replacement management |  | Yes | UE context |
| S-NSSAI availability information | Defines the S-NSSAI availability and/or alternative S-NSSAI for S-NSSAI | Conditional (NOTE 12) | Yes | UE context |
| Network Slice Replacement Type | Define the type of Network Slice Replacement. | Conditional (NOTE 12) | Yes | UE context |
| Slice Related Restrictions | Defines network policies for Slices subject to network control |  |  |  |
| List of S-NSSAIs | Defines the List of S-NSSAIs that are on demand | Conditional (NOTE 13) | No | UE context |
| Per S-NSSAI: Deregistration Inactivity Timer value | Defines the S-NSSAI deregistration inactivity timer value before removing the S-NSSAI from the Allowed Slices | (NOTE 14) | No | UE context |
| Charging related information | Defines information related to Charging |  |  |  |
| Charging information | Defines the containing CHF address and optionally the associated CHF instance ID and/or CHF set ID | Conditional (NOTE 15) | Yes | UE context |
| CHF group ID | The CHF Group ID for the UE | Conditional (NOTE 15) | Yes | UE context |
| Deferred activation/deactivation | Defines information related to deferred activation/deactivation of one or more AM policies |  |  |  |
| Deferred group of AM policies | Defines a group of AM policy attributes (e.g., UE-AMBR, RFSP index etc) that is to be activated at deferred activation time and deactivated at deferred deactivation time. | Optional (NOTE 16) | Yes | UE context |
| Deferred activation time | Defines the time when a group of AM policies becomes active. (The AM policy shall be inactive until that time). | Optional | Yes | UE context |
| Deferred deactivation time | Defines the time when a group of AM policies becomes inactive. (The AM policy shall be active until that time). | Optional | Yes | UE context |
| NOTE 1: If management of service area restrictions by PCF is enabled. NOTE 2: If management of RFSP index by PCF is enabled. NOTE 3: Either the list of allowed TAIs or the list of non-allowed TAIs are provided by the PCF. NOTE 4: Both the maximum number of allowed TAIs and the list of allowed TAIs may be sent by PCF. NOTE 5: If management of UE-AMBR by PCF is enabled. NOTE 6: If SMF selection management by PCF is enabled. NOTE 7: The List of S-NSSAIs contains S-NSSAIs, valid in the serving network, of the Allowed NSSAI. NOTE 8: If management of UE-Slice-MBR by PCF is enabled. NOTE 9: If management of 5G access stratum time distribution is enabled. NOTE 10: If 5G access stratum time distribution or (g)PTP time synchronization is enabled. NOTE 11: If required based on operator policy when the RFSP index provided by the PCF indicates a change in priority from 5G access to E-UTRAN access. NOTE 12: If slice replacement management by PCF is enabled. NOTE 13: Includes only the list of subscribed slices with network restriction policies for slice use by the UE. The list is empty if there are no S-NSSAIs that are on demand or the timer value are not set by the PCF. NOTE 14: The S-NSSAI deregistration timer is mandatory for every S-NSSAI in the list of S-NSSAIs that are on demand S-NSSAI. NOTE 15: Shall be included If the home operator policies indicates that the same CHF is selected by the PCF for the UE and the AMF, otherwise optional. NOTE 16: The minimum number of AM policies in the group is 1. The max number is unlimited. The AM policies are as defined in clause 6.1.2.1.1. |  |  |  |  |

The list of allowed TAIs indicates the TAIs where the UE is allowed to be registered, see clause 5.3.4 of TS 23.501 [2] for the description on how AMF uses this information.

The list of non-allowed TAIs indicates the TAIs where the UE is not allowed to be registered, see clause 5.3.4 of TS 23.501 [2] for the description on how AMF uses this information.

The Maximum number of allowed TAs indicates the maximum number of allowed Tracking Areas, the list of TAI is defined in the AMF and not explicitly provided by the PCF.

The RFSP Index for Allowed NSSAI and RFSP Index for Target NSSAI defines the RFSP Index for radio resource management functionality.

RFSP Index in Use Validity Time defines the time for which the RFSP Index in use will be used in MME after 5GS to EPS mobility as specified in clause 5.17.2.2 of TS 23.501 [2].

The UE-AMBR limits the aggregated bit rate across all Non-GBR QoS Flows of a UE in the serving network.

The list of UE-Slice-MBR defines the list of authorized UE-Slice-MBR allocated for a UE, how it is enforced is described in clause 5.7.1.10 of TS 23.501 [2].

The DNN replacement of unsupported DNNs indicates that the AMF shall contact the PCF for replacement of an unsupported DNN requested by the UE.

The List of S-NSSAIs defines the S-NSSAIs, valid in the serving network, of the Allowed NSSAI that contain DNN candidates for replacement by PCF.

The List of DNNs defines the DNN candidates for which the AMF shall contact the PCF for replacement if such a DNN is requested by a UE.

The 5G access stratum time distribution indicates the 5G access stratum time distribution parameters to be indicated to the NG-RAN via AMF.

The S-NSSAI availability information indicates whether the S-NSSAI is not available or is available, and/or an alternative S-NSSAI that the S-NSSAI can be replaced with.

The Network Slice Replacement Type indicates the Type of Network Slice Replacement. This is described in detail in clause 5.15.5.2.2a of TS 23.501 [2]

The Charging information includes CHF address(es) and if available, the associated CHF instance ID(s) and/or CHF set ID(s). The CHF group ID contains CHF Group ID of the CHF. This is described in detail in clause 6.3.11 of TS 23.501 [2] and in clause 6.1.1.4.

The Deferred activation time includes the time when an AM policy or a group of AM policies becomes active.

The Deferred deactivation time includes the time when an AM policy or a group of AM policies becomes inactive.

The Deferred group of AM policies includes an AM policy or a group of AM policies that becomes active at the deferred activation time and inactive at the deferred deactivation time.

* * * * End of Changes * * * *