# 5	Test environment for RF test

This section contains all the exceptions of the common test parameters specified in clause 4 for specific needs of test cases defined in TS 36.521-1 [21]. Exceptions specified in clause 5 overwrite the parameter settings of clause 4; exceptions defined within the test cases overwrite parameter settings of clause 4 and 5.

## 5.1	Requirements of test equipment

No common RF test environment requirements are specified in addition to the common requirements described in clause 4.2. Specific RF requirements are indicated within the test cases defined in TS 36.521-1 [21].

## 5.2	RF Reference system configurations

### 5.2.1	Common parameters for simulated E-UTRA cells

For BL/CE testing the same content of system information blocks SIB2, SIB3, SIB5 and SIB13 are used for broadcasting on BCCH and/or BCCH-BR.

#### 5.2.1.1	Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

- E-UTRA FDD single cell scenario

- E-UTRA TDD single cell scenario

- E-UTRA FDD intra-frequency multi cell scenario

- E-UTRA TDD intra-frequency multi cell scenario

Combination 2 applies to the following test case scenarios:

- E-UTRA FDD + MBMS

- E-UTRA TDD + MBMS

Combination 3 applies to the following test case scenarios:

- E-UTRA FDD intra-band carrier aggregation component carriers cell scenario

- E-UTRA FDD inter-band carrier aggregation component carriers cell scenario

- E-UTRA TDD intra-band carrier aggregation component carriers cell scenario

The combinations of system information blocks for test cases in TS 36.521-1 [21] is defined in table 5.2-1.1-1.

Table 5.2.1.1-1: Combinations of system information blocks

|  |  | System information block type |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB2 | SIB3 | SIB4 | SIB5 | SIB6 | SIB7 | SIB8 | SIB9 | SIB10 | SIB11 | SIB12 | SIB13 |
| 1 | X | X |  |  |  |  |  |  |  |  |  |  |
| 2 | X | X |  |  |  |  |  |  |  |  |  | X |
| 3 | X | X |  | X |  |  |  |  |  |  |  |  |

#### 5.2.1.2	Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables. SIB1 will be transmitted during subframes#5 which SFN mod 2 = 0, and SIB2+SIB3 will be transmitted during subframes#5 which SFN mod 2 = 1 with 8 radio frames periodicity. SIB5 will be transmitted during subframes#5 which SFN mod 2 = 1 with 64 radio frames periodicity

Table 5.2.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 8 | SIB2, SIB3 |

Table 5.2.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 8 | SIB2, SIB3 |
| 2 | 64 | SIB13 |

Table 5.2.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Mapping of system information blocks |
| --- | --- | --- |
| 1 | 8 | SIB2, SIB3 |
| 2 | 64 | SIB5 |

#### 5.2.1.3	Common contents of system information messages

- MasterInformationBlock

As defined in Table 4.4.3.2-1 without exceptions.

- SystemInformation

As defined in Table 5.2.1.3-1As defined in Table without exceptions.

Table 5.2.1.3-1: SystemInformation

| Derivation Path: Clause 4.4.3.2 Table 4.4.3.2-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r8 SEQUENCE { |  |  |  |
| sib-TypeAndInfo SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 5.2.1.1 and 5.2.1.2 |  |  |
| criticalExtensionsFuture SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

- SystemInformationBlockType1

As defined in Table 4.4.3.2-3 with the following exceptions:

Table 5.2.1.3-2: SystemInformationBlockType1 exceptions

| Derivation Path: Clause 4.4.3.2 Table 4.4.3.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1 ::= SEQUENCE { |  |  |  |
| si-WindowLength | ms40 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RF_INTF_MITIG |
| hsdn-Cell-r15 | Not present |  |  |
| cellSelectionInfoCE-v1530 | Not present |  |  |
| crs-IntfMitigConfig-r15 CHOICE { | Not present |  |  |
| crs-IntfMitigNumPRBs | n6 |  |  |
| } |  |  |  |
| cellBarred-CRS-r15 | notBarred |  |  |
| plmn-IdentityList-v1530 | Not present |  |  |
| posSchedulingInfoList-r15 | Not present |  |  |
| cellAccessRelatedInfo-5GC-r15 | Not present |  |  |
| ims-EmergencySupport5GC-r15 | Not present |  |  |
| eCallOverIMS-Support5GC-r15 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RF_INTF_MITIG | This condition applies for CRS interference mitigation testing |

- SystemInformationBlockType1-BR-r13

As defined in Table 4.4.3.2-3 with the following exceptions:

Table 5.2.1.3-2A: SystemInformationBlockType1-BR-r13 exceptions

| Derivation Path: Clause 4.4.3.2 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-BR-r13 ::= SEQUENCE { |  |  |  |
| si-WindowLength-BR-r13 | ms40 |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | RF_INTF_MITIG |
| hsdn-Cell-r15 | Not present |  |  |
| cellSelectionInfoCE-v1530 | Not present |  |  |
| crs-IntfMitigConfig-r15 CHOICE { | Not present |  |  |
| crs-IntfMitigNumPRBs | n6 |  |  |
| } |  |  |  |
| cellBarred-CRS-r15 | notBarred |  |  |
| plmn-IdentityList-v1530 | Not present |  |  |
| posSchedulingInfoList-r15 | Not present |  |  |
| cellAccessRelatedInfo-5GC-r15 | Not present |  |  |
| ims-EmergencySupport5GC-r15 | Not present |  |  |
| eCallOverIMS-Support5GC-r15 | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RF_INTF_MITIG | This condition applies for CRS interference mitigation testing |

- SystemInformationBlockType2

As defined in Table 4.4.3.3-1 with the following exceptions:

Table 5.2.1.3-3: SystemInformationBlockType2 exceptions

| Derivation Path: Clause 4.4.3.3 Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| timeAlignmentTimerCommon | infinity |  |  |
| } |  |  |  |

## 5.2A	Generic RF procedures

The UE test state used for testing is specified in the individual test cases in the corresponding test specification TS 36.521-1 [21] or TS 36.521-3 [34].

This clause describes UE test states which can be used in the initial condition of many test cases defined in TS 36.521-1 [21] and TS 36.521-3 [34].

### 5.2A.1	UE RF test states

Table 5.2A.1-1: The E-UTRAN UE states

|  |  | RRC | ECM | EMM | ESM | UE Test Mode |
| --- | --- | --- | --- | --- | --- | --- |
| State 2A-RF | Registered, Idle Mode, UE Test Mode Activated | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 2A-RF-CE | Registered, Idle Mode, Cell supporting BL/CE UE, UE Test Mode Activated | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 3A-RF | Generic Default RB Established, UE Test Mode Activated | RRC_CONNECTED 1 data radio bearer configured | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 3A-RF-CE | Generic Default RB Established, Cell supporting BL/CE UE, UE Test Mode Activated | RRC_CONNECTED 1 data radio bearer configured | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 3A-RF-DC1 | DC MCG/SCG Dedicated RB established, UE Test Mode Activated | RRC_CONNECTED 2 data radio bearers configured | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell 1 dedicated EPS bearer context(s) active on the PSCell | Active |
| State 3A-RF-DC2 | DC Split Default RB established, UE Test Mode Activated | RRC_CONNECTED 1 data radio bearer configured | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell and PSCell with UL transmission of PDCP SDUs on PSCell | Active |
| State 3A-RF-V2X | Generic Default RB Established, UE Test Mode Activated, V2X Setup | RRC_CONNECTED 1 data radio bearer configured | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 3B-RF | Reserved for E-UTRAN UE state for RRM testing as described in Table 7.2A.1-1 |  |  |  |  |  |
| State 4A-RF | Loopback Activation without looped data | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 4A-RF-CE | Loopback Activation without looped data, Cell supporting BL/CE UE | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |
| State 4A-RF-DC1 | DC MCG/SCG DRBs Loopback Activation without looped data | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell 1 dedicated EPS bearer context(s) active on the PSCell | Active |
| State 4A-RF-DC2 | DC Split DRB Loopback Activation without looped data | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell and PSCell | Active |
| State 4A-RF-V2X | Loopback Activation without looped data, V2X Setup | RRC_CONNECTED | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |

### 5.2A.1A	Registered, Idle Mode, UE Test Mode Activated (State 2A-RF)

#### 5.2A.1A.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

#### 5.2A.1A.2	Definition of system information messages

The default system information messages are used.

#### 5.2A.1A.3	Procedure

Table 5.2A.1A.3-1: UE registration with test mode activation procedure (state 1 to state 2A-RF)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 to 9a2 | Steps 1 to 9a2 as specified in the procedure in clause 4.5.2.3 take place | - | - |
| 10 | The SS transmits an ACTIVATE TEST MODE message to activate UE radio bearer test mode procedure. | <-- | RRC: DLInformationTransfer TC: ACTIVATE TEST MODE |
| 11 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE TEST MODE COMPLETE |
| 12 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 13 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 14 | The SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry |
| 15 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 16 | The SS transmits an RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to 4.8.2.2.1.1. This message includes the ATTACH ACCEPT message. The ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in ATTACH ACCEPT. | <-- | RRC: RRCConnectionReconfiguration NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 17 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of default bearer. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 18 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane if requested by the UE. |  |  |
| 18 | This message includes the ATTACH COMPLETE message. The ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in ATTACH COMPLETE. | --> | RRC: ULInformationTransfer NAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| - | EXCEPTION: Steps18a1 to 18b6 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that takes place if a particular configuration has been chosen | - | - |
| 18a1 | IF IMS voice not supported and pc_voice_PS_1_CS_2, pc_attach and pc_TAU_connected_in_IMS are set to TRUE (Note 1) THEN the UE transmits a TRACKING AREA UPDATE REQUEST message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST |
| 18a2 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 18a3 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 18b1 | ELSE IF IMS voice not supported and pc_voice_PS_1_CS_2, pc_attach and pc_TAU_idle_in_IMS are set to TRUE (Note 2) THEN the SS transmits an RRCConnectionRelease message to release the RRC connection. | <-- | RRC: RRCConnectionRelease |
| 18b2 | The UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 18b3 | The SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 18b4 | The UE transmits an RRCConnectionSetupComplete message including a TRACKING AREA UPDATE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST |
| 18b5 | The SS transmits a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 18b6 | The UE transmits a TRACKING AREA UPDATE COMPLETE message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 19 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE (State 2A-RF). | <-- | RRC: RRCConnectionRelease |
| NOTE 1: The procedure is used with specific message with no IMS voice network support. The UE is configured for voice domain preference IMS PS voice preferred, CS Voice as secondary and to initiate EPS attach. The UE implementation supports TAU in connected mode. NOTE 2: The procedure is used with specific message with no IMS voice network support. The UE is configured for voice domain preference IMS PS voice preferred, CS Voice as secondary and to initiate EPS attach. The UE implementation supports TAU in idle mode. |  |  |  |

#### 5.2A.1A.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A with the exceptions below.

Table 5.2A.1A.4-1: RRCConnectionRequest (Step 2 and 19b2)

| Derivation Path: Table 4.6.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r8 SEQUENCE { |  |  |  |
| ue-Identity | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.1A.4-2: UECapabilityInformation (Step 15)

| Derivation Path: Table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.1A.4-3: ATTACH ACCEPT (Step 16)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | '0000 0000'B | IMS voice over PS session in S1 mode not supported | NON pc_Disable_E-UTRA_NOIMSVoIP |
|  | '0000 0001'B | IMS voice over PS session in S1 mode supported | pc_Disable_E-UTRA_NOIMSVoIP |
| Additional update result | Not present |  |  |

Table 5.2A.1A.4-4: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (Step 16)

| Derivation Path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS QoS | See Reference default EPS bearer context #1 in table 6.6.1-1 |  | NON pc_Disable_E-UTRA_NOIMSVoIP |
|  | See Reference default EPS bearer context #2 in table 6.6.1-1 |  | pc_Disable_E-UTRA_NOIMSVoIP |
| Access point name | The SS defines a Default APN or, if the UE transmits an ESM INFORMATION RESPONSE message providing an APN, the SS shall use this value if it is not the IMS APN. If it is the default IMS APN, the SS defines a Default APN. |  | NON pc_Disable_E-UTRA_NOIMSVoIP |
|  | IMS.apn.epc.mnc<MNC>.mcc<MCC>.3gppnetwork.org The <MNC> and <MCC> are set to the same values as in IMSI. |  | pc_Disable_E-UTRA_NOIMSVoIP |
| PDN address |  |  | IPv4v6 |
| Length of PDN address contents | 5 octets |  |  |
| PDN type value | ‘001’B | IPv4 |  |
| PDN address information | IPv4 address | The SS provides a valid IPv4 address |  |
| ESM cause | IF "PDN type" IE in step 4 is 'IPv4v6' THEN '00110010'B ELSE Not present | "PDN type IPv4 only allowed" |  |

Table 5.2A.1A.4-5: TRACKING AREA UPDATE REQUEST (Steps 19a1 and 19b4)

Derivation Path: 36.508 Table 4.7.2-27 with condition combined_TA_LA

Table 5.2A.1A.4-6: TRACKING AREA UPDATE ACCEPT (Steps 19a2 and 19b5)

| Derivation Path: 36.508 Table 4.7.2-24 with condition combined_TA_LA |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | ‘0000 0000'B | IMS voice over PS session in S1 mode not supported |  |
| Additional update result | Not present |  |  |

### 5.2A.1AA	Registered, Idle Mode, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-RF-CE)

The same assumptions and definitions apply as in clause 5.2A.1A.

#### 5.2A.1AA.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

#### 5.2A.1AA.2	Definition of system information messages

The default system information messages are used with the exceptions below.

Table 5.2A.1AA.2-1: MasterInformationBlock

Derivation Path: 36.508 Table 4.4.3.2-1 using condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

Table 5.2A.1AA.2-2: SystemInformation-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-2A.

Table 5.2A.1AA.2-3: SystemInformationBlockType1-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-3A.

#### 5.2A.1AA.3	Procedure

For RF testing of BL-UE use the same procedure as in clause 5.2A.1A.3 with the exception below:

-	BR-BCCH replaces BCCH.

For RF testing of non-BL UE supporting CE use the procedure in Table 5.2A.1AA.3-1.

Table 5.2A.1AA.3-1: UE registration procedure (state 1 to state 2A-RF-CE)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | In addition to the system information broadcasted on BCCH the SS broadcasts SystemInformation-BR-r13, SystemInformationBlockType1-BR-r13 and other relevant system information blocks as required by the specified combinations of system information blocks specified for the test case on BCCH-BR. | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) |
| 2 to 9a2 | Same procedure for steps 2 to 9a2 as specified in the procedure in clause 4.5.2.3 | - | - |
| 10 to 19 | Same procedure for steps 10 to 19 as specified in the procedure in clause 5.2A.1A.3 | - | - |

#### 5.2A.1AA.4	Specific message contents

Same specific message content as in clause 5.2A.1A.4 with the exception below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

### 5.2A.2	Generic Default Radio Bearer Establishment, UE Test Mode Activated (State 3A-RF)

In tests referring to test mode State 3A-RF, in order to avoid any uncontrollable transmission of uplink U-plane data, test mode State 4A-RF (specified in 5.2A.3) may be used and configured instead.

#### 5.2A.2.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The Test USIM shall be inserted.

#### 5.2A.2.2	Definition of system information messages

The default system information messages are used.

#### 5.2A.2.3	Procedure

Table 5.2A.2.3-1: UE registration with default EPS bearer establishment and test mode activation procedures

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BCCH) |
| 2 to 18 | Steps 2 to 18 as specified in the procedure in clause 5.2A.1A.3 take place. | - | - |

For Rel-11 or higher UEs, that support frequencyBandRetrieval_r11, where the UE has not sent all supported band combinations in UECapabilityInformation (Step 15 of table 5.2A.2.3-1) the following procedure shall be executed after the completion of the procedure in table 5.2A.2.3-1:

Table 5.2A.2.3-2: Retrieval of additional UE capabilities for Rel-11 and higher UEs that support frequencyBandRetrieval_r11

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 to 3 | Steps 1 to 3 as specified in the procedure in clause 5.2A.7.3 take place. | - | - |

#### 5.2A.2.4	Specific message contents

All specific message contents shall be referred to clause 5.2A.1A.4 with the exception below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

### 5.2A.2A	DC MCG/SCG Dedicated RB established, UE Test Mode Activate (State 3A-RF-DC1)

#### 5.2A.2A.1	Initial conditions

System Simulator:

-	2 cells, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in Generic Default RB Established, UE Test Mode Activated (State 3A-RF).

#### 5.2A.2A.2	Definition of system information messages

The default system information messages are used.

#### 5.2A.2A.3	Procedure

The establishment of MCG/SCG dedicated radio bearer connection is assumed to always be mobile terminated.

Table 5.2A.2A.3-1: Procedure for MCG/SCG dedicated bearer establishment

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS adds the PSCell and configures a new SCG data radio bearer and the associated dedicated EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new SCG data radio bearer, associated with the dedicated EPS bearer context in the NAS message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

#### 5.2A.2A.4	Specific message contents

All specific message contents shall be referred to clause 5.2A.1A.4, with exceptions as below.

Table 5.2A.2A.4-1: RRCConnectionReconfiguration (Step 1, Table 5.2A.2A.3-1)

Derivation Path: 36.508, Table 4.6.1-8 using condition PSCell_Add_SCG_DRB

### 5.2A.2AA	Generic Default Radio Bearer Establishment, UE Test Mode Activated in cell supporting BL/CE UE (State 3A-RF-CE)

In tests referring to test mode State 3A-RF-CE, in order to avoid any uncontrollable transmission of uplink U-plane data, test mode State 4A-RF-CE (specified in 5.2A.3AA) may be used and configured instead.

#### 5.2A.2AA.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The Test USIM shall be inserted.

#### 5.2A.2AA.2	Definition of system information messages

The default system information messages are used with the exceptions below.

Table 5.2A.2AA.2-1: MasterInformationBlock

Derivation Path: 36.508 Table 4.4.3.2-1 using condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

Table 5.2A.2AA.2-2: SystemInformation-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-2A.

Table 5.2A.2AA.2-3: SystemInformationBlockType1-BR-r13

| Derivation Path: 36.508 Table 4.4.3.2-3A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| schedulingInfoList-BR-r13 SEQUENCE (SIZE (1..maxSI-Message)) OF SEQUENCE { | n entries | n is the same number of entries, and listed in the same order, as in SchedulingInfoList (without suffix) |  |
| si-Narrowband-r13[1] | 1 | Narrowband index 0 | 1.4 MHz BW |
| si-Narrowband-r13[1] | 2 | Narrowband index 1 | 3 MHz BW |
| si-Narrowband-r13[1] | 4 | Narrowband index 3 | 5 MHz BW or 15 MHz BW or 20 MHz BW |
| si-Narrowband-r13[1] | 6 | Narrowband index 5 | 10 MHz BW |
| si-TBS-r13[1] | b712 |  |  |
| …. |  |  |  |
| si-Narrowband-r13[n] | 1 | Narrowband index 0 | 1.4 MHz BW |
| si-Narrowband-r13[n] | 2 | Narrowband index 1 | 3 MHz BW |
| si-Narrowband-r13[n] | 4 | Narrowband index 3 | 5 MHz BW or 15 MHz BW or 20 MHz BW |
| si-Narrowband-r13[n] | 6 | Narrowband index 5 | 10 MHz BW |
| si-TBS-r13[n] | b712 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| 1.4 MHz BW | Used for cells with 1.4 MHz bandwidth |
| 3 MHz BW | Used for cells with 3 MHz bandwidth |
| 5 MHz BW | Used for cells with 5 MHz bandwidth |
| 10 MHz BW | Used for cells with 10 MHz bandwidth |
| 15 MHz BW | Used for cells with 15 MHz bandwidth |
| 20 MHz BW | Used for cells with 20 MHz bandwidth |

#### 5.2A.2AA.3	Procedure

For RF testing of BL-UE use the procedure in Table 5.2A.2AA.3-1. For RF testing of non-BL UE supporting CE use the procedure in Table 5.2A.2AA.3-2.

Table 5.2A.2AA.3-1: UE registration with default EPS bearer establishment and test mode activation procedures

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 |  | <-- | RRC: SYSTEM INFORMATION (BR-BCCH) |
| 2 to 18 | Steps 2 to 18 as specified in the procedure in clause 5.2A.1A.3 take place. | - | - |

Table 5.2A.2AA.3-2: UE registration procedure (state 1 to state 2A-RF-CE)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | In addition to the system information broadcasted on BCCH the SS broadcasts SystemInformation-BR-r13, SystemInformationBlockType1-BR-r13 and other relevant system information blocks as required by the specified combinations of system information blocks specified for the test case on BCCH-BR. | <-- | RRC: SYSTEM INFORMATION (BCCH and BCCH-BR) |
| 2 to 18 | Steps 2 to 18 as specified in the procedure in clause 5.2A.1A.3 take place. | - | - |

#### 5.2A.2AA.4	Specific message contents

All specific message contents shall be referred to clause 5.2A.1A.4 with the exceptions below:

-	In addition to any other conditions use the condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

Table 5.2A.2AA.4-1: MPDCCH-Config-r13

| Derivation Path: 36.508 Table 4.6.3-2B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| mpdcch-config-r13 CHOICE[1] { |  |  |  |
| setup SEQUENCE { |  |  |  |
| mpdcch-config-r13 CHOICE[1] { |  |  |  |
| setup SEQUENCE { |  |  |  |
| csi-NumRepetitionCE-r13 | sf1 |  |  |
| mpdcch-pdsch-HoppingConfig-r13 | off |  |  |
| mpdcch-StartSF-UESS-r13 CHOICE { |  |  |  |
| fdd-r13 | v1 |  | FDD |
| tdd-r13 | v1 |  | TDD |
| } |  |  |  |
| mpdcch-NumRepetition-r13 | r1 | No repetitions to reduce impact on legacy E-UTRA test cases for category M1 UEs. |  |
| mpdcch-Narrowband-r13 | 1 | Narrowband index 0 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| TDD | TDD cell environment |

### 5.2A.2B	DC Split Default RB established, UE Test Mode Activate (State 3A-RF-DC2)

#### 5.2A.2B.1	Initial conditions

Same as in sub-clause 5.2A.2A.1.

#### 5.2A.2B.2	Definition of system information messages

The default system information messages are used.

#### 5.2A.2B.3	Procedure

The establishment of split dedicated radio bearer connection is assumed to always be mobile terminated.

Table 5.2A.2B.3-1: Procedure for Dual Connectivity Split default bearer establishment

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 4 | The SS adds a PSCell and configures new split data radio bearer | <-- | RRC: RRCConnectionReconfiguration |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the split data radio bearer with UL transmission of PDCP SDUs on the PSCell | --> | RRC: RRCConnectionReconfigurationComplete |

#### 5.2A.2B.4	Specific message contents

All specific message contents shall be referred to clause 5.2A.1A.4, with exceptions as below.

Table 5.2A.2B.4-1: RRCConnectionReconfiguration (Step 4, Table 5.2A.2A.3-1)

Derivation Path: 36.508, Table 4.6.1-8 using condition PSCell_Add_Split_DRB

### 5.2A.2C	Generic Default Radio Bearer Establishment, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X)

#### 5.2A.2C.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	1 V2V transmitter for receiver characteristics or 1 V2V receiver for transmitter characteristics

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The Test USIM shall be inserted.

#### 5.2A.2C.2	Definition of system information messages

The default system information messages are used with the exceptions below:

Table 5.2A.1B.2-1: SystemInformationBlockType21

Derivation Path: 36.508 Table 4.4.3.3-19.

#### 5.2A.2C.3	Procedure

For RF testing of V2X Communication, use the same procedure as in clause 5.2A.2.3.

#### 5.2A.2C.4	Specific message contents

All specific message contents shall be referred to clause 5.2A.1A.4, with exceptions as below:

Table 5.2A.2A.4-1: RRCConnectionReconfiguration (Step 16, Table 5.2A.1A.3-1)

Derivation Path: 36.508, Table 4.6.1-8B using conditions SETUP and SCHEDULED

### 5.2A.3	Loopback Activation without looped data (State 4A-RF)

Editor’s note: Test mode State 4A-RF is used and configured as an alternative to test mode State 3A-RF, to avoid any uncontrollable transmission of uplink U-plane data.

#### 5.2A.3.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in Generic RB Establishment state, UE Test Mode Activated (State 3A-RF).

#### 5.2A.3.2	Definition of system information messages

The default system information messages are used.

#### 5.2A.3.3	Procedure

Table 5.2A.3.3-1: UE registration with default EPS bearer establishment and test mode activation procedures

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE |

#### 5.2A.3.4	Specific message contents

All specific message contents shall be referred to clause 4.5.4.4, with the exceptions below.

Table 5.2A.3.4-1: CLOSE UE TEST LOOP (in the preamble)
(Step 1 in Table 5.2A.3.3-1)

| Derivation Path: 36.509 clause 6.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | 1 1 1 1 |  |  |
| Skip indicator | 0 0 0 0 |  |  |
| Message type | 1 0 0 0 0 0 0 0 |  |  |
| UE test loop mode | 0 0 0 0 0 0 0 0 | UE test loop mode A |  |
| UE test loop mode A LB setup |  |  |  |
| Length of UE test loop mode A LB setup list in bytes | 0 0 0 0 0 0 1 1 | Length of one LB setup DRB (3 bytes) |  |
| LB setup DRB | 0 0 0 0 0 0 0 0, 0 0 0 0 0 0 0 0, 0 0 0 Q4 Q3 Q2 Q1 Q0 | UL PDCP SDU size = 0 bits (0 bytes) Q4..Q0 = Data Radio Bearer identity number for the default radio bearer. See 36.509 clause 6.1 |  |
| UE test loop mode B LB setup | Not present |  |  |
| UE test loop mode B LB setup | Not present |  |  |

### 5.2A.3A	DC MCG/SCG DRBs Loopback Activation without looped data (State 4A-RF-DC1)

Editor’s note: Test mode State 4B-RF is used and configured as an alternative to test mode State 3B-RF, to avoid any uncontrollable transmission of uplink U-plane data.

#### 5.2A.3A.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in DC MCG/SCG Dedicated RB established, UE Test Mode Activated (State 3A-RF-DC1).

#### 5.2A.3A.2	Definition of system information messages

Same as in sub-clause 5.2A.3.2.

#### 5.2A.3A.3	Procedure

Same as in sub-clause 5.2A.3.3.

#### 5.2A.3A.4	Specific message contents

Same as in sub-clause 5.2A.3.4.

### 5.2A.3AA	Loopback Activation without looped data in cell supporting BL/CE UE (State 4A-RF-CE)

Editor’s note: Test mode State 4A-RF-CE is used and configured as an alternative to test mode State 3A-RF-CE, to avoid any uncontrollable transmission of uplink U-plane data.

#### 5.2A.3AA.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in Generic RB Establishment state, UE Test Mode Activated (State 3A-RF).

#### 5.2A.3AA.2	Definition of system information messages

The default system information messages are used with the exceptions below.

Table 5.2A.3AA.2-1: MasterInformationBlock

Derivation Path: 36.508 Table 4.4.3.2-1 using condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

Table 5.2A.3AA.2-2: SystemInformation-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-2A.

Table 5.2A.2AA.2-3: SystemInformationBlockType1-BR-r13

Derivation Path: 36.508 Table 4.4.3.2-3A.

#### 5.2A.3AA.3	Procedure

Table 5.2A.3.3-1: UE registration with default EPS bearer establishment and test mode activation procedures

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE |

#### 5.2A.3AA.4	Specific message contents

Same as in clause 5.2A.3.4.

### 5.2A.3B	DC Split DRB Loopback Activation without looped data (State 4A-RF-DC2)

Editor’s note: Test mode State 4C-RF is used and configured as an alternative to test mode State 3C-RF, to avoid any uncontrollable transmission of uplink U-plane data.

#### 5.2A.3B.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in DC Split Default RB established, UE Test Mode Activated (State 3A-RF-DC2).

#### 5.2A.3B.2	Definition of system information messages

Same as in sub-clause 5.2A.3.2.

#### 5.2A.3B.3	Procedure

Same as in sub-clause 5.2A.3.3.

#### 5.2A.3B.4	Specific message contents

Same as in sub-clause 5.2A.3.4.

### 5.2A.3C	Loopback Activation without looped data, V2X Setup (State 4A-RF-V2X)

Editor’s note: Test mode State 4A-RF-V2X is used and configured as an alternative to test mode State 3A-RF-V2X, to avoid any uncontrollable transmission of uplink U-plane data.

#### 5.2A.3C.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	1 GNSS simulator

-	1 V2V transmitter for receiver characteristics or 1 V2V receiver for transmitter characteristics

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

-	The UE shall be in Generic RB Establishment state, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X).

#### 5.2A.3C.2	Definition of system information messages

Same as in sub-clause 5.2A.3.2.

#### 5.2A.3C.3	Procedure

Same as in sub-clause 5.2A.3.3.

#### 5.2A.3C.4	Specific message contents

All specific message contents shall be referred to clause 5.2A.3.4, with exceptions as below:

Table 5.2A.3C.4-1: ACTIVATE TEST MODE: V2X Communication

Derivation Path: 36.508, Table 4.7A-1 using condition UE TEST LOOP MODE E

Table 5.2A.3C.4-2: CLOSE UE TEST LOOP: V2X Communication

Derivation Path: 36.508, Table 4.7A-3 using condition UE TEST LOOP MODE E

### 5.2A.4	Procedure to configure SCC

Table 5.2A.4.-1: UE RRC reconfiguration with sCELLToAdd

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits an RRCConnectionReconfiguration(sCellToAddModList) message to establish the SCC(s). | <-- | RRC: RRCConnectionReconfiguration |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | If not all SCCs are established, then repeat step 1 and 2. |  |  |

#### 5.2 A.41.	Specific message contents

All specific message contents shall be referred to clause 4.5.2A.4, with exceptions as below.

##### 5.2A.4.1.1	Exceptions for all CA tests

RRCConnectionReconfiguration

Table 5.2A.4.1.1-1: RRCConnectionReconfiguration

Derivation Path: Clause 4.6.1 Table 4.6.1-8, condition SCell_AddMod

MAC configurations

Table 5.2A.4.1.1-2: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5 Table 5.5.1.1-1, condition Scell_AddMod |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| mac-MainConfig-v1020SEQUENCE { |  |  | SCell_AddMod |
| sCellDeactivationTimer-r10 | Not present |  |  |
| extendedBSR-Sizes-r10 | Not Present |  |  |
| extendedPHR-r10 | Not Present |  |  |
|  | Setup |  | UL CA |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |
| UL CA | This condition is used for UL CA. |

##### 5.2A.4.1.2	Exceptions for UL CA tests

RadioResourceConfigCommonSCell-r10-DEFAULT

Table 5.2A.4.1.2-1: RadioResourceConfigCommonSCell-r10-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | UL CA |
| ul-FreqInfo-r10 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r10 | Not Present | For FDD: If absent, the (default) value determined from the default TX-RX frequency separation defined in 3GPP TS 36.101 [27], table 5.7.3-1 applies. For TDD: This parameter is absent and it is equal to the downlink frequency. |  |
| ul-Bandwidth-r10 | Not Present | Same downlink bandwidth as used for target SCell |  |
| additionalSpectrumEmissionSCell-r10 | 31 (CA_NS_31) |  | Intra-B UL CA |
| additionalSpectrumEmissionSCell-r10 | 1 (NS_01) |  | Inter-B UL CA |
| } |  |  |  |
| soundingRS-UL-ConfigCommon-r10 | release |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL CA | When UL Carrier Aggregation is used. |
| Intra-B UL CA | Intra-band Carrier Aggregation |
| Inter-B UL CA | Inter-band Carrier Aggregation |

PhysicalConfigDedicatedSCell-r10-DEFAULT

Table 5.2A.4.1.2-2: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Present |  |  |
| ul-Configuration-r10 SEQUENCE { |  |  | UL CA |
| antennaInfoUL-r10 | Not Present |  |  |
| pusch-ConfigDedicatedSCell-r10 | Not present |  |  |
| uplinkPowerControlDedicatedSCell-r10 | UplinkPowerControlDedicatedSCell-r10-DEFAULT |  |  |
| cqi-ReportConfigSCell-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-r10 | Not present |  |  |
| soundingRS-UL-ConfigDedicated-v1020 | Not present |  |  |
| soundingRS-UL-ConfigDedicatedAperiodic-r10 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| UL CA | When UL Carrier Aggregation is used. |

### 5.2A.5	Exceptions for feICIC tests

#### 5.2A.5.1	Specific message contents

All specific message contents shall be referred to clause 4.6, with exceptions as below.

##### 5.2A.5.1.1	Neighbour cell info for all feICIC test cases

Table 5.2A.5.1.1-1: RRCConnectionReconfiguration

| Derivation Path: clause 4.6.1, Table 4.6.1-8: RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  | SRB2-DRB(n, m) |
| securityConfigHO | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.5.1.1-2: RadioResourceConfigDedicated-SRB2-DRB(n, m)

| Derivation Path: clause 4.6.3, Table 4.6.3-16 RadioResourceConfigDedicated-SRB2-DRB(n,m) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB(n, m) ::= SEQUENCE { |  |  |  |
| neighCellsCRS-Info-r11 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| CRS-AssistancedInfoList-r11 ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r11 |  |  |  |
| CRS-AssistanceInfo-r11 :: = SEQUENCE { |  | CRS information for all the configured neighbour cells |  |
| physCellId-r11 | Based on simulated cell requirements |  |  |
| antennaPortsCount-r11 | Based on simulated cell requirements |  |  |
| mbsfn-SubframeConfigList-r11 | Based on simulated cell requirements |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 5.2A.6	Exceptions for NAICS tests

#### 5.2A.6.1	NAICS specific RRC Connection reconfiguration procedure

##### 5.2A.6.1.1	Procedure

Table 5.2A.6.1.1-1: Procedure for RRC connection reconfiguration

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits and RRCConnectionReconfiguration message. | <-- | RRC: RRCConnectionReconfiguration |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRC: RRCConnectionReconfigurationComplete |

##### 5.2A.6.1.1	Specific message contents

Same as in 5.2A.6.2 with the condition “NAICS_ONLY”.

#### 5.2A.6.2	Specific message contents

All specific message contents shall be referred to clause 4.6, with exceptions as below.

##### 5.2A.6.2.1	RRCConnectionReconfiguration for setting up and releasing NAICS configuration in NAICS test cases

Table 5.2A.6.2.1-1: RRCConnectionReconfiguration

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| dedicatedInfoNASList | Not present |  | NAICS_ONLY, NAICS_RELEASE |
|  | Set according to specific message content |  | SRB2-DRB_NAICS(n, m) |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SRB2-DRB_NAICS_ONLY |  | NAICS_ONLY |
|  | RadioResourceConfigDedicated-SRB2-DRB_NAICS(n, m) |  | SRB2-DRB_NAICS(n, m) |
|  | RadioResourceConfigDedicated_NAICS_RELEASE |  | NAICS_RELEASE |
| securityConfigHO | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB2-DRB_NAICS(n,m) | Establishment of a SRB and DRB combination with n x AM DRB and m x UM DRB (including establishment of SRB2) plus NAICS information |
| NAICS_ONLY | Stand alone NAICS set-up |
| NAICS_RELEASE | Standalone release of all NAICS configurations |

Table 5.2A.6.2.1-2: RadioResourceConfigDedicated-SRB2-DRB_NAICS(n, m)

| Derivation Path: clause 4.6.3, Table 4.6.3-16 RadioResourceConfigDedicated-SRB2-DRB(n,m) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB_NAICS(n, m) ::= SEQUENCE { |  |  |  |
| neighCellsCRS-Info-r11 | NeighCellsCRS-Info_NAICS |  |  |
| } |  |  |  |

Table 5.2A.6.2.1-3: RadioResourceConfigDedicated-SRB2-DRB_NAICS_ONLY

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB_NAICS_ONLY ::= SEQUENCE { |  |  |  |
| neighCellsCRS-Info-r11 | NeighCellsCRS-Info_NAICS |  |  |
| } |  |  |  |

Table 5.2A.6.2.1-4: NeighCellsCRS-Info_NAICS

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| neighCellsCRS-Info-r11 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| naics-Info-r12 ::= CHOICE { |  |  |  |
| setup :: = SEQUENCE { |  |  |  |
| neighCellsToReleaseList-r12 | Not present |  |  |
| neighCellsToAddModList-r12 SEQUENCE (SIZE (1..maxNeighCell-r12)) OF NeighCellsInfo-r12 |  |  |  |
| NeighCellsInfo-r12 ::= SEQUENCE { |  |  |  |
| physCellId-r12 | Based on simulated cell requirements |  |  |
| p-b-r12 | Based on simulated cell requirements |  |  |
| crs-PortsCount-r12 | Based on simulated cell requirements |  |  |
| mbsfn-SubframeConfig-r12 | Not present |  |  |
| p-aList-r12 SEQUENCE (SIZE (1..maxP-a-PerNeighCell-r12)) OF P-a |  |  |  |
| P-a | dB-6 |  |  |
| P-a | dB-3 |  |  |
| P-a | dB0 |  |  |
| transmissionModeList-r12 | ‘01110110’B | Transmission modes 2,3,4,8 and 9 |  |
| resAllocGranularity-r12 | 1 |  |  |
| } |  |  |  |
| servCellp-a-r12 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.6.2.1-5: RadioResourceConfigDedicated_NAICS_RELEASE

| Derivation Path: 36.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB_NAICS_ONLY ::= SEQUENCE { |  |  |  |
| neighCellsCRS-Info-r11 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| naics-Info-r12 ::= CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 5.2A.7	Procedure to retrieve additional UE Capabilities for Rel-11 and higher UEs that support frequencyBandRetrieval_r11

#### 5.2A.7.1	Initial conditions

System Simulator:

-	1 cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 5

User Equipment:

- 	The Test USIM shall be inserted.

#### 5.2A.7.2	Definition of system information messages

The default system information messages are used.

#### 5.2A.7.3	Procedure

Table 5.2A.7.3-1: Procedure to retrieve additional UE capabilities

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | If the UE sent freqBandretrieval-r11 in previous UECapabilityInformation, the SS transmits a UECapabilityEnquiry message to initiate the UE radio access capability transfer procedure | <-- | RRC: UECapabilityEnquiry |
| 2 | The UE transmits a UECapabilityInformation message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation |
| 3 | If the UE did not report all supported E-UTRA bands relevant to the tested bands, repeat steps 1 and 2. |  |  |

#### 5.2A.7.4	Specific message contents

Table 5.2A.7.4-1: UECapabilityEnquiry (Step 1)

| Derivation Path: Clause 6.2.2 Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |
| ue-CapabilityRequest | eutra |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| requestedFrequencyBands-r11 SEQUENCE (SIZE (1..16)) OF SEQUENCE{ |  |  |  |
| FreqBandIndicator-r11 |  |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 5.2A.8	Exceptions for DL Control channel interference mitigation (Type A and Type B) tests

#### 5.2A.8.1	Specific message contents

All specific message contents shall be referred to clause 4.6, with exceptions as below.

##### 5.2A.8.1.1	Neighbour cell info for all CCIM test cases

Table 5.2A.5.1.1-1: RRCConnectionReconfiguration

| Derivation Path: clause 4.6.1, Table 4.6.1-8: RRCConnectionReconfiguration |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r8 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated | RadioResourceConfigDedicated-SRB2-DRB(n, m) |  | SRB2-DRB(n, m) |
| securityConfigHO | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2A.5.1.1-2: RadioResourceConfigDedicated-SRB2-DRB(n, m)

| Derivation Path: clause 4.6.3, Table 4.6.3-16 RadioResourceConfigDedicated-SRB2-DRB(n,m) |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-SRB2-DRB(n, m) ::= SEQUENCE { |  |  |  |
| neighCellsCRS-Info-r13 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| CRS-AssistancedInfoList-r13 ::= SEQUENCE (SIZE (1..maxCellReport)) OF CRS-AssistanceInfo-r13 |  |  |  |
| CRS-AssistanceInfo-r13 :: = SEQUENCE { |  | CRS information for all the configured neighbour cells |  |
| physCellId-r13 | Based on simulated cell requirements |  |  |
| antennaPortsCount-r13 | Based on simulated cell requirements |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 5.3	Default RRC message and information elements contents

### 5.3.1	Radio resource control information elements

As defined in clause 4.6.3 with the following exceptions:

Table 5.3.1-1: TDD-Config-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TDD-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| subframeAssignment | sa1 |  |  |
| specialSubframePatterns | ssp4 |  | RF |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RF | For all the RF tests specified in 36.521-1 |

Table 5.3.1-2: RadioResourceConfigCommonSIB-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon | RACH-ConfigCommon-DEFAULT |  |  |
| bcch-Config | BCCH-Config-DEFAULT |  |  |
| pcch-Config | PCCH-Config-DEFAULT |  |  |
| prach-Config | PRACH-ConfigSIB-DEFAULT |  |  |
| pdsch-ConfigCommon | PDSCH-ConfigCommon-DEFAULT |  |  |
| pusch-ConfigCommon | PUSCH-ConfigCommon-DEFAULT |  |  |
| pucch-ConfigCommon | PUCCH-ConfigCommon-DEFAULT |  |  |
| soundingRS-UL-ConfigCommon CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| uplinkPowerControlCommon | UplinkPowerControlCommon-DEFAULT |  |  |
| ul-CyclicPrefixLength | len1 |  |  |
| } |  |  |  |

Table 5.3.1-3: PRACH-Config-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| prach-ConfigInfo SEQUENCE { |  |  |  |
| prach-ConfigIndex | 3 |  | TDD |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDD | TDD cell environment |

Table 5.3.1-4: RadioResourceConfigCommonSCell-r10-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not Present |  | DL CA only |
| } |  |  |  |

Table 5.3.1-5: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not Present |  | DL CA only |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DL CA only | When DL CA without UL CA is used. |

### 5.3.2	Measurement information elements

As defined in clause 4.6.6 with the following exceptions:

Table 5.3.2-1: MeasDS-Config-DEFAULT

| Derivation Path: clause 4.6.6 Table 4.6.6-1B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasDS-Config-DEFAULT ::= CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| dmtc-PeriodOffset-r12 CHOICE { |  |  |  |
| ms40-r12 | 0 |  | LAA_Receiver |
| ms80-r12 | 0 |  | LAA_DemodCSI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LAA_Receiver | Used for LAA receiver characteristics test cases |
| LAA_DemodCSI | Used for LAA demodulation and CSI reporting test cases |

## 5.4	Default NAS message and information elements contents

## 5.5	Reference radio bearer configurations

### 5.5.1	SRB and DRB parameters

#### 5.5.1.1	MAC configurations

As defined in clause 4.8.2.1.5 with the following exceptions:

Table 5.5.1.1-1: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5, Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| ul-SCH-Config | Not present |  | SCell_AddMod |
| ul-SCH-Config SEQUENCE { |  |  |  |
| maxHARQ-Tx | n1 | Only one transmission per UL HARQ |  |
| } |  |  |  |
| drx-Config | Not present |  | SCell_AddMod |
| drx-Config CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| timeAlignmentTimerDedicated | infinity |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |

#### 5.5.1.2	Physical Layer configurations

Table 5.5.1.2-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: Clause 4.8.2.1.6 Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| pdsch-ConfigDedicated | PDSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  | Not present |  | RBC |
| pucch-ConfigDedicated | PUCCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  | Not present |  | RBC |
| pusch-ConfigDedicated | PUSCH-ConfigDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  | Not present |  | RBC |
| uplinkPowerControlDedicated | UplinkPowerControlDedicated-DEFAULT | See subclause 4.6.3 | SRB1 |
|  | Not present |  | RBC |
| tpc-PDCCH-ConfigPUCCH | Not present |  | SRB1 |
|  | TPC-PDCCH-Config-DEFAULT using condition PUCCH | See subclause 4.6.3 | RBC |
| tpc-PDCCH-ConfigPUSCH | Not present |  | SRB1 |
|  | TPC-PDCCH-Config-DEFAULT using condition PUSCH | See subclause 4.6.3 | RBC |
| cqi-ReportConfig | Not present |  | SRB1 |
|  | Not present |  | RBC |
| soundingRS-UL-ConfigDedicated | Not present |  | SRB1 |
|  | Not present |  | RBC |
| antennaInfo CHOICE { |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| schedulingRequestConfig | Not present |  | SRB1 |
|  | SchedulingRequest-Config-DEFAULT | See subclause 4.6.3 | SRB1 and HalfDuplex and (CEmodeA or CEmodeB) |
|  | Not present | See subclause 4.6.3 | RBC |
| } |  |  |  |
| cqi-ReportConfig-r10 | Not present |  | SCell_AddMod |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| SCell_AddMod | Addition or modification of SCell |
| CEmodeA | Used for CE mode A testing |
| CEmodeB | Used for CE mode B testing |
| HalfDuplex | Used during Type B HalfDuplex Operation |

#### 5.5.1.3	SRB and DRB combinations

##### 5.5.1.3.1	Combinations on DL-SCH and UL-SCH

5.5.1.3.1.1	SRB1 and SRB2 for DCCH + n x AM DRB + m x UM DRB, where n=1 and m=0

This SRB and DRB combination is setup in UE Registration procedure and the Generic Radio Bearer Establishment with UE Test Mode Activated using specific message content - the default RRCConnectionReconfiguration message with condition SRB2-DRB(n, m).

## 5.6	Test environment for NTN testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for IoT NTN UEs, unless otherwise specified.

### 5.6.1	UE location

UE shall determine its location during the test using any of the following means.

1.	UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test.

2.	Other options such as providing UE location via AT command are not precluded.

UE location provided to the UE before the start of the test for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows:

Longitude: 121.56076999

Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites)

Altitude: 0

### 5.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 5.6.2.0	Assumptions for Ephemeris generation

This section provides the assumptions used to generate Ephemeris information for GSO and NGSO orbits:

1.	Ephemeris information is generated for 3 types of satellites: GSO (inclination equal to 7º), LEO-600 and LEO-1200.

2.	UE location is as defined in section 5.6.1.

3.	PositionVelocity state vectors are used for EphemerisInfo signalling.

4.	For GSO, an altitude of 35,786 km is considered. For NGSO, 2 different altitudes are considered: 600 km and 1200 km.

5.	One-way delay between UE and satellite to be in the range of 119.375 ms to 128.79 ms for GSO and between 2 ms (lowest value for LEO orbit 600 km) and 6.67 ms (highest value for LEO orbit 1200 km)

6.  Doppler (frequency) shift up to [0.93] ppm for GSO and up to 24ppm for NGSO (LEO-600).

#### 5.6.2.1	Ephemeris for zero Doppler conditions

All eMTC NTN test cases defined in sections 6, 7 and 8 in TS 36.521-4 [73], unless otherwise stated, shall use the ephemeris configuration in tables 5.6.2.1-1 to 5.6.2.1-3, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test:

- In case UE supports only NGSO satellites, ephemeris values in Table 5.6.2.1-3 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

- In case UE supports both GSO and NGSO satellites, ephemeris values in Table 5.6.1.2.1-1 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 5.6.1.2.1-2 shall be used.

Table 5.6.2.1-1: SystemInformationBlockType31 – eMTC NTN Ephemeris Information for GSO satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1--2: SystemInformationBlockType31 – eMTC NTN Ephemeris Information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm. |  |  |  |

Table 5.6.2.1-3: SystemInformationBlockType31 –eMTC IoT NTN Ephemeris Information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 clause FFS |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r1 | 109528 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

### 5.6.3	eMTC NTN message contents for RF Tx/Rx, Demodulation tests

#### 5.6.3.1	ServingSatelliteInfo-r17 values in SIB31

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 5.6.2 (kept constant throughout the duration of each measurement in the test).

Table 5.6.3.1-1: ServingSatelliteInfo-r17 for RF tests

| Derivation Path: Table 4.4.3.3-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | See cl. 5.6.2 |  |  |
| positionY-r17 | See cl. 5.6.2 |  |  |
| positionZ-r17 | See cl. 5.6.2 |  |  |
| velocityVX-r17 | See cl. 5.6.2 |  |  |
| velocityVY-r17 | See cl. 5.6.2 |  |  |
| velocityVZ-r17 | See cl. 5.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| } |  |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| } |  |  |  |
| ] |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

# 6	Test environment for Signalling test

## 6.1	Requirements of test equipment

The requirements of test equipment specified in this subclause apply to Signalling test cases defined in TS 36.523-1 [18], in addition to the common requirements of test equipment specified in cause 4.2 of this specification.

Test equipment shall be able to simulate cells of Radio Access Technology (RAT) E-UTRA, UTRA, GSM or HRPD / 1xRTT. Regardless of respective RAT, the overall number and configuration of cells to be simulated simultaneously by test equipment shall not exceed the resources specified in the following Table 6.1-1:

Table 6.1-1: Maximum resources in terms of number / configuration of cells to be simulated simultaneously in a test setup

| Simulation of | Max. number / configuration of cells (SISO / SIMO) | Max. number / configuration of cells (MIMO) |
| --- | --- | --- |
| E-UTRA single-mode networks (FDD or TDD) | 3x cells | n/a |
| E-UTRA dual-mode networks (FDD and TDD) | 3x cells | n/a |
| E-UTRA networks involving Carrier Aggregation | 4x cells | n/a |
| Mixed E-UTRA / UTRA networks | 3x cells | n/a |
| Mixed E-UTRA / GSM networks | 3x cells | n/a |
| Mixed E-UTRA / HRPD or 1xRTT networks | 3x cells | n/a |
| Mixed E-UTRA / UTRA / GSM networks | 3x cells | n/a |
| Note 1: No differentiation between cell configuration types (as defined in clause 6.3.3) here, because these types are relevant to specific test cases and their TTCN-3 implementation only. Note 2: Only network scenarios specified in clause 4.4.1 and 6.3.2.1 have been covered. Note 3: In case of Carrier Aggregation, each cell can act as a PCell, an SCell, or a standalone cell (not used as a CA component carrier). In Release 10 a maximum of 1 SCell can be aggregated with a PCell. Note 4: Virtual Cells are not included in the maximum cell number as they do not require resources in the SS. |  |  |

Exceptions to the requirements outlined above are possible but need special evidence to be provided explicitly in the test case prose and should be allowed only if the test case purpose cannot be met otherwise.

Test equipment shall be able to simulate one or more UEs with PC5 interface for sidelink direct communication and/or sidelink direct discovery or V2X sidelink communication.

## 6.2	Reference test conditions

The reference test conditions specified in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified, in addition to the common reference test conditions specified in subclause 4.3 of this specification.

### 6.2.1	Physical channel allocations

#### 6.2.1.1	Antennas

If the UE has two or more Rx antennas, the same downlink signal is applied to each one, except if MIMO is tested. All UE Rx antennas shall be connected.

If the UE has one Rx antenna, the downlink signal is applied to it.

#### 6.2.1.2	Downlink physical channels and physical signals

Power allocation of downlink physical channels for Signalling test cases is specified in table 6.2.1.2-1.

Table 6.2.1.2-1: Power allocation for OFDM symbols and reference signals for Signalling test cases

| Physical Channel |  | EPRE Ratio |  | Comment |  |
| --- | --- | --- | --- | --- | --- |
| PBCH |  | PBCH_RA = 0 dB |  |  |  |
|  |  | PBCH_RB = 0 dB |  |  |  |
| PSS |  | PSS_RA = 0 dB |  |  |  |
| SSS |  | SSS_RA = 0 dB |  |  |  |
| PCFICH |  | PCFICH_RB = 0 dB |  |  |  |
| PDCCH |  | PDCCH_RA = 0 dB |  |  |  |
|  |  | PDCCH_RB = 0 dB |  |  |  |
| PDSCH (BCCH, BR-BCCH, CCCH, PCCH) |  | PDCCH_RA = 0 dB |  | To be consistent with default physical channel configuration in TS 36.331 [17], 9.2.4 |  |
|  |  | PDCCH_RB = 0 dB |  |  |  |
| PDSCH (DCCH, DTCH) |  | PDSCH_RA = -3 dB |  | To reduce interference from PDSCH of intra-frequency neighbour cells |  |
|  |  | PDSCH_RB = -3 dB |  | To reduce interference from PDSCH of intra-frequency neighbour cells |  |
| PHICH |  | PHICH_RB = 0 dB |  |  |  |
| PMCH |  | PMCH_RA = 0 dB |  |  |  |
| MBSFN RS |  | MBSFN RS_RA = 0dB |  |  |  |
| WUS |  | As per WUS-Config-v1560.powerBoost-r15 |  | According to TS 36.213 [29] clause 5.2 and the contents of Table 4.6.3-5 |  |
| PRS |  | PRS_RA = 0 dB |  |  |  |

NOTE:	MBSFN RS is not defined downlink physical channels in TS 36.211 [35].

#### 6.2.1.3	Mapping of downlink physical channels and signals to physical resources

Same as clause 4.3.3.3

#### 6.2.1.4	Uplink physical channels and physical signals

The SideLink Physical channels and Physical signals used and their powers relative to configured SS-UE power are specified in table 6.2.1.4-1.

Table 6.2.1.4-1: Relative power allocation for OFDM symbols and signals for SideLink test cases

| Physical Channel | EPRE Ratio |
| --- | --- |
| PSSS | 0 dB |
| SSSS | 0 dB |
| PSDCH | 0 dB |
| PSBCH | 0 dB |
| PSCCH | 0 dB |
| PSSCH | 0 dB |

#### 6.2.1.5	Mapping of uplink physical channels and signals to physical resources

[FFS].

### 6.2.2	Signal levels

#### 6.2.2.1	Downlink signal levels

The default settings of suitable cells and non-suitable cells for E-UTRA are specified in table 6.2.2.1-1.

Cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell in table 6.2.2.1-1.

Table 6.2.2.1-1: Default settings of suitable / non-suitable cells

| Power level type | E-UTRAN (Note 1-3) |  | UTRAN | GERAN |
| --- | --- | --- | --- | --- |
|  | Unit | Power level |  |  |
| Serving cell | dBm/15kHz | -85 | Table 6.1.1 (FDD) / 6.1.6a (TDD) [5] | Table 6.1.10 [5] |
| Suitable neighbour intra-frequency cell | dBm/15kHz | -91 | Table 6.1.2 (FDD) / 6.1.7 (TDD) [5] | n/a |
| Suitable neighbour inter-frequency cell | dBm/15kHz | -97 | Table 6.1.2 (FDD) / 6.1.7 (TDD) [5] | Table 6.1.10 [5] |
| Non-suitable cell | dBm/15kHz | -115 | Table 6.1.3 (FDD) / 6.1.8 (TDD) [5] | Table 6.1.11 [5] |
| Non-suitable "Off" cell | dBm/15kHz | ≤ -145 | Table 6.1.4 (FDD) / 6.1.9 (TDD) [5] | Cell is switched-off |
| Note 1: The power level is specified in terms of cell-specific RS EPRE instead of RSRP as RSRP is a measured value and cannot be directly controlled by the SS. Note 2: Power levels are specified based on the precondition that q-Hyst, a3-Offset and hysteresis are 0 dB. Note 3: The power level is specified at each UE Rx antenna. |  |  |  |  |

The default signal level uncertainty is specified in table 6.2.2.1-2 for any level specified, unless a tighter uncertainty is specified by a test case in TS 36.523-1 [18].

Table 6.2.2.1-2: SS signal level uncertainty

|  | Absolute signal level uncertainty for each cell | Relative signal level uncertainty between multiple cells |
| --- | --- | --- |
| Intra-frequency | +/-3 dB at each test port | +/-3 dB |
| Inter-frequency | +/-3 dB at each test port | See Note 1 |
| Note 1: For Inter-frequency cells the relative signal level uncertainty between multiple cells is determined by the absolute uncertainty of each cell, and does not have any additional constraint. |  |  |

Cell-specific RS EPRE setting should be equal to or higher than -115 dBm except for Non-suitable "Off" cell. The figure is chosen to ensure that for all bands the DL signal is within the RSRP measurement range specified in TS 36.133 [39] clauses 9.1.2 and 9.1.3, taking into account the SS default absolute signal level uncertainty.

NOTE: (The power spectral density of a white noise source; specified in TS 36.133 [39]) can be assumed to be -Infinity [dBm/15kHz] for all intra and inter frequency test cases. It is applicable to both idle mode and connected mode in TS 36.523-1 [18], unless otherwise specified in specific test cases.

For test cases requiring AWGN (Noc), the default level uncertainty is specified in table 6.2.2.1-3 for any level specified, unless a tighter uncertainty is specified by a test case in TS 36.523-1 [18].

Table 6.2.2.1-3: SS AWGN level uncertainty

|  | Absolute AWGN level uncertainty for each frequency |
| --- | --- |
| Intra-frequency | +/-3 dB at each test port |
| Inter-frequency | +/-3 dB at each test port |

#### 6.2.2.2	Measurement accuracy and side conditions

Measurement accuracy shall be considered in setting downlink power levels.

RSRP measurement accuracy in E-UTRA RRC_IDLE state is specified in table 6.2.2.2-1, derived from TS 36.133 [39] clauses 4.2.2.3 and 4.2.2.4. This measurement accuracy is applicable to idle mode test cases specified in TS 36.523-1 [18]. For the serving cell and suitable neighbour cells, the following side conditions shall be satisfied including the effect of signal level uncertainty.

- RSRP  -121 dBm

- RSRP Ês/Iot  -4 dB

- SCH_RP  -121 dBm

- SCH Ês/Iot  -4 dB

Table 6.2.2.2-1: RSRP measurement accuracy in E-UTRA RRC_IDLE state

|  | Absolute RSRP measurement accuracy | Relative RSRP measurement accuracy |
| --- | --- | --- |
| Intra-frequency | +/-6 dB | +/-3 dB |
| Inter-frequency | +/-6 dB | +/-5 dB |

RSRP measurement accuracy in E-UTRA RRC_CONNECTED state is specified in table 6.2.2.2-2, derived from TS 36.133 [39] clauses 9.1.2 and 9.1.3 selecting Normal condition. The ranges and side conditions in TS 36.133 [39] clauses 9.1.2 and 9.1.3 apply. This measurement accuracy is applicable to connected mode test cases specified in TS 36.523-1 [18]. For the serving cell and suitable neighbour cells, the following side conditions shall be satisfied including the effect of signal level uncertainty.

- RSRP  -124 dBm

- RSRP Ês/Iot > -6 dB- Io : -118 dBm/15kHz … -70 dBm/BWChannel (for absolute RSRP measurement accuracy)

- Io : -118 dBm/15kHz … -50 dBm/BWChannel (for relative RSRP measurement accuracy)

Table 6.2.2.2-2: RSRP measurement accuracy in E-UTRA RRC_CONNECTED state

|  | Absolute RSRP measurement accuracy | Relative RSRP measurement accuracy |
| --- | --- | --- |
| Intra-frequency | +/-6 dB | +/-3 dB |
| Inter-frequency | +/-6 dB | +/-6 dB |

Signal level difference between the serving cell and any suitable intra-frequency neighbour cell shall be nominally 6 dB to satisfy the measurement accuracy requirement and its side conditions specified in TS 36.133 [39]. This figure is chosen based on the following preconditions for intra-frequency cells.

- Interference to reference signals from reference signals of other cells is eliminated by Physical Cell Identity shifting as specified in TS 36.523-3 [20].

- Interference to reference signals from PDSCH with SI-RNTI of other cells is negligible because it’s sparse enough.

- Interference to reference signals from PDSCH of the serving cell is controlled by satisfying the conditions of clauses 6.2.2.1 and 6.2.2.2.

- Interference to P-SS/S-SS from P-SS/S-SS of other cells is eliminated by frame timing shifting as specified in TS 36.523-3 [20].

- Interference to P-SS/S-SS from PDSCH of other cells is eliminated by PDSCH resource allocation as specified in TS 36.523-3 [20].

#### 6.2.2.3	Uplink signal levels

The default settings of SS-UE is specified in table 6.2.2.3-1.

Table 6.2.2.3-1: Default settings of SS-UE power

| Power level type | SS-UE (Note 1) |  | Absolute signal level uncertainty for each SS-UE |
| --- | --- | --- | --- |
|  | Unit | Power level |  |
| SS-UE (default value) | dBm/15kHz | -85 | +/-3 dB |
| Note 1: The power level is specified at each Rx antenna of the UE under test. |  |  |  |

### 6.2.3	Default test frequencies

The default channel bandwidth of 1.4/3/5/10/20 MHz is applied to the signalling test. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

For sidelink direct discovery, the default channel bandwidth of 5/10/15/20 MHz is applied to the signalling test for all ProSe bands except band 14 where default bandwidth is 5/10 MHz and bands 31, 72 where default bandwidth is 5 MHz. For sidelink direct communication the default channel bandwidth of 10 MHz is applied to the signalling test except bands 31, 72 where bandwidth is 5 MHz.

For V2X communication, the default channel bandwidth of 10/20 MHz is applied to the signalling test for all V2X bands.

For eMTC NTN, the default channel bandwidth of 1.4 MHz is applied to the signalling test for all eMTC NTN bands.

For Band 13, Band 18,  Band 31 and Band 72, only one test frequency f1 is defined. All operating Bands except Band 13, Band 18, Band 31 and Band 72 can accommodate at least two test frequencies f1 and f2 (f1<f2). An additional test frequency f3 can be defined for the operating Bands with at least triple of the default bandwidth. The fourth test frequency f4 (f3<f1<f4<f2) is applicable to the operating Bands which have at least quadruple of the default bandwidth.

To the single cell signalling test with channel bandwidth different from the default bandwidths of the operating bands, Mid Range defined in clause 4.3.1 is applied.

#### 6.2.3.1	Test frequencies for signalling test

Test frequencies for signalling test are specified in table 6.2.3.1-1, 6.2.3.1-1a, 6.2.3.1-1d and 6.2.3.1-1e for FDD and table 6.2.3.1-2 and 6.2.3.1-2a for TDD. Except f4 and a few f1, f5 which are specified according to EARFCN of the concerned operating Bands, the majority of the test frequencies in table 6.2.3.1-1, 6.2.3.1-1a, 6.2.3.1-1d, 6.2.3.1-1e, 6.2.3.1-2 and 6.2.3.1-2a are specified in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 4.3.1.

Test frequencies for signalling test of MFBI are specified in table 6.2.3.1-1b and 6.2.3.1-1c for FDD and table 6.2.3.1-2b and 6.2.3.1-2c for TDD. Except f4 and a few f1, f5 which are specified according to EARFCN of the concerned operating Bands, the majority of the test frequencies in table 6.2.3.1-1b, 6.2.3.1-1c, 6.2.3.1-2b and 6.2.3.1-2c are specified in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 4.3.1.

Table 6.2.3.1-1: Test frequencies for E-UTRA FDD(5MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 1 | 60 | Mid | Mid | High | High | Low | Low | 18350 | 350 |
| 2 | 60 | Mid | Mid | High | High | Low | Low | 18950 | 950 |
| 3 | 75 | Mid | Mid | High | High | Low | Low | 19625 | 1625 |
| 4 | 45 | Mid | Mid | High | High | Low | Low | 20225 | 2225 |
| 5 | 25 | Mid | Mid | High | High | Low | Low | 20575 | 2575 |
| 6 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 7 | 70 | Mid | Mid | High | High | Low | Low | 21150 | 3150 |
| 8 | 35 | Mid | Mid | High | High | Low | Low | 21675 | 3675 |
| 9 | 35 | Mid | Mid | High | High | Low | Low | 22025 | 4025 |
| 10 | 60 | Mid | Mid | High | High | Low | Low | 22500 | 4500 |
| 12 | 17 | Low | Low | High | High | Mid | Mid | N/A | N/A |
| 14 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| … |  |  |  |  |  |  |  |  |  |
| 17 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 19 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 21 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 22 | 80 | Mid | Mid | High | High | Low | Low | 24950 | 6950 |
| ... |  |  |  |  |  |  |  |  |  |
| 23 | 20 | 25575 | 7575 | High | High | Low | Low | 25625 | 7625 |
| 24 | 342 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 25 | 65 | Mid | Mid | High | High | Low | Low | 26415 | 8415 |
| 26 | 35 | Mid | Mid | High | High | Low | Low | 26915 | 8915 |
| 27 | 17 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 28 | 45 | Mid | Mid | High | High | Low | Low | 27560 | 9560 |
| 30 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 31 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| … |  |  |  |  |  |  |  |  |  |
| 65 | 90 | Mid | Mid | High | High | Low | Low | 131422 | 65886 |
| 66 | 70+901 | Mid | Mid | High | High | Low | Low | 132272 | 66736 |
| 68 | 30 | Mid | Mid | High | High | Low | Low | 132867 | 67731 |
| … |  |  |  |  |  |  |  |  |  |
| 70 | 15+251 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 71 | 35 | Mid | Mid | High | High | Low | Low | 133247 | 68711 |
| 72 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 73 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 74 | 43 | Mid | Mid | High | High | Low | Low | 133882 | 69336 |
| 85 | 18 | Low | Low | High | High | Mid | Mid | N/A | N/A |
| 87 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 88 | 5 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Asymmetric operating band (UL + DL). NOTE 2: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-1a: Test frequencies for E-UTRA FDD(10MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 11 | 20 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 13 | 10 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 18 | 15 | Low | Low | N/A | N/A | N/A | N/A | N/A | N/A |
| 20 | 30 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 23 | 20 | Low | Low | High | High | N/A | N/A | N/A | N/A |

Table 6.2.3.1-1b: Test frequencies for E-UTRA FDD(5MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 2 | 25 | 60 | Mid | Mid | High | High | Low | Low | 18950 | 950 |
| 3 | 9 | 35 | Mid | Mid | High | High | Low | Low | 19824 | 1824 |
| 4 | 10 | 45 | Mid | Mid | High | High | Low | Low | 20225 | 2225 |
| 4 | 66 | 45 | Mid | Mid | High | High | Low | Low | 20225 | 2225 |
| 5 | 18 | 6 | Mid | Mid | N/A | N/A | N/A | N/A | N/A | N/A |
| 5 | 19 | 25 | Mid | Mid | High | High | Low | Low | 20585 | 2585 |
| 5 | 26 | 25 | Mid | Mid | High | High | Low | Low | 20575 | 2575 |
| 9 | 3 | 35 | Mid | Mid | High | High | Low | Low | 22025 | 4025 |
| 10 | 4 | 45 | Mid | Mid | High | High | Low | Low | 22425 | 4425 |
| 10 | 66 | 60 | Mid | Mid | High | High | Low | Low | 22425 | 4425 |
| 12 | 17 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 17 | 12 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 17 | 85 | 12 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 19 | 5 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 19 | 26 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 25 | 2 | 60 | Mid | Mid | High | High | Low | Low | 26415 | 8415 |
| 26 | 5 | 25 | Mid | Mid | High | High | Low | Low | 26915 | 8915 |
| 26 | 18 | 15 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 26 | 19 | 15 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 26 | 27 | 10 | Mid | Mid | High | High | Low | Low | N/A | N/A |
| 27 | 18 | 9 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 27 | 26 | 10 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 66 | 4 | 45 | Mid | Mid | High | High | Low | Low | 132247 | 66711 |
| 66 | 10 | 60 | Mid | Mid | High | High | Low | Low | 132247 | 66711 |
| 85 | 17 | 12 | Mid | Mid | High | High | N/A | N/A | N/A | N/A |

Table 6.2.3.1-1c: Test frequencies for E-UTRA FDD(10MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 18 | 5 | 6 | Low | Low | N/A | N/A | N/A | N/A | N/A | N/A |
| 18 | 26 | 15 | Low | Low | High | High | N/A | N/A | N/A | N/A |
| 18 | 27 | 9 | Low | Low | High | High | N/A | N/A | N/A | N/A |

Table 6.2.3.1-1d: Test frequencies for E-UTRA FDD(1.4MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 253 | 7 | Mid | Mid | High | High | Low | Low | 261318 | 228550 |
| 254 | 16.5 | Mid | Mid | High | High | Low | Low | 261432 | 228664 |
| 255 | 34 | Mid | Mid | High | High | Low | Low | 261724 | 228956 |
| 256 | 30 | Mid | Mid | High | High | Low | Low | 262039 | 229271 |

Table 6.2.3.1-1e: Test frequencies for E-UTRA FDD(3MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 |  | f2, f6 |  | f3, f7 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 106 | 5 | Mid1 | Mid1 | N/A | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: In the Rel-18 version of the specification, only EARFCN 70686 (NDL) and 134322 (NUL) is applicable for 3 MHz channel bandwidth. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.1-2: Test frequencies for E-UTRA TDD(5MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 | f2, f6 | F3, f7 | f4 |
| --- | --- | --- | --- | --- | --- |
| 33 | 20 | 36075 | High | Low | 36125 |
| 34 | 15 | Mid | High | Low | N/A |
| 35 | 60 | Mid | High | Low | 36700 |
| 36 | 60 | Mid | High | Low | 37300 |
| 37 | 20 | 37625 | High | Low | 37675 |
| … |  |  |  |  |  |
| 43 | 100 | Mid | High | Low | 44640 |
| 44 | 50 | Mid | High | Low | 46115 |
| 45 | 20 | 46665 | High | Low | 46715 |
| … |  |  |  |  |  |
| 53 | 11.5 | Low | High | N/A | N/A |
| 54 | 5 | Mid | N/A | N/A | N/A |

Table 6.2.3.1-2a: Test frequencies for E-UTRA TDD(20MHz)

| E-UTRA Operating Band | Bandwidth [MHz] | f1, f5 | f2, f6 | f3, f7 | f4 |
| --- | --- | --- | --- | --- | --- |
| 38 | 50 | Low | High | N/A | N/A |
| 39 | 40 | Low | High | N/A | N/A |
| 40 | 100 | Mid | High | Low | 39350 |
| 41 | 194 | Mid | High | Low | 40970 |
| 42 | 200 | Mid | High | Low | 43040 |
| … |  |  |  |  |  |
| 48 | 150 | Mid | High | Low | 56315 |

Table 6.2.3.1-2b: Test frequencies for E-UTRA TDD(5MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 | f2, f6 | F3, f7 | f4 |
| --- | --- | --- | --- | --- | --- | --- |
| 33 | 39 | 20 | 36075 | High | Low | 36125 |

Table 6.2.3.1-2c: Test frequencies for E-UTRA TDD(20MHz) MFBI

| E-UTRA Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 | f2, f6 | f3, f7 | f4 |
| --- | --- | --- | --- | --- | --- | --- |
| 38 | 41 | 50 | Low | High | N/A | N/A |
| 39 | 33 | 20 | Low | High | N/A | N/A |
| 41 | 38 | 50 | High | Low | Mid | N/A |

#### 6.2.3.2	Test frequencies for CA signalling test

Test frequencies for CA signalling testing are specified in Table 6.2.3.2-1 for CA Intra-Band contiguous case; in Tables 6.2.3.2-2 for CA Inter-band case and in Table 6.2.3.2-4 for Intra-Band Non-Contiguous case.

NOTE 1:	Alternative test frequencies for additional channel bandwidth combinations may need to be specified when new CA configurations or CA Bandwidth Combination Sets are introduced in TS 36.101 subclause 5.6A.

For CA Intra-Band contiguous scenarios then f1, f2 and f3 are used.

NOTE 2:	f2 and f3 are not adjacent frequencies; hence those cannot be used simultaneously as Pcell and Scell for CA Intra-Band contiguous scenarios.

For CA Inter-Band scenarios then f1, f2, f5 and f6 are used.

NOTE 3:	Table 6.2.3.2-2 specifies the test frequencies for operation with PCell in the frequency band mentioned first (= carrier 1) and SCell in the frequency band mentioned second (= carrier 2) in the E-UTRA CA Configuration name acc. to TS 36.101 subclause 5.6A (e.g. PCell in band 1 and SCell in band 5 for CA_1A-5A configuration), for operation with switched allocation of PCell and SCell. SCell in the frequency band mentioned first (= carrier 1) and PCell in the frequency band mentioned second (= carrier 2) in the E-UTRA CA Configuration name (given that UL is supported in both frequency bands in question).

NOTE 4:	For operation with switched allocation f1 is mapped to f5, f2 is mapped to f6, f5 is mapped to f1, f6 is mapped to f2

For CA Intra-Band non-contiguous scenarios then f1, f2 and f3 are used.

Table 6.2.3.2-1: Test frequencies for E-UTRA PCell and SCell for CA contiguous Intra-band operation

| E-UTRA CA Configuration | Width of Operating band [MHz] | CC Combination / NRB_agg | Test Frequency | CC NRB | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1C | 60 | 100 + 100 | f1 | CC 100 | 18300 | 1950 | 300 | 2140 |
|  |  |  | f2 | CC 100 | 18498 | 1969.8 | 498 | 2159.8 |
|  |  |  | f3 | CC 100 | 18102 | 1930.2 | 102 | 2120.2 |
| CA_2C | 60 | 100 + 100 | f1 | CC 100 | 18901 | 1880.1 | 901 | 1960.1 |
|  |  |  | f2 | CC 100 | 19099 | 1899.9 | 1099 | 1979.9 |
|  |  |  | f3 | CC 100 | 18703 | 1860.3 | 703 | 1940.3 |
| CA_3C | 60 | 100 + 100 | f1 | CC 100 | 19505 | 1740.5 | 1505 | 1835.5 |
|  |  |  | f2 | CC 100 | 19703 | 1760.3 | 1703 | 1855.3 |
|  |  |  | f3 | CC 100 | 19307 | 1720.7 | 1307 | 1815.7 |
| CA_5B | 25 | 50+50 | f1 | CC_50 | 20476 | 831.6 | 2476 | 876.6 |
|  |  |  | f2 | CC_50 | 20575 | 841.5 | 2575 | 886.5 |
|  |  |  | f3 | N/A | N/A | N/A | N/A | N/A |
| CA_7B | 70 | 75+25 | f1 | CC 100 | 21050 | 2530 | 3050 | 2650 |
|  |  |  | f2 | CC 100 | 21248 | 2549.8 | 3248 | 2669.8 |
|  |  |  | f3 | CC 100 | 20852 | 2510.2 | 2852 | 2630.2 |
| CA_7C | 70 | 100 + 100 | f1 | CC 100 | 21050 | 2530 | 3050 | 2650 |
|  |  |  | f2 | CC 100 | 21248 | 2549.8 | 3248 | 2669.8 |
|  |  |  | f3 | CC 100 | 20852 | 2510.2 | 2852 | 2630.2 |
| CA_8B | 35 | 50+50 | f1 | CC_50 | 21651 | 900.1 | 3651 | 945.1 |
|  |  |  | f2 | CC_50 | 21750 | 910 | 3750 | 955 |
|  |  |  | f3 | CC_50 | 21552 | 890.2 | 3552 | 935.2 |
| CA_12B | 17 | 25+25 | f1 | CC_25 | 23095 | 707.5 | 5095 | 737.5 |
|  |  |  | f2 | CC_25 | 23143 | 712.3 | 5143 | 742.3 |
|  |  |  | f3 | CC_25 | 23047 | 702.7 | 5047 | 732.7 |
| CA_23B | 20 | 50+50 | f1 | CC_50 | 25551 | 2005.1 | 7551 | 2185.1 |
|  |  |  | f2 | CC_50 | 25650 | 2015 | 7650 | 2195 |
|  |  |  | f3 | N/A | N/A | N/A | N/A | N/A |
| CA_27B | 17 | 25+25 | f1 | CC_25 | 27100 | 813 | 9100 | 858 |
|  |  |  | f2 | CC_25 | 27148 | 817.8 | 9148 | 862.8 |
|  |  |  | f3 | CC_25 | 27050 | 808 | 9050 | 853 |
| CA_38C | 50 | 100 + 100 | f1 | CC 100 | 37901 | 2585.1 | 37901 | 2585.1 |
|  |  |  | f2 | CC 100 | 38099 | 2604.9 | 38099 | 2604.9 |
|  |  |  | f3 | N/A | N/A | N/A | N/A | N/A |
| CA_39C | 40 | 100 + 50 | f1 | CC 100 | 38401 | 1895.1 | 38401 | 1895.1 |
|  |  |  | f2 | CC 50 | 38545 | 1909.5 | 38545 | 1909.5 |
|  |  |  | f3 | NA | NA | NA | NA | NA |
| CA_40C | 100 | 100 + 100 | f1 | CC 100 | 39051 | 2340.1 | 39051 | 2340.1 |
|  |  |  | f2 | CC 100 | 39249 | 2359.9 | 39249 | 2359.9 |
|  |  |  | f3 | CC 100 | 38853 | 2320.3 | 38853 | 2320.3 |
| CA_41C | 194 | 100 + 100 | f1 | CC 100 | 40521 | 2583.1 | 40521 | 2583.1 |
|  |  |  | f2 | CC 100 | 40719 | 2602.9 | 40719 | 2602.9 |
|  |  |  | f3 | CC 100 | 40323 | 2563.3 | 40323 | 2563.3 |
| CA_42C | 200 | 100+100 | f1 | CC 100 | 42491 | 3490.1 | 42491 | 3490.1 |
|  |  |  | f2 | CC 100 | 42689 | 3509.9 | 42689 | 3509.9 |
|  |  |  | f3 | CC 100 | 42293 | 3470.3 | 42293 | 3470.3 |
| CA_48C | 150 | 100 + 100 | f1 | CC 100 | 55891 | 3615.1 | 55891 | 3615.1 |
|  |  |  | f2 | CC 100 | 56089 | 3634.9 | 56089 | 3634.9 |
|  |  |  | f3 | CC 100 | 55693 | 3595.3 | 55693 | 3595.3 |
| CA_66B | 70+901 | 50+50 | f1 | CC_50 | 132322 | 1745 | 66786 | 2145 |
|  |  |  | f2 | CC_50 | 132421 | 1754.9 | 66885 | 2154.9 |
|  |  |  | f3 | CC_50 | 132223 | 1735.1 | 66687 | 2135.1 |
| CA_66C | 70+901 | 100+100 | f1 | CC 100 | 132322 | 1745 | 66786 | 2145 |
|  |  |  | f2 | CC 100 | 132520 | 1764.8 | 66984 | 2164.8 |
|  |  |  | f3 | CC 100 | 132124 | 1725.2 | 66588 | 2125.2 |
| CA_70C | 15+251 | 75+50 | f1 | CC 75 | 133047 | 1702.5 | 68411 | 2002.5 |
|  |  |  | f2 | CC 50 | N/A | N/A | 68531 | 2014.5 |
|  |  |  | f3 | N/A | N/A | N/A | N/A | N/A |
| NOTE 1: Asymmetric operating band (UL + DL) |  |  |  |  |  |  |  |  |

Table 6.2.3.2-2: Test frequencies for E-UTRA PCell and SCell for CA Inter-band operation and E-UTRA PCell and PSCell for DC Inter-band operation (two bands)

| E-UTRA CA Configuration | Width of Operating bands [MHz+MHz] |  | CC Combination / NRB_agg |  | Test Frequency |  | CC NRB |  | NUL |  | Frequency of Uplink [MHz] |  | NDL |  | Frequency of Downlink [MHz] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A, DC_1A-3A | 60+75 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
| CA_1A-5A, DC_1A-5A | 60+25 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [18550] |  | [1975] |  | [550] |  | [2165] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
| CA_1A-7A, DC_1A-7A | 60 + 70 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21400 |  | 2565 |  | 3400 |  | 2685 |  |
| CA_1A-8A, DC_1A-8A | 60 + 35 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
| CA_1A-11A | 60 + 20 |  | 100 + 50 |  | f1 |  | CC 100 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 18500 |  | 1970 |  | 500 |  | 2160 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
| CA_1A-18A | 60 + 15 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23900 |  | 820 |  | 5900 |  | 865 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_1A-19A, DC_1A-19A | 60+15 |  | 75 + 50 |  | f1 |  | CC 75 |  | 18324 |  | 1952.4 |  | 324 |  | 2142.4 |  |
|  |  |  |  |  | f2 |  | CC 75 |  | 18525 |  | 1972.5 |  | 525 |  | 2162.5 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 24100 |  | 840 |  | 6100 |  | 885 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_1A-20A | 60 + 30 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 24200 |  | 837 |  | 6200 |  | 796 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
| CA_1A-21A, DC_1A-21A | 60+15 |  | 75 + 75 |  | f1 |  | CC 75 |  | 18324 |  | 1952.4 |  | 324 |  | 2142.4 |  |
|  |  |  |  |  | f2 |  | CC 75 |  | 18525 |  | 1972.5 |  | 525 |  | 2162.5 |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 24525 |  | 1455.4 |  | 6525 |  | 1503.4 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_1A-26A | 60 + 35 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 26865 |  | 831.5 |  | 8865 |  | 876.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 26990 |  | 844 |  | 8990 |  | 889 |  |
| CA_1A-28A | 60+45 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_1A-38A | 60+50 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 38000 |  | 2595 |  | 38000 |  | 2595 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 38200 |  | 2615 |  | 38200 |  | 2615 |  |
| CA_1A-40A | 60 + 100 |  | 50 + 100 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 39150 |  | 2350 |  | 39150 |  | 2350 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 39550 |  | 2390 |  | 39550 |  | 2390 |  |
| CA_1A-41A | 60 + 194 |  | 50 + 100 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
| CA_1A-41C | 60+194 |  | 50+100+100 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_1A-42A | 60 + 200 |  | 75 + 100 |  | f1 |  | CC 75 |  | 18324 |  | 1952.4 |  | 324 |  | 2142.4 |  |
|  |  |  |  |  | f2 |  | CC 75 |  | 18525 |  | 1972.5 |  | 525 |  | 2162.5 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_1A-42C | 60+200 |  | 50+100+100 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_1A-46A 3 | 60+775 |  | 50+100 |  | f1 |  | CC 50 |  | 18300 |  | 1950 |  | 300 |  | 2140 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 18550 |  | 1975 |  | 550 |  | 2165 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_2A-4A, DC_2A-4A | 60 + 45 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
| CA_2A-5A | 60+25 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
| CA_2A-7A, DC_2A-7A | 60 + 70 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21400 |  | 2565 |  | 3400 |  | 2685 |  |
| CA_2A-12A | 60 + 17 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23060 |  | 704 |  | 5060 |  | 734 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_2A-13A, DC_2A-13A | 60 + 10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23230 |  | 782 |  | 5230 |  | 751 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_2A-14A | 60 + 10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23330 |  | 793 |  | 5330 |  | 763 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_2A-17A | 60+12 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23790 |  | 710 |  | 5790 |  | 740 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_2A-28A | 60+45 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_2A-29A | 60+11 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_2A-30A | 60+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_2A-46A 3 | 60+775 |  | 50+100 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_2A-66A | 60+(70+90)2 |  | 50+100 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_2A-71A | 60+35 |  | 50+50 |  | f1 |  | CC 50 |  | 18900 |  | 1880 |  | 900 |  | 1960 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19150 |  | 1905 |  | 1150 |  | 1985 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 133297 |  | 680.5 |  | 68761 |  | 634.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 133422 |  | 693 |  | 68886 |  | 647 |  |
| CA_3A-5A, DC_3A-5A | 75+25 |  | 50 + 50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [19900] |  | [1780] |  | [1900] |  | [1875] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
| CA_3C-5A | 75+25 |  | 100+100+50 |  | f1 |  | CC 100 |  | 19475 |  | 1737.5 |  | 1475 |  | 1832.5 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 19673 |  | 1757.3 |  | 1673 |  | 1852.3 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
| CA_3A-7A, DC_3A-7A | 75+70 |  | 50 + 50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [19900] |  | [1780] |  | [1900] |  | [1875] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21400 |  | 2565 |  | 3400 |  | 2685 |  |
| CA_3A-8A, DC_3A-8A | 75+35 |  | 50+50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
| CA_3A-11A | 75+20 |  | 75+50 |  | f1 |  | CC75 |  | 19674 |  | 1757.4 |  | 1674 |  | 1852.4 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_3A-19A, DC_3A-19A | 75+15 |  | 100+50 |  | f1 |  | CC 100 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 19850 |  | 1775 |  | 1850 |  | 1870 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 24100 |  | 840 |  | 6100 |  | 885 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_3A-20A, DC_3A-20A | 75+30 |  | 50+50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 24200 |  | 837 |  | 6200 |  | 796 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
| CA_3A-26A, DC_3A-26A | 75 + 35 |  | 50 + 50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 26865 |  | 831.5 |  | 8865 |  | 876.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 26990 |  | 844 |  | 8990 |  | 889 |  |
| CA_3A-27A | 75 + 17 |  | 50 + 25 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 25 |  | 27125 |  | 815.5 |  | 9125 |  | 860.5 |  |
|  |  |  |  |  | f6 |  | CC 25 |  | 27185 |  | 821.5 |  | 9185 |  | 866.5 |  |
| CA_3A-28A | 75+45 |  | 50+50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27435 |  | 725.5 |  | 9435 |  | 780.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_3A-69A | 75 + 50 |  | 50 + 50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 68086 |  | 2595 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 68286 |  | 2615 |  |
| CA_3A-32A | 75+44 |  | 50+50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 10140 |  | 1474 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 10310 |  | 1491 |  |
| CA_3A-38A | 75 + 50 |  | 50 + 50 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 38000 |  | 2595 |  | 38000 |  | 2595 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 38200 |  | 2615 |  | 38200 |  | 2615 |  |
| CA_3A-40A | 75 + 100 |  | 50 + 100 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 39150 |  | 2350 |  | 39150 |  | 2350 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 39550 |  | 2390 |  | 39550 |  | 2390 |  |
| CA_3A-41A | 75 + 194 |  | 100 + 100 |  | f1 |  | CC 100 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 19850 |  | 1775 |  | 1850 |  | 1870 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
| CA_3A-42A | 75 + 200 |  | 100 + 100 |  | f1 |  | CC 100 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 19850 |  | 1775 |  | 1850 |  | 1870 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_3A-42C | 75+200 |  | 50+100+100 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_3A-46A 3 | 75+775 |  | 50+100 |  | f1 |  | CC 50 |  | 19575 |  | 1747.5 |  | 1575 |  | 1842.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 19900 |  | 1780 |  | 1900 |  | 1875 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_4A-5A | 45+25 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
| CA_4A-7A, DC_4A-7A | 45+70 |  | 50+50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21400 |  | 2565 |  | 3400 |  | 2685 |  |
| CA_4A-12A, DC_4A-12A | 45+17 |  | 50+50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23130 |  | 711 |  | 5130 |  | 741 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_4A-13A, DC_4A-13A | 45+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23230 |  | 782 |  | 5230 |  | 751 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_4A-17A, DC_4A-17A | 45+12 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23790 |  | 710 |  | 5790 |  | 740 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_4A-28A | 45+45 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_4A-29A | 45+11 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_4A-30A | 45+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_4A-46A 3 | 54+775 |  | 50+100 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_4A-71A | 45+35 |  | 50+50 |  | f1 |  | CC 50 |  | 20175 |  | 1732.5 |  | 2175 |  | 2132.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20350 |  | 1750 |  | 2350 |  | 2150 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 133297 |  | 680.5 |  | 68761 |  | 634.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 133422 |  | 693 |  | 68886 |  | 647 |  |
| CA_5A-7A, DC_5A-7A | 25+70 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21400 |  | 2565 |  | 3400 |  | 2685 |  |
| CA_5A-12A, DC_5A-12A | 25+17 |  | 50+50 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23130 |  | 711 |  | 5130 |  | 741 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_5A-13A | 25+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20450 |  | 829 |  | 2450 |  | 874 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23230 |  | 782 |  | 5230 |  | 751 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_5A-17A | 25+12 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23790 |  | 710 |  | 5790 |  | 740 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_5A-25A | 25+65 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 26363 |  | 1882.5 |  | 8365 |  | 1962.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 26640 |  | 1910 |  | 8640 |  | 1990 |  |
| CA_5A-29A | 25 + 11 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_5A-30A | 25+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_5A-40A | 25 + 100 |  | 50 + 100 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 39150 |  | 2350 |  | 39150 |  | 2350 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 39600 |  | 2395 |  | 39600 |  | 2395 |  |
| CA_5A-40C | 25 + 100 |  | 50 + 100+100 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 39051 |  | 2340.1 |  | 39051 |  | 2340.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 39249 |  | 2359.9 |  | 39249 |  | 2359.9 |  |
| CA_5A-66A | 25+(70+90)2 |  | 50+100 |  | f1 |  | CC 50 |  | 20525 |  | 836.5 |  | 2525 |  | 881.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 20600 |  | 844 |  | 2600 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_7A-8A | 70+35 |  | 50+50 |  | f1 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [21400] |  | [2565] |  | [3400] |  | [2685] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
| CA_7A-12A | 70+17 |  | 50+50 |  | f1 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [21400] |  | [2565] |  | [3400] |  | [2685] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23130 |  | 711 |  | 5130 |  | 741 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_7A-20A, DC_7A-20A | 70+30 |  | 50 + 50 |  | f1 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [21400] |  | [2565] |  | [3400] |  | [2685] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 24200 |  | 837 |  | 6200 |  | 796 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
| CA_7A-22A | 70+80 |  | 50+50 |  | f1 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [21400] |  | [2565] |  | [3400] |  | [2685] |  |
|  |  |  |  |  | f1 |  | CC 50 |  | 24650 |  | 3415 |  | 6650 |  | 3515 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 25350 |  | 3485 |  | 7350 |  | 3585 |  |
| CA_7A-28A, DC_7A-28A | 70+45 |  | 50+50 |  | f1 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [21400] |  | [2565] |  | [3400] |  | [2685] |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_7A-46A 3 | 70+775 |  | 50+100 |  | f1 |  | CC 50 |  | 21100 |  | 2535 |  | 3100 |  | 2655 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | [21400] |  | [2565] |  | [3400] |  | [2685] |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_8A-11A | 35 + 20 |  | 50 + 50 |  | f1 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_8A-20A | 35 + 30 |  | 50 + 50 |  | f1 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 24200 |  | 837 |  | 6200 |  | 796 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
| CA_8A-27A | 35 + 17 |  | 50 + 25 |  | f1 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 25 |  | 27125 |  | 815.5 |  | 9125 |  | 860.5 |  |
|  |  |  |  |  | f6 |  | CC 25 |  | 27185 |  | 821.5 |  | 9185 |  | 866.5 |  |
| CA_8A-28A | 35 + 45 |  | 50 + 50 |  | f1 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_8A-40A | 35 + 100 |  | 50 + 100 |  | f1 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 39150 |  | 2350 |  | 39150 |  | 2350 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 39550 |  | 2390 |  | 39550 |  | 2390 |  |
| CA_8A-41A | 35 + 194 |  | 50 + 100 |  | f1 |  | CC 50 |  | 21651 |  | 900.1 |  | 3651 |  | 945.1 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_8A-41C | 35 + 194 |  | 50 + 100+100 |  | f1 |  | CC 50 |  | 21651 |  | 900.1 |  | 3651 |  | 945.1 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_8A-42A | 35+200 |  | 50 + 100 |  | f1 |  | CC 50 |  | 21625 |  | 897.5 |  | 3625 |  | 942.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 21750 |  | 910 |  | 3750 |  | 955 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_11A-18A | 20+15 |  | 50 + 50 |  | f1 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 23900 |  | 820 |  | 5900 |  | 865 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_11A-28A | 20+30 |  | 50 + 50 |  | f1 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_11A-41A | 20+194 |  | 50 + 100 |  | f1 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_11A-41C | 20+194 |  | 50 + 100+100 |  | f1 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_11A-42A | 20+200 |  | 50 + 100 |  | f1 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_11A-42C | 20+200 |  | 50 + 100+100 |  | f1 |  | CC 50 |  | 22800 |  | 1432.9 |  | 4800 |  | 1480.9 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 22900 |  | 1442.9 |  | 4900 |  | 1490.9 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_12A-25A | 17+65 |  | 50 + 50 |  | f1 |  | CC 50 |  | 23060 |  | 704 |  | 5060 |  | 734 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 26363 |  | 1882.5 |  | 8365 |  | 1962.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 26640 |  | 1910 |  | 8640 |  | 1990 |  |
| CA_12A-30A | 17+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 23060 |  | 704 |  | 5060 |  | 734 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_12A-66A | 17+(70+90)2 |  | 50 + 100 |  | f1 |  | CC 50 |  | 23060 |  | 704 |  | 5060 |  | 734 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_13A-66A | 10 + (70+90)2 |  | 50 + 100 |  | f1 |  | CC 50 |  | 23230 |  | 782 |  | 5230 |  | 751 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_14A-30A | 10+10 |  | 50 + 50 |  | f1 |  | CC 50 |  | 23330 |  | 793 |  | 5330 |  | 763 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_14A-66A | 10+(70+90)2 |  | 50 + 100 |  | f1 |  | CC 50 |  | 23330 |  | 793 |  | 5330 |  | 763 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_18A-28A | 15+30 |  | 50 + 50 |  | f1 |  | CC 50 |  | 23900 |  | 820 |  | 5900 |  | 865 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27360 |  | 718 |  | 9360 |  | 773 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27460 |  | 728 |  | 9460 |  | 783 |  |
| CA_19A-21A, DC_19A-21A | 15+15 |  | 50+75 |  | f1 |  | CC 50 |  | 24075 |  | 837.5 |  | 6075 |  | 882.5 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 24525 |  | 1455.4 |  | 6525 |  | 1503.4 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_19A-28A | 15+30 |  | 50+50 |  | f1 |  | CC 50 |  | 24075 |  | 837.5 |  | 6075 |  | 882.5 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27360 |  | 718 |  | 9360 |  | 773 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27460 |  | 728 |  | 9460 |  | 783 |  |
| CA_19A-42A | 15 + 200 |  | 50 + 100 |  | f1 |  | CC 50 |  | 24075 |  | 837.5 |  | 6075 |  | 882.5 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_19A-42C | 15+200 |  | 50+100+100 |  | f1 |  | CC 50 |  | 24075 |  | 837.5 |  | 6075 |  | 882.5 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_20A-28A | 30+45 |  | 50 + 50 |  | f1 |  | CC 50 |  | 24300 |  | 847 |  | 6300 |  | 806 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
| CA_20A-31A | 30+5 |  | 50 + 25 |  | f1 |  | CC 50 |  | 24300 |  | 847 |  | 6300 |  | 806 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
|  |  |  |  |  | f5 |  | CC 25 |  | 27785 |  | 455 |  | 9895 |  | 465 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_20A-32A | 30+44 |  | 50 + 50 |  | f1 |  | CC 50 |  | 24300 |  | 847 |  | 6300 |  | 806 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 10140 |  | 1474 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 10310 |  | 1491 |  |
| CA_20A-40A | 30+100 |  | 50 + 100 |  | f1 |  | CC 50 |  | 24300 |  | 847 |  | 6300 |  | 806 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 39150 |  | 2350 |  | 39150 |  | 2350 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 39550 |  | 2390 |  | 39550 |  | 2390 |  |
| CA_20A-67A | 30+20 |  | 50 + 50 |  | f1 |  | CC 50 |  | 24300 |  | 847 |  | 6300 |  | 806 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 24400 |  | 857 |  | 6400 |  | 816 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 67386 |  | 743 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 67486 |  | 753 |  |
| CA_21A-42A | 15+200 |  | 75 + 100 |  | f1 |  | CC 75 |  | 24525 |  | 1455.4 |  | 6525 |  | 1503.4 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_21A-42C | 15+200 |  | 75+100+100 |  | f1 |  | CC 75 |  | 24525 |  | 1455.4 |  | 6525 |  | 1503.4 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_23A-29A | 20+11 |  | 50 + 50 |  | f1 |  | CC 50 |  | 25550 |  | 2005 |  | 7550 |  | 2185 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 25650 |  | 2015 |  | 7650 |  | 2195 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_25A-26A | 65+35 |  | 50 + 50 |  | f1 |  | CC 50 |  | 26365 |  | 1882.5 |  | 8365 |  | 1962.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 26640 |  | 1910 |  | 8640 |  | 1990 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 26865 |  | 831.5 |  | 8865 |  | 876.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 26990 |  | 844 |  | 8990 |  | 889 |  |
| CA_25A-41A | 65+194 |  | 50 + 50 |  | f1 |  | CC 50 |  | 26365 |  | 1882.5 |  | 8365 |  | 1962.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 26640 |  | 1910 |  | 8640 |  | 1990 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 41540 |  | 2685 |  | 41540 |  | 2685 |  |
| CA_25A-41C | 65+194 |  | 50+100+100 |  | f1 |  | CC 50 |  | 26363 |  | 1882.5 |  | 8365 |  | 1962.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 26640 |  | 1910 |  | 8640 |  | 1990 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_26A-41A | 60 + 194 |  | 50 + 100 |  | f1 |  | CC 50 |  | 26865 |  | 831.5 |  | 8865 |  | 876.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 26990 |  | 844 |  | 8990 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
| CA_26A-41C | 60+194 |  | 50+100+100 |  | f1 |  | CC 50 |  | 26865 |  | 831.5 |  | 8865 |  | 876.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 26990 |  | 844 |  | 8990 |  | 889 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_28A-41A | 45+194 |  | 50 + 100 |  | f1 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
| CA_28A-41C | 45+194 |  | 50+100+100 |  | f1 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
| CA_28A-42A | 45+200 |  | 50 + 100 |  | f1 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_28A-42C | 45+200 |  | 50+100+100 |  | f1 |  | CC 50 |  | 27410 |  | 723 |  | 9410 |  | 778 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | 27610 |  | 743 |  | 9610 |  | 798 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_29A-30A1 | 11+10 |  | 50+50 |  | f1 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f6 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_29A-66A1 | 11+(70+90)2 |  | 50+100 |  | f1 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_29A-70A1 | 11+(15+25)2 |  | 50+75 |  | f1 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 75 |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_29A-70C1 | 11+(15+25)2 |  | 50+75+50 |  | f1 |  | CC 50 |  | N/A |  | N/A |  | 9720 |  | 723 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 68531 |  | 2014.5 |  |
| CA_30A-48A |  |  | 10 + 150 |  | 50 + 100 |  | f1 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  |  |  | f5 |  | CC 100 |  | 55990 |  | 3625 |  | 55990 |  | 3625 |  |
|  |  |  |  |  |  |  | f6 |  | CC 100 |  | 56640 |  | 3690 |  | 56640 |  | 3690 |  |
| CA_30A-66A | 10+(70+90)2 |  | 50+100 |  | f1 |  | CC 50 |  | 27710 |  | 2310 |  | 9820 |  | 2355 |  |
|  |  |  |  |  | f2 |  | N/A |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_39A-41A, DC_39A-41A | 40+194 |  | 100+100 |  | f1 |  | CC 100 |  | 38350 |  | 1890 |  | 38350 |  | 1890 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 38550 |  | 1910 |  | 38550 |  | 1910 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
| CA_41A-42A, DC_41A-42A | 194+200 |  | 100 + 100 |  | f1 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_41A-42C | 194+200 |  | 100+100+100 |  | f1 |  | CC100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42491 |  | 3490.1 |  | 42491 |  | 3490.1 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 42689 |  | 3509.9 |  | 42689 |  | 3509.9 |  |
| CA_41A-46A 3 | 194+775 |  | 100+100 |  | f1 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_41C-42A | 194+200 |  | 100+100+100 |  | f1 |  | CC 100 |  | 40521 |  | 2583.1 |  | 40521 |  | 2583.1 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 40719 |  | 2602.9 |  | 40719 |  | 2602.9 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
| CA_41A-48A | 194+150 |  | 100+100 |  | f1 |  | CC 100 |  | 40620 |  | 2593 |  | 40620 |  | 2593 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 41490 |  | 2680 |  | 41490 |  | 2680 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 55990 |  | 3625 |  | 55990 |  | 3625 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 56640 |  | 3690 |  | 56640 |  | 3690 |  |
| CA_42A-46A 3 | 200+775 |  | 100+100 |  | f1 |  | CC 100 |  | 42590 |  | 3500 |  | 42590 |  | 3500 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 43490 |  | 3590 |  | 43490 |  | 3590 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
| CA_46A-66A1,3 | 775+(70+90)2 |  | 100+100 |  | f1 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_46A-66C1,3 | 775+(70+90)2 |  | 100+100+100 |  | f1 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132322 |  | 1745 |  | 66786 |  | 2145 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132520 |  | 1764.8 |  | 66984 |  | 2164.8 |  |
| CA_46A-70A1,3 | 775+(15+25)2 |  | 100+75 |  | f1 |  | CC 100 |  | 50690 |  | 5540 |  | 50690 |  | 5540 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 53940 |  | 5865 |  | 53940 |  | 5865 |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 75 |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_46C-66A1,3 | 775+(70+90)2 |  | 100+100+100 |  | f1 |  | CC 100 |  | 50692 |  | 5540.2 |  | 50692 |  | 5540.2 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 50890 |  | 5560 |  | 50890 |  | 5560 |  |
|  |  |  |  |  | f5 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f6 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
| CA_66A-70A | (70+90) 2 + (15+25)2 |  | 100+75 |  | f1 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 75 |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_66A-70C | (70+90) 2 + (15+25)2 |  | 100+75+50 |  | f1 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 68531 |  | 2014.5 |  |
| CA_66A-71A | (70+90) 2 + 35 |  | 100+100+ 50 |  | f1 |  | CC 100 |  | 132422 |  | 1755 |  | 66886 |  | 2155 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 132572 |  | 1770 |  | 67036 |  | 2170 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 133297 |  | 680.5 |  | 68761 |  | 634.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 133422 |  | 693 |  | 68886 |  | 647 |  |
| CA_66C-70A | (70+90) 2 + (15+25)2 |  | 100+100+75 |  | f1 |  | CC 100 |  | 132322 |  | 1745 |  | 66786 |  | 2145 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 132520 |  | 1764.8 |  | 66984 |  | 2164.8 |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 75 |  | N/A |  | N/A |  | N/A |  | N/A |  |
| CA_66C-70C | (70+90) 2 + (15+25)2 |  | 100+100+75+50 |  | f1 |  | CC 100 |  | 132322 |  | 1745 |  | 66786 |  | 2145 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 132520 |  | 1764.8 |  | 66984 |  | 2164.8 |  |
|  |  |  |  |  | f5 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | N/A |  | N/A |  | 68531 |  | 2014.5 |  |
| CA_66C-71A | (70+90) 2 + 35 |  | 100+100+ 50 |  | f1 |  | CC 100 |  | 132322 |  | 1745 |  | 66786 |  | 2145 |  |
|  |  |  |  |  | f2 |  | CC 100 |  | 132520 |  | 1764.8 |  | 66984 |  | 2164.8 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 133297 |  | 680.5 |  | 68761 |  | 634.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 133422 |  | 693 |  | 68886 |  | 647 |  |
| CA_70A-71A | (15+25)2 + 35 |  | 75+50 |  | f1 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f2 |  | CC 75 |  | N/A |  | N/A |  | N/A |  | N/A |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 133297 |  | 680.5 |  | 68761 |  | 634.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 133422 |  | 693 |  | 68886 |  | 647 |  |
| CA_70C-71A | (15+25)2 + 35 |  | 75+50+50 |  | f1 |  | CC 75 |  | 133047 |  | 1702.5 |  | 68411 |  | 2002.5 |  |
|  |  |  |  |  | f2 |  | CC 50 |  | N/A |  | N/A |  | 68531 |  | 2014.5 |  |
|  |  |  |  |  | f5 |  | CC 50 |  | 133297 |  | 680.5 |  | 68761 |  | 634.5 |  |
|  |  |  |  |  | f6 |  | CC 50 |  | 133422 |  | 693 |  | 68886 |  | 647 |  |
| Note 1: Exceptionally the second band test frequencies of this CA combination shall be used for PCell and the first band test frequencies of this CA combination shall be used for the SCell respectively. Note 2: Asymmetric operating band (UL + DL). Note 3: Band 46 is restricted to E-UTRA operation when carrier aggregation is configured. In Rel-13 the downlink only operating band 46 is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.2-3: Void

Table 6.2.3.2-4: Test frequencies for E-UTRA PCell and SCell for CA non-contiguous Intra-band operation

| E-UTRA CA Configuration | Width of Operating band [MHz] | CC Combination / NRB_agg | Test Frequency | CC NRB | NUL | Frequency of Uplink [MHz] | NDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_2A-2A | 60 | 50 + 50 | f1 | CC 50 | 18650 | 1855 | 650 | 1935 |
|  |  |  | f2 | CC 50 | 19150 | 1905 | 1150 | 1985 |
|  |  |  | f3 | CC 50 | 18900 | 1880 | 900 | 1960 |
| CA_3A-3A | 75 | 50 + 50 | f1 | CC 50 | 19250 | 1715 | 1250 | 1810 |
|  |  |  | f2 | CC 50 | 19900 | 1780 | 1900 | 1875 |
|  |  |  | f3 | CC 50 | 19575 | 1747.5 | 1575 | 1842.5 |
| CA_4A-4A | 45 | 50 + 50 | f1 | CC 50 | 20000 | 1715 | 2000 | 2115 |
|  |  |  | f2 | CC 50 | 20350 | 1750 | 2350 | 2150 |
|  |  |  | f3 | CC 50 | 20175 | 1732.5 | 2175 | 2132.5 |
| CA_5A-5A | 25 | 50 + 50 | f1 | CC 50 | 20450 | 829 | 2450 | 874 |
|  |  |  | f2 | CC 50 | 20600 | 844 | 2600 | 889 |
|  |  |  | f3 | N/A | N/A | N/A | N/A | N/A |
| CA_7A-7A | 70 | 50 + 50 | f1 | CC 50 | 20800 | 2505 | 2800 | 2625 |
|  |  |  | f2 | CC 50 | 21400 | 2565 | 3400 | 2685 |
|  |  |  | f3 | CC 50 | 21100 | 2535 | 3100 | 2655 |
| CA_23A-23A | 20 | 25 + 50 | f1 | CC 25 | 25525 | 2002.5 | 7525 | 2182.5 |
|  |  |  | f2 | CC 50 | 25650 | 2015 | 7650 | 2195 |
|  |  |  | f3 | N/A | N/A | N/A | N/A | N/A |
| CA_25A-25A | 65 | 50 + 50 | f1 | CC 50 | 26090 | 1855 | 8090 | 1935 |
|  |  |  | f2 | CC 50 | 26640 | 1910 | 8640 | 1990 |
|  |  |  | f3 | CC 50 | 26365 | 1882,5 | 8365 | 1962.5 |
| CA_41A-41A | 194 | 100 + 100 | f1 | CC 100 | 39750 | 2506 | 39750 | 2506 |
|  |  |  | f2 | CC 100 | 41490 | 2680 | 41490 | 2680 |
|  |  |  | f3 | CC 100 | 40620 | 2593 | 40620 | 2593 |
| CA_41A-41C | 194 + 194 | 100 + 100 | f1 | CC 100 | 39750 | 2506 | 39750 | 2506 |
|  |  |  | f2 | CC 100 | 41490 | 2680 | 41490 | 2680 |
|  |  |  | f3 | CC 100 | 40620 | 2593 | 40620 | 2593 |
| CA_42A-42A | 200 | 100 + 100 | f1 | CC 100 | 41690 | 3410 | 41690 | 3410 |
|  |  |  | f2 | CC 100 | 43490 | 3590 | 43490 | 3590 |
|  |  |  | f3 | CC 100 | 42590 | 3500 | 42590 | 3500 |
| CA_66A-66A | 70+901 | 50 + 50 | f1 | CC 50 | 132022 | 1715 | 66486 | 2115 |
|  |  |  | f2 | CC 50 | 132622 | 1775 | 67086 | 2175 |
|  |  |  | f3 | CC 50 | 132322 | 1745 | 66786 | 2145 |
| NOTE 1: Asymmetric operating band (UL + DL) |  |  |  |  |  |  |  |  |

#### 6.2.3.3	Test frequencies for ProSe signalling test

Test frequencies for prose direct discovery and communication signalling testing are specified in Table 6.2.3.3-1 and Table 6.2.3.3-2, these are derived from section 4.3.1 using uplink mid range frequency (i.e. f1 in Table 6.2.3.1-1) for supported Prose Bands as specified in TS 36.101 table 5.6D.

Table 6.2.3.3-1: Test frequencies for E-UTRA cells and Sidelink Discovery

| E-UTRA ProSe Band | E-UTRA Operating Band | Default Operating BW MHz | f1 |  | f2 |  | f3 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 2 | 2 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 3 | 3 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 4 | 4 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 7 | 7 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 14 | 14 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 20 | 20 | 10 | Refer to Table 6.2.3.1-1a |  |  |  |  |  |  |  |
| 26 | 26 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 28 | 28 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 31 | 31 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 41 | 41 | 20 | Refer to Table 6.2.3.1-2a |  |  |  |  |  |  |  |
| 68 | 68 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |
| 72 | 72 | 5 | Refer to Table 6.2.3.1-1 |  |  |  |  |  |  |  |

Table 6.2.3.3-2: Test frequencies for E-UTRA cells and Sidelink Communication

| E-UTRA ProSe Band | E-UTRA Operating Band | Default Operating BW MHz | f1 |  | f2 |  | f3 |  | f4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL | NUL | NDL |
| 3 | 3 | 10 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 7 | 7 | 10 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 14 | 14 | 10 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 20 | 20 | 10 | Table 6.2.3.1-1a |  |  |  |  |  |  |  |
| 26 | 26 | 10 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 28 | 28 | 10 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 31 | 31 | 5 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 68 | 68 | 5 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |
| 72 | 72 | 5 | FFS | FFS | FFS | FFS | FFS | FFS | FFS | FFS |

#### 6.2.3.4	Test frequencies for MFBI frequency band priority adjustment signalling test

Test frequencies for MFBI frequency band priority adjustment signalling testing are specified in Table 6.2.3.4-1, these are derived from section 4.3.1 using uplink mid range frequency.

Table 6.2.3.4-1: MFBI frequency band priority adjustment test frequency/Inter-band CA

| E-UTRA CA Configuration | Width of Operating bands [MHz+MHz] | CC Combination / NRB_agg | E-UTRA Operating Band | MFBI Operating Band | MFBI Overlapping Band | f1, f5 |
| --- | --- | --- | --- | --- | --- | --- |
| CA_39A-41A | 40+194 | 100+100 | 39 | 38 | 41 | Mid (Note) |
| Note: For f1, Mid refer to Table 4.3.1.2.7-1. For f5, Mid refer to Table 4.3.1.5.26-1. |  |  |  |  |  |  |

#### 6.2.3.5	Test frequencies for V2X Communication

Test frequencies for V2X communication signalling testing are specified in Table 6.2.3.5-1, these are derived from section 4.3.1 using uplink mid, low and high range frequency for supported V2X communication Bands as specified in TS 36.101 table 5.6G.

Table 6.2.3.5-1: Test frequencies for V2X Communication

| E-UTRA V2X Communication Band | Bandwidth [MHz] | Default Operating BW MHz | f5 |  | f6 |  | f7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 47 | 60 | 10 | Mid | Mid | High | High | Low | Low |
| 47 | 60 | 20 | Mid | Mid | High | High | Low | Low |

## 6.3	Reference system configurations

The reference system configurations specified in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified, in addition to the common reference system configurations specified in subclause 4.4 of this specification.

For Signalling testing, MIMO (Multiple Input Multiple Output) is not applied for all cell configurations regardless of UE MIMO functionality. Only one SS Tx antenna is used.

One or two UE antennas are used for all signalling test cases. (*1)

(*1) Two UE antennas configuration is possible for UE diversity case.

### 6.3.1	Default parameter specific for simulated cells

Default parameters specific for simulated cells are specified in this subclause.

#### 6.3.1.1	Intra-frequency neighbouring cell list in SIB4 for E-UTRA cells

Intra-frequency neighbouring cell list for signalling test cases is defined in table 6.3.1.1-1. This table is referred to in the default contents of IE intraFreqNeighbouringCellList in SystemInformationBlockType4 defined in table 4.4.3.3-3.

Table 6.3.1.1-1: Intra-frequency neighbouring cell lists for E-UTRA cells

| cell ID | Test Frequency | intra-frequency neighbouring cell list |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |  | q-OffsetCell [n] |  |  |
|  |  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| Cell 1 | f1 | 3 | Cell 2 | Cell 4 | Cell 11 | dB0 | dB0 | dB0 |
| Cell 2 | f1 | 3 | Cell 1 | Cell 4 | Cell 11 | dB0 | dB0 | dB0 |
| Cell 4 | f1 | 3 | Cell 1 | Cell 2 | Cell 11 | dB0 | dB0 | dB0 |
| Cell 11 | f1 | 3 | Cell 1 | Cell 2 | Cell 4 | dB0 | dB0 | dB0 |
| Cell 3 | f2 | 1 | Cell 23 | - | - | dB0 | - | - |
| Cell 23 | f2 | 1 | Cell 3 | - | - | dB0 | - | - |

NOTE:	The intra-frequency E-UTRA neighbouring cell list for signalling NAS test cases when cells are on same PLMN is defined in table 6.3.2.3.1-1.

#### 6.3.1.2	Inter-frequency carrier frequency list in SIB5 for E-UTRA cells

Inter-frequency E-UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.2-1. This table is referred to in the default contents of IE interFreqCarrierFreqList in SystemInformationBlockType5 defined in table 4.4.3.3-4.

Table 6.3.1.2-1: Inter-frequency carrier frequency lists for E-UTRA cells

| cell ID | Test Frequency | interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  | 1 | 2 | 3 |
| Cell 1 Cell 2 Cell 4 Cell 11 | f1 | 3 | f2 | f3 | f5 |
| Cell 3 Cell 23 | f2 | 3 | f1 | f3 | f5 |
| Cell 6 | f3 | 3 | f1 | f2 | f5 |
| Cell 10 | f5 | 3 | f1 | f2 | f3 |
| Note 1: The inter-frequency E-UTRA carrier frequency list for signalling NAS test cases when cells are on same PLMN is defined in table 6.3.2.3.2-1. Note 2: Depending on the Band under test, f3 may not be applicable. Note 3: In case of Test frequency f1, f2 and f3, dl-CarrierFreq f5 as part of inter-frequency list is applicable only in case of multi-band scenarios. Note 4: In case of Test frequency f5, dl-CarrierFreq f1 as part of inter-frequency list is applicable only in case of multi-band scenarios. |  |  |  |  |  |

In the case of dual mode multi-cell network scenarios as defined in subclause 4.4.1.3, inter-frequency E-UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.2-2.

Table 6.3.1.2-2: Inter-frequency carrier frequency lists for E-UTRA cells in dual mode scenario

| cell ID | Test Frequency | interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  | 1 | 2 | 3 |
| Cell 1 Cell 2 Cell 4 | f1 | 3 | f2 | f5 | f6 |
| Cell 3 | f2 | 3 | f1 | f5 | f6 |
| Cell 10, Cell 30, Cell 31 | f5 | 3 | f1 | f2 | f6 |
| Cell 28, Cell 29 | f6 | 3 | f1 | f2 | f5 |

#### 6.3.1.3	UTRA carrier frequency list in SIB6 for E-UTRA cells

UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.3-1. This table is referred to in the default contents of IE carrierFreqListUTRA-FDD and carrierFreqListUTRA-TDD in SystemInformationBlockType6 defined in table 4.4.3.3-5.

Table 6.3.1.3-1: UTRA carrier frequency lists for E-UTRA cells

| interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- |
| number of entries | carrierFreq[n] |  |  |
|  | 1 | 2 | 3 |
| 3 | f8 | f9 | f10 |
| Note: Band VI has two entries. |  |  |  |

Table 6.3.1.3-2: Mapping of UTRA cell with TS 34.108 [5]

| UTRA cell | Frequency | UTRA cell in TS 34.108, clause 6.1 | UTRA frequency in TS 34.108 |
| --- | --- | --- | --- |
| Cell 5 | f8 | Cell 1 | High (Note 2) |
| Cell 7 | f8 | Cell 2 | High (Note 2) |
| Cell 8 | f9 | Cell 4 | Mid (Note 2) |
| Cell 9 | f10 | Cell 7 | Low |
| Note 1: The following simultaneous co-existences in the test are not allowed: - cells on f1 (eUTRA cell 1, cell 2, cell 4, cell 11) and f9 (UTRA cell 8); - cells on f2 (eUTRA cell 3, cell 12, cell 23) and f8 (UTRA cell 5, cell 7); - cells on f3 (eUTRA cell 6, Cell 13) and f10 (UTRA cell 9). Note 2: On UTRA Band VI, the Low range test frequency is applied to f9 for Cell 8, and f8 for Cell 5 and Cell 7. Note 3: The combinations of the referred TS 34.108 UTRA Cell index and frequency in this table override that specified in TS 34.108. |  |  |  |

#### 6.3.1.4	GERAN carrier frequency group list in SIB7 for E-UTRA cells

GERAN carrier frequency group list for signalling test cases is defined in table 6.3.1.4-1. This table is referred to in the default contents of IE carrierFreqsInfoList in SystemInformationBlockType7 defined in table 4.4.3.3-6.

Table 6.3.1.4-1: GERAN carrier frequency group list for E-UTRA cells

| carrierFreqsInfoList |  |  |  |  |
| --- | --- | --- | --- | --- |
| number of entries | index (n) | carrierFreqs[n] |  |  |
|  |  | startingARFCN[n] | explicitListOfARFCNs[n] |  |
|  |  |  | number of entries | ARFCN-ValueGERAN |
| 1 | 1 | f11 | 2 | f12, f13 |

Table 6.3.1.4-2: Mapping of GERAN cells with TS 51.010-1 [25]

| GERAN cell | Frequency | GERAN cell in TS 51.010-1, clause 40 |
| --- | --- | --- |
| Cell 24 | f11 | Cell A |
| Cell 25 | f12 | Cell D |
| Cell 26 | f13 | Cell B |

NOTE 2:	Unless otherwise stated, GERAN cells 24/25/26 take the default values of GERAN cells A/B/D as defined in TS 51.010 clause 40.

#### 6.3.1.5	CDMA2000 HRPD carrier frequency list in SIB8 for E-UTRA cells

CDMA2000 HRPD carrier frequency list for signalling test cases is defined in table 6.3.1.5-1. This table is referred to in the default contents of IE cellReselectionParametersHRPD in SystemInformationBlockTyp8 defined in table 4.4.3.3-7.

Table 6.3.1.5-1: CDMA2000 HRPD carrier frequency list for E-UTRA cells

| neighCellsPerFreqList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| number of entries | index (n) | arfcn[n] | physCellIdList[n] |  |  |
|  |  |  | number of entries | index | PhysCellIdCDMA2000 |
| 3 | 1 | f14 | 2 | 1 | Cell 15 |
|  |  |  |  | 2 | Cell 16 |
|  | 2 | f15 | 1 | 1 | Cell 17 |
|  | 3 | f16 | 1 | 1 | Cell 18 |

#### 6.3.1.6	CDMA2000 1xRTT carrier frequency list in SIB8 for E-UTRA cells

CDMA2000 1xRTT carrier frequency list for signalling test cases is defined in table 6.3.1.6-1. This table is referred to in the default contents of IE cellReselectionParameters1XRTT in SystemInformationBlockTyp8 defined in table 4.4.3.3-7.

Table 6.3.1.6-1: CDMA2000 1xRTT carrier frequency list for E-UTRA cells

| neighCellsPerFreqList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| number of entries | index (n) | arfcn[n] | physCellIdList[n] |  |  |
|  |  |  | number of entries | index | PhysCellIdCDMA2000 |
| 3 | 1 | f17 | 2 | 1 | Cell 19 |
|  |  |  |  | 2 | Cell 20 |
|  | 2 | f18 | 1 | 1 | Cell 21 |
|  | 3 | f19 | 1 | 1 | Cell 22 |

#### 6.3.1.7	E-UTRA carrier frequency list in SIB19 for UTRA cells

E-UTRA carrier frequency list for signalling test cases is defined in table 6.3.1.7-1. This table is referred to in the default contents of IE eutra-FrequencyAndPriorityInfoList in System Information Block type 19 defined in table 4.4.4.1-1.

Table 6.3.1.7-1: E-UTRA carrier frequency list for UTRA cells

| eutra-FrequencyAndPriorityInfoList |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| number of entries | earfcn[n] |  |  |  |  |
|  | 1 | 2 | 3 | 4 | - |
| 4 | f1 | f2 | f3 | f4 | - |
| Note: Depending on the Band under test, f2, f3 and f4 may not be applicable. |  |  |  |  |  |

#### 6.3.1.8	NR carrier frequency list in SIB24 for E-UTRA cells

NR carrier frequency list for signalling test cases is defined in table 6.3.1.8-1. This table is referred to in the default contents of IE carrierFreqListNR-r15 in System Information Block type 24 defined in Table 4.4.3.3-20.

Table 6.3.1.8-1: NR carrier frequency list for E-UTRA cells

| carrierFreqListNR-r15 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| number of entries | nr-arfcn[n] |  |  |  |  |
|  | 1 | 2 | 3 | 4 | - |
| 4 | NRf1 | NRf2 | NRf3 | NRf4 | - |
| Note 1: Depending on the NR Band under test, NRf2, NRf3 and NRf4 may not be applicable. Note 2: NRf1, NRf2, NRf3 and NRf4 are defined in TS 38.508-1[67] clause 4.4.2 and 6.2.3. |  |  |  |  |  |

### 6.3.2	Default configurations for NAS test cases

The default configurations specified in this subclause apply only to NAS test cases. They apply to all NAS test cases unless otherwise specified.

#### 6.3.2.1	Simulated network scenarios for NAS test cases

Simulated network scenarios for NAS test cases to be tested are specified in the pre-test conditions of each individual test case.

NOTE:	The number of cells specified does not necessarily correspond to the maximum number of resources to be configured simultaneously in test equipment. Please refer to Table 6.1-1 for such information.

Any combination is allowed with the following restrictions:

-	Cell E should not be used if Cell G or Cell H is used, otherwise two different PLMNs will be operated on the same frequency

-	a maximum 3 cells on the same frequency can be used, i.e. only 3 cells out of cells A, B, C, D and M may be used simultaneously in each individual test case when cells in the test case are in different PLMNs (refer to Table 6.3.2.2-3).

#### 6.3.2.2	Simulated NAS cells

Simulated NAS cells and default parameters are specified in table 6.3.2.2-1.

Unless otherwise specified, the default parameters specified in section 4 also apply to all NAS cells.

Table 6.3.2.2-1: Default parameters for simulated NAS cells

| NAS cell ID | Tracking Area |  |  |  | TA# list (Note 1) | GUTI (Note 2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | MME Identifier |  | M-TMSI |
|  |  | MCC | MNC |  |  | MME Group ID | MME Code |  |
| Cell A | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.8 [2]. |
| Cell B | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32770 | 1 |  |
| Cell C | TAI-3 | (Note 3) |  | 3 | TAI-3 | 32771 | 1 |  |
| Cell D | TAI-4 | (Note 3) |  | 4 | TAI-4 | 32772 | 1 |  |
| Cell E | TAI-12 | 002 | 101 | 3 | TAI-12 | 32777 | 1 |  |
| Cell F |  |  |  |  |  |  |  |  |
| Cell G | TAI-7 | (Note 4) | 02 | 1 | TAI-7 | 32775 | 1 |  |
| Cell H | TAI-8 | (Note 4) | 02 | 2 | TAI-8 | 32776 | 1 |  |
| Cell I | TAI-9 | 002 | 101 | 1 | TAI-9 | 32777 | 1 |  |
| Cell J | TAI-10 | 003 | 101 | 1 | TAI-10 | 32778 | 1 |  |
| Cell K | TAI-9 | 002 | 101 | 1 | TAI-9 | 32777 | 1 |  |
| Cell L | TAI-11 | 002 | 101 | 2 | TAI-11 | 32779 | 1 |  |
| Cell M | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Cell N | TAI-7 | (Note 4) | 02 | 1 | TAI-7 | 32780 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column GUTI indicates GUTI included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). Note 4: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.9.3). Note 5: Cell A is a serving cell and the other cells are suitable neighbour cells. The definitions are specified in subclause 6.2.2.1. |  |  |  |  |  |  |  |  |

Table 6.3.2.2-2: Default cell identifiers for simulated NAS cells when cells are in same PLMN

| NAS cell ID | Frequency | E-UTRAN Cell Identifier |  | Physical layer cell identity | rootSequenceIndex FDD | rootSequenceIndex TDD |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | eNB Identifier | Cell Identity |  |  |  |
| Cell A | f1 | ‘0000 0000 0000 0001 0001’B | '0000 0001'B | 1 | 22 | 0 |
| Cell B | f1 | ‘0000 0000 0000 0001 0001’B | '0000 0010'B | 2 | 86 | 8 |
| Cell C | f2 | ‘0000 0000 0000 0001 0010’B | '0000 0011'B | 3 | 22 | 0 |
| Cell D | f1 | ‘0000 0000 0000 0001 0001’B | '0000 0100'B | 4 | 150 | 16 |
| Cell E | NA | NA | NA | NA | NA | NA |
| Cell F | NA | NA | NA | NA | NA | NA |
| Cell G | NA | NA | NA | NA | NA | NA |
| Cell H | NA | NA | NA | NA | NA | NA |
| Cell I | NA | NA | NA | NA | NA | NA |
| Cell J | NA | NA | NA | NA | NA | NA |
| Cell M | f2 | ‘0000 0000 0000 0010 0001’B | '0001 0001'B | 17 | 86 | 8 |

Table 6.3.2.2-3: Default cell identifiers for simulated NAS cells when cells are in different PLMNs

| NAS cell ID | PLMN | Frequency | E-UTRAN Cell Identifier |  | Physical layer cell identity | rootSequenceIndex FDD | rootSequenceIndex TDD |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | eNB Identifier | Cell Identity |  |  |  |
| Cell A | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0001'B | 1 | 22 | 0 |
| Cell B | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0010'B | 2 | 86 | 8 |
| Cell C | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0011'B | 3 | 150 | 16 |
| Cell D | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0100'B | 4 | 214 | 24 |
| Cell E | MCC=002 MNC=101 | f2 | ‘0000 0000 0000 0011 0101’B | '0001 0111'B | 23 | 22 | 0 |
| Cell F | NA | NA | NA | NA | NA | NA | NA |
| Cell G | MCC = MCC in USIM MNC=02 | f2 | '0000 0000 0000 0010 0010'B | '0000 1011'B | 11 | 86 | 8 |
| Cell H | MCC= MCC in USIM MNC=02 | f2 | '0000 0000 0000 0010 0010'B | '0000 1100'B | 12 | 150 | 16 |
| Cell I | MCC=002 MNC=101 | f3 (Note 1) | '0000 0000 0000 0010 0011'B | '0000 1101'B | 13 | 22 | 0 |
| Cell J | MCC=003 MNC=101 | f4 (Note 1) | '0000 0000 0000 0010 0100'B | '0000 1110'B | 14 | 22 | 0 |
| Cell K | MCC=002 MNC=101 | f3 (Note 1) | '0000 0000 0000 0010 0011'B | '0000 1111'B | 15 | 86 | 8 |
| Cell L | MCC=002 MNC=101 | f3 (Note 1) | '0000 0000 0000 0010 0011'B | '0001 0000'B | 16 | 150 | 16 |
| Cell M | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0001 0001'B | 17 | 278 | 32 |
| Cell N | MCC = MCC in USIM MNC=02 | f1 | '0000 0000 0000 0010 0010'B | '0000 1011'B | 18 | 342 | NA |
| Note 1: The test frequency f3 or f4 is allocated to the cell if f1 and f2 are already allocated to the cells in the test. Otherwise, f1 or f2 is allocated, instead. |  |  |  |  |  |  |  |

#### 6.3.2.3	Broadcast system information

##### 6.3.2.3.1	Intra-frequency neighbouring cell list in SIB4 for E-UTRA NAS cells

Intra-frequency neighbouring cell list of the same PLMN for the NAS signalling test is defined in table 6.3.2.3.1-1 when SIB4 to be broadcast.

Table 6.3.2.3.1-1: Intra-freq. lists in SIB4 for NAS test cases (same PLMN)

| NAS cell ID | Test Frequency | Intra-frequency neighbouring cell list |  |  |
| --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |
|  |  |  | 1 | 2 |
| Cell A | f1 | 2 | Cell B | Cell D |
| Cell B | f1 | 2 | Cell A | Cell D |
| Cell D | f1 | 2 | Cell A | Cell B |
| Cell C | f2 | 1 | Cell M | N/A |
| Cell M | f2 | 1 | Cell C | N/A |

##### 6.3.2.3.2	Inter-frequency carrier frequency list in SIB5 for E-UTRA NAS cells

Inter-frequency neighbouring carrier and cell lists for NAS signalling test cases are defined in table 6.3.2.3.2-1 for same PLMN and table 6.3.2.3.2-2 for different PLMN.

Table 6.3.2.3.2-1: Inter-freq. lists in SIB5 for NAS test cases (same PLMN)

| NAS cell ID | Test Frequency | interFreqCarrierFreqList |  |  |
| --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |
|  |  |  | 1 | 2 |
| Cell A Cell B Cell D | f1 | 1 | f2 | NA |
| Cell C Cell M | f2 | 1 | f1 | NA |
| NOTE: Operating on Band 13, SIB5 is not broadcast. |  |  |  |  |

Table 6.3.2.3.2-2: Inter-freq. lists in SIB5 for NAS test cases (different PLMN)

| NAS cell ID | Test Frequency | interFreqCarrierFreqList |  |  | Condition |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  | 1 | 2 |  |
| Cell A Cell B Cell C Cell D Cell M | f1 | 0 | NA | NA |  |
| Cell G Cell H | f2 | 0 | NA | NA |  |
| Cell I Cell K Cell L | f3 | 0 | NA | NA | E-NOT-CONF |
|  |  | 1 | f2 |  | E-CONF |
| Cell J | f4 | 0 | NA | NA |  |
| Cell E | f2 | 1 | f3 | NA |  |

| Condition | Explanation |
| --- | --- |
| E-NOT-CONF | cell E is not configured in the test |
| E-CONF | cell E is configured in the test |

### 6.3.3	Cell configurations

For the purpose of test not involving Carrier Aggregation, three types of SS cell configurations are defined, full (Active) cell configuration, broadcast only cell configuration and minimum uplink cell configuration.

The abovementioned configurations can also be used in Carrier Aggregation test cases, either for PCells or SCells. However another configuration, the virtual cell configuration, can additionally be used for an SCell that is not active.

#### 6.3.3.1	Full cell configuration

Full cell configuration is also called active cell configuration. The cell configuration, in minimum, has all defined DL and UL physical channels configured, i.e.

in DL: 	PBCH, PCFICH, PDCCH, PDSCH, PHICH,

in UL: 	PRACH, PUCCH, PUSCH.

The DL and UL Reference and synchronization (both primary and secondary) signals are also configured.

#### 6.3.3.2	Minimum uplink cell configuration

In this cell configuration,

in DL: 	physical channels capable of transmission, i.e. PBCH, PCFICH, PDCCH, PDSCH are configured;

DL physical reference and synchronization (both primary and secondary) signals are also configured.

In UL: 	PRACH is configured. SS shall report any detection of PRACH preambles, in order to assign test verdicts. Decoding the preambles is not required.

#### 6.3.3.3	Broadcast only cell configuration

In this cell configuration,

in DL: 	physical channels capable of transmission, i.e. PBCH, PCFICH, PDCCH, PDSCH are configured;

DL physical reference and synchronization (both primary and secondary) signals are also configured.

in UL: 	no physical resources are configured, neither channels, nor signals.

#### 6.3.3.3A	Virtual cell configuration

In this cell configuration,

in DL: 	no physical resources are configured, neither channels, nor signals;

in UL: 	no physical resources are configured, neither channels, nor signals.

#### 6.3.3.4	Application of different cell configurations

By default, the cells specified in 36.523-1 are defined with the full cell configuration, unless it is explicitly specified as either the broadcast only, minimum uplink, or virtual cell configuration. The full cell configuration is suitable for UE to start camping, establish RRC connection or hand over from another active cell.

The broadcast only cells identified as ‘DL only’ can be applied in some RRC measurement test cases to those neighbour cells which UE shall not camp on during the test case execution.

The capability of a minimum uplink cell is much weaker than a full cell, but stronger than a broadcast only cell in UL. This cell configuration identified as ‘min UL’ can be applied in the idle mode test cases to those neighbour cells which a conformant UE shall not camp on during the test case execution.

The virtual cell configuration can only be used in Carrier Aggregation test cases as an SCell which is not active, i.e. the UE has it configured via RRC signalling but does not need to send or receive anything in this SCell.

If a cell configuration is specified in a test case it shall remain unchanged throughout the test case specification. In addition, there shall not be any requirement in the test specification or test implementation for a cell reconfiguration from one of the above configurations to the other.

For the UE conformance test, the broadcast only cell configuration can be replaced and implemented with the minimum uplink or full cell configurations. The minimum uplink cell configuration can also be implemented with the full cell configuration. The replacements in the implementation have no impact on the test purposes and the test requirements. The implementation guidelines are referred to 36.523-3.

### 6.3.4	SCell configurations

For the purpose of System Simulator resource management for Carrier Aggregation testing the following types of SCell configurations are defined:

-	Active SCell: A cell that becomes an SCell at any point of time during the test case and which, while being an SCell, is activated.

-	Inactive SCell: A cell that becomes an SCell at any point of time during the test case but is never activated while being an SCell.

SCell activation is defined as sending an Activation/Deactivation MAC Control Element (ref. 36.321 cl. 6.1.3.8) to the UE to activate the SCell.

The SCell type does not depend on what the cell is used for in the test case while not being an SCell and does not directly relate to the cell type specified in section 6.3.3. In particular an Inactive SCell can still be used as full cell or broadcast only cell (e.g., for measurements to be performed on such a cell), and may be used for UE dedicated traffic while not acting as an SCell.

Also note that an Active SCell will not become an Inactive SCell if the SCell is deactivated via Activation/Deactivation MAC Control Element or after the sCellDeactivationTimer timer expires.

### 6.3.5	Satellite ephemeris information for NTN

#### 6.3.5.1	GSO scenario

The satellite ephemeris information of GSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 0

-	Eccentricity: 0

-	Latitude: 0

-	Longitude: 121.56076999

-	Altitude: 35786 km

-	Speed: 0 km/s

In case of multi-cell NTN GSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information is defined in Table 6.3.5.1-1 unless otherwise specified in a test case.

Table 6.3.5.1-1: Satellite ephemeris information in multi-cell NTN GSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX/Y/Z | nta-Common | Koffset |
| --- | --- | --- | --- | --- | --- | --- |
| First | -16976014 | 27636499 | 0 | 0 | 8243100 | 550 |
| Second | -23550424 | 22301093 | 0 | 0 | 8248322 | 550 |
| Third | -14502736 | 29010891 | 0 | 0 | 8248322 | 550 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 36.523-1 [18] TC 22.5.23 running in GSO scenario, Ncell 50 is the serving cell in the preamble so it uses the satellite ephemeris information of First Cell, while Ncell 62 uses the satellite ephemeris information of Second Cell. |  |  |  |  |  |  |

#### 6.3.5.2	NGSO scenario

The satellite ephemeris information of NGSO is defined in Table 4.4.3.3-21 for eMTC NTN and Table 8.1.4.3.3-10 for NB-IoT NTN with below assumption:

-	Inclination: 90

-	Eccentricity: 0

-	Longitude: 121.56076999

-	Altitude: 600 km

-	Speed: 7.56 km/s

The UE position defined in clause 4.13 is the projection position from one of the satellite information to the ground.

All eMTC NTN and NB-IoT NTN test cases defined in TS 36.523-1 [18], unless otherwise stated, shall use the first row of Table 6.3.5.2-1 as the ephemeris configuration of the serving cell, corresponding to zero Doppler and constant delay for NGSO scenario, keeping such information constant throughout the duration of each measurement in the test.

In case of multi-cell NTN NGSO scenario, a maximum of 3 NTN cells can be used. The satellite ephemeris information is defined in Table 6.3.5.2-1 unless otherwise specified in a test case, keeping such information constant throughout the duration of each measurement in the test.

Table 6.3.5.2-1: Satellite ephemeris information in multi-cell NTN NGSO scenario

| Cell type (Note 1) | positionX | positionY | positionZ | velocityVX | velocityVY | velocityVZ | k-Mac | k-Offset | Fd (Note 2) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First | -2550081 | 4151097 | 2263034 | 21970 | -48164 | 114109 | 3 | 5 | 0.013 |
| Second | -2654549 | 4386991 | 1594205 | 14581 | -34487 | 120182 | 4 | 8 | 19.83 |
| Third | -2401021 | 3830217 | 2907041 | 28879 | -61410 | 105774 | 4 | 8 | -19.95 |
| Note 1: First Cell is the cell defined as default serving cell in the test case at the end of the preamble. Second Cell and Third Cell are used in multi-cell test cases with Second Cell being defined as the 1st neighbour cell and Third Cell being defined as the 2nd neighbour cell. For example, in TS 36.523-1 [18] TC 22.5.23 running in NGSO scenario, Ncell 50 is the serving cell in the preamble so it uses the satellite ephemeris information of First Cell, while Ncell 62 uses the satellite ephemeris information of Second Cell. Note 2: Fd is the frequency Doppler in the service link, unit is ppm. The values are not broadcasted but resemble those that are calculated at the UE based on the corresponding cell satellite ephemeris information. They need to be considered for UL & DL transmission in SS side. |  |  |  |  |  |  |  |  |  |

### 6.3.6	Signalling test environment for Vertical UEs

#### 6.3.6.1	eMTC NTN

This clause defines the signalling test environment which applies to all signalling test cases executed for eMTC NTN UEs, unless otherwise specified. This is in addition to the common test environment defined in clause 4.14.1.

-	Test frequencies specified in Table 6.2.3.1-1d are used.

-	For NGSO scenario, UE prediction of satellite trajectory shall be deactivated in the pre-test conditions by any preconfigured means.

NOTE:	As constant ephemeris information in SIB31 is used for the NGSO scenario, the frequency Doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE will not update the Doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of timer T317 (ul-SyncValidityDuration-r17).

## 6.4	Generic procedures

This clause describes UE test states which can be used in the initial condition of many test cases defined in TS 36.523-1 [18] in addition to the states already specified in clause 4.5 of this specification.

### 6.4.1	Initial UE states and setup procedures

#### 6.4.1.1	Initial UE states and setup procedures

Figure 6.4.1.1-1: Initial UE states

In order that the UE can set up a call or session in E-UTRAN, there are a number of setup procedures to be undertaken in a hierarchical sequence to move between known states. The sequences are shown in figure 6.4.1.1-1 and the status of the relevant protocols in the UE in the different states are given in table 6.4.1.1-1. State 3 is defined in clause 4.5.1.

Table 6.4.1.1-1: Protocol state for each initial UE state

|  |  | RRC | ECM | EMM | ESM |
| --- | --- | --- | --- | --- | --- |
| State 5 | Dedicated RB established | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active N dedicated EPS bearer context(s) active |
| State 5A | DC MCG/SCG Dedicated RB established | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell 1 dedicated EPS bearer context(s) active on the PCell 1 dedicated EPS bearer context(s) active on the PSCell |
| State 5B | DC Split Dedicated RB established | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active on the PCell 1 dedicated EPS bearer context(s) active as a Split DRB on the PCell and the PSCell with UL transmission of PDCP SDUs on PSCell |
| State 6 | Loopback Activated | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active N dedicated EPS bearer context(s) active |
| State 6A | DC MCG/SCG DRB Loopback Activated | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active 1 dedicated EPS bearer context(s) active on the PCell 1 dedicated EPS bearer context(s) active on the PSCell |
| State 6B | DC Split DRB Loopback Activated | RRC_CONNECTED | ECM_CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active 1 dedicated EPS bearer context(s) active as a Split DRB on the PCell and the PSCell with UL transmission of PDCP SDUs on PSCell |

#### 6.4.1.2	Dedicated Bearer Establishment (to state 5)

##### 6.4.1.2.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Generic RB established (State 3).

##### 6.4.1.2.2	Definition of system information messages

The default system information messages are used.

##### 6.4.1.2.3	Procedure

The establishment of dedicated radio bearer connection is assumed to always be mobile terminated.

Table 6.4.1.2.3-1: Procedure for dedicated bearer establishment

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS configures new data radio bearer(s) and the associated dedicated EPS bearer context(s). | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer(s), associated with the dedicated EPS bearer context(s) in the NAS message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

##### 6.4.1.2.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7.

#### 6.4.1.2A	DC MCG/SCG Dedicated Bearer Establishment (to state 5A)

##### 6.4.1.2A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Generic RB established (State 3).

##### 6.4.1.2A.2	Definition of system information messages

The default system information messages are used.

##### 6.4.1.2A.3	Procedure

The establishment of MCG/SCG dedicated radio bearer connection is assumed to always be mobile terminated.

Table 6.4.1.2A.3-1: Procedure for MCG/SCG dedicated bearer establishment

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS configures new MCG data radio bearer and the associated dedicated EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new MCG data radio bearer, associated with the dedicated EPS bearer context in the NAS message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 4 | The SS adds a PSCell and configures new SCG data radio bearer and the associated dedicated EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new SCG data radio bearer(s), associated with the dedicated EPS bearer context in the NAS message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |

##### 6.4.1.2A.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the following exceptions:

Table 6.4.1.2A.4-1: RRCConnectionReconfiguration (Step 1, Table 6.4.1.2A.3-1)

Derivation Path: 36.508, Table 4.6.1-8 using conditions AM-DRB-ADD(1)

Table 6.4.1.2A.4-2: RRCConnectionReconfiguration (Step 4, Table 6.4.1.2A.3-1)

Derivation Path: 36.508, Table 4.6.1-8 using conditions PSCell_Add_SCG_DRB

#### 6.4.1.2B	DC Split Dedicated Bearer Establishment (to state 5B)

##### 6.4.1.2B.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Generic RB established (State 3).

##### 6.4.1.2B.2	Definition of system information messages

The default system information messages are used.

##### 6.4.1.2B.3	Procedure

The establishment of split dedicated radio bearer connection is assumed to always be mobile terminated.

Table 6.4.1.2B.3-1: Procedure for Dual Connectivity Split dedicated bearer establishment

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS configures new MCG data radio bearer and the associated dedicated EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new MCG data radio bearer, associated with the dedicated EPS bearer context in the NAS message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| 4 | The SS adds a PSCell and configures a split data radio bearer with UL transmission of PDCP SDUs on the PSCel. | <-- | RRC: RRCConnectionReconfiguration |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the split data radio bearer | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | Void | - | - |

##### 6.4.1.2B.4	Specific message contents

All specific message contents shall be referred to clause 4.6 and 4.7 with the following exceptions:

Table 6.4.1.2B.4-1: RRCConnectionReconfiguration (Step 1, Table 6.4.1.2A.3-1)

Derivation Path: 36.508, Table 4.6.1-8 using conditions AM-DRB-ADD(1)

Table 6.4.1.2B.4-2: RRCConnectionReconfiguration (Step 4, Table 6.4.1.2A.3-1)

Derivation Path: 36.508, Table 4.6.1-8 using conditions PSCell_Add_Split_DRB

#### 6.4.1.3	Loopback Activation (to state 6)

##### 6.4.1.3.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in Dedicated Radio Bearer Established (State 5).

##### 6.4.1.3.2	Definition of system information messages

The default system information messages are used.

##### 6.4.1.3.3	Procedure

Table 6.4.1.3.3-1: Procedure for loopback activation

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits an ACTIVATE RB TEST MODE message to activate UE radio bearer test mode procedure. | <-- | RRC: DLInformationTransfer TC: ACTIVATE RB TEST MODE |
| 2 | The UE transmits an ACTIVATE RB TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer TC: ACTIVATE RB TEST MODE COMPLETE |
| 3 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- | RRC: DLInformationTransfer TC: CLOSE UE TEST LOOP |
| 4 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated (State 6). | --> | RRC: ULInformationTransfer TC: CLOSE UE TEST LOOP COMPLETE |

##### 6.4.1.3.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A.

#### 6.4.1.3A	DC MCG/SCG DRB Loopback Activation (to state 6A)

##### 6.4.1.3A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in DC MCG/SCG Dedicated Radio Bearer Established (State 5A).

##### 6.4.1.3A.2	Definition of system information messages

The default system information messages are used.

##### 6.4.1.3A.3	Procedure

Same procedure as in sub-clause 6.4.1.3.3.

##### 6.4.1.3A.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A.

#### 6.4.1.3B	DC Split DRB Loopback Activation (to state 6B)

##### 6.4.1.3B.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 4.4, unless otherwise specified in the test case.

User Equipment:

-	The UE shall be in DC Split Dedicated Radio Bearer Established (State 5B).

##### 6.4.1.3B.2	Definition of system information messages

The default system information messages are used.

##### 6.4.1.3B.3	Procedure

Same procedure as in sub-clause 6.4.1.3.3.

##### 6.4.1.3B.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 4.7A.

### 6.4.2	Test procedures

#### 6.4.2.1	Introduction

This section defines test procedures which can be used within test procedure sequences for test steps where checking the UE state is needed.

For each test procedure,

-	at the start of the test procedure,

-	the System Simulator condition and the value of system information messages are the ones applicable in the test case referring to this test procedure, as they are after the execution of the test step immediately preceding the test step where the test procedure is used;

-	the initial UE condition is one indicated in the test case referring to this procedure, as it is after the execution of the test step immediately preceding the test step where the test procedure is used.

-	at the end of the test procedure,

-	the System Simulator condition after the test procedure execution is complete is the same as before it is started (this should not be changed by the test procedure).

#### 6.4.2.2	Test procedure to check RRC_IDLE state

This procedure aims at checking whether the UE is in RRC_IDLE on a certain cell of a test case or not.

Table 6.4.2.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 0 | The SS waits 1 second. | - | - | - | - |
| 1 | The SS sends RRC Paging message with UE S-TMSI on the cell(s) specified in the test case. | <-- | RRC: Paging (PCCH) | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest message on the cell specified in the test case ? | --> | RRC: RRCConnectionRequest |  | P |
| 3 | The SS transmits a RRCConnectionSetup message | <-- | RRC: RRCConnectionSetup | - | - |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetup Complete NAS: SERVICE REQUEST | - | - |
| 5 - 6 | Void | - | - | - | - |
| 7 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand | - | - |
| 8 | The UE transmits a SecurityModeComplete message. | --> | RRC: SecurityModeComplete | - | - |
| 9 | The SS transmits a RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) according to 4.8.2.2.1.1, | <-- | RRC: RRCConnectionReconfiguration | - | - |
| 10 | The UE transmits an RRCConnectionReconfiguration complete message. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 11 | The SS release the RRC connection. | <-- | RRC: RRCConnectionRelease | - | - |

#### 6.4.2.3	Test procedure to check RRC_CONNECTED state

Table 6.4.2.3-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS sends UECapabilityEnquiry message to the UE. | <-- | UECapabilityEnquiry | - | - |
| 2 | Check: Does the UE send a UECapabilityInformation message? | --> | UECapabilityInformation |  | P |

#### 6.4.2.4	Test procedure Paging (for NAS testing)

This procedure aims at checking whether the UE is in registered with a certain S-TMSI.

For the PS domain this procedure is identical to the procedure in 6.4.2.2 except that the S-TMSI as indicated in step 1 is the one explicitly specified in the test step calling this procedure.

For the CS domain the test procedure is FFS (i.e. it shall be avoided to make use of CS paging in the test case design).

#### 6.4.2.5	Test procedure for no response to paging (for NAS testing)

This procedure aims at checking that the UE ignores paging messages with a specified identity.

The procedure is defined in table 6.4.2.5-1.

Table 6.4.2.5-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 0 | The SS waits 1 second. | - | - | - | - |
| 1 | The SS transmits a paging message using the UE identity and the CN domain which are both specified in the referring test step, and on the cell which is specified in the referring test step. | <-- | Paging | - | - |
| 2 | Check: Does the UE send an RRCConnectionRequest message on the cell where the paging was transmitted within the next 3s? | --> | RRCConnectionRequest |  | F |

#### 6.4.2.6	Test procedure to check that a dedicated EPS bearer context is active (for NAS testing)

This procedure aims at checking that a dedicated EPS bearer context is active.

The procedure is defined in table 6.4.2.6-1.

Table 6.4.2.6-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS modifies existing data radio bearer(s) and the associated EPS bearer context(s) with Bearer QoS update. | <-- | RRC: RRCConnectionReconfiguration NAS: MODIFY EPS BEARER CONTEXT REQUEST | - | - |
| 2 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the modification of the existing data radio bearer(s), associated with the EPS bearer context(s) in the NAS message. | --> | RRC: RRCConnectionReconfigurationComplete | - | - |
| 3 | Check: Does the UE transmit a MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer NAS: MODIFY EPS BEARER CONTEXT ACCEPT |  | P |

#### 6.4.2.7	Test procedure to check that UE is camped on a new E-UTRAN cell

This procedure aims at checking whether the UE is camping on a new E-UTRAN cell with different TAI of a test case or not.

The procedure is defined in table 6.4.2.7-1.

Table 6.4.2.7-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCConnectionRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest | - | - |
| 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 3 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | Check: Does the UE send a TRACKING AREA UPDATE COMPLETE on the cell specified in the test case? | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | P |
| 6 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |

NOTE 1:	The periodic tracking area updating timer T3412 is deactivated by default during the attach procedure (TS 36.508 clause 4.7.2).

NOTE 2:	The SS does not initiate authentication and NAS SECURITY MODE COMMAND are not performed (reuse of keys allocated during the attach procedure).

#### 6.4.2.7A	Test procedure to check that UE is camped on E-UTRAN cell upon mobility from another RAT

This procedure aims at checking whether the UE is camping on a E-UTRAN cell upon mobility from another RAT after Inter RAT reselection.

The procedure if PDP context was established before inter RAT reselection, is defined in table 6.4.2.7A-1.

Table 6.4.2.7A-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a RRCConnectionRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest | - | - |
| 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 3 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security (mapped security context) (Note 1). | <-- | RRC: DLInformationTransfer NAS: SECURITY MODE COMMAND | - | - |
| 5 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer NAS: SECURITY MODE COMPLETE | - | - |
| 6 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 7 | Check: Does the UE send a TRACKING AREA UPDATE COMPLETE on the cell specified in the test case? | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | P |
| 8 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| Note 1: Step 4 and 5 are executed only when UE and SS does not have a native security context. |  |  |  |  |  |

The procedure if PDP context was not established before inter RAT reselection, is defined in table 6.4.2.7A-2.

Table 6.4.2.7A-2: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: For Rel-8 and Rel-9 steps 1 to 6 are optional and depend on UE implementation. A Rel-10 UE directly starts attach procedure from step 6. | - | - | - | - |
| 1 | The UE transmits a RRCConnectionRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest | - | - |
| 2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup | - | - |
| 3 | The UE transmits a RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | The SS transmits a TRACKING AREA UPDATE REJECT message with cause #40 (No EPS bearer context activated) to force attach the UE | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE REJECT |  |  |
| - | EXCEPTION: Step 5 describes the behaviour that depends on UE behaviour (Note 1). | - | - | - | - |
| 5 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |
| - | EXCEPTION: Step 6 describes a behaviour which depends on the UE capability | - | - | - | - |
| 6 | IF NOT pc_Auto_PDN_Connectivity, the user initiates an attach by MMI or by AT command. (Note 2) | - | - | - | - |
| 7-22 | Steps 2 to 17 of the generic UE Registration procedure (4.5.2.3-1) are executed to successfully complete the Attach procedure. | - | - | - | - |
| Note 1: SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. Note 2: The request is assumed to be triggered by AT command AT+CGDCONT=1,"IP" followed by AT+CGACT=1 |  |  |  |  |  |

6.4.2.7A.1	Specific message contents

Table 6.4.2.7A.1-1: TRACKING AREA UPDATE REQUEST (Step 3, table 6.4.2.7A-1, 6.4.2.7A-2)

| Derivation Path: 36.508 clause 4.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS update type |  |  |  |
| EPS update type Value | '000'B | TA updating | TA_only |
|  | '001'B or ‘010’B | Combined TA/LA updating or combined TA/LA updating with IMSI attach | No_LAU_CombinedRAU_GERAN_UTRA |
|  | ‘010’B | Combined TA/LA updating with IMSI attach | LAU_CombinedRAU_GERAN_UTRA |
| "Active" flag | Any allowed value |  |  |
| NAS key set identifier |  |  |  |
| NAS key set identifier | The valid NAS key set identifier of the UE |  |  |
| TSC | '0'B | native security context (for KSIASME) | Native_Secuirty_Context |
|  | '1'B | mapped security context (for KSISGSN) | Mapped_Secuirty_Context |
| GPRS ciphering key sequence number | Not present or any allowed value |  |  |
| Old P-TMSI signature | Any allowed value |  |  |
| Additional GUTI | Not present or any allowed value |  |  |
| NonceUE | Not present or any allowed value |  |  |
| UE radio capability information update needed | Not present or any allowed value |  |  |
| EPS bearer context status | Not present or (octet 3 = ('00100000'B or '01100000'B) and octet 4 = '00000000'B) |  | Bearer_Established |
|  | Not present or (octet 3 = '00000000'B and octet 4 = '00000000'B) |  | No_Bearer_Established |

| Condition | Explanation |
| --- | --- |
| TA_only | See the definition below table 4.7.2-24. |
| combined_TA_LA | See the definition below table 4.7.2-24. |
| Native_Secuirty_Context | UE has performed AKA on the EUTRA Cell |
| Mapped_Secuirty_Context | UE has performed AKA on the UTRA/GERAN Cell |
| No_LAU_CombinedRAU_GERAN_UTRA | UE has not performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |
| LAU_CombinedRAU_GERAN_UTRA | UE has performed location area update procedure or a combined routing area update procedure in A/Gb or Iu mode |
| Bearer_Established | Bearer is Established when UE earlier camped at UTRAN/GERAN cell or UE previously successfully camped in EUTRA cell. |
| No_Bearer_Established | Bearer is not Established when UE earlier camped at UTRAN/GERAN cell and UE did not previously camped in EUTRA cell. |

This message is sent integrity protected when a valid security context exists and without integrity protection otherwise.

#### 6.4.2.7B	Test procedure to check that UE is camped on a new E-UTRAN cell / UP CIoT

This procedure aims at checking whether the UE is camping on a new E-UTRAN cell with different TAI of a test case or not.

The procedure is defined in table 6.4.2.7B-1.

Table 6.4.2.7B-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCConnectionResumeRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionResumeRequest | - | - |
| 2 | SS transmit an RRCConnectionResume message. | <-- | RRC: RRCConnectionResume | - | - |
| 3 | The UE transmits an RRCConnectionResumeComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionResumeComplete NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | Check: Does the UE send a TRACKING AREA UPDATE COMPLETE on the cell specified in the test case? | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | P |
| 6 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease | - | - |

Table 6.4.2.7B-2: RRCConnectionRelease (Step 6)

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |
| releaseCause | other |  |  |
| redirectedCarrierInfo | Not present |  |  |
| idleModeMobilityControlInfo | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 6.4.2.8	Test procedure to check that UE is camped on a new UTRAN cell

This procedure aims at checking whether the UE is camping on a new UTRAN cell of a test case or not.

The procedure is defined in table 6.4.2.8-1.

Table 6.4.2.8-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a RRC CONNECTION REQUEST message on the cell specified in the test case. | --> | RRC: RRC CONNECTION REQUEST | - | - |
| 2 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC: RRC CONNECTION SETUP | - | - |
| 3 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC: RRC CONNECTION SETUP COMPLETE | - | - |
| 4 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | RRC: INITIAL DIRECT TRANSFER NAS: ROUTING AREA UPDATE REQUEST | - | - |
| 5 | The SS transmits a SECURITY MODE COMMAND message. | <-- | RRC: SECURITY MODE COMMAND | - | - |
| 6 | The UE transmits a SECURITY MODE COMPLETE message. | --> | RRC: SECURITY MODE COMPLETE | - | - |
| 7 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | RRC: DOWNLINK DIRECT TRANSFER NAS: ROUTING AREA UPDATE ACCEPT | - | - |
| 8 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | RRC: UPLINK DIRECT TRANSFER NAS: ROUTING AREA UPDATE COMPLETE | - | - |
| 9 | The SS transmits a RRC CONNECTION RELEASE message. | <-- | RRC: RRC CONNECTION RELEASE | - | - |
| 10 | Check: Does the UE transmit an RRC CONNECTION RELEASE COMPLETE message on the cell specified in the test? | --> | RRC: RRC CONNECTION RELEASE COMPLETE | - | P |

NOTE:	The TS 34.108 [5] and TS 34.123-1 [7] use Network Mode of Operation I as default, for this reason a combined MM/GMM procedure is performed.

#### 6.4.2.9	Test procedure to check that UE is camped on a new GERAN cell

This procedure aims at checking whether the UE is camping on a new GERAN cell of a test case or not.

The procedure is defined in table 6.4.2.9-1.

Table 6.4.2.9-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE send a CHANNEL REQUEST message on the cell(s) specified in the test case? | --> | CHANNEL REQUEST | - | - |
| 2 | An uplink TBF is established in order to allow the UE to transmit a ROUTING AREA UPDATE REQUEST message signalling. | - | - | - | - |
| 3 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATING REQUEST | - | - |
| 4 | A downlink TBF is established by the SS in order to transmit the AUTHENTICATION AND CIPHERING REQUEST message | - | - | - | - |
| 5 | The SS transmits an AUTHENTICATION AND CIPHERING REQUEST message. | <-- | AUTHENTICATION AND CIPHERING REQUEST | - | - |
| 6 | An uplink TBF is established in order to allow the UE to transmit an AUTHENTICATION AND CIPHERING RESPONSE message | - | - | - | - |
| 7 | The UE transmits an AUTHENTICATION AND CIPHERING RESPONSE message. | --> | AUTHENTICATION AND CIPHERING RESPONSE | - | - |
| 8 | A downlink TBF is established by the SS in order to transmit the ROUTING AREA UPDATE ACCEPT message | - | - | - | - |
| 9 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATING ACCEPT | - | - |
| 10 | An uplink TBF is established in order to allow the UE to transmit a ROUTING AREA UPDATE COMPLETE message |  |  | - | - |
| 11 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATING COMPLETE | - | P |

NOTE:	The TS 51.010-1 [25] uses Network Mode of Operation I as default, for this reason a combined MM/GMM procedure is performed.

#### 6.4.2.10	Test procedure to check that UE performs tracking area updating procedure without ISR and security reconfiguration after successful completion of handover from UTRA

This procedure aims at checking whether the UE performs tracking area updating procedure where ISR is not activated and security reconfiguration after successful completion of handover from UTRA.

The procedure is defined in table 6.4.2.10-1.

Table 6.4.2.10-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an ULInformationTransfer message on the cell specified in the test case. This message includes a TRACKING AREA UPDATE REQUEST message. | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 2 | The SS transmits a DLInformationTransfer message on the cell specified in the test case. This message includes a TRACKING AREA UPDATE ACCEPT message. | <-- | RRC:DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 3 | The UE transmits an ULInformationTransfer message on the cell specified in the test case. This message includes a TRACKING AREA UPDATE COMPLETE message. | --> | RRC:ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE | - | - |
| - | EXCEPTION: Steps 4a1 to 4a2 describe behaviour that depends on the IE included in TRACKING AREA UPDATE REQUEST in step 1. | - | - | - | - |
| 4a1 | IF TRACKING AREA UPDATE REQUEST transmitted in step 1 does not contain a valid KSIASME THEN the SS transmits a DLInformationTransfer message on the cell specified in the test case. This message includes an AUTHENTICATION REQUEST message. | <-- | RRC:DLInformationTransfer NAS: AUTHENTICATION REQUEST | - | - |
| 4a2 | The UE transmits an ULInformationTransfer message on the cell specified in the test case. This message includes an AUTHENTICATION RESPONSE message. | --> | RRC:ULInformationTransfer NAS: AUTHENTICATION RESPONSE | - | - |
| 5 | The SS transmits a DLInformationTransfer message on the cell specified in the test case. This message includes a SECURITY MODE COMMAND message to take the native EPS security context into use. | <-- | RRC:DLInformationTransfer NAS: SECURITY MODE COMMAND | - | - |
| 6 | The UE transmits an ULInformationTransfer message on the cell specified in the test case. This message includes a SECURITY MODE COMPLETE message. | --> | RRC:ULInformationTransfer NAS: SECURITY MODE COMPLETE | - | - |
| 7 | The SS transmits an RRCConnectionReconfiguration message to perform intra cell handover and security reconfiguration on the cell specified in the test case. | <-- | RRC:RRCConnectionReconfiguration | - | - |
| 8 | Check: Does the UE transmit an RRCConnectionReconfigurationComplete message on the cell specified in the test case? | --> | RRC:RRCConnectionReconfigurationComplete | - | P |

Table 6.4.2.10-2: Message TRACKING AREA UPDATE REQUEST (step 1, Table 6.4.2.10-1)

| Derivation Path: 36.508 clause 4.7.2-27 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NAS key set identifier |  |  |  |
| NAS key set identifier | The valid NAS key set identifier of the UE | mapped security context (for KSISGSN) |  |
| TSC | 1 |  |  |
| Non-current native NAS key set identifier |  |  |  |
| NAS key set identifier | The valid NAS key set identifier of the UE | native security context (for KSIASME) |  |
| TSC | 0 |  |  |
| Old P-TMSI signature | any allowed value |  |  |
| Additional GUTI | any allowed value |  |  |

Table 6.4.2.10-3: RRCConnectionReconfiguration (step 7, Table 6.4.2.10-1)

Derivation Path: 36.508, Table 4.6.1-8

Table 6.4.2.10-4: SecurityConfigHO (IE in RRCConnectionReconfiguration, Table 6.4.2.10-3)

| Derivation Path: 36.508, Table 4.6.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SecurityConfigHO ::= SEQUENCE { |  |  |  |
| handoverType CHOICE { |  |  |  |
| intraLTE SEQUENCE { |  |  |  |
| keyChangeIndicator | TRUE |  |  |
| nextHopChainingCount | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

### 6.4.3	Reference test procedures for TTCN development

This clause describes reference test procedures for the purpose of TTCN development.

#### 6.4.3.1	UE triggered establishment of a dedicated EPS bearer context

Table 6.4.3.1-1: Test procedure sequence

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to request bearer resource allocation of dedicated EPS bearer associated with first PDN connectivity. (see Note 1) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a7 describe behaviour that depends on RRC state; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 2a3 | The UE transmits an RRCConnectionSetupComplete including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 2a4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 2a5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 2a6 | The SS transmits an RRCConnectionReconfiguration message to configure data radio bearer(s) associated with the existing EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) where n and m are the number of DRBs associated with existing EPS bearer contexts and configured respectively with RLC-AM and RLC-UM. | <-- | RRC: RRCConnectionReconfiguration |
| 2a7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ULInformationTransfer message including the BEARER RESOURCE ALLOCATION REQUEST message. | --> | RRC: ULInformationTransfer NAS: BEARER RESOURCE ALLOCATION REQUEST |
| 4 | The SS transmits an RRCConnectionReconfiguration message including the ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST message to establish the dedicated EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | The UE transmits an ULInformationTransfer message including the ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEDICATED EPS BEARER CONTEXT ACCEPT |
| Note 1: The request is assumed to be triggered by AT command +CGDSCONT, and +CGACT (activated). Note 2: If UE sends IP related data this shall be handled by the SS. |  |  |  |

#### 6.4.3.2	UE triggered establishment of a default EPS bearer context associated with an additional PDN

Table 6.4.3.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to request connectivity to an additional PDN. (see Note 1) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a7 describe behaviour that depends on RRC state; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 2a3 | The UE transmits an RRCConnectionSetupComplete including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 2a4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 2a5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 2a6 | The SS transmits an RRCConnectionReconfiguration message to configure data radio bearer(s) associated with the existing EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) where n and m are the number of DRBs associated with existing EPS bearer contexts and configured respectively with RLC-AM and RLC-UM. | <-- | RRC: RRCConnectionReconfiguration |
| 2a7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ULInformationTransfer message including the PDN CONNECTIVITY REQUEST message. | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST |
| 4 | The SS transmits an RRCConnectionReconfiguration message including the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message to establish the default EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| - | EXCEPTION: In parallel to the event described in step 6 below, the generic procedure for IP address allocation in the U-plane specified in TS 36.508 subclause 4.5A.1 takes place performing IP address allocation in the U-plane. | - | - |
| 6 | The UE transmits an ULInformationTransfer message including the ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |
| Note 1: The request is assumed to be triggered by AT command +CGDCONT and +CGACT (activated). Note 2: If UE sends IP related data this shall be handled by the SS. |  |  |  |

Table 6.4.3.2-2: Message PDN CONNECTIVITY REQUEST (step 3, Table 6.4.3.2-1)

| Derivation Path: TS 36.508 Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS bearer identity | 0000 | No EPS bearer identity assigned |  |
| Procedure transaction identity | PTI-1 | UE assigns a particular PTI not yet used between 1 and 254 |  |
| ESM information transfer flag | Not present | This IE is only used during an attach procedure. |  |
| Access point name | APN-1(New PDN name) | The requested PDN is different from default PDN |  |

#### 6.4.3.3	UE triggered modification of an EPS bearer context

Table 6.4.3.3-1: Test procedure sequence

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to request bearer resource modification of EPS bearer associated with first PDN connectivity. (see Note 1) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a7 describe behaviour that depends on RRC state; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 2a3 | The UE transmits an RRCConnectionSetupComplete including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 2a4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 2a5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 2a6 | The SS transmits an RRCConnectionReconfiguration message to configure data radio bearer(s) associated with the existing EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) where n and m are the number of DRBs associated with existing EPS bearer contexts and configured respectively with RLC-AM and RLC-UM. | <-- | RRC: RRCConnectionReconfiguration |
| 2a7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ULInformationTransfer message including the BEARER RESOURCE MODIFICATION REQUEST message. | --> | RRC: ULInformationTransfer NAS: BEARER RESOURCE MODIFICATION REQUEST |
| 4 | The SS transmits an RRCConnectionReconfiguration message including the MODIFY EPS BEARER CONTEXT REQUEST message to modify the EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: MODIFY EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | Check: Does the UE transmit an ULInformationTransfer message including the MODIFY EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer NAS: MODIFY EPS BEARER CONTEXT ACCEPT |
| Note 1: The request is assumed to be triggered by AT command +CGCMOD. Note 2: If UE sends IP related data this shall be handled by the SS. |  |  |  |

#### 6.4.3.4	UE triggered deletion of an EPS bearer context

Table 6.4.3.4-1: Test procedure sequence

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to request bearer resource release of dedicated EPS bearer associated with first PDN connectivity. (see Note 1) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a7 describe behaviour that depends on RRC state; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 2a3 | The UE transmits an RRCConnectionSetupComplete including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 2a4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 2a5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 2a6 | The SS transmits an RRCConnectionReconfiguration message to configure data radio bearer(s) associated with the existing EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) where n and m are the number of DRBs associated with existing EPS bearer contexts and configured respectively with RLC-AM and RLC-UM. | <-- | RRC: RRCConnectionReconfiguration |
| 2a7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ULInformationTransfer message including the BEARER RESOURCE MODIFICATION REQUEST message. | --> | RRC: ULInformationTransfer NAS: BEARER RESOURCE MODIFICATION REQUEST |
| 4 | The SS transmits an RRCConnectionReconfiguration message including the DEACTIVATE EPS BEARER CONTEXT REQUEST message to delete EPS bearer context. | <-- | RRC: RRCConnectionReconfiguration NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST |
| 5 | The UE transmits an RRCConnectionReconfigurationComplete message | --> | RRC: RRCConnectionReconfigurationComplete |
| 6 | The UE transmits an ULInformationTransfer message including the DEACTIVATE EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT |
| Note 1: The request is assumed to be triggered by AT command +CGACT (deactivated). Note 2: If UE sends IP related data this shall be handled by the SS. |  |  |  |

#### 6.4.3.5	UE triggered CS call

Table 6.4.3.5-1: Test procedure sequence

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to originate CS call. (see Note 1) | - | - |
| - | EXCEPTION: Steps 2a1 to 2b1 describe behaviour that depends on RRC state; the "lower case letter" identifies a step sequence that takes place depending on RRC state. | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 2a3 | The UE transmits an RRCConnectionSetupComplete including the EXTENDED SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: EXTENDED SERVICE REQUEST |
| 2b1 | ELSE The UE transmits an ULInformationTransfer message including the EXTENDED SERVICE REQUEST message. | --> | RRC: ULInformationTransfer NAS: EXTENDED SERVICE REQUEST |
| Note 1: The request is assumed to be triggered by AT command D. |  |  |  |

#### 6.4.3.6	UE triggered MO SMS over SGs

Table 6.4.3.6-1: Test procedure sequence

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Cause the UE to originate SMS message. (see Note 1) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a7 describe behaviour that depends on RRC state; the "lower case letter" identifies a step sequence that takes place if the UE is in RRC_IDLE state. | - | - |
| 2a1 | IF the UE is in RRC_IDLE state THEN the UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 2a3 | The UE transmits an RRCConnectionSetupComplete including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 2a4 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 2a5 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 2a6 | The SS transmits an RRCConnectionReconfiguration message to configure data radio bearer(s) associated with the existing EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(n, m) where n and m are the number of DRBs associated with existing EPS bearer contexts and configured respectively with RLC-AM and RLC-UM. | <-- | RRC: RRCConnectionReconfiguration |
| 2a7 | The UE transmits an RRCConnectionReconfigurationComplete message. | --> | RRC: RRCConnectionReconfigurationComplete |
| 3 | The UE transmits an ULInformationTransfer message including the UPLINK NAS TRANSPORT message to send the SMS message. | --> | RRC: ULInformationTransfer NAS: UPLINK NAS TRANSPORT |
| 4 | The SS transmits a DLInformationTransfer message including the DOWNLINK NAS TRANSPORT message to acknowledge receipt of the SMS message. | <-- | RRC: DLInformationTransfer NAS: DOWNLINK NAS TRANSPORT |
| 5 | The SS transmits a DLInformationTransfer message including the DOWNLINK NAS TRANSPORT message to send the delivery report. | <-- | RRC: DLInformationTransfer NAS: DOWNLINK NAS TRANSPORT |
| 6 | The UE transmits an ULInformationTransfer message including the UPLINK NAS TRANSPORT message to acknowledge receipt of the delivery report. | --> | RRC: ULInformationTransfer NAS: UPLINK NAS TRANSPORT |
| Note 1: The request is assumed to be triggered by AT command +CMGW and +CMSS. |  |  |  |

#### 6.4.3.7	CS fallback to UTRAN procedures (LAI of UTRAN cell same as the LAI received in combined Attach procedure in EUTRA cell)

All procedures specified in this clause are referred to the UTRA target cell where the UE has been redirected or handed over from the EUTRA cell after a CS call requested. The default message contents are found in TS 34.108 [5], clause 9.

The procedures in 6.4.3.7.1 – 6.4.3.7.4 are applied if the UE supports (EUTRA) RRC connection release with redirection and Multi Cell System Information to UTRAN. The procedures in 6.4.3.7.5 - 6.4.3.7.7 are applied if the UE supports PS HO from EUTRA to UTRAN.

##### 6.4.3.7.1	CS fallback to UTRAN with redirection / MT call (PS bearers not established)

Table 6.4.3.7.1-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRC CONNECTION REQUEST message. | --> | RRC CONNECTION REQUEST | - | - |
| 2 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 3 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| - | EXCEPTION: In parallel to the events described in step 4 to 15 the step specified in Table 6.4.3.7.1-2 takes place. | - | - | - | - |
| 4 | Check: Does the UE transmit a PAGING RESPONSE message? | --> | PAGING RESPONSE | - | P |
| 5-15 | Steps 7 to 17 of the generic test procedure in TS 34.108 [5] subclause 7.2.3.1.3 are performed using the UTRA reference radio bearer parameters and combination "UTRA Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Mobile terminating CS call is established. | - | - | - | - |
| 16 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. See Note 1. | <-- | SECURITY MODE COMMAND | - | - |
| 17 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 18 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 19 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| Note 1: A real network will initiate the security mode command procedure for the PS domain immediately after receiving the ROUTING AREA UPDATE REQUEST, but in this test procedure it was chosen to complete the procedure for CS domain first, in order to avoid the possibility of a security mode command procedure running in parallel with another RRC procedure. |  |  |  |  |  |

Table 6.4.3.7.1-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

##### 6.4.3.7.2	CS fallback to UTRAN with redirection / MO call (PS bearers not established)

Table 6.4.3.7.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRC CONNECTION REQUEST message. | --> | RRC CONNECTION REQUEST | - | - |
| 2 | The SS transmits an RRC CONNECTION SETUP message. | <-- | RRC CONNECTION SETUP | - | - |
| 3 | The UE transmits an RRC CONNECTION SETUP COMPLETE message. | --> | RRC CONNECTION SETUP COMPLETE | - | - |
| - | EXCEPTION: In parallel to the events described in step 4 to 15 the step specified in Table 6.4.3.7.2-2 takes place. | - | - | - | - |
| 4 | Check: Does the UE transmit a CM SERVICE REQUEST message? | --> | CM SERVICE REQUEST | - | P |
| 5-15 | Steps 6 to 16 of the generic test procedure in TS 34.108 [5] subclause 7.2.3.2.3 are performed using the UTRA reference radio bearer parameters and combination "UTRA Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Mobile originating CS call is established. | - | - | - | - |
| 16 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. See Note 1. | <-- | SECURITY MODE COMMAND | - | - |
| 17 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 18 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 19 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE | - | - |
| Note 1: A real network will initiate the security mode command procedure for the PS domain immediately after receiving the ROUTING AREA UPDATE REQUEST, but in this test procedure it was chosen to complete the procedure for CS domain first, in order to avoid the possibility of a security mode command procedure running in parallel with another RRC procedure. |  |  |  |  |  |

Table 6.4.3.7.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

##### 6.4.3.7.3	CS fallback to UTRAN with redirection / MT call (PS bearer established)

Table 6.4.3.7.3-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-19 | Steps 1-19 of table 6.4.3.7.1-1. NOTE: Mobile terminating CS call is established and Routing Area Update procedure is performed |  |  | - | - |
| 20 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | - | P |
| 21 | SS transmits a RADIO BEARER SETUP message, using the UTRA reference radio bearer parameters and combination "UTRA PS RB + Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Mobile originating packet switched session is established. | <-- | RADIO BEARER SETUP | - | - |
| 22 | The UE transmits a RADIO BEARER SETUP COMPLETE message | --> | RADIO BEARER SETUP COMPLETE | - | - |

##### 6.4.3.7.4	CS fallback to UTRAN with redirection / MO call (PS bearer established)

Table 6.4.3.7.4-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1-19 | Steps 1-19 of table 6.4.3.7.2-1. NOTE: Mobile originating CS call is set up and Routing Area Update procedure is performed | - | - | - | - |
| 20 | Check: Does the UE transmit a SERVICE REQUEST message? | --> | SERVICE REQUEST | - | P |
| 21 | SS transmits a RADIO BEARER SETUP message, using the UTRA reference radio bearer parameters and combination "UTRA PS RB + Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Mobile originating packet switched session is established. | <-- | RADIO BEARER SETUP | - | - |
| 22 | The UE transmits a RADIO BEARER SETUP COMPLETE message | --> | RADIO BEARER SETUP COMPLETE | - | - |

##### 6.4.3.7.5	CS fallback to UTRAN with Handover / MT call

Table 6.4.3.7.5-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message? | --> | HANDOVER TO UTRAN COMPLETE | - | P |
| - | EXCEPTION: In parallel to the events described in step 2a1 to 9 the steps specified in table 6.4.3.7.5-2 takes place. | - | - | - | - |
| - | Exception: Steps 2a1 to 2a10 and 2b1 to 2b9 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. Note: One sequence of steps 2a1 to 2a10 or 2b1 to 2b9 are executed | - | - | - | - |
| 2a1 | IF the UE does not determine the LAI of the new UTRAN cell, THEN Check: Does the UE transmit a LOCATION UPDATING REQUEST message? | --> | LOCATION UPDATING REQUEST | - | P |
| 2a2 | The SS transmits AUTHENTICATION REQUEST | <-- | AUTHENTICATION REQUEST | - | - |
| 2a3 | The UE transmits AUTHENTICATION RESPONSE | --> | AUTHENTICATION RESPONSE | - | - |
| 2a4 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a5 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a6 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 2a7 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a8 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a9 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2a10 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
|  | Exception: Steps 2b1 to 2b4 takes place if pc_UMI_ProcNeeded_DuringCSFB |  |  |  |  |
| 2b1 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2b2 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2b3 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2b4 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 2b5 | Check: Does the UE transmit a PAGING RESPONSE? | --> | PAGING RESPONSE | - | P |
| 2b6 | The SS transmits AUTHENTICATION REQUEST | <-- | AUTHENTICATION REQUEST | - | - |
| 2b7 | The UE transmits AUTHENTICATION RESPONSE | --> | AUTHENTICATION RESPONSE | - | - |
| 2b8 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2b9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 3-9 | Steps 11 to 17 of the generic test procedure in TS 34.108 subclause 7.2.3.1.3 are performed using the UTRA reference radio bearer parameters and combination "UTRA PS RB + Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Mobile terminating CS call is established. | - | - | - | - |
| 10 | The SS transmits a ROUTING AREA UPDATE ACCEPT. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 11 | The UE sends ROUTING AREA UPDATE COMPLETE. | --> | ROUTING AREA UPDATE COMPLETE | - | - |

Table 6.4.3.7.5-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

###### 6.4.3.7.5.1	Specific message contents

Table 6.4.3.7.5.1-1: Void

Table 6.4.3.7.5.1-2: LOCATION UPDATING ACCEPT (step 2a6 of table 6.4.3.7.5-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

##### 6.4.3.7.6	CS fallback to UTRAN with Handover / MO call

Table 6.4.3.7.6-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message? | --> | HANDOVER TO UTRAN COMPLETE | - | P |
| - | EXCEPTION: In parallel to the events described in step 2a1 to 9 the steps specified in table 6.4.3.7.6-2 takes place. | - | - | - | - |
| - | Exception: Steps 2a1 to 2a12 and 2b1 to 2b9 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. Note: One sequence of steps 2a1 to 2a12 or 2b1 to 2b9 are executed | - | - | - | - |
| 2a1 | IF the UE does not determine the LAI of the new UTRAN cell, THEN Check: Does the UE transmit a LOCATION UPDATING REQUEST message? | --> | LOCATION UPDATING REQUEST | - | P |
| 2a2 | The SS transmits AUTHENTICATION REQUEST | <-- | AUTHENTICATION REQUEST | - | - |
| 2a3 | The UE transmits AUTHENTICATION RESPONSE | --> | AUTHENTICATION RESPONSE | - | - |
| 2a4 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a5 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a6 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 2a7 | Check: Does the UE transmit a CM SERVICE REQUEST? | --> | CM SERVICE REQUEST | - | P |
| 2a8 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a10 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2a11 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 2a12 | The SS transmits a CM SERVICE ACCEPT message. | <-- | CM SERVICE ACCEPT | - | - |
|  | Exception: Steps 2b1 to 2b4 takes place if pc_UMI_ProcNeeded_DuringCSFB |  |  |  |  |
| 2b1 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2b2 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2b3 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2b4 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 2b5 | Check: Does the UE transmit a CM SERVICE REQUEST? | --> | CM SERVICE REQUEST | - | P |
| 2b6 | The SS transmits AUTHENTICATION REQUEST | <-- | AUTHENTICATION REQUEST | - | - |
| 2b7 | The UE transmits AUTHENTICATION RESPONSE | --> | AUTHENTICATION RESPONSE | - | - |
| 2b8 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2b9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 3 | The UE transmits a SETUP message. | --> | SETUP | - | - |
| 4-9 | Steps 11 to 16 of the generic test procedure in TS 34.108 subclause 7.2.3.2.3 are performed using the UTRA reference radio bearer parameters and combination "UTRA PS RB + Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Mobile originating CS call is established. | - | - | - | - |
| 10 | The SS transmits a ROUTING AREA UPDATE ACCEPT. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 11 | The UE sends ROUTING AREA UPDATE COMPLETE. | --> | ROUTING AREA UPDATE COMPLETE | - | - |

Table 6.4.3.7.6-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

###### 6.4.3.7.6.1	Specific message contents

None.

Table 6.4.3.7.6.1-1: LOCATION UPDATING ACCEPT (step 2a6 of table 6.4.3.7.6-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

##### 6.4.3.7.7	CS fallback to UTRAN with Handover / emergency call

Table 6.4.3.7.7-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message? | --> | HANDOVER TO UTRAN COMPLETE | - | P |
| - | EXCEPTION: In parallel to the events described in step 2a1 to 9 the steps specified in table 6.4.3.7.7-2 takes place. | - | - | - | - |
| - | Exception: Steps 2a1 to 2a12 and 2b1 to 2b9 describe behaviour that depends on the UE capability; the "lower case letter" identifies a step sequence that takes place if a capability is supported. Note: One sequence of steps 2a1 to 2a12 or 2b1 to 2b9 are executed | - | - | - | - |
| 2a1 | IF the UE does not determine the LAI of the new UTRAN cell, THEN Check: Does the UE transmit a LOCATION UPDATING REQUEST message? | --> | LOCATION UPDATING REQUEST | - | P |
| 2a2 | The SS transmits AUTHENTICATION REQUEST | <-- | AUTHENTICATION REQUEST | - | - |
| 2a3 | The UE transmits AUTHENTICATION RESPONSE | --> | AUTHENTICATION RESPONSE | - | - |
| 2a4 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a5 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a6 | The SS transmits a LOCATION UPDATING ACCEPT message. | <-- | LOCATION UPDATING ACCEPT | - | - |
| 2a7 | Check: Does the UE transmit a CM SERVICE REQUEST? | --> | CM SERVICE REQUEST | - | P |
| 2a8 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2a9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2a10 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2a11 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 2a12 | The SS transmits a CM SERVICE ACCEPT message. | <-- | CM SERVICE ACCEPT | - | - |
|  | Exception: Steps 2b1 to 2b4 takes place if pc_UMI_ProcNeeded_DuringCSFB |  |  |  |  |
| 2b1 | The SS transmits a SECURITY MODE COMMAND message for the PS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2b2 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 2b3 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 2b4 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 2b5 | Check: Does the UE transmit a CM SERVICE REQUEST? | --> | CM SERVICE REQUEST | - | P |
| 2b6 | The SS transmits AUTHENTICATION REQUEST | <-- | AUTHENTICATION REQUEST | - | - |
| 2b7 | The UE transmits AUTHENTICATION RESPONSE | --> | AUTHENTICATION RESPONSE | - | - |
| 2b8 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 2b9 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 3 | The UE transmits an EMERGENCY SETUP message. | --> | EMERGENCY SETUP | - | - |
| 4-9 | Steps 12 to 17 of the test procedure in TS 34.123-1 [7] subclause 13.2.1.1 are performed using the UTRA reference radio bearer parameters and combination "UTRA PS RB + Speech" according to subclause 4.8.3 and Table 4.8.3-1. NOTE: Emergency call is established.. | - | - | - | - |
| 10 | The SS transmits a ROUTING AREA UPDATE ACCEPT. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 11 | The UE sends ROUTING AREA UPDATE COMPLETE. | --> | ROUTING AREA UPDATE COMPLETE | - | - |

Table 6.4.3.7.7-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

###### 6.4.3.7.7.1	Specific message contents

UTRAN MOBILITY INFORMATION (step 2a10 & 2b1 of table 6.4.3.7.7-1): same as Table 6.4.3.7.5.1-1

Table 6.4.3.7.7.1-1: LOCATION UPDATING ACCEPT (step 2a6 of table 6.4.3.7.7-1)

| Derivation Path: TS 36.508 Table 4.7B.2-5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Mobile identity | Not present |  |  |

#### 6.4.3.8	CS fallback to GERAN procedures (LAI of GERAN cell same as the LAI received in combined Attach procedure in EUTRA cell)

All procedures specified in this clause are referred to the GERAN target cell where the UE has been redirected or handed over from the EUTRA cell after a CS call requested. The default message contents are found in TS 34.108 [5], clause 9.

The procedures in 6.4.3.8.1 – 6.4.3.8.4 are applied if the UE supports (EUTRA) RRC connection release with redirection or Cell Change order and Multi Cell System Information to GERAN.

##### 6.4.3.8.1	CS fallback to GERAN with redirection or CCO / MT call (DTM not supported)

Table 6.4.3.8.1-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a CHANNEL REQUEST message. | --> | CHANNEL REQUEST | - | - |
| 2 | The SS transmits an IMMEDIATE ASSIGNMENT message. | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 3 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 4 to 19 the steps specified in table 6.4.3.8.1-2 takes place | - | - | - | - |
| 4 | Check: Does the UE transmit a PAGING RESPONSE message? | --> | PAGING RESPONSE | - | P |
| 4A | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| - | EXCEPTION: Step 4B describes behaviour that depends on the UE capability. | - | - | - | - |
| 4B | IF pc_UTRA then the UE transmits a Utran Classmark message | --> | UTRAN CLASSMARK CHANGE | - | - |
| 5-19 | Steps 5 to 19 of the generic test procedure in TS 51.010-1 sub clause 10.3.3 are performed NOTE: Mobile terminating CS call is established. | - | - | - | - |
| 20 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 21 | The UE transmits RELEASE.. | --> | RELEASE | - | - |
| 22 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 23 | The SS transmits CHANNEL RELEASE, with GPRS Resumption Field set to 1 | <-- | CHANNEL RELEASE | - | - |
| 24-34 | Steps 1 to 11 of the generic test procedure in sub clause 6.4.2.9 are performed. | - | - | - | - |

Table 6.4.3.8.1-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a GPRS SUSPENSION REQUEST message. | --> | GPRS SUSPENSION REQUEST | - | - |

##### 6.4.3.8.2	CS fallback to GERAN with redirection or CCO / MO call (DTM not supported)

Table 6.4.3.8.2-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a CHANNEL REQUEST message. | --> | CHANNEL REQUEST | - | - |
| 2 | The SS transmits an IMMEDIATE ASSIGNMENT message. | <-- | IMMEDIATE ASSIGNMENT | - | - |
| 3 | Void | - | - | - | - |
| - | EXCEPTION: In parallel to the events described in step 4 to 17 the steps specified in table 6.4.3.8.2-2 takes place | - | - | - | - |
| 4 | Check: Does the UE transmit a CM SERVICE REQUEST message? | --> | CM SERVICE REQUEST | - | P |
| 4A | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE | - | - |
| - | EXCEPTION: Step 4B describes behaviour that depends on the UE capability. | - | - | - | - |
| 4B | IF pc_UTRA then the UE transmits a Utran Classmark message | --> | UTRAN CLASSMARK CHANGE | - | - |
| 5-17 | Steps 5 to 17 of the generic test procedure in TS 51.010-1 sub clause 10.2.3 are performed NOTE: Mobile originating CS call is established. | - | - | - | - |
| 18 | The SS transmits DISCONNECT. | <-- | DISCONNECT | - | - |
| 19 | The UE transmits RELEASE.. | --> | RELEASE | - | - |
| 20 | The SS transmits RELEASE COMPLETE. | <-- | RELEASE COMPLETE | - | - |
| 21 | The SS transmits CHANNEL RELEASE, with GPRS Resumption Field set to 1 | <-- | CHANNEL RELEASE | - | - |
| 22-32 | Steps 1 to 11 of the generic test procedure in sub clause 6.4.2.9 are performed. | - | - | - | - |

Table 6.4.3.8.2-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a GPRS SUSPENSION REQUEST message. | --> | GPRS SUSPENSION REQUEST | - | - |

##### 6.4.3.8.3	CS fallback to GERAN with PS Handover / MT call (EDTM not supported)

Same as test procedure sequence in 36.523-1 [18] clause 13.1.11.3.2 from steps 11 to end.

##### 6.4.3.8.4	CS fallback to GERAN with PS Handover / MO call (EDTM not supported)

Same as test procedure sequence in 36.523-1 [18] clause 13.1.12.3.2 from steps 4 to end.

##### 6.4.3.8.5	CS fallback to GERAN with PS Handover / MT call (EDTM supported)

Same as test procedure sequence in 36.523-1 [18] clause 13.1.13.3.2 from steps 7 to end.

#### 6.4.3.9	SRVCC Handover to UTRA

The procedure specified in this clause are referred to the UTRA target cell where the UE has been SRVCC handed over from the EUTRA cell. The default message contents are found in TS 34.108 [5], clause 9.

Table 6.4.3.9-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE transmit a HANDOVER TO UTRAN COMPLETE message? | --> | HANDOVER TO UTRAN COMPLETE | - | P |
| - | EXCEPTION: In parallel to the events described in step 2 to 7 the steps specified in table 6.4.3.9-2 takes place. | - | - | - | - |
| 2 | The SS transmits a SECURITY MODE COMMAND message for the CS domain. | <-- | SECURITY MODE COMMAND | - | - |
| 3 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE | - | - |
| 4 | The SS transmits an UTRAN MOBILITY INFORMATION message to notify CN information. | <-- | UTRAN MOBILITY INFORMATION | - | - |
| 5 | The UE transmits an UTRAN MOBILITY INFORMATION CONFIRM message. | --> | UTRAN MOBILITY INFORMATION CONFIRM | - | - |
| 6 | The SS transmits a TMSI REALLOCATION COMMAND message. | <-- | TMSI REALLOCATION COMMAND | - | - |
| 7 | The UE transmits a TMSI REALLOCATION COMPLETE message. | --> | TMSI REALLOCATION COMPLETE | - | - |
| 8 | The SS transmits a ROUTING AREA UPDATE ACCEPT. | <-- | ROUTING AREA UPDATE ACCEPT | - | - |
| 9 | The UE sends ROUTING AREA UPDATE COMPLETE. | --> | ROUTING AREA UPDATE COMPLETE | - | - |

Table 6.4.3.9-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

##### 6.4.3.9.1	Specific message contents

Table 6.4.3.9.1-1: SECURITY MODE COMMAND (step 2, Table 8.4.1.2.3.2-1)

| Derivation Path: 34.108 clause 9.1.1 (SECURITY MODE COMMAND message) |  |  |
| --- | --- | --- |
| Information Element | Condition | Value/remark |
| Ciphering mode info |  | Not Present |

#### 6.4.3.10	Offload to WLAN

The procedure specified in this clause is referred to the WLAN target AP where the UE has been offloaded from the EUTRA cell.

Table 6.4.3.10-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE associates with the WLAN AP and obtains the local IP address if it has not already done. Note 1 | - | - | - | - |
| - | EXCEPTION: Step 2 is optional, and is executed if initiated by UE | - | - | - | - |
| 2 | UE performs a dynamic selection of ePDG using DNS query | - | - | - | - |
| 3 | UE initiates Authentication and Authorization (EAP-AKA) using CHAP as defined in 33.402 [55] clause 6.5.3 | - | - | - | - |
| 4-9 | Check: The UE establishes IPsec tunnel using the IKEv2 protocol as defined in 3GPP TS 33.402 [55] clause 8.2.2, Figure 8.2.2-1; Procedure as defined in 23.402 [41] clause 8.2.3 steps 4-9 | - | - | - | P |
|  | Exception: Steps 10a1 to 10a3 takes place if UE is in RRC Connected state. Note 2 |  |  | - | - |
| 10a1-10a3 | The generic test procedure in TS 36.508 subclause 4.5A.15.3 is executed. | - | - | - | P |
| - | EXCEPTION: Steps 11a1 to 11a2b1 describes a transaction that depends on offload of IMS PDN. |  |  |  |  |
| 11a1 | IF IMS PDN is offloaded SS starts timer Timer_1 = 10 s (Note 3) |  |  |  |  |
| - | EXCEPTION: Steps 11a2a1 to 11a2a2 describe a transaction that depends on offload of IMS PDN and UE implementation |  |  |  |  |
| 11a2a1-11a2a2 | IF initiated by the UE, Re-Registration procedure according to TS 34.229-1 [43] sub clause C.XX takes place on WLAN AP. |  |  |  |  |
| 11a2b1 | The SS waits for Timer_1 expiry |  |  |  |  |
| Note 1: If the UE has not already associated with WLAN AP it may take long time (up to 60 seconds) to associate with WLAN AP as it may need to scan, authenticate, obtain IP address etc. Note 2: If UE is in RRC Idle state then as per 24.301[28] clause 6.4.2.2, the offloaded PDN is implicitly released. Note 3: A guarding time of [10] sec is suggested within which the procedure is expected to start. If the timer expires then the test procedure, from which the Procedure for IMS Re-Registration is called, shall advance to the next specified step. |  |  |  |  |  |

##### 6.4.3.10.1	Specific message contents

None

#### 6.4.3.11	Offload from WLAN

The procedure specified in this clause is referred to the EUTRA cell where the UE has been previously offloaded from the EUTRA cell and now the Offload from WLAN AP back to EUTRA cell happens.

Table 6.4.3.11-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | Exception: Steps 1 to 9 are executed if the UE is in RRC Idle state in EUTRA cell. |  |  | - | - |
| 1 | Check: Does UE transmit an RRCConnectionRequest message with establishmentCause set to ‘mo-Data’ followed by a SERVICE REQUEST message? | --> | SERVICE REQUEST | - |  |
| 2-9 | The SS establishes SRB2 and DRB associated with non offloaded default EPS bearer context by executing steps 3-10a7 in the generic test procedure in TS 36.508 subclause 4.5.3.3 | - | - | - | - |
| 10 | Check: Does the UE establish additional PDN connection for offloaded PDN by executing the generic test procedure in TS 36.508 subclause 4.5A.16.3. | - | - | - | P |
| 11 | Network initiated IPsec tunnel disconnection procedures defined by the IKEv2 protocol in IETF RFC 5996 [57] is executed on WLAN AP | - | - | - | - |

##### 6.4.3.11.1	Specific message contents

None

#### 6.4.3.12	Check UE does not offload to WLAN

The procedure specified in this clause is referred to the WLANAP.

Table 6.4.3.12-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | Exception: In parallel to step 1 parallel behaviour in table 6.4.3.12-2 may be optionally executed. | - | - | - | - |
| 1 | Check: IF UE initiates Authentication and Authorization (EAP-AKA) using CHAP as defined in 33.402 [55] clause 6.5.3 in the next 60 seconds | - | - | - | F |

Table 6.4.3.12-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE associates with the WLAN AP and obtains the local IP address if it has not already done | - | - | - | - |

##### 6.4.3.12.1	Specific message contents

None

#### 6.4.3.13	Check UE does not Offload to E-UTRAN

The procedure specified in this clause are referred to the EUTRA cell.

Table 6.4.3.13-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
|  | Exception: Steps 1a to 1b describe behaviour that depends on the state; the "lower case letter" identifies a step sequence that take place. IF UE is in ‘Idle’ state step 1a takes and if in ‘Connected’ state step 1b takes |  |  | - | - |
| 1a | Check: Does UE transmit an RRCConnectionRequest in the next 15 seconds | --> | RRCConnectionRequest | - | F |
| 1b | Check: Does the UE transmits a PDN CONNECTIVITY REQUEST message to request an additional PDN. In the next 15 seconds | --> | RRC: ULInformationTransfer NAS: PDN CONNECTIVITY REQUEST | - | F |

### 6.4.3.14	Procedure for UE initiated detach at non-switch-off

The purpose of this procedure is to allow a UE to perform UE initiated detaching.

Table 6.4.3.14-1: Procedure for optional UE initiated detaching

| Step | Procedure | Message Sequence |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a DETACH REQUEST message with the Detach type IE indicating “EPS detach” to regain normal service. | --> | DETACH REQUEST | - | - |
| 2 | The SS responds the DETACH ACCEPT message. | <-- | DETACH ACCEPT | - | - |

##### 6.4.3.13.1	Specific message contents

None

### 6.4.3A	Test case postambles

#### 6.4.3A.1	Introduction

In order to make test cases perfectly reproducible, the UE under test is switched off after the test procedure sequence of a test case is complete. In order to make it possible to run a number of test cases without any manual operation, it is important that the SS can properly handle any possible signalling from the UE between the end of the test procedure sequence and until the UE is off.

After the last verdict of a test procedure sequence is assigned, it is necessary to:

-	terminate any ongoing signalling procedure,

-	switch off the UE.

Usually, this can be performed by completing necessary TAU/LAU/RAU procedures, completing any ongoing voice call (CS or IMS), and switching off the UE with a detach procedure.

#### 6.4.3A.2	Reference end states

Test procedures in TS 36.523-1 may only partially terminate ongoing signalling procedures, provided they indicate a reference end state at the end of the test procedure sequence, so that a matching procedure in TS 36.523-3 can be executed to switch off the UE.

Reference end states definitions include the necessary pieces of information to predict UE behaviour during the postambles.

If any extra information was stored in the UE or the USIM due to the test procedure sequence, it should be removed explicitly by the test procedure sequence, before the UE can be left in a reference end state.

Table 6.4.3A.2-1: Reference end states

| Name | Description | Optional information |
| --- | --- | --- |
| E-UTRA idle (E1) | The UE: - is camped on an E-UTRA cell, and RRC idle and - registered for EPS services and EPS update status is "updated" and - is registered for non-EPS services and update status is updated (if CS fallback or SMS over SGs is supported) and - has one or more default EPS bearer context are active, zero or more dedicated EPS bearer contexts are active. | The test case may indicate that the UE is not registered for non-EPS services, though the UE supports CS fallback or SMS over SGs. |
| E-UTRA connected (E2) | Same as E-UTRA idle, except that: - the UE is RRC connected and - DRBs for all active EPS bearer contexts are established. | Same as for E-UTRA idle. |
| E-UTRA connected, T3440 started (E2_T3440) | Same as E-UTRA idle, except that: - the UE is RRC connected and - Timer T3440 is started in UE. | Same as for E-UTRA idle |
| E-UTRA test mode (E3) | Same as E-UTRA connected and - test mode is active and - test loop is closed. | Same as for E-UTRA idle |
| E-UTRA deregistered (E4) | The UE is: - camped on an E-UTRA cell, and idle and - not registered of EPS services and - not registered for non-EPS services. |  |
| E-UTRA manual selection (E5) | Same as E-UTRA idle, except that: - the UE is in manual PLMN selection mode | Same as for E-UTRA idle |
| UTRA idle (U1) | The UE is: - camped on a UTRA cell, RRC idle and - IMSI attached and update status is "updated" (if the UE supports CS domain) and - GPRS attached, GPRS update state is "updated", and zero or one or several PDP context(s) with interactive or background QoS are active. | The test case may indicate that the UE is not IMSI attached though the UE supports CS domain |
| UTRA connected (U2) | Same as UTRA idle, except that: - the UE is in CELL_DCH state and - a RAB associated with the active PDP context is established. | The test case may indicate that the UE is not IMSI attached though the UE supports CS domain. |
| UTRA handover (U3) | The UE: - was E-UTRA connected and - has completed a handover to UMTS and - hasn't yet sent or received any NAS signalling on the target UTRAN cell. | Same as for UTRA connected |
| UTRA CS fallback (U4) | The UE: - was E-UTRA idle or E-UTRA connected and - has transmitted an EXTENDED SERVICE REQUEST message due to MO or MT CS fallback call and - either the UE has completed a handover to UMTS or the UE has been redirected to UMTS and has established the RRC connection to a UTRA cell, and the target UTRA cell has activated security and RABs for all previously active EPS bearer contexts were established and - the UE hasn't yet sent or received any NAS signalling on the target UTRAN cell |  |
| UTRA CS call (U5) | The UE was in UTRAN CS fallback and the UE has completed LAU/RAU procedure if necessary and has established a CS call or the UE was in E-UTRAN connected and the UE has completed LAU/RAU procedure if necessary and has performed a handover to UTRAN with SRVCC. |  |
| UTRA deregistered (U6) | The UE is: - camped on a UTRA cell, and idle and - not registered GPRS/non-GPRS services |  |
| GERAN idle (G1) | The UE: - is camped on an GERAN cell, in idle mode and GPRS idle state and - is registered for PS services and GPRS update status is "updated" and zero or one or several PDP context(s) with interactive or background QoS are active. - is IMSI attached (if CS domain is supported) |  |
| GERAN PS handover (G2) | The UE: - was E-UTRA connected and - has performed a PS handover procedure to a GERAN cell and - hasn't yet sent or received any NAS signalling on the target GERAN cell. |  |
| GERAN CS fallback (G3) | The UE: - was E-UTRA idle or E-UTRA connected and - has transmitted an EXTENDED SERVICE REQUEST message due to MO or MT CS fallback call and - either the UE has completed a PS handover to GERAN or a CCO to GERAN or the UE has been redirected to GERAN and has entered dedicated mode, and if DTM is supported, - the UE hasn't yet sent or received any NAS signalling on the target GERAN cell |  |
| GERAN CS call (G4) | The UE was in GERAN CS fallback and the UE has established a CS call or the UE was in E-UTRAN connected and has performed a handover to GERAN with SRVCC. |  |
| GERAN deregistered (G5) | The UE is: - camped on a GERAN cell, and idle and - not registered GPRS/non-GPRS services |  |
| 1xRTT state 1 (1x1) | FFS |  |
| ... | ... |  |
| HRPD state 1 (H1) | FFS |  |
| ... | ... |  |

## 6.5	Default RRC message and information element contents

The default RRC message and information element contents specified in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified, in addition to the default RRC message and information element contents specified in subclause 4.6 of this specification.

### 6.5.1	Measurement information elements

#### -	MeasConfig-DEFAULT

Table 6.5.1-1: MeasConfig-DEFAULT

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig-DEFAULT ::= SEQUENCE { |  |  |  |
| measObjectToRemoveList | Not present |  |  |
| measObjectToAddModList | Not present |  |  |
| reportConfigToRemoveList | Not present |  |  |
| reportConfigToAddModList | Not present |  |  |
| measIdToRemoveList | Not present |  |  |
| measIdToAddModList | Not present |  |  |
| quantityConfig | QuantityConfig-DEFAULT |  |  |
| measGapConfig | Not present |  |  |
|  | MeasGapConfig-GP1 |  | INTER-FREQ, UTRAN |
|  | MeasGapConfig-GP2 |  | GERAN, INTER-RAT |
|  | MeasGapConfig-CE |  | intraFreq-CE-NeedForGaps |
| s-Measure | Not present |  |  |
| preRegistrationInfoHRPD | Not present |  |  |
| speedStatePars | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| INTER-FREQ | For E-UTRA inter-freq measurements |
| UTRAN | For inter-RAT measurements with UTRAN |
| GERAN | For inter-RAT measurements with GERAN |
| INTER-RAT | For inter-RAT measurements with UTRAN and GERAN |
| intraFreq-CE-NeedForGaps | For UE having set pc_intraFreq-CE-NeedForGaps to TRUE. |

#### -	MeasGapConfig-GP1

Table 6.5.1-2: MeasGapConfig-GP1

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-GP1 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 30 | TGRP = 40 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	MeasGapConfig-GP2

Table 6.5.1-3: MeasGapConfig-GP2

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-GP2 ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp1 | 30 | TGRP = 80 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	MeasGapConfig-CE

Table 6.5.1-4: MeasGapConfig-CE

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasGapConfig-CE ::= CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| gapOffset CHOICE { |  |  |  |
| gp0 | 20 | TGRP = 40 ms |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

## 6.6	Default NAS message and information element contents

The default NAS message and information element contents specified in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified, in addition to the default NAS message and information element contents specified in subclause 4.7 of this specification.

### 6.6.1	Reference default EPS bearer contexts

The following table defines Reference default EPS bearer contexts. Default EPS bearer context1 is the default "default EPS bearer context" which is used in the common procedures and test cases where no particular default EPS bearer context is specified.

Table 6.6.1-1: Reference default EPS bearer contexts

| Parameters | Reference default EPS bearer context #1 | Reference default EPS bearer context #2 | Reference default EPS bearer context #3 |
| --- | --- | --- | --- |
| EPS QoS |  |  |  |
| QCI (Note 1) | 9 (non-GBR QCI) | 5 (non-GBR QCI) | 69 (Note 5) (non-GBR QCI) |
| Maximum bit rate for uplink | 64 kbps (Note 2) | 64 kbps (Note 2) | 64 kbps (Note 2) |
| Maximum bit rate for downlink | 64 kbps (Note 2) | 64 kbps (Note 2) | 64 kbps (Note 2) |
| Guaranteed bit rate for uplink | 64 kbps (Note 2) | 64 kbps (Note 2) | 64 kbps (Note 2) |
| Guaranteed bit rate for downlink | 64 kbps (Note 2) | 64 kbps (Note 2) | 64 kbps (Note 2) |
| Maximum bit rate for uplink (extended) | 0 | 0 | 0 |
| Maximum bit rate for downlink (extended) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | 0 | 0 |
| Negotiated QoS | Note 3, Note 4 | Note 3, Note 4 | Note 3, Note 4 |
| Negotiated LLC SAPI | Note 4 | Note 4 | Note 4 |
| Radio priority | Note 4 | Note 4 | Note 4 |
| APN-AMBR | Not present | Not present | Not present |
| Configuration protocol | PPP | PPP | PPP |
| Note 1: For all non-GBR QCIs, the maximum and guaranteed bit rates shall be ignored. Note 2: According to TS 24.301, the UE ignores these parameters for a non-GBR QCI. Note 3: Parameters included for UEs capable of UTRAN according to TS 34.123-3 clause 8.10. Note 4: Parameters included for UEs capable of GERAN according to TS 51.010 subclause 40.5. Note 5: QCI 69 is used for Mission Critical delay sensitive signalling. |  |  |  |

### 6.6.2	Reference dedicated EPS bearer contexts

The following table defines Reference dedicated EPS bearer contexts. Dedicated EPS bearer context #1 is the default "dedicated EPS bearer context" which is used in the common procedures and test cases where no particular dedicated EPS bearer context is specified.

Table 6.6.2-1: Reference dedicated EPS bearer contexts, first set

| Parameters | Reference dedicated EPS bearer context #1 | Reference dedicated EPS bearer context #2 | Reference dedicated EPS bearer context #3 | Reference dedicated EPS bearer context #4 | Reference dedicated EPS bearer context #5 |
| --- | --- | --- | --- | --- | --- |
| Linked EPS bearer identity | Reference default EPS bearer #1 | Reference default EPS bearer #1 | Reference default EPS bearer #2 | Reference default EPS bearer #2 | Reference default EPS bearer #3 |
| EPS QoS |  |  |  |  |  |
| QCI (Note 1) | 1 (GBR QCI) | 5 (non-GBR QCI) | 2 (GBR QCI) | 1 (GBR QCI) | 65 (Note 8) (GBR QCI) |
| Maximum bit rate for uplink | 384 kbps | 384 kbps (Note 2) | 2104 kbps (Note 7) | 384 kbps | 384 kbps |
| Maximum bit rate for downlink | 384 kbps | 384 kbps (Note 2) | 2104 kbps (Note 7) | 384 kbps | 384 kbps |
| Guaranteed bit rate for uplink | 128 kbps | 128 kbps (Note 2) | 2104 kbps (Note 7) | 128 kbps | 128 kbps |
| Guaranteed bit rate for downlink | 128 kbps | 128 kbps (Note 2) | 2104 kbps (Note 7) | 128 kbps | 128 kbps |
| Maximum bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Maximum bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| TFT |  |  |  |  |  |
| TFT operation code | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" |
| E bit | 0 | 0 | 0 | 0 | 0 |
| Packet filters (Note 5) | 1, 2 | 3 | 5 | 4 | 4, 10 (Note 9) |
| Negotiated QoS |  |  |  |  |  |
| Traffic Class | conversational | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | conversational | conversational | conversational |
| Delivery Order | 'no' | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | ’no’ | 'no' | 'no' |
| Delivery of erroneous SDU | 'no' | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | ’no’ | 'no' | 'no' |
| Maximum SDU size | 150 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 1400 | 150 | 150 |
| Maximum bit rate for uplink | 384 kbps | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 432 kbps | 384 kbps | 384 kbps |
| Maximum bit rate for downlink | 384 kbps | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 432 kbps | 384 kbps | 384 kbps |
| Residual BER | 5*10-2 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 10-5 | 5*10-2 | 5*10-2 |
| SDU error ratio | 10-2 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 7*10-3 | 10-2 | 10-2 |
| Transfer delay | 80 ms | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 130 ms | 80 ms | 75 ms |
| Traffic Handling priority | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Signalling Indication | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Source Statistics Descriptor | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 1 | 0 |
| Maximum bit rate for downlink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Maximum bit rate for uplink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | See "PDP ContextDchForLTE" (Note 3); See "PDP context3" (Note 6) | 0 | 0 | 0 |
| Negotiated LLC SAPI | 3 (Note 6) | See "PDP context3" (Note 6) | - | 3 (Note 6) | - |
| Radio priority | 1 (Note 6) | See "PDP context3" (Note 6) | - | 1 (Note 6) | - |
| Protocol configuration options | - | - | - | - | - |
| Note 1: For all non-GBR QCIs, the maximum and guaranteed bit rates shall be ignored. Note 2: According to TS 24.301, the UE ignores these parameters for a non-GBR QCI. Note 3: Parameters included for UEs capable of UTRAN according to TS 34.123-3 clause 8.10 (table 8.10.1 and 8.10.2). Note 5: This row refers to the reference packet filters defined in the tables below. For each reference dedicated EPS bearer context, a list of reference packet filter numbers is provided. Note 6: Parameter included for UEs capable of GERAN; when value is not provided then value from TS 51.010 subclause 40.5 is applied. Note 7: This value shall reflect the highest possible bandwidth requirement of the H.264 codec to EPS bearer mappings as specified in Appendix E of TS 26.114 [64]. Note 8: QCI 65 is used for MCPTT voice. Note 9: MCPTT: Packet filter #4 for audio stream, packet filter #10 for media control messages. |  |  |  |  |  |

Table 6.6.2-1A: Reference dedicated EPS bearer contexts, second set

| Parameters | Reference dedicated EPS bearer context #6 | Reference dedicated EPS bearer context #7 | Reference dedicated EPS bearer context #8 | Reference dedicated EPS bearer context #9 | Reference dedicated EPS bearer context #10 |
| --- | --- | --- | --- | --- | --- |
| Linked EPS bearer identity | Reference default EPS bearer #1 | Reference default EPS bearer #2 | Reference default EPS bearer #2 | Reference default EPS bearer #3 | Reference default EPS bearer #3 |
| EPS QoS |  |  |  |  |  |
| QCI (Note 1) | 8 (non-GBR QCI) | 79 (non-GBR QCI) | 66 (GBR QCI) | 70 (Note 4) (non-GBR QCI) | 67 (Note 5) (GBR QCI) |
| Maximum bit rate for uplink | 384 kbps (Note 2) | 384 kbps (Note 2) | 384 kbps | 384 kbps (Note 2) | 2104 kbps |
| Maximum bit rate for downlink | 384 kbps (Note 2) | 384 kbps (Note 2) | 384 kbps | 384 kbps (Note 2) | 2104 kbps |
| Guaranteed bit rate for uplink | 128 kbps (Note 2) | 128 kbps (Note 2) | 128 kbps | 128 kbps (Note 2) | 2104 kbps |
| Guaranteed bit rate for downlink | 128 kbps (Note 2) | 128 kbps (Note 2) | 128 kbps | 128 kbps (Note 2) | 2104 kbps |
| Maximum bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Maximum bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for uplink (extended) | 0 | 0 | 0 | 0 | 0 |
| Guaranteed bit rate for downlink (extended) | 0 | 0 | 0 | 0 | 0 |
| TFT |  |  |  |  |  |
| TFT operation code | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" | "create new TFT" |
| E bit | 0 | 0 | 0 | 0 | 0 |
| Packet filters (Note 3) | 6, 7 | 8 | 9 | 11 | 4, 5, 10 (Note 6) |
| Negotiated QoS |  |  |  |  |  |
| Traffic Class | - | - | Conversational | - | - |
| Delivery Order | - | - | ‘no’ | - | - |
| Delivery of erroneous SDU | - | - | ‘no’ | - | - |
| Maximum SDU size | - | - | 150 | - | - |
| Maximum bit rate for uplink | - | - | 384 kbps | - | - |
| Maximum bit rate for downlink | - | - | 384 kbps | - | - |
| Residual BER | - | - | 5*10-2 | - | - |
| SDU error ratio | - | - | 10-2 | - | - |
| Transfer delay | - | - | 75 ms | - | - |
| Traffic Handling priority | - | - | 0 | - | - |
| Guaranteed bit rate for uplink | - | - | 0 | - | - |
| Guaranteed bit rate for downlink | - | - | 0 | - | - |
| Signalling Indication | - | - | 0 | - | - |
| Source Statistics Descriptor | - | - | 0 | - | - |
| Maximum bit rate for downlink (extended) | - | - | 0 | - | - |
| Guaranteed bit rate for downlink (extended) | - | - | 0 | - | - |
| Maximum bit rate for uplink (extended) | - | - | 0 | - | - |
| Guaranteed bit rate for uplink (extended) | - | - | 0 | - | - |
| Negotiated LLC SAPI | - | - | - | - | - |
| Radio priority | - | - | - | - | - |
| Protocol configuration options | - | - | - | - | - |
| Note 1: For all non-GBR QCIs, the maximum and guaranteed bit rates shall be ignored. Note 2: According to TS 24.301, the UE ignores these parameters for a non-GBR QCI. Note 3: This row refers to the reference packet filters defined in the tables below. For each reference dedicated EPS bearer context, a list of reference packet filter numbers is provided. Note 4: QCI 70 is used for MCData (Mission Critical Data). Note 5: QCI 67 is used for MCVideo (Mission Critical Video). Note 6: MCVideo: Packet filter #4 for audio stream, packet filter #5 for video stream, packet filter #10 for transmission control messages. |  |  |  |  |  |

Table 6.6.2-2: Reference packet filter #1

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 0 1 0 0 0 0 | DL only filter,ID=0 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 | 0 to 7 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 2 Value | 31 160 + EPS Bearer ID - 6 |  |  |
| Component type 3 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 3 Value | 17 | UDP |  |
| Note 1: remoteAddress should be set to the address of an IP server able to send a flow of downlink IP/UDP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-3: Reference packet filter #2

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 2 Value | 61 000 + EPS Bearer ID - 6 |  |  |
| Component type 3 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 3 Value | 17 | UDP |  |
| Note 1: remoteAddress should be set to the address of an IP server able to process a flow of uplink IP/UDP packets received from the UE. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #1. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-4: Reference packet filter #3

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 0 1 0 | Bidirectional filter, ID=2 |  |
| Evaluation precedence | 0 0 0 0 1 1 1 1 | Lowest priority |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Note 1: remoteAddress should be set to the address of an IP server able to process a uplink IP packets from the UE and transmit downlink IP packets to the UE. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-5: Reference packet filter #4

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 0 1 1 | Bidirectional,ID=3 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-6: Reference packet filter #5

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 0 | Bidirectional,ID=4 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS video media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-7: Reference packet filter #6

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 0 1 0 0 0 0 | DL only filter,ID=0 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 | 0 to 7 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 1 | ICMP | remote IPv4 |
| Component type 2 Value | 58 | ICMPv6 | remote IPv6 |
| Note 1: remoteAddress is the address of a network entity and the filter matches for ICMP/ICMPv6 packets sent to the UE with remote address being source address. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-8: Reference packet filter #7

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 1 | ICMP | Ping |
| Component type 2 Value | 58 | ICMPv6 | Ping |
| Note 1: remoteAddress is the address of a network entity and the filter matches for ICMP/ICMPv6 packets sent by the UE with remote address being destination address. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #6. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-9: Reference packet filter #8

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 0 0 0 0 1 | UL only filter, ID=1 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID - 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 0 0 1 0 0 0 0 | IPv4 remote address type | remoteIPv4 |
|  | 0 0 1 0 0 0 0 0 | IPv6 remote address type | remoteIPv6 |
| Component type 1 Value | remoteAddress 255.255.255.255 | See note 1 | remoteIPv4 |
|  | remoteAddress ff:ff:ff:ff:ff:ff:ff:ff: ff:ff:ff:ff:ff:ff:ff:ff | See note 1 | remoteIPv6 |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: remoteAddress should be set to the address of an IP server able to process a flow of uplink IP/UDP packets received from the UE. When configured together with packet filter #1, remoteAddress is the same as that for packet filter #1. remoteIPv4 applies if the UE has acquired an IPv4 address only, remoteIPv6 applies if the UE has acquired an IPv6 address only, or both an IPv6 and an IPv4 address. |  |  |  |

Table 6.6.2-10: Reference packet filter #9

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 1 | Bidirectional filter, ID=5 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 | 0 to 7 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 1 | Remote port range type |  |
| Component type 1 Value | media port | SS speech media port as used in the SDP negotiation (RTP remote port); see Note 1 |  |
|  | media port + 1 | RTCP remote port; see Note 1 |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |
| Note 1: Acc. to TS 26.114 and RFC 4566 a "media port" can be understood as the transport port to which a media stream is sent. |  |  |  |

Table 6.6.2-11: Reference packet filter #10

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 0 | Bidirectional,ID=4 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 1 Value | media port | Port used for media control message |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 17 | UDP |  |

Table 6.6.2-12: Reference packet filter #11

| Derivation path: 24.008 table 10.5.162 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Identifier | 0 0 1 1 0 1 0 0 | Bidirectional,ID=4 |  |
| Evaluation precedence | (0 0 0 0 0 0 0 0) + EPS Bearer ID – 6 + 8 | 8 to 15 |  |
| Component type 1 ID | 0 1 0 1 0 0 0 0 | Single remote port type |  |
| Component type 1 Value | media port | Port used for the data stream (e.g. MSRP) |  |
| Component type 2 ID | 0 0 1 1 0 0 0 0 | Protocol identifier/Next header type |  |
| Component type 2 Value | 6 | TCP |  |

## 6.6A	Default SMS over SGs message and information element contents

The default SMS over SGs message and information element contents specified in this subclause apply to all SMS over SGs Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified. All the messages and information elements are listed in alphabetical order.

### 6.6A.1	CM-sublayer messages

#### -	CP-ACK

This message is sent between the SS and the UE, in both directions.

Table 6.6A.1-1: Message CP-ACK

| Derivation Path: 24.011 clause 7.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | ‘1001’B | SMS messages |  |
| Transaction identifier | Any allowed value |  |  |
| Message type | ‘00000100’B | CP-ACK |  |

#### -	CP-DATA

This message is sent between the SS and the UE, in both directions.

Table 6.6A.1-2: Message CP-DATA

| Derivation Path: 24.011 clause 7.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Protocol discriminator | ‘1001’B | SMS messages |  |
| Transaction identifier | Any allowed value |  |  |
| Message type | ‘00000001’B | CP-DATA |  |
| CP-User data | RP-ACK RPDU or RP-DATA RPDU (as specified in the test case) |  |  |

### 6.6A.2	Short Message Relay Layer (SM-RL) messages

#### -	RP-ACK RPDU

This message is sent between the SS and the UE, in both directions.

Table 6.6A.2-1: Message RP-ACK RPDU

| Derivation Path: 24.011 clause 7.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RP-Message Type | ‘010’B | RP-ACK_PDU uplink | Uplink_SMS |
|  | ‘011’B | RP-ACK_PDU downlink | Downlink_SMS |
| RP-Message Reference | Same as in associated (preceding) RP-DATA RPDU |  |  |
| RP-User Data | Not present or any allowed value |  | Uplink_SMS |
|  | Not present |  | Downlink_SMS |

| Condition | Explanation |
| --- | --- |
| Uplink_SMS | This condition applies when the message is sent by the UE to the SS. |
| Downlink_SMS | This condition applies when the message is sent by the SS to the UE. |

#### -	RP-DATA RPDU

This message is sent between the SS and the UE, in both directions.

Table 6.6A.2-2: Message RP-DATA RPDU

| Derivation Path: 24.301 clause 8.2.23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RP-Message Type | ‘001’B | RP-DATA_PDU downlink | Downlink_SMS |
|  | ‘000’B | RP-DATA_PDU uplink | Uplink_SMS |
| RP-Message Reference | Any allowed value |  |  |
| RP-Originator Address | Any allowed value | originating Service Centre address | Downlink_SMS |
|  | Not present |  | Uplink_SMS |
| RP-Destination Address | Not present |  | Downlink_SMS |
|  | Any allowed value | destination Service Centre address | Uplink_SMS |
| RP-User Data | SMS-SUBMIT or SMS-DELIVER (as set in the test case) |  |  |

| Condition | Explanation |
| --- | --- |
| Uplink_SMS | See the definition below table 6.6A.2-1. |
| Downlink_SMS | See the definition below table 6.6A.2-1. |

### 6.6A.3	Short Message Transfer Layer (SM-TL) messages

#### -	SMS-DELIVER

This message is sent by the SS to the UE.

Table 6.6A.3-1: Message SMS-DELIVER

| Derivation Path: 23.040 clause 9.2.2.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TP-MTI | ‘00’B | SMS-DELIVER |  |
| TP-MMS | ‘1’B | No more messages are waiting for the MS in this SC |  |
| TP-RP | Any allowed value |  |  |
| TP-UDHI | ‘0’B |  |  |
| TP-SRI | ‘0’B |  |  |
| TP-OA | Any allowed value |  |  |
| TP-PID | ‘00000000’B |  |  |
| TP-DCS | Any allowed value |  |  |
| TP-SCTS | Any allowed value |  |  |
| TP-UDL | 160 |  |  |
| TP-UD (140 octets) | text of message (160 characters) | The 160 characters in TP-UD shall include at least one occurrence of each character in the default alphabet (see 3GPP TS 23.038, clause 6.2.1). |  |

#### -	SMS-SUBMIT

This message is sent by the UE to the SS.

Table 6.6A.3-2: Message SMS-SUBMIT

| Derivation Path: 23.040 clause 9.2.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| TP-MTI | ‘01’B | SMS-SUBMIT |  |
| TP-RD | Any allowed value |  |  |
| TP-VPF | Any allowed value |  |  |
| TP-RP | Any allowed value |  |  |
| TP-UDHI | Not present or any allowed value |  |  |
| TP-SRR | Not present or any allowed value |  |  |
| TP-MR | Any allowed value |  |  |
| TP-DA | Any allowed value |  |  |
| TP-PID | ‘00000000’B |  |  |
| TP-DCS | Any allowed value |  |  |
| TP-VP | Not present or any allowed value |  |  |
| TP-UDL | 160 |  |  |
| TP-UD (140 octets) | text of message (160 characters) |  |  |

## 6.6B	Reference radio bearer configurations

### 6.6B.1	SRB and DRB parameters and combinations

#### 6.6B.1.1	SRB and DRB parameters

##### 6.6B.1.1.1	Physical Layer configurations

Table 6.6B.1.1.1-1: PhysicalConfigDedicated-DEFAULT

| Derivation Path: TS 36.508 clause 4.8.2.1.6, Table 4.8.2.1.6-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-DEFAULT ::= SEQUENCE { |  |  |  |
| cqi-ReportConfig | CQI-ReportConfig-DEFAULT using condition CQI_PERIODIC | See subclause 4.6.3 | SRB1 or RBC or RBC-HO or HO-TO-EUTRA |
| soundingRS-UL-ConfigDedicated | SoundingRS-UL-ConfigDedicated-DEFAULT | See subclause 4.6.3 | (SRB1 or RBC or RBC-HO or HO-TO-EUTRA) and not (CEmodeA or CEmodeB) |
|  | Not present | Not applicable for CAT M1 | CEmodeA or CEmodeB |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SRB1 | Used at configuration of SRB1 during RRC connection (re-)establishment |
| RBC | Used at configuration of a radio bearer combination during SRB2+DRB establishment |
| 2TX | Used for cells with two antenna ports |
| RBC-HO | Used during Handover |
| HO-TO-EUTRA | Inter-RAT handover to E-UTRA |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

## 6.7	Timer Tolerances

The timer tolerances specified for the test environment in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified

All the timers used during testing are within a tolerance margin given by the equation below. If for a specific test a different tolerance value is required then this should be specified in the relevant test document (i.e. the document where the test is described).

Timer tolerance = 10%, or 5  RTT, whichever value is the greater.

Where RTT = 8 TTIs for FDD, and RTT= Maximum RTT from Table 6.7-1 for TDD (see Note).

NOTE:	Since the exact RTT for TDD varies depending on the UL/DL configurations and subframe number [29], the maximum RTT is defined in Table 6.7-1.

Table 6.7-1: Maximum RTT for TDD UL/DL configurations

| UL/DL Configuration | Maximum RTT (TTIs) |
| --- | --- |
| 0 | 10 |
| 1* | 11 |
| 2 | 12 |
| 3 | 15 |
| 4 | 16 |
| 5 | TBD |
| 6 | 11 |
| *Note: Default UL/DL configuration defined in Table 4.6.3-23 of TS 36.508 |  |

## 6.8	SideLink reference configuration

The reference test configurations specified in 6.8.1 apply to all SideLink Signalling test cases defined in TS 36.523-1 [18].

The reference test configurations specified in 6.8.2 apply to all V2X SideLink Signalling test cases defined in TS 36.523-1 [18].

### 6.8.1	Reference configuration for Direct Communication

ROHC compression is not applied in the signalling test cases simulating in coverage and out-of-coverage scenarios.

Unless otherwise specified, ciphering is not configured on SLRB.

#### 6.8.1.1	ProSe Direct Communication Preconfiguration for out-of-network coverage operation

The IE SL-Preconfiguration contains the resource pool information for sidelink direct communication.

Table 6.8.1.1-1: SL-Preconfiguration

| Derivation Path: 36.331 clause 9.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SL-Preconfiguration-r12 ::= SEQUENCE { |  |  |  |
| preconfigGeneral-r12 SEQUENCE { |  | SL-PreconfigGeneral-r12 |  |
| rohc-Profiles-r12 SEQUENCE { |  | -- PDCP configuration |  |
| profile0x0001-r12 | false |  |  |
| profile0x0002-r12 | false |  |  |
| profile0x0004-r12 | false |  |  |
| profile0x0006-r12 | false |  |  |
| profile0x0101-r12 | false |  |  |
| profile0x0102-r12 | false |  |  |
| profile0x0104-r12 | false |  |  |
| } |  |  |  |
| carrierFreq-r12 | f1 as per table 6.2.3.3-2 using the same band as cell1 | ARFCN-ValueEUTRA-r9 |  |
| maxTxPower-r12 | 0 |  |  |
| additionalSpectrumEmission-r12 | 1 |  |  |
| sl-bandwidth-r12 | see table 6.2.3.3-2 using the same bandwidth as cell1 |  |  |
| tdd-ConfigSL-r12 SEQUENCE { |  | TDD-ConfigSL-r12 |  |
| subframeAssignment | none | FDD |  |
| } |  |  |  |
| reserved-r12 | ‘0000000000000000000’B |  |  |
| } |  |  |  |
| preconfigSync-r12 SEQUENCE { |  | SL-PreconfigSync-r12 |  |
| syncCP-Len-r12 | normal | SL-CP-Len-r12 |  |
| syncOffsetIndicator1-r12 | 1 | SL-OffsetIndicatorSync-r12 Synchronisation resources are present in those SFN and subframes which satisfy the relation: (SFN*10+ Subframe Number) mod 40 = SL-OffsetIndicatorSync |  |
| syncOffsetIndicator2-r12 | 2 | SL-OffsetIndicatorSync-r12 |  |
| syncTxParameters-r12 | 23 | P0-SL-r12 |  |
| syncTxThreshOoC-r12 | 0 | (-110dBm/15kHz) |  |
| filterCoefficient-r12 | fc0 |  |  |
| syncRefMinHyst-r12 | dB0 |  |  |
| syncRefDiffHyst-r12 | dB0 |  |  |
| } |  |  |  |
| preconfigComm-r12 SEQUENCE (SIZE (1..maxSL-TxPool-r12)) OF SL-PreconfigCommPool-r12 { | 1 entry | SL-PreconfigCommPoolList4-r12 |  |
| SL-PreconfigCommPool-r12 SEQUENCE { |  | -- This IE is same as SL-CommResourcePool with rxParametersNCell absent |  |
| sc-CP-Len-r12 | normal | SL-CP-Len-r12 |  |
| sc-Period-r12 | sf40 | SL-PeriodComm-r12 |  |
| sc-TF-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | SL-OffsetIndicator-r12 |  |
| subframeBitmap-r12 | 00011000 00000000 00000000 00000000 00000000 | bs40-r12 | SLCONF1 |
|  | 00000110 00000000 00000000 00000000 00000000 |  | SLCONF2 |
| } |  |  |  |
| sc-TxParameters-r12 | 0 | INTEGER (-126..31) |  |
| data-CP-Len-r12 | Normal | SL-CP-Len-r12 |  |
| data-TF-ResourceConfig-r12 SEQUENCE { |  | SL-TF-ResourceConfig-r12 |  |
| prb-Num-r12 | 13 |  | BW5 |
|  | 25 |  | BW10 |
| prb-Start-r12 | 0 |  |  |
| prb-End-r12 | 24 |  | BW5 |
|  | 49 |  | BW10 |
| offsetIndicator-r12 | 0 | SL-OffsetIndicator-r12 |  |
| subframeBitmap-r12 | 00000000 11111111 11111111 00000000 00000000 | bs40-r12 | SLCONF1 |
|  | 00000000 00000000 00000000 11111111 11111111 |  | SLCONF2 |
| } |  |  |  |
| dataHoppingConfig-r12 SEQUENCE { |  | SL-HoppingConfigComm-r12 |  |
| hoppingParameter-r12 | 0 | INTEGER (0..504), |  |
| numSubbands-r12 | ns1 | ENUMERATED {ns1, ns2, ns4} |  |
| rb-Offset-r12 | 0 | INTEGER (0..110) |  |
| } |  |  |  |
| dataTxParameters-r12 | 0 | INTEGER (-126..31) |  |
| trpt-Subset-r12 | 010 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| BW5 | when sl-bandwidth-r12 is set to 5 MHz |
| BW10 | when sl-bandwidth-r12 is set to 10 MHz |
| SLCONF1 | SideLink Configuration 1 |
| SLCONF2 | SideLink Configuration 2 (a limited number of Radio Parameters which differ to those in SLCONF1 are provided to allow means for easily recognisable different UE transmissions when UE respects them). |

### 6.8.2	Reference configuration for V2X Sidelink Communication

ROHC compression is not applied in the signalling test cases simulating in coverage and out-of-coverage scenarios.

Ciphering is not configured on SLRB.

#### 6.8.2.1	V2X Sidelink Communication Preconfiguration for out-of-network coverage operation

The V2X Sidelink communication preconfigurations for single V2X component carrier or V2X intra-band contiguous multi-carrier are defined in Table 4.10.1.1-1 and Table 4.10.1.1-2 respectively.

# 7	Test environment for RRM tests

This section contains all the exceptions of the common test parameters specified in clause 4 for specific needs of test cases defined in TS 36.521-3 [34], TS 34.121-1 [50] and TS 34.122 [51]. Exceptions specified in clause 7 overwrite the parameter settings of clause 4; exceptions defined within the test cases overwrite parameter settings of clause 4 and 7.

This clause describes UE test states which can be used in the initial condition for TS 36.521-3 [34], TS 34.121-1 [50] and TS 34.122 [51].

## 7.1	Requirements of test equipment

<void>

## 7.2	RRM Reference system configurations

### 7.2.1	Common parameters for simulated E-UTRA cells

#### 7.2.1.1	Combinations of system information blocks

<void>

#### 7.2.1.2	Scheduling of system information blocks

<void>

#### 7.2.1.3	Common contents of system information messages

- SystemInformationBlockType2

As defined in Table 4.4.3.3-1 with the following exceptions:

Table 7.2.1.3-1: SystemInformationBlockType2 exceptions

| Derivation Path: Clause 4.4.3.3, Table 4.4.3.3-1 SystemInformationBlockType2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| mbsfn-SubframeConfigList SEQUENCE | Not Present | MBSFN not applicable for eMTC RRM Test cases | CEmodeA, CEmodeB |
| mbsfn-SubframeConfigList ::= SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  | FDD |
| radioframeAllocationPeriod | n1 | Every radio frame is with MBSFN subframe |  |
| radioframeAllocationOffset | 0 |  |  |
| subframeAllocation CHOICE { |  |  |  |
| oneFrame | ‘111111’B | Subframe 1, 2, 3, 6, 7, 8 is used for MBSFN |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

- SystemInformationBlockType3

As defined in Table 4.4.3.3-2 with the following exceptions:

Table 7.2.1.3-2: SystemInformationBlockType3 exceptions

| Derivation Path: Clause 4.4.3.3, Table 4.4.3.3-2 SystemInformationBlockType3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| neighCellConfig | ‘10’B (The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell) |  | FDD with E-UTRA FDD neighbour cell |
|  | ‘01’B (No MBSFN sub frames are present in all neighbor cells) |  | CEmodeA, CEmodeB |

| Condition | Explanation |
| --- | --- |
| FDD with E-UTRA FDD neighbour cell | FDD cell environment with a E-UTRA FDD neighbour cell |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

- SystemInformationBlockType5

As defined in Table 4.4.3.3-3 with the following exceptions:

Table 7.2.1.3-3: SystemInformationBlockType5 exceptions

| Derivation Path: Clause 4.4.3.3, Table 4.4.3.3-4 SystemInformationBlockType5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| neighCellConfig[n] | ‘10’B (The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell) |  | FDD with E-UTRA FDD neighbour cell |
|  | ‘11’B (Different UL/DL allocation in neighbouring cells for TDD compared to the serving cell) |  | FDD with E-UTRA TDD neighbour cell, TDD with E-UTRA FDD neighbour cell |
|  | ‘01’B (No MBSFN sub frames are present in all neighbor cells) |  | CEmodeA, CEmodeB |

| Condition | Explanation |
| --- | --- |
| FDD with E-UTRA FDD neighbour cell | FDD cell environment with a E-UTRA FDD neighbour cell |
| FDD with E-UTRA TDD neighbour cell | FDD cell environment with a E-UTRA TDD neighbour cell |
| TDD with E-UTRA FDD neighbour cell | TDD cell environment with a E-UTRA FDD neighbour cell |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

- SystemInformationBlockType7

As defined in Table 4.4.3.3-6 with the following exceptions:

Table 7.2.1.3-4: SystemInformationBlockType7 exceptions

| Derivation Path: Clause 4.4.3.3, Table 4.4.3.3-6 SystemInformationBlockType7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| commonInfo SEQUENCE { |  |  |  |
| p-MaxGERAN | 33 (33 dBm) |  | GSM 400 & GSM 900 & GSM 850 & GSM 700 |
|  | 30 (30 dBm) |  | DCS 1800 & PCS 1900 |
| } |  |  |  |

### 7.2.2	Common parameters for simulated GERAN cells

#### 7.2.2.1	Mapping of GERAN cells

Unless otherwise stated, GERAN cells take the default values defined in Table 7.2.2.1-1.

Table 7.2.2.1-1: Mapping of GERAN cells with TS 51.010-1 [25]

| GERAN cell | Frequency | GERAN cell in TS 51.010-1, clause 40 |
| --- | --- | --- |
| Cell 24 | f11 | Cell A |
| Cell 25 | f12 | Cell D |
| Cell 26 | f13 | Cell B |

## 7.2A	Generic RRM procedures

### 7.2A.1	UE RRM test states

Table 7.2A.1-1: The E-UTRAN UE states

|  |  | RRC | ECM | EMM | ESM | UE Test Mode |
| --- | --- | --- | --- | --- | --- | --- |
| State 2A-RF | Registered, Idle Mode, UE Test Mode Activated | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 |
| State 2A-RF-CE | Registered, Idle Mode, Cell supporting BL/CE UE, UE Test Mode Activated | Refer to Table 5.2A.1-1AA | Refer to Table 5.2A.1-1AA | Refer to Table 5.2A.1-1AA | Refer to Table 5.2A.1-1AA | Refer to Table 5.2A.1-1AA |
| State 3A-RF | Generic Default RB Established, UE Test Mode Activated | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 |
| State 3A-RF-CE | Generic Default RB Established, Cell supporting BL/CE UE, UE Test Mode Activated | Refer to Table 5.2A.2AA | Refer to Table 5.2A.2AA | Refer to Table 5.2A.2AA | Refer to Table 5.2A.2AA | Refer to Table 5.2A.2AA |
| State 3A-RF-DC1 | DC MCG/SCG Dedicated RB established, UE Test Mode Activated | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 |
| State 3A-RF-DC2 | DC Split Default RB established, UE Test Mode Activated | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 | Refer to Table 5.2A.1-1 |
| State 3B-RF | Generic Default RB Established, UE Test Mode Activated, pre-registered on HRPD | RRC_CONNECTED 1 data radio bearer configured | ECM-CONNECTED | EMM-REGISTERED | 1 default EPS bearer context active | Active |

### 7.2A.2	UE Registration, UE Test Mode Activated (State 2A-RF)

As described in clause 5.2A.1A.

### 7.2A.2A	UE Registration, UE Test Mode Activated in cell supporting BL/CE UE (State 2A-RF-CE)

As described in clause 5.2A.1AA.

### 7.2A.3	Generic Default Radio Bearer Establishment, UE Test Mode Activated (State 3A-RF)

As described in clause 5.2A.2.

### 7.2A.3A	DC MCG/SCG Dedicated RB established, UE Test Mode Activated (State 3A-RF-DC1)

As described in clause 5.2A.2A.

### 7.2A.3AA	Generic Default Radio Bearer Establishment, UE Test Mode Activated in cell supporting BL/CE UE (State 3A-RF-CE)

As described in clause 5.2A.2AA.

### 7.2A.3B	DC Split Default RB established, UE Test Mode Activated (State 3A-RF-DC2)

As described in clause 5.2A.2B.

### 7.2A.3C	Generic Default Radio Bearer Establishment, UE Test Mode Activated, V2X Setup (State 3A-RF-V2X)

As described in clause 5.2A.2C.

### 7.2A.4	Generic Default Radio Bearer Establishment, UE Test Mode Activated, pre-registration on HRPD (State 3B-RF)

Editor’s note: This section is incomplete. The following aspects are either missing or not yet determined:
Other than UATI Request message and UATI Assignment message are FFS

#### 7.2A.4.1	Initial conditions

System Simulator:

-	Cell 1

-	Cell 1 is transmitting SystemInformationBlockType8

User Equipment:

- 	The Test USIM shall be inserted.

#### 7.2A.4.2	Definition of system information messages

As described in clause 4.5.2B.2.

#### 7.2A.4.3	Procedure

Table 7.2A.4.3-1: UE registration with default EPS bearer establishment, test mode activation and HRPD pre-registration procedures (state 1 to state 3B-RF)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 to 18 | Steps 1 to 18 as specified in the procedure in clause 5.2A.1A.3 take place. | - | - |
| 19 to 36 | Same procedure for steps 17 to 30D as specified in the procedure in clause 4.5.2B.3. |  |  |

#### 7.2A.4.4	Specific message contents

As described in clauses 4.5.2B.4 and 7.2A.2.

### 7.2A.5	Procedure to configure SCC

As described in clause 5.2A.4.

### 7.2A.6	Exceptions for feICIC tests

As described in clause 5.2A.5.

## 7.2B	Other generic RRM procedures

### 7.2B.1	Tracking area updating procedure

The procedure is defined in table 7.2B.1-1.

Table 7.2B.1-1: Tracking area updating procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits system information on the cell specified in the test case. | <-- | RRC: SYSTEM INFORMATION (BCCH, BCCH-BR) Note 5. |
| - | EXCEPTION: If all EPS bearer contexts are inactive in UE, for Rel-8 and Rel-9, steps 2 to 8a are optional and depend on UE implementation. If all EPS bearer contexts are inactive in UE, a Rel-10 or higher UE directly starts attach procedure from step 9a. | - | - |
| 2 | The UE transmits an RRCConnectionRequest message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest |
| 3 | The SS transmits an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| 4 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete NAS: TRACKING AREA UPDATE REQUEST |
| - | EXCEPTION: If all EPS bearer contexts are marked as inactive in the EPS bearer context status IE included in the TRACKING AREA UPDATE REQUEST message then, events described in steps 5a to 26a. Otherwise, events described in steps 5 to 7. | - | - |
| 5 | The SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE ACCEPT |
| 6 | The UE transmits a TRACKING AREA UPDATE COMPLETE | --> | RRC: ULInformationTransfer NAS: TRACKING AREA UPDATE COMPLETE |
| 7 | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease |
| 5a | The SS responds with TRACKING AREA UPDATE REJECT message with cause #40 (No EPS bearer context activated) to force attach the UE. | <-- | RRC: DLInformationTransfer NAS: TRACKING AREA UPDATE REJECT |
| 6a | EXCEPTION: Step 7a describes the behaviour that depends on UE behaviour (Note 3). | - | - |
| 7a | The SS transmits an RRCConnectionRelease message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease |
| - | EXCEPTION: Step 8a describes a behaviour which depends on the UE capability | - | - |
| 8a | IF NOT pc_Auto_PDN_Connectivity, the user initiates an attach by MMI or by AT command (NOTE 4). | - | - |
| 9a to 27a | Steps 2 to 20 of the generic UE Registration procedure (5.2A.1A.3) take place to complete the Attach procedure | - | - |
| NOTE 1: The periodic tracking area updating timer T3412 is deactivated by default during the attach procedure (TS 36.508 clause 4.7.2). NOTE 2: The SS does not initiate authentication and NAS SECURITY MODE COMMAND are not performed (reuse of keys allocated during the attach procedure). NOTE 3: The SS waits for 1.5 second to receive the Attach Request on the existing RRC Connection. In case Attach Request is not received within 1.5 second, existing RRC Connection is released. NOTE 4: The request is assumed to be triggered by AT command AT+CGDCONT=1,"IP" followed by AT+CGACT=1. NOTE 5: For BL UE testing and for non-BL UE enhanced coverage testing the SS shall broadcast SystemInformation-BR-r13, SystemInformationBlockType1-BR-r13 and other relevant system information blocks as required by the specified combinations of system information blocks specified for the test case on BCCH-BR. For non-BL UE normal coverage testing the SS may exclude broadcasting of system information on BCCH-BR. |  |  |  |

Specific message contents

Table 7.2B.1-1a: MasterInformationBlock (Step 1 and following steps)

Derivation Path: 36.508 Table 4.4.3.2-1 using condition CEmodeB if the test case specifically tests CE mode B else use condition CEmodeA.

Table 7.2B.1-1a1: SystemInformation-BR-r13 (Step 1 and following steps)

Derivation Path: 36.508 Table 4.4.3.2-2A.

Table 7.2B.1-1a2: SystemInformationBlockType1-BR-r13 (Step 1 and following steps)

Derivation Path: 36.508 Table 4.4.3.2-3A.

Table 7.2B.1-2: TRACKING AREA UPDATE ACCEPT (Step 5)

| Derivation Path: 36.508 Table 4.7.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS network feature support | ‘0000 0000'B | IMS voice over PS session in S1 mode not supported |  |
| Additional update result | Not present |  |  |

## 7.3	Default RRC message and information elements contents

### 7.3.1	Contents of RRC messages

FFS.

### 7.3.2	Radio resource control information elements

Editor’s note: Unless otherwise stated in the test parameters or messages exceptions in the respective test cases in TS 36.521-3 [34], PRACH configuration index and exceptional message as specified in Table 7.3.2-1 is used.

As defined in clause 4.6.3 with the following exceptions:

Table 7.3.2-1: PRACH-Config-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PRACH-Config-DEFAULT ::= SEQUENCE { |  |  |  |
| prach-ConfigIndex | 3 |  | TDD |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TDD | TDD cell environment |

Table 7.3.2-2: RadioResourceConfigCommonSCell-r10-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-13A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSCell-r10 ::= SEQUENCE { |  |  |  |
| mbsfn-SubframeConfigList-r10 SEQUENCE (SIZE (1..maxMBSFN-Allocations)) OF SEQUENCE { |  |  | FDD |
| radioframeAllocationPeriod | n1 | Every radio frame is with MBSFN subframe |  |
| radioframeAllocationOffset | 0 |  |  |
| subframeAllocation CHOICE { |  |  |  |
| oneFrame | ‘111111’B | Subframe 1, 2, 3, 6, 7, 8 is used for MBSFN |  |
| } |  |  |  |
| } |  |  |  |
| ul-Configuration-r10 | Not Present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| FDD | FDD cell environment |

Table 7.3.2-3: PhysicalConfigDedicatedSCell-r10-DEFAULT

| Derivation Path: Clause 4.6.3 Table 4.6.3-6A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicatedSCell-r10 ::= SEQUENCE { |  |  |  |
| ul-Configuration-r10 | Not Present |  |  |
| } |  |  |  |

### 7.3.3	Measurement information elements

As defined in clause 4.6.6 with the following exceptions:

Table 7.3.3-0: MeasDS-Config-DEFAULT

| Derivation Path: clause 4.6.6 Table 4.6.6-1B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasDS-Config-DEFAULT ::= CHOICE { |  |  |  |
| release | NULL |  |  |
| setup SEQUENCE { |  |  |  |
| dmtc-PeriodOffset-r12 CHOICE { |  |  |  |
| ms40-r12 | 10 |  | LAA_MeasProc |
| ms40-r12 | 0 |  | LAA_MeasPerf |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| LAA_MeasProc | Used for LAA measurements procedures test cases |
| LAA_MeasPerf | Used for LAA measurement performance test cases |

Table 7.3.3-1: MeasObjectEUTRA-GENERIC (Freq)

| Derivation Path: Clause 4.6.6, Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| neighCellConfig | 10'B (The MBSFN subframe allocations of all neighbour cells are identical to or subsets of that in the serving cell) |  | FDD with E-UTRA FDD neighbour cell |
|  | ‘11’B (Different UL/DL allocation in neighbouring cells for TDD compared to the serving cell) |  | FDD with E-UTRA TDD neighbour cell, TDD with E-UTRA FDD neighbour cell |
|  | ‘01’B (No MBSFN sub frames are present in all neighbor cells) |  | CEmodeA, CEmodeB |

| Condition | Explanation |
| --- | --- |
| FDD with E-UTRA FDD neighbour cell | FDD cell environment with a E-UTRA FDD neighbour cell |
| FDD with E-UTRA TDD neighbour cell | FDD cell environment with a E-UTRA TDD neighbour cell |
| TDD with E-UTRA FDD neighbour cell | TDD cell environment with a E-UTRA FDD neighbour cell |
| CEmodeA | CE mode A test environment |
| CEmodeB | CE mode B test environment |

## 7.3A	Default UTRA message and information element contents

### 7.3A.1	UTRA RRC messages

As defined in 4.7B with the following exceptions:

Contents of RRC CONNECTION SETUP message: UM

| Information Element | Condition | Value/remark | Version | Index |
| --- | --- | --- | --- | --- |
| - System specific capability update requirement list |  | 1 entry |  | RCSU-019 |
| - System specific capability update requirement |  | GSM |  | RCSU-019a |
| CHOICE specification mode |  | Complete specification | Rel-5 | RCSU-020 |
| - Complete specification |  |  | Rel-5 | RCSU-021 |

## 7.4	Default NAS message and information elements contents

## 7.5	Reference radio bearer configurations

### 7.5.1	SRB and DRB parameters

#### 7.5.1.1	MAC configurations

As defined in clause 4.8.2.1.5 with the following exceptions:

Table 7.5.1.1-1: MAC-MainConfig-RBC

| Derivation Path: Clause 4.8.2.1.5 Table 4.8.2.1.5-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-RBC ::= SEQUENCE { |  |  |  |
| mac-MainConfig-v1020SEQUENCE { |  |  | SCell_AddMod |
| sCellDeactivationTimer-r10 | Not present |  |  |
| extendedBSR-Sizes-r10 | Not Present |  |  |
| extendedPHR-r10 | Not Present |  |  |
|  | Setup |  | UL CA |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SCell_AddMod | Addition or modification of Scell |
| UL CA | This condition is used for UL CA. |

## 7.6	Test environment for NTN testing

This section defines the test environment which applies to all RRM test cases executed for IoT NTN UEs, unless otherwise specified.

### 7.6.1	UE location

Same as in section 5.6.1.

### 7.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 7.6.2.0	Assumptions for Ephemeris generation

Same as in section 5.6.2.0.

#### 7.6.2.1	Ephemeris for elevation angle 30º

All eMTC NTN RRM test cases defined in section 14 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 7.6.2.1-1a to 7.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites or both GSO and NGSO satellites, ephemeris values in Table 7.6.2.1-3a or 7.6.2.1-3b shall be used.

-	Tables 7.6.2-4a to 7.6.2-4d shall only be used if the test case under test is only applicable to GEO satellites.

Table 7.6.2.1-1a: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-1b: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 8393544277 |  |  |  |  |  |
| eccentricity-r17 |  | 0 |  |  |  |  |  |
| periapsis-r17 |  | 266532360 |  |  |  |  |  |
| longitude-r17 |  | 224998499 |  |  |  |  |  |
| inclination-r17 |  | 2125399 |  |  |  |  |  |
| meanAnomaly-r17 |  | 118496066 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-2a: Void

Table 7.6.2.1-2b: Void

Table 7.6.2.1-3a: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2935552 |  |  |  |  |  |
| positionY-r17 |  | 4895460 |  |  |  |  |  |
| positionZ-r17 |  | 1188458 |  |  |  |  |  |
| velocityVX-r17 |  | 8506 |  |  |  |  |  |
| velocityVY-r17 |  | -23314 |  |  |  |  |  |
| velocityVZ-r17 |  | 118253 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-3b: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 273626880 |  |  |  |  |  |
| eccentricity-r17 |  | 769300 |  |  |  |  |  |
| periapsis-r17 |  | 198830479 |  |  |  |  |  |
| longitude-r17 |  | 89834718 |  |  |  |  |  |
| inclination-r17 |  | 65473992 |  |  |  |  |  |
| meanAnomaly-r17 |  | 7822790 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 7.6.2.1-4a: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-4b: SystemInformationBlockType31 – eMTC NTN Serving cell Ephemeris Information orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions

| Derivation Path: Table 7.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 200160368 |  |  |
| longitude-r17 | 24831613 |  |  |
| inclination-r17 | 5309231 |  |  |
| meanAnomaly-r17 | 66742914 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-4c: SystemInformationBlockType33 (Rel-18)/Test equipment configuration (Rel-17) – eMTC NTN Neighbour cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 7.6.2.1-4d: SystemInformationBlockType33 (Rel-18)/Test equipment configuration (Rel-17) – eMTC NTN Neighbour cell Ephemeris Information (orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 7.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 ::= CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

#### 7.6.2.2	Ephemeris for eMTC NTN RRM UL timing accuracy test cases

Table 7.6.2.2-1: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information for GSO satellites (minimum Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -16944523 |  |  |  |  |  |
| positionY-r17 |  | 27362209 |  |  |  |  |  |
| positionZ-r17 |  | 4019393 |  |  |  |  |  |
| velocityVX-r17 |  | -336 |  |  |  |  |  |
| velocityVY-r17 |  | -208 |  |  |  |  |  |
| velocityVZ-r17 |  | 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |  |  |  |  |

Table 7.6.2.2-2: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2771542 |  |  |  |  |  |
| positionY-r17 |  | 4509243 |  |  |  |  |  |
| positionZ-r17 |  | 2455572 |  |  |  |  |  |
| velocityVX-r17 |  | 20837 |  |  |  |  |  |
| velocityVY-r17 |  | -46176 |  |  |  |  |  |
| velocityVZ-r17 |  | 109528 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |  |  |  |  |

Table 7.6.2.2-3: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -16989970 |  |  |  |  |  |
| positionY-r17 |  | 27591785 |  |  |  |  |  |
| positionZ-r17 |  | 1413211 |  |  |  |  |  |
| velocityVX-r17 |  | 390 |  |  |  |  |  |
| velocityVY-r17 |  | -65 |  |  |  |  |  |
| velocityVZ-r17 |  | 5945 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |  |  |  |  |

Table 7.6.2.2-4: SystemInformationBlockType31-eMTC NTN Serving cell Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2654549 |  |  |  |  |  |
| positionY-r17 |  | 4386991 |  |  |  |  |  |
| positionZ-r17 |  | 1594205 |  |  |  |  |  |
| velocityVX-r17 |  | 14581 |  |  |  |  |  |
| velocityVY-r17 |  | -34487 |  |  |  |  |  |
| velocityVZ-r17 |  | 120182 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |  |  |  |  |

Table 7.6.2.2-5: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information for GSO satellites (maximum negative Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |  |  |  |  |

Table 7.6.2.2-6: SystemInformationBlockType31- eMTC NTN Serving cell Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: Table 7.6.3.1-1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2401021 |  |  |  |  |  |
| positionY-r17 |  | 3830217 |  |  |  |  |  |
| positionZ-r17 |  | 2907041 |  |  |  |  |  |
| velocityVX-r17 |  | 28879 |  |  |  |  |  |
| velocityVY-r17 |  | -61410 |  |  |  |  |  |
| velocityVZ-r17 |  | 105774 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -0-19.95 ppm |  |  |  |  |  |  |  |

### 7.6.3	eMTC NTN message contents for RRM tests

#### 7.6.3.1	ServingSatelliteInfo-r17 values in SIB31

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31 shall be periodically signalled to the UE during test based on the Ephemeris information in clause 7.6.2 (kept constant throughout the duration of the test).

Table 7.6.3.1-1: ServingSatelliteInfo-r17 for RRM tests (Serving cell)

| Derivation Path: Table 4.4.3.3-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | TC_EF_SV |
| positionX-r17 | See cl. 7.6.2 |  |  |
| positionY-r17 | See cl. 7.6.2 |  |  |
| positionZ-r17 | See cl. 7.6.2 |  |  |
| velocityVX-r17 | See cl. 7.6.2 |  |  |
| velocityVY-r17 | See cl. 7.6.2 |  |  |
| velocityVZ-r17 | See cl. 7.6.2 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | TC_EF_OP |
| semiMajorAxis-r17- | See cl. 7.6.2 |  |  |
| eccentricity-r17 | See cl. 7.6.2 |  |  |
| periapsis-r17 | See cl. 7.6.2 |  |  |
| longitude-r17 | See cl. 7.6.2 |  |  |
| inclination-r17 | See cl. 7.6.2 |  |  |
| meanAnomaly-r17 | See cl. 7.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| } |  |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TC_EF_SV | Test cases using ephemeris with Position Velocity format according to section 7.6.2 |
| TC_EF_OP | Test cases using ephemeris with Orbital format according to section 7.6.2 |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

#### 7.6.3.2	SystemInformationBlockType33

Table 7.6.3.2-1: SystemInformationBlockType33 (Rel-18)/Test equipment configuration (Rel-17)

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| satelliteId-r18 | 1 |  |  |
| ephemerisInfo-r18 | See cl. 7.6.2 |  |  |
| nta-CommonParameters SEQUENCE { |  |  |  |
| nta-Common-r18 | 0 |  |  |
| nta-CommonDrift-r18 | 0 |  |  |
| nta-CommonDriftVariation-r18 | 0 |  |  |
| } |  |  |  |
| epochTime-r18 | Not present |  |  |
| k-Mac-r18 | Not present |  |  |
| t-ServiceStartNeigh-r18 | Not present |  |  |
| } |  |  |  |
| neighValidityDuration-r18 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| NOTE 1: In Rel-17, SIB33 is not signalled to the device but information in this table can be used to configure test equipment. |  |  |  |

# 8	NB-IoT test environment

## 8.1	NB-IoT Common test environment

### 8.1.1	NB-IoT Environmental conditions

Same environmental conditions as the ones described in section 4.1 apply to NB-IoT UEs.

### 8.1.2	NB-IoT Common requirements of test equipment

Same common requirements of test equipment as the ones described in section 4.2 apply to NB-IoT with the following exceptions:

-	Only FDD Mode is required for Rel-13 and Rel-14 NB-IoT

-	Both FDD and TDD Mode are required for Rel-15 and forward NB-IoT

-	Only FDD Mode is required for Rel-17 and forward NB-IoT NTN

-	Supported physical channels for NB-IoT are:

| Physical channel | Minimum number | Comments |
| --- | --- | --- |
| NPBCH | 1 | Narrowband Physical Broadcast Channel |
| NPDCCH | 1 | The Narrowband Physical Downlink Control channel carries control information. |
| NPDSCH | 1 | Narrowband Physical Downlink Shared Channel |
| NPUSCH | 1 | Narrowband Physical Uplink Shared Channel |
| NPRACH | 1 | Narrowband Physical Random Access Channel |

-	Supported physical signals for NB-IoT are:

| Physical signal | Minimum number | Comments |
| --- | --- | --- |
| Narrowband Reference Signal | NA | DL |
| Narrowband Primary Synchronization Signal | NA | DL |
| Narrowband Secondary Synchronization Signal | NA | DL |
| Demodulation Reference Signal | NA | UL |
| Wake-Up Signal | NA | DL |

### 8.1.3	NB-IoT Reference test conditions

This clause contains the reference test conditions, which apply to all NB-IoT test cases unless otherwise specified.

#### 8.1.3.1	NB-IoT Test frequencies

NB-IoT is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 42, 43, 65, 66, 70, 71, 72, 73, 74, 85, 87, 88 and 103 which are defined in Table 5.5-1 in [21]. For satellite access (NTN), NB-IoT is designed to operate in the E-UTRA operating bands 253, 254, 255 and 256 which are defined in Table 5.2-1 in [74]. NB-IoT system operates in HD-FDD and TDD duplex mode or in TDD mode.

The test frequencies are based on the E-UTRA frequency bands defined in the core specifications.

The raster spacing is 100 kHz.

E-UTRA/FDD is designed to operate in paired bands of 3GPP TS 36.101 [27] and TS 36.102 [74]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.1.

E-UTRA/TDD is designed to operate in unpaired bands of 3GPP TS 36.101 [27]. The reference test frequencies for the RF and Signalling test environment for each of the operating bands are defined in sub clause 8.1.3.1.2.

USA & Canada emission requirements for specific operating bands are indicated by network signalling value NS_04 on indicated bands in clause 8.1.3.1.1. The normative reference for this requirement is TS 36.101 [27] clause 5.5F.

##### 8.1.3.1.1	NB-IoT FDD Mode Test frequencies

8.1.3.1.1.1	NB-IoT FDD reference test frequencies for operating band 1

Table 8.1.3.1.1.1-1: NB-IoT standalone Test frequencies for operating band 1

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 18001 | 0 | 1920.1 | 1 | -0.5 | 2110.1 |
| Mid Range | 18300 | 0 | 1950.0 | 300 | -0.5 | 2140.0 |
| High Range | 18599 | 0 | 1979.9 | 599 | -0.5 | 2169.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.1-2: NB-IoT in-band Test frequencies for operating band 1

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 18007 | 0 | 1920.7000 | 7 | -2 | 2110.6925 |
| Low Range2 | 18060 | -2 | 1925.9900 | 60 | -1 | 2115.9975 |
| Low Range3 | 18069 | -2 | 1926.8900 | 69 | -1 | 2116.8975 |
| Mid Range1 | 18291 | 0 | 1949.1000 | 291 | -2 | 2139.0925 |
| Mid Range2 | 18310 | -2 | 1950.9900 | 310 | -1 | 2140.9975 |
| Mid Range3 | 18319 | -2 | 1951.8900 | 319 | -1 | 2141.8975 |
| High Range1 | 18593 | 0 | 1979.3000 | 593 | 1 | 2169.3075 |
| High Range2 | 18560 | -2 | 1975.9900 | 560 | -1 | 2165.9975 |
| High Range3 | 18569 | -2 | 1976.8900 | 569 | -1 | 2166.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.1-3: NB-IoT guard-band Test frequencies for operating band 1

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 18001 | 0 | 1920.1000 | 1 | 1 | 2110.1075 |
| Mid Range | 5 | 18276 | 0 | 1947.6000 | 276 | 1 | 2137.6075 |
|  | 10 | 18346 | 0 | 1954.6000 | 346 | 1 | 2144.6075 |
| High Range | Any | 18599 | 0 | 1979.9000 | 599 | -2 | 2169.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.2	NB-IoT FDD reference test frequencies for operating band 2

Table 8.1.3.1.1.2-1: NB-IoT standalone Test frequencies for operating band 2

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 18601 | 0 | 1850.1 | 601 | -0.5 | 1930.1 |
| Mid Range | 18900 | 0 | 1880.0 | 900 | -0.5 | 1960.0 |
| High Range | 19199 | 0 | 1909.9 | 1199 | -0.5 | 1989.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-1a: NB-IoT standalone Test frequencies for operating band 2 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 18602 | 0 | 1850.2 | 602 | -0.5 | 1930.2 |
| Mid Range | 18900 | 0 | 1880.0 | 900 | -0.5 | 1960.0 |
| High Range | 19198 | 0 | 1909.8 | 1198 | -0.5 | 1989.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-2: NB-IoT in-band Test frequencies for operating band 2

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 18606 | 0 | 1850.6000 | 606 | -2 | 1930.5925 |
| Low Range2 | 18660 | -2 | 1855.9900 | 660 | -1 | 1935.9975 |
| Low Range3 | 18669 | -2 | 1856.8900 | 669 | -1 | 1936.8975 |
| Mid Range1 | 18891 | 0 | 1879.1000 | 891 | -2 | 1959.0925 |
| Mid Range2 | 18910 | -2 | 1880.9900 | 910 | -1 | 1960.9975 |
| Mid Range3 | 18919 | -2 | 1881.8900 | 919 | -1 | 1961.8975 |
| High Range1 | 19194 | 0 | 1909.4000 | 1194 | 1 | 1989.4075 |
| High Range2 | 19160 | -2 | 1905.9900 | 1160 | -1 | 1985.9975 |
| High Range3 | 19169 | -2 | 1906.8900 | 1169 | -1 | 1986.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-2a: NB-IoT in-band Test frequencies for operating band 2 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 18606 | 0 | 1850.6000 | 606 | -2 | 1930.5925 |
| Low Range2 | 18660 | -2 | 1855.9900 | 660 | -1 | 1935.9975 |
| Low Range3 | 18669 | -2 | 1856.8900 | 669 | -1 | 1936.8975 |
| Mid Range1 | 18891 | 0 | 1879.1000 | 891 | -2 | 1959.0925 |
| Mid Range2 | 18910 | -2 | 1880.9900 | 910 | -1 | 1960.9975 |
| Mid Range3 | 18919 | -2 | 1881.8900 | 919 | -1 | 1961.8975 |
| High Range1 | 19194 | 0 | 1909.4000 | 1194 | 1 | 1989.4075 |
| High Range2 | 19160 | -2 | 1905.9900 | 1160 | -1 | 1985.9975 |
| High Range3 | 19169 | -2 | 1906.8900 | 1169 | -1 | 1986.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.2-3: NB-IoT guard-band Test frequencies for operating band 2

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 18601 | 0 | 1850.1000 | 601 | 1 | 1930.1075 |
| Mid Range | 5 | 18876 | 0 | 1877.6000 | 876 | 1 | 1957.6075 |
|  | 10 | 18946 | 0 | 1884.6000 | 946 | 1 | 1964.6075 |
| High Range | Any | 19199 | 0 | 1909.9000 | 1199 | -2 | 1989.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.2-3a: NB-IoT guard-band Test frequencies for operating band 2 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 18602 | 0 | 1850.2000 | 602 | 1 | 1930.2075 |
| Mid Range | 5 | 18876 | 0 | 1877.6000 | 876 | 1 | 1957.6075 |
|  | 10 | 18946 | 0 | 1884.6000 | 946 | 1 | 1964.6075 |
| High Range | Any | 19198 | 0 | 1909.8000 | 1198 | -2 | 1989.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.3	NB-IoT FDD reference test frequencies for operating band 3

Table 8.1.3.1.1.3-1: NB-IoT standalone Test frequencies for operating band 3

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 19201 | 0 | 1710.1 | 1201 | -0.5 | 1805.1 |
| Mid Range | 19575 | 0 | 1747.5 | 1575 | -0.5 | 1842.5 |
| High Range | 19949 | 0 | 1784.9 | 1949 | -0.5 | 1879.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.3-2: NB-IoT in-band Test frequencies for operating band 3

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 19206 | 0 | 1710.6000 | 1206 | -2 | 1805.5925 |
| Low Range2 | 19260 | -2 | 1715.9900 | 1260 | -1 | 1810.9975 |
| Low Range3 | 19269 | -2 | 1716.8900 | 1269 | -1 | 1811.8975 |
| Mid Range1 | 19566 | 0 | 1746.6000 | 1566 | -2 | 1841.5925 |
| Mid Range2 | 19585 | -2 | 1748.4900 | 1585 | -1 | 1843.4975 |
| Mid Range3 | 19594 | -2 | 1749.3900 | 1594 | -1 | 1844.3975 |
| High Range1 | 19944 | 0 | 1784.4000 | 1944 | 1 | 1879.4075 |
| High Range2 | 19910 | -2 | 1780.9900 | 1910 | -1 | 1875.9975 |
| High Range3 | 19919 | -2 | 1781.8900 | 1919 | -1 | 1876.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.3-3: NB-IoT guard-band Test frequencies for operating band 3

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 19201 | 0 | 1710.1000 | 1201 | 1 | 1805.1075 |
| Mid Range | 5 | 19551 | 0 | 1745.1000 | 1551 | 1 | 1840.1075 |
|  | 10 | 19621 | 0 | 1752.1000 | 1621 | 1 | 1847.1075 |
| High Range | Any | 19949 | 0 | 1784.9000 | 1949 | -2 | 1879.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.4	NB-IoT FDD reference test frequencies for operating band 4

Table 8.1.3.1.1.4-1: NB-IoT standalone Test frequencies for operating band 4

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 19951 | 0 | 1710.1 | 1951 | -0.5 | 2110.1 |
| Mid Range | 20175 | 0 | 1732.5 | 2175 | -0.5 | 2132.5 |
| High Range | 20399 | 0 | 1754.9 | 2399 | -0.5 | 2154.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-1a: NB-IoT standalone Test frequencies for operating band 4 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 19952 | 0 | 1710.2 | 1952 | -0.5 | 2110.2 |
| Mid Range | 20175 | 0 | 1732.5 | 2175 | -0.5 | 2132.5 |
| High Range | 20398 | 0 | 1754.8 | 2398 | -0.5 | 2154.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-2: NB-IoT in-band Test frequencies for operating band 4

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 19957 | 0 | 1710.7000 | 1957 | -2 | 2110.6925 |
| Low Range2 | 20010 | -2 | 1715.9900 | 2010 | -1 | 2115.9975 |
| Low Range3 | 20019 | -2 | 1716.8900 | 2019 | -1 | 2116.8975 |
| Mid Range1 | 20166 | 0 | 1731.6000 | 2166 | -2 | 2131.5925 |
| Mid Range2 | 20185 | -2 | 1733.4900 | 2185 | -1 | 2133.4975 |
| Mid Range3 | 20194 | -2 | 1734.3900 | 2194 | -1 | 2134.3975 |
| High Range1 | 20393 | 0 | 1754.3000 | 2393 | 1 | 2154.3075 |
| High Range2 | 20360 | -2 | 1750.9900 | 2360 | -1 | 2150.9975 |
| High Range3 | 20369 | -2 | 1751.8900 | 2369 | -1 | 2151.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-2a: NB-IoT in-band Test frequencies for operating band 4 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 19957 | 0 | 1710.7000 | 1957 | -2 | 2110.6925 |
| Low Range2 | 20010 | -2 | 1715.9900 | 2010 | -1 | 2115.9975 |
| Low Range3 | 20019 | -2 | 1716.8900 | 2019 | -1 | 2116.8975 |
| Mid Range1 | 20166 | 0 | 1731.6000 | 2166 | -2 | 2131.5925 |
| Mid Range2 | 20185 | -2 | 1733.4900 | 2185 | -1 | 2133.4975 |
| Mid Range3 | 20194 | -2 | 1734.3900 | 2194 | -1 | 2134.3975 |
| High Range1 | 20393 | 0 | 1754.3000 | 2393 | 1 | 2154.3075 |
| High Range2 | 20360 | -2 | 1750.9900 | 2360 | -1 | 2150.9975 |
| High Range3 | 20369 | -2 | 1751.8900 | 2369 | -1 | 2151.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.4-3: NB-IoT guard-band Test frequencies for operating band 4

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 19951 | 0 | 1710.1000 | 1951 | 1 | 2110.1075 |
| Mid Range | 5 | 20151 | 0 | 1730.1000 | 2151 | 1 | 2130.1075 |
|  | 10 | 20221 | 0 | 1737.1000 | 2221 | 1 | 2137.1075 |
| High Range | Any | 20399 | 0 | 1754.9000 | 2399 | -2 | 2154.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.4-3a: NB-IoT guard-band Test frequencies for operating band 4 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 19952 | 0 | 1710.2000 | 1952 | 1 | 2110.2075 |
| Mid Range | 5 | 20151 | 0 | 1730.1000 | 2151 | 1 | 2130.1075 |
|  | 10 | 20221 | 0 | 1737.1000 | 2221 | 1 | 2137.1075 |
| High Range | Any | 20398 | 0 | 1754.8000 | 2398 | -2 | 2154.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.5	NB-IoT FDD reference test frequencies for operating band 5

Table 8.1.3.1.1.5-1: NB-IoT standalone Test frequencies for operating band 5

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20401 | 0 | 824.1 | 2401 | -0.5 | 869.1 |
| Mid Range | 20525 | 0 | 836.5 | 2525 | -0.5 | 881.5 |
| High Range | 20649 | 0 | 848.9 | 2649 | -0.5 | 893.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-1a: NB-IoT standalone Test frequencies for operating band 5 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20402 | 0 | 824.2 | 2402 | -0.5 | 869.2 |
| Mid Range | 20525 | 0 | 836.5 | 2525 | -0.5 | 881.5 |
| High Range | 20648 | 0 | 848.8 | 2648 | -0.5 | 893.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-2: NB-IoT in-band Test frequencies for operating band 5

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20406 | 0 | 824.6000 | 2406 | -2 | 869.5925 |
| Low Range2 | 20460 | -2 | 829.9900 | 2460 | -1 | 874.9975 |
| Low Range3 | 20469 | -2 | 830.8900 | 2469 | -1 | 875.8975 |
| Mid Range1 | 20516 | 0 | 835.6000 | 2516 | -2 | 880.5925 |
| Mid Range2 | 20535 | -2 | 837.4900 | 2535 | -1 | 882.4975 |
| Mid Range3 | 20544 | -2 | 838.3900 | 2544 | -1 | 883.3975 |
| High Range1 | 20644 | 0 | 848.4000 | 2644 | 1 | 893.4075 |
| High Range2 | 20610 | -2 | 844.9900 | 2610 | -1 | 889.9975 |
| High Range3 | 20619 | -2 | 845.8900 | 2619 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-2a: NB-IoT in-band Test frequencies for operating band 5 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20406 | 0 | 824.6000 | 2406 | -2 | 869.5925 |
| Low Range2 | 20460 | -2 | 829.9900 | 2460 | -1 | 874.9975 |
| Low Range3 | 20469 | -2 | 830.8900 | 2469 | -1 | 875.8975 |
| Mid Range1 | 20516 | 0 | 835.6000 | 2516 | -2 | 880.5925 |
| Mid Range2 | 20535 | -2 | 837.4900 | 2535 | -1 | 882.4975 |
| Mid Range3 | 20544 | -2 | 838.3900 | 2544 | -1 | 883.3975 |
| High Range1 | 20644 | 0 | 848.4000 | 2644 | 1 | 893.4075 |
| High Range2 | 20610 | -2 | 844.9900 | 2610 | -1 | 889.9975 |
| High Range3 | 20619 | -2 | 845.8900 | 2619 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.5-3: NB-IoT guard-band Test frequencies for operating band 5

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 20401 | 0 | 824.1000 | 2401 | 1 | 869.1075 |
| Mid Range | 5 | 20501 | 0 | 834.1000 | 2501 | 1 | 879.1075 |
|  | 10 | 20571 | 0 | 841.1000 | 2571 | 1 | 886.1075 |
| High Range | Any | 20649 | 0 | 848.9000 | 2649 | -2 | 893.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.5-3a: NB-IoT guard-band Test frequencies for operating band 5 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 20402 | 0 | 824.2000 | 2402 | 1 | 869.2075 |
| Mid Range | 5 | 20501 | 0 | 834.1000 | 2501 | 1 | 879.1075 |
|  | 10 | 20571 | 0 | 841.1000 | 2571 | 1 | 886.1075 |
| High Range | Any | 20648 | 0 | 848.8000 | 2648 | -2 | 893.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.6	Void

8.1.3.1.1.7	NB-IoT FDD reference test frequencies for operating band 7

Table 8.1.3.1.1.7-1: NB-IoT standalone Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 20751 | 0 | 2500.1 | 2751 | -0.5 | 2620.1 |
| Mid Range | 21100 | 0 | 2535.0 | 3100 | -0.5 | 2655.0 |
| High Range | 21449 | 0 | 2569.9 | 3449 | -0.5 | 2689.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.7-2: NB-IoT in-band Test frequencies for operating band 7

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 20757 | 0 | 2500.7000 | 2757 | -2 | 2620.6925 |
| Low Range2 | 20810 | -2 | 2505.9900 | 2810 | -1 | 2625.9975 |
| Low Range3 | 20819 | -2 | 2506.8900 | 2819 | -1 | 2626.8975 |
| Mid Range | 21091 | 0 | 2534.1000 | 3091 | -2 | 2654.0925 |
| Mid Range2 | 21110 | -2 | 2535.9900 | 3110 | -1 | 2655.9975 |
| Mid Range3 | 21119 | -2 | 2536.8900 | 3119 | -1 | 2656.8975 |
| High Range | 21443 | 0 | 2569.3000 | 3443 | 1 | 2689.3075 |
| High Range2 | 21410 | -2 | 2565.9900 | 3410 | -1 | 2685.9975 |
| High Range3 | 21419 | -2 | 2566.8900 | 3419 | -1 | 2686.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.7-3: NB-IoT guard-band Test frequencies for operating band 7

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 20751 | 0 | 2500.1000 | 2751 | 1 | 2620.1075 |
| Mid Range | 5 | 21076 | 0 | 2532.6000 | 3076 | 1 | 2652.6075 |
|  | 10 | 21146 | 0 | 2539.6000 | 3146 | 1 | 2659.6075 |
| High Range | Any | 21449 | 0 | 2569.9000 | 3449 | -2 | 2689.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.8	NB-IoT FDD reference test frequencies for operating band 8

Table 8.1.3.1.1.8-1: NB-IoT standalone Test frequencies for operating band 8

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 21451 | 0 | 880.1 | 3451 | -0.5 | 925.1 |
| Mid Range | 21625 | 0 | 897.5 | 3625 | -0.5 | 942.5 |
| High Range | 21799 | 0 | 914.9 | 3799 | -0.5 | 959.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.8-2: NB-IoT in-band Test frequencies for operating band 8

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 21456 | 0 | 880.6000 | 3456 | -2 | 925.5925 |
| Low Range2 | 21510 | -2 | 885.9900 | 3510 | -1 | 930.9975 |
| Low Range3 | 21519 | -2 | 886.8900 | 3519 | -1 | 931.8975 |
| Mid Range | 21616 | 0 | 896.6000 | 3616 | -2 | 941.5925 |
| Mid Range2 | 21635 | -2 | 898.4900 | 3635 | -1 | 943.4975 |
| Mid Range3 | 21644 | -2 | 899.3900 | 3644 | -1 | 944.3975 |
| High Range | 21794 | 0 | 914.4000 | 3794 | 1 | 959.4075 |
| High Range2 | 21760 | -2 | 910.9900 | 3760 | -1 | 955.9975 |
| High Range3 | 21769 | -2 | 911.8900 | 3769 | -1 | 956.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.8-3: NB-IoT guard-band Test frequencies for operating band 8

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 21451 | 0 | 880.1000 | 3451 | 1 | 925.1075 |
| Mid Range | 5 | 21601 | 0 | 895.1000 | 3601 | 1 | 940.1075 |
|  | 10 | 21671 | 0 | 902.1000 | 3671 | 1 | 947.1075 |
| High Range | Any | 21799 | 0 | 914.9000 | 3799 | -2 | 959.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.9 to 8.1.3.1.1.10	Void

8.1.3.1.1.11	NB-IoT FDD reference test frequencies for operating band 11

Table 8.1.3.1.1.11-1: NB-IoT standalone Test frequencies for operating band 11

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 22751 | 0 | 1428.0 | 4751 | -0.5 | 1476.0 |
| Mid Range | 22850 | 0 | 1437.9 | 4850 | -0.5 | 1485.9 |
| High Range | 22949 | 0 | 1447.8 | 4949 | -0.5 | 1495.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.11-2: NB-IoT in-band Test frequencies for operating band 11

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 22757 | 0 | 1428.6000 | 4757 | -2 | 1476.5925 |
| Low Range2 | 22810 | -2 | 1433.8900 | 4810 | -1 | 1481.8975 |
| Low Range3 | 22819 | -2 | 1434.7900 | 4819 | -1 | 1482.7975 |
| Mid Range1 | 22841 | 0 | 1437.0000 | 4841 | -2 | 1484.9925 |
| Mid Range2 | 22860 | -2 | 1438.8900 | 4860 | -1 | 1486.8975 |
| Mid Range3 | 22869 | -2 | 1439.7900 | 4869 | -1 | 1487.7975 |
| High Range1 | 22943 | 0 | 1447.2000 | 4943 | 1 | 1495.2075 |
| High Range2 | 22910 | -2 | 1443.8900 | 4910 | -1 | 1491.8975 |
| High Range3 | 22919 | -2 | 1444.7900 | 4919 | -1 | 1492.7975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.11-3: NB-IoT guard-band Test frequencies for operating band 11

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 22751 | 0 | 1428.0000 | 4751 | 1 | 1476.0075 |
| Mid Range | 22826 | 0 | 1435.5000 | 4826 | 1 | 1483.5075 |
| High Range | 22949 | 0 | 1447.8000 | 4949 | -2 | 1495.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.12	NB-IoT FDD reference test frequencies for operating band 12

Table 8.1.3.1.1.12-1: NB-IoT standalone Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23179 | 0 | 715.9 | 5179 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-1a: NB-IoT standalone Test frequencies for operating band 12 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23011 | 0 | 699.1 | 5011 | -0.5 | 729.1 |
| Mid Range | 23095 | 0 | 707.5 | 5095 | -0.5 | 737.5 |
| High Range | 23178 | 0 | 715.8 | 5178 | -0.5 | 745.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-2: NB-IoT in-band Test frequencies for operating band 12

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23016 | 0 | 699.6000 | 5016 | -2 | 729.5925 |
| Low Range2 | 23070 | -2 | 704.9900 | 5070 | -1 | 734.9975 |
| Low Range3 | 23079 | -2 | 705.8900 | 5079 | -1 | 735.8975 |
| Mid Range1 | 23086 | 0 | 706.6000 | 5086 | -2 | 736.5925 |
| Mid Range2 | 23105 | -2 | 708.4900 | 5105 | -1 | 738.4975 |
| Mid Range3 | 23114 | -2 | 709.3900 | 5114 | -1 | 739.3975 |
| High Range1 | 23174 | 0 | 715.4000 | 5174 | 1 | 745.4075 |
| High Range2 | 23140 | -2 | 711.9900 | 5140 | -1 | 741.9975 |
| High Range3 | 23149 | -2 | 712.8900 | 5149 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-2: NB-IoT in-band Test frequencies for operating band 12 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23016 | 0 | 699.6000 | 5016 | -2 | 729.5925 |
| Low Range2 | 23070 | -2 | 704.9900 | 5070 | -1 | 734.9975 |
| Low Range3 | 23079 | -2 | 705.8900 | 5079 | -1 | 735.8975 |
| Mid Range1 | 23086 | 0 | 706.6000 | 5086 | -2 | 736.5925 |
| Mid Range2 | 23105 | -2 | 708.4900 | 5105 | -1 | 738.4975 |
| Mid Range3 | 23114 | -2 | 709.3900 | 5114 | -1 | 739.3975 |
| High Range1 | 23174 | 0 | 715.4000 | 5174 | 1 | 745.4075 |
| High Range2 | 23140 | -2 | 711.9900 | 5140 | -1 | 741.9975 |
| High Range3 | 23149 | -2 | 712.8900 | 5149 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3: NB-IoT guard-band Test frequencies for operating band 12

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23179 | 0 | 715.9000 | 5179 | -2 | 745.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.12-3a: NB-IoT guard-band Test frequencies for operating band 12 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23011 | 0 | 699.1000 | 5011 | 1 | 729.1075 |
| Mid Range | 5 | 23071 | 0 | 705.1000 | 5071 | 1 | 735.1075 |
|  | 10 | 23141 | 0 | 712.1000 | 5141 | 1 | 742.1075 |
| High Range | Any | 23178 | 0 | 715.8000 | 5178 | -2 | 745.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.13	NB-IoT FDD reference test frequencies for operating band 13

Table 8.1.3.1.1.13-1: NB-IoT standalone Test frequencies for operating band 13

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23181 | 0 | 777.1 | 5181 | -0.5 | 746.1 |
| Mid Range | 23230 | 0 | 782.0 | 5230 | -0.5 | 751.0 |
| High Range | 23279 | 0 | 786.9 | 5279 | -0.5 | 755.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-1a: NB-IoT standalone Test frequencies for operating band 13 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23181 | 0 | 777.1 | 5181 | -0.5 | 746.1 |
| Mid Range | 23230 | 0 | 782.0 | 5230 | -0.5 | 751.0 |
| High Range | 23278 | 0 | 786.8 | 5278 | -0.5 | 755.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-2: NB-IoT in-band Test frequencies for operating band 13

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23187 | 0 | 777.7000 | 5187 | -2 | 746.6925 |
| Low Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Low Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| Mid Range1 | 23221 | 0 | 781.1000 | 5221 | -2 | 750.0925 |
| Mid Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Mid Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| High Range1 | 23273 | 0 | 786.3000 | 5273 | 1 | 755.3075 |
| High Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| High Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-2a: NB-IoT in-band Test frequencies for operating band 13 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23187 | 0 | 777.7000 | 5187 | -2 | 746.6925 |
| Low Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Low Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| Mid Range1 | 23221 | 0 | 781.1000 | 5221 | -2 | 750.0925 |
| Mid Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| Mid Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| High Range1 | 23273 | 0 | 786.3000 | 5273 | 1 | 755.3075 |
| High Range2 | 23240 | -2 | 782.9900 | 5240 | -1 | 751.9975 |
| High Range3 | 23249 | -2 | 783.8900 | 5249 | -1 | 752.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.13-3: NB-IoT guard-band Test frequencies for operating band 13

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23181 | 3 | 777.1150 | 5181 | 1 | 746.1075 |
| Mid Range | 5 | 23206 | 3 | 779.6150 | 5206 | 1 | 748.6075 |
|  | 10 | 23276 | 3 | 786.6150 | 5276 | 1 | 755.6075 |
| High Range | Any | 23279 | -3 | 786.8850 | 5279 | -2 | 755.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.13-3a: NB-IoT guard-band Test frequencies for operating band 13 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23181 | 3 | 777.1150 | 5181 | 1 | 746.1075 |
| Mid Range | 5 | 23206 | 3 | 779.6150 | 5206 | 1 | 748.6075 |
|  | 10 | 23276 | 3 | 786.6150 | 5276 | 1 | 755.6075 |
| High Range | Any | 23278 | -3 | 786.7850 | 5278 | -2 | 755.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.14	NB-IoT FDD reference test frequencies for operating band 14

Table 8.1.3.1.1.14-1: NB-IoT standalone Test frequencies for operating band 14

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23281 | 0 | 788.1 | 5281 | -0.5 | 758.1 |
| Mid Range | 23330 | 0 | 793.0 | 5330 | -0.5 | 763.0 |
| High Range | 23379 | 0 | 797.9 | 5379 | -0.5 | 767.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.14-2: NB-IoT in-band Test frequencies for operating band 14

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23287 | 0 | 788.7000 | 5287 | -2 | 758.6925 |
| Low Range2 | 23340 | -2 | 793.9900 | 5340 | -1 | 763.9975 |
| Low Range3 | 23349 | -2 | 794.8900 | 5349 | -1 | 764.8975 |
| Mid Range1 | 23321 | 0 | 792.1000 | 5321 | -2 | 762.0925 |
| Mid Range2 | 23340 | -2 | 793.9900 | 5340 | -1 | 763.9975 |
| Mid Range3 | 23349 | -2 | 794.8900 | 5349 | -1 | 764.8975 |
| High Range1 | 23373 | 0 | 797.3000 | 5373 | 1 | 767.3075 |
| High Range2 | 23340 | -2 | 793.9900 | 5340 | -1 | 763.9975 |
| High Range3 | 23349 | -2 | 794.8900 | 5349 | -1 | 764.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.14-3: NB-IoT guard-band Test frequencies for operating band 14

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 23281 | 3 | 788.1150 | 5281 | 1 | 758.1075 |
| Mid Range | 5 | 23306 | 3 | 790.6150 | 5306 | 1 | 760.6075 |
|  | 10 | 23376 | 3 | 797.6150 | 5376 | 1 | 767.6075 |
| High Range | Any | 23379 | -3 | 797.8850 | 5379 | -2 | 767.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.15 to 8.1.3.1.1.16	Void

8.1.3.1.1.17	NB-IoT FDD reference test frequencies for operating band 17

Table 8.1.3.1.1.17-1: NB-IoT standalone Test frequencies for operating band 17

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23731 | 0 | 704.1 | 5731 | -0.5 | 734.1 |
| Mid Range | 23790 | 0 | 710.0 | 5790 | -0.5 | 740.0 |
| High Range | 23849 | 0 | 715.9 | 5849 | -0.5 | 745.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-1a: NB-IoT standalone Test frequencies for operating band 17 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23732 | 0 | 704.2 | 5732 | -0.5 | 734.2 |
| Mid Range | 23790 | 0 | 710.0 | 5790 | -0.5 | 740.0 |
| High Range | 23848 | 0 | 715.8 | 5848 | -0.5 | 745.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-2: NB-IoT in-band Test frequencies for operating band 17

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23737 | 0 | 704.7000 | 5737 | -2 | 734.6925 |
| Low Range2 | 23790 | -2 | 709.9900 | 5790 | -1 | 739.9975 |
| Low Range3 | 23799 | -2 | 710.8900 | 5799 | -1 | 740.8975 |
| Mid Range1 | 23781 | 0 | 709.1000 | 5781 | -2 | 739.0925 |
| Mid Range2 | 23800 | -2 | 710.9900 | 5800 | -1 | 740.9975 |
| Mid Range3 | 23809 | -2 | 711.8900 | 5809 | -1 | 741.8975 |
| High Range1 | 23843 | 0 | 715.3000 | 5843 | 1 | 745.3075 |
| High Range2 | 23810 | -2 | 711.9900 | 5810 | -1 | 741.9975 |
| High Range3 | 23819 | -2 | 712.8900 | 5819 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-2a: NB-IoT in-band Test frequencies for operating band 17 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23737 | 0 | 704.7000 | 5737 | -2 | 734.6925 |
| Low Range2 | 23790 | -2 | 709.9900 | 5790 | -1 | 739.9975 |
| Low Range3 | 23799 | -2 | 710.8900 | 5799 | -1 | 740.8975 |
| Mid Range1 | 23781 | 0 | 709.1000 | 5781 | -2 | 739.0925 |
| Mid Range2 | 23800 | -2 | 710.9900 | 5800 | -1 | 740.9975 |
| Mid Range3 | 23809 | -2 | 711.8900 | 5809 | -1 | 741.8975 |
| High Range1 | 23843 | 0 | 715.3000 | 5843 | 1 | 745.3075 |
| High Range2 | 23810 | -2 | 711.9900 | 5810 | -1 | 741.9975 |
| High Range3 | 23819 | -2 | 712.8900 | 5819 | -1 | 742.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-3: NB-IoT guard-band Test frequencies for operating band 17

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23731 | 0 | 704.1000 | 5731 | 1 | 734.1075 |
| Mid Range | 23766 | 0 | 707.6000 | 5766 | 1 | 737.6075 |
| High Range | 23849 | 0 | 715.9000 | 5849 | -2 | 745.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.17-3a: NB-IoT guard-band Test frequencies for operating band 17 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23732 | 0 | 704.2000 | 5732 | 1 | 734.2075 |
| Mid Range | 23766 | 0 | 707.6000 | 5766 | 1 | 737.6075 |
| High Range | 23848 | 0 | 715.8000 | 5848 | -2 | 745.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.18	NB-IoT FDD reference test frequencies for operating band 18

Table 8.1.3.1.1.18-1: NB-IoT standalone Test frequencies for operating band 18

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23851 | 0 | 815.1 | 5851 | -0.5 | 860.1 |
| Mid Range | 23925 | 0 | 822.5 | 5925 | -0.5 | 867.5 |
| High Range | 23999 | 0 | 829.9 | 5999 | -0.5 | 874.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.18-2: NB-IoT in-band Test frequencies for operating band 18

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 23857 | 0 | 815.7000 | 5857 | -2 | 860.6925 |
| Low Range2 | 23910 | -2 | 820.9900 | 5910 | -1 | 865.9975 |
| Low Range3 | 23919 | -2 | 821.8900 | 5919 | -1 | 866.8975 |
| Mid Range1 | 23916 | 0 | 821.6000 | 5916 | -2 | 866.5925 |
| Mid Range2 | 23935 | -2 | 823.4900 | 5935 | -1 | 868.4975 |
| Mid Range3 | 23944 | -2 | 824.3900 | 5944 | -1 | 869.3975 |
| High Range1 | 23993 | 0 | 829.3000 | 5993 | 1 | 874.3075 |
| High Range2 | 23960 | -2 | 825.9900 | 5960 | -1 | 870.9975 |
| High Range3 | 23969 | -2 | 826.8900 | 5969 | -1 | 871.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.18-3: NB-IoT guard-band Test frequencies for operating band 18

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 23851 | 0 | 815.1000 | 5851 | 1 | 860.1075 |
| Mid Range | 23901 | 0 | 820.1000 | 5901 | 1 | 865.1075 |
| High Range | 23999 | 0 | 829.9000 | 5999 | -2 | 874.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.19	NB-IoT FDD reference test frequencies for operating band 19

Table 8.1.3.1.1.19-1: NB-IoT standalone Test frequencies for operating band 19

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24001 | 0 | 830.1 | 6001 | -0.5 | 875.1 |
| Mid Range | 24075 | 0 | 837.5 | 6075 | -0.5 | 882.5 |
| High Range | 24149 | 0 | 844.9 | 6149 | -0.5 | 889.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.19-2: NB-IoT in-band Test frequencies for operating band 19

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 24007 | 0 | 830.7000 | 6007 | -2 | 875.6925 |
| Low Range2 | 24060 | -2 | 835.9900 | 6060 | -1 | 880.9975 |
| Low Range3 | 24069 | -2 | 836.8900 | 6069 | -1 | 881.8975 |
| Mid Range1 | 24066 | 0 | 836.6000 | 6066 | -2 | 881.5925 |
| Mid Range2 | 24085 | -2 | 838.4900 | 6085 | -1 | 883.4975 |
| Mid Range3 | 24094 | -2 | 839.3900 | 6094 | -1 | 884.3975 |
| High Range1 | 24143 | 0 | 844.3000 | 6143 | 1 | 889.3075 |
| High Range2 | 24110 | -2 | 840.9900 | 6110 | -1 | 885.9975 |
| High Range3 | 24119 | -2 | 841.8900 | 6119 | -1 | 886.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.19-3: NB-IoT guard-band Test frequencies for operating band 19

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24001 | 0 | 830.1000 | 6001 | 1 | 875.1075 |
| Mid Range | 24051 | 0 | 835.1000 | 6051 | 1 | 880.1075 |
| High Range | 24149 | 0 | 844.9000 | 6149 | -2 | 889.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.20	NB-IoT FDD reference test frequencies for operating band 20

Table 8.1.3.1.1.20-1: NB-IoT standalone Test frequencies for operating band 20

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24151 | 0 | 832.1 | 6151 | -0.5 | 791.1 |
| Mid Range | 24300 | 0 | 847.0 | 6300 | -0.5 | 806.0 |
| High Range | 24449 | 0 | 861.9 | 6449 | -0.5 | 820.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.20-2: NB-IoT in-band Test frequencies for operating band 20

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 24157 | 0 | 832.7000 | 6157 | -2 | 791.6925 |
| Low Range2 | 24210 | -2 | 837.9900 | 6210 | -1 | 796.9975 |
| Low Range3 | 24219 | -2 | 838.8900 | 6219 | -1 | 797.8975 |
| Mid Range1 | 24291 | 0 | 846.1000 | 6291 | -2 | 805.0925 |
| Mid Range2 | 24310 | -2 | 847.9900 | 6310 | -1 | 806.9975 |
| Mid Range3 | 24319 | -2 | 848.8900 | 6319 | -1 | 807.8975 |
| High Range1 | 24443 | 0 | 861.3000 | 6443 | 1 | 820.3075 |
| High Range2 | 24410 | -2 | 857.9900 | 6410 | -1 | 816.9975 |
| High Range3 | 24419 | -2 | 858.8900 | 6419 | -1 | 817.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.20-3: NB-IoT guard-band Test frequencies for operating band 20

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 24151 | 3 | 832.1150 | 6151 | 1 | 791.1075 |
| Mid Range | 5 | 24276 | 3 | 844.6150 | 6276 | 1 | 803.6075 |
|  | 10 | 24346 | 3 | 851.6150 | 6346 | 1 | 810.6075 |
| High Range | Any | 24449 | -3 | 861.8850 | 6449 | -2 | 820.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.21	NB-IoT FDD reference test frequencies for operating band 21

Table 8.1.3.1.1.21-1: NB-IoT standalone Test frequencies for operating band 21

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24451 | 0 | 1448.00 | 6451 | -0.5 | 1496.00 |
| Mid Range | 24525 | 0 | 1455.40 | 6525 | -0.5 | 1503.40 |
| High Range | 24599 | 0 | 1462.80 | 6599 | -0.5 | 1510.80 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.21-2: NB-IoT in-band Test frequencies for operating band 21

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 24457 | -3 | 1448.5850 | 6457 | -2 | 1496.5925 |
| Low Range2 | 24510 | -2 | 1453.8900 | 6510 | -1 | 1501.8975 |
| Low Range3 | 24519 | -2 | 1454.7900 | 6519 | -1 | 1502.7975 |
| Mid Range | 24516 | -3 | 1454.4850 | 6516 | -2 | 1502.4925 |
| Mid Range2 | 24535 | -2 | 1456.3900 | 6535 | -1 | 1504.3975 |
| Mid Range3 | 24544 | -2 | 1457.2900 | 6544 | -1 | 1505.2975 |
| High Range | 24593 | 3 | 1462.2150 | 6593 | 1 | 1510.2075 |
| High Range2 | 24560 | -2 | 1458.8900 | 6560 | -1 | 1506.8975 |
| High Range3 | 24569 | -2 | 1459.7900 | 6569 | -1 | 1507.7975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.21-3: NB-IoT guard-band Test frequencies for operating band 21

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 24451 | 0 | 1448.0000 | 6451 | 1 | 1496.0075 |
| Mid Range | 5 | 24501 | 0 | 1453.0000 | 6501 | 1 | 1501.0075 |
|  | 10 | 24571 | 0 | 1460.0000 | 6571 | 1 | 1508.0075 |
| High Range | Any | 24599 | 0 | 1462.8000 | 6599 | -2 | 1510.7925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.22 to 8.1.3.1.1.24	Void

8.1.3.1.1.25	NB-IoT FDD reference test frequencies for operating band 25

Table 8.1.3.1.1.25-1: NB-IoT standalone Test frequencies for operating band 25

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26041 | 0 | 1850.10 | 8041 | -0.5 | 1930.1 |
| Mid Range | 26365 | 0 | 1882.50 | 8365 | -0.5 | 1962.5 |
| High Range | 26689 | 0 | 1914.90 | 8689 | -0.5 | 1994.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-1a: NB-IoT standalone Test frequencies for operating band 25 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26042 | 0 | 1850.20 | 8042 | -0.5 | 1930.2 |
| Mid Range | 26365 | 0 | 1882.50 | 8365 | -0.5 | 1962.5 |
| High Range | 26688 | 0 | 1914.80 | 8688 | -0.5 | 1994.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-2: NB-IoT in-band Test frequencies for operating band 25

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26046 | 0 | 1850.6000 | 8046 | -2 | 1930.5925 |
| Low Range2 | 26100 | -2 | 1855.9900 | 8100 | -1 | 1935.9975 |
| Low Range3 | 26109 | -2 | 1856.8900 | 8109 | -1 | 1936.8975 |
| Mid Range1 | 26356 | 0 | 1881.6000 | 8356 | -2 | 1961.5925 |
| Mid Range2 | 26375 | -2 | 1883.4900 | 8375 | -1 | 1963.4975 |
| Mid Range3 | 26384 | -2 | 1884.3900 | 8384 | -1 | 1964.3975 |
| High Range1 | 26684 | 0 | 1914.4000 | 8684 | 1 | 1994.4075 |
| High Range2 | 26650 | -2 | 1910.99 | 8650 | -1 | 1990.9975 |
| High Range3 | 26659 | -2 | 1911.89 | 8659 | -1 | 1991.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-2a: NB-IoT in-band Test frequencies for operating band 25 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26046 | 0 | 1850.6000 | 8046 | -2 | 1930.5925 |
| Low Range2 | 26100 | -2 | 1855.9900 | 8100 | -1 | 1935.9975 |
| Low Range3 | 26109 | -2 | 1856.8900 | 8109 | -1 | 1936.8975 |
| Mid Range1 | 26356 | 0 | 1881.6000 | 8356 | -2 | 1961.5925 |
| Mid Range2 | 26375 | -2 | 1883.4900 | 8375 | -1 | 1963.4975 |
| Mid Range3 | 26384 | -2 | 1884.3900 | 8384 | -1 | 1964.3975 |
| High Range1 | 26684 | 0 | 1914.4000 | 8684 | 1 | 1994.4075 |
| High Range2 | 26650 | -2 | 1910.99 | 8650 | -1 | 1990.9975 |
| High Range3 | 26659 | -2 | 1911.89 | 8659 | -1 | 1991.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-3: NB-IoT guard-band Test frequencies for operating band 25

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26041 | 0 | 1850.1000 | 8041 | 1 | 1930.1075 |
| Mid Range | 26341 | 0 | 1880.1000 | 8341 | 1 | 1960.1075 |
| High Range | 26689 | 0 | 1914.9000 | 8689 | -2 | 1994.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.25-3a: NB-IoT guard-band Test frequencies for operating band 25 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26041 | 0 | 1850.1000 | 8041 | 1 | 1930.1075 |
| Mid Range | 26341 | 0 | 1880.1000 | 8341 | 1 | 1960.1075 |
| High Range | 26688 | 0 | 1914.8000 | 8688 | -2 | 1994.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.26	NB-IoT FDD reference test frequencies for operating band 26

Table 8.1.3.1.1.26-1: NB-IoT standalone Test frequencies for operating band 26

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26691 | 0 | 814.1 | 8691 | -0.5 | 859.1 |
| Mid Range | 26865 | 0 | 831.5 | 8865 | -0.5 | 876.5 |
| High Range | 27039 | 0 | 848.9 | 9039 | -0.5 | 893.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-1a: NB-IoT standalone Test frequencies for operating band 26 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26692 | 0 | 814.2 | 8692 | -0.5 | 859.2 |
| Mid Range | 26865 | 0 | 831.5 | 8865 | -0.5 | 876.5 |
| High Range | 27038 | 0 | 848.8 | 9038 | -0.5 | 893.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-2: NB-IoT in-band Test frequencies for operating band 26

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26696 | 0 | 814.6000 | 8696 | -2 | 859.5925 |
| Low Range2 | 26750 | -2 | 819.9900 | 8750 | -1 | 864.9975 |
| Low Range3 | 26759 | -2 | 820.8900 | 8759 | -1 | 865.8975 |
| Mid Range1 | 26856 | 0 | 830.6000 | 8856 | -2 | 875.5925 |
| Mid Range2 | 26875 | -2 | 832.4900 | 8875 | -1 | 877.4975 |
| Mid Range3 | 26884 | -2 | 833.3900 | 8884 | -1 | 878.3975 |
| High Range1 | 27034 | 0 | 848.4000 | 9034 | 1 | 893.4075 |
| High Range2 | 27000 | -2 | 844.9900 | 9000 | -1 | 889.9975 |
| High Range3 | 27009 | -2 | 845.8900 | 9009 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-2a: NB-IoT in-band Test frequencies for operating band 26 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 26696 | 0 | 814.6000 | 8696 | -2 | 859.5925 |
| Low Range2 | 26750 | -2 | 819.9900 | 8750 | -1 | 864.9975 |
| Low Range3 | 26759 | -2 | 820.8900 | 8759 | -1 | 865.8975 |
| Mid Range1 | 26856 | 0 | 830.6000 | 8856 | -2 | 875.5925 |
| Mid Range2 | 26875 | -2 | 832.4900 | 8875 | -1 | 877.4975 |
| Mid Range3 | 26884 | -2 | 833.3900 | 8884 | -1 | 878.3975 |
| High Range1 | 27034 | 0 | 848.4000 | 9034 | 1 | 893.4075 |
| High Range2 | 27000 | -2 | 844.9900 | 9000 | -1 | 889.9975 |
| High Range3 | 27009 | -2 | 845.8900 | 9009 | -1 | 890.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-3: NB-IoT guard-band Test frequencies for operating band 26

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26691 | 0 | 814.1000 | 8691 | 1 | 859.1075 |
| Mid Range | 26841 | 0 | 829.1000 | 8841 | 1 | 874.1075 |
| High Range | 27039 | 0 | 848.9000 | 9039 | -2 | 893.8925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.26-3a: NB-IoT guard-band Test frequencies for operating band 26 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 26692 | 0 | 814.2000 | 8692 | 1 | 859.2075 |
| Mid Range | 26841 | 0 | 829.1000 | 8841 | 1 | 874.1075 |
| High Range | 27038 | 0 | 848.8000 | 9038 | -2 | 893.7925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.27	Void

8.1.3.1.1.28	NB-IoT FDD reference test frequencies for operating band 28

Table 8.1.3.1.1.28-1: NB-IoT standalone Test frequencies for operating band 28

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 27211 | 0 | 703.1 | 9211 | -0.5 | 758.1 |
| Mid Range | 27435 | 0 | 725.5 | 9435 | -0.5 | 780.5 |
| High Range | 27659 | 0 | 747.9 | 9659 | -0.5 | 802.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.28-2: NB-IoT in-band Test frequencies for operating band 28

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 27216 | 0 | 703.6000 | 9216 | -2 | 758.5925 |
| Low Range2 | 27270 | -2 | 708.9900 | 9270 | -1 | 763.9975 |
| Low Range3 | 27279 | -2 | 709.8900 | 9279 | -1 | 764.8975 |
| Mid Range1 | 27426 | 0 | 724.6000 | 9426 | -2 | 779.5925 |
| Mid Range2 | 27420 | -2 | 723.9900 | 9420 | -1 | 778.9975 |
| Mid Range3 | 27429 | -2 | 724.8900 | 9429 | -1 | 779.8975 |
| High Range1 | 27654 | 0 | 747.4000 | 9654 | 1 | 802.4075 |
| High Range2 | 27620 | -2 | 743.9900 | 9620 | -1 | 798.9975 |
| High Range3 | 27629 | -2 | 744.8900 | 9629 | -1 | 799.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.28-3: NB-IoT guard-band Test frequencies for operating band 28

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 27211 | 0 | 703.1000 | 9211 | 1 | 758.1075 |
| Mid Range | 5 | 27411 | 0 | 723.1000 | 9411 | 1 | 778.1075 |
|  | 10 | 27456 | 0 | 727.6000 | 9456 | 1 | 782.6075 |
| High Range | Any | 27659 | 0 | 747.9000 | 9659 | -2 | 802.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.29 to 8.1.3.1.1.30	Void

8.1.3.1.1.31	NB-IoT FDD reference test frequencies for operating band 31

Table 8.1.3.1.1.31-1: NB-IoT standalone Test frequencies for operating band 31

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 27761 | 0 | 452.60 | 9871 | -0.5 | 462.6 |
| Mid Range | 27785 | 0 | 455.00 | 9895 | -0.5 | 465 |
| High Range | 27809 | 0 | 457.40 | 9919 | -0.5 | 467.4 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.31-2: NB-IoT in-band Test frequencies for operating band 31

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 27766 | -3 | 453.0850 | 9876 | -2 | 463.0925 |
| Low Range2 | 27794 | 0 | 455.9000 | 9904 | 1 | 465.9075 |
| Low Range3 | 27803 | 0 | 456.8000 | 9913 | 1 | 466.8075 |
| Mid Range1 | 27776 | -3 | 454.0850 | 9886 | -2 | 464.0925 |
| Mid Range2 | 27794 | 0 | 455.9000 | 9904 | 1 | 465.9075 |
| Mid Range3 | 27803 | 0 | 456.8000 | 9913 | 1 | 466.8075 |
| High Range1 | 27804 | 3 | 456.9150 | 9914 | 1 | 466.9075 |
| High Range2 | 27794 | 0 | 455.9000 | 9904 | 1 | 465.9075 |
| High Range3 | 27803 | 0 | 456.8000 | 9913 | 1 | 466.8075 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 5 MHz and NB-IoT PRB 17 NOTE 3: Related to LTE channel BW 5 MHz and NB-IoT PRB 22 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.31-3: NB-IoT guard-band Test frequencies for operating band 31

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 27761 | 0 | 452.6000 | 9871 | 1 | 462.6075 |
| Mid Range | 27761 | 0 | 452.6000 | 9871 | 1 | 462.6075 |
| High Range | 27809 | 0 | 457.4000 | 9919 | -2 | 467.3925 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.32 to 8.1.3.1.1. 64	Void

8.1.3.1.1.65	NB-IoT FDD reference test frequencies for operating band 65

Table 8.1.3.1.1.65-1: NB-IoT standalone Test frequencies for operating band 65

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131073 | 0 | 1920.1 | 65537 | -0.5 | 2110.1 |
| Mid Range | 131522 | 0 | 1965.0 | 65986 | -0.5 | 2155.0 |
| High Range | 131971 | 0 | 2009.9 | 66435 | -0.5 | 2199.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing NOTE 2: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.65-2: NB-IoT in-band Test frequencies for operating band 65

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 131078 | 0 | 1920.6000 | 65542 | -2 | 2110.5925 |
| Low Range2 | 131132 | -2 | 1925.9900 | 65596 | -1 | 2115.9975 |
| Low Range3 | 131141 | -2 | 1926.8900 | 65605 | -1 | 2116.8975 |
| Mid Range1 | 131513 | 0 | 1964.1000 | 65977 | -2 | 2154.0925 |
| Mid Range2 | 131532 | -2 | 1965.9900 | 65996 | -1 | 2155.9975 |
| Mid Range3 | 131541 | -2 | 1966.8900 | 66005 | -1 | 2156.8975 |
| High Range1 | 131966 | 0 | 2009.4000 | 66430 | 1 | 2199.4075 |
| High Range2 | 131932 | -2 | 2005.9900 | 66396 | -1 | 2195.9975 |
| High Range3 | 131941 | -2 | 2006.8900 | 66405 | -1 | 2196.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.65-3: NB-IoT guard-band Test frequencies for operating band 65

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 131073 | 0 | 1920.1000 | 65537 | 1 | 2110.1075 |
| Mid Range | 5 | 131498 | 0 | 1962.6000 | 65962 | 1 | 2152.6075 |
|  | 10 | 131568 | 0 | 1969.6000 | 66032 | 1 | 2159.6075 |
| High Range | Any | 131971 | 0 | 2009.9000 | 66435 | -2 | 2199.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

8.1.3.1.1.66	NB-IoT FDD reference test frequencies for operating band 66

Table 8.1.3.1.1.66-1: NB-IoT standalone Test frequencies for operating band 66

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131973 | 0 | 1710.1 | 66437 | -0.5 | 2110.1 |
| Mid Range | 132322 | 0 | 1745.0 | 66786 | -0.5 | 2145.0 |
| High Range | 132671 | 0 | 1779.9 | 67135 | -0.5 | 2179.9 |
| High Range3 | 132421 | 0 | 1754.9 | 66885 | -0.5 | 2154.9 |
| High Range4 | 132571 | 0 | 1769.0 | 67035 | -0.5 | 2169.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing NOTE 2: Only paired part of the band considered for NB-IoT NOTE 3: Applicable to NB-IoT FDD MFBI where band 66 overlaps band 4 NOTE 4: Applicable to NB-IoT FDD MFBI where band 66 overlaps band 10 |  |  |  |  |  |  |

Table 8.1.3.1.1.66-1a: NB-IoT standalone Test frequencies for operating band 66 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 131974 | 0 | 1710.2 | 66438 | -0.5 | 2110.2 |
| Mid Range | 132322 | 0 | 1745.0 | 66786 | -0.5 | 2145.0 |
| High Range | 132670 | 0 | 1779.8 | 67134 | -0.5 | 2179.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing NOTE 2: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.66-2: NB-IoT in-band Test frequencies for operating band 66

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 131978 | 0 | 1710.6000 | 66442 | -2 | 2110.5925 |
| Low Range2 | 132032 | -2 | 1715.9900 | 66496 | -1 | 2115.9975 |
| Low Range3 | 132041 | -2 | 1716.8900 | 66505 | -1 | 2116.8975 |
| Mid Range1 | 132313 | 0 | 1744.1000 | 66777 | -2 | 2144.0925 |
| Mid Range2 | 132432 | -2 | 1755.9900 | 66896 | -1 | 2155.9975 |
| Mid Range3 | 132441 | -2 | 1756.8900 | 66905 | -1 | 2156.8975 |
| High Range1 | 132666 | 0 | 1779.4000 | 67130 | 1 | 2179.4075 |
| High Range2 | 132632 | -2 | 1775.9900 | 67096 | -1 | 2175.9975 |
| High Range3 | 132641 | -2 | 1776.8900 | 67105 | -1 | 2176.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.66-2a: NB-IoT in-band Test frequencies for operating band 66 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 131978 | 0 | 1710.6000 | 66442 | -2 | 2110.5925 |
| Low Range2 | 132032 | -2 | 1715.9900 | 66496 | -1 | 2115.9975 |
| Low Range3 | 132041 | -2 | 1716.8900 | 66505 | -1 | 2116.8975 |
| Mid Range1 | 132313 | 0 | 1744.1000 | 66777 | -2 | 2144.0925 |
| Mid Range2 | 132432 | -2 | 1755.9900 | 66896 | -1 | 2155.9975 |
| Mid Range3 | 132441 | -2 | 1756.8900 | 66905 | -1 | 2156.8975 |
| High Range1 | 132666 | 0 | 1779.4000 | 67130 | 1 | 2179.4075 |
| High Range2 | 132632 | -2 | 1775.9900 | 67096 | -1 | 2175.9975 |
| High Range3 | 132641 | -2 | 1776.8900 | 67105 | -1 | 2176.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |

Table 8.1.3.1.1.66-3: NB-IoT guard-band Test frequencies for operating band 66

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 131973 | 0 | 1710.1000 | 66437 | 1 | 2110.1075 |
| Mid Range | 5 | 132298 | 0 | 1742.6000 | 66762 | 1 | 2142.6075 |
|  | 10 | 132468 | 0 | 1759.6000 | 66932 | 1 | 2159.6075 |
| High Range | Any | 132671 | 0 | 1779.9000 | 67135 | -2 | 2179.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

Table 8.1.3.1.1.66-3a: NB-IoT guard-band Test frequencies for operating band 66 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 131974 | 0 | 1710.2000 | 66438 | 1 | 2110.2075 |
| Mid Range | 5 | 132298 | 0 | 1742.6000 | 66762 | 1 | 2142.6075 |
|  | 10 | 132468 | 0 | 1759.6000 | 66932 | 1 | 2159.6075 |
| High Range | Any | 132670 | 0 | 1779.8000 | 67134 | -2 | 2179.7925 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 2: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

8.1.3.1.1.67 to 8.1.3.1.1.69	Void

8.1.3.1.1.70	NB-IoT FDD reference test frequencies for operating band 70

Table 8.1.3.1.1.70-1: NB-IoT standalone Test frequencies for operating band 70

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 132973 | 0 | 1695.1 | 68337 | -0.5 | 1995.1 |
| Mid Range | 133047 | 0 | 1702.5 | 68411 | -0.5 | 2002.5 |
| High Range | 133121 | 0 | 1709.9 | 68485 | -0.5 | 2009.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.70-2: NB-IoT in-band Test frequencies for operating band 70

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 132979 | -3 | 1695.6850 | 68343 | -2 | 1995.6925 |
| Mid Range | 3 | 133038 | -3 | 1701.5850 | 68402 | -2 | 2001.5925 |
|  | 10 PRB 30 | 133057 | -2 | 1703.4900 | 68421 | -1 | 2003.4975 |
|  | 10 PRB 35 | 133066 | -2 | 1704.3900 | 68430 | -1 | 2004.3975 |
| High Range | Any | 133115 | 3 | 1709.3150 | 68479 | 1 | 2009.3075 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.70-3: NB-IoT guard-band Test frequencies for operating band 70

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 132973 | 0 | 1695.1000 | 68337 | 1 | 1995.1075 |
| Mid Range | 5 | 133023 | 0 | 1700.1000 | 68387 | 1 | 2000.1075 |
|  | 10 | 133093 | 0 | 1707.1000 | 68457 | 1 | 2007.1075 |
| High Range | Any | 133121 | 0 | 1709.9000 | 68485 | -2 | 2009.8925 |
| NOTE 1: Void NOTE 2: Applicable to either 3.75 kHz or 15 kHz UL sub-carrier spacing NOTE 3: Only paired part of the band considered for NB-IoT |  |  |  |  |  |  |  |

8.1.3.1.1.71	NB-IoT FDD reference test frequencies for operating band 71

Table 8.1.3.1.1.71-1: NB-IoT standalone Test frequencies for operating band 71

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133123 | 0 | 663.1000 | 68587 | -0.5 | 617.1000 |
| Mid Range | 133297 | 0 | 680.5000 | 68761 | -0.5 | 634.5000 |
| High Range | 133471 | 0 | 697.9000 | 68935 | -0.5 | 651.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-1a: NB-IoT standalone Test frequencies for operating band 71 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133124 | 0 | 663.2000 | 68588 | -0.5 | 617.2000 |
| Mid Range | 133297 | 0 | 680.5000 | 68761 | -0.5 | 634.5000 |
| High Range | 133470 | 0 | 697.8000 | 68934 | -0.5 | 651.8000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-2: NB-IoT in-band Test frequencies for operating band 71

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 133129 | 0 | 663.7000 | 68593 | -2 | 617.6925 |
| Low Range2 | 133182 | -2 | 668.9900 | 68646 | -1 | 622.9975 |
| Low Range3 | 133191 | -2 | 669.8900 | 68655 | -1 | 623.8975 |
| Mid Range1 | 133288 | 0 | 679.6000 | 68752 | -2 | 633.5925 |
| Mid Range2 | 133307 | -2 | 681.4900 | 68771 | -1 | 635.4975 |
| Mid Range3 | 133316 | -2 | 682.3900 | 68780 | -1 | 636.3975 |
| High Range1 | 133465 | 0 | 697.3000 | 68929 | 1 | 651.3075 |
| High Range2 | 133432 | -2 | 693.9900 | 68896 | -1 | 647.9975 |
| High Range3 | 133441 | -2 | 694.8900 | 68905 | -1 | 648.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-2a: NB-IoT in-band Test frequencies for operating band 71 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 133129 | 0 | 663.7000 | 68593 | -2 | 617.6925 |
| Low Range2 | 133182 | -2 | 668.9900 | 68646 | -1 | 622.9975 |
| Low Range3 | 133191 | -2 | 669.8900 | 68655 | -1 | 623.8975 |
| Mid Range1 | 133288 | 0 | 679.6000 | 68752 | -2 | 633.5925 |
| Mid Range2 | 133307 | -2 | 681.4900 | 68771 | -1 | 635.4975 |
| Mid Range3 | 133316 | -2 | 682.3900 | 68780 | -1 | 636.3975 |
| High Range1 | 133465 | 0 | 697.3000 | 68929 | 1 | 651.3075 |
| High Range2 | 133432 | -2 | 693.9900 | 68896 | -1 | 647.9975 |
| High Range3 | 133441 | -2 | 694.8900 | 68905 | -1 | 648.8975 |
| NOTE 1: Related to LTE channel BW 3 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.71-3: NB-IoT guard-band Test frequencies for operating band 71

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 133123 | 3 | 663. 1150 | 68587 | 1 | 617.1075 |
| Mid Range | 5 | 133273 | 3 | 678. 1150 | 68737 | 1 | 632.1075 |
|  | 10 | 133343 | 3 | 685. 1150 | 68807 | 1 | 639.1075 |
| High Range | Any | 133471 | -3 | 697. 8850 | 68935 | -2 | 651.8925 |
| NOTE 1: Void. NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.71-3a: NB-IoT guard-band Test frequencies for operating band 71 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 133124 | 3 | 663. 2150 | 68588 | 1 | 617.2075 |
| Mid Range | 5 | 133273 | 3 | 678. 1150 | 68737 | 1 | 632.1075 |
|  | 10 | 133343 | 3 | 685. 1150 | 68807 | 1 | 639.1075 |
| High Range | Any | 133470 | -3 | 697. 7850 | 68934 | -2 | 651.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.72	Void

8.1.3.1.1.73	NB-IoT FDD reference test frequencies for operating band 73

Table 8.1.3.1.1.73-1: NB-IoT standalone Test frequencies for operating band 73

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133523 | 0 | 450.1000 | 68997 | -0.5 | 461.1000 |
| Mid Range | 133547 | 0 | 452.5000 | 69011 | -0.5 | 462.5000 |
| High Range | 133571 | 0 | 454.9000 | 69035 | -0.5 | 464.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.73-2: NB-IoT in-band Test frequencies for operating band 73

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 133556 | 0 | 453.4000 | 69020 | 1 | 463.4000 |
| Mid Range | 133556 | 0 | 453.4000 | 69020 | 1 | 463.4000 |
| High Range | 133556 | 0 | 453.4000 | 69020 | 1 | 463.4000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Defined for 5 MHz LTE Cell Bandwidth. |  |  |  |  |  |  |

Table 8.1.3.1.1.73-3: NB-IoT guard-band Test frequencies for operating band 73

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 133523 | 0 | 450.1000 | 68997 | 1 | 461.1000 |
| Mid Range | 5 | 133523 | 0 | 450.1000 | 68997 | 1 | 461.1000 |
| High Range | Any | 133571 | 0 | 454.9000 | 69035 | 1 | 464.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.74 to 8.1.3.1.1.84	Void

8.1.3.1.1.85	NB-IoT FDD reference test frequencies for operating band 85

Table 8.1.3.1.1.85-1: NB-IoT standalone Test frequencies for operating band 85

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134003 | 0 | 698,1000 | 70367 | -0.5 | 728,1000 |
| Mid Range | 134082 | 0 | 706,0000 | 70446 | -0.5 | 736,0000 |
| High Range | 134181 | 0 | 715,9000 | 70545 | -0.5 | 745,9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-1a: NB-IoT standalone Test frequencies for operating band 85 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134004 | 0 | 698,2000 | 70368 | -0.5 | 728,2000 |
| Mid Range | 134082 | 0 | 706,0000 | 70446 | -0.5 | 736,0000 |
| High Range | 134180 | 0 | 715,8000 | 70544 | -0.5 | 745,8000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-2: NB-IoT in-band Test frequencies for operating band 85

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 134062 | -2 | 703,9900 | 70426 | -1 | 733,9975 |
| Low Range2 | 134071 | -2 | 704,8900 | 70435 | -1 | 734,8975 |
| Mid Range1 | 134102 | -2 | 707,9900 | 70466 | -1 | 737,9975 |
| Mid Range2 | 134111 | -2 | 708,8900 | 70475 | -1 | 738,8975 |
| High Range1 | 134142 | -2 | 711,9900 | 70506 | -1 | 741,9975 |
| High Range2 | 134151 | -2 | 712,8900 | 70515 | -1 | 742,8975 |
| NOTE 1: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 3: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-2a: NB-IoT in-band Test frequencies for operating band 85 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 134062 | -2 | 703,9900 | 70426 | -1 | 733,9975 |
| Low Range2 | 134071 | -2 | 704,8900 | 70435 | -1 | 734,8975 |
| Mid Range1 | 134102 | -2 | 707,9900 | 70466 | -1 | 737,9975 |
| Mid Range2 | 134111 | -2 | 708,8900 | 70475 | -1 | 738,8975 |
| High Range1 | 134142 | -2 | 711,9900 | 70506 | -1 | 741,9975 |
| High Range2 | 134151 | -2 | 712,8900 | 70515 | -1 | 742,8975 |
| NOTE 1: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 3: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.85-3: NB-IoT guard-band Test frequencies for operating band 85

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134098 | 0 | 707,6000 | 70462 | 1 | 737,6075 |
| Mid Range | 10 | 134138 | 0 | 711,6000 | 70502 | 1 | 741,6075 |
| High Range | Any | 134178 | 0 | 715,6000 | 70542 | -2 | 745,6075 |
| NOTE 1: Void. NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

Table 8.1.3.1.1.85-3a: NB-IoT guard-band Test frequencies for operating band 85 with NS_04

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134004 | 0 | 698.2000 | 70368 | 1 | 728.2075 |
| Mid Range | 10 | 134092 | 0 | 707.0000 | 70456 | 1 | 737.0075 |
| High Range | Any | 134180 | 0 | 715.8000 | 70544 | -2 | 745.7925 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.86	FFS

8.1.3.1.1.87	NB-IoT FDD reference test frequencies for operating band 87

Table 8.1.3.1.1.87-1: NB-IoT standalone Test frequencies for operating band 87

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134183 | 0 | 410.1000 | 70547 | -0.5 | 420.1000 |
| Mid Range | 134207 | 0 | 412.5000 | 70571 | -0.5 | 422.5000 |
| High Range | 134231 | 0 | 414.9000 | 70595 | -0.5 | 424.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.87-2: NB-IoT in-band Test frequencies for operating band 87

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134216 | 0 | 413.4000 | 70580 | 1 | 423.4000 |
| Mid Range | 134216 | 0 | 413.4000 | 70580 | 1 | 423.4000 |
| High Range | 134216 | 0 | 413.4000 | 70580 | 1 | 423.4000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Defined for 5 MHz LTE Cell Bandwidth. |  |  |  |  |  |  |

Table 8.1.3.1.1.87-3: NB-IoT guard-band Test frequencies for operating band 87

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134183 | 0 | 410.1000 | 70547 | 1 | 420.1000 |
| Mid Range | 5 | 134183 | 0 | 410.1000 | 70547 | 1 | 4210.1000 |
| High Range | Any | 134231 | 0 | 414.9000 | 70595 | 1 | 424.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.88	NB-IoT FDD reference test frequencies for operating band 88

Table 8.1.3.1.1.88-1: NB-IoT standalone Test frequencies for operating band 88

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134233 | 0 | 412.1000 | 70597 | -0.5 | 422.1000 |
| Mid Range | 134257 | 0 | 414.5000 | 70621 | -0.5 | 424.5000 |
| High Range | 134281 | 0 | 416.9000 | 70645 | -0.5 | 426.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.88-2: NB-IoT in-band Test frequencies for operating band 88

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134266 | 0 | 415.4000 | 70630 | 1 | 425.4000 |
| Mid Range | 134266 | 0 | 415.4000 | 70630 | 1 | 425.4000 |
| High Range | 134266 | 0 | 415.4000 | 70630 | 1 | 425.4000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing NOTE 2: Defined for 5 MHz LTE Cell Bandwidth. |  |  |  |  |  |  |

Table 8.1.3.1.1.88-3: NB-IoT guard-band Test frequencies for operating band 88

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 134233 | 0 | 412.1000 | 70597 | 1 | 422.1000 |
| Mid Range | 5 | 134233 | 0 | 412.1000 | 70597 | 1 | 422.1000 |
| High Range | Any | 134281 | 0 | 416.9000 | 70645 | 1 | 426.9000 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.1.89 to 8.1.3.1.1.102	Void

8.1.3.1.1.103	NB-IoT FDD reference test frequencies for operating band 103

Table 8.1.3.1.1.103-1: NB-IoT standalone Test frequencies for operating band 103

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134283 | 0 | 787.1 | 70647 | -0.5 | 757.1 |
| Mid Range | 134287 | 0 | 787.5 | 70651 | -0.5 | 757.5 |
| High Range | 134291 | 0 | 787.9 | 70655 | -0.5 | 757.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.103-1a: NB-IoT standalone Test frequencies for operating band 103 with NS_04

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 134284 | 0 | 787.2 | 70648 | -0.5 | 757.2 |
| Mid Range | 134287 | 0 | 787.5 | 70651 | -0.5 | 757.5 |
| High Range | 134290 | 0 | 787.8 | 70654 | -0.5 | 757.8 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

8.1.3.1.1.104 to 8.1.3.1.1.252	FFS

Note:	Sub-clauses 8.1.3.1.1.104 to 8.1.3.1.1.252 are marked as FFS as bands 104 to 252 are not yet defined in TS 36.101 [27] or TS 36.102 [74].

8.1.3.1.1.253	NB-IoT FDD reference test frequencies for operating band 253

Table 8.1.3.1.1.253-1: NB-IoT standalone Test frequencies for operating band 253

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261270 | 0 | 1668.1 | 228502 | 0 | 1518.1 |
| Mid Range | 261304 | 0 | 1671.5 | 228536 | 0 | 1521.5 |
| High Range | 261338 | 0 | 1674.9 | 228570 | 0 | 1524.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.254	NB-IoT FDD reference test frequencies for operating band 254

Table 8.1.3.1.1.254-1: NB-IoT standalone Test frequencies for operating band 254

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261340 | 0 | 1610.1 | 228572 | 0 | 2483.6 |
| Mid Range | 261422 | 0 | 1618.3 | 228654 | 0 | 2491.8 |
| High Range | 261503 | 0 | 1626.4 | 228735 | 0 | 2499.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.255	NB-IoT FDD reference test frequencies for operating band 255

Table 8.1.3.1.1.255-1: NB-IoT standalone Test frequencies for operating band 255

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261505 | 0 | 1626.6 | 228737 | 0 | 1525.1 |
| Mid Range | 261674 | 0 | 1643.5 | 228906 | 0 | 1542.0 |
| High Range | 261843 | 0 | 1660.4 | 229075 | 0 | 1558.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

8.1.3.1.1.256	NB-IoT FDD reference test frequencies for operating band 256

Table 8.1.3.1.1.256-1: NB-IoT standalone Test frequencies for operating band 256

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 261845 | 0 | 1980.1 | 229077 | 0 | 2170.1 |
| Mid Range | 261994 | 0 | 1995.0 | 229226 | 0 | 2185.0 |
| High Range | 262143 | 0 | 2009.9 | 229375 | 0 | 2199.9 |
| NOTE 1: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

##### 8.1.3.1.2	NB-IoT TDD Mode Test frequencies

8.1.3.1.2.1 to 8.1.3.1.2.40	Void

8.1.3.1.2.41	NB-IoT TDD reference test frequencies for operating band 41

Table 8.1.3.1.1.41-1: NB-IoT standalone Test frequencies for operating band 41

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 39651 | 0 | 2496.1 | 39651 | -0.5 | 2496.1 |
| Mid Range | 40620 | 0 | 2593.0 | 40620 | -0.5 | 2593.0 |
| High Range | 41589 | 0 | 2689.9 | 41589 | -0.5 | 2689.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.41-2: NB-IoT in-band Test frequencies for operating band 41

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 39657 | 0 | 2496.7000 | 39657 | -2 | 2496.6925 |
| Low Range2 | 39710 | -2 | 2501.9900 | 39710 | -1 | 2501.9975 |
| Low Range3 | 39719 | -2 | 2502.8900 | 39719 | -1 | 2502.8975 |
| Mid Range1 | 40611 | 0 | 2592.1000 | 40611 | -2 | 2592.0925 |
| Mid Range2 | 40630 | -2 | 2593.9900 | 40630 | -1 | 2593.9975 |
| Mid Range3 | 40639 | -2 | 2594.8900 | 40639 | -1 | 2594.8975 |
| High Range1 | 41583 | 0 | 2689.3000 | 41583 | 1 | 2689.3075 |
| High Range2 | 41550 | -2 | 2685.9900 | 41550 | -1 | 2685.9975 |
| High Range3 | 41559 | -2 | 2686.8900 | 41559 | -1 | 2686.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.1.41-3: NB-IoT guard-band Test frequencies for operating band 41

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 39651 | 3 | 2496.1150 | 39651 | 1 | 2496.1075 |
| Mid Range | 5 | 40596 | 3 | 2590.6150 | 40596 | 1 | 2590.6075 |
|  | 10 | 40666 | 3 | 2597.6150 | 40666 | 1 | 2597.6075 |
| High Range | Any | 41589 | -3 | 2689.8850 | 41589 | -2 | 2689.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.2.42	NB-IoT TDD reference test frequencies for operating band 42

Table 8.1.3.1.2.42-1: NB-IoT standalone Test frequencies for operating band 42

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 41591 | 0 | 3400.1 | 41591 | -0.5 | 3400.1 |
| Mid Range | 42590 | 0 | 3500.0 | 42590 | -0.5 | 3500.0 |
| High Range | 43589 | 0 | 3599.9 | 43589 | -0.5 | 3599.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.2.42-2: NB-IoT in-band Test frequencies for operating band 42

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 41597 | 0 | 3400.7000 | 41597 | -2 | 3400.6925 |
| Low Range2 | 41650 | -2 | 3405.9900 | 41650 | -1 | 3405.9975 |
| Low Range3 | 41659 | -2 | 3406.8900 | 41659 | -1 | 3406.8975 |
| Mid Range1 | 42581 | 0 | 3499.1000 | 42581 | -2 | 3499.0925 |
| Mid Range2 | 42600 | -2 | 3500.9900 | 42600 | -1 | 3500.9975 |
| Mid Range3 | 42609 | -2 | 3501.8900 | 42609 | -1 | 3501.8975 |
| High Range1 | 43583 | 0 | 3599.3000 | 43583 | 1 | 3599.3075 |
| High Range2 | 43550 | -2 | 3595.9900 | 43550 | -1 | 3595.9975 |
| High Range3 | 43559 | -2 | 3596.8900 | 43559 | -1 | 3596.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.2.42-3: NB-IoT guard-band Test frequencies for operating band 42

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 41591 | 3 | 3400.1150 | 41591 | 1 | 3400.1075 |
| Mid Range | 5 | 42566 | 3 | 3497.6150 | 42566 | 1 | 3497.6075 |
|  | 10 | 42636 | 3 | 3504.6150 | 42636 | 1 | 3504.6075 |
| High Range | Any | 43589 | -3 | 3599.8850 | 43589 | -2 | 3599.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

8.1.3.1.2.43	NB-IoT TDD reference test frequencies for operating band 43

Table 8.1.3.1.2.43-1: NB-IoT standalone Test frequencies for operating band 43

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range | 39651 | 0 | 2496.1 | 39651 | -0.5 | 2496.1 |
| Mid Range | 40620 | 0 | 2593.0 | 40620 | -0.5 | 2593.0 |
| High Range | 41589 | 0 | 2689.9 | 41589 | -0.5 | 2689.9 |
| NOTE 1: Applicable to either 3.75 kHz or 15 kHz NB-IoT UL subcarrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.2.43-2: NB-IoT in-band Test frequencies for operating band 43

| Test Frequency ID | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- |
| Low Range1 | 43597 | 0 | 3600.7000 | 43597 | -2 | 3600.6925 |
| Low Range2 | 43650 | -2 | 3605.9900 | 43650 | -1 | 3605.9975 |
| Low Range3 | 43659 | -2 | 3606.8900 | 43659 | -1 | 3606.8975 |
| Mid Range1 | 44581 | 0 | 3699.1000 | 44581 | -2 | 3699.0925 |
| Mid Range2 | 44600 | -2 | 3700.9900 | 44600 | -1 | 3700.9975 |
| Mid Range3 | 44609 | -2 | 3701.8900 | 44609 | -1 | 3701.8975 |
| High Range1 | 45583 | 0 | 3799.3000 | 45583 | 1 | 3799.3075 |
| High Range2 | 45550 | -2 | 3795.9900 | 45550 | -1 | 3795.9975 |
| High Range3 | 45559 | -2 | 3796.8900 | 45559 | -1 | 3796.8975 |
| NOTE 1: Related to LTE channel BW 5 MHz NOTE 2: Related to LTE channel BW 10 MHz and NB-IoT PRB 30 NOTE 3: Related to LTE channel BW 10 MHz and NB-IoT PRB 35 NOTE 4: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |

Table 8.1.3.1.2.43-3: NB-IoT guard-band Test frequencies for operating band 43

| Test Frequency ID | LTE Cell Bandwidth [MHz] | NUL | MUL | Frequency of Uplink [MHz] | NDL | MDL | Frequency of Downlink [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low Range | Any | 43591 | 3 | 3600.1150 | 43591 | 1 | 3600.1075 |
| Mid Range | 5 | 44566 | 3 | 3697.6150 | 44566 | 1 | 3697.6075 |
|  | 10 | 44636 | 3 | 3704.6150 | 44636 | 1 | 3704.6075 |
| High Range | Any | 45589 | -3 | 3799.8850 | 45589 | -2 | 3799.8925 |
| NOTE 1: Void NOTE 2: Defined for NB-IoT UL subcarrier spacing 15 kHz. Also applicable for 3.75 kHz UL sub-carrier spacing |  |  |  |  |  |  |  |

#### 8.1.3.2	NB-IoT Radio conditions

##### 8.1.3.2.1	NB-IoT Normal propagation condition

Same normal propagation condition as defined in clause 4.3.2.1 apply.

#### 8.1.3.3	NB-IoT Physical channel allocations

##### 8.1.3.3.1	NB-IoT Antennas

As the UE has one Rx antenna, the downlink signal is applied to it.

##### 8.1.3.3.2	NB-IoT Downlink physical channels and physical signals

The Downlink Physical channels and Physical signals used and their relative powers are specified for either single or two SS Tx antenna in table 8.1.3.3.2-1.  According to this table, no boosting needs to be signalled (as power ratios are fixed between NRS EPRE and all DL channels for a given number of DL antenna ports).

According to TS 36.213 [29] clause 16.2.2, if higher layer parameter operationModeInfo indicates ‘00’ for a cell, the ratio of NRS EPRE to CRS EPRE is given by the parameter nrs-CRS-PowerOffset if the parameter nrs-CRS-EPRE-Ratio is provided by higher layers, and the ratio of NRS EPRE to CRS EPRE may be assumed to be 0 dB if the parameter nrs-CRS-EPRE-Ratio is not provided by higher layers.

Table 8.1.3.3.2-1: NB-IoT Power allocation for OFDM symbols and reference signals (1 or 2 SS Tx antenna)

| Physical Channel / Signal |  | EPRE Ratio |  |
| --- | --- | --- | --- |
| NPBCH |  | According to TS 36.213 [29] clause 16.2.2, a UE may assume the ratio of NPBCH EPRE to NRS EPRE among NPBCH REs (not applicable to NPBCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports. |  |
| NPSS |  | Power allocation for NPSS is defined as equal to NRS EPRE = -85 dBm/15 kHz in order to have a constant EPRE (an absolute value is used as there is no NRS transmission when NPSS is transmitted). |  |
| NSSS |  | Power allocation for NSSS is defined as equal to NRS EPRE = -85 dBm/15 kHz in order to have a constant EPRE (an absolute value is used as there is no NRS transmission when NPSS is transmitted). |  |
| NPDCCH |  | According to TS 36.213 [29] clause 16.2.2, a UE may assume the ratio of NPDCCH EPRE to NRS EPRE among NPDCCH REs (not applicable to NPDCCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports. |  |
| NPDSCH |  | According to TS 36.213 [29] clause 16.2.2, a UE may assume the ratio of NPDSCH EPRE to NRS EPRE among NPDSCH REs (not applicable to NPDSCH REs with zero EPRE) is 0 dB for an NB-IoT cell with one NRS antenna port and -3 dB for an NB-IoT cell with two NRS antenna ports. |  |
| NRS |  | - If the number of NRS antenna ports is one, the EPRE of NRS and the EPRE of all NB-IoT DL channels is the same - If the number of NRS antenna ports is two, the EPRE per antenna port of NRS port is 3dB larger compared to the EPRE per antenna port of all NB-IoT DL channels - This means no signalling support of power offsets - The set of values for power offset between NRS and LTE CRS is {-6, -4.77, -3, -1.77, 0, 1, 1.23, 2, 3, 4, 4.23, 5, 6, 7, 8, 9} dB. |  |
| NWUS |  | According to TS 36.213 [29] clause 16.2.2, the ratio of NWUS EPRE to NRS EPRE is 0 dB. |  |

##### 8.1.3.3.3	NB-IoT Mapping of downlink physical channels and signals to physical resources

Parameters for mapping of NB-IoT downlink physical channels and signals are specified as follows:

Normal Cyclic Prefix

, Physical layer cell identity	= 0 is used as the default physical layer cell identity

For RF testing, the mapping of DL physical channels to resource element is defined TS 36.521-1 [21] Annex C.1.

Table 8.1.3.3.3-1: NB-IoT Mapping of DL Physical Channels and Signals to Resource Elements (FDD)

| Physical channel / signal | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPBCH | - NPBCH is transmitted in subframe 0 in every radio frame - NPBCH consists of 8 independently decodable blocks of 80 ms duration - The time interval where MIB remains unchanged is 640 ms - NPBCH does not use the first 3 symbols in a subframe in in-band operation. - For stand-alone and guard-band, the first 3 symbols (of the subframe transmitting NPBCH) contain no NPBCH. | Occupies any of the 12 subcarriers not reserved for transmission of reference signals | Mapping rule is specified in TS 36.211 [35] sub clause 10.2.4.4 |
| NPSS | - NPSS is transmitted in subframe 5 - NPSS uses the last 11 OFDM symbols of subframes in which NB-PSS occurs for normal CP - NB-IoT PSS/SSS do not use: - the LTE PDCCH control region - REs used by LTE CRS - The periodicity of NPSS transmission is 10ms. | NPSS is mapped to sub-carriers #0-10 of the NB-IoT carrier | Mapping rule is specified in TS 36.211 [35] sub clause 10.2.7.1.2 |
| NSSS | - NSSS is transmitted in subframe 9 - NSSS uses the last 11 OFDM symbols of subframes in which NB-SSS occurs for normal CP - NB-IoT PSS/SSS do not use: - the LTE PDCCH control region - REs used by LTE CRS - NSSS periodicity is 20ms | The number of subcarriers for NSSS is 12 | Mapping rule is specified in TS 36.211 [35] sub clause 10.2.7.2.2 |
| NPDCCH | - NPDCCH on a given NB-IoT carrier are not mapped to the subframes containing NPSS/NSSS/PBCH on that carrier - NPDCCH are not be mapped onto resources elements used for NRS - NPDCCH are not overlapped with PBCH, PSS, SSS, or CRS - One or two NPDCCHs can be transmitted in a subframe. - In in-band, first three OFDM symbols are not used for NPDCCH - In stand-alone and guard-band, all OFDM symbols are available for NPDCCH - NPDCCH and NPDSCH are multiplexed only based on TDM at subframe level - It means that only cross subframe scheduling is supported - The start of an NPDCCH search space is >=4ms after the end of the last NPDCCH search space | - NPDCCH is transmitted on an aggregation of one or two consecutive narrowband control channel elements (NCCEs), where a narrowband control channel element corresponds to 6 consecutive subcarriers in a subframe where NCCE 0 occupies subcarriers 0 through 5 and NCCE 1 occupies subcarriers 6 through 11. - REG is not defined for NPDCCH | Mapping rule is specified in TS 36.211 [35] sub clause 10.2.5.5 |
| NPDSCH | - The start of NB-PDSCH transmission is >=4ms later than the end of its associated DL assignment - NPDSCH on a given NB-IoT carrier are not mapped to the subframes containing NPSS/NSSS/PBCH on that carrier - NPDSCH resources elements should be different from the ones used for NRS - NPDSCH resources elements should be different from the ones used for CRS | Occupies any of the 12 subcarriers not reserved for transmission of reference signals | Mapping rule is specified in TS 36.211 [35] sub clause 10.2.3.4 |
| NRS | - Narrowband reference signals are transmitted in all NB-IoT downlink subframes in a cell supporting NPDSCH transmission. - NRS is not transmitted in subframes that are not NB-IoT downlink subframes, except if these subframes contain NPBCH or NPDSCH carrying SystemInformationBlockType1-NB where NRS shall be transmitted. - Narrowband reference signals are transmitted in subframes #0 and #4 and in subframes #9 not containing NSSS [when no NB-IoT downlink subframes configuration has not been established]. - The narrowband reference signals shall not be mapped to subframes containing NPSS or NSSS. - NRS are transmitted on one or two antenna ports 0 to 1 (refer to Figure 10.2.6.2-1 in TS 36.211 [35]). | 2 subcarriers per antenna port in any OFDM symbols where NRS is transmitted |  |
| NWUS | - NWUS and its associated NB-IoT paging occasion subframes are on the same NB-IoT carrier. - NWUS shall not be transmitted in subframes #4 carrying SystemInformationBlockType1-NB, those subframes are counted in the NWUS mapping but are not used for transmission of NWUS. | The number of subcarriers for NWUS is 12. The same antenna port shall be used for all symbols of the NWUS within a subframe. | Mapping rule is specified in TS 36.211 [35] sub clause 10.2.6B.2 |

Table 8.1.3.3.3-2: NB-IoT Mapping of DL Physical Channels to Resource Elements (TDD)

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| NPBCH | Same as in Table 8.1.3.3.3-1, except for - NPBCH is transmitted in subframe 9 in every radio frame | Same as in Table 8.1.3.3.3-1 |  |
| NPSS | Same as in Table 8.1.3.3.3-1 | Same as in Table 8.1.3.3.3-1 |  |
| NSSS | Same as in Table 8.1.3.3.3-1, except for - NSSS is transmitted in subframe 0 | Same as in Table 8.1.3.3.3-1 |  |
| NPDCCH | Same as in Table 8.1.3.3.3-1, except for - Except in a special subframe when NPDCCH is transmitted in more than one subframe, - In a special subframe where the NPDCCH is transmitted in one subframe, NPDCCH are mapped into DwPTS - In a special subframe where the NPDCCH is transmitted in more than one subframe, NPDCCH are not mapped to NRS locations when the subframe is not a special subframe. | Same as in Table 8.1.3.3.3-1 |  |
| NPDSCH | Same as in Table 8.1.3.3.3-1, except for - Except in a special subframe when >1, NPDSCH resources elements should be different from the ones used for NRS - NPDSCH resources elements should be different from the ones used for CRS - In a special subframe, if , NPDSCH are mapped into DwPTS - In a special subframe, if , NPDSCH are not mapped to NRS locations in subframes which are not special subframes. | Same as in Table 8.1.3.3.3-1 |  |
| NRS | Same as in Table 8.1.3.3.3-1, except for - Narrowband reference signals are transmitted in subframes #9 and in subframes #0 not containing NSSS [when no ddNB-IoT downlink subframes configuration has not been established]. | Same as in Table 8.1.3.3.3-1 |  |

##### 8.1.3.3.4	NB-IoT Uplink physical channels and physical signals

The NB-IoT uplink setting in table 8.1.3.3.4-1 is used unless otherwise specified in a test case.

Table 8.1.3.3.4-1: NB-IoT Default Uplink setting

|  | Unit | Channel bandwidth |
| --- | --- | --- |
|  |  | 200 kHz |
| Number of Tones |  | 1 |
| Sub-carrier spacing | kHz | 15 |

##### 8.1.3.3.5	NB-IoT Mapping of uplink physical channels and signals to physical resources

[FFS]

#### 8.1.3.4	NB-IoT Signal levels

##### 8.1.3.4.1	NB-IoT Downlink signal levels

The downlink power settings in table 8.1.3.4.1-1 are used unless otherwise specified in a test case.

Table 8.1.3.4.1-1: NB-IoT Default Downlink power levels

|  | Unit | Channel bandwidth |
| --- | --- | --- |
|  |  | 200 kHz |
| Number of Tones |  | 12 |
| Channel BW Power | dBm | -74 |
| NRS EPRE | dBm/15kHz | -85 |
| Note 1: The channel bandwidth powers are informative, based on -85dBm/15kHz NRS_EPRE, then scaled according to the number of tones in DL and rounded to the nearest integer dBm value. Note 2: The power level is specified at the UE Rx antenna |  |  |

It is [FFS] whether there is a requirement to specify constant power throughout all OFDM symbols, and if so how unallocated Resource elements should be treated.

The default signal level uncertainty is +/-3dB at each test port, for any level specified. If the uncertainty value is critical for the test purpose a tighter uncertainty is specified for the related test case in TS 36.521-1 [21] Annex F or in TS 36.521-3 [34] Annex F

##### 8.1.3.4.2	NB-IoT Uplink signal levels

[FFS]

#### 8.1.3.5	NB-IoT Standard test signals

##### 8.1.3.5.1	NB-IoT Downlink test signals

[FFS]

##### 8.1.3.5.2	NB-IoT Uplink test signals

[FFS]

#### 8.1.3.6	NB-IoT Physical layer parameters

##### 8.1.3.6.1	NB-IoT Downlink physical layer parameters

8.1.3.6.1.1	NB-IoT Physical layer parameters for DCI format N0

Default NB-IoT physical layer parameters for DCI format N0 are specified in table 8.1.3.6.1.1-1.

Table 8.1.3.6.1.1-1: NB-IoT Physical layer parameters for DCI format N0

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format N0/format N1differentiation | format N0 | "0" |
| Subcarrier indication | 0 if not specified othewise depending on test case (6 bits as defined in section 16.5.1.1 of 36.213 [29]) | - |
| Resource assignment | Depending on test parameters (3 bits as defined in section 16.5.1.1 of 36.213 [29]) | - |
| Scheduling delay | 0 (2 bits as defined in section 16.5.1 of of 36.213 [29]) | “00” |
| Modulation and coding scheme | Depending on test parameters (4 bits as defined in section 16.5.1.2 of 36.213 [29]) | - |
| Redundancy version | 0 (1 bit as defined in clause 16.5.1.2 of 36.213 [29]) | "0" |
| Repetition number | NRep = 1 (3 bits as defined in clause 16.5.1.2 of 36.213 [29]) | “000” |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 [14] (1 bit) | - |
| DCI subframe repetition number | R = 2 (according to NPDCCH configuration of Table 8.1.6.3-3)(2 bits as defined in section 16.6 in 36.213 [29]) | “00” |
| HARQ process number | Mandatory present if 2 HARQ processes are configured in the UE, default value (HARQ process) "0". Otherwise not present(1 bit as defined in section 16.5.1.2 of 36.213 [29]) | - |

8.1.3.6.1.2	NB-IoT Physical layer parameters for DCI format N1

Default NB-IoT physical layer parameters for DCI format N1 are specified in table 8.1.3.6.1.2-1 for random access procedure initiated by a NPDCCH order, in table 8.1.3.6.1.2-2 for the scheduling of one NPDSCH codeword in one cell scrambled by RA-RNTI/C-RNTI and in table 8.1.3.6.1.2-2 for the scheduling of one NPDSCH codeword in one cell scrambled by G-RNTI.

Table 8.1.3.6.1.2-1: NB-IoT Physical layer parameters for DCI format N1 (for random access procedure initiated by a NPDCCH order)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format N0/format N1 differentiation | format N1 | “1” |
| NPDCCH order indicator | 1 | “1” |
| Starting number of NPRACH repetitions | n1 2 bits as defined in section 16.3.1 of 36.213 [29]) | “00” |
| Subcarrier indication of NPRACH | 12 (6 bits as defined in section 16.3.1 of 36.213 [29]) | “001100” |
| NOTE 1: All the remaining bits in format N1 are set to one |  |  |

Table 8.1.3.6.1.2-2: NB-IoT Physical layer parameters for DCI format N1 (for the scheduling of one NPDSCH codeword in one cell scrambled by RA-RNTI/C-RNTI)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for format N0/format N1 differentiation | format N1 | “1” |
| NPDCCH order indicator | 0 | “0” |
| Scheduling delay | 0 (3 bits as defined in clause16.4.1 of 36.213 [29]) | “000” |
| Resource assignment | Depending on test parameters (3 bits as defined in section 16.4.1.3 of 36.213 [29]) | - |
| Modulation and coding scheme | Depending on test parameters (4 bits as defined in section 16.4.1.5 of 36.213 [29]) | - |
| Repetition number | NRep = 1 for normal coverage and 32 for enhanced coverage test cases (see RRM_NTN_Enh in table 8.1.4.3.2-3) (4 bits as defined in section 16.4.1.3 of 36.213 [29]) | “0000” for normal coverage and “0101” for enhanced coverage. |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 36.321 [14] (1 bit) | - |
| HARQ-ACK resource | 0 if not specified otherwise depending on test case (4 bits as defined in section 16.4.2 of 36.213 [29]) | - |
| DCI subframe repetition number | R = 2 (according to NPRACH configuration of Table 8.1.6.3-5) (2 bits as defined in section 16.6 in 36.213 [29]) | “00” |
| HARQ process number | Mandatory present if 2 HARQ processes are configured in the UE, default value (HARQ process) "0". Otherwise not present (1 bit as defined in section 16.4.1.5 of 36.213 [29]) | - |

Table 8.1.3.6.1.2-3: NB-IoT Physical layer parameters for DCI format N1 (for the scheduling of one NPDSCH codeword in one cell scrambled by G-RNTI)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Information for SC-MCCH change notification | 0 (2 bit as defined in section 5.8a of 36.331 [17]) | “00” |
| Scheduling delay | 0 (3 bits as defined in clause16.4.1 of 36.213 [29]) | “000” |
| Resource assignment | Depending on test parameters (3 bits as defined in section 16.4.1.3 of 36.213 [29]) | - |
| Modulation and coding scheme | Depending on test parameters (4 bits as defined in section 16.4.1.5 of 36.213 [29]) | - |
| Repetition number | NRep = 1 (4 bits as defined in section 16.4.1.3 of 36.213 [29]) | “0000” |
| DCI subframe repetition number | R = 2 (according to SC-MTCH configuration of Table 8.1.6.1-15a) (2 bits as defined in section 16.6 in 36.213 [29]) | “00” |
| HARQ process number | Mandatory present if 2 HARQ processes are configured in the UE, default value (HARQ process) "0". Otherwise not present (1 bit as defined in section 16.4.1.5 of 36.213 [29]) | - |

8.1.3.6.1.3	(Void)

8.1.3.6.1.4	(Void)

8.1.3.6.1.5	NB-IoT Physical layer parameters for DCI format N2

Default NB-IoT physical layer parameters for DCI format N2 are specified in table 8.1.3.6.1.5-1 for paging, in table 8.1.3.6.1.5-2 for direct indication and in table 8.1.3.6.1.5-3 for scheduling of one NPDSCH codeword carrying SC-MCCH in one cell.

Table 8.1.3.6.1.5-1: NB-IoT Physical layer parameters for DCI format N2 (for paging)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for paging/direct indication differentiation | 1 | “1” |
| Resource assignment | Depending on test parameters (3 bits as defined in section 16.4.1.3 of 36.213 [29]) | - |
| Modulation and coding scheme (transport block 1) | Depending on test parameters (4 bits as defined in section 16.4.1.5 of 36.213 [29]) | - |
| Repetition number | NRep = 1 (4 bits as defined in section 16.4.1.3 of 36.213 [29]) | “0000” |
| DCI subframe repetition number | R = 1 (according to PCCH configuration of Table 8.1.6.3-2) (3 bits as defined in section 16.6 of 36.213 [29]) | “000” |

Table 8.1.3.6.1.5-2: NB-IoT Physical layer parameters for DCI format N2 (for direct indication)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Flag for paging/direct indication differentiation | 0 | “0” |
| Direct Indication information | 1 (8 bits provide direct indication of system information update and other fields, as defined in 36.331 [17]) | “00000001” |
| NOTE 1: Reserved information bits are added until the size is equal to that of format N2 with Flag=1 |  |  |

Table 8.1.3.6.1.5-3: NB-IoT Physical layer parameters for DCI format N2 (for one NPDSCH codeword carrying SC-MCCH in one cell)

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Information for SC-MCCH change notification | 0 (1 bit as defined in section 5.8a of 36.331 [17]) | “0” |
| Resource assignment | Depending on test parameters (3 bits as defined in section 16.4.1.3 of 36.213 [29]) | - |
| Modulation and coding scheme (transport block 1) | Depending on test parameters (4 bits as defined in section 16.4.1.5 of 36.213 [29]) | - |
| Repetition number | NRep = 1 (4 bits as defined in section 16.4.1.3 of 36.213 [29]) | “0000” |
| DCI subframe repetition number | R = 1 (according to SC-MCCH configuration of Table 8.1.4.3.3-7) (3 bits as defined in section 16.6 of 36.213 [29]) | “000” |

### 8.1.4	NB-IoT Reference system configurations

The reference system configurations specified in this sub clause apply to all NB-IoT test cases unless otherwise specified.

#### 8.1.4.1	NB-IoT Simulated network scenarios

Simulated network scenarios to be tested are listed in this sub clause.

NOTE 1:	The number of cells specified does not necessarily correspond to the maximum number of resources to be configured simultaneously in test equipment. Please refer to clause 8.3.1 for such information.

NOTE 2:	Void.

##### 8.1.4.1.1	NB-IoT Single cell network scenarios

For NB-IoT basic single cell environment, Ncell 1 is used.

##### 8.1.4.1.2	NB-IoT single mode multi cell network scenarios

For NB-IoT basic intra-frequency multi cell environment, Ncell 1, Ncell 2 and Ncell 4 are used.

For NB-IoT basic inter-frequency multi cell environment, Ncell 1 Ncell 3 and Ncell 6 are used.

For NB-IoT basic inter-band cell environment, Ncell 1 and Ncell 10 are used.

For NB-IoT multi tracking area intra-frequency multi cell environment, Ncell 1 and Ncell 11 are used.

For NB-IoT multi tracking area inter-frequency multi cell environment, Ncell 1 and Ncell 23 are used.

For NB-IoT multi PLMN inter-frequency multi cell environment, Ncell 1, Ncell 12, Ncell 13 and Ncell 14 are used.

#### 8.1.4.2	NB-IoT Simulated cells

NOTE 1:	Void

NOTE 2:	Test frequency and range defined in table 8.1.4.2-1 do not apply to 36.521-1 test cases.

Test frequencies and simulated cells are defined in table 8.1.4.2-1. For NB-IoT cells, f1 is the default test frequency.

Default parameters for simulated cells are specified in table 8.1.4.2-1A and table 8.1.4.2-2.

Common parameters for simulated cells are specified in subclauses 8.1.4.3.

Table 8.1.4.2-1: Definition of test frequencies and simulated NB-IoT cells – Batch1

| Test frequency | RAT | Operating band | Range | Simulated cells |
| --- | --- | --- | --- | --- |
| f1 | NB-IoT | Operating band under test | Mid (Note 1) | Ncell 1, Ncell 1a, Ncell 2, Ncell 4, Ncell5, Ncell 6a, Ncell 11 (Note 2), Ncell 18 |
| f2 | NB-IoT | Operating band under test | High (Note 1) | Ncell 3, Ncell 12, Ncell 23 |
| f3 | NB-IoT | Operating band under test | Low (Note 1) | Ncell 6, Ncell 13 |
| f4 | NB-IoT | Operating band under test | (Note 1) | Ncell 14 |
| f5 | NB-IoT | Operating band for inter-band cells | Mid (Note 1) | Ncell 10 |
| Note 1: For signalling test, see clause 8.3.2.3 (note that f4 is not defined for signalling tests). Note 2: For signalling test, simultaneous co-existence of Ncell 2 with Ncell 11 is not allowed. |  |  |  |  |

Table 8.1.4.2-1A: Default NB-IoT parameters for simulated NB-IoTcells – Batch1

| cell ID | NB-IoT Cell Identifier |  | Physical layer cell identity |
| --- | --- | --- | --- |
|  | eNB Identifier | Cell Identity |  |
| Ncell 1 | '0000 0000 0000 0000 0001'B | '0000 0000'B | 0 |
| Ncell 1a | '0000 0000 0000 0000 0001'B | '0000 0001'B | 1 |
| Ncell 2 | '0000 0000 0000 0000 0001'B | '0000 0010'B | 2 |
| Ncell 3 | '0000 0000 0000 0000 0010'B | '0000 0011'B | 3 |
| Ncell 4 | '0000 0000 0000 0000 0011'B | '0000 0100'B | 4 |
| Ncell 5 | '0000 0000 0000 0000 0011'B | '0000 0101'B | 5 |
| Ncell 6 | '0000 0000 0000 0000 0100'B | '0000 0110'B | 6 |
| Ncell 6a | '0000 0000 0000 0000 0100'B | '0000 0110'B | 6 |
| Ncell 10 | '0000 0000 0000 0000 0101'B | '0000 1010'B | 10 |
| Ncell 11 | '0000 0000 0000 0000 0110'B | '0000 1011'B | 11 |
| Ncell 12 | '0000 0000 0000 0000 0010'B | '0000 1100'B | 12 |
| Ncell 13 | '0000 0000 0000 0000 0100'B | '0000 1101'B | 13 |
| Ncell 14 | '0000 0000 0000 0000 0111'B | '0000 1110'B | 14 |
| Ncell 18 | '0000 0000 0000 0000 0100'B | '0001 0010'B | 18 |
| Ncell 23 | '0000 0000 0000 0000 0110'B | '0001 0111'B | 23 |

Table 8.1.4.2-2: Default NAS parameters for simulated NB-IoT cells – Batch1

| cell ID | Tracking Area |  |  |  | TA# list (Note 1) | GUTI (Note 2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | MME Identifier |  | M-TMSI |
|  |  | MCC | MNC |  |  | MME Group ID | MME Code |  |
| Ncell 1 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.8 [2]. |
| Ncell 1a | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 2 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 3 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 4 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 5 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Ncell 6 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 6a | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Ncell 10 | TAI-10 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 11 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Ncell 12 | TAI-3 | 002 | 11 | 1 | TAI-3 | 32769 | 1 |  |
| Ncell 13 | TAI-4 | 003 | 21 | 1 | TAI-4 | 32769 | 1 |  |
| Ncell 14 | TAI-5 | 004 | 31 | 1 | TAI-5 | 32769 | 1 |  |
| Ncell 18 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 23 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32769 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column GUTI indicates GUTI included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). |  |  |  |  |  |  |  |  |

Table 8.1.4.2-3: Default NAS parameters for simulated NB-IoT cells – Batch2

| NAS cell ID | Tracking Area |  |  |  | TA# list (Note 1) | GUTI (Note 2) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TA# | PLMN |  | TAC |  | MME Identifier |  | M-TMSI |
|  |  | MCC | MNC |  |  | MME Group ID | MME Code |  |
| Ncell 50 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 | Arbitrarily selected according to TS 23.003 subclause 2.8 [2]. |
| Ncell 51 | TAI-2 | (Note 3) |  | 2 | TAI-2 | 32770 | 1 |  |
| Ncell 52 | TAI-3 | (Note 3) |  | 3 | TAI-3 | 32771 | 1 |  |
| Ncell 53 | TAI-4 | (Note 3) |  | 4 | TAI-4 | 32772 | 1 |  |
| Ncell 54 | TAI-12 | 002 | 101 | 3 | TAI-12 | 32777 | 1 |  |
| Ncell 55 | TAI-7 | (Note 4) | 02 | 1 | TAI-7 | 32775 | 1 |  |
| Ncell 56 | TAI-8 | (Note 4) | 02 | 2 | TAI-8 | 32776 | 1 |  |
| Ncell 57 | TAI-9 | 002 | 101 | 1 | TAI-9 | 32777 | 1 |  |
| Ncell 58 | TAI-10 | 003 | 101 | 1 | TAI-10 | 32778 | 1 |  |
| Ncell 59 | TAI-9 | 002 | 101 | 1 | TAI-9 | 32777 | 1 |  |
| Ncell 60 | TAI-11 | 002 | 101 | 2 | TAI-11 | 32779 | 1 |  |
| Ncell 61 | TAI-1 | (Note 3) |  | 1 | TAI-1 | 32769 | 1 |  |
| Ncell 62 | TAI-7 | (Note 4) | 02 | 1 | TAI-7 | 32775 | 1 |  |
| Ncell 63 | TAI-13 | (Note 4) | 03 | 1 | TAI-13 | 32770 | 1 |  |
| Note 1: The value(s) in the column TA# list indicates TAI(s) included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 2: The value in the column GUTI indicates GUTI included in the response messages of the registration procedure (ATTACH ACCEPT or TRACKING AREA UPDATE ACCEPT) when the UE performs the registration procedure on a corresponding cell. Note 3: Set to the same Mobile Country Code and Mobile Network Code stored in EFIMSI on the test USIM card (subclause 4.9.3). Note 4: Set to the same Mobile Country Code stored in EFIMSI on the test USIM card (subclause 4.9.3). Note 5: Ncell 50 is a serving cell and the other cells are suitable neighbour cells. The definitions are specified in subclause 8.3.2.2.1. |  |  |  |  |  |  |  |  |

Table 8.1.4.2-4: Default cell identifiers for simulated NB-IoT cells – Batch2 when cells are in same PLMN

| cell ID | Frequency | NB-IoT Cell Identifier |  | Physical layer cell identity |
| --- | --- | --- | --- | --- |
|  |  | eNB Identifier | Cell Identity |  |
| Ncell 50 | f1 | ‘0000 0000 0000 0001 0001’B | '0000 0001'B | 1 |
| Ncell 51 | f1 | ‘0000 0000 0000 0001 0001’B | '0000 0010'B | 2 |
| Ncell 52 | f2 | ‘0000 0000 0000 0001 0010’B | '0000 0011'B | 3 |
| Ncell 53 | f1 | ‘0000 0000 0000 0001 0001’B | '0000 0100'B | 4 |
| Ncell 54 | NA | NA | NA | NA |
| Ncell 55 | NA | NA | NA | NA |
| Ncell 56 | NA | NA | NA | NA |
| Ncell 57 | NA | NA | NA | NA |
| Ncell 58 | NA | NA | NA | NA |
| Ncell 61 | f2 | ‘0000 0000 0000 0010 0001’B | '0001 0001'B | 17 |

Table 8.1.4.2-5: Default cell identifiers for simulated NB-IoT cells – Batch2 when cells are in different PLMNs

| NAS cell ID | PLMN | Frequency | NB-IoT Cell Identifier |  | Physical layer cell identity |
| --- | --- | --- | --- | --- | --- |
|  |  |  | eNB Identifier | Cell Identity |  |
| Ncell 50 | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0001'B | 1 |
| Ncell 51 | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0010'B | 2 |
| Ncell 52 | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0011'B | 3 |
| Ncell 53 | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0000 0100'B | 4 |
| Ncell 54 | MCC=002 MNC=101 | f2 | ‘0000 0000 0000 0011 0101’B | '0001 0111'B | 23 |
| Ncell 55 | MCC = MCC in USIM MNC=02 | f2 | '0000 0000 0000 0010 0010'B | '0000 1011'B | 11 |
| Ncell 56 | MCC= MCC in USIM MNC=02 | f2 | '0000 0000 0000 0010 0010'B | '0000 1100'B | 12 |
| Ncell 57 | MCC=002 MNC=101 | f3 (Note 1) | '0000 0000 0000 0010 0011'B | '0000 1101'B | 13 |
| Ncell 58 | MCC=003 MNC=101 | f4 (Note 1) | '0000 0000 0000 0010 0100'B | '0000 1110'B | 14 |
| Ncell 59 | MCC=002 MNC=101 | f3 (Note 1) | '0000 0000 0000 0010 0011'B | '0000 1111'B | 15 |
| Ncell 60 | MCC=002 MNC=101 | f3 (Note 1) | '0000 0000 0000 0010 0011'B | '0001 0000'B | 16 |
| Ncell 61 | MCC/MNC= MCC/MNC in USIM | f1 | '0000 0000 0000 0010 0001'B | '0001 0001'B | 17 |
| Ncell 62 | MCC = MCC in USIM MNC=02 | f1 | '0000 0000 0000 0010 0010'B | '0001 0010'B | 18 |
| Ncell 63 | MCC = MCC in USIM MNC=03 | f1 | '0000 0000 0000 0010 0010'B | '0001 0011'B | 19 |
| Note 1: The test frequency f3 or f4 is allocated to the cell if f1 and f2 are already allocated to the cells in the test. Otherwise, f1 or f2 is allocated instead. Note that f4 is not defined for signalling tests (see clause 8.3.2.3). |  |  |  |  |  |

Table 8.1.4.2-6 defines TAC values only for Idle Mode Test Cases defined in TS 36.523-1 section 22.2.

Table 8.1.4.2-6: Tracking Area Code (TAC) for NB-IoT cells – Batch1 for Idle Mode test cases

| cell ID | TAC |
| --- | --- |
| Ncell 1 | 1 |
| Ncell 2 | 2 |
| Ncell 3 | 3 |
| Ncell 4 | 4 |
| Ncell 6 | 6 |
| Ncell 11 | 11 |
| Ncell 12 | 12 |
| Ncell 13 | 13 |
| Ncell 14 | 14 |
| Ncell 23 | 23 |

#### 8.1.4.3	NB-IoT Common parameters for simulated cells

The parameters specified in this sub clause apply to all simulated cells unless otherwise specified.

##### 8.1.4.3.1	NB-IoT Common configurations of system information blocks

###### 8.1.4.3.1.1	NB-IoT Combinations of system information blocks

The combination of system information blocks required by a test case depends on the test case scenario. In this clause, the following combinations of system information blocks are defined.

Combination 1 is the default combination which applies to the following test case scenarios:

-	single cell scenario

Combination 2 applies to the following test case scenarios:

-	intra-frequency multi cell scenario

-	intra-frequency multi cell scenario with neighbouring cell related information

Combination 3 applies to the following test case scenarios:

-	inter-frequency multi cell scenario

Combination 4 applies to the following test case scenarios:

- access barring single cell scenario

Combination 5 applies to the following test case scenarios:

-	SC-PTM single cell scenario (SC-PTM cell broadcasting SIB20-NB)

Combination 6 applies to the following test case scenarios:

-	single cell on multi carriers scenario

Combination 7 applies to the following test case scenarios:

-	single cell NPRACH resources using preamble format 2 on non-anchor carriers scenario

Combination 8 is the default combination which applies to the following test case scenarios:

-	single NTN cell scenario

Combination 9 is the default combination which applies to the following test case scenarios:

-	intra-frequency multi NTN cell scenario

Combination 10 is the default combination which applies to the following test case scenarios:

-	intra-frequency multi NTN cell scenario with neighbouring NTN cell related information

Combination 11 is the default combination which applies to the following test case scenarios:

-	inter-frequency multi NTN cell scenario

Combination 12 is the default combination which applies to the following test case scenarios:

-	single NTN cell on multi NTN carriers scenario

Combination 13 applies to the following test case scenarios:

-	access barring single NTN cell scenario

Combination 14 applies to the following test case scenarios:

-	intra-frequency multi NTN cell on discontinuous coverage scenario with neighbouring NTN cell related information

Combination 15 applies to the following test case scenarios:

-	intra-frequency multi NTN cell scenario with satellite assistance information for neighbour cells.

-	intra-frequency multi NTN cell scenario for RRM with satellite assistance information for neighbour cells with Test equipment configuration (Rel-18).

Table 8.1.4.3.1.1-1: Combinations of system information blocks

|  | System information block type |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Combination No. | SIB2-NB | SIB3-NB | SIB4-NB | SIB5-NB | SIB14-NB | SIB16-NB | SIB20-NB | SIB22-NB | SIB23-NB | SIB31-NB | SIB32-NB | SIB33-NB |
| 1 | X |  |  |  |  |  |  |  |  |  |  |  |
| 2 | X | X | X |  |  |  |  |  |  |  |  |  |
| 3 | X | X |  | X |  |  |  |  |  |  |  |  |
| 4 | X | X |  |  | X |  |  |  |  |  |  |  |
| 5 | X |  |  |  |  |  | X |  |  |  |  |  |
| 6 | X |  |  |  |  |  |  | X |  |  |  |  |
| 7 | X |  |  |  |  |  |  | X | X |  |  |  |
| 8 | X |  |  |  |  |  |  |  |  | X |  |  |
| 9 | X | X |  |  |  |  |  |  |  | X |  |  |
| 10 | X | X | X |  |  |  |  |  |  | X |  |  |
| 11 | X | X |  | X |  |  |  |  |  | X |  |  |
| 12 | X |  |  |  |  |  |  | X |  | X |  |  |
| 13 | X | X |  |  | X |  |  |  |  | X |  |  |
| 14 | X | X |  |  |  | X |  |  |  | X |  | X |
| 15 | X | X |  |  |  |  |  |  |  | X |  | X |

###### 8.1.4.3.1.2	NB-IoT Scheduling of system information blocks

The scheduling configurations for combinations of system information blocks are defined in the following tables.

Table 8.1.4.3.1.2-1: Scheduling for combination 1

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |

Table 8.1.4.3.1.2-2: Scheduling for combination 2

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB4-NB |

Table 8.1.4.3.1.2-3: Scheduling for combination 3

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB5-NB |

Table 8.1.4.3.1.2-4: Scheduling for combination 4

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB14-NB |

Table 8.1.4.3.1.2-5: Scheduling for combination 5

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB20-NB |

Table 8.1.4.3.1.2-6: Scheduling for combination 6

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-7: Scheduling for combination 7

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB22-NB |
| 3 | 64 | every16thRF | 256 | SIB23-NB |

Table 8.1.4.3.1.2-8: Scheduling for combination 8

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |

Table 8.1.4.3.1.2-8a: Scheduling for combination 8 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB |
| 2 | 256 | every2thRF | 256 | SIB31-NB |

Table 8.1.4.3.1.2-9: Scheduling for combination 9

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB, SIB31-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |

Table 8.1.4.3.1.2-9a: Scheduling for combination 9 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB, SIB31-NB |
| 2 | 256 | every2thRF | 256 | SIB3-NB |

Table 8.1.4.3.1.2-10: Scheduling for combination 10

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB3-NB |
| 4 | 64 | every16thRF | 256 | SIB4-NB |

Table 8.1.4.3.1.2-11: Scheduling for combination 11

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB3-NB |
| 4 | 64 | every16thRF | 256 | SIB5-NB |

Table 8.1.4.3.1.2-12: Scheduling for combination 12

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-12a: Scheduling for combination 12 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB |
| 2 | 256 | every2thRF | 256 | SIB31-NB |
| 3 | 256 | every2thRF | 256 | SIB22-NB |

Table 8.1.4.3.1.2-13: Scheduling for combination 13

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB3-NB |
| 4 | 64 | every16thRF | 256 | SIB14-NB |

Table 8.1.4.3.1.2-14: Scheduling for combination 14

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB, SIB31-NB |
| 2 | 64 | every16thRF | 256 | SIB3-NB |
| 3 | 64 | every16thRF | 256 | SIB33-NB |
| 4 | 64 | every16thRF | 256 | SIB16-NB |

Table 8.1.4.3.1.2-15: Scheduling for combination 15

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 64 | every8thRF | 552 | SIB2-NB |
| 2 | 64 | every16thRF | 256 | SIB31-NB |
| 3 | 64 | every16thRF | 256 | SIB33-NB |
| 4 | 64 | every16thRF | 256 | SIB3-NB |

Table 8.1.4.3.1.2-15a: Scheduling for combination 15 under RRM enh coverage

| Scheduling Information No. | Periodicity [radio frames] | Repetition pattern [radio frames] | TB size [bits] | Mapping of system information blocks |
| --- | --- | --- | --- | --- |
| 1 | 256 | every2thRF | 552 | SIB2-NB |
| 2 | 256 | every2thRF | 256 | SIB31-NB |
| 3 | 256 | every2thRF | 256 | SIB33-NB |
| 4 | 256 | every2thRF | 256 | SIB3-NB |

##### 8.1.4.3.2	NB-IoT Common contents of system information messages

###### -	MasterInformationBlock-NB

The MasterInformationBlock-NB includes the system information transmitted on BCH.

Table 8.1.4.3.2-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r13 | 2 | TBS = 208 bits, 16 repetitions |  |
|  | 5 | TBS = 328 bits, 16 repetitions | NTN |
| systemInfoValueTag-r13 | 0 |  |  |
| ab-Enabled-r13 | FALSE |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| } |  |  |  |
| inband-DifferentPCI-r13 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r13 | same | same number of ports as NRS |  |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r13 SEQUENCE { |  |  | Guardband |
| rasterOffset-r13 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| spare | ‘000’B |  |  |
| } |  |  |  |
| standalone-r13 SEQUENCE { |  |  | Standalone |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| } |  |  |  |
| additionalTransmissionSIB1-r15 | FALSE |  |  |
| ab-Enabled-5GC-r16 | FALSE |  |  |
| partEARFCN-17 CHOICE { |  |  |  |
| spare | '00'B |  |  |
| earfcn-LSB | 2 least significant bits of the EARFCN for NTN bands where 100 kHz raster is used. |  | NTN |
| } |  |  |  |
| spare | ‘0000 00’B |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Inband-Same | In-band with same PCI test environment |
| Inband-Different | In-band with different PCI test environment |
| PRB30 | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 30 |
| PRB35 | For 10 MHz LTE Cell with In-Band NB-IoT in PRB 35 |
| PRB17 | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 17 |
| PRB22 | For 5 MHz LTE Cell with In-Band NB-IoT in PRB 22 |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| NTN | Non-Terrestrial Networks test environment |

###### -	MasterInformationBlock-TDD-NB

The MasterInformationBlock-TDD-NB includes the system information transmitted on BCH in TDD.

Table 8.1.4.3.2-1A: MasterInformationBlock-TDD-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-TDD-NB ::= SEQUENCE { |  |  |  |
| systemFrameNumber-MSB-r15 | A valid value as defined in TS 36.331 [17] |  |  |
| hyperSFN-LSB-r15 | A valid value as defined in TS 36.331 [17] |  |  |
| schedulingInfoSIB1-r15 | 2 | 16 repetitions |  |
| systemInfoValueTag-r15 | 0 |  |  |
| ab-Enabled-r15 | FALSE |  |  |
| operationModeInfo-r15 CHOICE { |  |  |  |
| inband-SamePCI-r15 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r15 | 23 | As specified in TS 36.213 [29] Clause 16.8 | PRB30 |
|  | 24 | As specified in TS 36.213 [29] Clause 16.8 | PRB35 |
|  | 7 | As specified in TS 36.213 [29] Clause 16.8 | PRB17 |
|  | 8 | As specified in TS 36.213 [29] Clause 16.8 | PRB22 |
| sib-InbandLocation-r15 | lower |  |  |
| } |  |  |  |
| inband-DifferentPCI-r15 SEQUENCE { |  |  | Inband_Different |
| eutra-NumCRS-Ports-r15 | same | same number of ports as NRS |  |
| rasterOffset-r15 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| sib-InbandLocation-r15 | lower |  |  |
| spare | ’00’B |  |  |
| } |  |  |  |
| guardband-r15 SEQUENCE { |  |  | Guardband |
| rasterOffset-r15 | 2.5*(2MDL+1) kHz | MDL See subclause 8.1.3.1 |  |
| sib-GuardbandInfo-r15 CHOICE { |  |  |  |
| sib-GuardbandAnchor-r15 SEQUENCE { |  |  | sib-GuardbandAnchor |
| spare | '0'B |  |  |
| } |  |  |  |
| sib-GuardbandGuardband-r15 SEQUENCE { |  |  | sib-GuardbandGuardband |
| spare | '0'B |  |  |
| } |  |  |  |
| sib-GuardbandInbandSamePCI-r15 SEQUENCE { |  |  | sib-GuardbandInbandSamePCI |
| spare | '0'B |  |  |
| } |  |  |  |
| sib-GuardbandinbandDiffPCI-r15 SEQUENCE { |  |  | sib-GuardbandinbandDiffPCI |
| sib-EUTRA-NumCRS-Ports-r15 | same |  |  |
| } |  |  |  |
| eutra-Bandwitdh-r15 | bw5or10 |  | EUTRA system bandwidth Value 5 or 10 MHz |
|  | Bw15or20 |  | EUTRA system bandwidth Value 15 or 20 MHz |
| } |  |  |  |
| standalone-r15 SEQUENCE { |  |  | Standalone |
| sib-InbandLocation-r15 | lower |  |  |
| spare | ‘0000 0’B |  |  |
| } |  |  |  |
| } |  |  |  |
| sib1-CarrierInfo-r15 | anchor |  |  |
| ab-Enabled-5GC-r16 | FALSE |  |  |
| spare | ‘0000 0000’B |  |  |
| } |  |  |  |

###### -	SystemInformation-NB

The SystemInformation-NB message is used to convey one or more System Information Blocks. All the SIBs included are transmitted with the same periodicity.

Table 8.1.4.3.2-2: SystemInformation-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformation-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| systemInformation-r13 SEQUENCE { |  |  |  |
| sib-TypeAndInfo-r13 SEQUENCE (SIZE (1..maxSIB)) OF CHOICE {} | See subclause 8.1.4.3.1 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType1-NB

SystemInformationBlockType1-NB contains information relevant when evaluating if a UE is allowed to access a cell and defines the scheduling of other system information.

Table 8.1.4.3.2-3: SystemInformationBlockType1-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType1-NB ::= SEQUENCE { |  |  |  |
| hyperSFN-MSB-r13 | A valid value as defined in TS 36.331 [17] |  |  |
| cellAccessRelatedInfo-r13 SEQUENCE { |  |  |  |
| plmn-IdentityList-r13 SEQUENCE (SIZE (1.. maxPLMN-r11)) OF PLMN-IdentityInfo-NB-r13 SEQUENCE { | 1 entry |  |  |
| PLMN-IdentityInfo-NB-r13[1] SEQUENCE { |  | entry 1 |  |
| plmn-Identity-r13 SEQUENCE { |  |  |  |
| mcc | See table 8.1.4.2-2 |  |  |
| mnc | See table 8.1.4.2-2 |  |  |
| } |  |  |  |
| cellReservedForOperatorUse-r13 | notReserved |  |  |
| attachWithoutPDN-Connectivity-r13 | true |  | ATTACH_WITHOUT_PDN |
|  | Not present |  | ATTACH_WITH_PDN |
| } |  |  |  |
| } |  |  |  |
| trackingAreaCode-r13 | See table 8.1.4.2-2 |  |  |
| cellIdentity-r13 | Cell ID for the simulated cell |  |  |
| cellBarred-r13 | notBarred |  |  |
|  | barred |  | NTN |
| intraFreqReselection-r13 | notAllowed |  |  |
| } |  |  |  |
| cellSelectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) | For RF/RRM test cases | RF |
|  | -34 (-34dB) | For RRM NB-IoT NTN enhanced coverage test cases | RRM_NTN_Enh |
|  | -106 dBm | For signalling test cases | Signalling |
| q-QualMin-r13 | -20 (-20dB) |  |  |
| } |  |  |  |
| p-Max-r13 | Not Present |  |  |
| freqBandIndicator-r13 | Operating band under test. |  |  |
| freqBandInfo-r13 | Not Present |  |  |
| multiBandInfoList-r13 | Not Present |  |  |
| downlinkBitmap-r13 | Not Present |  |  |
| eutraControlRegionSize-r13 | n2 |  | Inband-Same, Inband-Different |
|  | Not Present |  | Standalone, Guard |
| nrs-CRS-PowerOffset-r13 | dB0 |  | Inband-Same |
|  | Not Present |  | Inband-Different, Standalone, Guardband |
| schedulingInfoList-r13 | See subclause 8.1.4.3.1 |  |  |
| si-WindowLength-r13 | ms160 |  |  |
|  | ms640 | For RRM NB-IoT NTN enhanced coverage test cases | RRM_NTN_Enh |
| si-RadioFrameOffset-r13 | Not Present |  |  |
| systemInfoValueTagList-r13 | Not Present |  |  |
| lateNonCriticalExtension | Not Present |  |  |
| nonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1350 | TDD, SibType>22, NTN |
| cellSelectionInfo-v1350 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1430 |  |
| cellSelectionInfo-v1430 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1450 |  |
| nrs-CRS-PowerOffset-v1450 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1530 |  |
| tdd-Parameters-r15 | Not Present |  |  |
| tdd-Parameters-r15 SEQUENCE { |  |  | TDD |
| tdd-Config-r15 SEQUENCE { |  |  |  |
| subframeAssignment-r15 | sa1 |  |  |
| specialSubframePatterns-r15 | ssp6 |  |  |
| } |  |  |  |
| tdd-SI-CarrierInfo-r15 | anchor |  |  |
| tdd-SI-SubframesBitmap-r15 | Not Present |  |  |
| } |  |  |  |
| schedulingInfoList-v1530 | Not Present |  |  |
| schedulingInfoList-v1530 SEQUENCE (SIZE (1..maxSI-Message-NB-r13)) OF SchedulingInfo-NB-v1530 { | Actual number of entries depends on system information combination in clause 8.1.4.3.1.1 | Includes the same number of entries, and listed in the same order, as in schedulingInfoList-r13 | SibType>22 |
| SchedulingInfo-NB-v1530[x] | See clause 8.1.4.3.1.2 | entry x (x denotes the index of the entry) |  |
| } |  |  |  |
| nonCriticalExtension | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1610 | NTN |
| cellAccessRelatedInfo-5GC-r16 | Not Present |  |  |
| nonCriticalExtension SEQUENCE { |  | SystemInformationBlockType1-NB-v1700 |  |
| cellAccessRelatedInfo-NTN-r17 SEQUENCE { |  |  |  |
| cellBarred-NTN-r17 | notBarred |  |  |
| plmn-IdentityList-v1700 | Not present |  |  |
| } |  |  |  |
| nonCriticalExtension | Not Present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| ATTACH_WITH_PDN | If the UE shall perform ATTACH with PDN connectivity procedure. |
| ATTACH_WITHOUT_PDN | If the UE may perform ATTCH without PDN connectivity procedure. |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| RF | For RF/RRM test cases |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |
| Signalling | For Signalling test cases |
| NTN | Non-Terrestrial Networks test environment |
| SibType>22 | When the system information combination in use (see clause 8.1.4.3.1) includes at least one SIB with index exceeding 22 (i.e. 23 or more) |

##### 8.1.4.3.3	NB-IoT Common contents of system information blocks

###### -	SystemInformationBlockType2-NB

The IE SystemInformationBlockType2-NB contains radio resource configuration information that is common for all UEs.

Table 8.1.4.3.3-1: SystemInformationBlockType2-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| radioResourceConfigCommon-r13 SEQUENCE {} | RadioResourceConfigCommonSIB-NB-DEFAULT | See subclause 8.1.6 |  |
| ue-TimersAndConstants-r13 SEQUENCE { |  |  |  |
| t300-r13 | ms10000 |  |  |
| t301-r13 | ms10000 |  |  |
| t310-r13 | ms1000 |  |  |
| n310-r13 | n2 |  |  |
| t311-r13 | ms1000 |  |  |
| n311-r13 | n1 |  |  |
| } |  |  |  |
| freqInfo-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Not present | Default UL EARFCN applies | Standalone |
|  | See subclause 8.1.3.1 |  | Inband_Same, Inband_Different, Guardband |
| additionalSpectrumEmission-r13 | 1 (NS_01) | A-MPR doesn’t apply by default. See TS 36.101 table 6.2.4F -1. |  |
|  | 4 (NS_04) |  | NS_04 |
| } |  |  |  |
| timeAlignmentTimerCommon-r13 | infinity |  |  |
| multiBandInfoList-r13 | Not present |  |  |
| lateNonCriticalExtension SEQUENCE { | Not present |  | FDD |
| freqInfo-v1530 SEQUENCE { |  |  | TDD |
| tdd-UL-DL-AlignmentOffset-r15 | khz0 |  |  |
| } |  |  |  |
| } |  |  |  |
| cp-Reestablishment-r14 | Not present |  |  |
| servingCellMeasInfo-r14 | Not present |  |  |
| cqi-Reporting-r14 | Not present |  |  |
| enhancedPHR-r15 | Not present |  |  |
| freqInfo-v1530 | Not present |  |  |
| cp-EDT-r15 | Not present |  |  |
| up-EDT-r15 | Not present |  |  |
| earlySecurityReactivation-r16 | Not present |  |  |
| cp-EDT-5GC-r16 | Not present |  |  |
| up-EDT-5GC-r16 | Not present |  |  |
| cp-PUR-EPC-r16 | Not present |  |  |
| up-PUR-EPC-r16 | Not present |  |  |
| cp-PUR-5GC-r16 | Not present |  |  |
| up-PUR-5GC-r16 | Not present |  |  |
| rai-ActivationEnh-r16 | Not present |  |  |
| gnss-PositionFixDurationReporting-r18 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| Inband_Same | In-band with same PCI test environment |
| Inband_Different | In-band with different PCI test environment |
| Standalone | Standalone test environment |
| Guardband | Guard-band test environment |
| NS_04 | When using the NS_04 frequency table variants in subclause 8.1.3.1 |

###### -	SystemInformationBlockType3-NB

The IE SystemInformationBlockType3-NB contains cell re-selection information common for intra-frequency, inter-frequency as well as intra-frequency cell re-selection information other than neighbouring cell related.

Table 8.1.4.3.3-2: SystemInformationBlockType3-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType3-NB-r13 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon-r13 SEQUENCE { |  |  |  |
| q-Hyst-r13 | dB0 | To reduce interference between intra-frequency multiple cells |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE-r13 { |  |  |  |
| s-NonIntraSearch-r13 | 31 (62dB) |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo-r13 SEQUENCE { |  |  |  |
| q-RxLevMin-r13 | -70 (-140 dBm) | For RF/RRM test cases |  |
|  | -106 dBm | For signalling test cases |  |
| q-QualMin-r13 | Not present |  |  |
| p-Max-r13 | Not present |  |  |
| s-IntraSearchP-r13 | 31 (62dB) |  |  |
| t-Reselection-r13 | 0 |  |  |
| } |  |  |  |
| freqBandInfo-r13 | Not present |  |  |
| multiBandInfoList-r13 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| t-Service-r17 | Not present |  |  |
| t-Service-r17 | 2 |  | DiscontinousCoverage |
| satelliteAssistanceInfo-r18 | Not present |  |  |
| satelliteAssistanceInfo-r18 SEQUENCE (SIZE (1.. maxSat-r17)) OF SatelliteId-r18 { | 1 entry |  | DiscontinousCoverage |
| SatelliteId-r18[1] | 2 | entry 1 SatelliteId-r18 for the neighbour cell |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| DiscontinousCoverage | For NB-IoT NTN enhancement discontinous coverage test case |

###### -	SystemInformationBlockType4-NB

The IE SystemInformationBlockType4-NB contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters.

Table 8.1.4.3.3-3: SystemInformationBlockType4-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType4-NB-r13 ::= SEQUENCE { |  |  |  |
| intraFreqNeighCellList-r13 SEQUENCE (SIZE (1..maxCellIntra)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested. When Qoffset configuration is tested, see table 8.3.3.1.1-1 |  |
| intraFreqExcludedCellList SEQUENCE-r13 (SIZE (1..maxExcludedCell)) OF SEQUENCE {} | Not present | Not required unless exclude-listed cell list configuration is tested. When exclude-listed cell list configuration is tested, see table FFS. |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType5-NB

The IE SystemInformationBlockType5-NB contains information relevant only for inter-frequency cell re-selection i.e. information about other NB-IOT frequencies and inter-frequency neighbouring cells relevant for cell re-selection. The IE includes cell re-selection parameters common for a frequency.

Table 8.1.4.3.3-4: SystemInformationBlockType5-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType5-NB-r13 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList-r13 SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | The same number of entries as the configured inter-freq carriers. For Signalling test cases see clause 8.3.2.3 | n denotes the index of the entry |  |
| dl-CarrierFreq-r13[n] SEQUENCE { | Downlink EARFCN under test. For Signalling test cases see clause 8.3.2.3 |  |  |
| carrierFreq-r13[n] | Downlink EARFCN under test. For Signalling test cases see clause 8.3.2.3 |  |  |
| carrierFreqOffset-r13 [n] | Not present |  |  |
| carrierFreqOffset-r13 [n] | Offset of the NB-IOT channel number under test to the EARFCN under test |  | RRM-InterFreq |
| } |  |  |  |
| q-RxLevMin-r13[n] | -65 (-130 dBm) | For RF/RRM test cases |  |
|  | -106 dBm | For signalling test cases |  |
| q-QualMin-r13[n] | Not present |  |  |
| p-Max-r13[n] | Not present |  |  |
| q-OffsetFreq-r13[n] | dB0 | Qoffset doesn’t apply by default. |  |
| interFreqNeighCellList-r13[n] SEQUENCE (SIZE (1..maxCellInter)) OF SEQUENCE {} | Not present | Not required unless Qoffset configuration is tested |  |
| interFreqExcludedCellList-r13[n] SEQUENCE (SIZE (1..maxExcludedCell)) OF SEQUENCE {} | Not present | Not required unless exclude-listed cell list configuration is tested |  |
| multiBandInfoList-r13 | Not present |  |  |
| } |  |  |  |
| t-Reselection-r13 | s0 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM-InterFreq | Used for RRM NB-IOT inter-frequency mobility scenarios |

###### -	SystemInformationBlockType14-NB

The IE SystemInformationBlockType14-NB contains the AB parameters.

Table 8.1.4.3.3-5: SystemInformationBlockType14-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType14-NB-r13 ::= SEQUENCE { |  |  |  |
| ab-Param-r13 CHOICE { |  |  |  |
| ab-Common-r13 SEQUENCE { |  |  |  |
| ab-Category-r13 | a | AB applies to all UEs |  |
| ab-BarringBitmap-r13 | '1111111111'B | All UE classes 0-9 |  |
| ab-BarringExceptionData-r13 | Not present |  |  |
| ab-BarringForSpecialAC-r13 | '11111'B | All UE classes 11-15 |  |
| } |  |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType15-NB

The IE SystemInformationBlockType15-NB contains the MBMS Service Area Identities (SAI) of the current and/ or neighbouring carrier frequencies.

Table 8.1.4.3.3-5A: SystemInformationBlockType15-NB

FFS

###### -	SystemInformationBlockType16-NB

The IE SystemInformationBlockType16-NB contains information related to GPS time and Coordinated Universal Time (UTC). The UE may use the parameters provided in this system information block to obtain the UTC, the GPS and the local time.

Table 8.1.4.3.3-6: SystemInformationBlockType16-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType16-NB-r13 ::= SEQUENCE { |  |  |  |
| timeInfo-r11 SEQUENCE { |  |  |  |
| timeInfoUTC-r11 | 0 |  |  |
| dayLightSavingTime-r11 | Not present |  |  |
| leapSeconds-r1 | Not present |  |  |
| localTimeOffset-r11 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| timeReferenceInfo-r15 | Not present |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType20-NB

The IE SystemInformationBlockType20-NB contains the information required to acquire the control information associated with transmission of MBMS using SC-PTM.

Table 8.1.4.3.3-7: SystemInformationBlockType20-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType20-NB-r14 ::= SEQUENCE { |  |  |  |
| npdcch-SC-MCCH-Config-r14 SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-SC-MCCH-r14 | r8 |  |  |
| npdcch-StartSF-SC-MCCH-r14 | v1dot5 |  |  |
| npdcch-Offset-SC-MCCH-r14 | zero |  |  |
| } |  |  |  |
| sc-mcch-CarrierConfig-r14 CHOICE { |  |  |  |
| dl-CarrierIndex-r14 | 0 |  |  |
| } |  |  |  |
| sc-mcch-RepetitionPeriod-r14 | rf32 |  |  |
| sc-mcch-Offset-r14 | 1 |  |  |
| sc-mcch-ModificationPeriod-r14 | rf512 |  |  |
| sc-mcch-SchedulingInfo-r14 | Not present |  |  |
| lateNonCriticalExtension |  |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType22-NB

The IE SystemInformationBlockType22-NB contains radio resource configuration for paging and random access procedure on non-anchor carriers.

Table 8.1.4.3.3-8: SystemInformationBlockType22-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType22-NB-r14 ::= SEQUENCE { |  |  |  |
| dl-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF DL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| DL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| dl-CarrierConfig-r14 | DL-CarrierConfigCommon-NB-DEFAULT |  |  |
| pcch-Config-r14 | Not present |  |  |
| wus-Config-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| ul-ConfigList-r14 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-r14 SEQUENCE { |  |  |  |
| UL-ConfigCommon-NB-r14[1] SEQUENCE { |  |  |  |
| ul-CarrierFreq-r14 | See subclause 8.1.3.1 |  |  |
| nprach-ParametersList-r14 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| pagingWeightAnchor-r14 | Not present |  |  |
| nprach-ProbabilityAnchorList-r14 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType23-NB

The IE SystemInformationBlockType23-NB contains radio resource configuration for NPRACH resources using preamble format 2 on non-anchor carriers.

Table 8.1.4.3.3-9: SystemInformationBlockType23-NB

| Derivation Path: 36.331 clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType23-NB-r15 ::= SEQUENCE { |  |  |  |
| ul-ConfigList-v1530 SEQUENCE (SIZE (1.. maxNonAnchorCarriers-NB-r14)) OF UL-ConfigCommon-NB-v1530 SEQUENCE { | 1 entry |  |  |
| nprach-ParametersListFmt2-r15 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF NPRACH-ParametersFmt2-NB-r15 SEQUENCE { | 1 entry |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms8 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n36 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n24 |  |  |
| npdcch-CarrierIndex-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| nprach-ParametersListFmt2EDT-r15 | Not present |  |  |
| } |  |  |  |
| ul-ConfigListMixed-v1530 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType31-NB

The IE SystemInformationBlockType31-NB contains satellite assistance information for the serving cell. SystemInformationBlockType31-NB is only signalled in an NTN cell.

Table 8.1.4.3.3-10: SystemInformationBlockType31-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors | See cl. 6.3.5.1 |  | GSO |
|  | See cl. 6.3.5.2 |  | NGSO |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 8243100 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r17 | 0 |  |  |
| nta-CommonDriftVariation-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s240 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 550 |  | GSO |
|  | See cl. 6.3.5.2 |  | NGSO |
| k-Mac-r17 | Not present |  | GSO |
|  | See cl. 6.3.5.2 |  | NGSO |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| servingSatelliteInfo-v1820 | Not present |  |  |
| servingSatelliteInfo-v1820 SEQUENCE { |  |  | NTN_enh |
| satelliteId-r18 | Not present |  |  |
| referenceLocation-r18 CHOICE { |  |  |  |
| fixedReferenceLocation-r18 | '2273BA566355'H |  |  |
| } |  |  |  |
| distanceThresh-r18 | 500 |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |
| NTN_enh. | For NB-IoT NTN enhancement test case |

###### -	SystemInformationBlockType32-NB

The IE SystemInformationBlockType32-NB contains satellite assistance information for prediction of discontinuous coverage. SystemInformationBlockType32-NB is only signalled in a NTN cell.

Table 8.1.4.3.3-11: SystemInformationBlockType32-NB

| Derivation Path: TS 36.331 [17], clause 6.7.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType32-NB-r17 ::= SEQUENCE { |  |  |  |
| satelliteInfoList-r17 | FFS |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

###### -	SystemInformationBlockType33-NB

The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells.

Table 8.1.4.3.3-12: SystemInformationBlockType33-NB

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry |  |  |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| satelliteId-r18 | 2 |  |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors | See cl. 6.3.5.1 | Note 1 | GSO |
|  | See cl. 6.3.5.2 | Note 1 | NGSO |
| } |  |  |  |
| nta-CommonParameters SEQUENCE { |  |  |  |
| nta-Common-r18 | See cl. 6.3.5.1 | Note 1 | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r18 | 0 |  |  |
| nta-CommonDriftVariation-r18 | 0 |  |  |
| } |  |  |  |
| epochTime-r18 | Not present |  |  |
| k-Mac-r18 | Not present |  | GSO |
|  | See cl. 6.3.5.2 | Note 1 | NGSO |
| t-ServiceStartNeigh-r18 | Not present |  |  |
| } |  |  |  |
| neighValidityDuration-r18 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| NOTE 1: The NeigthSatelllite-Info-r18 should convey the parameters matching the parameters of a corresponding neighbour cell according to the cell type definitions in the cl 6.3.5.1 for GSO and cl 6.3.5.2 for NGSO. |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

### 8.1.5	NB-IoT Generic procedures

#### 8.1.5.0	General

This clause describes NB-IoT UE test states which can be used in the initial condition of many test cases defined in TS 36.521-1 [21], TS 36.523-1 [18] and TS 36.521-3 [34] or other procedures defined in this specification. This section also defines a set of procedures to bring the UE into these states.

#### 8.1.5.1	NB-IoT UE test states

NB-IoT UE test states defined in this section support Control Plane CIoT EPS Optimization (CP) and User Plane CIoT EPS Optimization (UP).

Figure 8.1.5.1-1: NB-IoT UE Test States for Basic Generic Procedures

Table 8.1.5.1-1: The NB-IoT UE states

|  |  | RRC | ECM | EMM | ESM | UE Test Mode |
| --- | --- | --- | --- | --- | --- | --- |
| State 1-NB | Switched OFF | ----- | ----- | ----- | ----- | ----- |
| State 2-NB | Attach, Connected Mode | RRC_CONNECTED 0, 1 or 2 data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 0 or 1 default EPS bearer context active and N (0 ≤ N ≤ 1) dedicated EPS bearer active as specified in the test cases. | Not active |
| State 2A-NB | Attach, Connected Mode, UE Test Mode Activated | RRC_CONNECTED 0, 1 or 2 data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 0 or 1 default EPS bearer context active and N (0 ≤ N ≤ 1) dedicated EPS bearer active as specified in the test cases. | Active |
| State 2B-NB | Attach, Connected Mode, UE Loopback Activated UL User data transmission pending | RRC_CONNECTED 0,1 or 2 data radio bearers configured as specified in the test cases | ECM-CONNECTED | EMM-REGISTERED | 0 or 1 default EPS bearer context active and N (0 ≤ N ≤ 1) dedicated EPS bearer active as specified in the test cases. | Active and Closed |
| State 3-NB | Registered, Idle Mode | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 0 or 1 default EPS bearer context active as specified in the test cases. | Not active |
| State 3A-NB | Registered, Idle Mode, UE Test Mode Activated | RRC_IDLE | ECM-IDLE | EMM-REGISTERED | 0 or 1 default EPS bearer context active as specified in the test cases. | Active |

NOTE 1: 	Refer to TS 24.301 [28] subclause 5.5.1.1 for more details on the ESM state.

NOTE 2: 	Refer to TS 36.509 [38] for details regarding UE test mode and UE Loopback.

#### 8.1.5.2	NB-IoT UE Attach, Connected mode (State 2-NB)

##### 8.1.5.2.0	General

UE registration depends on the specific configuration of a UE, which is expressed by PICS according to TS 36.523-2 [19], and specific test case conditions.

The following UE configurations and test case conditions are used in this procedure.

| Configuration | Condition | Explanation | Default values |
| --- | --- | --- | --- |
| S1_U_DataTransfer | UE_S1_U_CIoT_Optimisation AND TC_Require_S1_U_DataTransfer | User Plane CIoT Optimisation or S1_U is supported by UE (UE capability), pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer, and both the Network and test case require Attach with S1_U Data Transfer | UE_S1_U_DataTransfer =pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer TC_Require_S1_U_DataTransfer =FALSE |

##### 8.1.5.2.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2

User Equipment:

- 	The Test UICC shall be inserted. This shall contain a USIM application on UICC.

-	UE is in State 1-NB switched off.

##### 8.1.5.2.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5.2.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITHOUT_PDN for NOT S1_U_DataTransfer and ATTACH_WITH_PDN for S1_U_DataTransfer.

##### 8.1.5.2.3	Procedure

Table 8.1.5.2.3-1: NB-IoT UE connected mode procedure (state 1-NB to state 2-NB)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS transmits system information. | <-- | RRC: SYSTEM INFORMATION-NB (BCCH) |
| 1A | Switch-on the UE | - | - |
| 2 | UE transmits an RRCConnectionRequest-NB message. | --> | RRC: RRCConnectionRequest-NB |
| 3 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB |
| - | EXCEPTION: Steps 4a1 and 4b1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - |
| 4a1 | IF (NOT S1_U_DataTransfer AND px_Do AttachWithoutPDN THEN The UE transmits an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message. An ESM DUMMY MESSAGE is piggybacked in ATTACH REQUEST. | --> | RRC: RRCConnectionSetupComplete-NB NAS: ATTACH REQUEST NAS: ESM DUMMY MESSAGE |
| 4b1 | ELSE The UE transmits an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and to initiate the Attach procedure by including the ATTACH REQUEST message. A PDN CONNECTIVITY REQUEST message is piggybacked in ATTACH REQUEST. | --> | RRC: RRCConnectionSetupComplete-NB NAS: ATTACH REQUEST NAS: PDN CONNECTIVITY REQUEST |
| 5 | The SS transmits an AUTHENTICATION REQUEST message to initiate the EPS authentication and AKA procedure. | <-- | RRC: DLInformationTransfer-NB NAS: AUTHENTICATION REQUEST |
| 6 | The UE transmits an AUTHENTICATION RESPONSE message and establishes mutual authentication. | --> | RRC: ULInformationTransfer-NB NAS: AUTHENTICATION RESPONSE |
| 7 | The SS transmits a NAS SECURITY MODE COMMAND message to activate NAS security. | <-- | RRC: DLInformationTransfer-NB NAS: SECURITY MODE COMMAND |
| 8 | The UE transmits a NAS SECURITY MODE COMPLETE message and establishes the initial security configuration. | --> | RRC: ULInformationTransfer-NB NAS: SECURITY MODE COMPLETE |
| - | EXCEPTION: Steps 9a1 to 9a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred. | - | - |
| 9a1 | IF the UE sets the ESM information transfer flag in the PDN CONNECTIVITY REQUEST message sent in step 4b1 THEN the SS transmits an ESM INFORMATION REQUEST message to initiate exchange of protocol configuration options and/or APN. | <-- | RRC: DLInformationTransfer-NB NAS: ESM INFORMATION REQUEST |
| 9a2 | The UE transmits an ESM INFORMATION RESPONSE message to transfer protocol configuration options and/or APN. | --> | RRC: ULInformationTransfer-NB NAS: ESM INFORMATION RESPONSE |
| - | EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on UE configuration and test case condition; the "lower case letter" identifies a step sequence that take place if S1_U_DataTransfer condition is evaluated as true. | - | - |
| 10a1 | IF S1_U_DataTransfer THEN the SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 10a2 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 11 | The SS transmits a UECapabilityEnquiry-NB message to initiate the UE radio access capability transfer procedure. | <-- | RRC: UECapabilityEnquiry-NB |
| 12 | The UE transmits a UECapabilityInformation-NB message to transfer UE radio access capability. | --> | RRC: UECapabilityInformation-NB |
| - | EXCEPTION: Steps 13a1 to 13c1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured for S1_U_DataTransfer and if not then if it is configured to do Attach Without PDN or not. | - | - |
| 13a1 | IF S1_U_DataTransfer THEN the SS transmits a RRCConnectionReconfiguration-NB message to establish the default bearer with condition DRB(1) according to [8.1.8]. This message includes the ATTACH ACCEPT message and the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in the ATTACH ACCEPT | <-- | RRC: RRCConnectionReconfiguration-NBNAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| 13a2 | The UE transmits an RRCConnectionReconfigurationComplete-NB message to confirm the establishment of default bearer, | --> | RRC: RRCConnectionReconfigurationComplete-NB |
| 13b1 | ELSE IF px_DoAttachWithoutPDN THEN SS transmits an ATTACH ACCEPT message and an ESM DUMMY MESSAGE is piggybacked in the ATTACH ACCEPT. | <-- | RRC: DLInformationTransfer-NB NAS: ATTACH ACCEPT NAS: ESM DUMMY MESSAGE |
| 13c1 | ELSE SS transmits an ATTACH ACCEPT message and an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message is piggybacked in the ATTACH ACCEPT If PDN type "IP" was included in the PDN CONNECTIVITY REQUEST step 4b1 then the network shall include the PDN type and the PDN address information within the PDN address IE in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent to the UE | <-- | RRC: DLInformationTransfer-NB NAS: ATTACH ACCEPT NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |
| - | EXCEPTION: Steps 14a1 to 14b1 describe behaviour which depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - |
| 14a1 | IF px_DoAttachWithoutPDN THEN The UE transmit an ATTACH COMPLETE message and an ESM DUMMY MESSAGE is piggybacked in the ATTACH COMPLETE. | --> | RRC: ULInformationTransfer-NB NAS: ATTACH COMPLETE NAS: ESM DUMMY MESSAGE |
| - | EXCEPTION: IF not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in steps 13a1 or 13c1 THEN IF pc_User_Plane_CIoT_Optimisation or pc_S1_U_DataTransfer THEN In parallel to the event described in step 14b1 below the generic procedure for IP address allocation in the U-plane specified in TS 36.508 [18] subclause 4.5A.1 takes place performing IP address allocation in the U-plane. ELSE In parallel to the events described in step 14b1 below the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - |
| 14 b1 | ELSE UE transmits and ATTACH COMPLETE message and an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message is piggybacked in the ATTACH COMPLETE. | --> | RRC: ULInformationTransfer-NBNAS: ATTACH COMPLETE NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |

##### 8.1.5.2.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 8.1.7 with the exceptions below.

Table 8.1.5.2.4-1: RRCConnectionRequest-NB (step 2, Table 8.1.5.2.3-1)

| Derivation Path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| ue-Identity-r13 | Any allowed value |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.5.2A	NB-IoT UE Attach, Connected mode, UE Test Mode Activated (State 2A-NB)

##### 8.1.5.2A.0	General

UE registration depends on the specific configuration of a UE, which is expressed by PICS according to TS 36.523-2 [19], and specific test case conditions.

The UE configurations and test case conditions described in clause 8.1.5.2 are used in this procedure.

##### 8.1.5.2A.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2

User Equipment:

- 	The Test UICC shall be inserted. This shall contain a USIM application on UICC.

-	UE is in State 1-NB switched off.

##### 8.1.5.2A.2	Definition of system information messages

The same default system information messages are used as in clause 8.1.5.2.2.

NOTE:	Loopback mode A/B is applicable only IF S1_U_DataTransfer; loopback mode G/H/I is applicable only IF NOT S1_U_DataTransfer.

##### 8.1.5.2A.3	Procedure

Table 8.1.5.2A.3-1: NB-IoT UE Attach, connected mode and test mode activated procedure (state 2-NB to state 2A-NB)

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 0 | Switch-on the UE | - | - |
| 1-9a2 | Same procedure for steps 0 to 9a2 as specified in the procedure in clause 8.1.5.2.3 | - | - |
| 10 | The SS transmits an ACTIVATE TEST MODE message to activate test mode procedure. | <-- | RRC: DLInformationTransfer-NB TC: ACTIVATE TEST MODE |
| 11 | The UE transmits an ACTIVATE TEST MODE COMPLETE message. | --> | RRC: ULInformationTransfer-NB TC: ACTIVATE TEST MODE COMPLETE |
| 12a1-16b1 | Same procedure for steps 10a1 to 14b1 as specified in the procedure in clause 8.1.5.2.3 | - | - |

##### 8.1.5.2A.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 8.1.7 with the exceptions below.

Table 8.1.5.2A.4-1: ACTIVATE TEST MODE (step 10, Table 8.1.5.2A.3-1)

| Derivation Path: Table 4.7A-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UE test loop mode | '00000110'B |  | UE TEST LOOP MODE G (NOTE 1) |
|  | '00000111'B |  | UE TEST LOOP MODE H (NOTE 1) |
|  | '00001000'B |  | UE TEST LOOP MODE I (NOTE 1) |
|  | '00000000'B |  | UE TEST LOOP MODE A (NOTE 2) |
|  | '00000001'B |  | UE TEST LOOP MODE B (NOTE 2) |
|  | '00000101'B |  | UE TEST LOOP MODE F |
| NOTE 1: UE TEST LOOP MODE G/H/I is applicable only IF NOT S1_U_DataTransfer. NOTE 2: UE TEST LOOP MODE A/B is applicable only IF S1_U_DataTransfer. |  |  |  |

#### 8.1.5.2B	NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB)

##### 8.1.5.2B.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 8.1.4, unless otherwise specified in the test case.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2

User Equipment:

-	The NB-IoT UE shall be in NB-IoT UE Connected mode, NB-IoT UE Test Mode Activated (State 2A-NB).

##### 8.1.5.2B.2	Definition of system information messages

The same default system information messages are used as in clause 8.1.5.2.2.

NOTE:	Loopback mode A/B is applicable only IF S1_U_DataTransfer; loopback mode G/H/I is applicable only IF NOT S1_U_DataTransfer.

##### 8.1.5.2B.3	Procedure

Table 8.1.5.2B.3-1: NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated procedure (state 2A-NB to state 2B-NB)

| Step | Procedure | Direction | Message Sequence |
| --- | --- | --- | --- |
|  |  | UE - SS | Message |
| 1 | The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. | <-- | RRC: DLInformationTransfer-NB TC: CLOSE UE TEST LOOP |
| 2 | The UE transmits a CLOSE UE TEST LOOP COMPLETE message. | --> | RRC: ULInformationTransfer-NB TC: CLOSE UE TEST LOOP COMPLETE |

##### 8.1.5.2B.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 8.1.7 and 4.7 with the exceptions below for Test loop mode G/H/I/F (for test loop A/B the default message content according to clause 4.7A shall be applied).

Table 8.1.5.2B.4-1: CLOSE UE TEST LOOP (step 1, Table 8.1.5.2B.3-1) for test loop mode G/H/I/F

| Derivation path: Table 4.7A-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE test loop mode | '00000110'B | UE test loop mode G setup | TL_MODE_G |
|  | '00000111'B | UE test loop mode H setup | TL_MODE_H |
|  | '00001000'B | UE test loop mode I setup | TL_MODE_I |
|  | '00000101'B | UE test loop mode F setup | TL_MODE_F |
| Operation mode and repetitions |  |  | TL_MODE_G, TL_MODE_H |
| M0 | 0 |  | return_via_EMM_SMC |
|  | 1 |  | NBIOT_return_via_RLC |
| R6..R0 | '0000001'B | 1 The received DL message in uplink shall be looped back 1 time (once) |  |
| Uplink data delay | '00000000'B | T_delay_modeG timer=0 sec |  |
| SC-MTCH ID | ‘0100’H | same value as g-RNTI indicated in the default SCPTMConfiguration-NB message | TL_MODE_F |

| Condition | Explanation |
| --- | --- |
| TL_MODE_G | Test loop mode G is applied |
| TL_MODE_H | Test loop mode H is applied |
| TL_MODE_I | Test loop mode I is applied |
| TL_MODE_F | Test loop mode F is applied |
| return_via_EMM_SMC | data is returned in uplink at the EMM entity for UE test loop mode G or the SMC SAP for UE test loop mode H |
| NBIOT_return_via_RLC | data is returned in uplink at the RLC AM-SAP of SRB1bis for NB-IoT UE used in MAC and RLC test cases |

#### 8.1.5.3	NB-IoT UE Registered, Idle Mode (State 3-NB)

##### 8.1.5.3.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 8.1.4, unless otherwise specified in the test case.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2

User Equipment:

-	The NB-IoT UE shall be in NB-IoT UE Attach, Connected mode (State 2-NB).

##### 8.1.5.3.2	Definition of system information messages

The same default system information messages are used as in clause 8.1.5.2.2.

##### 8.1.5.3.3	Procedure

Table 8.1.5.3.3-1: NB-IoT Registered Idle Mode procedure (state 2-NB to state 3-NB)

| Step | Procedure | Direction | Message Sequence |
| --- | --- | --- | --- |
|  |  | UE - SS | Message |
| 1 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease-NB |

##### 8.1.5.3.4	Specific message contents

None.

#### 8.1.5.3A	NB-IoT UE Registered, Idle Mode, UE Test Mode Activated (State 3A-NB)

##### 8.1.5.3A.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the basic single cell environment, as defined in subclause 8.1.4, unless otherwise specified in the test case.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2

User Equipment:

-	The NB-IoT UE shall be in NB-IoT UE Attach, Connected mode (State 2A-NB).

##### 8.1.5.3A.2	Definition of system information messages

The same default system information messages are used as in clause 8.1.5.2.2.

##### 8.1.5.3A.3	Procedure

Table 8.1.5.3A.3-1: NB-IoT Registered, Idle Mode, NB-IoT UE Test Mode Activated (state 2A-NB to state 3A-NB)

| Step | Procedure | Direction | Message Sequence |
| --- | --- | --- | --- |
|  |  | UE - SS | Message |
| 1 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease-NB |

##### 8.1.5.3A.4	Specific message contents

None.

#### 8.1.5.4	Void

### 8.1.5A	Other generic procedures

#### 8.1.5A.1	Procedure for IP address allocation in the CP CIoT

The purpose of this procedure is to allow the successful completion of IP address allocation if it is initiated by the UE therefore the result from the execution of the Procedure for IP address allocation in the CP CIoT shall not lead to assignment of a verdict.

Depending on the UE configuration there may be unpredictable delay in the start of the procedure. A guarding time of 1.2 sec (for satellite access (NTN) 2.48 sec) is suggested within which the procedure is expected to start. If the timer expires then the test procedure, from which the Procedure for IP address allocation in the CP CIoT is called, shall advance to the next specified step.

The procedure only applies for IP data. The procedure is not applicable for Non IP type data and SMS.

Table 8.1.5A.1-1: Procedure for IP address allocation in the CP CIoT

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1 to 2 below describe behaviour that depends on the contents of the latest PDN CONNECTIVITY REQUEST message sent by the UE prior to this procedure. | - | - |
| 1 | If the "PDN type" in the latest PDN CONNECTIVITY REQUEST message prior to this procedure was 'IPv6' or 'IPv4v6' then stateless address auto configuration may occurs over SRB1bis, UE may send an ICMPv6 Router Solicitation | --> | RRC: ULInformationTransfer-NB NAS: ESM DATA TRANSPORT |
| 2 | The SS sends an ICMPv6 Router Advertisement containing the IPv6 prefix. | <-- | RRC: DLInformationTransfer-NB NAS: ESM DATA TRANSPORT |

#### 8.1.5A.2	Test procedure to check UE response to Paging for Control Plane CIoT MT access

##### 8.1.5A.2.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-IDLE.

##### 8.1.5A.2.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.2.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, Condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.2.3	Procedure

Table 8.1.5A.2.3-1: UE response to Paging for Control Plane CIoT MT access

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging-NB | - | - |
| 2 | Check: Does the UE transmit an RRCConnectionRequest-NB message "mt-access"? | --> | RRC: RRCConnectionRequest-NB | - | P |
| 3 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| - | EXCEPTION: Steps 4a1a1 to 4a1b3b1 describe behaviour which depends on UE implementation; the "lower case letter" identifies a step sequence which takes place depending on whether the UE is configured or not to establish PDN upon establishing RRC connection and whether UE has pending data to transmit. | - | - | - | - |
| 4a1a1 | IF UE is configured to establish PDN upon establishing RRC connection and UE has pending data to transmit THEN Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message containing a ESM DATA TRANSPORT message in the ESM message container IE? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | P |
| 4a1b1 | ELSE Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message not containing user data (there is not yet a PDN) not containing a PDN CONNECTIVITY REQUEST message? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | - | P |
| 4a1b2 | SS starts wait timer 2 sec. Note: The Service request procedure success is guarded in the UE by timer T3417 = 5 sec after which expiry without any DL action the UE will consider the procedure as failed. | - | - | - | - |
| - | EXCEPTION: Steps 4a1b3a1 to 4a1b3a4 describe behaviour which depends on UE implementation; the "lower case letter" identifies a step sequence which takes place depending on whether the UE is configured or not to establish PDN in RRC-CONNECTED after it sends the CONTROL PLANE SERVICE REQUEST message. | - | - | - | - |
| 4a1b3a1 | The UE transmits a PDN CONNECTIVITY REQUEST message. | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | - |
| 4a1b3a2 | Stop timer wait timer 2 sec. | - | - | - | - |
| 4a1b3a3 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 4a1b3a3 TNEN In parallel to the events described in steps 4a1b3a4 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - |
| 4a1b3a4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer-NB NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 4a1b3b1 | Timer 2 sec expires | - | - | - | - |
| 5 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |
| 6 | For NTN access, SS waits for timer 2* TNTN,offset expiry (Note 1) | - | - | - | - |
| Note 1: TNTN,offset is specified in TS 36.523-3 [20] clause 7A.14.2.3). |  |  |  |  |  |

##### 8.1.5A.2.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.2.4-1: Message RRCConnectionRequest-NB (step 2, Table 8.1.5A.2.3-1)

| Derivation path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| establishmentCause-r13 | mt-Access |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.2.4-2: Message RRCConnectionSetupComplete-NB (steps 4a1a1, 4a1b1, Table 8.1.5A.2.3-1)

| Derivation path: Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | the same value as included in the RRCConnectionSetup-NB message received from SS |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| attachWithoutPDN-Connectivity-r13 | True |  | px_DoAttachWithoutPDN |
|  | Not Present |  | NOT px_DoAttachWithoutPDN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.2.4-3: PDN CONNECTIVITY REQUEST (steps 4a1a1, 4a1b3a1, Table 8.1.5A.2.3-1)

| Derivation path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not Present or Any allowed value |  |  |
| Protocol configuration options | Not Present |  |  |
| Header compression configuration | Any allowed value |  | pc_HCCPCIoT AND pc_IP_PDN |
| Extended protocol configuration options | Not Present or Any allowed value |  |  |

Table 8.1.5A.2.4-4: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 4a1b3a3, Table 8.1.5A.2.3-1)

| Derivation path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Header compression configuration | 0000H | No Compression profile NOTE: For the purposes of CIoT (NAS) testing regardless of the Compression files supported by the UE and indicated in the PDN CONNECTIVITY REQUEST, the SS does not agree header compression to be applied. | pc_HCCPCIoT AND pc_IP_PDN |
| Control plane only indication | '0001'B | PDN connection can be used for control plane CIoT EPS optimization only |  |
| Extended protocol configuration options |  | The content of the IE below uses the same fields and Conditions (and their meaning) as those defined in TS 36.508 [18] for the IE 'Protocol configuration options' |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control | '11111111 11111111'B | Unrestricted Max number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send via a PDN connection per 6 minute interval |  |

Table 8.1.5A.2.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 4a1b3a4, Table 8.1.5A.2.3-1)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Extended protocol configuration options | Not present or any allowed value |  |  |

#### 8.1.5A.3	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer non-SMS transport

##### 8.1.5A.3.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-IDLE.

##### 8.1.5A.3.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.3.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.3.3	Procedure

Table 8.1.5A.3.3-1: Control Plane CIoT MO user data transfer non-SMS transport

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | Check: Does the UE transmit an RRCConnectionRequest-NB message? | --> | RRC: RRCConnectionRequest-NB | - | P |
| 2 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| - | EXCEPTION: Steps 3a1 to 3b1 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - | - | - |
| 3a1a1 | Void. | - | - | - | - |
| 3a1b1 | IF px_DoAttachWithoutPDN THEN Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | - | P |
| 3a1b2 | Check: Does the UE transmit a PDN CONNECTIVITY REQUEST message? | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | P |
| 3a2 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 3a2 TNEN In parallel to the events described in steps 3a3 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - |
| 3a3 | Check: Does the UE transmit an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer-NB NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | P |
| 3a4 | Check: Does the UE send an ESM DATA TRANSPORT message containing user data? | --> | RRC: ULInformationTransfer-NB NAS: ESM DATA TRANSPORT | - | P |
| 3b1 | ELSE (NOT px_DoAttachWithoutPDN) Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message containing user data encapsulated in an ESM DATA TRANSPORT message included in the ESM message container IE? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | P |
| 4 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |
| 5 | For NTN access, SS waits for timer 2* TNTN,offset expiry (Note 1) | - | - | - | - |
| Note 1: TNTN,offset in specified in TS 36.523-3 [20] clause 7A.14.2.3). |  |  |  |  |  |

##### 8.1.5A.3.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.3.4-1: Message RRCConnectionRequest-NB (step 1, Table 8.1.5A.3.3-1)

| Derivation path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| establishmentCause-r13 | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3.4-2: Message RRCConnectionSetupComplete-NB (steps 3a1a1, 3a1b1, Table 8.1.5A.3.3-1)

| Derivation path: Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | the same value as included in the RRCConnectionSetup-NB message received from SS |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| attachWithoutPDN-Connectivity-r13 | True |  | px_DoAttachWithoutPDN |
|  | Not Present |  | NOT px_DoAttachWithoutPDN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3.4-3: Message CONTROL PLANE SERVICE REQUEST (step 3b1, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.2-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Data service type |  |  |  |
| ..Data service type value | mobile originating request |  |  |
| ESM message container | ESM DATA TRANSPORT message, User data container not checked | If the ESM DATA TRANSPORT message 'User data container' IE needs to be checked and then its content shall be specified in TC |  |

Table 8.1.5A.3.4-4: PDN CONNECTIVITY REQUEST (steps 3a1a1, 3a1b2, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not Present or Any allowed value |  |  |
| Protocol configuration options | Not Present |  |  |
| Header compression configuration | Any allowed value |  | pc_HCCPCIoT AND pc_IP_PDN |
| Extended protocol configuration options | Not Present or Any allowed value |  |  |

Table 8.1.5A.3.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (step 3a2, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Header compression configuration | 0000H | No Compression profile NOTE: For the purposes of CIoT (NAS) testing regardless of the Compression files supported by the UE and indicated in the PDN CONNECTIVITY REQUEST, the SS does not agree header compression to be applied. | pc_HCCPCIoT AND pc_IP_PDN |
| Control plane only indication | '0001'B | PDN connection can be used for control plane CIoT EPS optimization only |  |
| Extended protocol configuration options |  | The content of the IE below uses the same fields and Conditions (and their meaning) as those defined in TS 36.508 [18] for the IE 'Protocol configuration options' |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control | '11111111 11111111'B | Unrestricted Max number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send via a PDN connection per 6 minute interval |  |

Table 8.1.5A.3.4-6: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (step 3a3, Table 8.1.5A.3.3-1)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Extended protocol configuration options | Not present or any allowed value |  |  |

#### 8.1.5A.3A	Test procedure to check UE initiation of Control Plane CIoT MO user data transfer SMS transport

##### 8.1.5A.3A.1	Initial conditions

System Simulator:

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-IDLE.

##### 8.1.5A.3A.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.3A.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.3A.3	Procedure

Table 8.1.5A.3A.3-1: Control Plane CIoT MO user data transfer SMS transport

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | Check: Does the UE transmit an RRCConnectionRequest-NB message? | --> | RRC: RRCConnectionRequest-NB | - | P |
| 2 | SS transmits an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 3a1 | Void. | - | - | - | - |
| 3a2 | Void. | - | - | - | - |
| 3a3 | Void. | - | - | - | - |
| 3b1 | Check: Does the UE transmit an RRCConnectionSetupComplete-NB message and a CONTROL PLANE SERVICE REQUEST message , data service type="mobile originating request", integrity protected and partially ciphered and including the SMS in the NAS message container IE, and, not containing a PDN CONNECTIVITY REQUEST message? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST | - | P |
| 3b2 | SS starts wait timer 2 sec. Note: The Service request procedure success is guarded in the UE by timer T3417 = 5 sec after which expiry without any DL action the UE will consider the procedure as failed. | - | - | - | - |
| - | EXCEPTION: Steps 3b3a1 to 3b3b1 describe behaviour which depends on UE implementation; the "lower case letter" identifies a step sequence which takes place depending on whether the UE is configured or not to establish PDN in RRC-CONNECTED after it sends the CONTROL PLANE SERVICE REQUEST message. | - | - | - | - |
| 3b3a1 | IF px_nonSMSTransport_CP_CIoT AND px_DoAttachWithoutPDN THEN The UE transmits a PDN CONNECTIVITY REQUEST message. | --> | RRC: ULInformationTransfer-NB NAS: PDN CONNECTIVITY REQUEST | - | - |
| 3b3a2 | Stop timer wait timer 2 sec. | - | - | - | - |
| 3b3a3 | The SS transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | - | - |
| - | EXCEPTION: IF pc_IP_PDN AND not all IP address information was allocated in the ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST message sent in step 3b3a3 TNEN In parallel to the events described in steps 3b3a4 the Generic 'Procedure for IP address allocation in the CP CIoT' described in TS 36.508 [18], clause 8.1.5A.1 takes place. | - | - | - | - |
| 3b3a4 | The UE transmits an ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT message. | --> | RRC: ULInformationTransfer-NB NAS:ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT | - | - |
| 3b3b1 | Timer 2 sec expires | - | - | - | - |
| 4 | The SS transmits a SERVICE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: SERVICE ACCEPT | - | - |
| 5 | For NTN access, SS waits for timer 2* TNTN,offset expiry (Note 1) | - | - | - | - |
| Note 1: TNTN,offset is specified in TS 36.523-3 [20] clause 7A.14.2.3). |  |  |  |  |  |

##### 8.1.5A.3A.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.3A.4-1: Message RRCConnectionRequest-NB (step 1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 8.1.6.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| establishmentCause-r13 | mo-data |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3A.4-2: Message RRCConnectionSetupComplete-NB (steps 3a1, 3b1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 8.1.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | the same value as included in the RRCConnectionSetup-NB message received from SS |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| attachWithoutPDN-Connectivity-r13 | True |  | px_DoAttachWithoutPDN |
|  | Not Present |  | NOT px_DoAttachWithoutPDN |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.5A.3A.4-3: Message CONTROL PLANE SERVICE REQUEST (steps 3a1, 3b1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.2-28. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Data service type |  |  |  |
| ..Data service type value | mobile originating request |  |  |
| NAS message container | SMS - content not checked | If the SMS needs to be checked and then its content shall be specified in TC |  |

Table 8.1.5A.3A.4-4: PDN CONNECTIVITY REQUEST (steps 3a1, 3b3a1, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.3-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Access point name | Not Present or Any allowed value |  |  |
| Protocol configuration options | Not Present |  |  |
| Header compression configuration | Any allowed value |  | pc_HCCPCIoT AND pc_IP_PDN |
| Extended protocol configuration options | Not Present or Any allowed value |  |  |

Table 8.1.5A.3A.4-5: Message ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST (steps 3a2, 3b3a3, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.3-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Header compression configuration | 0000H | No Compression profile NOTE: For the purposes of CIoT (NAS) testing regardless of the Compression files supported by the UE and indicated in the PDN CONNECTIVITY REQUEST, the SS does not agree header compression to be applied. | pc_HCCPCIoT AND pc_IP_PDN |
| Control plane only indication | '0001'B | PDN connection can be used for control plane CIoT EPS optimization only |  |
| Extended protocol configuration options |  | The content of the IE below uses the same fields and Conditions (and their meaning) as those defined in TS 36.508 [18] for the IE 'Protocol configuration options' |  |
| Container ID n | ‘0003’H | n assigned to next available number | DNS IPv6 |
| Length of container ID n contents |  | Length value determined by the TTCN implementation |  |
| Container ID n contents | IPv6 address | DNS IPv6 Address |  |
| Container ID n+1 | ‘000D’H | n assigned to next available number | DNS IPv4 |
| Length of container ID n+1 contents |  | Length value determined by the TTCN implementation |  |
| Container ID n+1 contents | IPv4 address | DNS IPv4 Address |  |
| Serving PLMN rate control | '11111111 11111111'B | Unrestricted Max number of uplink ESM DATA TRANSPORT messages including User data container IEs the UE is allowed to send via a PDN connection per 6 minute interval |  |

Table 8.1.5A.3A.4-6: Message ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT (steps 3a3, 3b3a4, Table 8.1.5A.3A.3-1)

| Derivation path: Table 4.7.3-4 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Protocol configuration options | Not present |  |  |
| Extended protocol configuration options | Not present or any allowed value |  |  |

#### 8.1.5A.4	Test procedure to check release of PDN connectivity before leaving RRC-CONNECTED for attach without PDN

##### 8.1.5A.4.1	Initial conditions

-	1 NB-IoT cell, default parameters.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2.

User Equipment:

- 	The UE is in RRC-CONNECTED.

-	px_DoAttachWithoutPDN=TRUE

##### 8.1.5A.4.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.4.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITHOUT_PDN.

##### 8.1.5A.4.3	Procedure

Table 8.1.5A.4.3-1: Release of PDN connectivity before leaving RRC-CONNECTED for attach without PDN

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| - | EXCEPTION: Steps 1a1 and 1b2 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence which may take place depending on whether the UE is configured to do Attach Without PDN or not. | - | - | - | - |
| 1 | SS starts timer 60 sec | - | - | - | - |
| - | EXCEPTION: Steps 2a1-2b3 describe behaviour that depends on UE implementation; the "lower case letter" identifies a step sequence which may take place depending on whether the UE will initiate PDN deactivation. | - | - | - | - |
| 2a1 | Check: Does the UE transmit a PDN DISCONNECT REQUEST message? | --> | RRC: ULInformationTransfer NAS: PDN DISCONNECT REQUEST | - | P |
| 2a2 | The SS transmits a DEACTIVATE EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 2a3 | Check: Does the UE transmit a DEACTIVATE EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | P |
| 2a4 | SS stops 60 sec timer. | - | - | - | - |
| 2b1 | 60 sec timer expires. | - | - | - | - |
| 2b2 | SS releases the PDN connectivity by transmitting a DEACTIVATE EPS BEARER CONTEXT REQUEST message. | <-- | RRC: DLInformationTransfer-NB NAS: DEACTIVATE EPS BEARER CONTEXT REQUEST | - | - |
| 2b3 | Check: Does the UE transmit a DEACTIVATE EPS BEARER CONTEXT ACCEPT message? | --> | RRC: ULInformationTransfer-NB NAS: DEACTIVATE EPS BEARER CONTEXT ACCEPT | - | P |

##### 8.1.5A.4.4	Specific message contents

All specific message contents shall be referred to clause 8.1.6 and 4.7 with the exceptions below.

Table 8.1.5A.4.4-1: DEACTIVATE EPS BEARER CONTEXT REQUEST (step 2a2, Table 8.1.5A.4.3-1)

| Derivation Path: Table 4.7.3-12, Condition UE-INITIATED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ESM cause | 00100100 | regular deactivation |  |

Table 8.1.5A.4.4-2: DEACTIVATE EPS BEARER CONTEXT REQUEST (step 2b2, Table 8.1.5A.4.3-1)

| Derivation Path: Table 4.7.3-12, Condition NETWORK-INITIATED |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ESM cause | 00100100 | regular deactivation |  |

#### 8.1.5A.5	Test procedure to check that NB-IoT UE is camped on a new NB-IOT cell

This procedure verifies whether the UE has camped on a new NB-IoT cell with different TAI.

The procedure is defined in table 8.1.5A.5-1.

Table 8.1.5A.5-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCConnectionRequest-NB message on the cell specified in the test case. | --> | RRC: RRCConnectionRequest-NB | - | - |
| 2 | SS transmit an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 3 | The UE transmits an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionSetupComplete-NB NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | Check: Does the UE send a TRACKING AREA UPDATE COMPLETE on the cell specified in the test case? | --> | RRC: ULInformationTransfer-NB NAS: TRACKING AREA UPDATE COMPLETE | - | P |
| 6 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease-NB | - | - |

#### 8.1.5A.6	Test procedure to check that NB-IoT UE resume RRC connection on a new NB-IOT cell

This procedure verifies whether the UE resumes RRC connection on a new NB-IoT cell with different TAI.

The procedure is defined in table 8.1.5A.6-1.

Table 8.1.5A.6-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits an RRCConnectionResumeRequest-NB message on the cell specified in the test case. | --> | RRC: RRCConnectionResumeRequest-NB | - | - |
| 2 | SS transmits an RRCConnectionResume-NB message. | <-- | RRC: RRCConnectionResume-NB | - | - |
| 3 | The UE transmits an RRCConnectionResumeComplete-NB message to confirm the successful completion of the connection establishment and a TRACKING AREA UPDATE REQUEST message is sent to update the registration of the actual tracking area. | --> | RRC: RRCConnectionResumeComplete-NB NAS: TRACKING AREA UPDATE REQUEST | - | - |
| 4 | SS responds with TRACKING AREA UPDATE ACCEPT message. | <-- | RRC: DLInformationTransfer-NB NAS: TRACKING AREA UPDATE ACCEPT | - | - |
| 5 | Check: Does the UE send a TRACKING AREA UPDATE COMPLETE on the cell specified in the test case? | --> | RRC: ULInformationTransfer-NB NAS: TRACKING AREA UPDATE COMPLETE | - | P |
| 6 | The SS transmits an RRCConnectionRelease-NB message to release RRC connection and move to RRC_IDLE. | <-- | RRC: RRCConnectionRelease-NB | - | - |

#### 8.1.5A.7

#### 8.1.5A.8	Test procedure to check RRC_CONNECTED state for NB-IoT

Table 8.1.5A.8-1: Test procedure sequence

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message/PDU/SDU |  |  |
| 1 | The SS sends UECapabilityEnquiry-NB message to the UE. | <-- | UECapabilityEnquiry-NB | - | - |
| 2 | Check: Does the UE send a UECapabilityInformation-NB message? | --> | UECapabilityInformation-NB | - | P |

#### 8.1.5A.9	Test Procedure to establish radio bearers in User Plane

##### 8.1.5A.9.1	Initial conditions

System Simulator:

-	Parameters are set to the default parameters for the NB-IoT single cell environment, as defined in subclause 8.1.4.1.1, unless otherwise specified in the test case.

-	The procedure shall be performed under ideal radio conditions as defined in clause 8.2

User Equipment:

-	The NB-IoT UE shall be in NB-IoT Registered, Idle mode (State 3-NB) for condition S1_U_DataTransfer.

##### 8.1.5A.9.2	Definition of system information messages

The default system information messages are used with the exception(s) below.

Table 8.1.5A.9.2-1: SystemInformationBlockType1-NB

Derivation Path: Table 8.1.4.3.2-3, condition ATTACH_WITH_PDN.

##### 8.1.5A.9.3	Procedure

The establishment of generic radio bearer connection is assumed to always be mobile terminated.

Table 8.1.5A.9.3-1: Generic Radio Bearer establishment procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | RRC: Paging-NB |
| 2 | UE transmits an RRCConnectionRequest-NB message. | --> | RRC: RRCConnectionRequest-NB |
| 3 | SS transmit an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB |
| 4 | The UE transmits an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and to initiate the session management procedure by including the CONTROL PLANE SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST |
| 5 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 6 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 7 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration-NB message is using condition DRB(1). | <-- | RRC: RRCConnectionReconfiguration-NB |
| 8 | The UE transmits an RRCConnectionReconfigurationComplete-NB message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete-NB |

##### 8.1.5A.9.4	Specific message contents

All specific message contents shall be referred to clause 4.6, 4.7 and 8.1.6.

### 8.1.5B	Common test environment for Vertical UEs

#### 8.1.5B.1	NB-IoT NTN

This clause defines the common test environment which applies to all test cases executed for NB-IoT NTN UEs, unless otherwise specified.

-	Test configures following IEs with condition NTN of MasterInformationBlock-NB in Table 8.1.4.3.2-1, SystemInformationBlockType1-NB in Table 8.1.4.3.2-3, SystemInformationBlockType31-NB in Table 8.1.4.3.3-10 and RadioResourceConfigCommonSIB-NB-DEFAULT in Table 8.1.6.3-9.

-	Test uses system information combinations 8 to 13.

### 8.1.6	NB-IoT Default RRC message and information elements contents

This clause contains the default values of common RRC messages and information elements, which unless indicated otherwise in specific clauses of TS 36.521-1 [21], TS 36.521-3 [34], TS 36.523-1 [18] and other clauses in this specification. All the messages and information elements are listed in alphabetical order.

#### 8.1.6.1	NB-IoT Contents of RRC messages

#### –	DLInformationTransfer-NB

Table 8.1.6.1-1: DLInformationTransfer-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DLInformationTransfer-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| dlInformationTransfer-r13 SEQUENCE { |  |  |  |
| dedicatedInfoNAS-r13 | Set according to specific message content |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	Paging-NB

Table 8.1.6.1-2: Paging-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging-NB ::= SEQUENCE { |  |  |  |
| pagingRecordList-r13 SEQUENCE (SIZE (1..maxPageRec)) OF SEQUENCE { | 1 entry |  |  |
| ue-Identity-r13[1] CHOICE { |  |  |  |
| s-TMSI | Set to the value of the S-TMSI of the UE |  |  |
| } |  |  |  |
| } |  |  |  |
| systemInfoModification-r13 | Not present |  |  |
| systemInfoModification-eDRX-r13 | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### –	PURConfigurationRequest-NB

Table 8.1.6.1-2a: PURConfigurationRequest-NB

| Derivation path: 36.331 sub-clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PURConfigurationRequest-NB-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| purConfigurationRequest-r16 SEQUENCE { |  |  |  |
| pur-ConfigRequest-r16 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionReconfiguration-NB

Table 8.1.6.1-3: RRCConnectionReconfiguration-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfiguration-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReconfiguration-r13 SEQUENCE { |  |  |  |
| dedicatedInfoNASList-r13 | Not present |  |  |
|  | Set according to specific message content |  | NB-DRB(n) |
| radioResourceConfigDedicated-r13 | Not present |  |  |
|  | RadioResourceConfigDedicated-NB-DRB(n) |  | NB-DRB(n) |
| fullConfig-r13 | Not present |  |  |
|  | Present |  | Reestab |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NB-DRB | Establishment of additional n x AM DRB:s (maximum number of AM DRBs: 2) |
| Reestab | This field is optionally present, need ON upon the first reconfiguration after RRC connection re-establishment; otherwise the field is not present |

#### –	RRCConnectionReconfigurationComplete-NB

Table 8.1.6.1-4: RRCConnectionReconfigurationComplete-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReconfigurationComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReconfigurationComplete-r13 SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionReestablishment-NB

Table 8.1.6.1-5: RRCConnectionReestablishment-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishment-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| rrcConnectionReestablishment-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 | RadioResourceConfigDedicated-NB-SRB |  |  |
| nextHopChainingCount | 0 |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionReestablishmentComplete-NB

Table 8.1.6.1-6: RRCConnectionReestablishmentComplete-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentComplete-r13 SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionReestablishmentRequest-NB

Table 8.1.6.1-7: RRCConnectionReestablishmentRequest-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReestablishmentRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionReestablishmentRequest-r13 SEQUENCE { |  |  |  |
| ue-Identity-r13 | Set according to specific message content |  |  |
| reestablishmentCause-r13 | Set according to specific message content |  |  |
| spare | Present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionReject-NB

Table 8.1.6.1-8: RRCConnectionReject-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionReject-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionReject-r13 SEQUENCE { |  |  |  |
| extendedWaitTime | 3(seconds) |  |  |
| rrc-SuspendIndication-r13 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionRelease-NB

Table 8.1.6.1-9: RRCConnectionRelease-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| releaseCause-r13 | other |  |  |
| resumeIdentity-r13 | Not present |  |  |
| extendedWaitTime-r13 | Not present |  |  |
| redirectedCarrierInfo | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionRequest-NB

Table 8.1.6.1-10: RRCConnectionRequest-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionRequest-r13 SEQUENCE { |  |  |  |
| ue-Identity-r13 CHOICE { |  |  |  |
| s-TMSI | Any allowed value |  |  |
| } |  |  |  |
| establishmentCause-r13 | Present but contents not checked |  |  |
| multiToneSupport-r13 | Not checked |  |  |
| multiCarrierSupport-r13 | Not checked |  |  |
| spare | Present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionResume-NB

Table 8.1.6.1-11: RRCConnectionResume-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResume-NB ::= SEQUENCE { |  | (Note 1) |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionResume-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 | RadioResourceConfigDedicated-NB-DRB(n) |  |  |
| nextHopChainingCount-r13 | 0 |  |  |
| drb-ContinueROHC-r13 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Note 1: The RRCConnectionResume-NB (SRB-1 DCCH message) is sent on the same MAC PDU as Contention Resolution (RA Msg4). |  |  |  |

#### –	RRCConnectionResumeComplete-NB

Table 8.1.6.1-12: RRCConnectionResumeComplete-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeComplete-r13 SEQUENCE { |  |  |  |
| selectedPLMN-Identity-r13 | Not checked |  |  |
| dedicatedInfoNAS-r13 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionResumeRequest-NB

Table 8.1.6.1-13: RRCConnectionResumeRequest-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionResumeRequest-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionResumeRequest-r13 SEQUENCE { |  |  |  |
| resumeID-r13 | Present but contents not checked |  |  |
| resumeMAC-I-r13 | Present but contents not checked |  |  |
| resumeCause-r13 | Present but contents not checked |  |  |
| spare | Present but contents not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionSetup-NB

Table 8.1.6.1-14: RRCConnectionSetup-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetup-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionSetup-r13 SEQUENCE { |  |  |  |
| radioResourceConfigDedicated-r13 | RadioResourceConfigDedicated-NB-SRB |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	RRCConnectionSetupComplete-NB

Table 8.1.6.1-15: RRCConnectionSetupComplete-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcConnectionSetupComplete-r13 SEQUENCE { |  |  |  |
| selectedPLMN-Identity | 1 |  |  |
| s-TMSI-r13 | Not checked |  |  |
| registeredMME | Not checked |  |  |
| dedicatedInfoNAS | Present but contents not checked |  |  |
| attachWithoutPDN-Connectivity-r13 | Not checked |  |  |
| up-CIoT-EPS-Optimisation-r13 | Not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SCPTMConfiguration-NB

Table 8.1.6.1-15a: SCPTMConfiguration-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SCPTMConfiguration-NB-r14 ::= SEQUENCE { |  |  |  |
| sc-mtch-InfoList-r14 SEQUENCE (SIZE (0.. maxSC-MTCH-NB-r14)) OF SEQUENCE { | 1 entry |  |  |
| sc-mtch-CarrierConfig-r14 CHOICE { |  |  |  |
| dl-CarrierIndex-r14 | 0 | INTEGER (0.. maxNonAnchorCarriers-NB-r14) |  |
| } |  |  |  |
| mbmsSessionInfo-r14 SEQUENCE { |  |  |  |
| tmgi-r13 SEQUENCE { |  |  |  |
| plmn-Id-r9 CHOICE { |  |  |  |
| plmn-Index-r9 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| serviceId-r9 | ‘000001’H | OCTET STRING (SIZE (3)) |  |
| } |  |  |  |
| sessionId-r13 | Not present |  |  |
| } |  |  |  |
| g-RNTI-r14 | ‘0100’H |  |  |
| sc-mtch-SchedulingInfo-r14 | Not present |  |  |
| sc-mtch-neighbourCell-r14 | Not present |  |  |
| npdcch-NPDSCH-MaxTBS-SC-MTCH-r14 | n680 |  |  |
| npdcch-NumRepetitions-SC-MTCH-r14 | r16 |  |  |
| npdcch-StartSF-SC-MTCH-r14 | v4 |  |  |
| npdcch-Offset-SC-MTCH-r14 | zero |  |  |
| } |  |  |  |
| scptm-NeighbourCellList-r14 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |

#### –	UECapabilityEnquiry-NB

Table 8.1.6.1-16: UECapabilityEnquiry-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityEnquiry-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r13 SEQUENCE { |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UECapabilityInformation-NB

Table 8.1.6.1-17: UECapabilityInformation-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UECapabilityInformation-NB ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueCapabilityInformation-r13 SEQUENCE { |  |  |  |
| ue-Capability-Container-r13 SEQUENCE { |  |  |  |
| accessStratumRelease-r13 | Same as indicated in TC applicability in TS 36.523-2 [19] |  |  |
| ue-Category-NB-r13 | Not checked |  |  |
| multipleDRB-r13 | Not checked |  |  |
| pdcp-Parameters-r13 | Not checked |  |  |
| phyLayerParameters-r13 SEQUENCE { |  |  |  |
| multiTone-r13 | Not checked |  |  |
| multiCarrier-r13 | Not checked |  |  |
| } |  |  |  |
| rf-Parameters-r13 | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| ue-RadioPagingInfo-r13 SEQUENCE { |  |  |  |
| ue-Category-NB-r13 | Not checked |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	ULInformationTransfer-NB

Table 8.1.6.1-18: ULInformationTransfer-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ULInformationTransfer-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ulInformationTransfer-r13 SEQUENCE { |  |  |  |
| dedicatedInfoNAS-r13 | Present but contents not checked |  |  |
| lateNonCriticalExtension | Not checked |  |  |
| nonCriticalExtension SEQUENCE {} | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.6.2	NB-IoT System information blocks

See subclause 8.1.4.3.3 in this document.

#### 8.1.6.3	NB-IoT Radio resource control information elements

#### –	BCCH-Config-NB-DEFAULT

Table 8.1.6.3-1: BCCH-Config-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| BCCH-Config-NB-DEFAULT::= SEQUENCE { |  |  |  |
| modificationPeriodCoeff-r13 | n32 | To provide reliable delivery of SI change notifications. |  |
| } |  |  |  |

#### –	DL-CarrierConfigCommon-NB-DEFAULT

Table 8.1.6.3-1A: DL-CarrierConfigCommon-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DL-CarrierConfigCommon-NB -DEFAULT::= SEQUENCE { |  |  |  |
| dl-CarrierFreq-r14 | See subclause 8.1.3.1 |  |  |
| downlinkBitmapNonAnchor-r14 CHOICE { |  |  |  |
| useNoBitmap | NULL |  |  |
| } |  |  |  |
| dl-GapNonAnchor-r14 CHOICE { |  |  |  |
| useNoGap-r14 | NULL |  |  |
| } |  |  |  |
| inbandCarrierInfo-r14 SEQUENCE { |  |  | NON-ANCHOR-INBAND |
| samePCI-Indicator-r14 | Not present |  |  |
| eutraControlRegionSize-r14 | n2 |  |  |
| } |  |  |  |
| nrs-PowerOffsetNonAnchor-r14 | dB0 |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NON-ANCHOR-INBAND | The non-anchor carrier is an inband carrier |

#### –	PCCH-Config-NB-DEFAULT

Table 8.1.6.3-2: PCCH-Config-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PCCH-Config-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| defaultPagingCycle-r13 | rf128 | Typical value in real network. |  |
| nB-r13 | oneT |  |  |
| npdcch-NumRepetitionPaging-r13 | r8 |  |  |
|  | r32 |  | RRM_NTN_Enh |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

#### –	NPDCCH-ConfigDedicated-NB-DEFAULT

Table 8.1.6.3-3: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r16 |  |  |
| npdcch-StartSF-USS-r13 | v4 |  |  |
| npdcch-Offset-USS-r13 | zero |  |  |
| } |  |  |  |

#### –	NPDSCH-ConfigCommon-NB-DEFAULT

Table 8.1.6.3-4: NPDSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nrs-Power-r13 | 44 (dBm) |  |  |
| } |  |  |  |

#### –	NPRACH-ConfigSIB-NB-DEFAULT

Table 8.1.6.3-5: NPRACH-ConfigSIB-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| nprach-CP-Length-r13 | us66dot7 |  |  |
| rsrp-ThresholdsPrachInfoList-r13 | Not present | Only one resource |  |
| nprach-ParametersList-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| nprach-Periodicity-r13 | ms80 |  |  |
| nprach-StartTime-r13 | ms32 |  |  |
| nprach-SubcarrierOffset-r13 | n12 |  |  |
| nprach-NumSubcarriers-r13 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r13 | oneThird |  |  |
| maxNumPreambleAttemptCE-r13 | n6 |  |  |
| numRepetitionsPerPreambleAttempt-r13 | n1 |  |  |
| npdcch-NumRepetitions-RA-r13 | r16 |  |  |
|  | r32 |  | RRM_NTN_Enh |
| npdcch-StartSF-CSS-RA-r13 | v4 |  |  |
| npdcch-Offset-RA-r13 | zero |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

#### –	NPUSCH-ConfigCommon-NB-DEFAULT

Table 8.1.6.3-6: NPUSCH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-Msg4-r13 SEQUENCE (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF { ACK-NACK-NumRepetitions-NB-r13 } | r8 | 1 entry |  |
| srs-SubframeConfig-r13 | Not present |  |  |
| dmrs-Config-r13 SEQUENCE { |  |  |  |
| threeTone-BaseSequence-r13 | Not present |  |  |
| threeTone-CyclicShift-r13 | 0 |  |  |
| sixTone-BaseSequence-r13 | Not present |  |  |
| sixTone-CyclicShift-r13 | 0 |  |  |
| twelveTone-BaseSequence-r13 | Not present |  |  |
| } |  |  |  |
| ul-ReferenceSignalsNPUSCH-r13 SEQUENCE { |  |  |  |
| groupHoppingEnabled-r13 | FALSE |  |  |
| groupAssignmentPUSCH-13 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	NPUSCH-ConfigDedicated-NB-DEFAULT

Table 8.1.6.3-7: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r8 | Default |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

#### –	RACH-ConfigCommon-NB-DEFAULT

Table 8.1.6.3-8: RACH-ConfigCommon-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| preambleTransMax-CE-r13 | n6 |  |  |
| powerRampingParameters-r13 SEQUENCE { |  |  |  |
| powerRampingStep | dB2 |  |  |
| preambleInitialReceivedTargetPower | dBm-104 |  |  |
| } |  |  |  |
| rach-InfoList-r13 (SIZE (1.. maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| ra-ResponseWindowSize-r13 | pp10 |  |  |
| mac-ContentionResolutionTimer-r13 | pp8 |  |  |
| } |  |  |  |
| connEstFailOffset-r13 | Not present |  |  |
| } |  |  |  |

#### –	RadioResourceConfigCommonSIB-NB-DEFAULT

Table 8.1.6.3-9: RadioResourceConfigCommonSIB-NB-DEFAULT

| Derivation Path: TS 36.331 [17], clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigCommonSIB-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rach-ConfigCommon-r13 | RACH-ConfigCommon-NB-DEFAULT |  |  |
| bcch-Config-r13 | BCCH-Config-NB-DEFAULT |  |  |
| pcch-Config-r13 | PCCH-Config-NB-DEFAULT |  |  |
|  | PCCH-Config-NB-DEFAULT with condition RRM_NTN_Enh |  | RRM_NTN_Enh |
| nprach-Config-r13 | NPRACH-ConfigSIB-NB-DEFAULT |  |  |
|  | NPRACH-ConfigSIB-NB-DEFAULT with condition RRM_NTN_Enh |  | RRM_NTN_Enh |
| npdsch-ConfigCommon-r13 | NPDSCH-ConfigCommon-NB-DEFAULT |  |  |
| npusch-ConfigCommon-r13 | NPUSCH-ConfigCommon-NB-DEFAULT |  |  |
| dl-Gap-r13 | Not present |  |  |
| uplinkPowerControlCommon-r13 | UplinkPowerControlCommon-NB-DEFAULT |  |  |
| nprach-Config-v1330 | Not present |  |  |
| nprach-Config-v1450 | Not present |  |  |
| nprach-Config-v1530 | NPRACH-ConfigSIB-NB-v1530-DEFAULT |  | TDD |
| dl-Gap-v1530 | Not present |  |  |
| wus-Config-v1530 | Not present |  |  |
| nprach-Config-v1550 | NPRACH-ConfigSIB-NB-v1550-DEFAULT |  | TDD |
| gwus-Config-r16 | Not present |  |  |
| nrs-NonAnchorConfig-r16 | Not present |  |  |
| ue-SpecificDRX-CycleMin-r16 | Not present |  |  |
| ntn-ConfigCommon-r17 | Not present |  |  |
| ntn-ConfigCommon-r17 SEQUENCE { |  |  | NTN |
| ta-Report-r17 | enabled |  |  |
| t318-r17 | ms8000 |  |  |
| nprach-TxDurationFmt01-r17 | Not present |  |  |
| nprach-TxDurationFmt01-r17 SEQUENCE { |  |  | NGSO |
| nprach-TxDurationFmt01-r17 | n32 |  |  |
| } |  |  |  |
| nprach-TxDurationFmt2-r17 | Not present |  |  |
| nprach-TxDurationFmt2-r17 SEQUENCE { |  |  | NGSO |
| nprach-TxDurationFmt2-r17 | n8 |  |  |
| } |  |  |  |
| npusch-TxDuration-r17 | Not present |  |  |
| npusch-TxDuration-r17 SEQUENCE { |  |  | NGSO |
| npusch-TxDuration-r17 | ms128 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| NTN | Non-Terrestrial Networks test environment |
| NGSO | Non-Geosynchronous Orbit test environment |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

#### –	RadioResourceConfigDedicated-NB-SRB

Table 8.1.6.3-10: RadioResourceConfigDedicated-NB-SRB

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-SRB ::= SEQUENCE { |  |  |  |
| srb-ToAddModList-r13 SEQUENCE (SIZE (1)) OF SEQUENCE {} | 1 entry, with value SRB-ToAddMod-NB-DEFAULT | See subclause 8.1.8.2 |  |
| drb-ToAddModList-r13 | Not present |  |  |
| drb-ToReleaseList-r13 | Not present |  |  |
| mac-MainConfig CHOICE { |  |  |  |
| explicitValue-r13 | MAC-MainConfig-NB-SRB | See subclause 8.1.8.2 |  |
| } |  |  |  |
| physicalConfigDedicated-r13 | PhysicalConfigDedicated-NB-DEFAULT | See subclause 8.1.8.2 |  |
| rlf-TimersAndConstants-r13 | Not present |  |  |
| } |  |  |  |

#### –	RadioResourceConfigDedicated-NB-DRB(n)

Table 8.1.6.3-11: RadioResourceConfigDedicated-NB-DRB(n)

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-DRB(n) ::= SEQUENCE { |  | n is the number of AM RLC DRBs (0..2) |  |
| srb-ToAddModList-r13 | Not present |  |  |
| drb-ToAddModList-r13 SEQUENCE (SIZE (1.. maxDRB-NB-r13)) OF SEQUENCE { | n |  |  |
| drb-ToAddMod[n]-r13 | DRB-ToAddMod-NB-DEFAULT (n) | n AM RLC DRBs See subclause 8.1.8.2 | n>0 |
| } |  |  |  |
| drb-ToReleaseList-r13 | Not present |  |  |
| mac-MainConfig-r13 | Not present |  |  |
| physicalConfigDedicated-r13 | Not present |  |  |
| rlf-TimersAndConstants-r13 | Not present |  |  |
| } |  |  |  |

#### -	RadioResourceConfigDedicated-NB-DRB-ADD(bid)

Table 8.1.6.3-11A: RadioResourceConfigDedicated-NB-DRB-ADD(bid)

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-DRB-ADD(bid) ::= SEQUENCE { |  | bid is the bearer identity |  |
| srb-ToAddModList-r13 | Not present |  |  |
| drb-ToAddModList-r13 SEQUENCE (SIZE (1.. maxDRB-NB-r13)) OF SEQUENCE { | one entry |  |  |
| drb-ToAddMod-r13[1] | DRB-ToAddMod-NB-DEFAULT (bid) | See subclause 8.1.8.2 |  |
| } |  |  |  |
| drb-ToReleaseList-r13 | Not present |  |  |
| mac-MainConfig-r13 | Not present |  |  |
| physicalConfigDedicated-r13 | Not present |  |  |
| rlf-TimersAndConstants-r13 | Not present |  |  |
| } |  |  |  |

#### -	RadioResourceConfigDedicated-NB-DRB-REL(bid)

Table 8.1.6.3-11B: RadioResourceConfigDedicated-NB-DRB-REL(bid)

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-DRB-REL(bid) ::= SEQUENCE { |  | bid is the bearer identity |  |
| srb-ToAddModList-r13 | Not present |  |  |
| drb-ToAddModList-r13 | Not present |  |  |
| drb-ToReleaseList-r13 SEQUENCE (SIZE (1.. maxDRB-NB-r13)) OF | one entry |  |  |
| drb-Identity[1] | bid |  |  |
| mac-MainConfig-r13 | Not present |  |  |
| physicalConfigDedicated-r13 | Not present |  |  |
| rlf-TimersAndConstants-r13 | Not present |  |  |
| } |  |  |  |

#### –	RLC-Config-NB-SRB-RECONFIG

Table 8.1.6.3-12: RLC-Config-NB-SRB-RECONFIG

| Derivation Path: 36.331 clause 6.7.3, 9.2.1.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-NB-SRB-RECONFIG ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC-r13 SEQUENCE { |  |  |  |
| t-PollRetransmit-r13 | ms15000 |  |  |
| maxRetxThreshold-r13 | t6 |  |  |
| } |  |  |  |
| dl-AM-RLC-r13 SEQUENCE { |  |  |  |
| enableStatusReportSN-Gap-r13 | Not present | Default |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	SRB-ToAddModList-NB-RECONFIG

Table 8.1.6.3-13: SRB-ToAddModList-NB-RECONFIG

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRB-ToAddModList-NB ::= SEQUENCE (SIZE (1)) OF SEQUENCE { | 1 Entries |  |  |
| rlc-Config-r13 CHOICE { |  |  |  |
| explicitValue-r13 | RLC-Config-NB-SRB-RECONFIG |  |  |
| logicalChannelConfig-r13 CHOICE { |  |  |  |
| defaultValue | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

#### –	UplinkPowerControlCommon-NB-DEFAULT

Table 8.1.6.3-14: UplinkPowerControlCommon-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlCommon-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-NominalNPUSCH-r13 | -85 (-85 dBm) |  |  |
| alpha-r13 | al1 (1) |  |  |
| deltaPreambleMsg3-r13 | 4 |  |  |
| } |  |  |  |

#### –	UplinkPowerControlDedicated-NB-DEFAULT

Table 8.1.6.3-15: UplinkPowerControlDedicated-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UplinkPowerControlDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| p0-UE-NPUSCH-r13 | 0 | Default |  |
| } |  |  |  |

#### -	RadioResourceConfigDedicated-NB-DRB-Mod

Table 8.1.6.3-16: RadioResourceConfigDedicated-NB-DRB-Mod

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioResourceConfigDedicated-NB-DRB ::= SEQUENCE { |  |  |  |
| srb-ToAddModList-r13 | Not present |  |  |
| drb-ToAddModList-r13 | DRB-ToAddModList-NB-RECONFIG |  |  |
| } |  |  |  |
| drb-ToReleaseList-r13 | Not present |  |  |
| mac-MainConfig-r13 | Not present |  |  |
| physicalConfigDedicated-r13 | Not present |  |  |
| rlf-TimersAndConstants-r13 | Not present |  |  |
| } |  |  |  |

#### -	NPRACH-ConfigSIB-NB-v1530-DEFAULT

Table 8.1.6.3-17: NPRACH-ConfigSIB-NB-v1530-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-v1530-DEFAULT ::= SEQUENCE { |  |  |  |
| tdd-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-ParametersListTDD-r15 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| { |  |  |  |
| nprach-Parameters-r15 SEQUENCE { |  |  |  |
| nprach-Periodicity-r15 | ms640 |  |  |
| nprach-StartTime-r15 | ms10 |  |  |
| nprach-SubcarrierOffset-r15 | n12 |  |  |
| nprach-NumSubcarriers-r15 | n12 |  |  |
| nprach-SubcarrierMSG3-RangeStart-r15 | oneThird |  |  |
| npdcch-NumRepetitions-RA-r15 | r16 |  |  |
| npdcch-StartSF-CSS-RA-r15 | v4 |  |  |
| npdcch-Offset-RA-r15 | zero |  |  |
| nprach-NumCBRA-StartSubcarriers-r15 | n8 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### -	NPRACH-ConfigSIB-NB-v1550-DEFAULT

Table 8.1.6.3-18: NPRACH-ConfigSIB-NB-v1550-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPRACH-ConfigSIB-NB-v1550-DEFAULT ::= SEQUENCE { |  |  |  |
| tdd-Parameters-v1550 SEQUENCE { |  |  |  |
| nprach-ParametersListTDD-v1550 SEQUENCE (SIZE (1..maxNPRACH-Resources-NB-r13)) OF SEQUENCE { | 1 entry |  |  |
| { |  |  |  |
| maxNumPreambleAttemptCE-v1550 | n3 |  |  |
| numRepetitionsPerPreambleAttempt-v1550 | n1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

#### 8.1.6.4	NB-IoT Security control information elements

See subclause 4.6.4 in this document.

#### 8.1.6.5	NB-IoT Other information elements

#### –	RRC-TransactionIdentifier-DL

Table 8.1.6.5-1: RRC-TransactionIdentifier-DL

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRC-TransactionIdentifier-DL ::= | 0..3 |  |  |

#### –	RRC-TransactionIdentifier-UL

Table 8.1.6.5-2: RRC-TransactionIdentifier-UL

| Derivation Path: 36.331 clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRC-TransactionIdentifier-UL ::= | 0..3 | The same value as the value of RRC-TransactionIdentifier-DL in the downlink message initiating the procedure |  |

### 8.1.7	NB-IoT Default NAS message and information element contents

Default values and common NAS messages and information elements are defined in section 4.7 of this document.

### 8.1.7A	NB-IoT Default TC message and information element contents

Default values of common TC (Test Control, see [38]) messages and information elements are defined in section 4.7A of this document.

### 8.1.8	NB-IoT Reference radio bearer configurations

#### 8.1.8.1	General

#### 8.1.8.2	NB-IoT SRB and DRB parameters and combinations

##### 8.1.8.2.1	NB-IoT SRB and DRB parameters

8.1.8.2.1.1	NB-IoT SRB configurations

Table 8.1.8.2.1.1-1: SRB-ToAddMod-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SRB-ToAddMod-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| rlc-Config-r13 CHOICE { |  |  |  |
| defaultValue |  | Default values defined in subclause 9.2.1.1 of TS 36.331 |  |
| } |  |  |  |
| logicalChannelConfig CHOICE { |  |  |  |
| defaultValue |  | Default values defined in subclause 9.2.1.1 of TS 36.331 |  |
| } |  |  |  |
| } |  |  |  |

8.1.8.2.1.2	NB-IoT DRB PDCP configurations

8.1.8.2.1.2.1	DRB PDCP configurations for AM RLC

Table 8.1.8.2.1.2.1-1: PDCP-Config-NB-DRB

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCP-Config-NB-DRB ::= SEQUENCE { |  |  |  |
| discardTimer-r13 | Infinity |  |  |
| headerCompression-r13 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.8.2.1.3	NB-IoT DRB RLC configurations

8.1.8.2.1.3.1	NB-IoT DRB AM RLC configurations

Table 8.1.8.2.1.3.1-1: RLC-Config-NB-DRB

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RLC-Config-NB-DRB ::= CHOICE { |  |  |  |
| am SEQUENCE { |  |  |  |
| ul-AM-RLC-r13 SEQUENCE { |  |  |  |
| t-PollRetransmit-r13 | ms25000 |  |  |
| maxRetxThreshold-r13 | t8 |  |  |
| } |  |  |  |
| dl-AM-RLC-r13 SEQUENCE { |  |  |  |
| enableStatusReportSN-Gap-r13 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.8.2.1.4	NB-IoT DRB Logical Channel configurations

Table 8.1.8.2.1.4-1: LogicalChannelConfig-NB-DRB

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LogicalChannelConfig-NB-DRB ::= SEQUENCE { |  |  |  |
| priority-r13 | 13 | low logical channel priority |  |
| logicalChannelSR-Prohibit-r13 | TRUE |  |  |
| } |  |  |  |

8.1.8.2.1.5	NB-IoT MAC configurations

Table 8.1.8.2.1.5-1: MAC-MainConfig-NB-SRB

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MAC-MainConfig-NB-SRB ::= SEQUENCE { |  |  |  |
| ul-SCH-Config-r13 SEQUENCE { |  |  |  |
| periodicBSR-Timer-r13 | pp8 | Default |  |
| retxBSR-Timer-r13 | infinity | Default |  |
| } |  |  |  |
| drx-Config-r13 CHOICE { |  |  |  |
| release | NULL |  |  |
| } |  |  |  |
| timeAlignmentTimerDedicated-r13 | infinity |  |  |
| logicalChannelSR-Config-r13 CHOICE { |  |  |  |
| logicalChannelSR-ProhibitTimer-r13 | pp2048 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.1.8.2.1.6	NB-IoT Physical Layer configurations

Table 8.1.8.2.1.6-1: PhysicalConfigDedicated-NB-DEFAULT

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PhysicalConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| carrierConfigDedicated-r13 | Not present | Anchor carrier |  |
| npdcch-ConfigDedicated-r13 | NPDCCH-ConfigDedicated-NB-DEFAULT | See subclause 8.1.6.3 |  |
|  | NPDCCH-ConfigDedicated-NB-DEFAULT with condition RRM_NTN_Enh | See subclause 8.1.6.3 | RRM_NTN_enh |
| npusch-ConfigDedicated-r13 | NPUSCH-ConfigDedicated-NB-DEFAULT | See subclause 8.1.6.3 |  |
| uplinkPowerControlDedicated-r13 | UplinkPowerControlDedicated-NB-DEFAULT | See subclause 8.1.6.3 |  |
| npdsch-ConfigDedicated-v1800 | Not present |  |  |
| npdsch-ConfigDedicated-v1800 SEQUENCE { |  |  | DL HARQ |
| downlinkHARQ-FeedbackDisabledBitmap-NB-r18 CHOICE { |  |  |  |
| setup | ’00’B | HARQ feedback enabled for HARQ process ID 0 and HARQ process ID 1 |  |
| } |  |  |  |
| downlinkHARQ-FeedbackDisabledDCI-NB-r18 | Not present |  |  |
| } |  |  |  |
| npusch-ConfigDedicated-v1800 | Not present |  |  |
| npusch-ConfigDedicated-v1800 SEQUENCE { |  |  | UL HARQ |
| uplinkHARQ-mode-r18 CHOICE { |  |  |  |
| setup | ’11’B | HARQ mode A for HARQ process ID 0 and HARQ process ID 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_enh | For RRM NB-IoT NTN enhanced coverage test cases |
| DL HARQ | When DL HARQ feedback is enabled in IoT NTN enhancement test case. |
| UL HARQ | When UL HARQ mode A is used in IoT NTN enhancement test case. |

8.1.8.2.1.7	NB-IoT DRB configurations

Table 8.1.8.2.1.7-1: DRB-ToAddMod-NB-DEFAULT(bid)

| Derivation Path: 36.331 clause 6.7.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRB-ToAddMod-NB-DEFAULT(bid) ::= SEQUENCE { |  | bid is the bearer identity (1..2) |  |
| eps-BearerIdentity-r13 | bid+4 |  |  |
| drb-Identity-r13 | Bid |  |  |
| pdcp-Config-r13 | PDCP-Config-NB-DRB |  |  |
| rlc-Config-r13 | RLC-Config-NB-DRB |  |  |
| logicalChannelIdentity-r13 | bid+3 |  |  |
| logicalChannelConfig-r13 | LogicalChannelConfig-NB-DRB |  |  |
| } |  |  |  |

### 8.1.9	NB-IoT Common test USIM parameters

#### 8.1.9.1	General

As defined in clause 4.9; i.e. there are no specific NB-IoT requirements in addition to the default test USIM parameters, unless specified otherwise in a test case defined in TS 36.523-1 [18].

For PLMN (re-)selection, the UE will decide on the PLMN to select based on ‘E-UTRA’ as Access Technology in the PLMN selector of USIM Elementary Files PLMNwAcT, OPLMNwAcT and HPLMNwAcT and then use its device category (= NB-IoT) to properly access the NB-IoT network.

## 8.2	NB-IoT Test environment for RF test

This section contains all the exceptions of the NB-IoT common test parameters specified in clause 8.1 for specific needs of test cases defined in TS 36.521-1 [21]. Exceptions specified in clause 8.2 overwrite the parameter settings of clause 8.1; exceptions defined within the test cases overwrite parameter settings of clause 8.1 and 8.2.

### 8.2.1	NB-IoT Requirements of test equipment

No NB-IoT common RF test environment requirements are specified in addition to the NB-IoT common requirements described in clause 8.1.2. Specific RF requirements are indicated within the test cases defined in TS 36.521-1 [21].

### 8.2.2	NB-IoT RF Reference system configurations

#### 8.2.2.1	NB-IoT Common parameters for simulated E-UTRA cells

##### 8.2.2.1.1		NB-IoT Combinations of system information blocks

The NB-IoT combination of system information blocks required by a test case depends on the test case scenario. In clause 8.1.4 of this document, the NB-IoT combinations of system information blocks are defined.

Combination 1, defined in clause 8.1.4, is used by default in test cases defined in TS 36.521-1 [21].

##### 8.2.2.1.2	NB-IoT Scheduling of system information blocks

Same NB-IoT scheduling of system information blocks as defined in clause 8.1.4.3.1.2.

##### 8.2.2.1.3	NB-IoT Common contents of system information messages

MasterInformationBlock-NB

As defined in Table 8.1.4.3.2-1.

SystemInformation-NB

As defined in Table 8.1.4.3.2-2 without exceptions.

SystemInformationBlockType1-NB

As defined in Table 8.1.4.3.2-3 without exceptions.

SystemInformationBlockType2-NB

As defined in Table 8.1.4.3.3-1 without exceptions.

SystemInformationBlockType31-NB

Table 8.2.2.1.3-1: SystemInformationBlockType31-NB

| Derivation Path: TS 36.508 clause 8.1.4.3.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | See cl. 8.2.6.2 |  |  |
| positionY-r17 | See cl. 8.2.6.2 |  |  |
| positionZ-r17 | See cl. 8.2.6.2 |  |  |
| velocityVX-r17 | See cl. 8.2.6.2 |  |  |
| velocityVY-r17 | See cl. 8.2.6.2 |  |  |
| velocityVZ-r17 | See cl. 8.2.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDrift-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| nta-CommonDriftVariation-r17 | 0 |  | GSO |
|  | 0 |  | NGSO |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s900 |  |  |
| epochTime-r17 | Not present |  |  |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

### 8.2.2A	NB-IoT Generic RF procedures

The NB-IoT UE test state used for testing is specified in the individual test cases in the corresponding test specification TS 36.521-1 [21] or TS 36.521-3 [34].

RF NB-IoT UE test states are covered by common NB-IoT UE test states defined in clause 8.1.5 of this document.

### 8.2.3	NB-IoT Default RRC message and information elements contents

#### 8.2.3.1	NB-IoT Radio resource control information elements

As defined in clause 8.1.6.3 with the following exceptions.

Table 8.2.3.1-1: NPDCCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: Clause 8.1.6.3 Table 8.1.6.3-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPDCCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| npdcch-NumRepetitions-r13 | r1 |  |  |
|  | r32 |  | RRM_NTN_Enh |
| npdcch-StartSF-USS-r13 | v8 |  |  |
| npdcch-Offset-USS-r13 | oneFourth |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| RRM_NTN_Enh | For RRM NB-IoT NTN enhanced coverage test cases |

Table 8.2.3.1-2: NPUSCH-ConfigDedicated-NB-DEFAULT

| Derivation Path: Clause 8.1.6.3 Table 8.1.6.3-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NPUSCH-ConfigDedicated-NB-DEFAULT ::= SEQUENCE { |  |  |  |
| ack-NACK-NumRepetitions-r13 | r1 |  |  |
| npusch-AllSymbols-r13 | TRUE | Default |  |
| groupHoppingDisabled-r13 | Not present | Default |  |
| } |  |  |  |

### 8.2.4	NB-IoT Default NAS message and information elements contents

As defined in clause 8.1.7 without exceptions.

### 8.2.5	NB-IoT Reference radio bearer configurations

#### 8.2.5.1	NB-IoT SRB and DRB parameters

As defined in clause 8.1.8.2.1 without exception.

### 8.2.6	Test environment for NB-IoT NTN RF/Demod testing

This section defines the test environment which applies to all RF and DEMOD test cases executed for NB-IoT NTN UEs, unless otherwise specified.

#### 8.2.6.1	UE location

Same as in section 5.6.1.

#### 8.2.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 8.2.6.2.0	Assumptions for Ephemeris generation

Same as 5.6.2.0.

#### 8.2.6.2.1	Ephemeris for zero Doppler conditions

All NB-IoT NTN test cases defined in sections 6, 7 and 8 in TS 36.521-4 [73], unless otherwise stated, shall use the ephemeris configuration in tables 8.2.6.2.1-1 to 8.2.6.2.1-3, corresponding to zero Doppler and constant delay for the type of satellite under test, keeping such information constant throughout the duration of each measurement in the test:

- In case UE supports only NGSO satellites, ephemeris values in Table 8.2.6.2.1-3 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 8.2.6.2.1-2 shall be used.

- In case UE supports both GSO and NGSO satellites, ephemeris values in Table 8.2.6.2.1-1 shall be used except for test cases 8.2.1.1.1 and 8.3.1.1.1, where ephemeris values in Table 8.2.6.2.1-2 shall be used.

Table 8.2.6.2.1-1: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 Table 8.2.2.1.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 8.2.6.2.1-2: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 Table 8.2.2.1.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm. |  |  |  |

Table 8.2.6.2.1-3: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: TS 36.508 Table 8.2.2.1.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

#### 8.2.6.3	NB-IoT NTN message contents for RF Tx/Rx, Demodulation tests

#### 8.2.6.3.1	ServingSatelliteInfo-r17 values in SIB31-NB

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31-NB shall be periodically signalled to the UE during test based on the Ephemeris information in clause 8.2.6.1 (kept constant throughout the duration of each measurement in the test).

## 8.3	NB-IoT Test environment for Signalling test

### 8.3.1	NB-IoT Requirements of test equipment

The requirements of test equipment specified in this subclause apply to NB-IoT Signalling test cases defined in TS 36.523-1 [18], in addition to the common NB-IoT requirements of test equipment specified in cause 8.1.2 and 8.1.3 of this specification. Test equipment shall be able to simulate NB-IoT cells, based on the E-UTRA platform as Radio Access Technology (RAT), in the following deployment scenarios:

- In-band operation with E-UTRA

- Guard-band operation with E-UTRA

- Standalone operation

Depending on the respective deployment scenario, the overall number and configuration of cells to be simulated simultaneously by test equipment shall not exceed the resources specified in the following Table 8.3.1-1:

Table 8.3.1-1: Maximum resources in terms of number / configuration of Ncells to be simulated simultaneously in a test setup

| Simulation of |  | Max. number / configuration of cells (SISO) |
| --- | --- | --- |
| NB-IoT network in standalone operation |  | 4x cells |
| NB-IoT network in E-UTRA guard-band operation (Note 1) |  | 4x cells |
| NB-IoT network in E-UTRA in-band operation (Note 1) |  | 4x cells |
| Note 1: E-UTRA cells are not configured. |  |  |

Exceptions to the requirements outlined above are possible but need special evidence to be provided explicitly in the test case prose and should be allowed only if the test case purpose cannot be met otherwise.

### 8.3.2	NB-IoT Reference test conditions

#### 8.3.2.1	NB-IoT Physical channel allocations

As defined in clause 8.1.3.3.

#### 8.3.2.2	NB-IoT Signal levels

##### 8.3.2.2.1	NB-IoT Downlink signal levels

The default settings of suitable cells and non-suitable cells for NB-IoT are specified in table 8.3.2.2.1-1. Cells which are expected to be undetectable for UE under test shall fulfil the condition of non-suitable "Off" cell.

Table 8.3.2.2.1-1: Default settings of suitable / non-suitable cells

| Power level type |  | NB-IoT (Note 1-3) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Unit |  | Power level |  |
| Serving cell |  | dBm/15kHz |  | -85 |  |
| Suitable neighbour intra-frequency cell |  | dBm/15kHz |  | -91 |  |
| Suitable neighbour inter-frequency cell |  | dBm/15kHz |  | -97 |  |
| Non-suitable cell |  | dBm/15kHz |  | -120 (Note 4) |  |
| Non-suitable "Off" cell |  | dBm/15kHz |  | ≤ -150 |  |
| Note 1: The power level is specified in terms of cell-specific NRS EPRE. Note 2: Power levels are specified based on the precondition that q-Hyst, a3-Offset and hysteresis are 0 dB. Note 3: The power level is specified at the UE Rx antenna (assumption = single UE Rx antenna). Note 4: Calculation of non-suitable cell power level is based on the reference sensitivity (without repetitions) for UE category NB1 as per the minimum requirements specified in TS 36.101 [27] clause 7.3.1F.1. |  |  |  |  |  |

The default signal level uncertainty is specified in table 8.3.2.2.1-2 for any level specified, unless a tighter uncertainty is specified by a test case in TS 36.523-1 [18].

Table 8.3.2.2.1-2: SS signal level uncertainty

|  | Absolute signal level uncertainty for each cell | Relative signal level uncertainty between multiple cells |
| --- | --- | --- |
| Intra-frequency | +/-3 dB at each test port | +/-3 dB |
| Inter-frequency | +/-3 dB at each test port | See Note 1 |
| Note 1: For Inter-frequency cells the relative signal level uncertainty between multiple cells is determined by the absolute uncertainty of each cell, and does not have any additional constraint. |  |  |

#### 8.3.2.3	NB-IoT Default test frequencies

The default channel bandwidth of 200 kHz is applied to the signalling test. The test frequencies are defined so that no frequency overlapping takes place, in order to avoid unnecessary inter-frequency interference.

All operating bands can accommodate at least three test frequencies f1, f2 and f3 (f3<f1<f2).

For NB-IoT in-band and guard-band signalling testing, the NB-IoT frequency to be tested are low/mid/high range and are associated to the LTE frequency low/mid/high range as specified in tables 4.3.1.1.x. The E-UTRA channel bandwidth to be tested is 10MHz and NB-IoT PRB 30 for in-band and 5 MHz for guard-band for all operating bands for all test cases; unless the specific channel bandwidth is specified for the operating band below:

For Band 31 in-band testing, the E-UTRA channel bandwidth to be tested is 5 MHz.

NB-IoT NTN in-band and guard-band signalling testing for Band 253, Band 254, Band 255 and Band 256 is not supported in this version as per TS 36.102 [74] clause 5.4B.2.

##### 8.3.2.3.1	NB-IoT FDD Mode Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.1-1 for FDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.1.

Table 8.3.2.3.1-1: Test frequencies for NB-IoT FDD (200 kHz)

| NB-IoT Operating Band |  |  | Bandwidth [MHz] |  |  | f1, f5 |  |  |  |  |  | f2 |  |  |  |  |  | f3 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NUL |  |  | NDL |  |  | NUL |  |  | NDL |  |  | NUL |  |  | NDL |  |  |
| 1 |  |  | 60 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 2 |  |  | 60 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 3 |  |  | 75 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 4 |  |  | 45 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 5 |  |  | 25 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  | 70 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 8 |  |  | 35 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  | 20 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 12 |  |  | 17 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 13 |  |  | 10 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 14 |  |  | 10 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  | 12 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 18 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 19 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 20 |  |  | 30 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 21 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  | 65 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 26 |  |  | 35 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  | 45 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  |  | 90 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 66 |  |  | 70+901 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 70 |  |  | 15 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 71 |  |  | 35 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 73 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 85 |  |  | 18 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 87 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 88 |  |  | 5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 103 |  |  | 1 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 253 |  |  | 7 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 254 |  |  | 16.5 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 255 |  |  | 34 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| 256 |  |  | 30 |  |  | Mid |  |  | Mid |  |  | High |  |  | High |  |  | Low |  |  | Low |  |  |
| NOTE 1: Asymmetric operating band (UL + DL). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.1-1a for FDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.1.

Table 8.3.2.3.1-1a: Test frequencies for NB-IoT FDD MFBI (200 kHz)

| NB-IoT Operating Band |  | MFBI Overlapping Band |  | Bandwidth [MHz] |  | f1, f5 |  |  |  | f2 |  |  |  | f3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | NUL |  | NDL |  | NUL |  | NDL |  | NUL |  | NDL |  |
| 2 |  | 25 |  | 60 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 10 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 4 |  | 66 |  | 45 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 5 |  | 18 |  | 15 |  | Mid |  | Mid |  | 23999 |  | 5999 |  | Low |  | Low |  |
| 5 |  | 19 |  | 15 |  | Mid |  | Mid |  | 20609 |  | 2609 |  | Low |  | Low |  |
| 5 |  | 26 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 12 |  | 17 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 17 |  | 12 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 17 |  | 85 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 5 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 19 |  | 26 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 25 |  | 2 |  | 60 |  | Mid |  | Mid |  | 26638 |  | 8638 |  | Low |  | Low |  |
| 26 |  | 5 |  | 25 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 18 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 26 |  | 19 |  | 15 |  | Mid |  | Mid |  | High |  | High |  | Low |  | Low |  |
| 66 |  | 4 |  | 45 |  | Mid |  | Mid |  | 132421 |  | 66885 |  | Low |  | Low |  |
| 66 |  | 10 |  | 60 |  | Mid |  | Mid |  | 132571 |  | 67035 |  | Low |  | Low |  |
| 85 |  | 17 |  | 12 |  | Mid |  | Mid |  | High |  | High |  | 134072 |  | 70436 |  |

Test frequencies for signalling test are specified in table 8.3.2.3.1-2 for TDD in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.

Table 8.3.2.3.1-2: Test frequencies for NB-IoT TDD (200 kHz)

| NB-IoT Operating Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 41 | 60 | Mid | Mid | High | High | Low | Low |

##### 8.3.2.3.2	NB-IoT TDD Mode Test frequencies for signalling test

Test frequencies for signalling test are specified in table 8.3.2.3.2-1 in terms of Low, Mid and High which are referred to the Low Range, Mid Range and High Range in clause 8.1.3.1.2

Table 8.3.2.3.2-1: Test frequencies for NB-IoT TDD (200 kHz)

| NB-IoT Operating Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 41 | 194 | Mid | Mid | High | High | Low | Low |
| 42 | 200 | Mid | Mid | High | High | Low | Low |
| 43 | 200 | Mid | Mid | High | High | Low | Low |

Test frequencies for signalling test of MFBI are specified in Table 8.3.2.3.2-1a for TDD in terms of Low, Mid and High which are referred to as Low Range, Mid Range and High range in clause 8.1.3.1.2.

Table 8.3.2.3.2-1a: Test frequencies for NB-IoT TDD MFBI (200 kHz)

| NB-IoT Operating Band | MFBI Overlapping Band | Bandwidth [MHz] | f1, f5 |  | f2 |  | f3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NUL | NDL | NUL | NDL | NUL | NDL |
| 41 | 38 | 50 | Mid | Mid | High | High | Low | Low |

### 8.3.3	NB-IoT Reference system configurations

The reference system configurations specified in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified, in addition to the common reference system configurations specified in subclause 8.1.4 of this specification.

Only one SS Tx antenna is used.

One UE antenna is used for all signalling test cases.

#### 8.3.3.1	NB-IoT Default parameters specific for simulated cells

Default parameters specific for simulated cells are specified in this subclause.

##### 8.3.3.1.1	Intra-frequency neighbouring cell list in SIB4-NB for NB-IoT cells

Intra-frequency neighbouring cell list for signalling test cases is defined in table 8.3.3.1.1-1. This table is referred to in the default contents of IE intraFreqNeighbouringCellList in SystemInformationBlockType4-NB defined in table 8.1.4.3.3-3.

Table 8.3.3.1.1-1: Intra-frequency neighbouring cell lists for NB-IoT cells

| cell ID | Test Frequency | intra-frequency neighbouring cell list |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |  | q-OffsetCell[n] |  |  |
|  |  |  | 1 | 2 | 3 | 1 | 2 | 3 |
| Ncell 1 | f1 | 3 | Ncell 2 | Ncell 4 | Ncell 11 | dB0 | dB0 | dB0 |
| Ncell 2 | f1 | 3 | Ncell 1 | Ncell 4 | Ncell 11 | dB0 | dB0 | dB0 |
| Ncell 4 | f1 | 3 | Ncell 1 | Ncell 2 | Ncell 11 | dB0 | dB0 | dB0 |
| Ncell 11 | f1 | 3 | Ncell 1 | Ncell 2 | Ncell 4 | dB0 | dB0 | dB0 |
| Ncell 3 | f2 | 1 | Ncell 23 | - | - | dB0 | - | - |
| Ncell 23 | f2 | 1 | Ncell 3 | - | - | dB0 | - | - |

Intra-frequency neighbouring cell list of the same PLMN for the NAS signalling test is defined in table 8.3.3.1.1-2 when SIB4-NB is broadcast.

Table 8.3.3.1.1-2: Intra-freq. lists in SIB4 for NAS test cases (same PLMN)

| NAS cell ID | Test Frequency | Intra-frequency neighbouring cell list |  |  |
| --- | --- | --- | --- | --- |
|  |  | number of entries | physCellId[n] |  |
|  |  |  | 1 | 2 |
| Ncell 50 | f1 | 2 | Ncell 51 | Ncell 53 |
| Ncell 51 | f1 | 2 | Ncell 50 | Ncell 53 |
| Ncell 53 | f1 | 2 | Ncell 50 | Ncell 51 |
| Ncell 52 | f2 | 1 | Ncell 61 | N/A |
| Ncell 61 | f2 | 1 | Ncell 52 | N/A |

##### 8.3.3.1.2	Inter-frequency carrier frequency list in SIB5-NB for NB-IoT cells

Inter-frequency E-UTRA carrier frequency list for signalling test cases is defined in table 8.3.3.1.2-1. This table is referred to in the default contents of IE interFreqCarrierFreqList in SystemInformationBlockType5-NB defined in table 8.1.4.3.3-4.

Table 8.3.3.1.2-1: Inter-frequency carrier frequency lists for NB-IoT cells

| cell ID | Test Frequency | interFreqCarrierFreqList |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  | 1 | 2 | 3 |
| Ncell 1 Ncell 2 Ncell 4 Ncell 11 | f1 | 3 | f2 | f3 | f5 |
| Ncell 3 Ncell 23 | f2 | 3 | f1 | f3 | f5 |
| Ncell 6 | f3 | 3 | f1 | f2 | f5 |
| Ncell 10 | f5 | 3 | f1 | f2 | f3 |
| Note 1: The inter-frequency carrier frequency list for signalling NAS test cases when cells are on same PLMN is defined in table 8.3.3.1.2-2. Note 2: In case of Test frequency f1, f2 and f3, dl-CarrierFreq f5 as part of inter-frequency list is applicable only in case of multi-band scenarios. Note 3: In case of Test frequency f5, dl-CarrierFreq f1 as part of inter-frequency list is applicable only in case of multi-band scenarios. |  |  |  |  |  |

Inter-frequency neighbouring carrier and cell lists for NAS signalling test cases are defined in table 8.3.3.1.2-2 for same PLMN and table 8.3.3.1.2-3 for different PLMN.

Table 8.3.3.1.2-2: Inter-freq. lists in SIB5-NB for NAS test cases (same PLMN)

| NAS cell ID | Test Frequency | interFreqCarrierFreqList |  |  |
| --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |
|  |  |  | 1 | 2 |
| Ncell 50 Ncell 51 Ncell 53 | f1 | 1 | f2 | NA |
| Ncell 52 Ncell 61 | f2 | 1 | f1 | NA |

Table 8.3.3.1.2-3: Inter-freq. lists in SIB5-NB for NAS test cases (different PLMN)

| NAS cell ID | Test Frequency | interFreqCarrierFreqList |  |  | Condition |
| --- | --- | --- | --- | --- | --- |
|  |  | number of entries | dl-CarrierFreq[n] |  |  |
|  |  |  | 1 | 2 |  |
| Ncell 50 Ncell 51 Ncell 52 Ncell 53 Ncell 62 | f1 | 0 | NA | NA |  |
| Ncell 55 Ncell 56 | f2 | 0 | NA | NA |  |
| Ncell 57 Ncell 59 Ncell 60 | f3 | 0 | NA | NA | 54-NOT-CONF |
|  |  | 1 | f2 |  | 54-CONF |
| Ncell 54 | f2 | 1 | f3 | NA |  |

| Condition | Explanation |
| --- | --- |
| 54-NOT-CONF | Ncell 54 is not configured in the test |
| 54-CONF | Ncell 54 is configured in the test |

#### 8.3.3.2	NB-IoT Default configurations for NAS test cases

As defined in clause 8.1.4.

#### 8.3.3.3	NB-IoT Cell configurations

All Ncells are assumed to be full cells unless specified otherwise in a test case defined in TS 36.523-1 [18].

### 8.3.4	NB-IoT Generic signalling procedures

#### 8.3.4.1	NB-IoT Initial UE states and setup procedures

As defined in clause 8.1.5.

#### 8.3.4.2	NB-IoT Reference procedures and test procedures for TTCN development

As defined in clause 8.1.5A.

#### 8.3.4.3	NB-IoT Test case postambles for TTCN development

As defined in clause 10.5 of TS 36.523-3 [20].

### 8.3.5	NB-IoT Default RRC message and information element contents

As defined in clause 8.1.6.

### 8.3.6	NB-IoT Default NAS message and information element contents

As defined in clause 8.1.7.

### 8.3.7	NB-IoT Timer tolerances

The timer tolerances specified for the test environment in this subclause apply to all Signalling test cases defined in TS 36.523-1 [18] unless otherwise specified.

All the timers used during testing are within a tolerance margin given in Table 8.3.7.-1 below. For NB-IoT and its typically longer timer values across all protocol layers, common timer tolerances are applicable independently from protocol layer resp. test case group. If for a specific test a different tolerance value is required then this should be specified in the relevant test document (i.e. the document where the test is described).

Table 8.3.7-1: Timer tolerances for protocol timers

|  | timer value | < 500 ms | not applicable |
| --- | --- | --- | --- |
| 500 ms ≤ | timer value | < 10000 ms | timer tolerance = 10 % |
| 10000 ms ≤ | timer value | < 40000 ms | timer tolerance = 5 % |
| 40000 ms ≤ | timer value |  | timer tolerance = 2.5 % |

### 8.3.8	Signalling test environment for Vertical UEs

#### 8.3.8.1	NB-IoT NTN

This clause defines the signalling test environment which applies to all signalling test cases executed for NB-IoT NTN UEs, unless otherwise specified. This is in addition to the common test environment defined in clause 8.1.5B.1.

-	Test frequencies specified in Table 8.3.2.3.1-1 are used.

-	For NGSO scenario, UE prediction of satellite trajectory shall be deactivated in the pre-test conditions by any preconfigured means.

NOTE:	As constant ephemeris information in SIB31-NB is used for the NGSO scenario, the frequency Doppler and transmission delay values remain same during the test execution. Therefore, prediction of satellite trajectory needs to be deactivated in the UE, so that the UE will not update the Doppler and delay pre-compensation with the assumption that the satellite is moving before the expiry of timer T317 (ul-SyncValidityDuration-r17).

-	For Rel-13 to Rel-15 NB-IoT test cases applicable to a UE supporting NTN only access in NB-IoT (pc_NB_ntn_only_Connectivity_EPC):

-	UE is pre-configured with the UE position defined in clause 4.13.

-	The mapping of the system information combination in terrestrial networks (TN) to a corresponding system information combination in non-terrestrial networks (NTN) is defined in Table 8.3.8.1-1:

Table 8.3.8.1-1: Mapping of the TN to NTN system information combinations

| System Information Combination in TN environment | System Information Combination in NTN environment |
| --- | --- |
| c1 | c8 |
| c2 | c10 |
| c3 | c11 |
| c4 | c13 |
| c6 | c12 |

## 8.4	NB-IOT Test environment for RRM tests

This section contains all the exceptions of the common test parameters specified in clause 8.1 for specific needs of NB-IoT test cases defined in TS 36.521-3 [34]. Exceptions specified in clause 8.4 overwrite the parameter settings of clause 8.1; exceptions defined within the test cases overwrite parameter settings of clause 8.1 and 8.4.

### 8.4.1	NB-IoT Requirements of test equipment

No NB-IoT common RRM test environment requirements are specified in addition to the NB-IoT common requirements described in clause 8.1.2. Specific RRM requirements are indicated within the test cases defined in TS 36.521-3 [34].

### 8.4.2	NB-IoT RRM Reference system configurations

#### 8.4.2.1	NB-IoT Common parameters for simulated NB-IoT cells

##### 8.4.2.1.1	NB-IoT Combinations of system information blocks

The NB-IoT combination of system information blocks required by a test case depends on the test case scenario. In clause 8.1.4 of this document, the NB-IoT combinations of system information blocks are defined.

##### 8.4.2.1.2	NB-IoT Scheduling of system information blocks

Same NB-IoT scheduling of system information blocks as defined in clause 8.1.4.3.1.2.

##### 8.4.2.1.3	NB-IoT Common contents of system information messages

-	MasterInformationBlock-NB

As defined in Table 8.1.4.3.2-1 with the following exceptions.

Table 8.4.2.1.3-1: MasterInformationBlock-NB

| Derivation Path: 36.331 clause 6.7.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MasterInformationBlock-NB ::= SEQUENCE { |  |  |  |
| operationModeInfo-r13 CHOICE { |  |  |  |
| inband-SamePCI-r13 SEQUENCE { |  |  | Inband-Same |
| eutra-CRS-SequenceInfo-r13 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

- SystemInformation-NB

As defined in Table 8.1.4.3.2-2 without exceptions.

- SystemInformationBlockType1-NB

As defined in Table 8.1.4.3.2-3 without exceptions.

- SystemInformationBlockType2-NB

As defined in Table 8.1.4.3.3-1 without exceptions.

- SystemInformationBlockType31-NB

Table 8.4.2.1.3-2: SystemInformationBlockType31-NB

| Derivation Path: Table 8.1.4.3.3-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  | [TC_EF_SV] |
| positionX-r17 | See cl. 8.4.6.2 |  |  |
| positionY-r17 | See cl. 8.4.6.2 |  |  |
| positionZ-r17 | See cl. 8.4.6.2 |  |  |
| velocityVX-r17 | See cl. 8.4.6.2 |  |  |
| velocityVY-r17 | See cl. 8.4.6.2 |  |  |
| velocityVZ-r17 | See cl. 8.4.6.2 |  |  |
| } |  |  |  |
| orbitalParameters SEQUENCE { |  |  | [TC_EF_OP] |
| semiMajorAxis-r17 | See cl. 8.4.6.2 |  |  |
| eccentricity-r17 | See cl. 8.4.6.2 |  |  |
| periapsis-r17 | See cl. 8.4.6.2 |  |  |
| longitude-r17 | See cl. 8.4.6.2 |  |  |
| inclination-r17 | See cl. 8.4.6.2 |  |  |
| anomaly-r17 | See cl. 8.4.6.2 |  |  |
| } |  |  |  |
| } |  |  |  |
| nta-CommonParameters-17 SEQUENCE { |  |  |  |
| nta-Common-r17 | 0 |  |  |
| } |  |  |  |
| ul-SyncValidityDuration-r17 | s900 |  | GSO |
|  | S5 |  | NGSO |
| k-Offset-r17 | 258 |  | GSO |
|  | 14 |  | NGSO |
| k-Mac-r17 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| TC_EF_SV | Test cases using ephemeris with State Vector format according to section 8.4.6.2 |
| TC_EF_OP | Test cases using ephemeris with Orbital format according to section 8.4.6.2 |
| GSO | Geosynchronous Orbit scenario |
| NGSO | Non-geosynchronous Orbit scenario |

### 8.4.2A	NB-IoT Generic RRM procedures

The NB-IoT UE test state used for testing is specified in the individual test cases in the corresponding test specification TS 36.521-3 [34].

RRM NB-IoT UE test states are covered by common NB-IoT UE test states defined in clause 8.1.5 of this document.

### 8.4.3	NB-IoT Default RRC message and information elements contents

#### 8.4.3.1	NB-IoT Radio resource control information elements

As defined in clause 8.1.6.3 without exceptions.

### 8.4.4	NB-IoT Default NAS message and information elements contents

As defined in clause 8.1.7 without exceptions.

### 8.4.5	NB-IoT Reference radio bearer configurations

#### 8.4.5.1	NB-IoT SRB and DRB parameters

As defined in clause 8.1.8.2.1 without exception.

### 8.4.6	Test environment for NB-IoT NTN RRM testing

This section defines the test environment which applies to all RRM test cases executed for NB-IoT NTN UEs, unless otherwise specified.

#### 8.4.6.1	UE location

Same as in section 7.6.1.

#### 8.4.6.2	Ephemeris Information

Ephemeris information in this section has been generated as described in clause FFS in TR36.905 [FFS].

#### 8.4.6.2.0	Assumptions for Ephemeris generation

Same as in section 7.6.2.0.

#### 8.4.6.2.1	Ephemeris for elevation angle 30º

All NB-IoT NTN RRM test cases defined in in section 13 in TS 36.521-3 [34], unless otherwise stated, shall use ephemeris configuration in tables 8.4.6.2.1-1a to 8.4.6.2.1-3b, corresponding to an elevation angle of 30º for the type of satellite under test, keeping such information constant throughout the duration of the test:

-	In case UE supports only NGSO satellites or both GSO and NGSO satellites, ephemeris values in Table 8.4.6.2.1-3a or 8.4.6.2.1-3b shall be used.

-	Tables 8.4.6.2-4a to 8.4.6.2-4d shall only be used if the test case under test is only applicable to GEO satellites.

Table 8.4.6.2.1-1a: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (stateVectors format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-1b: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (orbitalParameters format) for GSO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17- |  | 8393544277 |  |  |  |  |  |
| eccentricity-r17 |  | 0 |  |  |  |  |  |
| periapsis-r17 |  | 266532360 |  |  |  |  |  |
| longitude-r17 |  | 224998499 |  |  |  |  |  |
| inclination-r17 |  | 2125399 |  |  |  |  |  |
| meanAnomaly-r17 |  | 118496066 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-2a: Void

Table 8.4.6.2.1-2b: Void

Table 8.4.6.2.1-3a: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (stateVectors format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2935552 |  |  |  |  |  |
| positionY-r17 |  | 4895460 |  |  |  |  |  |
| positionZ-r17 |  | 1188458 |  |  |  |  |  |
| velocityVX-r17 |  | 8506 |  |  |  |  |  |
| velocityVY-r17 |  | -23314 |  |  |  |  |  |
| velocityVZ-r17 |  | 118253 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-3b: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information (orbitalParameters format) for NGSO (LEO-1200) satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |  |  |  |  |
| semiMajorAxis-r17 |  | 273626880 |  |  |  |  |  |
| eccentricity-r17 |  | 769300 |  |  |  |  |  |
| periapsis-r17 |  | 198830479 |  |  |  |  |  |
| longitude-r17 |  | 89834718 |  |  |  |  |  |
| inclination-r17 |  | 65473992 |  |  |  |  |  |
| meanAnomaly-r17 |  | 7822790 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.03 degrees, one-way delay equal to 6.67 ms and Doppler equal to 17.55 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |  |  |  |  |

Table 8.4.6.2.1-4a: SystemInformationBlockType31-NB – NB-IoT NTN Serving cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4b: SystemInformationBlockType31-NB – NB-IoT NTN Serving cell Ephemeris Information orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 200160368 |  |  |
| longitude-r17 | 24831613 |  |  |
| inclination-r17 | 5309231 |  |  |
| meanAnomaly-r17 | 66742914 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.035 degrees, one-way delay equal to 127.33 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4c: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17) – NB-IoT NTN Neighbour cell Ephemeris Information (stateVectors format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| stateVectors SEQUENCE { |  |  |  |
| positionX-r17 | -6457459 |  |  |
| positionY-r17 | 31531426 |  |  |
| positionZ-r17 | 4004156 |  |  |
| velocityVX-r17 | -387 |  |  |
| velocityVY-r17 | -66 |  |  |
| velocityVZ-r17 | -104 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: stateVectors ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

Table 8.4.6.2.1-4d: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17) – NB-IoT NTN Neighbour cell Ephemeris Information (orbitalParameters format) for GEO satellites (serving cell 30º elevation angle conditions)

| Derivation Path: Table 8.4.6.3.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| ephemerisInfo-r18 CHOICE { |  |  |  |
| orbitalParameters SEQUENCE { |  |  |  |
| semiMajorAxis-r17- | 8393544277 |  |  |
| eccentricity-r17 | 0 |  |  |
| periapsis-r17 | 249232221 |  |  |
| longitude-r17 | 228396393 |  |  |
| inclination-r17 | 12328441 |  |  |
| meanAnomaly-r17 | 67231401 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Neighbour cell 1 Satellite-UE elevation angle equal to 32.17 degrees, one-way delay equal to 128.143 ms and Doppler equal to 0 ppm. NOTE 2: orbitalParameters ephemeris format shall be used in IoT NTN RRM test cases listed in Table FFS in TS36.521-3 [34]. |  |  |  |

#### 8.4.6.2.2	Ephemeris for NB-IoT NTN RRM UL timing accuracy test cases

Table 8.4.6.2.2-1: SystemInformationBlockType31-NB – IoT NTN Ephemeris Information for GSO satellites (minimum Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -16944523 |  |  |  |  |  |
| positionY-r17 |  | 27362209 |  |  |  |  |  |
| positionZ-r17 |  | 4019393 |  |  |  |  |  |
| velocityVX-r17 |  | -336 |  |  |  |  |  |
| velocityVY-r17 |  | -208 |  |  |  |  |  |
| velocityVZ-r17 |  | 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |  |  |  |  |

Table 8.4.6.2.2-2: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for NGSO (LEO-1200) satellites (minimum Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2771542 |  |  |  |  |  |
| positionY-r17 |  | 4509243 |  |  |  |  |  |
| positionZ-r17 |  | 2455572 |  |  |  |  |  |
| velocityVX-r17 |  | 20837 |  |  |  |  |  |
| velocityVY-r17 |  | -46176 |  |  |  |  |  |
| velocityVZ-r17 |  | 109528 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm. |  |  |  |  |  |  |  |

Table 8.4.6.2.2-3: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for GSO satellites (maximum positive Doppler)

| Derivation Path: Table clause 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -16989970 |  |  |  |  |  |
| positionY-r17 |  | 27591785 |  |  |  |  |  |
| positionZ-r17 |  | 1413211 |  |  |  |  |  |
| velocityVX-r17 |  | 390 |  |  |  |  |  |
| velocityVY-r17 |  | -65 |  |  |  |  |  |
| velocityVZ-r17 |  | 5945 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to 0.15 ppm |  |  |  |  |  |  |  |

Table 8.4.6.2.2-4: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2654549 |  |  |  |  |  |
| positionY-r17 |  | 4386991 |  |  |  |  |  |
| positionZ-r17 |  | 1594205 |  |  |  |  |  |
| velocityVX-r17 |  | 14581 |  |  |  |  |  |
| velocityVY-r17 |  | -34487 |  |  |  |  |  |
| velocityVZ-r17 |  | 120182 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30.11 degrees, one-way delay equal to 3.60 ms and Doppler equal to 19.83 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle |  |  |  |  |  |  |  |

Table 8.4.6.2.2-5: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for GSO  satellites (maximum negative Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -17129898 |  |  |  |  |  |
| positionY-r17 |  | 27505094 |  |  |  |  |  |
| positionZ-r17 |  | 1413939 |  |  |  |  |  |
| velocityVX-r17 |  | 116 |  |  |  |  |  |
| velocityVY-r17 |  | 378 |  |  |  |  |  |
| velocityVZ-r17 |  | -5944 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 30 degrees, one-way delay equal to 128.77 ms and Doppler equal to -0.15 ppm. NOTE 2: This configuration can be skipped as it is the same as default 30º elevation angle. |  |  |  |  |  |  |  |

Table 8.4.6.2.2-6: SystemInformationBlockType31-NB – IoT NTN Serving cell Ephemeris Information for NGSO (LEO-600)  satellites (maximum negative Doppler)

| Derivation Path: Table 8.4.2.1.3-2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| SystemInformationBlockType31-NB-r17 ::= SEQUENCE { |  |  |  |  |  |  |  |
| servingSatelliteInfo-r17 SEQUENCE { |  |  |  |  |  |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |  |  |  |  |
| stateVectors SEQUENCE { |  |  |  |  |  |  |  |
| positionX-r17 |  | -2401021 |  |  |  |  |  |
| positionY-r17 |  | 3830217 |  |  |  |  |  |
| positionZ-r17 |  | 2907041 |  |  |  |  |  |
| velocityVX-r17 |  | 28879 |  |  |  |  |  |
| velocityVY-r17 |  | -61410 |  |  |  |  |  |
| velocityVZ-r17 |  | 105774 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 149.91 degrees, one-way delay equal to 3.66 ms and Doppler equal to -0-19.95 ppm |  |  |  |  |  |  |  |

#### 8.4.6.3	NB-IoT NTN message contents for RRM tests

#### 8.4.6.3.1	ServingSatelliteInfo-r17 values in SIB31-NB

The ServingSatelliteInfo-r17 values including the EphemerisInfo-r17 values in SIB31-NB shall be periodically signalled to the UE during test based on the Ephemeris information in clause 8.4.6.2 (kept constant throughout the duration of the test).

#### 8.4.6.3.2	SystemInformationBlockType33-NB

Table 8.4.6.3.2-1: SystemInformationBlockType33-NB (Rel-18)/Test equipment configuration (Rel-17)

| Derivation Path: TS 36.331 [17], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType33-NB-r18 ::= SEQUENCE { |  |  |  |
| neighSatelliteInfoList-r18 SEQUENCE (SIZE(1..maxSat-r18)) OF NeighSatelliteInfo-r18 { | 1 entry for test cases requiring one neighbour cell |  | One neighbour cell required |
| NeighSatelliteInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| satelliteId-r18 | 1 |  |  |
| ephemerisInfo-r18 | See cl. 8.4.6.2 |  |  |
| nta-CommonParameters SEQUENCE { |  |  |  |
| nta-Common-r18 | 0 |  |  |
| nta-CommonDrift-r18 | 0 |  |  |
| nta-CommonDriftVariation-r18 | 0 |  |  |
| } |  |  |  |
| epochTime-r18 | Not present |  |  |
| k-Mac-r18 | Not present |  |  |
| t-ServiceStartNeigh-r18 | Not present |  |  |
| } |  |  |  |
| neighValidityDuration-r18 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |
| NOTE 1: In Rel-17, SIB33 is not signalled to the device but information in this table can be used to configure test equipment. |  |  |  |