| 3GPP TS 23.281 V18.2.0 (2022-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	10

1	Scope	11

2	References	11

3	Definitions, symbols and abbreviations	12

3.1	Definitions	12

3.2	Symbols	13

3.3	Abbreviations	13

4	Introduction	13

5	Architectural requirements	13

5.1	Media routing requirements	13

5.2	MCVideo group affiliation and MCVideo group de-affiliation	14

5.3	Device inventory requirements	14

5.4	Device discovery requirements (off-network)	14

5.5	Bearer management	14

5.5.1	General	14

5.5.2	EPS bearer considerations	14

5.5.3	EPS unicast bearer considerations for MCVideo	14

5.5.4	MBMS bearer management	15

5A	Involved business relationships	15

6	MCVideo Functional model	15

6.1	Functional model description	15

6.1.1	On-network functional model	15

6.1.2	Off-network functional model	16

6.2	Functional entities description	16

6.2.1	General	16

6.2.2	MCVideo service application plane	16

6.2.2.1	General	16

6.2.2.2	Common services core	16

6.2.2.3	MCVideo application service	16

6.2.2.3.1	MCVideo client	16

6.2.2.3.2	MCVideo server	17

6.2.2.3.3	Media distribution function	17

6.2.2.3.4	Media mixer	17

6.2.2.3.5	MCVideo user database	18

6.2.2.3.6	Transmission control server	18

6.2.2.3.7	Transmission control participant	18

6.2.2.3.8	MC gateway server	18

6.3	Reference points	18

6.3.1	Reference point MCVideo-1 (between the MCVideo client and the MCVideo server)	18

6.3.2	Reference point MCVideo-2 (between the MCVideo server and the MCVideo user database)	18

6.3.3	Reference point MCVideo-3 (between the MCVideo server and the MCVideo server and between the MCVideo server and the MC gateway server)	19

6.3.4	Reference point MCVideo-4 (between the transmission control participant and the transmission control server)	19

6.3.4A	Reference point MCVideo-5 (between the MCVideo server and the EPS)	19

6.3.4B	Reference point MCVideo-6 (between the MCVideo server and the EPS)	19

6.3.5	Reference point MCVideo-7 (between the media distribution function and the media mixer)	19

6.3.6	Reference point MCVideo-8 (between the media distribution function and the media mixer)	19

6.3.7	Reference point MCVideo-9 (between the transmission control participant and the transmission control server)	20

6.3.8	Reference point MCVideo-10 (between the MC gateway server and the MC gateway server in a different MCVideo system)	20

6A	Identities	20

6B	Application of functional model to deployments	20

7	Procedures and information flows	20

7.1	Group call	20

7.1.1	General	20

7.1.2	On-network group call	20

7.1.2.1	General	20

7.1.2.2	Information flows for group call in on-network	20

7.1.2.2.1	Group call request (MCVideo client – MCVideo server)	20

7.1.2.2.2	Group call request	21

7.1.2.2.3	Group call response	21

7.1.2.2.4	Group call response (MCVideo client – MCVideo server)	22

7.1.2.2.5	Group call release request	22

7.1.2.2.6	Group call release request (MCVideo client – MCVideo server)	22

7.1.2.2.7	Group call release response	22

7.1.2.2.8	Group call rejoin request	23

7.1.2.2.9	Group call rejoin response	23

7.1.2.2.10	Group join request	23

7.1.2.2.11	Group join response	24

7.1.2.2.12	Group call leave request	24

7.1.2.2.12a	Group call leave request	24

7.1.2.2.13	Group call leave response	24

7.1.2.2.13a	Group call leave response	25

7.1.2.2.14	Void	25

7.1.2.2.15	Void	25

7.1.2.2.16	Void	25

7.1.2.2.17	Void	25

7.1.2.2.18	MCVideo emergency group call request	25

7.1.2.2.19	MCVideo emergency group call response	26

7.1.2.2.20	MCVideo in-progress emergency group state cancel request	26

7.1.2.2.21	MCVideo in-progress emergency group state cancel response	27

7.1.2.2.22	MCVideo imminent peril group call request	27

7.1.2.2.23	MCVideo imminent peril group call response	27

7.1.2.2.24	MCVideo imminent peril group call cancel request	27

7.1.2.2.25	MCVideo imminent peril group call cancel response	28

7.1.2.3	Group call within one MC system	28

7.1.2.3.1	Group call models	28

7.1.2.3.1.1	Pre-arranged group call	28

7.1.2.3.1.2	Chat group call	33

7.1.2.3.2	Exiting group call due to de-affiliation	39

7.1.2.3.3	MCVideo user leaving a group call	40

7.1.2.4	Broadcast group call	41

7.1.2.4.1	General	41

7.1.2.4.2	Common broadcast group call procedure	41

7.1.2.5	Emergency and imminent peril procedures	41

7.1.2.5.1	MCVideo emergency group call	41

7.1.2.5.1.1	MCVideo emergency group call commencement	41

7.1.2.5.1.2	MCVideo group call upgraded to an MCVideo emergency group call	44

7.1.2.5.1.3	MCVideo in-progress emergency group state cancel	46

7.1.2.5.2	MCVideo imminent peril group call	48

7.1.2.5.2.1	MCVideo imminent peril group call commencement	48

7.1.2.5.2.2	Imminent peril group call upgrade	50

7.1.2.5.2.3	MCVideo imminent peril group call cancel	52

7.1.2.6	MCVideo emergency alert (on-network and off-network)	53

7.1.3	Off-network group communications	53

7.1.3.1	General	53

7.1.3.2	Information flows for off-network group communications	53

7.1.3.2.1	Group communication announcement	53

7.1.3.2.2	Group communication answer response	54

7.1.3.2.3	MCVideo upgrade to emergency group communication	54

7.1.3.2.4	MCVideo emergency group communication cancel	55

7.1.3.2.5	MCVideo upgrade to imminent peril group communication	55

7.1.3.2.6	MCVideo imminent peril group communication cancel	55

7.1.3.2.7	MCVideo emergency alert announcement	55

7.1.3.2.8	MCVideo emergency alert cancel announcement	56

7.1.3.3	Group communication setup	56

7.1.3.3.1	General	56

7.1.3.3.2	Procedure	56

7.1.3.4	Passive join to group communication	58

7.1.3.4.1	General	58

7.1.3.4.2	Procedure	58

7.1.3.5	Active join to group communication	59

7.1.3.5.1	General	59

7.1.3.5.2	Procedure	59

7.1.3.6	Broadcast group communication	60

7.1.3.7	Group communication release due to inactivity	61

7.1.3.8	Emergency and imminent peril procedures	61

7.1.3.8.1	Emergency group communication	61

7.1.3.8.2	MCVideo imminent peril	62

7.1.3.9	MCVideo emergency alert	63

7.1.3.9.1	General	63

7.2	Private call	63

7.2.1	General	63

7.2.2	Private call on-network	63

7.2.2.1	General	63

7.2.2.2	Information flows for private call in on-network	63

7.2.2.2.1	MCVideo private call request (MCVideo client – MCVideo server)	63

7.2.2.2.2	MCVideo private call request	64

7.2.2.2.3	MCVideo private call response (MCVideo client – MCVideo server)	65

7.2.2.2.4	MCVideo private call response	65

7.2.2.2.5	MCVideo call end request	66

7.2.2.2.6	MCVideo emergency private call request (MCVideo client to MCVideo server)	66

7.2.2.2.7	MCVideo emergency private call request	67

7.2.2.2.8	Emergency MCVideo private call response (MCVideo client – MCVideo server)	67

7.2.2.2.9	Emergency MCVideo private call response	67

7.2.2.2.10	MCVideo progress indication	68

7.2.2.2.11	MCVideo functional alias resolution response	68

7.2.2.3	Private call within one MC system	68

7.2.2.3.1	Private call setup in automatic commencement mode	68

7.2.2.3.2	Private call setup in manual commencement mode	70

7.2.2.3.2.1	Description	70

7.2.2.3.2.2	Procedure	70

7.2.2.3.3	Private call release	72

7.2.2.3.3.1	Client initiated	72

7.2.2.3.3.2	Server initiated	73

7.2.2.4	MCVideo emergency private call	74

7.2.2.4.1	MCVideo emergency private call commencement	74

7.2.2.4.2	MCVideo private call emergency upgrade	76

7.2.3	Off-network private communications	77

7.2.3.1	General	77

7.2.3.2	Information flows for off-network private communications	77

7.2.3.2.1	Private communication request	77

7.2.3.2.2	Private communication answer response	78

7.2.3.2.3	Private communication release request	78

7.2.3.2.4	Private communication release response	78

7.2.3.3	Use of ProSe for off-network private communications	79

7.2.3.4	Automatic commencement private communication	79

7.2.3.4.1	General	79

7.2.3.4.2	Procedure	79

7.2.3.5	Manual commencement private communication	81

7.2.3.5.1	General	81

7.2.3.5.2	Procedure – Communication accepted	81

7.2.3.5.3	Procedure – Communication rejected/ignored	83

7.2.3.6	Private communication release	84

7.2.3.6.1	General	84

7.2.3.6.2	Procedure	84

7.3	Video pull	85

7.3.1	General	85

7.3.2	On-network video pull	85

7.3.2.1	General	85

7.3.2.2	Information flows for on-network video pull	85

7.3.2.2.1	MCVideo pull from server request	86

7.3.2.2.2	MCVideo pull from server response	86

7.3.2.2.3	MCVideo pull from server complete request	86

7.3.2.2.4	MCVideo pull from server complete response	86

7.3.2.3	One-to-one video pull	87

7.3.2.3.1	General	87

7.3.2.3.2	One-to-one video pull – call setup	87

7.3.2.3.3	One-to-one video pull – call release	88

7.3.2.4	One-from-server video pull	88

7.3.2.4.1	General	88

7.3.2.4.2	Procedure	88

7.3.3	Off-network video pull	89

7.3.3.1	General	89

7.3.3.2	Information flows for off-network video pull	90

7.3.3.3	Video pull to self	90

7.3.3.3.1	General	90

7.3.3.3.2	Procedure	90

7.4	Video push	92

7.4.1	General	92

7.4.2	On-network video push	92

7.4.2.1	General	92

7.4.2.2	Information flows for on-network video push	92

7.4.2.2.1	Remote video push request	92

7.4.2.2.2	Remote video push response	93

7.4.2.2.3	Remote video push release request	93

7.4.2.2.4	MCVideo push to server request	94

7.4.2.2.5	MCVideo push to server response	94

7.4.2.2.6	MCVideo push to server complete request	94

7.4.2.2.7	MCVideo push to server complete response	95

7.4.2.3	One-to-one video push	95

7.4.2.3.1	General	95

7.4.2.3.2	One-to-one video push – call setup	95

7.4.2.3.3	One-to-one video push – call release	96

7.4.2.4	One-to-server video push	96

7.4.2.4.1	General	96

7.4.2.4.2	Procedure	96

7.4.2.5	Remotely initiated video push	98

7.4.2.5.1	General	98

7.4.2.5.2	Remotely initiated video push – call setup	98

7.4.2.5.3	Remotely initiated video push – call release by authorized user	99

7.4.2.6	Remotely initiated video push to group	100

7.4.2.6.1	General	100

7.4.2.6.2	Remotely initiated video push to group – call setup	100

7.4.2.6.3	Remotely initiated video push to group – call release by authorized user	101

7.4.3	Off-network video push	102

7.4.3.1	General	102

7.4.3.2	Information flows for off-network video push	102

7.4.3.2.1	Remote video push request	102

7.4.3.2.2	Video push trying response	102

7.4.3.2.3	Notification of video push	102

7.4.3.3	Video push to another MCVideo user	103

7.4.3.3.1	General	103

7.4.3.3.2	Procedure	103

7.4.3.4	Remotely initiated video push	104

7.4.3.4.1	General	104

7.4.3.4.2	Procedure	104

7.4.3.5	Remotely initiated video push to a group	106

7.4.3.5.1	General	106

7.4.3.5.2	Procedure	106

7.5	Capability information sharing	107

7.5.1	General	107

7.5.2	On-network capability information sharing	107

7.5.2.1	General	107

7.5.2.2	Information flows for on-network capability information sharing	107

7.5.2.2.1	Update MCVideo capabilities info request	107

7.5.2.2.2	Update MCVideo capabilities info response	108

7.5.2.2.3	Get MCVideo capabilities info request	108

7.5.2.2.4	Get MCVideo capabilities info response	108

7.5.2.2.5	Subscribe MCVideo capabilities info request	108

7.5.2.2.6	Subscribe MCVideo capabilities info response	109

7.5.2.2.7	Notify MCVideo capabilities info request	109

7.5.2.2.8	Notify MCVideo capabilities info response	109

7.5.2.3	Update MCVideo capabilities information at the MCVideo server	109

7.5.2.4	Retrieve MCVideo capabilities information by the MCVideo client	110

7.5.2.5	Subscription and notification for MCVideo capabilities information	110

7.5.3	Off-network capability information sharing	111

7.5.3.1	General	111

7.5.3.2	Information flows for Off-network capability information sharing	112

7.5.3.2.1	Capability request	112

7.5.3.2.2	Capability announcement	112

7.5.3.2.3	Activity status request	112

7.5.3.2.4	Activity status announcement	113

7.5.3.3	Periodic capability announcements	113

7.5.3.3.1	General	113

7.5.3.3.2	Procedure	113

7.5.3.4	Request capabilities from client(s)	114

7.5.3.4.1	General	114

7.5.3.4.2	Request clients with particular capabilities	114

7.5.3.4.3	Request capabilities from a particular client	115

7.5.3.5	Request activity status from client(s)	116

7.5.3.5.1	General	116

7.5.3.5.2	Request activity status of group members	116

7.5.3.5.3	Request activity status from a particular client	117

7.6	Ambient viewing call	118

7.6.1	General	118

7.6.2	Information flows for ambient viewing call	118

7.6.2.1	Ambient viewing call request	118

7.6.2.2	Ambient viewing call response	119

7.6.2.3	Ambient viewing call release request	119

7.6.2.4	Ambient viewing call release response	119

7.6.2.5	Ambient viewing call release notification	120

7.6.3	Procedures	120

7.6.3.1	Remotely initiated ambient viewing call setup procedure	120

7.6.3.2	Locally initiated ambient viewing call setup procedure	121

7.6.3.3	Ambient viewing call release – server initiated	122

7.6.3.4	Ambient viewing call release – "viewing" user initiated	123

7.6.3.5	Ambient viewing call release – "viewed to" user initiated	124

7.7	Transmission control	125

7.7.1	Transmission control for on-network MCVideo service	125

7.7.1.1	General	125

7.7.1.2	Information flows for transmission control for on-network	125

7.7.1.2.1	General	125

7.7.1.2.2	Transmit media request	125

7.7.1.2.3	Transmit media granted	126

7.7.1.2.4	Transmit media rejected	126

7.7.1.2.5	Media transmission notification	126

7.7.1.2.6	Receive media request	126

7.7.1.2.7	Receive media response	127

7.7.1.2.8	Media reception notification	127

7.7.1.2.9	Queue position info	127

7.7.1.2.10	Transmission revoked	128

7.7.1.2.11	Queue position request	128

7.7.1.2.12	Transmit media cancel request	128

7.7.1.2.13	Transmit media cancel response	128

7.7.1.2.14	Transmit media cancel request notify	129

7.7.1.2.15	Transmit media end request	129

7.7.1.2.16	Transmit media end response	129

7.7.1.2.17	Remote transmit media request	129

7.7.1.2.18	Remote transmit media response	130

7.7.1.2.19	Remote transmit media end request	130

7.7.1.2.20	Remote transmit media cancel response	130

7.7.1.2.21	Media reception end request	130

7.7.1.2.22	Media reception end response	131

7.7.1.2.23	Media reception override notification	131

7.7.1.2.24	Transmit media end notify	131

7.7.1.3	Transmission control within one MC system for MCVideo service	132

7.7.1.3.1	Transmission control during an MCVideo session	132

7.7.1.3.2	Reception control during an MCVideo session	133

7.7.1.3.2A	End media reception – receiving user initiated	134

7.7.1.3.2B	End media reception – transmission control server initiated	135

7.7.1.3.2C	Reception control on overridden – mandatory mode	135

7.7.1.3.2D	Reception control on overridden – negotiated mode	136

7.7.1.3.3	Transmission revoke during an MCVideo session	137

7.7.1.3.4	Queue position during an MCVideo session	138

7.7.1.3.5	Transmit media request cancellation from the video transmission queue	139

7.7.1.3.5.1	Transmit media request cancellation from the queue – MCVideo user initiated	139

7.7.1.3.5.2	Transmit media request cancellation from the queue - transmission control server initiated	140

7.7.1.3.6	End a media transmission during an MCVideo session	141

7.7.1.3.6.1	End a media transmission – MCVideo user initiated	141

7.7.1.3.6.2	End a media transmission – transmission control server initiated	142

7.7.1.3.6.3	End a media transmission – remote MCVideo user initiated	143

7.7.1.3.7	Remotely initiated media transmission during an MCVideo session	144

7.7.2	Off-network transmission control	145

7.7.2.1	General	145

7.7.2.2	Information flows for off-network transmission control	145

7.7.2.2.1	Transmission request	145

7.7.2.2.2	Transmission granted	146

7.7.2.2.3	Transmission release	146

7.7.2.2.4	Transmission rejected	146

7.7.2.2.5	Transmission revoked	147

7.7.2.2.6	Transmission arbitration taken	147

7.7.2.2.7	Transmission arbitration release	147

7.7.2.3	Initializing transmission control – single arbitrator approach	148

7.7.2.3A	Initializing transmission control – self arbitration approach	149

7.7.2.4	Transmission permission granted	149

7.7.2.5	Transmission permission rejected	150

7.7.2.6	Releasing transmission permission	151

7.7.2.7	Transmission override	152

7.7.2.8	Transmission override (revoke self)	153

7.7.2.9	Transmission arbitration release	155

7.7.2.9.1	Transmission arbitration release	155

7.7.2.9.2	Transmission arbitration release with delegation	155

7.7.2.10	Simultaneous transmission requests	156

7.8	MCVideo service configuration	157

7.9	Affiliation and de-affiliation to/from MCVideo group(s)	157

7.9a	Functional alias management	158

7.10	Use of MBMS transmission (on-network)	158

7.10.1	Information flows for MBMS Transmission	158

7.10.1.1	General	158

7.10.1.2	MapGroupToBearer	158

7.10.1.3	UnmapGroupFromBearer	159

7.10.2	Use of pre-established MBMS bearers	159

7.10.3	Use of dynamic MBMS bearer establishment	159

7.10.4	Video call connect and disconnect over MBMS	160

7.10.4.1	General	160

7.10.4.2	Procedure	160

7.10.4.2.1	Call connect over MBMS	160

7.10.4.2.2	Call disconnect over MBMS	161

7.10.5	 Switching from MBMS bearer to unicast bearer	162

7.11	Simultaneous session for MCVideo calls (on-network)	162

7.12	User authentication and authorization for MCVideo service	162

7.13	Support for multiple devices	163

7.14	Location information (on-network)	163

7.15	MCVideo resource management (on-network)	163

7.16	MCVideo client query	164

7.16.1	General	164

7.16.2	Information flows for MCVideo client query	164

7.16.2.1	MCVideo client query request	164

7.16.2.2	MCVideo client query response	164

7.16.3	MCVideo client query procedure	164

7.17	MCVideo adaptation during MCVideo communication	165

7.17.1	General	165

7.17.2	Information flows for MCVideo adaptation	165

7.17.2.1	MCVideo communication parameter update request	165

7.17.2.2	MCVideo communication parameter update response	166

7.17.3	MCVideo adaptation procedure	166

7.17.3.1	MCVideo adaptation during group communication procedure – transmitting client triggered	166

7.17.3.2	MCVideo adaptation during group communication procedure – receiving user triggered	167

7.17.3.3	MCVideo adaptation during private communication procedure – transmitting client triggered	168

7.17.3.4	MCVideo adaptation during group communication procedure – receiving user triggered	169

7.18	Use of ProSe capabilities in off-network MCVideo communications	170

7.18.1	General	170

7.18.2	Procedures	171

Annex A (normative): MCVideo related configuration data	172

A.1	General	172

A.2	MCVideo UE configuration data	172

A.3	MCVideo user profile configuration data	173

A.4	MCVideo related Group configuration data	182

A.5	MCVideo service configuration data	184

Annex B (informative): Change history	186

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

This document specifies the functional model, procedures and information flows needed for the mission critical video (MCVideo) service. Support for both MCVideo group streaming and MCVideo private streaming operating in on-network and off-network modes of operation is specified.

The corresponding service requirements are defined in 3GPP TS 22.280 [2], and 3GPP TS 22.281 [3].

The present document is applicable primarily to mission critical video service using E-UTRAN access based on the common functional architecture for mission critical services defined in 3GPP TS 23.280 [6] and the EPC architecture defined in 3GPP TS 23.401 [8].

The MCVideo service can be used for public safety applications and also for general commercial applications e.g. utility companies and railways.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.280: "Mission Critical Common Requirements (MCCoRe); Stage 1".

[3]	3GPP TS 22.281: "Mission Critical Video services".

[4]	3GPP TS 23.002: "Network Architecture".

[5]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[7]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[9]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[10]	Void

[11]	3GPP TS 23.203: "Policy and charging control architecture".

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 reference point; Stage 3".

[14]	3GPP TS 33.180: "Security of the mission critical service".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MCVideo client: An instance of an MC service client that provides the client application function for the MCVideo service.

MCVideo group: An MC service group configured for MCVideo service.

MCVideo group affiliation: An MC service group affiliation for MCVideo.

MCVideo group de-affiliation: An MC service group de-affiliation for MCVideo.

MCVideo group home system: The mission critical system where the MCVideo group is defined.

MCVideo group host server: The MCVideo server within a mission critical system which provides centralised support for MCVideo services of an MCVideo group defined in a MCVideo group home system.

MCVideo ID: An instance of an MC service ID within the MCVideo service.

MCVideo server: An instance of an MC service server that provides the server application function for the MCVideo service.

MCVideo service: A video communication service supporting applications for mission critical organizations and mission critical applications for other businesses and organizations (e.g., utilities, railways) with strong security, high availability, reliability and priority handling.

MCVideo system: The collection of applications, services, and enabling capabilities required to provide Mission Critical video for a Mission Critical Organization.

MCVideo UE: An MC service UE that can be used to participate in MCVideo services.

MCVideo user: An MC service user who is authorized for MCVideo services via an MCVideo UE.

Transmission control: Video transmitting control mechanism in an MCVideo service that determines which participants have the authority to transmit video, and determines the onward downlink video transmission during an video call.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.280 [2] apply:

Mission Critical

Mission Critical Applications

Mission Critical Service

Mission Critical Organization

Mission Critical System

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.281 [3] apply:

Real Time

Real Time Video

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.280 [6] apply:

MC service client

MC service group

MC service group affiliation

MC service group de-affiliation

MC service group home system

MC service group host

MC service ID

MC service server

## 3.2	Symbols

For the purposes of the present document, the following symbols given in 3GPP TS 22.280 [2] apply:

Nc2

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

E-UTRAN			Evolved Universal Terrestrial Radio Access Network

EPC			Evolved Packet Core

GCS AS			Group Communication Service Application Server

GCSE_LTE			Group Communication Service Enabler over LTE

HTTP			Hyper Text Transfer Protocol

IMS			IP Multimedia Subsystem

MC			Mission Critical

MCVideo			Mission Critical Video

MCVideo group ID		MCVideo group identity

MCVideo ID			MCVideo user identity

ProSe			Proximity-based Services

SIP			Session Initiated Protocol

# 4	Introduction

The MCVideo service supports video media communication between several users (i.e. group call), where each user has the ability to gain access to the permission to stream video in an arbitrated manner. The MCVideo service also supports private calls between two users.

The MCVideo architecture is based on the functional architecture for mission critical communication services defined in 3GPP TS 23.280 [6].

# 5	Architectural requirements

## 5.1	Media routing requirements

The video media flow for a private call shall be routed according to one of the following two options:

a)	Option 1:

1)	Through the primary MCVideo system if both users in the call belong to the same organisation; or

2)	Through the primary MCVideo system of each users, if the users in the call do not belong to the same organisation.

b)	Option 2: The video media flow may be routed locally, under the control of the primary MCVideo system, through an entity allowing the duplication of the media flow to the primary MCVideo system of each user.

The video media flow for a group call shall be routed to the group home MCVideo system.

## 5.2	MCVideo group affiliation and MCVideo group de-affiliation

MCVideo group affiliation shall be as specified in clause 5.2.5 of 3GPP TS 23.280 [6]. In addition, the following requirements shall be fulfilled by the MCVideo service for MCVideo users affiliated to MCVideo groups:

-	MCVideo users receive notifications for MCVideo group call setup and invitations for their affiliated MCVideo group(s).

-	MCVideo users select an affiliated MCVideo group to initiate a new group MCVideo call or transmit media in an existing MCVideo group call.

-	MCVideo users receive media and events from their affiliated MCVideo group(s).

## 5.3	Device inventory requirements

The MCVideo service shall provide device inventory capabilities for MCVideo UEs. The device inventory capabilities shall include:

-	device information registration;

-	device information storage; and

-	device information query.

## 5.4	Device discovery requirements (off-network)

The MCVideo service shall provide device discovery for devices as requested by the MCVideo user according to the MC service provider policy for off-network operations.

## 5.5	Bearer management

### 5.5.1	General

The MCVideo UE shall use the APNs as defined in subclause 5.2.7.0 of 3GPP TS 23.280 [6]. The MCVideo UE shall use the MC services APN as defined in subclause 5.2.7.0 of 3GPP TS 23.280 [6] for the SIP-1 reference point.

### 5.5.2	EPS bearer considerations

The EPS bearer considerations specified in subclause 5.2.7.2 of 3GPP TS 23.280 [6] shall apply.

### 5.5.3	EPS unicast bearer considerations for MCVideo

For an MCVideo call session request, resources shall be requested utilising interaction with dynamic PCC. The MCVideo system shall request resources over Rx to a PCRF. It is recommended that the dedicated bearer(s) for video media and control of the video media (i.e. MCVideo-4 and MCVideo-7) utilise the QCI values depending on the MCVideo mode of the MCVideo call/session, as per table 5.5.3-1. For transmission and reception control signalling, the QCI value of 69 is recommended as specified in 3GPP TS 23.203 [11]. The request of resources over Rx shall include an application identifier for MCVideo in order for the PCRF to evaluate the correct QCI.

Table 5.5.3-1: QCI values to use for EPS unicast bearers for each MCVideo mode

| MCVideo mode | QCI value utilised (as specified in 3GPP TS 23.203 [11]) |
| --- | --- |
| Urgent real-time mode | 67 |
| Non-urgent real-time mode | 67 |
| Non real-time mode | 4 |

The MCVideo audio media and video media may transmit over separate EPS bearers, in which case the priority for each bearer is determined by the operator policy. Depending on operator policy, the MCVideo system may be able to request modification of the priority (ARP) of an existing bearer without the need to initiate a new dedicated GBR bearer.

NOTE:	Operator policy takes into account regional/national requirements.

When MCVideo audio media and video media are multiplexed into one EPS bearer, then different QCI and/or ARP values are not possible for video and audio media.

### 5.5.4	MBMS bearer management

The MBMS bearer management for MC services is specified in subclause 5.2.7.1 of 3GPP TS 23.280 [6].

# 5A	Involved business relationships

The description of the involved business relationships for the MCVideo service is contained in clause 6 of 3GPP TS 23.280 [6].

# 6	MCVideo Functional model

## 6.1	Functional model description

### 6.1.1	On-network functional model

Figure 6.1.1-1 shows the functional model for the application plane of MCVideo service for on-network operations.

Figure 6.1.1-1: Functional model for application plane of MCVideo service

In the model shown in figure 6.1.1-1, the following apply:

-	The MCVideo server is an instantiation of a MC service server in accordance with 3GPP TS 23.280 [6].

-	The MCVideo server is an instantiation of a GCS AS in accordance with 3GPP TS 23.468 [9].

-	MCVideo-9 carries signalling over multicast bearer between the transmission control server of the MCVideo server and the transmission control participant of the MCVideo UE.

-	MCVideo-4 carries signalling over unicast bearer between the transmission control server of the MCVideo server and the transmission control participant of the MCVideo UE.

### 6.1.2	Off-network functional model

Figure 6.1.2-1 shows the functional model for the application plane of MCVideo service for off-network operations.

Figure 6.1.2-1: Functional model for application plane of off-network MCVideo service

## 6.2	Functional entities description

### 6.2.1	General

Each subclause is a description of a functional entity and does not imply a physical entity.

### 6.2.2	MCVideo service application plane

#### 6.2.2.1	General

Entities within the application plane of MCVideo service provide application control, media transmission control and distribution functions.

#### 6.2.2.2	Common services core

The description of the common services core entities are contained in common functional architecture for MC services over LTE in 3GPP TS 23.280 [6].

#### 6.2.2.3	MCVideo application service

##### 6.2.2.3.1	MCVideo client

The MCVideo client functional entity acts as the user agent for all MCVideo application transactions.

The MCVideo client is responsible for remote device control. This functional entity is located in the UE for both on-network and off-network operations.

##### 6.2.2.3.2	MCVideo server

The MCVideo server functional entity provides centralised support for MCVideo services.

All the MCVideo clients supporting users belonging to a single group are required to use the same MCVideo server for that group. An MCVideo client supporting a user involved in multiple groups can have relationships with multiple MCVideo servers.

The MCVideo server is an instantiation of a MC service server in accordance with 3GPP TS 23.280 [6].

The MCVideo server functional entity represents a specific instantiation of the GCS AS described in 3GPP TS 23.468 [9] to control multicast and unicast operations for group communications.

The MCVideo server functional entity is supported by the SIP AS, HTTP client and HTTP server functional entities of the signalling control plane.

This MCVideo server provides support for centralised media transmission control for on-network and distributed media transmission control for off-network operation.

The MCVideo server is responsible for managing and providing the device information that can participate in MCVideo communications. The device information is further associated to MCVideo users to manage remote device control authorization. The device information is provisioned to the MCVideo server by the MCVideo service provider, mission critical organization and the MCVideo user.

The MCVideo server performing the functional alias controlling role is responsible for:

-	interfacing with the functional alias management server for functional alias policy;

-	functional alias activation, deactivation, take over and interrogation support for MCVideo user.

The controlling roles for group call, private call and functional alias are independent with each other.

##### 6.2.2.3.3	Media distribution function

The media distribution function is responsible for the distribution of media to MCVideo clients. By means of information provided by the MCVideo server (e.g. IP addresses, transport layer ports), it will provide the following functionality:

-	provide for the reception of uplink MCVideo UE media transmission by means of the MCVideo-7 reference point;

-	storing the received media stream as MCVideo content files;

-	replicate the media as needed for distribution to those participants using unicast transport;

-	distribute downlink media to MCVideo UEs by IP unicast transmission to those participants utilizing unicast transport by means of the MCVideo-7 reference point;

-	distribute downlink media to MCVideo UEs using multicast downlink transport of media for the call by means of the MCVideo-8 reference point; and

-	provide a media mixing function where multiple media streams are combined into a single media stream for transmission to the MCVideo UE.

NOTE 1:	If media mixing function occurs within the media distribution function, it operates independently of the media mixer in the UE.

NOTE 2:	A media mixing function within the media distribution function is not possible where the media is end to end encrypted.

##### 6.2.2.3.4	Media mixer

This functional entity exists on the UE and provides support for sending and receiving one or multiple media streams. It also provides support for combining multiple media streams into one media stream through the enforcement of media policy information. It supports the storing of a media stream as MCVideo content files.

##### 6.2.2.3.5	MCVideo user database

This functional entity contains information of the MCVideo user profile associated with an MCVideo ID that is held by the MCVideo service provider at the application plane. The MCVideo user profile is determined by the mission critical organization, the MCVideo service provider, and potentially the MCVideo user.

The MCVideo ID is a MC service ID as described in clause 8 of 3GPP TS 23.280 [6]. The MCVideo user profile is a MC service user profile as described in clause 10.1.4 of 3GPP TS 23.280 [6].

The MCVideo user profile can be co-located with other MC service user profiles and stored in a common MC service user database.

##### 6.2.2.3.6	Transmission control server

This functional entity provides support for centralised transmission control for on-network and distributed transmission control for off-network operation. It may schedule transmission requests according to uplink criteria from different transmission control participants, send an notification to all transmission control participants to allow them to receive the video according to downlink criteria if the transmission request is granted, and provide queuing in cases of contention. Transmission control applies to all MCVideo communications including group call and private call. For on-network operation, this functional entity is located with the MCVideo server. For off-network operation, this functional entity is located in the UE.

##### 6.2.2.3.7	Transmission control participant

The transmission control participant functional entity is responsible for handling outgoing transmission requests and the incoming video stream invitations and notifications. This functional entity is located in the UE for both on-network and off-network operations.

##### 6.2.2.3.8	MC gateway server

The MC gateway server provides support for MCVideo interconnection services with a partner MCVideo system in a different trust domain whilst providing topology hiding. It acts as a proxy for one or more MCVideo servers in the partner MCVideo system without needing to expose the MCVideo servers in the primary MCVideo system outside the trusted domain of the primary MCVideo system. It may be a role of the MCVideo server described in subclause 6.2.2.3.2 of the present document.

The MC gateway server is responsible for relaying call control and floor control signalling messages, and media between MCVideo servers within the MCVideo system and the interconnected MCVideo system.

Editor's note:	It is FFS whether the gateway MC server can act as a signalling proxy as defined in 3GPP TS 33.180 [14]

## 6.3	Reference points

### 6.3.1	Reference point MCVideo-1 (between the MCVideo client and the MCVideo server)

The MCVideo-1 reference point, which exists between the MCVideo client and the MCVideo server, is used for MCVideo application signalling for establishing a session in support of MCVideo service.

### 6.3.2	Reference point MCVideo-2 (between the MCVideo server and the MCVideo user database)

The MCVideo-2 reference point, which exists between the MCVideo server and the MCVideo user database, is used by the MCVideo server to obtain information about a specific user.

### 6.3.3	Reference point MCVideo-3 (between the MCVideo server and the MCVideo server and between the MCVideo server and the MC gateway server)

The MCVideo-3 reference point exists between the MCVideo server and another MCVideo server and between the MCVideo server and the MC gateway server, and is used for MCVideo application signalling.

### 6.3.4	Reference point MCVideo-4 (between the transmission control participant and the transmission control server)

The MCVideo-4 reference point, which exists between the transmission control participant and the transmission control server, is used for MCVideo transmission control signalling over unicast. The MCVideo-4 reference point uses the SGi reference point defined in 3GPP TS 23.002 [4].

### 6.3.4A	Reference point MCVideo-5 (between the MCVideo server and the EPS)

The MCVideo-5 reference point, which exists between the MCVideo and the EPS, is used, subject to the conditions below, by the MCVideo server to obtain unicast bearers with appropriate QoS from the EPS. It utilises the Rx interface of the EPS according to 3GPP TS 23.203 [11].

MCVideo-5 is not used when the MCVideo service provider and the PLMN operator do not have an operational agreement for QoS control to be provided directly from the MCVideo service provider domain.

MCVideo-5 may be used when the MCVideo service provider and the PLMN operator have an operational agreement where QoS control is provided directly from the MCVideo service provider domain.

NOTE:	Any coordination between the P-CSCF use of Rx and the MCVideo server use of Rx (via MCVideo-5) from the MCVideo service provider domain is not specified in this release of this specification.

### 6.3.4B	Reference point MCVideo-6 (between the MCVideo server and the EPS)

The MCVideo-6 reference point, which exists between the MCVideo server and the EPS, is used to request the allocation and activation of multicast transport resources for MCVideo application usage. The MCVideo-6 reference point uses the MB2-C interface as defined in 3GPP TS 29.468 [13].

### 6.3.5	Reference point MCVideo-7 (between the media distribution function and the media mixer)

The MCVideo-7 reference point, which exists between the media distribution function and the media mixer, is used to exchange unicast media between the media distribution function of the MCVideo server and the media mixer of the MCVideo client. The MCVideo-7 reference point uses the SGi reference point defined in 3GPP TS 23.002 [4].

### 6.3.6	Reference point MCVideo-8 (between the media distribution function and the media mixer)

The MCVideo-8 reference point, which exists between the media distribution function and the media mixer, is used by the media distribution function of the MCVideo server to send multicast media to the media mixer of the MCVideo client. The MCVideo-8 reference point uses the MB2-U interface defined in 3GPP TS 23.468 [9].

### 6.3.7	Reference point MCVideo-9 (between the transmission control participant and the transmission control server)

The MCVideo-9 reference point, which exists between the transmission control participant and the transmission control server, is used MCVideo transmission control signalling over multicast. The MCVideo-9 reference point uses the MB2-U interface defined in 3GPP TS 23.468 [9].

### 6.3.8	Reference point MCVideo-10 (between the MC gateway server and the MC gateway server in a different MCVideo system)

The MCVideo-10 reference point, which exists between the MC gateway server and the MC gateway server in an interconnected MCVideo system for MCVideo application signalling for establishing MCVideo sessions, shall use the SIP-3 reference point for transport and routing of signalling. Floor control signalling and media are also transferred using the MCVideo-10 reference point between interconnected MCVideo systems.

# 6A	Identities

The MCVideo service specific identities (e.g. MCVideo ID, MCVideo group ID) are described in clause 8 of 3GPP TS 23.280 [6].

# 6B	Application of functional model to deployments

The application of the functional model to deployments, and description of various deployment scenarios for the MCVideo service, can be found in clause 9 of 3GPP TS 23.280 [6].

# 7	Procedures and information flows

## 7.1	Group call

### 7.1.1	General

Group calls are enabled in both on-network and off-network.

### 7.1.2	On-network group call

#### 7.1.2.1	General

This subclause contains procedures for group call across a single and multiple MCVideo servers, and associated functions such as emergency call, broadcast call and others.

Two variations of group call model are described in subclause 7.1.2.3, the pre-arranged group call and the chat group call. Each of the subsequent group call types in subclause 7.1.2.4 onwards are applicable to either call model.

#### 7.1.2.2	Information flows for group call in on-network

##### 7.1.2.2.1	Group call request (MCVideo client – MCVideo server)

Table 7.1.2.2.1-1 describes the information flow group call request from the MCVideo client to the MCVideo server.

Table 7.1.2.2.1-1: Group call request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is requested |
| SDP offer | M | Media parameters of MCVideo clients |
| Implicit transmit media request | O | When originating client requests the permission to transmit media, this element shall be included |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |
| Requested priority | O | Application priority level requested for this call |

##### 7.1.2.2.2	Group call request

Table 7.1.2.2.2-1 describes the information flow group call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.2-1: Group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is initiated |
| SDP offer | M | Media parameters of MCVideo server |
| Broadcast indicator | O | Indicates that the group call request is for a broadcast group call |

##### 7.1.2.2.3	Group call response

Table 7.1.2.2.3-1 describes the information flow group call response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.3-1: Group call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is requested |
| SDP answer | M | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |

##### 7.1.2.2.4	Group call response (MCVideo client – MCVideo server)

Table 7.1.2.2.4-1 describes the information flow group call response from the MCVideo client to the MCVideo server.

Table 7.1.2.2.4-1: Group call response (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the target MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is initiated |
| SDP answer | M | Media parameters selected |
| Result | M | Result of the group call request (success or failure) |

##### 7.1.2.2.5	Group call release request

Table 7.1.2.2.5-1 describes the information flow group call release request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.5-1: Group call release request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is released |

##### 7.1.2.2.6	Group call release request (MCVideo client – MCVideo server)

Table 7.1.2.2.6-1 describes the information flow group call release request from the MCVideo client to the MCVideo server.

Table 7.1.2.2.6-1: Group call release request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is released |

##### 7.1.2.2.7	Group call release response

Table 7.1.2.2.7-1 describes the information flow group call release response from the MCVideo server to the MCVideo server and from the MCVideo client to the MCVideo server.

Table 7.1.2.2.7-1: Group call release response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is released |

##### 7.1.2.2.8	Group call rejoin request

Table 7.1.2.2.8-1 describes the information flow group call rejoin request from the MCVideo server to the MCVideo server and from the MCVideo client to the MCVideo server.

Table 7.1.2.2.8-1: Group call rejoin request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the re-joining MCVideo group member |
| Functional alias | O | The functional alias of the re-joining MCVideo group member |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |
| SDP offer | M | Media parameters of MCVideo client |

##### 7.1.2.2.9	Group call rejoin response

Table 7.1.2.2.9-1 describes the information flow group call rejoin response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.9-1: Group call rejoin response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member rejoining the group call |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |
| SDP answer | M | Media parameters selected |

##### 7.1.2.2.10	Group join request

Table 7.1.2.2.10-1 describes the information flow group join request from the MCVideo server to the MCVideo server and from the MCVideo client to the MCVideo server.

Table 7.1.2.2.10-1: Group join request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member joining the group communications for the group |
| MCVideo group ID | M | The MCVideo group ID of the group to which the group communication is requested |
| SDP offer | M | Media parameters of MCVideo client |
| Implicit transmit media request | O | An indication that the user is also requesting the permission to transmit video |
| Requested priority | O | Application priority level requested for this call |

##### 7.1.2.2.11	Group join response

Table 7.1.2.2.11-1 describes the information flow group join response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.11-1: Group join response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member joining the group communications for the group |
| MCVideo group ID | M | The MCVideo group ID of the group to which the group communication is requested |
| SDP answer | M | Media parameters selected |

##### 7.1.2.2.12	Group call leave request

Table 7.1.2.2.12-1 describes the information flow group call leave request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.12-1: Group call leave request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member which has been de-affiliated |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |

##### 7.1.2.2.12a	Group call leave request

Table 7.1.2.2.12a-1 describes the information flow group call leave request from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo server.

Table 7.1.2.2.12a-1: Group call leave request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member leaving the group call |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |

##### 7.1.2.2.13	Group call leave response

Table 7.1.2.2.13-1 describes the information flow group call leave response from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo server.

Table 7.1.2.2.13-1: Group call leave response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member which has been de-affiliated |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |

##### 7.1.2.2.13a	Group call leave response

Table 7.1.2.2.13a-1 describes the information flow group call leave response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.13a-1: Group call leave response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the MCVideo group member leaving the group call |
| MCVideo group ID | M | The MCVideo group ID of the group on which the call is on-going |

##### 7.1.2.2.14	Void

##### 7.1.2.2.15	Void

##### 7.1.2.2.16	Void

##### 7.1.2.2.17	Void

##### 7.1.2.2.18	MCVideo emergency group call request

Table 7.1.2.2.18-1 describes the information flow emergency group call request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.18-1: MCVideo emergency group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Emergency indicator | M | Indicates that the group call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested priority | O | Priority level requested for the call |
| Implicit transmit media request (see NOTE) | O | Indicates that the originating client requests the permission to transmit media. |
| NOTE: This element shall be included only when the originating client requests the the permission to transmit media. |  |  |

##### 7.1.2.2.19	MCVideo emergency group call response

Table 7.1.2.2.19-1 describes the information flow emergency group call response from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.19-1: MCVideo emergency group call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Result | M | Result of the MCVideo emergency group call request (success or failure) |

##### 7.1.2.2.20	MCVideo in-progress emergency group state cancel request

Table 7.1.2.2.20-1 describes the information flow MCVideo in-progress emergency group state cancel request from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo server.

NOTE:	In Rel-14 and Rel-13 versions of this specification the name of this information flow is "MCVideo emergency group call cancel request".

Table 7.1.2.2.20-1: MCVideo in-progress emergency group state cancel request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the cancelling party |
| MCVideo group ID | M | The MCVideo group ID on which the MCVideo in-progress emergency state is to be cancelled. |
| Alert indicator | O | Indicates whether the emergency alert of the cancelling party is to be cancelled |

##### 7.1.2.2.21	MCVideo in-progress emergency group state cancel response

Table 7.1.2.2.21-1 describes the information flow MCVideo in-progress emergency group state cancel response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

NOTE:	In Rel-14 and Rel-13 versions of this specification the name of this information flow is "MCVideo emergency group call cancel response".

Table 7.1.2.2.21-1: MCVideo in-progress emergency group state cancel response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the cancelling party |
| MCVideo group ID | M | The MCVideo group ID on which the MCVideo in-progress emergency state is to be cancelled. |

##### 7.1.2.2.22	MCVideo imminent peril group call request

Table 7.1.2.2.22-1 describes the information flow MCVideo imminent peril group call request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.22-1: MCVideo imminent peril group call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Requested priority | O | Priority level requested for the call |
| Imminent peril indicator | M | Indicates that the group call request is an imminent peril call |

##### 7.1.2.2.23	MCVideo imminent peril group call response

Table 7.1.2.2.23-1 describes the information flow MCVideo imminent peril group call response from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.23-1: MCVideo imminent peril group call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the calling party |
| MCVideo group ID | M | The MCVideo group ID on which the call is to be conducted |
| Result | M | Result of the MCVideo imminent peril group call request (success or failure) |

##### 7.1.2.2.24	MCVideo imminent peril group call cancel request

Table 7.1.2.2.24-1 describes the information flow MCVideo imminent peril group call cancel request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.24-1: MCVideo imminent peril group call cancel request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the cancelling party |
| MCVideo group ID | M | The MCVideo group ID on which the imminent peril is to be cancelled |

##### 7.1.2.2.25	MCVideo imminent peril group call cancel response

Table 7.1.2.2.25-1 describes the information flow MCVideo imminent peril group call cancel response from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.1.2.2.25-1: MCVideo imminent peril group call cancel response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the cancelling party |
| MCVideo group ID | M | The MCVideo group ID on which the imminent peril is to be cancelled |

#### 7.1.2.3	Group call within one MC system

##### 7.1.2.3.1	Group call models

###### 7.1.2.3.1.1	Pre-arranged group call

7.1.2.3.1.1.1	General

A pre-arranged group call is initiated by one of the affiliated group members. The initiation of a pre-arranged group call results in all other affiliated group members being invited. Media plane resources are reserved (on-demand) or a pre-established session is associated during the group call setup procedure and released (if on-demand session) or de-associated (if pre-established session) when the call is released. SIP signalling is used to setup and release pre-arranged group calls.

7.1.2.3.1.1.2	Pre-arranged group call setup

The procedure enables the scenario where an MCVideo client is initiating an MCVideo group call with unicast signalling for communicating with the affiliated members of that group.

Procedures in figure 7.1.2.3.1.1.2-1 are the signalling control plane procedures for the MCVideo client initiating establishment of an MCVideo group call with a pre-arranged group i.e., MCVideo users on client 1, client 2 and client 3 belong to the same group which is defined in the group management server.

Pre-conditions:

1.	A pre-arranged group is an MCVideo group that is pre-defined with MCVideo group ID and member list in the group management server. All members of the group belong to the same MC system.

2.	It is assumed that MCVideo users on MCVideo client 1, MCVideo client 2 and MCVideo client 3 are already registered for receiving MCVideo service and affiliated.

3.	The MCVideo client 1 may have an activated functional alias to be used.

4.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

5.	The MCVideo user on MCVideo client 1 may have bound a functional alias to the MCVideo group ID (3GPP TS 23.280 [6]).

Figure 7.1.2.3.1.1.2-1: Pre-arranged group call setup

1.	User at MCVideo client 1 would like to initiate an MCVideo group call with a selected group (identified by MCVideo group ID). The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo group call.

NOTE 1:	The selected functional alias is not changed during the group call, i.e. a MCVideo client uses the same functional alias until the group call is released or the MCVideo client has left the group call.

2.	MCVideo client 1 sends a group call request towards the MCVideo server via the SIP core, which hosts the group selected by the user and identified by MCVideo group ID. The group call request also contains the MCVideo group ID and an SDP offer containing the MCVideo client media parameters. If the MC service user of MCVideo client 1 has selected a functional alias, then the group call request contains that functional alias. If there is a transmit media request, then the group call request contains an indication for the implicit transmit media request.

3.	MCVideo server checks whether the user of MCVideo client 1 is authorized to initiate a group call for the selected group. If a functional alias is present, the MCVideo server checks whether it is allowed to be used and if it has been activated for the user. If authorized and the group call is ongoing for that MCVideo group ID, the MCVideo server adds the requesting MCVideo client 1 to the existing MCVideo group call and notifies the MCVideo client 1 that the MCVideo group call is already in progress. Otherwise, MCVideo server resolves the MCVideo group ID to determine the members of that group and their affiliation status, based on the information from the group management server.

If the functional alias is provided only in the group call request, or via binding, the MCVideo server proceeds with the value that is provided. If the functional alias is provided in both the group call request and via binding, it is up to the MCVideo server implementation to determine a value for the functional alias to be used.

NOTE 2:	MCVideo server can have already retrieved the user/group configuration data and locally cached. If the user/group configuration data is not locally cached on the MCVideo server then MCVideo server requests the user/group configuration data from the MCVideo user database/group management server.

4.	MCVideo server includes information that it communicates using MCVideo service, offers the same media parameters or a subset of the media parameters contained in the initial received request and sends the corresponding group call request via the SIP core towards the MCVideo clients of each of those affiliated group members. MCVideo users are notified about the incoming group call and, if present, the functional alias of the initiating MC service user is displayed. The MCVideo server indicates whether acknowledgement is required for the call.

5.	The receiving MCVideo clients accept the group call request, and a group call response is sent to the group host MCVideo server. This response may contain an acknowledgement. The conditions for sending acknowledgement may be based on configuration.

6.	MCVideo server sends the group call response including the selected media parameters to the MCVideo client 1 through the signalling path to inform about successful call establishment.

NOTE 3:	Step 6 can occur at any time following step 4b, and prior to step 7 depending on the conditions to proceed with the call.

7.	If the initiating MCVideo user requires the acknowledgement from affiliated MCVideo group members, and the required MCVideo group members do not acknowledge the call setup within a configured time (the "acknowledged call setup timeout"), then the MCVideo server may proceed with or abandon the call and then notify the initiating MCVideo user that the acknowledgements did not include all required members according to group policy. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the MCVideo group call.

8.	MCVideo client 1, client 2 and client 3 have successfully established media plane and transmission control for communication.

7.1.2.3.1.1.3	Release pre-arranged group call

The procedure enables the scenario where an MCVideo server initiates the termination of an ongoing MCVideo group call for all the participants of that group call, since at least one of the termination conditions are met e.g., last participant leaving, second last participant leaving, initiator leaving, or minimum number of affiliated MCVideo group members are not present.

Procedures in figure 7.1.2.3.1.1.3-1 are the signalling control plane procedures for the MCVideo server initiating termination of an ongoing MCVideo group call.

Figure 7.1.2.3.1.1.3-1: Release pre-arranged group call

1.	It is assumed that MCVideo users on MCVideo client 1, client 2 and client 3 are already part of the ongoing group call (e.g., as a result of pre-arranged group call setup).

2.	MCVideo server would like to release the MCVideo group call which is ongoing e.g., last participant leaving, initiator leaving, or minimum number of affiliated group members are not present.

3.	MCVideo server identifies the participants of the ongoing group call and generates group call release request to release ongoing session.

4.	MCVideo server sends a group call release request via SIP core towards each participant of the ongoing group call.

5.	MCVideo users are notified about the release of the group call.

6.	MCVideo client(s) receiving group call release request, acknowledge towards the MCVideo server by sending a group call release response.

7. MCVideo client 1, client 2 and client 3 have successfully released the media transmission control and media plane resources associated with the group call that is terminated.

7.1.2.3.1.1.4	Late entry pre-arranged group call

Procedures in figure 7.1.2.3.1.1.4-1 are the signalling control plane procedures for the MCVideo server requesting a newly affiliated member or a member coming back from out of coverage to join an ongoing MCVideo group call.

Pre-conditions:

1.	MCVideo group is previously defined on the group management server with MCVideo users affiliated to that group. All members of the group belong to the same MC system.

2.	It is assumed that MCVideo users on MCVideo client 2 to MCVideo client n are on an ongoing call.

3.	Optionally, the MCVideo client 1 may have activated functional alias to be used.

4.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.1.2.3.1.1.4-1: Late entry pre-arranged group call

1.	MCVideo server determines that MCVideo client 1 which is newly affiliated or coming back from out of coverage has to be invited to join an ongoing group call (late entry).

2.	MCVideo server generates group call request including the information such as MCVideo service identifier (possible for the SIP core to route the request to the MCVideo server), MC service group ID of the group invited to join, offer one or more media types and sends towards the MCVideo client 1 via SIP core.

3.	MCVideo user at MCVideo client 1 is notified about the incoming group call.

4.	Upon MCVideo user at MCVideo client 1 accepting the incoming group call request, MCVideo client 1 sends the group call response including the selected media types to the MCVideo server through the signalling path. If the incoming group call request is rejected by the MCVideo client 1, the MCVideo server should not resend the group call request

5.	MCVideo client 1 is successfully added to the ongoing group call and MCVideo users at MCVideo client 1 to MCVideo client n may be notified about the MCVIDEO client 1 joining the group call.

6.	A notification with the information of media transmissions in the group call is sent to MCVideo client1.

Editor's Note: Checking of functional alias of MCVideo client 1 by the MCVideo server is FFS.

7.1.2.3.1.1.5	Rejoining call

Procedures in figure 7.1.2.3.1.1.5-1 are the signalling control plane procedures for the MCVideo client to rejoin an ongoing MCVideo group call (e.g. coming back from out of coverage).

Pre-condition:

1	It is assumed that MCVideo users on MCVideo client 2 to MCVideo client n are on an ongoing call.

2.	Optionally, the MCVideo client 1 may have activated functional alias to be used.

3.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.1.2.3.1.1.5-1: Rejoin call

1.	MCVideo client 1 has necessary information for rejoining an ongoing group call, then the MCVideo client 1 initiates group call rejoin request including the ongoing group call information. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo rejoin request.

2.	MCVideo server checks whether the MCVideo client 1 can rejoin the ongoing call (e.g. based upon affiliation status) and checks the functional alias, if present.

3.	MCVideo client 1 is informed that the group call rejoin is successful by sending a group call rejoin response.

4.	MCVideo client 1 is successfully added to the ongoing group call and MCVideo users at MCVideo client 1 to MCVideo client n may be notified about the MCVideo client 1 joining the group call and the functional alias of MCVideo client 1 may be displayed.

5.	A notification with the information of media transmissions in the group call is sent to MCVideo client 1.

###### 7.1.2.3.1.2	Chat group call

7.1.2.3.1.2.1	General

In a chat group (restricted) call model, the MCVideo user individually joins a group call without being explicitly invited by the MCVideo server. The establishment of a chat group (restricted) call does not result in other group members being invited.

Figure 7.1.2.3.1.2.2-1 describes the basic procedure for the MCVideo client initiating an MCVideo group call which uses the chat group (restricted) call model. The chat group (restricted) call model can be used to realize the video conferencing service where only users that have been configured as participants for the video conferencing group are allowed to join the group communications for the given group.

Chat group join mechanism:

-	Each MCVideo client sends a group join request when the MCVideo user wants to participate in the group communication for the group. (This message does not impact the MCVideo user's membership in the group; the MCVideo server will verify that the MCVideo user is an authorized member of the group.)

-	The group join request may include a transmit media request. It is assumed that the group join request will be delivered from MCVideo client to MCVideo server using SIP.

-	The group join request is used to indicate to the MCVideo server that the MCVideo user associated with the given MCVideo client wishes to participate (begin to receive notifications for media transmissions) from the group.

-	The group join request shall cause the MCVideo server to generate an implicit affiliation for the MCVideo user to the group, if the user is not already affiliated to the group.

-	The group join request contains the information needed to negotiate media parameters (on demand) or to associate a pre-established session between MCVideo server and MCVideo client for the group call. The group join request can take the form of a SIP invite.

-	A selected functional alias is not changed by a MCVideo client during the whole participation within a chat group call, i.e. a MCVideo client uses the same functional alias selected when joining the chat group call until the chat group call is released or the MCVideo client leaves the chat group call.

Subsequent participation in a group call when the group is using the chat model:

-	Once an MCVideo client successfully joins a group call which is using the chat model, the MCVideo client connects to the media plane for the media transmission if the media transmission is currently ongoing.

-	If the MCVideo group call is not currently ongoing (i.e.: when MCVideo clients on the group call are not sending or receiving media, and the time out between transmission control exchanges has expired) then the newly joined MCVideo client will only have pre-established its media parameters for the call.

-	If the newly joined MCVideo user wishes to transmit media to the other joined users of the group using the chat model, then the MCVideo client shall use a normal transmission control procedure for transmitting the media.

-	Subsequent group call media transmissions are controlled using transmission control signalling.

-	The MCVideo server may tear down the media plane between successive group calls using the chat model, or the MCVideo server may allow the media plane to remain up between successive group calls using the chat model depending on resources.

Leaving and releasing a chat group:

-	When a user wants to leave a chat group call, the client shall send a group call leave request to the server and release the media plane.

-	The server can release a chat group call by sending a group call release to all joined clients. A server initiated release also releases the media plane for all joined clients.

7.1.2.3.1.2.2	Chat group call setup

MCVideo client 1, client 2, and client 3 are served by the home MCVideo service provider in figure 7.1.2.3.1.2.2-1.

Pre-conditions:

1.	MCVideo user 2 and MCVideo user 3 have previously joined (affiliated) to the group call. MCVideo client 1, client 2, and client 3 are registered and all users (MCVideo user 1, user 2, and user 3) have been authenticated and authorized to use the MCVideo service.

2.	MCVideo client 1, MCVideo client 2 and MCVideo client 3 may have activated functional alias(es) configured to be used during the group call communication. No call is currently in progress for the group.

3.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

4.	The MCVideo user on MCVideo client 1 may have bound a functional alias to the MCVideo group ID (3GPP TS 23.280 [6]).

Figure 7.1.2.3.1.2.2-1: MCVideo chat group call

1.	MCVideo user 1 indicates to join the group communication for the group. This may include a transmit media request.

1a.	MCVideo client 1 sends a group join request with the MCVideo group ID of the desired group. It contains the MCVideo user's MCVideo ID and the MCVideo client media parameters. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo group join request. If there is a request for media transmission, then the group join request contains an indication of an implicit transmit media request.

1b.	The MCVideo server receives the group join request. MCVideo server generates an implicit affiliation (if the MCVideo user is not already affiliated to the group) and verifies that MCVideo user 1 is authorized to affiliate to the group by following the affiliation procedure for MCVideo. If a functional alias is present, the MCVideo server checks whether it is allowed to be used and if it has been activated for the user.

If the functional alias is provided only in the group join request, or via binding, the MCVideo server proceeds with the value that is provided. If the functional alias is provided in both the group join request and via binding, it is up to the MCVideo server implementation to determine a value for the functional alias to be used.

1c.	The MCVideo server replies with a group join response indicating the acceptance of the group join request and also returns the MCVideo server selected media parameters for the group call in the group join response.

2.	If MCVideo user 1 requests to transmit media by sending transmit media request to MCVideo server, the MCVideo server establishes the media plane (if not already established) for the call.

3.	Transmission control will continue to be used by the transmission control participants associated with MCVideo client 1, MCVideo client 2 and MCVideo client 3 for the duration of the call. .If present, the functional alias of MCVideo client 1, MCVideo client 2 and MCVideo client 3 are displayed where appropriate.

7.1.2.3.1.2.3	Release chat group call

The procedure describes the case where the MCVideo server releases an ongoing MCVideo group call for all the participants of that group call, since at least one of the conditions for release are met e.g. due to chat duration expiry, last participant leaving or initiator leaving.

NOTE 1:	The procedure for an MCVideo user leaving the group call is a different procedure.

Procedures in figure 7.1.2.3.1.2.3-1 are the procedures for the MCVideo server initiating the release of an ongoing MCVideo group call.

The following precondition applies:

- A group call is ongoing between MCVideo clients 1, 2 and 3

Figure 7.1.2.3.1.2.3-1: Release chat group call

1.	MCVideo server would like to release the MCVideo group call which is ongoing e.g., due to chat duration expiry, last participant leaving, or initiator leaving.

2.	MCVideo server identifies the participants of the ongoing group call and generates group call release request to release the ongoing session.

3.	MCVideo server sends a group call release request towards each participant of the ongoing group call.

NOTE 2:	The group call release request can also be sent over SIP signalling on the signalling plane.

4.	MCVideo users are notified about the release of the group call.

5.	Optionally the MCVideo client(s) receiving group call release request, may send a group call release response to the MCVideo server.

NOTE 3:	The MCVideo client can send group call release response when the group call release request is sent using a unicast bearer.

6.	MCVideo client 1, client 2 and client 3 release the transmission control and media plane resources associated with the group call that is released. Successful release of the group call does not affect the status of affiliation of any of the clients.

7.1.2.3.1.2.4	void

7.1.2.3.1.2.5	Late entry chat group call, newly joined group member

Procedures in figure 7.1.2.3.1.2.5-1 are those for a group member entering an ongoing MCVideo group call, i.e. performing a late entry.

Pre-conditions:

1.	MCVideo user 2 and MCVideo user 3 have previously joined to the group. MCVideo client 1, client 2, and client 3 are registered and all users (MCVideo user 1, user 2, and user 3) have been authenticated and authorized to use the MCVideo service.

2.	MCVideo user 1 indicates to join the group communication for the group.

3.	MCVideo users using MCVideo client 1 to MCVideo client n may have activated functional alias(es) configured to be used during the group call communication.

4	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

5.	The MCVideo user on MCVideo client 1 may have bound a functional alias to the MCVideo group ID (3GPP TS 23.280 [6]).

Figure 7.1.2.3.1.2.5-1: Late entry of a newly joined group member

1.	MCVideo client 1 sends a group join request with the MCVideo group ID of the desired group. It contains the MCVideo user's MCVideo ID and the MCVideo client media parameters. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo group join request. If there is a request to transmit, then the group join request contains an indication of an implicit transmit media request.

2.	The MCVideo server receives the group join request. MCVideo server generates an implicit affiliation (if the MCVideo user is not already affiliated to the group) and verifies that MCVideo user 1 is authorized to affiliate to the group. If a functional alias is present, the MCVideo server checks whether it is allowed to be used and if it has been activated for the user.

If the functional alias is provided only in the group join request, or via binding, the MCVideo server proceeds with the value that is provided. If the functional alias is provided in both the group join request and via binding, it is up to the MCVideo server implementation to determine a value for the functional alias to be used.

3.	The MCVideo server replies with a group join response indicating the acceptance of the group join request.

4.	Media plane between MCVideo client 1 and MCVideo server is established using media plane control signalling.

5.	MCVideo users at MCVideo client 2 and MCVideo client 3 may be notified about the MCVideo client 1 joining the group call, and the functional alias of MCVideo client 1 may be displayed.

6.	The MCVideo server may send (6a) Media transmission notification to MCVideo client 1, indicating the current transmitter. Alternatively the MCVideo server may send (6b) Transmit media granted, (6b) Transmit media rejected or (6d) Queue position info.

7.	Transmission control will continue to be used by the transmission control participants associated with MCVideo client 1, MCVideo client 2 and MCVideo client 3. If present, the functional alias of MCVideo client 1, MCVideo client 2 and MCVideo client 3 are displayed where appropriate.

7.1.2.3.1.2.6	Late entry chat group call, MCVideo client coming back from out of coverage

Procedures in figure 7.1.2.3.1.2.6-1 are those for an MCVideo client coming back from out of coverage during an ongoing MCVideo group call.

Pre-conditions:

1.	MCVideo users using MCVideo client 1, MCVideo client 2 and MCVideo client 3 are in an ongoing group call when MCVideo client1 goes out of radio coverage.

2.	MCVideo client1 returns from out of coverage while the group call is still ongoing.

3.	MCVideo users using MCVideo client 1 to MCVideo client n may have activated functional alias(es) configured to be used during the group call communication.

4.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.1.2.3.1.2.6-1: Late entry of an MCVideo client returning from out of coverage

1.	MCVideo client 1 or MCVideo server detects that MCVideo client 1 has returned from out of coverage.

NOTE 1:	How the MCVideo client or MCVideo server detects that the client has returned from out of coverage is out of scope of the present document.

2.	Media plane between MCVideo client 1 and MCVideo server is re-established using media plane control signalling.

NOTE 2:	Depending on how long MCVideo client 1 was out of coverage, this step can include signalling from MCVideo client 1 to rejoin the chat group.

3.	The MCVideo server may send (3a) Media transmission notification to MCVideo client 1, indicating the current transmitter. Alternatively, the MCVideo server may send (3b) Transmit media granted, (3c) Transmit media rejected or (3d) Queue position info.

4.	MCVideo users at MCVideo client 2 and MCVideo client 3 may be notified about the MCVideo client 1 returning to the group call, and the functional alias of MCVideo client 1 may be displayed.

5.	Transmission control will continue to be used by the transmission control participants associated with MCVideo client 1, MCVideo client 2 and MCVideo client 3. If present, the functional alias of MCVideo client 1, MCVideo client 2 and MCVideo client 3 are displayed where appropriate.

##### 7.1.2.3.2	Exiting group call due to de-affiliation

Procedures in figure 7.1.2.3.2-1 are the signalling control plane procedures for the MCVideo server requesting a newly de-affiliated member to leave an ongoing MCVideo group call.

Pre-conditions:

1.	MCVideo group is previously defined on the group management server with MCVideo users affiliated to that group. All members of the group belong to the same MC system.

2.	MCVideo users on MCVideo client 1 to MCVideo client n are on an ongoing call.

3.	MCVideo client 1 has been de-affiliated from the MCVideo group.

Figure 7.1.2.3.2-1: Exiting MCVideo group call due to de-affiliation

1.	MCVideo client 1 which has been de-affiliated is instructed to leave the ongoing group call.

2.	MCVideo server sends a group call leave request to MCVideo client 1.

3.	MCVideo user at MCVideo client 1 is notified about leaving the group call.

4.	MCVideo client 1 sends the group call leave response and leaves the group call.

5.	MCVideo client 1 is now removed from the ongoing group call and MCVideo users at MCVideo client 2 to MCVideo client n may be notified that MCVideo client 1 has left the group call.

##### 7.1.2.3.3	MCVideo user leaving a group call

Procedures in figure 7.1.2.3.3-1 are the signalling control plane procedures for the MCVideo user leaving an ongoing MCVideo group call.

Pre-conditions:

1.	MCVideo group is previously defined on the group management server with MCVideo users affiliated to that group. All members of the group belong to the same MC system.

2.	MCVideo users on MCVideo client 1 to MCVideo client n are on an ongoing call.

3.	MCVideo user 1 indicates to leave the group call

Figure 7.1.2.3.3-1: MCVideo user leaving a group call

1.	MCVideo client 1 sends a group call leave request to the MCVideo server.

2.	MCVideo server sends a group call leave response to MCVideo client 1.

3.	Release of transmission control and media plane resources associated with the group call for MCVideo client 1.

NOTE 1:	MCVideo server checks if group call termination conditions are met, e.g. last participant leaving, second last participant leaving, initiator leaving, minimum number of affiliated MCVideo group members are not present . If at least one of the group call termination conditions is met, the MCVideo server releases the group call for all participants (see subclauses 7.1.2.3.1.1.3 Release pre-arranged group call and 7.1.2.3.1.2.3 Release chat group call).

4.	MCVideo users at MCVideo client 2 to MCVideo client n may be notified that the MCVideo user of client 1 has left the group call.

NOTE 2:	The MCVideo server does not perform late entry for MCVideo users that have left the group call.

#### 7.1.2.4	Broadcast group call

##### 7.1.2.4.1	General

A broadcast group call is a special group call where the initiating MCVideo user expects no response from the other MCVideo users, i.e. no response from the recipients is required to start media transmission and the call ends when the media transmission is complete.

##### 7.1.2.4.2	Common broadcast group call procedure

Only the call originator can transmit media during the broadcast group call and the broadcast group call is released when the transmission is complete.

Figure 7.1.2.4.2-1 illustrates the common procedure both for group-broadcast group call and user-broadcast group call.

Pre-condition:

1.	MCVideo client 1 and MCVideo client 2 are members of a group-broadcast group/user-broadcast group.

Figure 7.1.2.4.2-1: Broadcast group call

1.	MCVideo user at MCVideo client 1 initiates the broadcast group call setup procedure with the indication of broadcast group call. The signalling procedure is identical to the group call setup as described in subclause 7.1.2.3.1.1 with the inclusion of the parameter for broadcast group call indicator.

2.	MCVideo client 1 starts to transmit media.

NOTE 1:	Only the call originating MCVideo user is allowed to transmit media on broadcast group call.

NOTE 2:	A broadcast group call transmitted on a user-broadcast group has priority over group calls involving users within the user hierarchy. A broadcast group call transmitted on a group-broadcast group has priority over group calls on its subordinate groups.

3.	If the media transmission from call originating MCVideo user is complete, the broadcast group call is released.

#### 7.1.2.5	Emergency and imminent peril procedures

##### 7.1.2.5.1	MCVideo emergency group call

###### 7.1.2.5.1.1	MCVideo emergency group call commencement

The procedure describes the case where an MCVideo client is initiating an MCVideo emergency group call with the affiliated MCVideo group members of that MCVideo group. An MCVideo client in the MCVideo emergency state gains elevated access privilege for all of the MCVideo user's mission critical applications. Initiating an MCVideo emergency group call puts the MCVideo group into the in-progress emergency state. While in the in-progress emergency state, all MCVideo group calls in the MCVideo group are processed as MCVideo emergency group calls by the MCVideo server until the emergency state of the MCVideo group is cancelled.

Figure 7.1.2.5.1.1-1 shows the procedures for the MCVideo client initiating establishment of an MCVideo emergency group call with an MCVideo group i.e., MCVideo users on MCVideo client 1, MCVideo client 2 and MCVideo client 3 belong to the same MCVideo group which is defined on group management server.

NOTE 1:	For simplicity, a single MCVideo server is shown in place of a user home MCVideo server and a group hosting MCVideo server.

Pre-conditions:

1.	The MCVideo group is previously defined on the group management server with MCVideo client 2 and MCVideo client 3 affiliated to that MCVideo group.

2.	All members of the MCVideo group belong to the same MC system.

3.	The initiating MCVideo client 1 has been provisioned with an MCVideo group that has been designated via provisioning as the MCVideo emergency group.

4	MCVideo client 1, MCVideo client 2 and MCVideo client 3 may have an activated functional alias to be used during the emergency group communication.

5.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

NOTE 2:	Alternatively, the client could have been provisioned for emergency behaviour on the selected group.

Figure 7.1.2.5.1.1-1: MCVideo emergency group call

1.	The user at the MCVideo client 1 initiates an MCVideo emergency group call. MCVideo client 1 sets its MCVideo emergency state. The MCVideo emergency state of MCVideo client 1 is retained until explicitly cancelled by the user of MCVideo client 1.

NOTE 3: While MCVideo client 1 is in the emergency state, all MCVideo group and private calls initiated by MCVideo client 1 are initiated as MCVideo emergency calls.

2.	MCVideo client 1 sends an MCVideo emergency group call request towards the MCVideo server. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo emergency group call request. The request contains an indication of the MCVideo emergency. The MCVideo server records the identity of the MCVideo user that initiated the MCVideo emergency group call until the MCVideo emergency state of the group is cancelled. Once an MCVideo emergency call has been initiated, the MCVideo group is considered to be in an in-progress emergency state until the emergency state of the group cancelled. If configured to send an MCVideo emergency alert when initiating an MCVideo emergency group call, the request also contains an indication that an MCVideo emergency alert is to be initiated. The request may contain an indication of an implicit transmit media request.

NOTE 4:	While the MCVideo group is in the in-progress emergency state, all MCVideo group calls within the group are processed as emergency group calls by the MCVideo server. MCVideo group members that are not in the emergency state do not indicate emergency in group call requests but can set the requested priority of all group call requests to high priority.

3.	The MCVideo server implicitly affiliates MCVideo client 1 to the emergency group if the client is not already affiliated.

4.	MCVideo server checks whether the provided functional alias is allowed to be used and has been activated for the MCVideo user, and whether the MCVideo user of MCVideo client 1 is authorized for initiation of MCVideo emergency calls on the indicated MCVideo group, and if authorized, it resolves the MCVideo group ID to determine the members of that MCVideo group and their affiliation status, based on the information from group management server.

5.	The MCVideo server configures the priority of the underlying bearers for all participants in the MCVideo group.

NOTE 5:	Successive calls during the MCVideo group's in-progress emergency state will all receive the adjusted bearer priority.

6.	MCVideo server sends the MCVideo emergency group call request towards the MCVideo clients of each of those affiliated MCVideo group members. The request contains an indication of the in-progress emergency. The request contains an indication of an MCVideo emergency alert if the request from the originator indicated MCVideo emergency alert.

7.	MCVideo users are notified of the incoming MCVideo group call, and, if available, the functional alias of the initiating user is displayed.

8.	The receiving MCVideo clients send the MCVideo emergency group call response to the MCVideo server to acknowledge the MCVideo emergency group call request. For a multicast call, these acknowledgements are not sent.

9.	The MCVideo server sends the MCVideo emergency group call response to the MCVideo user 1 to inform the successful MCVideo emergency call establishment.

NOTE 6:	Step 9 can occur at any time following step 5, and prior to step 10 depending on the conditions to proceed with the call.

MCVideo client 1, MCVideo client 2 and MCVideo client 3 have successfully established media plane for communication. MCVideo transmission control participant 1, transmission control participant 2 and transmission control participant 3 exchange transmission control information e.g., MCVideo client 1 receives the transmit media granted information over the established media plane, while the other MCVideo client's receive media available notification with forced reception information. MCVideo client 1 indicates to the MCVideo user that the permission is granted to transmit media, while the other MCVideo clients in the MCVideo emergency group call will be receiving that media. MCVideo client 1 can override other clients in the call except those that are also in the MCVideo emergency state.

###### 7.1.2.5.1.2	MCVideo group call upgraded to an MCVideo emergency group call

The procedure describes the case where an authorized MCVideo user is upgrading an MCVideo group call to an MCVideo emergency group call while the MCVideo group call is already in progress.

Procedures in figure 7.1.2.5.1.2-1 are the signalling control plane procedures for the MCVideo client upgrading an MCVideo group call on an MCVideo group to an MCVideo emergency group call.

NOTE 1:	For simplicity, a single MCVideo server is shown in place of a user home MCVideo server and a group hosting MCVideo server.

Pre-conditions:

1.	The MCVideo group is previously defined on the group management server with MCVideo client 2 and MCVideo client 3 affiliated to that MCVideo group.

2.	All members of the MCVideo group belong to the same MC system.

3.	An MCVideo group call is already in progress.

4.	The initiating MCVideo client 1 has been configured to send an MCVideo emergency alert when upgrading an MCVideo emergency group call.

Figure 7.1.2.5.1.2-1: MCVideo group call upgraded to an MCVideo emergency group call

1.	The MCVideo user at MCVideo client 1 initiates a group emergency. MCVideo client 1 sets its MCVideo emergency state. The MCVideo emergency state of MCVideo client 1 is retained until explicitly cancelled by the user of MCVideo client 1.

NOTE 2: While MCVideo client 1 is in the emergency state, all MCVideo group and private calls initiated by MCVideo client 1 are initiated as MCVideo emergency calls.

2.	MCVideo client 1 requests the MCVideo server to upgrade the MCVideo group to an in-progress emergency state by sending an MCVideo emergency group call request. If configured to send an MCVideo alert when initiating an MCVideo emergency upgrade, the request also contains an indication that an MCVideo alert is to be initiated. The request may contain an indication of an implicit transmit media request.

NOTE 3:	While the MCVideo group is in the in-progress emergency state, all MCVideo group calls within the group are processed as emergency group calls by the MCVideo server. MCVideo group members that are not in the emergency state do not indicate emergency in group call requests but can set the requested priority of all group call requests to high priority.

3.	The MCVideo server adjusts the priority of the underlying bearer for all participants in the MCVideo group.

4.	MCVideo server sends the MCVideo emergency group call request towards the MCVideo clients of each of those affiliated MCVideo group members. The request contains an indication of an MCVideo emergency alert if the request from the originator indicated MCVideo emergency alert.

5.	MCVideo users are notified of the in-progress emergency state of the MCVideo group.

6.	The receiving MCVideo clients send the MCVideo emergency group call response to the MCVideo server to acknowledge the MCVideo group emergency request. For a multicast call, these acknowledgements are not sent.

7.	The MCVideo server sends the MCVideo emergency group call response to the MCVideo user 1 to confirm the upgrade request. If the MCVideo emergency request contained an implicit transmit media request, the OK message contains the result of the implicit transmit media request.

NOTE 4:	Step 7 can occur at any time following step 3, and prior to step 8 depending on the conditions to proceed with the call.

MCVideo client 1, MCVideo client 2 and MCVideo client 3 continue with the MCVideo group call, which has been transformed into an MCVideo emergency group call. MCVideo client 1 can override other clients in the call except those that are also in the MCVideo emergency state.

###### 7.1.2.5.1.3	MCVideo in-progress emergency group state cancel

NOTE 1:	In Rel-14 and Rel-13 versions of this specification the title of this subclause is "MCVideo emergency group call cancel".

The procedure describes the case where an MCVideo client cancels an MCVideo group's in-progress emergency state. The emergency state of the group may alternatively be cancelled by the emergency alert cancellation procedure specified in 3GPP TS 23.280 [16], subclause 10.10.1.2.2.2.

Procedures in figure 7.1.2.5.1.3-1 are the signalling control plane procedures for the MCVideo client cancelling an in-progress emergency of a group.

NOTE 2:	For simplicity, a single MCVideo server is shown in place of a user home MCVideo server and a group hosting MCVideo server.

NOTE 3:	The end of the MCVideo emergency group call does not cancel the MCVideo group's in-progress emergency state. It is explicitly cancelled by an authorized user using this procedure, or by the emergency alert cancellation procedure specified in 3GPP TS 23.280 [5], subclause 10.10.1.2.2.2.

Pre-conditions:

1.	The MCVideo group is previously defined on the group management server with MCVideo client 2 and MCVideo client 3 affiliated to that MCVideo group.

2.	All members of the MCVideo group belong to the same MC system.

3.	MCVideo group members have been notified about the in-progress emergency.

4.	The MCVideo group is in the in-progress emergency state and has prioritized bearer support.

5.	MCVideo client 1 previously initiated the in-progress emergency for the group.

Figure 7.1.2.5.1.3-1: MCVideo in-progress emergency group state cancel

1.	The user at the MCVideo client 1 initiates an MCVideo in-progress emergency group state cancel.

NOTE 4:	An MCVideo user authorized to cancel in-progress emergencies on the MCVideo group can also be authorised to cancel the MCVideo emergency alert in addition to the initiator. However, only the initiator can cancel the initiator's local MCVideo emergency state.

2.	MCVideo client 1 sends an MCVideo in-progress emergency group state cancel request to the MCVideo server.

NOTE 5:	If an MCVideo emergency alert relating to MCVideo client 1 is in effect together with an MCVideo in-progress emergency group state on the MCVideo group, the MCVideo emergency alert of MCVideo client 1 can be cancelled at the same time. In that case, the MCVideo in-progress emergency group state request carries an indication that the emergency alert of MCVideo client 1 is also being cancelled.

NOTE 6:	If an MCVideo in-progress emergency group state cancel request is received by the MCVideo server while a group member that is in the emergency state is transmitting, the MCVideo in-progress emergency group state cancel request is rejected by the MCVideo server.

3.	MCVideo server resolves the MCVideo group ID to determine the members of that MCVideo group and their affiliation status, based upon the information from group management server.

4.	The MCVideo server adjusts the priority of the underlying bearer; priority treatment is no longer required. The MCVideo server cancels/resets the emergency in-progress state of the MCVideo group.

5.	The MCVideo server sends an MCVideo in-progress emergency group state cancel request to the MCVideo group members.

6.	MCVideo group members are notified of the MCVideo in-progress emergency group state cancel.

7.	The receiving MCVideo clients send the MCVideo in-progress emergency group state cancel response to the MCVideo server to acknowledge the MCVideo in-progress emergency group state cancel. For a multicast call scenario, these acknowledgements are not sent.

8.	The MCVideo server sends the MCVideo in-progress emergency group state cancel response to the MCVideo user 1 to confirm the MCVideo in-progress emergency group state cancel. If the MCVideo in-progress emergency group state cancel request (in step 2) contained the "Alert indicator" IE, the MCVideo client 1 resets its local emergency status.

NOTE 7:	Step 8 can occur at any time following step 4, depending on the conditions to proceed with the call.

##### 7.1.2.5.2	MCVideo imminent peril group call

###### 7.1.2.5.2.1	MCVideo imminent peril group call commencement

The procedure focuses on the case where an authorized MCVideo user is initiating an imminent peril group call for communicating with the affiliated MCVideo members of that MCVideo group. This procedure will gain elevated access privilege for the MCVideo client if it is not already in that state. The access privilege for other applications will not necessarily be affected.

Procedures in figure 7.1.2.5.2.1-1 are the signalling control plane procedures for the MCVideo client initiating establishment of an imminent peril group call with an MCVideo group i.e., MCVideo users on MCVideo client 1, MCVideo client 2 and MCVideo client 3 belong to the same MCVideo group which is defined on MCVideo group management server.

Pre-conditions:

1.	The MCVideo group is previously defined on the group management server with MCVideo client 2 and MCVideo client 3 affiliated to that MCVideo group.

2.	All members of the MCVideo group belong to the same MC system.

3.	The initiating MCVideo client 1 has been provisioned with an MCVideo group that has been designated in the provisioning to be used for imminent peril communications.

4.	MCVideo client 1, MCVideo client 2 and MCVideo client 3 may have an activated functional alias to be used during the emergency group communication.

5.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

NOTE 1:	Alternatively, the client could have been provisioned for imminent peril behaviour on the selected group.

Figure 7.1.2.5.2.1-1: MCVideo imminent peril group call

1.	The user at the MCVideo client 1 initiates an imminent peril group call.

2.	MCVideo client 1 sends an MCVideo imminent peril group call request towards the MCVideo server. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo imminent peril group call request. The request contains an indication of the in-progress imminent peril. The MCVideo server records the identity of the MCVideo user that initiated the imminent peril group call until the in-progress imminent peril state is cancelled. Once an imminent peril group call has been initiated, the MCVideo group is considered to be in an in-progress imminent peril state until cancelled. The request may contain an indication of an implicit transmit media request.

3.	The MCVideo server implicitly affiliates MCVideo client 1 to the imminent peril group if the client is not already affiliated.

4.	MCVideo server checks whether the provided functional alias is allowed to be used and has been activated for the MCVideo user, and whether the MCVideo user of MCVideo client 1 is authorized for initiation of imminent peril group calls on the indicated MCVideo group, and if authorized, it resolves the MCVideo group ID to determine the members of that MCVideo group and their affiliation status, based on the information from group management server.

5.	The MCVideo server configures the priority of the underlying bearers for all participants in the MCVideo group.

NOTE 2:	Successive calls during the in-progress imminent peril state will all receive the adjusted bearer priority.

6.	MCVideo server sends the imminent peril group call request towards the MCVideo clients of each of those affiliated MCVideo group members. The request contains an indication of the in-progress imminent peril.

7.	MCVideo users are notified of the incoming imminent peril call, and, if available, the functional alias of the initiating user is displayed.

8.	The receiving MCVideo clients send the MCVideo imminent peril group call response to the MCVideo server to acknowledge the imminent peril call request. For a multicast call, these acknowledgements are not set.

9.	The MCVideo server sends the MCVideo imminent peril group call response to the MCVideo user 1 to inform the successful imminent peril call establishment. If the MCVideo imminent peril request contained an implicit transmit media request, the OK message contains the result of the implicit transmit media request.

NOTE 3:	Step 9 can occur at any time following step 5, and prior to step 10 depending on the conditions to proceed with the imminent peril call.

MCVideo client 1, MCVideo client 2 and MCVideo client 3 have successfully established media plane for communication. MCVideo transmission control participant 1, transmission control participant 2 and transmission control participant 3 exchange transmission control information e.g., MCVideo client 1 receives the transmit media granted information over the established media plane, while the other MCVideo clients receive media available notification with forced reception mode information. MCVideo client 1 indicates to the MCVideo user that the permission is granted to transmit media, while the other MCVideo clients in the imminent peril call will be receiving that media.

###### 7.1.2.5.2.2	Imminent peril group call upgrade

The procedure focuses on the case where an authorized MCVideo user is upgrading an MCVideo group call to an imminent peril group call while the MCVideo group call is already in progress.

Procedures in figure 7.1.2.5.2.2-1 are the signalling control plane procedures for the MCVideo client upgrading an MCVideo group call on an MCVideo group to an imminent peril group call.

Pre-conditions:

1.	The MCVideo group is previously defined on the group management server with MCVideo client 1, MCVideo client 2 and MCVideo client 3 affiliated to that MCVideo group.

2.	All members of the MCVideo group belong to the same MC system.

3.	An MCVideo group call is already in progress.

Figure 7.1.2.5.2.2-1: MCVideo group call upgrade to an imminent peril group call

1.	The MCVideo user at MCVideo client 1 initiates an imminent peril call.

2.	MCVideo client 1 requests the MCVideo server to upgrade the MCVideo group to an in-progress imminent peril state by sending an MCVideo imminent peril group call request. The request may contain an indication of an implicit transmit media request.

3.	The MCVideo server adjusts the priority of the underlying bearer for all participants in the MCVideo group.

4.	MCVideo server sends the MCVideo imminent peril group call request towards the MCVideo clients of each of those affiliated MCVideo group members.

5.	MCVideo users are notified of the in-progress imminent peril state of the MCVideo group.

6.	The receiving MCVideo clients send the MCVideo imminent peril group call response to the MCVideo server to acknowledge the MCVideo imminent peril group call request. For a multicast call, these acknowledgements are not set.

7.	The MCVideo server sends the MCVideo imminent peril group call response to the MCVideo user 1 to confirm the upgrade request. If the MCVideo imminent peril group call request contained an implicit transmit media request, the OK message contains the result of the implicit transmit media request.

NOTE:	Step 7 can occur at any time following step 4, and prior to step 8 depending on the conditions to proceed with the call.

MCVideo client 1, MCVideo client 2 and MCVideo client 3 continue with the MCVideo group call, which has been transformed into an imminent peril group call.

###### 7.1.2.5.2.3	MCVideo imminent peril group call cancel

The procedure focuses on the case where an authorized MCVideo user cancels an MCVideo group's in-progress imminent peril state.

Procedures in figure 7.1.2.5.2.3-1 are the signalling control plane procedures for the MCVideo client cancelling an MCVideo group's in-progress imminent peril state.

NOTE 1:	The end of the imminent peril call does not cancel the MCVideo group's in-progress imminent peril state. It is explicitly cancelled by an authorized user.

Pre-conditions:

1.	The MCVideo group is previously defined on the group management server with MCVideo client 1, MCVideo client 2 and MCVideo client 3 affiliated to that MCVideo group.

2.	All members of the MCVideo group belong to the same MC system.

3.	The MCVideo group is an in-progress imminent peril state and has prioritized bearer support.

4.	MCVideo group members have been notified about the MCVideo group's in-progress imminent peril state.

5.	MCVideo client 1 previously initiated the in-progress imminent peril.

Figure 7.1.2.5.2.3-1: MCVideo imminent peril group call cancel

1.	The user at the MCVideo client 1 initiates an imminent peril cancel.

2.	MCVideo client 1 sends an MCVideo imminent peril group call cancel request to the MCVideo server.

3.	The MCVideo server adjusts the priority of the underlying bearer; priority treatment is no longer required. The MCVideo server cancels/resets the in-progress imminent peril state.

4.	MCVideo server resolves the MCVideo group ID to determine the members of that MCVideo group and their affiliation status, based upon the information from group management server.

5.	The MCVideo server sends an MCVideo imminent peril group call cancel request to the MCVideo group members.

6.	MCVideo group members are notified of the in-progress imminent peril cancel.

7.	The receiving MCVideo group members send the MCVideo imminent peril group call cancel response to the MCVideo server to acknowledge the in-progress MCVideo imminent peril group call cancel request. For a multicast scenario, these acknowledgements are not set.

8.	The MCVideo server sends the MCVideo imminent peril group call cancel response to the MCVideo user 1 to confirm the MCVideo imminent peril group call cancel request.

NOTE 2:	Step 8 can occur at any time following step 4, depending on the conditions to proceed with the call.

#### 7.1.2.6	MCVideo emergency alert (on-network and off-network)

The MCVideo server shall support the procedures and related information flows as specified in subclauses 10.10 of 3GPP TS 23.280 [6] with the following clarifications:

-	The MC service ID is the MCVideo ID;

-	The MC service server is the MCVideo server;

-	The MC service group ID is the MCVideo group ID and

-	The MC service user profile index is the MCVideo user profile index.

### 7.1.3	Off-network group communications

#### 7.1.3.1	General

This subclause contains procedures for group communications directly between multiple MCVideo clients without involving the network to provide associated functions. Off-network MCVideo communications are based on ProSe capabilities as described in clause 7.18.

The group communication setup is described in subclause 7.1.3.3 and two variations of joining an existing communication group communication are described, the passive variant in subclause 7.1.3.4 and the active variant in subclause 7.1.3.5.

NOTE:	When the MCVideo client receives a message that is not addressed to either its MCVideo ID or to any MCVideo group of which the MCVideo client is an affiliated group member, the MCVideo client ignores the message.

#### 7.1.3.2	Information flows for off-network group communications

##### 7.1.3.2.1	Group communication announcement

Table 7.1.3.2.1-1 describes the information flow for the group communication announcement sent from the MCVideo client to other MCVideo clients.

Table 7.1.3.2.1-1: Group communication announcement

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user sending the announcement |
| MCVideo group ID | M | The MCVideo group ID towards which the announcement was sent |
| Media codec | M | The media characteristics like codec, resolution, frame rate and video mode to be used for the communication |
| Multi-cast port | M | Multicast port number for media |
| Transmission control port number | M | Port number for transmission control protocol |
| Announcement period | M | Period of the group communication announcement |
| Encryption parameters | O | Encryption parameters to be used for the communication, if the communication is to be encrypted |
| Confirm mode indication | O | Indicates whether the receiving MCVideo user needs to confirm participation |
| Emergency indication | O | Indicates that the MCVideo group communication is an MCVideo emergency communication |
| Imminent peril indication | O | Indicates that the MCVideo group communication is an MCVideo imminent peril communication |
| Broadcast communication indication | O | Indicates that the MCVideo communication is a broadcast communication |
| Video push indication | O | Indicates that the MCVideo communication is for video push |
| Video pull indication | O | Indicates that the MCVideo communication is for video pull |

##### 7.1.3.2.2	Group communication answer response

Table 7.1.3.2.2-1 describes the information flow for the group communication answer response sent from the MCVideo client to other MCVideo clients.

Table 7.1.3.2.2-1: Group communication answer response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user sending the response |
| MCVideo group ID | M | The identity of the MCVideo group on which the call is requested |

##### 7.1.3.2.3	MCVideo upgrade to emergency group communication

Table 7.1.3.2.3-1 describes the information flow for the MCVideo upgrade to emergency sent from the MCVideo client to other MCVideo clients.

Table 7.1.3.2.3-1: MCVideo upgrade to emergency group communication

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user upgrading the communication |
| MCVideo group ID | M | The MCVideo group ID towards which the upgrade was sent |

##### 7.1.3.2.4	MCVideo emergency group communication cancel

Table 7.1.3.2.4-1 describes the information flow for the MCVideo emergency group communication cancel sent from the MCVideo client to other MCVideo clients.

Table 7.1.3.2.4-1: MCVideo emergency group communication cancel

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user cancelling the upgrade |
| MCVideo group ID | M | The MCVideo group ID towards which the cancel was sent |

##### 7.1.3.2.5	MCVideo upgrade to imminent peril group communication

Table 7.1.3.2.5-1 describes the information flow for the MCVideo upgrade to imminent peril sent from the MCVideo client to other MCVideo clients.

Table 7.1.3.2.5-1: MCVideo upgrade to imminent peril group communication

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user upgrading the communication |
| MCVideo group ID | M | The MCVideo group ID towards which the upgrade was sent |

##### 7.1.3.2.6	MCVideo imminent peril group communication cancel

Table 7.1.3.2.6-1 describes the information flow for MCVideo imminent peril group communication cancel sent from the MCVideo client to other MCVideo clients.

Table 7.1.3.2.6-1: MCVideo imminent peril group communication cancel

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user cancelling the upgrade |
| MCVideo group ID | M | The MCVideo group ID towards which the cancel was sent |

##### 7.1.3.2.7	MCVideo emergency alert announcement

Table 7.1.3.2.7-1 describes the information flow for the MCVideo emergency alert announcement sent from the MCVideo client to the other MCVideo clients, for the procedures defined in 3GPP TS 23.280 [6].

Table 7.1.3.2.7-1: MCVideo emergency alert announcement

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user sending emergency alert |
| MCVideo group ID | M | The MCVideo group ID towards which the alert was sent |
| Organization name | M | The alerting user's mission critical organization name |
| Location | O | The alerting client's location, if known |

##### 7.1.3.2.8	MCVideo emergency alert cancel announcement

Table 7.1.3.2.8-1describes the information flow for the MCVideo emergency alert cancel sent from the MCVideo client to other MCVideo clients, for the procedures defined in 3GPP TS 23.280 [6].

Table 7.1.3.2.8-1: MCVideo emergency alert cancel announcement

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user cancelling emergency alert |
| MCVideo group ID | M | The MCVideo group ID towards which the cancel was sent |

#### 7.1.3.3	Group communication setup

##### 7.1.3.3.1	General

A group communication setup is initiated by one member of a pre-configured MCVideo group and ends up with an established MCVideo group between multiple MCVideo clients ready for video sharing.

##### 7.1.3.3.2	Procedure

Figure 7.1.3.3.2-1 describes procedures to establish a MCVideo group communication with other MCVideo clients.

Once the MCVideo group communication is established the MCVideo clients send Group communication announcement periodically.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1 to client N.

2.	MCVideo client 1 to MCVideo client N are members of the same MCVideo group.

3.	MCVideo user 1 has initiated MCVideo group communication.

Figure 7.1.3.3.2-1: MCVideo group communication setup

1.	MCVideo client 1 sends a Group communication announcement message to the MCVideo group.

2.	Upon receiving the Group communication announcement message, all MCVideo clients that are not part of an ongoing group communication for the MCVideo group indicated in the Group communication announcement message set parameters for the media plane as described in the Group communication announcement message.

3.	The MCVideo clients send a group communication answer response to the MCVideo group confirming the participation in the MCVideo group communication. On receiving a group communication answer response message from at least one MCVideo client, other MCVideo clients need not send a group communication answer response message. If the Group communication announcement message included a confirm mode indication, then the MCVideo clients should send a group communication answer response message.

NOTE 1:	MCVideo client 1 should wait for at least one group communication answer response message before transmitting video to the MCVideo group.

4.	MCVideo client 1 checks the participants of the MCVideo group communication through the received group communication answer response messages.

NOTE 2:	Due to the movement of the participants (in and out of the radio coverage) during the off-network group communication, the Group communication announcement message, including parameters for media delivery, is periodically sent.

NOTE 3:	Group communication announcement may include user ID, group ID, media type codec, control port for transmission video, video resolution and video frame rate, video mode, last chage time of video mode etc.

5.	MCVideo client 1, MCVideo client 2 and MCVideo client 3 have successfully established the media plane and transmission control for communication.

#### 7.1.3.4	Passive join to group communication

##### 7.1.3.4.1	General

There is ongoing group communication between multiple MCVideo clients. Another MCVideo client, after becoming aware of that group communication and after it has received periodically sent communication parameters, joins the group with or without sending an indication to other group members.

##### 7.1.3.4.2	Procedure

Figure 7.1.3.4.2-1 describes procedures to join a MCVideo group communication passively.

The MCVideo client X passively waits for a Group communication announcement message. Upon receiving a periodic Group communication announcement message, MCVideo client X establishes media plane and joins the MCVideo group communication.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1 to client N and client X.

2.	MCVideo client 1 to MCVideo client N and MCVideo client X are members of the same MCVideo group.

3.	MCVideo client 1 to MCVideo client N are part of an ongoing MCVideo group communication.

Figure 7.1.3.4.2-1: Passive join to MCVideo group communication

1.	Another MCVideo client X enters in the communication range after the MCVideo group communication establishment. The MCVideo client X does not do anything and passively waits for a periodic Group communication announcement message.

2.	An MCVideo client which is part of the ongoing MCVideo group communication eventually sends a periodic Group communication announcement message.

3.	Upon receiving the periodic Group communication announcement message MCVideo client X sets the parameters for  media plane as described in the Group communication announcement message.

4.	If the Group communication announcement message included a confirm mode indication, MCVideo client X may sends a group communication answer response towards MCVideo group to inform all MCVideo group members of its participation the MCVideo group communication.

5.	MCVideo group communication continues with the MCVideo client X.

#### 7.1.3.5	Active join to group communication

##### 7.1.3.5.1	General

There is ongoing group communication between multiple MCVideo clients. Another MCVideo client, after becoming aware of that group communication, sends a request to get the required communication parameters and after feedback joins the group with or without sending an indication to other group members.

##### 7.1.3.5.2	Procedure

Figure 7.1.3.5.2-1 describes procedures to join a MCVideo group communication actively.

The MCVideo client X sends a Group communication announcement message towards the MCVideo group.

Upon receiving a Group communication announcement message with new communication parameters, such as communication identifier, a MCVideo client participating in the on-going group communication for the MCVideo group sends a Group communication announcement message.

The Group communication announcement message contains parameters for the on-going MCVideo group communication. MCVideo client X upon receiving such a Group communication announcement message establishes the media plane as described in the message and joins the MCVideo group communication.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1 to client N and client X.

2.	MCVideo client 1 to MCVideo client N and MCVideo client X are members of the same MCVideo group.

3.	MCVideo client 1 to MCVideo client N are part of an ongoing MCVideo group communication.

Figure 7.1.3.5.2-1: Active join to MCVideo group communication

1.	Another MCVideo client X enters in the communication range after the MCVideo group communication establishment. The MCVideo client X sends a Group communication announcement message to start a newMCVideo group communication.

2.	Upon receiving a Group communication announcement message with new communication parameters, such as communication identifier, another MCVideo client, which is part of the ongoing MCVideo group communication sends a Group communication announcement message.

3.	Upon receiving the Group communication announcement message MCVideo client X sets parameters for the media plane as described in the Group communication announcement message.

4.	If the Group communication announcement message included a confirm mode indication, MCVideo client X may send a group communication answer response towards MCVideo group to inform all MCVideo group members of its participation the MCVideo group communication.

5.	MCVideo group communication continues with the MCVideo client X.

#### 7.1.3.6	Broadcast group communication

Figure 7.1.3.6-1 describes procedures for broadcast group communication.

Broadcast group communication over group-broadcast group and over user-broadcast group are handled without distinction in off-network.

Figure 7.1.3.6-1: Broadcast group communication

1.	MCVideo client 1 initiates group communication setup as described in subclause 7.1.3.3. The group communication announcement message includes the indication of broadcast group communication.

2.	MCVideo client 1 initiating broadcast group communication starts to transmit video.

NOTE:	Other clients of the group member are not allowed to transmit media in broadcast group communication.

3.	The broadcast group communication is released as described in subclause 7.1.3.7.

#### 7.1.3.7	Group communication release due to inactivity

If the on-going MCVideo group communication is inactive for a specific duration, the participating MCVideo clients release the MCVideo group communication. Inactivity is defined as absence of any transmission to or from other communication participants.

NOTE:	Inactivity time can be set according to the policy of MC service provider.

#### 7.1.3.8	Emergency and imminent peril procedures

##### 7.1.3.8.1	Emergency group communication

The off-network emergency group communication is a special case of off-network (non-emergency) group communication as defined in subclause 7.1.3.2, subclause 7.1.3.3, subclause 7.1.3.4, subclause 7.1.3.5, subclause 7.1.3.6 and subclause 7.1.3.7. The following are modifications to the aforementioned subclauses to support MCVideo emergency group communications:

-	As a pre-condition, the client initiating the emergency communication has previously been provisioned with an MCVideo group designated as the MCVideo emergency group. The MCVideo client initiates MCVideo emergency group communications on this group. Alternatively, the MCVideo client could have been provisioned for emergency behaviour on the selected MCVideo group.

-	The group communication announcement contains an indication that the MCVideo group communication is to be an MCVideo emergency communication regardless of whether or not the MCVideo client is in communication. Group communication participants learn of the MCVideo group's in-progress emergency state from the indication.

-	Every communication initiated by the MCVideo client will be an emergency communication while the MCVideo client is in MCVideo emergency state.

-	The MCVideo client enters the MCVideo emergency state when initiating an MCVideo emergency communication or emergency alert. Only the MCVideo user of the MCVideo client can clear the client's local MCVideo emergency state.

-	The MCVideo group in-progress emergency state is cancelled when the communication ends.

-	The MCVideo user who initiated the emergency communication, or upgraded the group communication to emergency group communication, or an authorized user, may cancel the in-progress emergency state with an MCVideo emergency group communication cancel message. The message is sent to the MCVideo group.

-	When the MCVideo group is no longer in the in-progress emergency state, every client returns the value of ProSe Per Packet Priority to its default value.

-	An in-progress MCVideo group communication is upgraded to an emergency group communication when the periodic group communication announcement contains the emergency indicator.

-	The value of ProSe Per Packet Priority is upgraded according to emergency state of MCVideo group communication.

-	An MCVideo group communication upgraded to emergency group communication does not affect the state of other MCVideo communication(s) in the client.

-	The MCVideo group remains in the in-progress emergency state until the emergency group communication ends or the in-progress emergency state is cancelled.

##### 7.1.3.8.2	MCVideo imminent peril

The off-network imminent peril group communication is a special case of off-network (non-imminent peril) group communication as defined in subclause 7.1.3.2, subclause 7.1.3.3, subclause 7.1.3.4, subclause 7.1.3.5, subclause 7.1.3.6 and subclause 7.1.3.7. The following are modifications to the aforementioned subclauses to support MCVideo imminent peril communications:

-	As a pre-condition, the user initiating the imminent peril communication has previously been provisioned with an MCVideo group to be used as the MCVideo imminent peril group. The MCVideo client initiates MCVideo imminent peril group communications on this group. Alternatively, the MCVideo client could have been provisioned for imminent peril behaviour on the selected MCVideo group.

-	The group communication announcement contains an indication that the MCVideo group communication is to be an MCVideo imminent peril communication. Group communication participants learn of the MCVideo group's in-progress imminent peril condition from the indication.

-	The MCVideo group in-progress imminent peril state is considered cancelled when the communication ends.

-	The MCVideo user who initiated the in-progress imminent peril communication or upgraded the group communication to imminent peril group communication, or an authorized user may cancel the in-progress imminent peril state with an MCVideo imminent peril group communication cancel message. The message is sent to the communication participants.

-	When the MCVideo group is no longer in the in-progress imminent peril state, every MCVideo client returns the value of ProSe Per Packet Priority to its default value.

-	An in-progress MCVideo group communication is upgraded to an imminent peril group communication when the periodic group communication announcement contains the imminent peril indicator.

-	The value of ProSe Per Packet Priority is upgraded according to imminent peril state of MCVideo group communication.

-	The MCVideo group remains in the in-progress imminent peril state until the imminent peril group communication ends or the in-progress imminent peril state is cancelled.

#### 7.1.3.9	MCVideo emergency alert

##### 7.1.3.9.1	General

The MCVideo client use emergency alert initiation and emergency alert cancel procedures defined in 3GPP TS 23.280 [6], with the following clarifications:

-	The MC service ID is the MCVideo ID; and

-	The MC service group ID is the MCVideo group ID.

## 7.2	Private call

### 7.2.1	General

Private calls are enabled in both on-network and off-network.

Private calls can be setup in two different commencement modes, automatic commencement mode and manual commencement mode.

### 7.2.2	Private call on-network

#### 7.2.2.1	General

The private call setup in automatic commencement mode is described in subclause 7.2.2.3.1, manual commencement mode in subclause 7.2.2.3.2 and private call release (client or server initiated) in subclause 7.2.2.3.3.

#### 7.2.2.2	Information flows for private call in on-network

##### 7.2.2.2.1	MCVideo private call request (MCVideo client – MCVideo server)

Table 7.2.2.2.1-1 describes the information flow MCVideo private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.1-1: MCVideo private call request (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID (see NOTE) | O | The MCVideo ID of the called party |
| Functional alias (see NOTE) | O | The functional alias of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request | O | An indication that the user is also requesting the permission to transmit video |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| Requested priority | O | Application priority level requested for this call |
| NOTE: Either an MCVideo ID or a functional alias shall be present, but not both. |  |  |

##### 7.2.2.2.2	MCVideo private call request

Table 7.2.2.2.2-1 describes the information flow MCVideo private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.2-1: MCVideo private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Transmit media request indication | M | This element indicates whether transmission control will be used for the private call. |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Implicit transmit media request (see NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| Push indication | O | Indicates that the private call request is for a one-to-one push call |
| Pull indication | O | Indicates that the private call request is for a one-to-one pull call |
| NOTE: This element shall not be included when the request is sent from the MCVideo server to the MCVideo client. |  |  |

##### 7.2.2.2.3	MCVideo private call response (MCVideo client – MCVideo server)

Table 7.2.2.2.3-1 describes the information flow MCVideo private call response from the MCVideo client to the MCVideo server.

Table 7.2.2.2.3-1: MCVideo private call response (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | O | The MCVideo ID of the called party |
| SDP answer | M | Media parameters selected |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request (see NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| NOTE: This element shall be included when the response is associated to a request including a pull indication. |  |  |

##### 7.2.2.2.4	MCVideo private call response

Table 7.2.2.2.4-1 describes the information flow MCVideo private call response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.4-1: MCVideo private call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | O | The MCVideo ID of the called party |
| Acceptance confirmation | O | An indication whether the user has positively accepted the call. |
| SDP answer | M | Media parameters selected |
| Implicit transmit media request (see NOTE) | O | An indication that the user is also requesting the permission to transmit video |
| NOTE: This element shall not be included when the request is sent from the MCVideo server to the MCVideo client. |  |  |

##### 7.2.2.2.5	MCVideo call end request

Table 7.2.2.2.5-1 describes the information flow MCVideo call end request from the MCVideo client to the MCVideo server, from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.5-1: MCVideo call end request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| MCVideo call end reason | O | Indicate that the reason to end the call, e.g., due to migration, or due to user request. |

##### 7.2.2.2.6	MCVideo emergency private call request (MCVideo client to MCVideo server)

Table 7.2.2.2.6-1 describes the information flow MCVideo emergency private call request from the MCVideo client to the MCVideo server.

Table 7.2.2.2.6-1: MCVideo emergency private call request (MCVideo client to MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media |
| SDP offer | M | Media parameters of MCVideo client. |
| Requested priority | O | Priority level requested for the call |

##### 7.2.2.2.7	MCVideo emergency private call request

Table 7.2.2.2.7-1 describes the information flow MCVideo emergency private call request from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.7-1: MCVideo emergency private call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Emergency indicator | M | Indicates that the private call request is an MCVideo emergency call |
| Alert indicator | M | Indicates whether an emergency alert is to be sent |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit transmit media request | O | An indication that the user is also requesting to transmit media. |
| SDP offer | M | Media parameters of MCVideo client. |

##### 7.2.2.2.8	Emergency MCVideo private call response (MCVideo client – MCVideo server)

Table 7.2.2.2.8-1 describes the information flow emergency MCVideo private call response from the MCVideo client to the MCVideo server.

Table 7.2.2.2.8-1: Emergency MCVideo private call response (MCVideo client – MCVideo server)

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | O | The MCVideo ID of the called party |
| SDP answer | M | Media parameters selected |
| Requested commencement mode | O | An indication of the commencement mode to be used. |

##### 7.2.2.2.9	Emergency MCVideo private call response

Table 7.2.2.2.9-1 describes the information flow Emergency MCVideo private call response from the MCVideo server to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.2.2.2.9-1: Emergency MCVideo private call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | O | The MCVideo ID of the called party |
| Acceptance confirmation | O | An indication whether the user has positively accepted the call. |
| SDP answer | M | Media parameters selected |

##### 7.2.2.2.10	MCVideo progress indication

Table 7.2.2.2.10-1 describes the information flow MCVideo progress indication from the MCVideo server to the MCVideo client.

Table 7.2.2.2.10-1: MCVideo progress indication information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | M | The MCVideo ID of the called party |
| Progress indication | O | Indication to the caller. |

##### 7.2.2.2.11	MCVideo functional alias resolution response

Table 7.2.2.2.11-1 describes the information flow MCVideo functional alias resolution response from the MCVideo server to the MCVideo client.

Table 7.2.2.2.11-1: MCVideo functional alias resolution response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| MCVideo ID | M | The corresponding MCVideo ID of the called functional alias |

#### 7.2.2.3	Private call within one MC system

##### 7.2.2.3.1	Private call setup in automatic commencement mode

The procedure describes the scenario where an MCVideo user is initiating an MCVideo private call for communicating with another MCVideo user, with or without transmission control enabled, in an automatic commencement mode.

Procedures in figure 7.2.2.3.1-1 are the basic signalling control plane procedures for the MCVideo client initiating establishment of MCVideo private call with the chosen MCVideo user.

Pre-conditions:

1.	MCVideo users on MCVideo client 1 and MCVideo client 2 are already registered for receiving MCVideo service.

2.	The calling MCVideo user has selected automatic commencement mode for the call; or

3.	The called MCVideo client is set to automatic commencement mode.

4.	Optionally, the MCVideo client 1 may have a functional alias activated to be used.

5.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.2.2.3.1-1: Private call setup in automatic commencement mode– MCVideo users in the same MC system

1.	User at MCVideo client 1 would like to initiate an MCVideo private call for the chosen MCVideo user. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo private call.

2.	MCVideo client 1 sends an MCVideo private call request towards the MCVideo server (via SIP core) using a service identifier as defined in 3GPP TS 23.228 [5] for MCVideo, for establishing a private call with the chosen MCVideo user. The MCVideo private call request contains the MCVideo ID or the functional alias of invited user, an SDP offer containing one or more media types. The MCVideo client 1 may include a Requested commencement mode that indicates that the call is to be established in automatic commencement mode if automatic commencement mode is requested by the initiating user.

NOTE 1:	As part of this step, MCVideo client 1 and MCVideo client 2 set up a security association (when no functional alias is present), if end-to-end encryption is used for this call.

3.	MCVideo server checks whether the MCVideo user at MCVideo client 1 is authorized to initiate the private call, and that MCVideo user at MCVideo client 2 is authorized to receive the private call. MCVideo server verifies whether the provided functional alias, if present, can be used and has been activated for the user. If the MCVideo private call request contains a functional alias instead of an MCVideo ID as called party, the MCVideo server shall resolve the functional alias to the corresponding MCVideo ID(s) for which the functional alias is active. The MCVideo server shall also check whether MCVideo client 1 is allowed to use the functional alias of MCVideo client 2 to setup a private call and whether MCVideo client 2 is allowed to receive a private call from MCVideo client 1 using the functional alias. If the MCVideo private call request requested automatic commencement mode then the MCVideo server also checks whether the MCVideo user at MCVideo client 1 is authorized to initiate a private call in automatic commencement mode.

NOTE 2:	When a functional alias is shared among multiple MCVideo users, only one target MCVideo user can be target of the private call request. The MCVideo server resolves the associated MCVideo IDs of the functional alias and determines a MCVideo ID by help of criteria not defined here (e.g. location, time etc). This determination can include rejection of the call, if no suitable MCVideo ID is selected.

4a.	If the MCVideo private call request contains only the functional alias instead of an MCVideo ID for the called party, the MCVideo server responds with a functional alias resolution response message that contains the resolved MCVideo ID back to MCVideo client 1.

4b.	If the MCVideo server replies with a MCVideo functional alias resolution response message, the MCVideo client 1 abandons the first MCVideo private call request in step 2 and sends a new MCVideo private call request towards the resolved MCVideo ID.

NOTE 3:	MCVideo client 1 and MCVideo client 2 set up a security association for the media, if end-to-end encryption is used for this call.

5.	MCVideo server may provide a progress indication to MCVideo client 1 to indicate progress in the call setup process.

NOTE 4:	Step 5 can occur at any time following step 4b, and prior to step 9.

6.	If authorized, MCVideo server includes information that it communicates using MCVideo service, offers the same media types or a subset of the media types contained in the initial received request, includes the requested automatic commencement mode indication based on a requested automatic commencement mode by the calling user or based upon the setting of the called MCVideo client and sends the corresponding MCVideo private call request towards the MCVideo client 2, including the MC service ID and, if available the functional alias of the calling MCVideo user 1. If the called MCVideo user has registered to the MCVideo service with multiple MCVideo UEs and has designated the MCVideo UE for receiving the private calls, then the incoming MCVideo private call request is delivered only to the designated MCVideo UE.

7.	The receiving MCVideo client 2 notifies the user about the incoming private call. If the functional alias of the calling user is included, it is displayed.

8.	The receiving MCVideo client 2 accepts the private call automatically, and an MCVideo private call response is sent to the MCVideo server (via SIP core).

9.	Upon receiving the MCVideo private call response from MCVideo client 2 accepting the private call request, the MCVideo server informs the MCVideo client 1 about successful call establishment.

10.	MCVideo client 1 and MCVideo client 2 have successfully established media plane and transmission control for communication and both users can transmit media.

##### 7.2.2.3.2	Private call setup in manual commencement mode

###### 7.2.2.3.2.1	Description

Figure 7.2.2.3.2.2-1 describes the basic procedure for the MCVideo client initiating an MCVideo private call that uses manual commencement mode. The flow may use a transmit media request in the MCVideo private call request indicating that the originator will be granted permission to transmit when the call starts and eliminates the need for a separate initial transmit media request message during media plane establishment. Alternatively, the call initiation may be sent without the transmit media request, which allows the called party to transmit media request first.

###### 7.2.2.3.2.2	Procedure

Both clients are served by the primary MC service provider in figure 7.2.2.3.2.2-1.

Pre-conditions:

1.	MCVideo client 1 and MCVideo client 2 are both registered and their respective users, MCVideo user 1 and MCVideo user 2, are authenticated and authorized to use the MCVideo service.

2.	The calling MCVideo user has selected manual commencement mode or has not specified a commencement mode for the call; and

3.	The called MCVideo client is set to manual commencement mode.

4.	Optionally, the MCVideo client 1 may have a functional alias activated to be used.

5.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.2.2.3.2.2-1: MCVideo private call in manual commencement mode– MCVideo users in the same MC system

1.	MCVideo user at MCVideo client 1 would like to initiate an MCVideo private call for the selected MCVideo user. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo private call.

2.	MCVideo client 1 sends an MCVideo private call request addressed to the MC service ID of MCVideo user 2 using an MCVideo service identifier as defined in 3GPP TS 23.228 [5] (possible for the SIP core to route the request to the MCVideo server). The MCVideo private call request contains the MC service ID or the functional alias of invited user and an SDP offer containing one or more media types. The MCVideo client 1 may include a requested commencement mode that indicates that the call is to be established in manual commencement mode if manual commencement mode is requested by the initiating user.

NOTE 1:	As part of this step, MCVideo client 1 and MCVideo client 2 set up a security association (when no functional alias is present), if end-to-end encryption is used for this call.

3.	The MCVideo server confirms that both MCVideo users are authorized for the private call. MCVideo server verifies whether the provided functional alias, if present, can be used and has been activated for the user. If the MCVideo private call request contains a functional alias instead of an MCVideo ID as called party, the MCVideo server shall resolve the functional alias to the corresponding MCVideo ID(s) for which the functional alias is active. The MCVideo server shall also check whether MCVideo client 1 is allowed to use the functional alias of MCVideo client 2 to setup a private call and whether MCVideo client 2 is allowed to receive a private call from MCVideo client 1 using the functional alias. The MCVideo server checks the commencement mode setting of the called MCVideo client and also checks whether the MCVideo user at MCVideo client 1 is authorized to initiate a call in manual commencement mode.

NOTE 2:	When a functional alias is shared among multiple MCVideo users, only one target MCVideo user can be target of the private call request. The MCVideo server resolves the associated MCVideo IDs of the functional alias and determines a MCVideo ID by help of criteria not defined here (e.g. location, time etc). This determination can include rejection of the call, if no suitable MCVideo ID is selected.

4a.	If the MCVideo private call request contains only the functional alias instead of an MCVideo ID for the called party, the MCVideo server responds with a functional alias resolution response message that contains the resolved MCVideo ID back to MCVideo client 1.

4b.	If the MCVideo server provided the corresponding MCVideo ID, the MCVideo client 1 abandons the first MCVideo private call request in step 2 and sends a new MCVideo private call request containing the resolved MCVideo ID.

NOTE 3:	MCVideo client 1 and MCVideo client 2 set up a security association for the media, if end-to-end encryption is used for this call.

5.	The MCVideo server includes information that it communicates using MCVideo service, offers the same media types or a subset of the media types contained in the initial received request and sends an MCVideo private call request for the call to MCVideo client 2, including the MC service ID, and, if available the functional alias of the calling MCVideo user 1. If the called MCVideo user has registered to the MCVideo service with multiple MCVideo UEs and has designated the MCVideo UE for receiving the private calls, then the incoming MCVideo private call request is delivered only to the designated MCVideo UE.

6.	MCVideo server may provide a progress indication to MCVideo client 1 to indicate progress in the call setup process.

NOTE 4:	Step 6 can occur at any time following step 4b, and prior to step 7b.

7a.	The MCVideo user is alerted. MCVideo client 2 sends an MCVideo ringing to the MCVideo server.

7b.	The MCVideo server sends an MCVideo ringing to MCVideo client 1, indicating that MCVideo client 2 is being alerted. If the functional alias of the calling user is included, it is displayed.

8.	MCVideo user 2 is notified and has accepted the call using manual commencement mode (i.e., has taken some action to accept via the user interface).

9.	The MCVideo client 2 sends an MCVideo private call response to the MCVideo server. If MCVideo user 2 has not accepted the incoming call, the MCVideo client 2 sends a call failure response to the MCVideo server without adding reason for call failure.

10.	The MCVideo server sends an MCVideo private call response to MCVideo client 1 indicating that MCVideo user 2 has accepted the call, including the accepted media parameters.

11.	The media plane and transmission control for communication is established.

##### 7.2.2.3.3	Private call release

###### 7.2.2.3.3.1	Client initiated

The procedure describes the scenario where an MCVideo client is requesting to release an ongoing MCVideo private call (with or without transmission control) and the call established in either of the two commencement modes (manual or automatic).

Procedures in figure 7.2.2.3.3.1-1 are the basic signalling control plane procedures for the MCVideo client initiating the release of an ongoing MCVideo private call.

Pre-condition:

1.	It is assumed that MCVideo users on MCVideo client 1 and MCVideo client 2 are already registered for receiving MCVideo service and are involved in private call as described in subclause 7.2.2.3.1 and subclause 7.2.2.3.2.

Figure 7.2.2.3.3.1-1: Private call release – client initiated

1.	User at MCVideo client 1 would like to release an ongoing MCVideo private call with MCVideo client 2.

2.	MCVideo client 1 sends an MCVideo call end request towards the MCVideo server (via SIP core), for tearing down the private call with the other client.

3.	MCVideo server sends the corresponding MCVideo call end request towards the MCVideo client specified in the original MCVideo call end request.

4.	MCVideo user is notified about the release of the private call.

5.	The receiving MCVideo client 2 acknowledges the MCVideo call end request with a MCVideo call end response.

6.	After receiving the MCVideo call end response from MCVideo client 2, the MCVideo server generates an MCVideo call end response for the MCVideo client 1's MCVideo call end request.

7.	MCVideo clients release all the media plane resources used for the private call. Further, if the private call was established with transmission control, transmission control resources are released.

###### 7.2.2.3.3.2	Server initiated

The procedure describes the scenario where an MCVideo server is terminating an ongoing MCVideo private call  and the call established in either of the two commencement modes (manual or automatic), upon conditions to terminate call e.g., MCVideo administrator configured maximum duration for MCVideo private calls has expired or timed out due to MCVideo private call without transmission/reception.

Procedures in figure 7.2.2.3.3.2-1 are the basic signalling control plane procedures for the MCVideo server initiating termination of an ongoing MCVideo private call.

Pre-condition:

1.	It is assumed that MCVideo users on MCVideo client 1 and MCVideo client 2 are already registered for receiving MCVideo service and are involved in private call established either in manual or automatic commencement mode.

Figure 7.2.2.3.3.2-1: End private call – server initiated

1.	Upon conditions to terminate call e.g., MCVideo administrator configured maximum duration for MCVideo private calls has expired or timed out due to MCVideo private call without transmission/reception, the MCVideo server decides to initiate termination of an ongoing MCVideo private call between MCVideo client 1 and MCVideo client 2.

2.	MCVideo server sends an MCVideo call end request towards the MCVideo clients 1 and 2 (via SIP core), for tearing down the private call between them.

3.	MCVideo users at client 1 and client 2 are notified about the termination of the private call.

4.	The MCVideo call end request receiving MCVideo clients 1 and 2 acknowledge the request with MCVideo call end response.

5.	MCVideo clients release all the media plane resources used for the private call. Further, if the private call was established with transmission control, transmission control resources are released.

#### 7.2.2.4	MCVideo emergency private call

##### 7.2.2.4.1	MCVideo emergency private call commencement

This procedure describes the case where an authorized MCVideo user is initiating an MCVideo emergency private call with unicast signalling for communicating with another MCVideo user. An MCVideo client in the MCVideo emergency state gains elevated access privilege for all of the MCVideo user's mission critical applications.

Procedures in figure 7.2.2.4.1-1 are the procedures for the MCVideo client initiating establishment of an MCVideo emergency private call.

Pre-conditions:

1.	Both members of the MCVideo private call belong to the same MCVideo system.

2.	The initiating MCVideo client 1 has been configured to send an MCVideo emergency alert prior to initiating an MCVideo emergency private call.

3.	Optionally, the MCVideo client 1 may have a functional alias activated to be used.

4.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.2.2.4.1-1 MCVideo emergency private call

1.	The user at the MCVideo client 1 initiates an MCVideo emergency private call. MCVideo client 1 sets its MCVideo emergency state. MCVideo client 1 retains the MCVideo emergency state until explicitly cancelled by the user of MCVideo client 1.

NOTE 1:	While MCVideo client 1 is in the emergency state, all MCVideo group and private calls initiated by MCVideo client 1 are initiated as MCVideo emergency calls.

2.	MCVideo client 1 sends an MCVideo emergency private call request towards the MCVideo server. The request contains an indication of the MCVideo emergency. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo emergency private call. The MCVideo server verifies whether the provided functional alias can be used. The MCVideo server records the identity of the MCVideo user that initiated the MCVideo emergency private call until the MCVideo emergency is cancelled. If the MCVideo client is configured to send an MCVideo emergency alert when initiating an MCVideo emergency private call, the request also contains an indication that an MCVideo emergency alert is to be initiated.

3.	MCVideo server sends the MCVideo emergency private call request towards the target MCVideo client. The request contains an indication of the in-progress emergency. The request contains an indication of an MCVideo emergency alert if the request from the originator indicated MCVideo emergency alert. If available the request contains the functional alias of the calling MCVideo user 1.

4.	The MCVideo user on MCVideo client 2 is notified of the incoming MCVideo emergency private call. If the functional alias of the calling user is included it is displayed.

NOTE 2:	MCVideo client 2 does not set its emergency state as a result of receiving the MCVideo emergency private call.

5.	The receiving MCVideo client acknowledges the MCVideo emergency private call request to the MCVideo server via a MCVideo emergency private call response.

6.	The MCVideo server adjusts the priority of the underlying bearer.

7.	The MCVideo server informs MCVideo client 1 about the successful MCVideo emergency private call establishment via a MCVideo emergency private call response.

8.	MCVideo client 1 and MCVideo client 2 establish the media plane and transmission control for video communication.

NOTE 3:	The priority for the MCVideo emergency private call is retained until cancelled according to system policy (e.g. call end) or cancelled by an authorized MCVideo user.

NOTE 4:	The initiating MCVideo user's MCVideo emergency state is retained by the system until cancelled as in subclause 7.1.2.6, or by the emergency alert cancellation procedure specified in 3GPP TS 23.280 [5] subclause 10.10.1.2.2.2. The initiating MCVideo user's MCVideo emergency state is also retained locally by the MCVideo client until explicitly cancelled by the MCVideo user.

##### 7.2.2.4.2	MCVideo private call emergency upgrade

This procedure describes the case where an authorized MCVideo user is upgrading a private call to an MCVideo emergency private call while the private call is already in progress.

Procedures in figure 7.2.2.4.2-1 are the signalling procedures for the MCVideo client upgrading a private call to an MCVideo emergency private call.

Pre-conditions:

1.	Both members of the private call belong to the same MCVideo system.

2.	A private call is already in progress.

Figure 7.2.2.4.2-1: MCVideo private call upgrade

1.	The MCVideo user at MCVideo client 1 initiates an emergency. MCVideo client 1 sets its MCVideo emergency state. MCVideo client 1 retains the MCVideo emergency state until explicitly cancelled by the user of MCVideo client 1.

NOTE 1:	While MCVideo client 1 is in the emergency state, all MCVideo group and private calls initiated by MCVideo client 1 are initiated as MCVideo emergency calls.

2.	MCVideo client 1 requests the MCVideo server to upgrade the private call to in-progress emergency by sending an MCVideo emergency private call request. If configured to send an MCVideo alert when initiating an MCVideo emergency upgrade, the request also contains an indication that an MCVideo alert is to be initiated. The request may contain an indication of an implicit transmit media request.

3.	MCVideo server sends the MCVideo emergency request towards MCVideo client 2, the MCVideo client of the other participant.

4.	The MCVideo user of MCVideo client 2 is notified of the in-progress emergency of the MCVideo emergency private call.

NOTE 2:	MCVideo client 2 does not set its emergency state as a result of receiving the MCVideo emergency private call upgrade.

5.	The receiving MCVideo client acknowledges the MCVideo emergency request to the MCVideo server.

6.	The MCVideo server adjusts the priority of the underlying bearer for both participants in the private call. The priority is retained until the call ends.

7.	The MCVideo server confirms the upgrade request to MCVideo client 1. If the MCVideo emergency request contained an implicit transmit media request, the MCVideo emergency private call response message contains the result of the implicit transmit media request.

8.	MCVideo client 1 and MCVideo client 2 continue with the private call, which has been transformed into an MCVideo emergency private call. MCVideo client 1 can override MCVideo client 2 unless MCVideo client 2 is also in the MCVideo emergency state.

### 7.2.3	Off-network private communications

#### 7.2.3.1	General

The private call setup in automatic commencement mode is described in subclause 7.2.3.4, manual commencement mode in subclause 7.2.3.5 and private call release in subclause 7.2.3.6.

#### 7.2.3.2	Information flows for off-network private communications

##### 7.2.3.2.1	Private communication request

Table 7.2.3.2.1-1 describes the information flow for the private communication request sent from one MCVideo client to another MCVideo client.

Table 7.2.3.2.1-1: Private communication request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the communication |
| MCVideo ID | M | The identity of the MCVideo user with whom communication was requested |
| SDP offer | M | SDP with media information offered by the caller |
| Video push indication | O | Indicates that the MCVideo private communication is for video push |
| Video pull indication | O | Indicates that the MCVideo private communication is for video pull |

NOTE 1:	For video push, 'the identity of the MCVideo user requesting the communication' refers to the user who will be the source of the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will receive the video.

NOTE 2:	For video pull, 'the identity of the MCVideo user requesting the communication' refers to the user who wants to receive the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will be the source of the video.

##### 7.2.3.2.2	Private communication answer response

Table 7.2.3.2.2-1 describes the information flow for the response of private communication request sent from one MCVideo client to another MCVideo client.

Table 7.2.3.2.2-1: Private communication answer response

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the communication |
| MCVideo ID | M | The identity of the MCVideo user with whom communication was requested |
| SDP answer | M | SDP with media information accepted by the callee |

NOTE 1:	For video push, 'the identity of the MCVideo user requesting the communication' refers to the user who will be the source of the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will receive the video.

NOTE 2:	For video pull, 'the identity of the MCVideo user requesting the communication' refers to the user who wants to receive the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will be the source of the video.

##### 7.2.3.2.3	Private communication release request

Table 7.2.3.2.3-1 describes the information flow for the private communication release request sent from one MCVideo client to another MCVideo client.

Table 7.2.3.2.3-1: Private communication release request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the communication |
| MCVideo ID | M | The identity of the MCVideo user with whom communication was requested |
| MCVideo private communication release reason | O | Reason for the private communication release. |

NOTE 1:	For video push, 'the identity of the MCVideo user requesting the communication' refers to the user who will be the source of the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will receive the video.

NOTE 2:	For video pull, 'the identity of the MCVideo user requesting the communication' refers to the user who wants to receive the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will be the source of the video.

##### 7.2.3.2.4	Private communication release response

Table 7.2.3.2.4-1 describes the information flow for the private communication release response sent from one MCVideo client to another MCVideo client.

Table 7.2.3.2.4-1: Private communication release response

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the communication |
| MCVideo ID | M | The identity of the MCVideo user with whom communication was requested |

NOTE 1:	For video push, 'the identity of the MCVideo user requesting the communication' refers to the user who will be the source of the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will receive the video.

NOTE 2:	For video pull, 'the identity of the MCVideo user requesting the communication' refers to the user who wants to receive the video, and 'the identity of the MCVideo user with whom the communication was requested' refers to the user who will be the source of the video.

#### 7.2.3.3	Use of ProSe for off-network private communications

The use of ProSe capabilities for off-network private communications is described in clause 7.18.

#### 7.2.3.4	Automatic commencement private communication

##### 7.2.3.4.1	General

A private call request is sent by a MCVideo client directly to another MCVideo client, which checks the request, establishes the MCVideo session without further user interaction and notifies its user that the MCVideo session has been established.

##### 7.2.3.4.2	Procedure

Figure 7.2.3.4.2-1 describes procedures to establish an off-network automatic commencement private communication between MCVideo user A MCVideo user B.

Pre-conditions:

1.	MCVideo user A has initiated automatic commencement private communication with MCVideo user B.

2.	MCVideo user B has indicated MCVideo client B as the designated MCVideo client for MCVideo private communications.

3.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client A has discovered MCVideo client B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

Editor's Note:	How MCVideo client A determines that MCVideo client B is the designated MCVideo client of MCVideo user B for private communications is FFS.

Figure 7.2.3.4.2-1: Off-network automatic commencement private communication

1.	The MCVideo client A sends the Private communication request towards MCVideo client associated with MCVideo user B. The Private communication request contains an indication for automatic commencement and the SDP offer.

2.	On receiving a Private communication request with an indication for automatic commencement, the MCVideo client checks if it is the designated MCVideo client for off-network MCVideo private communications.

3a.	The designated MCVideo client B automatically accepts the Private communication request, and sends a Private communication answer response, indicating the successful receipt of communication request to the MCVideo client A. The Private communication answer response contains the SDP answer.

3b.	The designated MCVideo client B notifies the MCVideo user B about the incoming Private communication request.

NOTE 1:	Step 3a and step 3b can occur in any order.

4.	The MCVideo client A and the MCVideo client B establish the media plane for Private Communication.

NOTE 2:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client.

5.	MCVideo media is transmitted from the MCVideo client A to the MCVideo client B and is presented to the MCVideo user B.

NOTE 3:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

#### 7.2.3.5	Manual commencement private communication

##### 7.2.3.5.1	General

A private call request is sent by a MCVideo client directly to another MCVideo client, which checks the request, establishes the MCVideo session without further user interaction and notifies its user about the incoming MCVideo session request. The MCVideo user may accept, reject or ignore the MCVideo session request.

##### 7.2.3.5.2	Procedure – Communication accepted

Figure 7.2.3.5.2-1 describes procedures to establish an off-network manual commencement private communication between MCVideo user A MCVideo user B, which is accepted by the MCVideo user B.

Pre-conditions:

1.	MCVideo user A has initiated manual commencement private communication with MCVideo user B.

2.	MCVideo user B has indicated MCVideo client B as the designated MCVideo client for MCVideo private communications, if the MCVideo user B has signed on to the MCVideo service with multiple MCVideo clients.

3.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client A has discovered MCVideo clients B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

Editor's Note:	How MCVideo client A determines that MCVideo client B is the designated MCVideo client of MCVideo user B for private communications is FFS.

Figure 7.2.3.5.2-1: Off-network manual commencement private communication – Accepted

1.	The MCVideo client A sends the Private communication request towards MCVideo client associated with MCVideo user B. The Private communication request contains an indication for manual commencement and the SDP offer.

2.	On receiving a Private communication request, the MCVideo client checks if it is the designated MCVideo client for off-network MCVideo private communications.

3a.	The designated MCVideo client B presents the incoming Private communication request to the MCVideo user B.

3b.	The MCVideo client B sends back a Private communication ringing response to the MCVideo client A.

NOTE 1:	Step 3a and step 3b can occur in any order.

4.	The MCVideo client A notifies the ringing status to the MCVideo user A.

5.	The MCVideo user B accepts the Private communication request.

6.	The MCVideo client B sends the Private communication answer response to the MCVideo client A. The Private communication response contains the SDP answer.

7.	The MCVideo client A and the MCVideo client B establish the media plane for Private communication.

NOTE 2:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client.

8.	MCVideo media is transmitted from the MCVideo client A to the MCVideo client B and is presented to the MCVideo user B.

NOTE 3:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

##### 7.2.3.5.3	Procedure – Communication rejected/ignored

Figure 7.2.3.5.3-1 describes procedures to initiate an off-network manual commencement private communication between MCVideo user A MCVideo user B, which is rejected or ignored by the MCVideo user B.

Pre-conditions:

1.	MCVideo user A has initiated manual commencement private communication with MCVideo user B.

2.	MCVideo user B has indicated MCVideo client B as the designated MCVideo client for MCVideo Private Communications, if the MCVideo user B has signed on to the MCVideo service with multiple MCVideo clients.

3.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client A has discovered MCVideo clients B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

Editor's Note:	How MCVideo client A determines that MCVideo client B is the designated MCVideo client of MCVideo user B for private communications is FFS.

Figure 7.2.3.5.3-1: Off-network manual commencement private communication – Rejected or Ignored

1.	The MCVideo client A sends the private communication request towards MCVideo client associated with MCVideo user B. The private communication request contains an indication for manual commencement and the SDP offer.

2.	On receiving a Private communication request, the MCVideo client checks if it is the designated MCVideo client for off-network MCVideo private communications.

3a.	The designated MCVideo client B presents the incoming Private communication request to the MCVideo user B.

3b.	The MCVideo client B sends back a Private communication ringing response to the MCVideo client A.

NOTE 1:	Step 3a and step 3b can occur in any order.

4.	The MCVideo client A notifies the ringing status to the MCVideo user A.

5.	The MCVideo user B rejects (or ignores) the Private communication request.

NOTE 2:	The MCVideo client B determines that the MCVideo user B has ignored the Private communication request, if the MCVideo user B does not respond within a configured time limit.

6.	The MCVideo client B sends a Private communication reject response to the MCVideo client A. The Private communication reject response may indicate that the MCVideo user B has ignored the Private communication request.

7.	The MCVideo client A notifies the MCVideo user A that the Private communication request was rejected (or ignored).

#### 7.2.3.6	Private communication release

##### 7.2.3.6.1	General

Either of the MCVideo users involved in a private communication may release the unidirectional media plane or the whole private communication at any time.

##### 7.2.3.6.2	Procedure

Figure 7.2.3.6.2-1 describes procedures to release an ongoing MCVideo Private communication (either automatic commencement private communication or manual commencement private communication).

Either of the MCVideo users, the initiator or the receiver of the Private communication can request to release the Private communication.

The Private communication release request can be to release an individual (unidirectional) media plane or to terminate the Private communication. Based on the request the requested media plane or both the media planes are terminated in step 3.

Pre-conditions:

1.	MCVideo client A and MCVideo client B are engaged in a Private communication.

2.	MCVideo user A at MCVideo client A initiates release of the Private communication.

Figure 7.2.3.6.2-1: Off-network Private communication release

1,	The MCVideo client A sends the Private communication release request to the MCVideo client B.

2a.	The MCVideo client B sends a Private communication release response indicating successful receipt of the request to the MCVideo client A.

2b.	The MCVideo client B notifies the MCVideo user B about the Private communication release request.

NOTE:	Step 2a and step 2b can occur in any order.

3.	MCVideo client A and MCVideo client B release the requested media plane. If both the media planes are released, the Private communication is terminated.

## 7.3	Video pull

### 7.3.1	General

MCVideo pull is enabled in both on-network and off-network mode.

For on-network MCVideo pull is supported between MCVideo clients, or between a MCVideo client and a MCVideo server.

For off-network MCVideo pull is supported between MCVideo clients only.

### 7.3.2	On-network video pull

#### 7.3.2.1	General

A MCVideo user triggers its MCVideo client to pull a video either from another MCVideo client or from a MCVideo server.

#### 7.3.2.2	Information flows for on-network video pull

The following information flows for private call specified in subclause 7.2.2.2 is used for on-network video pull:

-	MCVideo private call request (MCVideo client – MCVideo server)

-	MCVideo private call request (MCVideo server – MCVideo client)

-	MCVideo private call response (MCVideo client – MCVideo server)

-	MCVideo private call response (MCVideo server – MCVideo client)

-	MCVideo call end request

##### 7.3.2.2.1	MCVideo pull from server request

Table 7.3.2.2.1-1 describes the information flow MCVideo pull from server request from the MCVideo client to the MCVideo server.

Table 7.3.2.2.1-1: MCVideo pull from server request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| File URL | M | URL of the video file to be streamed |
| SDP offer | M | Media parameters of MCVideo client |
| Requested priority | O | Application priority level requested for this call |

##### 7.3.2.2.2	MCVideo pull from server response

Table 7.3.2.2.2-1 describes the information flow MCVideo pull from server response from the MCVideo server to the MCVideo client.

Table 7.3.2.2.2-1: MCVideo pull from server response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| SDP answer | M | Media parameters selected |

##### 7.3.2.2.3	MCVideo pull from server complete request

Table 7.3.2.2.3-1 describes the information flow MCVideo pull from server complete request from the MCVideo client to the MCVideo server or from the MCVideo server to the MCVideo client.

Table 7.3.2.2.3-1: MCVideo pull from server complete request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Cause | M | End of communication cause |

##### 7.3.2.2.4	MCVideo pull from server complete response

Table 7.3.2.2.4-1 describes the information flow MCVideo pull from server complete response from the MCVideo client to the MCVideo server or from the MCVideo server to the MCVideo client.

Table 7.3.2.2.4-1: MCVideo pull from server complete response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |

#### 7.3.2.3	One-to-one video pull

##### 7.3.2.3.1	General

One-to-one video pull is a private call that only allows the called party to transmit video to the calling party, and the private call ends when the video transmission is completed.

##### 7.3.2.3.2	One-to-one video pull – call setup

The procedure describes the case where an MCVideo user is initiating an MCVideo private call to pull video from called MCVideo user in an automatic or manual commencement mode. Only the called party is allowed to transmit video.

Procedures in figure 7.3.2.3.2-1 are the basic signalling control plane procedures for the MCVideo client initiating establishment of MCVideo private call with the chosen MCVideo user for video pull.

Pre-conditions:

1.	Optionally, MCVideo client 1 may have a functional alias activated to be used.

2.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.3.2.3.2-1: One-to-one video pull

1.	MCVideo user on MCVideo client 1 initiates video pull from MCVideo client 2, by sending a private call request to the MCVideo server using a service identifier as defined in 3GPP TS 23.228 [5] for MCVideo, for establishing a private call with the chosen MCVideo user for video pull. The MCVideo private call request contains MCVideo ID of invited user, an SDP offer containing one or more media types and the video pull indication to indicate that invited user is requested to transmit video to the requester. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo private call request.

2.	MCVideo server checks whether the MCVideo user at MCVideo client 1 is authorized to initiate the private call for video pull, and that MCVideo user at MCVideo client 2 is authorized to receive the private call for video pull. MCVideo server verifies whether the provided functional alias, if present, can be used and has been activated for the user.

3.	MCVideo server may provide a progress indication to MCVideo client 1 to indicate progress in the call setup process.

NOTE 1:	Step 3 can occur at any time following step 4, and prior to step 6.

4.	If authorized, MCVideo server includes information that it communicates using MCVideo service, offers the same media types or a subset of the media types contained in the initial received request, includes the requested automatic commencement mode indication based on a requested commencement mode by the calling user or based upon the setting of the called MCVideo client and sends the corresponding MCVideo private call request towards the MCVideo client 2, including the MCVideo ID, and, if available the functional alias of the calling MCVideo user 1. If the called MCVideo user has registered to the MCVideo service with multiple MCVideo UEs and has designated the MCVideo UE for receiving the private calls, then the incoming MCVideo private call request is delivered only to the designated MCVideo UE.

5.	The MCVideo client 2 notifies the user about the incoming private call for video pull. If a functional alias of the calling user is included it is displayed.

6.	The receiving MCVideo client 2 accepts the private call for video pull, and sends an MCVideo private call response to the MCVideo server with an implicit transmit media request.

7.	Upon receiving the MCVideo private call response from MCVideo client 2 accepting the private call request for video pull, the MCVideo server informs the MCVideo client 1 about successful call establishment via a private call response message.

8.	MCVideo client 1 and MCVideo client 2 have successfully established media plane and transmission control for media communication.

NOTE 2:	When the video transmission is completed, the private call is released.

##### 7.3.2.3.3	One-to-one video pull – call release

The private call release procedure specified in subclause 7.2.2.3.3 is used for one-to-one video pull call release.

#### 7.3.2.4	One-from-server video pull

##### 7.3.2.4.1	General

One-from-server video pull is a private call between the calling party and the MCVideo server that allows the calling party to request streaming of  a video from the server where it is stored in a file. The one-from-server video pull ends typically when the end of the video file has been reached or when the transmission is stopped by the calling user or by the MCVideo server.

NOTE: This procedure fulfills the requirements for the stage 1 replay of stored video feature.

##### 7.3.2.4.2	Procedure

Procedure in figure 7.3.2.4.2-1 is the basic signalling control plane procedures for the MCVideo client initiating establishment of MCVideo video pull from the MCVideo server.

Pre-conditions:

1.	MCVideo user 1 on MCVideo client 1 has been authenticated and authorized to use MCVideo service.

2.	MCVideo client 1 is registered.

3.	MCVideo client 1 has been received the URL of a video file available on the MCVideo server, e.g. within an MCData SDS.

Figure 7.3.2.4.2-1: One-from-server video pull

1.	MCVideo user on MCVideo client 1 initiates the pull of a video from the server and sends an MCVideo pull from server request to the MCVideo server. The request contains the URL of the video file and an SDP offer with media description.

2.	MCVideo server checks whether MCVideo client 1 is authorized to pull the requested video from the server.

3.	MCVideo server accepts the pull from server request and sends an MCVideo pull from server response. The response contains the SDP answer.

NOTE 1:	The SDP may contain a media description that allows the receiving party to remote control the stream (e.g. rewind, fast forward).

4.	MCVideo client 1 and MCVideo server have successfully established the media plane and transmission control for the video stream.

5.	Video stream is transmitted from MCVideo server to MCVideo client 1. The MCVideo client 1 may be able to remote control the video stream, subject to SDP negotiation.

6.	MCVideo user  on MCVideo client 1 stops the video pull from the server and sends an MCVideo pull from server complete request.

7.	MCVideo server acknowledges the end of the communication.

NOTE 2:	Alternatively, based on internal criteria (e.g. maximum duration of the video playback, end of the video file), the MCVideo server can end the transmission.

### 7.3.3	Off-network video pull

#### 7.3.3.1	General

A MCVideo client pulls a video from another MCVideo client directly, i.e. without involving the network.

Off-network video pull communications are based on ProSe capabilities as described in clause 7.18.

#### 7.3.3.2	Information flows for off-network video pull

The following information flows for private call specified in subclause 7.2.3.2 are used for off-network video pull:

-	Private communication request; and

-	Private communication answer response.

#### 7.3.3.3	Video pull to self

##### 7.3.3.3.1	General

A MCVideo user requests another MCVideo user to deliver a video from its MCVideo client.

##### 7.3.3.3.2	Procedure

Figure 7.3.3.3.2-1 describes procedures for an off-network MCVideo video pull, inititated by MCVideo user A at MCVideo client A with another MCVideo client B, to pull a video to MCVideo client A.

MCVideo client B can be an autonomous MCVideo client or can be a human controlled MCVideo client. In either case, following procedure should be followed.

Pre-conditions:

1.	MCVideo user A has initiated MCVideo video pull with MCVideo user B.

2.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

3.	MCVideo client A has discovered MCVideo client B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

Figure 7.3.3.3.2-1: Off-network video pull to self

1.	The MCVideo client A sends a Private communication request towards the MCVideo client B. The Private communication request includes a video pull request and indicates MCVideo client A as the intended recipient of the video pull. Private communication request contains the SDP offer.

2a.	The MCVideo client B notifies the MCVideo user B about the incoming video pull request.

2b. The MCVideo client B automatically accepts the video pull request, and sends a Private communication answer response indicating the acceptance of the video pull request. The Private communication answer response contains SDP answer.

NOTE 1:	Step 2a and step 2b can occur in any order.

3.	The MCVideo client A notifies MCVideo user A about the incoming Private communication answer response as an indication of acceptance of MCVideo video pull request.

4.	The MCVideo client A and the MCVideo client B establish the media plane for communication.

NOTE 2:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client.

5.	Media is transmitted from MCVideo client B to MCVideo client A and presented to the MCVideo user A.

NOTE 3:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

## 7.4	Video push

### 7.4.1	General

MCVideo push is enabled in both on-network and off-network mode.

For on-network MCVideo push is supported between MCVideo clients, or between a MCVideo client and a MCVideo server.

For off-network MCVideo push is supported between MCVideo clients only.

### 7.4.2	On-network video push

#### 7.4.2.1	General

A MCVideo user triggers its MCVideo client to push a video to another MCVideo client or a MCVideo server.

A MCVideo user triggers its MCVideo client to trigger a remote video push, i.e. a second MCVideo client pushes a video to a third MCVideo client or to a MCVideo group.

#### 7.4.2.2	Information flows for on-network video push

The following information flows for private call specified in subclause 7.2.2.2 is used for on-network video push:

-	MCVideo private call request (MCVideo client – MCVideo server)

-	MCVideo private call request (MCVideo server – MCVideo client)

-	MCVideo private call response (MCVideo client – MCVideo server)

-	MCVideo private call response (MCVideo server – MCVideo client)

-	MCVideo call end request

##### 7.4.2.2.1	Remote video push request

Table 7.4.2.2.1-1 describes the information flow remote video push request from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.4.2.2.1-1: Remote video push request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the call originating party |
| Functional alias | O | The functional alias of the originating party |
| MCVideo ID | M | The MCVideo ID of the source party who should transmit media |
| MCVideo ID | O (see NOTE) | The MCVideo ID of the destination party who should receive media |
| MCVideo group ID | O (see NOTE) | The MCVideo group ID of the destination group whose affiliated group members should receive media |
| SDP offer | O | Media parameters of MCVideo client. |
| Requested commencement mode | O | An indication that is included if the user is requesting a particular commencement mode |
| Requested priority | O | Application priority level requested for this call |
| NOTE: MCVideo ID information element is present if remote video push to destination user is requested. MCVideo group ID information element is present if remote video push to group is requested. |  |  |

##### 7.4.2.2.2	Remote video push response

Table 7.4.2.2.2-1 describes the information flow remote video push request from the MCVideo server to the MCVideo client and from the MCVideo client to the MCVideo server.

Table 7.4.2.2.2-1: Remote video push response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the call originating party |
| MCVideo ID | M | The MCVideo ID of the source party who should transmit media |
| MCVideo ID | O (see NOTE) | The MCVideo ID of the destination party who should receive media |
| MCVideo group ID | O (see NOTE) | The MCVideo group ID of the destination group whose affiliated group members should receive media |
| Acceptance confirmation | O | An indication whether the user or group members have positively accepted the call. |
| SDP answer | M | Media parameters selected |
| NOTE: MCVideo ID information element is present if remote video push to destination user is requested. MCVideo group ID information element is present if remote video push to group is requested. |  |  |

##### 7.4.2.2.3	Remote video push release request

Table 7.4.2.2.3-1 describes the information flow Remote video push release request from the MCVideo client to the MCVideo server and from the MCVideo server to the MCVideo client.

Table 7.4.2.2.3-1: Remote video push release request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the call originating party |
| MCVideo ID | M | The MCVideo ID of the source party who should transmit media |
| MCVideo ID | O (see NOTE) | The MCVideo ID of the destination party who should receive media |
| MCVideo group ID | O (see NOTE) | The MCVideo group ID of the destination group whose affiliated group members should receive media |
| NOTE: MCVideo ID information element is present if remote video push to destination user is requested. MCVideo group ID information element is present if remote video push to group is requested. |  |  |

##### 7.4.2.2.4	MCVideo push to server request

Table 7.4.2.2.4-1 describes the information flow MCVideo push to server request from the MCVideo client to the MCVideo server.

Table 7.4.2.2.4-1: MCVideo push to server request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Time limit | O | Requested duration of the stream to be recorded |
| SDP offer | M | Media parameters of MCVideo client |
| Requested priority | O | Application priority level requested for this call |

##### 7.4.2.2.5	MCVideo push to server response

Table 7.4.2.2.5-1 describes the information flow MCVideo push to server response from the MCVideo server to the MCVideo client.

Table 7.4.2.2.5-1: MCVideo push to server response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Time limit | O | Accepted or imposed duration limit of the stream to be recorded |
| File URL | M | URL of the file the stream will be recorded to |
| SDP answer | M | Media parameters selected |

##### 7.4.2.2.6	MCVideo push to server complete request

Table 7.4.2.2.6-1 describes the information flow MCVideo push to server complete request from the MCVideo client to the MCVideo server or from the MCVideo server to the MCVideo client.

Table 7.4.2.2.6-1: MCVideo push to server complete request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |
| Cause | M | End of communication cause |

##### 7.4.2.2.7	MCVideo push to server complete response

Table 7.4.2.2.7-1 describes the information flow MCVideo push to server complete response from the MCVideo client to the MCVideo server or from the MCVideo server to the MCVideo client.

Table 7.4.2.2.7-1: MCVideo push to server complete response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the calling party |

#### 7.4.2.3	One-to-one video push

##### 7.4.2.3.1	General

One-to-one video push is a private call that only allows the calling party to transmit video to the called party, and the private call ends when the video transmission is completed.

##### 7.4.2.3.2	One-to-one video push – call setup

Procedures in figure 7.4.2.3.2-1 are the basic signalling control plane procedures for the MCVideo client initiating establishment of MCVideo private call with the chosen MCVideo user for video push.

Pre-conditions:

1.	This procedure is initiated either manually by the authorized MCVideo users at MCVideo client 1, or automatically due to triggers set by authorized MCVideo users at MCVideo client 1.

2.	Optionally, MCVideo client 1 may have a functional alias activated to be used.

3.	The MCVideo server may have subscribed to the MCVideo functional alias controlling server within the MC system for functional alias activation/de-activation updates.

Figure 7.4.2.3.2-1: One-to-one video push

1.	MCVideo client 1 initiates push video to MCVideo client 2, and sends a private call request to the MCVideo server using a service identifier as defined in 3GPP TS 23.228 [5] for MCVideo, for establishing a private call with the chosen MCVideo user for video push. The MCVideo private call request contains MCVideo ID of invited user, an SDP offer containing one or more media types and the video push indication to indicate that requestor is requesting to transmit video to the called party. The MCVideo user at MCVideo client 1 may include a functional alias used within the MCVideo private call request.

2.	MCVideo server checks whether the MCVideo user at MCVideo client 1 is authorized to initiate the private call for video push, and that MCVideo user at MCVideo client 2 is authorized to receive the private call for video push. MCVideo server verifies whether the provided functional alias, if present, can be used and has been activated for the user.

3.	MCVideo server may provide a progress indication to MCVideo client 1 to indicate progress in the call setup process.

NOTE 1:	Step 3 can occur at any time following step 4, and prior to step 6.

4.	If authorized, MCVideo server includes information that it communicates using MCVideo service, offers the same media types or a subset of the media types contained in the initial received request, includes the requested automatic commencement mode indication based on a requested commencement mode by the calling user or based upon the setting of the called MCVideo client and sends the corresponding MCVideo private call request towards the MCVideo client 2, including the MCVideo ID and, if available the functional alias of the calling MCVideo user 1. If the called MCVideo user has registered to the MCVideo service with multiple MCVideo UEs and has designated the MCVideo UE for receiving the private calls, then the incoming MCVideo private call request is delivered only to the designated MCVideo UE.

5.	The MCVideo client 2 notifies the user about the incoming private call for video push. If the functional alias of the calling user is included it is displayed.

6.	The MCVideo client 2 accepts the private call for video push, and sends an MCVideo private call response to the MCVideo server.

7.	Upon receiving the MCVideo private call response from MCVideo client 2 accepting the private call request for video push, the MCVideo server informs the MCVideo client 1 about successful call establishment via a private call response message.

8.	MCVideo client 1 and MCVideo client 2 have successfully established media plane and transmission control for media communication.

NOTE 2:	When the video transmission is completed, the private call is released.

##### 7.4.2.3.3	One-to-one video push – call release

The private call release procedure specified in subclause 7.2.2.3.3 is used for one-to-one video push call release.

#### 7.4.2.4	One-to-server video push

##### 7.4.2.4.1	General

One-to-server video push is a private call between the calling party and the MCVideo server that allows the calling party to transmit video to the server for it to be recorded to a file. The one-to-server video push ends when the video transmission is stopped by the calling party or by the MCVideo server.

NOTE:	This procedure fulfils part of the requirements for the stage 1 video processing capabilities feature.

##### 7.4.2.4.2	Procedure

Procedure in figure 7.4.2.4.2-1 is the basic signalling control plane procedures for the MCVideo client initiating establishment of MCVideo video push to the MCVideo server.

Pre-conditions:

1.	MCVideo user 1 on MCVideo client 1 has been authenticated and authorized to use MCVideo service.

2.	MCVideo client 1 is registered.

Figure 7.4.2.4.2-1: One-to-server video push

1.	MCVideo user on MCVideo client 1 initiates the push of a video to the server and sends an MCVideo push to server request to the MCVideo server. The request contains an SDP offer with media.

2.	MCVideo server checks whether MCVideo client 1 is authorized to push a video to the server.

3.	MCVideo server accepts the push to server request and sends an MCVideo push to server response. The response contains the SDP answer and the URL of the file where the video stream will be recorded.

NOTE 1:	Push request and response can include a negotiated maximum duration for the transmission.

4.	MCVideo client 1 and MCVideo server have successfully established the media plane and transmission control for the video stream.

5.	Video stream is transmitted from MCVideo client 1 to MCVideo server. The MCVideo server records the stream to the file whose URL has been given in the MCVideo push to server response.

NOTE 2:	Notifications of new media availability can be sent by the initiating MCVideo user or by the MCVideo server using the URL received in the MCVideo push to server response.

NOTE 3:	MCVideo clients which receive the notification can initiate a pull of the video stream from the server using the URL provided in the notification, at their convenience, using the procedure defined in subclause 7.3.2.4.2.

6.	MCVideo user on MCVideo client 1, or MCVideo client 1 based on pre-defined criteria (e.g. duration), stops the video push to the server and sends an MCVideo push to server complete request.

7.	MCVideo server acknowledges the end of the communication.

NOTE 4:	Alternatively, based on internal criteria (e.g. maximum duration of the recorded video), the MCVideo server can end the transmission.

#### 7.4.2.5	Remotely initiated video push

##### 7.4.2.5.1	General

Remotely initiated video push is a private call remotely initiated by a user that only allows a source user to transmit video to a destination, and the private call ends when the video transmission is completed or released by the authorized user who remotely initiated video push call.

##### 7.4.2.5.2	Remotely initiated video push – call setup

The procedure describes the case where an authorized MCVideo user is initiating an remotely initiated video push call from source MCVideo user to a destination MCVideo user. Only the source user is allowed to transmit video to the destination user.

Procedures in figure 7.4.2.5.2-1 are the basic signalling control plane procedures for the MCVideo client remotely initiating establishment of MCVideo private call between the source and destination MCVideo users.

Figure 7.4.2.5.2-1: Remotely initiated video push – call setup

1.	MCVideo user on MCVideo client 3 initiates remote video push from MCVideo client 1 to MCVideo client 2, by sending a remote video push request to the MCVideo server for establishing a one-to-one video push call between MCVideo client 1 and MCVideo client 2. The remote video push call request contains MCVideo ID of source (MCVideo client 1) and destination (MCVideo client 2) users

2.	MCVideo server checks whether the MCVideo user at MCVideo client 3 is authorized to remotely initiate video push call between the source user and the destination user. MCVideo server also checks if there is any on-going call between the source user and the destination user.

3.	If MCVideo client 3 is authorized and there is no on-ongoing private call between source user and destination user, the MCVideo server sends the remote video push request message to MCVideo client 1 including the information of MCVideo ID of source and destination users.

4.	The MCVideo client 1 accepts the remote video push request and establishes a one-to-one video push call between MCVideo client 1 and MCVideo client 2 as described in the subclause 7.4.2.3.2. The video is transmitted from MCVideo client 1 to MCVideo Client 2.

5.	The MCVideo client 1 sends a remote video push response message to MCVideo server indicating success or failure of call establishment between MCVideo client 1 and MCVideo client 2.

6.	Upon receiving the remote video push response from MCVideo client 1, the MCVideo server informs the MCVideo client 3 about success or failure of the call establishment by sending a remote video push response message.

NOTE:	Steps 5 and 6 can occur before step 4.

##### 7.4.2.5.3	Remotely initiated video push – call release by authorized user

The procedure describes the case where an authorized MCVideo user is releasing an remotely initiated video push call.

Procedures in figure 7.4.2.5.3-1 are the basic signalling control plane procedures for the MCVideo client releasing the remotely initiated video push call between the source and destination users.

Figure 7.4.2.5.3-1: Remotely initiated video push – call release by authorized user

1.	MCVideo user on MCVideo client 3 initiates release of remotely initiated video push call between MCVideo client 1 and MCVideo client 2, by sending a remote video push release request to the MCVideo server for releasing a one-to-one video push call between MCVideo client 1 and MCVideo client 2. The remote video push release request contains MCVideo ID of source user (MCVideo client 1) and destination user (MCVideo client 2).

2.	MCVideo server checks whether the MCVideo user at MCVideo client 3 is authorized to release the remotely initiated video push call between MCVideo client 1 and MCVideo client 2, and whether there is any on-going one-to-one video push call between MCVideo client 1 and MCVideo client 2.

3.	MCVideo server sends the remote video push release request to MCVideo client 1 for releasing the on-going one-to-one video push call between MCVideo client 1 and MCVideo client 2.

4.	The MCVideo client 1 accepts the remote video push release request and performs the release procedure for the one-to-one video push call between MCVideo client 1 and MCVideo client 2 as described in the subclause 7.4.2.3.3.

5.	The MCVideo client 1 sends a remote video push release response to the MCVideo server indicating the release of the one-to-one video push call between MCVideo client 1 and MCVideo client 2.

6.	Upon receiving the remote video push release response from MCVideo client 1, the MCVideo server informs the MCVideo client 3 about the one-to-one video push call release by sending a remote video push release response message.

NOTE:	Steps 5 and 6 can occur before step 4.

#### 7.4.2.6	Remotely initiated video push to group

##### 7.4.2.6.1	General

Remotely initiated video push to group is a type of broadcast group call that only allows the originating user to remotely request a source user to transmit video to the group, and the group call ends when the video transmission is completed or the authorized user releases the remotely initiated video push to group call.

##### 7.4.2.6.2	Remotely initiated video push to group – call setup

The procedure describes the case where an authorized MCVideo user is initiating video push to group call for transmitting video from a source MCVideo user to the group. Only the source MCVideo user is allowed to transmit video.

Procedures in figure 7.4.2.6.2-1 are the basic signalling control plane procedures for the MCVideo client remotely initiating a video push to group call.

Pre-conditions:

1.	MCVideo client 1 and MCVideo client 2 are affiliated members of the group X.

2.	The user on MCVideo client 1 is authorized to remotely initiate a video push to group call to enable transmitting the video from the user on MCVideo client 2.

Figure 7.4.2.6.2-1: Remotely initiated video push to group – call setup procedure

1.	MCVideo user on MCVideo client 1 remotely initiates a video push to group from MCVideo client 2, by sending a remote video push request to the MCVideo server. The remote video push request contains MCVideo ID of the source user (user of MCVideo client 2) and the MCVideo group ID of the group X to which the video transmission is requested.

2.	MCVideo server checks whether the MCVideo user at MCVideo client 1 is authorized to remotely initiate a video push to group, and also checks whether MCVideo client 2 is transmitting video in any on-going MCVideo group call on group X. If MCVideo client 2 is already transmitting video to group X then a suitable response is provided in step 6 to MCVideo client 1.

3.	The MCVideo server sends a remote video push request initiated by MCVideo client 1 to MCVideo client 2. The remote video push request contains MCVideo ID of the source user, the MCVideo group ID of the group X to which the video transmission is to be initiated by the source user and an SDP offer containing one or more media types.

4.	MCVideo client 2 initiates a MCVideo broadcast group call on group X. The MCVideo client 2 begins to transmit video to the group X.

5.	The MCVideo client 2 sends a remote video push response message to MCVideo server indicating success or failure in remotely initiating the video push to group X.

6.	The MCVideo server sends a remote video push response message to MCVideo client 1 indicating success or failure in remotely initiating the video push to group X by the MCVideo client 2.

NOTE:	Steps 5 and 6 can occur before step 4.

##### 7.4.2.6.3	Remotely initiated video push to group – call release by authorized user

The procedure describes the case where an MCVideo user is remotely releasing a video push to group call for ending the video transmission from the source MCVideo user.

Procedures in figure 7.4.2.6.3-1 are the basic signalling control plane procedures for the MCVideo client remotely releasing a video push to group call.

Pre-condition:

-	MCVideo client 1 and MCVideo client 2 are affiliated members of the group X and there is an on-going broadcast group call on group X from MCVideo client 2 remotely initiated by MCVideo client 1 as described in subclause 7.4.2.6.2.

Figure 7.4.2.6.3-1: Remotely initiated video push to group – call release by authorized user

1.	MCVideo user on MCVideo client 1 remotely initiates a release of the video push to group call from MCVideo client 2, by sending a remote video push release request to the MCVideo server. The remote video push release request contains MCVideo ID of the source user (user of MCVideo client 2) and the MCVideo group ID of the group X to which the video transmission is to be released.

2.	MCVideo server checks whether the MCVideo user at MCVideo client 1 is authorized to remotely release a video push to group call, and also checks whether MCVideo client 2 is transmitting video in an on-going MCVideo group call on group X remotely initiated by MCVideo client 1. If MCVideo client 2 is not transmitting video on group X then a suitable response is provided in step 5 to MCVideo client 1.

3.	The MCVideo server sends a remote video push release request initiated by MCVideo client 1 to MCVideo client 2. The remote video push release request contains MCVideo ID of the source user, the MCVideo group ID of the group X to which the video transmission is to be released.

4.	The MCVideo client 2 sends a remote video push release response message to MCVideo server indicating the release of video push to group call.

5.	The MCVideo server sends a remote video push release response message to MCVideo client 1 indicating the release of video push to group call.

6.	MCVideo client 2 stops the video transmission to the group X  and releases the MCVideo broadcast group call on group X remotely initiated by MCVideo client 1.

NOTE:	Step 6 can occur before steps 4 and 5.

### 7.4.3	Off-network video push

#### 7.4.3.1	General

A MCVideo user triggers its MCVideo client to push a video to another MCVideo client.

A MCVideo user triggers its MCVideo client to trigger a remote video push, which requests a second MCVideo client to push a video to a third MCVideo client or to a MCVideo group.

Off-network remote video push works without involving the network.

Off-network video push communications are based on ProSe capabilities as described in clause 7.18.

#### 7.4.3.2	Information flows for off-network video push

##### 7.4.3.2.1	Remote video push request

Table 7.4.3.2.1-1 describes the information flow for the remote video push request sent from one MCVideo client to other MCVideo client(s).

Table 7.4.3.2.1-1: Remote video push request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user remotely requesting the video push |
| MCVideo ID | M | The identity of the MCVideo user from whom the video push is requested |
| Video information | O | Information identifying the video |
| MCVideo group ID | O | The MCVideo group ID to which the video is to be transmitted. |

##### 7.4.3.2.2	Video push trying response

Table 7.4.3.2.2-1 describes the information flow for the video push trying response sent from one MCVideo client to another MCVideo client.

Table 7.4.3.2.2-1: Video push trying response

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user sending the trying response |
| MCVideo ID | M | The identity of the MCVideo user to whom the trying response is sent |
| MCVideo ID | M | The identity of the MCVideo user to whom the video is attempted |

##### 7.4.3.2.3	Notification of video push

Table 7.4.3.2.3-1 describes the information flow for the notification of video push sent from one MCVideo client to another MCVideo client.

Table 7.4.3.2.3-1: Notification of video push

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user sending the notification |
| MCVideo ID | M | The identity of the MCVideo user to whom the notification is sent |
| MCVideo ID | M | The identity of the MCVideo user to whom the video is pushed |

#### 7.4.3.3	Video push to another MCVideo user

##### 7.4.3.3.1	General

The MCVideo Push is the capability of a MCVideo user to push a video to another MCVideo user. A MCVideo user can push a video which is:

-	being received by the MCVideo user from another a MCVideo client; or

-	being recorded live on the MCVideo client.

##### 7.4.3.3.2	Procedure

Figure 7.4.3.3.2-1 describes procedures for an off-network MCVideo video push, inititated by MCVideo client B with another MCVideo client C, to push a video received from MCVideo client A to MCVideo client C.

MCVideo client A or MCVideo client C or both can be autonomous MCVideo clients or can be human controlled MCVideo clients. In any such combination, following the procedure is followed.

MCVideo client A and MCVideo client B may belong to the same MCVideo user.

Pre-conditions:

1.	MCVideo client A and MCVideo client B are engaged in MCVideo communication, where MCVideo client B is receiving media from MCVideo client A.

2.	MCVideo user B has initiated MCVideo video push with MCVideo user C.

3.	MCVideo client B and MCVideo client C are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

4.	MCVideo client B has discovered MCVideo client C in proximity, associated with MCVideo user C, using ProSe Discovery procedures.

Figure 7.4.3.3.2-1: Off-network Video push to another MCVideo user

1.	The MCVideo client B sends a Private communication request towards MCVideo client C. The Private communication request indicates that the request is for video push and contains the SDP offer.

2a.	The MCVideo client C may notify the MCVideo user C about the MCVideo Push request (if the MCVideo client C is human controlled)

2b.	The MCVideo client C automatically accepts the Private communication request for video push and sends a Private communication answer response to MCVideo client B. The Private communication answer response contains SDP answer.

NOTE 1:	Step 2a and step 2b can occur in any order.

3a.	If configured, the MCVideo client B notifies MCVideo client A about the video being pushed to MCVideo client C by sending a notification message to MCVideo client A.

3b.	If configured, the MCVideo client A may notify the MCVideo user A about the video being pushed to MCVideo client C.

4.	The MCVideo client B and MCVideo client C establish the media plane for communication.

5.	Video being received by MCVideo client B from MCVideo client A is transmitted from MCVideo client B to MCVideo client C and presented to the MCVideo user C.

NOTE 2:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client(s).

NOTE 3:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

#### 7.4.3.4	Remotely initiated video push

##### 7.4.3.4.1	General

The MCVideo push is the capability of a MCVideo user to push a video from a second MCVideo user to a third MCVideo user.

##### 7.4.3.4.2	Procedure

Figure 7.4.3.4.2-1 describes procedures for a remotely initiated off-network MCVideo video push, inititated by MCVideo user A at MCVideo client A with another MCVideo client B, to push a video to MCVideo client C.

MCVideo client B can be an autonomous MCVideo client or can be a human controlled MCVideo client. In either case, following procedure is followed.

Pre-conditions:

1.	MCVideo user A has remotely initiated MCVideo video push with MCVideo user B.

2.	MCVideo client A and MCVideo client B are members of the same ProSe Discovery group and are ProSe 1:1 direct communication capable.

3.	MCVideo client A has discovered MCVideo client B in proximity, associated with MCVideo user B, using ProSe Discovery procedures.

Figure 7.4.3.4.2-1: Remotely initiated off-network video push

1.	The MCVideo client A sends a Video push request towards the MCVideo client B. The Video push request indicates MCVideo client C as the intended recipient.

2a.	The MCVideo client B checks whether there is on-going priviate communication with MCVideo client C. If there is on-going private communication with MCVideo client C, then step 3, step 4 and step 5 are skipped, else MCVideo client B enables ProSe layer to discover MCVideo client C as specified in subclause 7.2.3.

NOTE 1:	MCVideo client A may provide the IP address of the MCVideo client C as part of the Video push request to the MCVideo B client. If such is the case, MCVideo client B does not require discovering MCVideo client C.

2b.	The MCVideo client B notifies the MCVideo user B about the incoming Video push request (if MCVideo client B is human controlled).

2c.	The MCVideo client B sends a video push trying response to MCVideo client A indicating that it is attempting to establish a connection with MCVideo client C,

NOTE 2:	Step 2a, 2b and 2c can occur in any order.

3.	Once the required details of MCVideo client C are obtained (either by discovery or from MCVideo client A) the MCVideo client B accepts the Video push request, and sends a Private communication request to MCVideo client C. The Private communication request indicates that the request was in response to the video push request from the MCVideo client A. The Private communication request contains the SDP offer.

4a.	MCVideo client C sends a Private communication answer response to MCVideo client B in response to the Private communication request. The Private communication answer response contains the SDP answer.

4b: The MCVideo client C notifies MCVideo user C about the incoming Private communication request as an indication of incoming video.

NOTE 3:	Step 4a and step 4b can occur in any order.

5.	Upon receiving a Private communication answer response from the MCVideo client C, the MCVideo client B sends a notification message to the MCVideo client A indicating that MCVideo client C has accepted the request.

6.	MCVideo client A notifies the MCVideo user A about the acceptance of the request by MCVideo client C (if the MCVideo client A is human controlled)

7.	The MCVideo client B and the MCVideo client C establish the media plane for communication.

NOTE 4:	If a MCVideo client fails to establish the communication, the MCVideo client should send a Private communication failed response indicating the failure reason to the appropriate MCVideo client.

8: Media is transmitted from MCVideo client B to MCVideo client C and presented to the MCVideo user C.

NOTE 5:	If a Private communication failed response is received by a MCVideo client, before or after establishing the media session, if already established, the session is terminated and the MCVideo user is notified about the failure and its reason, if any.

NOTE 6:	If the MCVideo client B fails to transmit the video to MCVideo client C, it should send a notification to MCVideo client A, indicating the reason.

#### 7.4.3.5	Remotely initiated video push to a group

##### 7.4.3.5.1	General

The MCVideo push is the capability of a MCVideo user to push a video from a second MCVideo user to a MCVideo group.

##### 7.4.3.5.2	Procedure

Figure 7.4.3.5.2-1 describes procedures for an off-network remote video push request, initiated by MCVideo user A at MCVideo client A towards a MCVideo client B, to initiate a video push to MCVideo group G.

MCVideo client B can be an autonomous MCVideo client or can be a human controlled MCVideo client. In either case, following procedure should be followed.

Pre-conditions:

1.	MCVideo client A and MCVideo client B are members of MCVideo group G.

2.	MCVideo user A has initiated remote video push request with MCVideo user B to initiate a video push to MCVideo group G.

Figure 7.4.3.5.2-1: Remotely initiated video push to a group

1.	MCVideo client A sends a remote video push request towards the MCVideo group. The remote video push request indicates MCVideo client B as the intended target of the request and MCVideo group G as the intended recipient.

2.	Upon receiving the remote video push request as the intended target of the request, the MCVideo client B notifies the MCVideo user B of the remote video push request, if the MCVideo client B is human controlled. Other MCVideo group members notify their respective MCVideo users of the remote video push request.

3.	The MCVideo client B automatically accepts the remote video push request and sends a group communication announcement message with video push indication towards the MCVideo group, as described in subclause 7.1.3.3. The group communication announcement message contains an SDP body and an indication that the group communication announcement is for video push.

## 7.5	Capability information sharing

### 7.5.1	General

The MCVideo UE can store its capabilities information at the MCVideo server which can be retrieved by authorized users or MCVideo group members during on-network operations. The MCVideo UE can share their capabilities periodically or on-demand to another MCVideo UE or to MCVideo group members during off-network operations.

### 7.5.2	On-network capability information sharing

#### 7.5.2.1	General

The MCVideo UE can store and update its capabilities information at the MCVideo server. Authorized MCVideo users or MCVideo group members can retrieve the capabilities information from the MCVideo server directly or via subscription and notification.

#### 7.5.2.2	Information flows for on-network capability information sharing

##### 7.5.2.2.1	Update MCVideo capabilities info request

Table 7.5.2.2.1-1 describes the information flow update MCVideo capabilities info request from the MCVideo client to the MCVideo server.

Table 7.5.2.2.1-1: Update MCVideo capabilities info request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the MCVideo user who is logged into the MCVideo UE |
| MCVideo capabilities | M | List of capabilities available at the MCVideo UE |

NOTE:	The device identification associated with MCVideo UE used for providing the MCVideo capabilities when the user logs on to multiple MCVideo UEs is specified by stage 3.

##### 7.5.2.2.2	Update MCVideo capabilities info response

Table 7.5.2.2.2-1 describes the information flow update MCVideo capabilities info response from the MCVideo server to the MCVideo client.

Table 7.5.2.2.2-1: Update MCVideo capabilities info response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure |

##### 7.5.2.2.3	Get MCVideo capabilities info request

Table 7.5.2.2.3-1 describes the information flow get MCVideo capabilities info request from the MCVideo client to the MCVideo server.

Table 7.5.2.2.3-1: Get MCVideo capabilities info request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the requesting MCVideo user |
| MCVideo ID list (see NOTE) | O | List of Identities of the target MCVideo users |
| MCVideo group ID list (see NOTE) | O | List of Identities of the target MCVideo groups |
| NOTE: At least one of the information elements shall be present. |  |  |

##### 7.5.2.2.4	Get MCVideo capabilities info response

Table 7.5.2.2.4-1 describes the information flow get MCVideo capabilities info response from the MCVideo server to the MCVideo client.

Table 7.5.2.2.4-1: Get MCVideo capabilities info response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure |
| MCVideo ID (see NOTE) | M | Identity of the MCVideo user whose MCVideo capabilities are requested |
| MCVideo capabilities (see NOTE) | M | List of capabilities available at the requested MCVideo user's UE |
| NOTE: The MCVideo capabilities correspond to the MCVideo ID. MCVideo ID and MCVideo capabilities information elements shall not be present if the Result is failure. Mulitple pairs of MCVideoID and MCVideo capabilities information elements may be appended. Only the affiliated group members in the requested MCVideo group ID list are returned. |  |  |

##### 7.5.2.2.5	Subscribe MCVideo capabilities info request

Table 7.5.2.2.5-1 describes the information flow subscribe MCVideo capabilities info request from the MCVideo client to the MCVideo server.

Table 7.5.2.2.5-1: Subscribe MCVideo capabilities info request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the requesting MCVideo user |
| MCVideo ID | O | Identity of the target MCVideo user |

##### 7.5.2.2.6	Subscribe MCVideo capabilities info response

Table 7.5.2.2.6-1 describes the information flow subscribe MCVideo capabilities info response from the MCVideo server to the MCVideo client.

Table 7.5.2.2.6-1: Subscribe MCVideo capabilities info response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure |

##### 7.5.2.2.7	Notify MCVideo capabilities info request

Table 7.5.2.2.7-1 describes the information flow notify MCVideo capabilities info request from the MCVideo server to the MCVideo client.

Table 7.5.2.2.7-1: Notify MCVideo capabilities info request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the MCVideo user logged in at the target MCVideo UE |
| MCVideo capabilities | M | List of capabilities available at the target MCVideo user's UE |

##### 7.5.2.2.8	Notify MCVideo capabilities info response

Table 7.5.2.2.8-1 describes the information flow notify MCVideo capabilities info response from the MCVideo client to the MCVideo server.

Table 7.5.2.2.8-1: Notify MCVideo capabilities info response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure |

#### 7.5.2.3	Update MCVideo capabilities information at the MCVideo server

The procedure for updating the MCVideo capabilities information at the MCVideo server is described in figure 7.5.2.3-1.

Figure 7.5.2.3-1: Update MCVideo capabilities information at the MCVideo server

1.	The MCVideo client sends a update MCVideo capabilities info request message to MCVideo server to update the capabilities information available at the MCVideo client

2.	The MCVideo server stores the updated MCVideo capabilities information received from the MCVideo client.

3.	The MCVideo server provides a Update MCVideo capabilities info response indicating success or failure.

#### 7.5.2.4	Retrieve MCVideo capabilities information by the MCVideo client

The procedure for retrieval of MCVideo capabilities information by the MCVideo client is described in figure 7.5.2.4-1.

Pre-conditions:

-	The MCVideo server has received MCVideo capabilities information from MCVideo clients and has stored this information;

-	The MCVideo user is authorized to access the MCVideo capabilities information from the MCVideo server.

-	The requesting MCVideo user is within the same MCVideo system as the requested MCVideo users and the requested MCVideo groups.

Figure 7.5.2.4-1: Retrieve MCVideo capabilities information by the MCVideo client

1.	The MCVideo client requests the MCVideo capabilities information by specifying some criteria (e.g. parameters, MCVideo ID).

2.	The MCVideo server checks whether the user of the MCVideo client is authorized to retrieve the requested MCVideo capabilities information.

3.	If authorized, the MCVideo server provides the requested MCVideo capabilities information to the MCVideo client.

Editor's note:	It is FFS for the case when the requesting MCVideo user is not within the same MCVideo system and the requested MCVideo users and the requested groups.

#### 7.5.2.5	Subscription and notification for MCVideo capabilities information

The procedure for subscription for MCVideo capabilities information as described in figure 7.5.2.5-1 is used by the MCVideo client to indicate to the MCVideo server that it wishes to receive updates of MCVideo capabilities information for which it is authorized.

Pre-conditions:

-	The MCVideo server has some MCVideo capabilities information stored.

Figure 7.5.2.5-1: Subscription for MCVideo capabilities information

1.	The MCVideo client subscribes to the MCVideo capabilities information stored at the MCVideo server using the subscribe MCVideo capabilities info request.

2.	The MCVideo server provides a subscribe MCVideo capabilities info response to the MCVideo client indicating success or failure of the request.

The procedure for notification of MCVideo capabilities information as described in figure 7.5.2.5-2 is used by the MCVideo server to inform the MCVideo client that new or updated MCVideo capabilities information is available.

Pre-conditions:

-	The MCVideo client has subscribed to the MCVideo capabilities information

-	The MCVideo server has received and stored new or updated MCVideo capabilities information.

Figure 7.5.2.5-2: Notification of MCVideo capabilities information

1.	The MCVideo server provides the notification to the MCVideo client, who previously subscribed for the MCVideo capabilities information.

2.	The MCVideo client provides a notify MCVideo capabilities info response to the MCVideo server.

If the MCVideo server has notified the MCVideo client about new or updated MCVideo capabilities information through this procedure, the MCVideo client may then follow the procedure described in subclause 7.5.2.4 in order to retrieve that MCVideo capabilities information.

### 7.5.3	Off-network capability information sharing

#### 7.5.3.1	General

Each MCVideo client within a MCVideo group needs to share its video capabilities with other members of the MCVideo group.

Video capability sharing can be done by sending information periodically as described in subclause 7.5.3.3 or on request as described in subclause 7.5.3.4.

The receiving MCVideo clients need to store and update the video capability information from the sharing MCVideo client.

Off-network video capability information sharing is based on ProSe capabilities as described in clause 7.18.

#### 7.5.3.2	Information flows for Off-network capability information sharing

##### 7.5.3.2.1	Capability request

Table 7.5.3.2.1-1 describes the information flow for the capability request sent from the MCVideo client to other MCVideo client(s).

Table 7.5.3.2.1-1: Capability request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the capabilities |
| MCVideo ID | O | The identity of the MCVideo user towards whom the capability request was sent. |
| MCVideo group ID | O | The MCVideo group ID towards which the request was sent |
| Search criteria | O | A criteria filter for the capability request (e.g. category tags, video capabilities etc.) |

##### 7.5.3.2.2	Capability announcement

Table 7.5.3.2.2-1 describes the information flow for the capability announcement sent from the MCVideo client to other MCVideo client(s).

Table 7.5.3.2.2-1: Capability announcement

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user announcing the capabilities |
| MCVideo ID | O | The identity of the MCVideo user towards whom the capability announcement was sent. |
| MCVideo group ID | O | The MCVideo group ID towards which the announcement was sent |
| Category tags | O | Category tags associated with a specific MCVideo UE or a video |
| Video capabilities | O | Set of video capabilities (e.g. codecs), camera capabilities (e.g. self-activated camera, wide field of view etc.) |
| Location | O | The location of the MCVideo user announcing the capabilities, if known |

##### 7.5.3.2.3	Activity status request

Table 7.5.3.2.3-1 describes the information flow for the activity status request sent from the MCVideo client to other MCVideo client(s).

Table 7.5.3.2.3-1: Activity status request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the activity status |
| MCVideo ID | O | The identity of the MCVideo user towards whom the activity status request was sent. |
| MCVideo group ID | O | The MCVideo group ID towards which the activity status request was sent |

##### 7.5.3.2.4	Activity status announcement

Table 7.5.3.2.4-1 describes the information flow for the activity status announcement sent from the MCVideo client to other MCVideo client(s).

Table 7.5.3.2.4-1: Activity status announcement

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user announcing the activity status |
| Activity status | M | Current status of MCVideo UE's activities (e.g. receiving video, transmitting video, or recording video) |
| Video details | O | Details of the video like category tags or bit rate. |
| MCVideo ID | O | The identity of the MCVideo user towards whom the activity status announcement was sent. |
| MCVideo group ID | O | The MCVideo group ID towards which the activity status announcement was sent |

#### 7.5.3.3	Periodic capability announcements

##### 7.5.3.3.1	General

The MCVideo client periodically provides its video capability information to other members of the MCVideo group.

##### 7.5.3.3.2	Procedure

Figure 7.5.3.3.2-1 describes procedures for periodic capability announcements.

Each MCVideo client periodically sends a Capability announcement messages to the MCVideo group. This Capability announcement message is received by all other members of MCVideo group members.

Upon receiving such a Capability announcement message, the MCVideo client stores/updates the information about the transmitting MCVideo client.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1.

2.	MCVideo client 1 to MCVideo client N are members of the same MCVideo group.

Figure 7.5.3.3.2-1: Periodic capability announcements

1.	MCVideo client 1 periodically sends a Capability announcement message, which contains its capability information with other relevant information.

2.	Other MCVideo clients, upon receiving a Capability announcement message from MCVideo client 1, cache its presence and capability information along with other relevant information.

NOTE:	Capability announcement may include specific category tags (e.g. certain video camera), video capabilities of the client, location, camera angles, camera orientation, etc.

#### 7.5.3.4	Request capabilities from client(s)

##### 7.5.3.4.1	General

The MCVideo client requests video capability information from other members of the MCVideo group.

##### 7.5.3.4.2	Request clients with particular capabilities

Figure 7.5.3.4.2-1 describes procedures for a mechanism to request other MCVideo clients having particular characteristics to share their capabilities.

MCVideo client sends a Capability request message to other MCVideo clients with search criteria. The search criteria may include MCVideo user ID or MCVideo client ID or a set of capabilities or a particular category of capabilities, or a mix of such criterions, etc.

Upon receiving the Capability request message, the MCVideo client that fulfils the search criteria responds to the received Capability request message with a Capability announcement message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1.

2.	MCVideo client 1 to MCVideo client N are members of the same MCVideo group.

Figure 7.5.3.4.2-1: Request clients with particular capabilities

1.	MCVideo client 1 sends a Capability request message with search criteria to request MCVideo clients of the MCVideo group, with particular characteristics, to share their capabilities.

2.	Upon receiving a Capability request message with search criteria, all MCVideo clients which fulfil the search criteria respond with a Capability announcement message, which contains its capability information with other relevant information.

NOTE 1:	If the Capability request message does not contain a search criteria, all the MCVideo clients that receive the Capability request message respond with a Capability announcement message.

3.	MCVideo clients, upon receiving a Capability announcement message from another MCVideo client, cache its presence and capability information along with other relevant information.

NOTE 2:	Capability announcement may include specific category tags (e.g. certain video camera), video capabilities of the client, location, camera angles, camera orientation, etc.

##### 7.5.3.4.3	Request capabilities from a particular client

Figure 7.5.3.4.3-1 describes procedures for a mechanism to request a particular MCVideo client in proximity, for its capabilities.

MCVideo client sends a Capability request message to the other MCVideo client.

Upon receiving the Capability request message, the MCVideo client responds to the received Capability request message by sending a Capability announcement message to the sender of the Capability request message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo client 2 is pre-configured in MCVideo client 1.

2.	MCVideo client 1 has discovered MCVideo client 2 in proximity using ProSe Discovery procedures.

Figure 7.5.3.4.3-1: Request capabilities from a particular client

1.	MCVideo client 1 sends a Capability request message towards MCVideo client 2.

2.	Upon receiving the Capability request message the MCVideo client 2 responds with a Capability announcement message, which contains its capability information with other relevant information.

3.	MCVideo client 1, upon receiving a Capability announcement message, caches MCVideo client 2's presence and capability information along with other relevant information.

NOTE:	Capability announcement may include specific category tags (e.g. certain video camera), video capabilities of the client, location, camera angles, camera orientation, etc.

#### 7.5.3.5	Request activity status from client(s)

##### 7.5.3.5.1	General

The MCVideo client requests activity status information from other members of the MCVideo group.

##### 7.5.3.5.2	Request activity status of group members

Figure 7.5.3.5.2-1 describes procedures to request activity status of other MCVideo clients having particular characteristics.

MCVideo client sends an activity status request message to other MCVideo clients with search criteria. The search criteria may include MCVideo user ID or MCVideo client ID or a set of capabilities or a particular category of capabilities, or a mix of such criterions, etc.

Upon receiving the activity status request message, the MCVideo client that fulfils the search criteria responds to the received activity status request message with an activity status announcement message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo group and its mapping to ProSe Layer-2 Group ID are pre-configured in MCVideo client 1.

2.	MCVideo client 1 to MCVideo client N are members of the same MCVideo group.

Figure 7.5.3.5.2-1: Request activity status of group members

1.	MCVideo client 1 sends an activity status request message with search criteria to request MCVideo clients of the MCVideo group, with particular characteristics, to share their activity status.

2.	Upon receiving a activity status request message with search criteria, all MCVideo clients which fulfil the search criteria respond with an activity status announcement message, which contains its activity status information.

NOTE:	If the activity status request message does not contain a search criteria, all the MCVideo clients that receive the activity status request message respond with an activity status announcement message.

3.	MCVideo clients, upon receiving an activity status announcement message from another MCVideo client, cache its presence and activity status information.

##### 7.5.3.5.3	Request activity status from a particular client

Figure 7.5.3.5.3-1 describes procedures to request a particular MCVideo client in proximity, for its activity status.

MCVideo client sends an activity status request message to the other MCVideo client.

Upon receiving the activity status request message, the MCVideo client responds to the received activity status request message by sending an activity status announcement message to the sender of the activity status request message.

Pre-conditions:

1.	Information for ProSe direct communications corresponding to the MCVideo client 2 is pre-configured in MCVideo client 1.

2.	MCVideo client 1 has discovered MCVideo client 2 in proximity using ProSe Discovery procedures.

Figure 7.5.3.5.3-1: Request activity status from a particular client

1.	MCVideo client 1 sends an activity status request message towards MCVideo client 2.

2.	Upon receiving the activity status request message, the MCVideo client 2 responds with an activity status announcement message, which contains its activity status information.

3.	MCVideo client 1, upon receiving an activity status announcement message, caches MCVideo client 2's presence and activity status information.

## 7.6	Ambient viewing call

### 7.6.1	General

The ambient viewing call is a type of a private MCVideo call that only allows a "viewed to" user to transmit media to a "viewing" user such that there is no indication on the MCVideo UE of the "viewed to" user about the call and the media transmission.

NOTE 1:	"viewed to" user refers to the user who is transmitting media in an ambient viewing call.

NOTE 2:	"viewing" user refers to the user who is receiving media in an ambient viewing call.

There are two types of ambient viewing call as below:

-	Remotely initiated ambient viewing is initiated by the authorized user (e.g., dispatcher) who wants to view to another user. In this case, the "viewed to" user is the called party, and shall automatically accept the call without causing any indication about the call and transmit the media to the "viewing" user.

-	Locally initiated ambient viewing is initiated by an authorized user who wants another user to view to the MCVideo UE communication. In this case, the "viewed to" user is the calling party and shall automatically transmit the media to the "viewing" user without causing any indication about the call processing and media transmission.

### 7.6.2	Information flows for ambient viewing call

#### 7.6.2.1	Ambient viewing call request

Table 7.6.2.1-1 describes the information flow for the ambient viewing call request from MCVideo client and MCVideo sever and MCVideo server to the MCVideo client.

Table 7.6.2.1-1: Ambient viewing call request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the "viewing" user to view |
| MCVideo ID | M | The MCVideo ID of the "viewed to" user |
| SDP offer | M | Media parameters of MCVideo client. |
| Ambient viewing type | M | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |

#### 7.6.2.2	Ambient viewing call response

Table 7.6.2.2-1 describes the information flow ambient viewing call response from the MCVideo client to the MCVideo server and MCVideo server to the MCVideo client.

Table 7.6.2.2-1: Ambient viewing call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the "viewing" user |
| MCVideo ID | M | The MCVideo ID of the "viewed to" user |
| SDP answer | M | Media parameters |
| Ambient viewing type | M | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |

#### 7.6.2.3	Ambient viewing call release request

Table 7.6.2.3-1 describes the information flow ambient viewing call release request from the MCVideo client to the MCVideo server and MCVideo server to the MCVideo client.

Table 7.6.2.3-1: Ambient viewing call release request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the "viewing" user |
| MCVideo ID | M | The MCVideo ID of the "viewed to" user |
| Ambient viewing type | M | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |

#### 7.6.2.4	Ambient viewing call release response

Table 7.6.2.4-1 describes the information flow ambient viewing call release response from the MCVideo client to the MCVideo server and MCVideo server to the MCVideo client.

Table 7.6.2.4-1: Ambient viewing call release response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the "viewing" user |
| MCVideo ID | M | The MCVideo ID of the "viewed to" user |
| Ambient viewing type | M | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |

#### 7.6.2.5	Ambient viewing call release notification

Table 7.6.2.5-1 describes the information flow ambient viewing call release notification from the MCVideo server to the MCVideo client.

Table 7.6.2.5-1: Ambient viewing call release notification

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the user to view |
| MCVideo ID | M | The MCVideo ID of the user to be viewed to |
| Call release reason | M | The reason for call release by the MCVideo server |
| Ambient viewing type | M | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |

### 7.6.3	Procedures

#### 7.6.3.1	Remotely initiated ambient viewing call setup procedure

The MCVideo service provides the capability for an authorised user to initiate a remotely initiated ambient viewing call.

Figure 7.6.3.1-1 illustrates the high level procedure of remotely initiated ambient viewing call setup procedure.

Pre-conditions:

-	MCVideo client 1 is the client of the authorized user who is authorized to invoke a remotely initiated ambience viewing call with MCVideo client 2.

-	MCVideo user 1 is the "viewing" user at MCVideo client 1, and MCVideo user 2 is the "viewed to" user at MCVideo client 2.

Figure 7.6.3.1-1: Remotely initiated ambient viewing call setup

1.	MCVideo client 1 initiates a remotely initiated ambient viewing call to MCVideo client 2 by sending an ambient viewing request to MCVideo server. The remotely initiated ambient viewing call type is included.

2.	The MCVideo server performs an authorization check for the authorized user 1 for the remotely initiated ambient viewing call. If authorization fails, the MCVideo server provides a failure response to MCVideo client 1.

3.	The MCVideo server sends the ambient viewing call request to MCVideo client 2.

4.	MCVideo client 2 returns the ambient viewing call response to the MCVideo server.

5.	MCVideo server provides an ambient viewing call response to MCVideo client 1, indicating that whether the remotely initiated ambient viewing call is set up successfully or not.

6.	The transmission control server of the MCVideo server then sends a media transmit granted message to MCVideo client 2 according to the ambient viewing type received in step 1.

7.	After receiving the media transmit granted message, the MCVideo client to be viewed to transmit video to the MCVideo client 1 for viewing.

NOTE:	MCVideo client 2 does not provide any indication of the ambient viewing call release to its user.

#### 7.6.3.2	Locally initiated ambient viewing call setup procedure

The MCVideo service provides the capability for an authorised user to initiate a locally initiated ambient viewing call.

Figure 7.6.3.2-1 illustrates the high level procedure of locally initiated ambient viewing call setup procedure.

Pre-conditions:

-	MCVideo client 2 is the client of the authorized user who is authorized to invoke a locally initiated ambient viewing call from MCVideo client 2.

-	MCVideo user 1 is the "viewing" user at MCVideo client 1, and MCVideo user 2 is the "viewed to" user at MCVideo client 2.

Figure 7.6.3.2-1: Locally initiated ambient viewing call setup

1.	MCVideo client 2 initiates a locally initiated ambient viewing call to MCVideo client 1 by sending an ambient viewing request to MCVideo server. The locally initiated ambient viewing call type is included.

2.	The MCVideo server performs an authorization check for the authorized user 2 for the locally initiated ambient viewing call. If authorization fails, the MCVideo server provides a failure response to MCVideo client 2.

3.	The MCVideo server sends the ambient viewing call request to MCVideo client 1.

4.	MCVideo client 1 returns the ambient viewing call response to the MCVideo server.

5.	MCVideo server provides an ambient viewing call response to MCVideo client 2, indicating that whether the locally initiated ambient viewing call is set up successfully or not.

6.	The transmission control server of the MCVideo server then sends a media transmit granted message to MCVideo client 2 according to the ambient viewing type received in step 1.

7.	After receiving the media transmit granted message, the MCVideo client to be viewed to, transmits video to the MCVideo client 1 for viewing.

NOTE:	MCVideo client 2 does not provide any indication of the ambient viewing call release to its user.

#### 7.6.3.3	Ambient viewing call release – server initiated

Figure 7.6.3.3-1 illustrates the information flow for ambient viewing call release – server initiated when trigger by the MCVideo administrator. This procedure is applied for both remotely initiated ambient viewing call and the locally initiated ambient viewing call.

Pre-conditions:

-	MCVideo client 1 is the MCVideo client of the authorized user, who initiated the ambient viewing call at MCVideo client 2.

-	There is an ongoing ambient viewing call between MCVideo client 2 and MCVideo client 1.

-	MCVideo user 1 is the "viewing" user at MCVideo client 1, and MCVideo user 2 is the "viewed to" user at MCVideo client 2.

Figure 7.6.3.3-1: Ambient viewing call release – server initiated

1.	The ambient viewing call release criteria by the MCVideo administrator.

2.	The MCVideo server sends an ambient viewing call release request to MCVideo client 2.

3.	MCVideo client 2 stops transmitting media to MCVideo client 1.

NOTE:	MCVideo client 2 does not provide any indication of the ambient viewing call release to its user.

4.	MCVideo client 2 provides an ambient viewing call release response to the MCVideo server.

5.	The MCVideo server sends an ambient viewing call release notification to MCVideo client 1 together with a reason code identifying that the call was released.

6.	MCVideo client 1 notifies the authorized MCVideo user 1.

#### 7.6.3.4	Ambient viewing call release – "viewing" user initiated

Figure 7.6.3.4-1 illustrates the information flow for ambient viewing call release – "viewing" user initiated. This procedure is applied for both remotely initiated ambient viewing call and the locally initiated ambient viewing call.

Pre-conditions:

-	MCVideo client 1 is the client of the authorized user, who is authorized to release the ambient viewing call at MCVideo client 2.

-	There is an ongoing ambient viewing call between MCVideo client 2 and MCVideo client 1.

-	MCVideo user 1 is the "viewing" user at MCVideo client 1, and MCVideo user 2 is the "viewed to" user at MCVideo client 2.

Figure 7.6.3.4-1: Ambient viewing call release – "viewing" user initiated

1.	The authorized user 1 at MCVideo client 1 initiates the ambient viewing call release to MCVideo client 2 by sending an ambient viewing call release request to the MCVideo server.

2.	The MCVideo server sends an ambient viewing call release request to MCVideo client 2.

3.	MCVideo client 2 stops transmitting media to MCVideo client 1.

NOTE:	MCVideo client 2 does not provide any indication of the ambient viewing call release to its user.

4.	MCVideo client 2 provides an ambient viewing call release response to the MCVideo server.

5.	The MCVideo server provides the ambient viewing call release response to MCVideo client 1.

#### 7.6.3.5	Ambient viewing call release – "viewed to" user initiated

Figure 7.6.3.5-1 illustrates the information flow for ambient viewing call release – "viewed to" user initiated. This procedure is only applied for the locally initiated ambient viewing call.

Pre-conditions:

-	MCVideo client 2 is the client of the authorized user, who is authorized to release the locally initiated ambient viewing call at MCVideo client 2.

-	There is an ongoing ambient viewing call between MCVideo client 1 and MCVideo client 2.

-	MCVideo user 1 is the current user at MCVideo client 1 who is viewing, and MCVideo user 2 is the current user at MCVideo client 2 who is being viewed to.

Figure 7.6.3.5-1: Ambient viewing call release – "viewed to" user initiated

1.	The authorized user 2 at MCVideo client 2 initiates the ambient viewing call release by sending an ambient viewing call release request to the MCVideo server.

2.	The MCVideo server provides an ambient viewing call release request to MCVideo client 1.

3.	MCVideo client 1 is notified about the ambient viewing call release.

4.	MCVideo client 1 provides an ambient viewing call release response to the MCVideo server.

5.	The MCVideo server provides the ambient viewing call release response to MCVideo client 2.

6.	MCVideo client 1 stops transmitting media to MCVideo client 2.

NOTE:	MCVideo client 1 does not provide any indication of the ambient viewing call release to its user.

## 7.7	Transmission control

### 7.7.1	Transmission control for on-network MCVideo service

#### 7.7.1.1	General

The procedure is for providing a transmission control to MCVideo UE in an on-network case and applies for both private call and group call. Transmission control is performed by using transmission control information flows between the transmission control participant and the transmission control server.

#### 7.7.1.2	Information flows for transmission control for on-network

##### 7.7.1.2.1	General

When the transmission control server receives a transmit media request from the transmission control participant, it decides whether to give a grant or not. The result is informed to the requesting transmission control participant. When the transmission control participant receives a transmit media granted message, it can send video media over the uplink bearer established beforehand. The transmit media revoked message can be used as part of revoke. The transmit media queue status request can be used to know current position in the queue for media transmission.

Some transmission control information flows can also piggyback call control information flows to provide efficient call setup and clearing:

-	Call setup request is optionally carried in transmit media request (uplink); and

-	Call release request is optionally carried in media transmission release (uplink).

##### 7.7.1.2.2	Transmit media request

Table 7.7.1.2.2-1 describes the information flow transmit media request, from the transmission control participant to the transmission control server, which is used to request the transmit media request. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.2-1: Transmit media request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media transmission is requested |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Media priority | O | Priority of the media |

##### 7.7.1.2.3	Transmit media granted

Table 7.7.1.2.3-1 describes the information flow transmit media granted, from the transmission control server to the transmission control participant, which is used to indicate that a transmit media request is granted and media transmission is possible. This information flow is sent in unicast (to the granted transmission control participant).

Table 7.7.1.2.3-1: Transmit media granted

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission control participant is required |

##### 7.7.1.2.4	Transmit media rejected

Table 7.7.1.2.4-1 describes the information flow transmit media rejected, from the transmission control server to the transmission control participant, which is used to indicate that a transmit media request is rejected. This information flow is sent in unicast (to the refused transmission control participant).

Table 7.7.1.2.4-1: Transmit media rejected

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Rejection cause | O | Indicates the cause for transmit media rejection |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission control participant is required |

##### 7.7.1.2.5	Media transmission notification

Table 7.7.1.2.5-1 describes the information flow media transmission notification, from the transmission control server to the transmission control participant, which is used to indicate that a media transmission is available from another user. This information flow is sent in unicast (to the receiving transmission control participant).

Table 7.7.1.2.5-1: Media transmission notification

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user who is transmitting the media |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Media reception mode (see NOTE) | M | Indicates the media reception mode (manual or forced). |
| NOTE: In the case of manual reception mode, to receive media, an explicit request from the user is required, whereas an explicit request is not required for the forced reception mode. |  |  |

##### 7.7.1.2.6	Receive media request

Table 7.7.1.2.6-1 describes the information flow receive media request, from the transmission control participant to the transmission control server, which is used to request the reception of the media from another user. This information flow is sent in unicast (to the transmission control server).

Table 7.7.1.2.6-1: Receive media request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user who is requesting the reception of the media |
| Source MCVideo ID | M | Identify of the user who is transmitting the media |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.7	Receive media response

Table 7.7.1.2.7-1 describes the information flow receive media response, from the transmission control server to the transmission control participant, which is used to indicate whether the media reception is possible or not. This information flow is sent in unicast (to the receiving transmission control participant).

Table 7.7.1.2.7-1: Receive media response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates whether media reception is possible as per the request |
| Rejection cause | O | Indicates the cause for rejecting the media receive request |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.8	Media reception notification

Table 7.7.1.2.8-1 describes the information flow media reception notification, from the transmission control server to the transmission control participant, which is used to indicate that a media reception has been initiated to a user. This information flow is sent in unicast (to the transmitting transmission control participant).

Table 7.7.1.2.8-1: Media reception notification

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user who is receiving the media |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.9	Queue position info

Table 7.7.1.2.9-1 describes the information flow queue position info, from the transmission control server to the transmission control participant, which is used to indicate that the transmit media request is queued and the queue position to the transmit media requesting UE. The MCVideo server and the MCVideo client support queuing of the transmit media requests shall support this information flow. This information flow is sent in unicast (to the queued transmission control participant).

Table 7.7.1.2.9-1: Queue position info

| Information element | Status | Description |
| --- | --- | --- |
| Queue position info | M | Position of the queued transmit media request in the queue |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission control participant is required |

##### 7.7.1.2.10	Transmission revoked

Table 7.7.1.2.10-1 describes the information flow transmission revoked, from the transmission control server to the transmission control participant, which is used to indicate that the on-going video transmission is queued and the queue position is provided or the on-going video transmission is terminated. This information flow is sent in unicast (to the queued transmission control participant).

Table 7.7.1.2.10-1: Transmission revoked

| Information element | Status | Description |
| --- | --- | --- |
| Revoke reason | M | The reason for revoke like queue or termination of on-going video transmission |
| Queue position info | O (see NOTE 1) | Position of the queued transmit media request in the queue |
| Media identifier | O (see NOTE 2) | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission control participant is required |
| NOTE 1: If revoke reason is queuing, then queue position info is included. NOTE 2: If revoke reason is termination, then media identifier is included. |  |  |

##### 7.7.1.2.11	Queue position request

Table 7.7.1.2.11-1 describes the information flow queue position request, from the transmission control participant to the transmission control server, which is used to request the position in the video transmission queue. The MCVideo server and the MCVideo client support queuing of the transmission control requests shall support this information flow. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.11-1: Queue position request

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.12	Transmit media cancel request

Table 7.7.1.2.12-1 describes the information flow transmit media cancel request, from the transmission control participant to the transmission control server. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.12-1: Transmit media cancel request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media transmission is requested for cancellation |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.13	Transmit media cancel response

Table 7.7.1.2.13-1 describes the information flow transmit media cancel response, from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.13-1: Transmit media cancel response

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.14	Transmit media cancel request notify

Table 7.7.1.2.14-1 describes the information flow transmit media cancel request notify, from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.14-1: Transmit media cancel request notify

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.15	Transmit media end request

Table 7.7.1.2.15-1 describes the information flow transmit media end request, from the transmission control participant to the transmission control server and from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control server/transmission control participant.

Table 7.7.1.2.15-1: Transmit media end request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media transmission is requested to be terminated. |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.16	Transmit media end response

Table 7.7.1.2.16-1 describes the information flow transmit media end response, from the transmission control participant and the transmission control server and from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control server/transmission control participant.

Table 7.7.1.2.16-1: Transmit media end response

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.17	Remote transmit media request

Table 7.7.1.2.17-1 describes the information flow remote transmit media request, from the transmission control participant to the transmission control server. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.17-1: Remote transmit media request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user who remotely initiated the media transmission of another user. |
| MCVideo ID | M | Identity of the user whose media transmission is requested |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Media priority | O | Priority of the media |

##### 7.7.1.2.18	Remote transmit media response

Table 7.7.1.2.18-1 describes the information flow remote transmit media response, from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control participant.

Table 7.7.1.2.18-1: Remote transmit media response

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.19	Remote transmit media end request

Table 7.7.1.2.19-1 describes the information flow remote transmit media end request, from the transmission control participant to the transmission control server. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.19-1: Remote transmit media end request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media transmission is requested for cancellation |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.20	Remote transmit media cancel response

Table 7.7.1.2.20-1 describes the information flow transmit media end response, from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.20-1: Remote transmit media end response

| Information element | Status | Description |
| --- | --- | --- |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.21	Media reception end request

Table 7.7.1.2.21-1 describes the information flow media reception end request, from the transmission control server to the transmission control participant and from the transmission control participant to the transmission control server.

Table 7.7.1.2.21-1: Media reception end request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media receipt is requested for cancellation |
| Media identifier | M | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.22	Media reception end response

Table 7.7.1.2.22-1 describes the information flow media reception end response, from the transmission control server to the transmission control participant and from the transmission control participant to the transmission control server.

Table 7.7.1.2.22-1: Media reception end response

| Information element | Status | Description |
| --- | --- | --- |
| /Media identifier | M | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.23	Media reception override notification

Table 7.7.1.2.23-1 describes the information flow media reception override notification, from the transmission control server to the transmission control participant.

Table 7.7.1.2.23-1: Media reception override notification

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user who is requesting the reception of the media |
| MCVideo ID | O | Identify of the user of the overriding media |
| Media identifier | M | Identifies the communication of overriding media, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| MCVideo ID | O | Identify of the user of the overridden media |
| Media identifier | M | Identifies the communication of overridden media, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.1.2.24	Transmit media end notify

Table 7.7.1.2.24-1 describes the information flow transmit media end notify, from the transmission control server to the transmission control participant. This information flow is sent in unicast to the transmission control server.

Table 7.7.1.2.24-1: Transmit media end notify

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose media transmission has been released |
| Media identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

#### 7.7.1.3	Transmission control within one MC system for MCVideo service

##### 7.7.1.3.1	Transmission control during an MCVideo session

Figure 7.7.1.3.1-1 describes the procedure for transmission control between the transmission control participant and the transmission control server during an MCVideo session. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server and there is no ongoing media transmission.

Figure 7.7.1.3.1-1: Transmission control during an MCVideo session

1.	Transmission control participant A wants to send video media over the session.

2.	Transmission control participant A sends a transmit media request message to transmission control server which includes transmission priority and other information as necessary.

3.	Transmission control server makes the determination on what action (grant, deny, or queue) to take on the request based on transmission control criteria (e.g., group policy) and determines to accept the transmit media request from transmission control participant A.

4.	Transmission control server responds with a transmit media granted message (4a) to transmission control participant A. Transmission control server may send transmit media rejected message (4b) indicating the cause of rejection or queue position info message (4c) indicating that the transmit media request has been queued.

5.	The transmit media granted shall cause the user of UE A where the transmission control participant A is located to be notified.

6.	Transmission control participant A starts sending video media over the session established to the transmission control server.

##### 7.7.1.3.2	Reception control during an MCVideo session

Figure 7.7.1.3.2-1 describes the procedure for the reception control between the transmission control participant and the transmission control server during an MCVideo session. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2-1: Reception control during an MCVideo session

1.	The transmission control server determines the reception mode for downlink transmission control participants for the media transmission permitted to transmission control participant A. The manual and forced reception modes are determined based on configurations (e.g. auto-receive video transmissions, auto-receive emergency video transmissions) as described in Annex A.

2.	Transmission control server sends a media transmission notification message to the transmission control participant B including information about the video transmitter and the reception mode.

3.	The receipt of the media transmission notification is used to inform the user of UE B the details of video transmission and the video transmitter.

4.	Based on the notification received, the user of UE B may want take actions (receive, reject or cancel) regarding the media available for reception.

5.	If user of UE B wants to receive the media or it required to receive the media via forced reception, it causes the transmission control participant B to send a media receive request message to the transmission control server.

6.	Transmission control server makes the determination on what action (e.g., real-time stream downlink, make stream downloadable, reject) to take on the request based on reception control criteria (e.g., allowed simultaneous reception) and determines to accept the media receive request from transmission control participant B. Transmission control server will also determine the availability of the media at the server to initiate the downlink reception.

7.	Transmission control server sends a receive media response message to transmission control participant B.

NOTE:	Step 5 to 7 may be omitted when auto-receive (i.e. forced reception mode) transmission mode is used based on configurations (e.g. for emergency group calls, imminent peril calls etc) as described in Annex A.

8.	Transmission control server sends video media to transmission control participant B.

9. Optionally, the transmission control server sends media reception notification message to transmission control participant A which includes the information about the video recipient (transmission control participant B).

##### 7.7.1.3.2A	End media reception – receiving user initiated

Figure 7.7.1.3.2A-1 describes the procedure for the receiving user initiated media reception end. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2A-1: End media reception – receiving user initiated

1.	The MCVideo user determines to end a receiving video stream.

2.	The user sends an media reception end request including the video stream to be ended to the transmission control server.

3.	The transmission control server returns a media reception end response.

4.	Upon receiving the media reception end request, the transmission control server stops sending the video stream selected to the transmission control participant B.

5.	If the end media reception at transmission control participant B causes the video transmission continuation condition to fail, then the transmission control server initiates the media transmission end procedure as described in subclause 7.7.1.3.6.2.

##### 7.7.1.3.2B	End media reception – transmission control server initiated

Figure 7.7.1.3.2B-1 describes the procedure for the transmission control server initiated media reception end. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2B-1: End media reception – transmission control server initiated

1.	The transmission control server determines to end a video stream transmitting to transmission control participant B according to events or configurations.

2.	The transmission control server sends an media reception end request including the video stream to be ended to the transmission control participant B.

3.	The MCVideo user is notified about the media reception end.

4.	The transmission control participants B returns a media reception end response.

5.	The transmission control server stops sending the video stream selected to the transmission control participant B.

6.	If the end media reception at transmission control participants B causes the continuation conditions to fail, then the transmission control server initiates media transmission end procedure as described in subclause 7.7.1.3.6.2.

##### 7.7.1.3.2C	Reception control on overridden – mandatory mode

Figure 7.7.1.3.2C-1 describes the procedure for the reception control on overridden with mandatory mode that the transmission control server determines the video stream to be overridden. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server.

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2C-1: Reception control on overriding/overridden – mandatory mode

1.	A new media stream is to be delivered to the transmission control participant B while the maximum number of simultaneous streams is reached. The transmission control server determines to override another media stream being received by the transmission control participant B accord to the media stream characteristics.

2.	Transmission control server sends a media reception override notification message to the transmission control participant B including information about the video stream being overridden and stop the downlink delivery of the overridden video stream to the transmission control participant B. The overriding video stream information maybe also included.

3.	The user of UE B is notified about the details of overriding/overridden.

4.	The transmission control server initiates the media reception end procedure as described in subclause 7.7.1.3.2B. 5.	The transmission control server performs the normal reception control procedure as described in subclause 7.7.1.3.2.

##### 7.7.1.3.2D	Reception control on overridden – negotiated mode

Figure 7.7.1.3.2D-1 describes the procedure for the reception control on overridden with negotiated mode that the transmission control participant determines the video stream to be overridden. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A has been granted the permission to transmit media.

Figure 7.7.1.3.2D-1: Reception control on overriding/overridden – negotiated mode

1.	A new media stream is to be delivered to the transmission control participant B while the maximum number of simultaneous streams is received. The transmission control server determines to override another media stream being received by the transmission control participant B according to the media stream characteristics.

2.	Transmission control server sends a media transmission notification message to the transmission control participant B including information about the new video stream.

3.	The user of transmission control participant B is notified about the maximum number of simultaneous streams received. The MCVideo user determines to reject the video stream or accept the video streaming by overriding a receiving video stream.

4.	The transmission control participant initiates the media reception end procedure as described in subclause 7.7.1.3.2A.

5.	Then the transmission control participant B requests to receive the new video stream as the normal reception control procedure described in subclause 7.7.1.3.2.

##### 7.7.1.3.3	Transmission revoke during an MCVideo session

Figure 7.7.1.3.3-1 describes the procedure for transmission revoke during an MCVideo session. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant B has been granted the permission to transmit media and there may be ongoing media transmission from transmission control participant B.

4.	The maximum media transmission limit for the MCVideo session is reached.

Figure 7.7.1.3.3-1: Transmission revoke during an MCVideo session

1.	Transmission control participant A wants to send video media over the session.

2.	Transmission control participant A sends a transmit media request message to transmission control server which includes transmission priority and other information as necessary.

3.	Transmission control server determines to accept the transmit media request from transmission control participant A and decides to pre-empt the on-going video transmission from transmission control participant B by queuing or ending the on-going video transmission.

4.	Transmission control server responds with a transmission revoked message (4a) to transmission control participant B with the action of pre-emption whether queued or terminated. Transmission control server responds with a transmit media granted message (4b) to transmission control participant A.

5.	The transmission revoked shall cause the transmission control participant B to be notified of the revoke (5a) of the on-going video transmission by queuing or termination. The transmit media granted shall cause the user of UE A where the transmission control participant A is located to be notified.

6.	Transmission control participant A starts sending video media over the session established to the transmission control server.

##### 7.7.1.3.4	Queue position during an MCVideo session

Figure 7.7.1.3.4-1 describes the procedure where the transmission control is conducted for the MCVideo session already established between MCVideo clients (with transmit media granted to transmission control participant B) and server (with an revoke determination at transmission control server). Only two UEs involved in the session are shown for the simplicity.

Figure 7.7.1.3.4-1: Queue status during an MCVideo session

1.	It is assumed that transmission control participant B has been granted permission to transmit media and is transmitting video media. There are several other transmission control participants (including transmission control participant A) requesting video transmission which get queued at the transmission control server.

2.	Transmission control participant A would like to know its current position in the video transmission queue.

3.	Transmission control participant A sends a queue position request message to the transmission control server.

4.	Transmission control server determines the current queue position of transmission control participant A from the video transmission queue.

5.	Transmission control server responds with the current position in queue position info message.

6.	User at transmission control participant A is informed about the current queue position.

##### 7.7.1.3.5	Transmit media request cancellation from the video transmission queue

###### 7.7.1.3.5.1	Transmit media request cancellation from the queue – MCVideo user initiated

Figure 7.7.1.3.5.1-1 illustrates the procedure for transmit media request cancellation from the video transmission queue initiated by the MCVideo user. The MCVideo user may be an authorized user who has rights to cancel the transmit media requests of other MCVideo users, whose transmit media requests are in video transmission queue.

Pre-conditions:

-	It is assumed that transmission control participant B has been granted the permission for video transmission and is transmitting video. There are several other transmission control participants (including transmission control participant A and transmission control participant C) requesting the permission to transmit media which have been queued at the transmission control server.

Figure 7.7.1.3.5.1-1: Transmit media request cancellation from queue initiated by MCVideo user

1.	The transmission control participant A wants to remove the transmit media request from the video transmission queue. If transmission control participant A is an authorized MCVideo user with the rights to cancel another MCVideo user's transmit media request, the authorized MCVideo user may request for transmit media request cancellation for one or more transmission control participants, whose transmit media request needs to be removed from the video transmission queue.

2.	The transmission control participant A sends a transmit media cancel request (initiating MCVideo ID) message to the transmission control server. If the transmission control participant A wants to remove the transmit media request(s) of other participant(s), the target participant(s)' MCVideo ID should be included in this message.

3.	The transmission control server shall check whether the requesting transmission control participant has authorization to cancel the transmit media request(s). If authorized, the transmit media request(s) will be removed from the video transmission queue. When the on-going transmissions are completed and are within the limit of the maximum simultaneous transmissions, the transmission control server will process the transmit media request from the updated video transmission queue.

4.	If the transmit media cancel request in step 3 is sent by an authorized user (e.g., dispatcher) to cancel the transmit media request(s) of other participant(s) from the video transmission queue, the transmit media cancel request notify message is sent to the transmission control participant whose transmit media request was cancelled from the video transmission queue.

5.	The transmission control server provides a transmit media cancel response to the transmission control participant A when the transmit media request cancellation is completed. Optionally, the new queue position information may be notified to the transmission control participants whose transmit media requests are in the video transmission queue (not shown in the figure).

###### 7.7.1.3.5.2	Transmit media request cancellation from the queue - transmission control server initiated

Figure 7.7.1.3.5.2-1 illustrates the procedure for transmit media request cancellation from the queue initiated by the transmission control server. Only two UEs involved in the session are shown for the simplicity.

Pre-conditions:

-	It is assumed that transmission control participant B has been granted the permission to transmit media and is transmitting video. There are several other transmission control participants (including transmission control participant A and participant C) requesting the permission to transmit media which have been queued at the transmission control server.

Figure 7.7.1.3.5.2-1: Transmit media request cancellation from queue initiated by transmission control server

1.	The transmission control server removes the transmit media request from the video transmission queue based on policy. e.g., expiration of a timer. In the case when transmission control server receives repeated transmit media requests from a transmission control participant while the limit for maximum simultaneous media transmissions has reached, the new transmit media request is accepted and added into the video transmission queue and the existing/former transmit media request is removed from the video transmission queue or the new transmit media request is rejected and the existing/former transmit media request of this transmission control participant is retained in the video transmission queue.

2.	The transmission control server sends a transmit media cancel request notify to the transmission control participant(s) whose transmit media request is removed from the video transmission queue.

3.	Optionally, the new queue position information is notified to the other transmission control participants whose transmit media requests are queued.

##### 7.7.1.3.6	End a media transmission during an MCVideo session

###### 7.7.1.3.6.1	End a media transmission – MCVideo user initiated

Figure 7.7.1.3.6.1-1 describes the procedure for ending a media transmission during an MCVideo session by a MCVideo user. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A and participant B have been granted the permission to transmit media and there are ongoing media transmission from transmission control participant A and participant B.

Figure 7.7.1.3.6.1-1: End a media transmission by an MCVideo user

1.	Transmission control participant A wants to end its media transmission in a session and sends a transmit media end request message to transmission control server.

2.	Transmission control server terminates the on-going media transmission from transmission control participant A.

3.	Transmission control server responds with a transmit media end response message to transmission control participant A indicating that the transmission control server has terminated the media transmission.

4.	Transmission control server sends a transmit media end notify message to transmission control participant B indicating that the media transmission from transmission control participant A is terminated by the user.

5.	The transmit media end notification shall cause the user of the transmission control participant B to be notified of the media transmission termination by the sending user.

###### 7.7.1.3.6.2	End a media transmission – transmission control server initiated

Figure 7.7.1.3.6.2-1 describes the procedure for ending a media transmission during an MCVideo session by the transmission control server. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant A and participant B have been granted the permission to transmit media and there are ongoing media transmission from transmission control participant A and participant B.

Figure 7.7.1.3.6.2-1: End a media transmission by the transmission control server

1.	Transmission control server terminates the on-going media transmission from transmission control participant A due to a termination event (e.g. revoke for another priority media transmission or there are no receiving participants).

2.	Transmission control server sends a transmit media end request message to transmission control participant A indicating the reason for ending the media transmission.

3.	Transmission control participant A responds with a transmit media end response message to transmission control server as acknowledgement

4.	Transmission control server sends a transmit media end notify message to transmission control participant B indicating that the media transmission from transmission control participant A is terminated by the transmission control server.

5.	The transmit media end notification shall cause the user of the transmission control participant B to be notified of the media transmission termination by the transmission control server.

###### 7.7.1.3.6.3	End a media transmission – remote MCVideo user initiated

Figure 7.7.1.3.6.3-1 describes the procedure for ending a media transmission during an MCVideo session by a remote MCVideo user. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server.

3.	Transmission control participant B has been granted the permission to transmit media and there is ongoing media transmission from transmission control participant B.

4.	Transmission control participant A is authorized to end the on-going media termination from transmission control participant B.

Figure 7.7.1.3.6.3-1: End a media transmission by a remote MCVideo user

1.	Transmission control participant A sends a remote transmit media end request message to transmission control server including the MCVideo ID of the user at the UE of the transmission control participant B and the MCVideo ID of the user at the UE of the transmission control participant A.

2.	Transmission control server checks whether the user at the UE of transmission control participant A is authorized to end the on-going media transmission from transmission control participant B. If authorized, the transmission control server ends the media transmission from transmission control participant B.

3.	Transmission control server ends the on-going media transmission from transmission control participant B. The transmission control server shall send transmit media end notify messages to the transmission control participants receiving the media transmission from transmission control participant B, indicating that the transmission has ended.

4.	Transmission control server responds with a remote transmit media end response message to transmission control participant A indicating that the transmission control server has ended the requested media transmission.

##### 7.7.1.3.7	Remotely initiated media transmission during an MCVideo session

Figure 7.7.1.3.7-1 describes the procedure for remotely initiated media transmission during an MCVideo session by an authorized user. Only two UEs involved in the session are shown for the simplicity.

Pre-condition:

1.	MCVideo session is established between MCVideo clients (client A and client B) and MCVideo server

2.	Transmission control is established between transmission control participants and transmission control server and there is no ongoing media transmission.

3.	The user of the UE at the transmission control participant A is authorized to remotely initiate a media transmission from transmission control participant B

Figure 7.7.1.3.7-1: Remotely initiated media transmission during an MCVideo session

1.	Transmission control participant A sends a remote transmit media request message to transmission control server including the MCVideo ID of the user at the UE of the transmission control participant B and the MCVideo ID of the user at the UE of the transmission control participant A.

2.	Transmission control server checks whether the user at the UE of transmission control participant A is authorized to remotely initiate a media transmission from transmission control participant B. If authorized, the transmission control server makes the determination on what action (grant, deny, or queue) to take on the request based on transmission control criteria (e.g., group policy) and determines to accept the remotely initiated transmit media request from transmission control participant A.

3.	Transmission control server sends a transmit media granted message to transmission control participant B.

4.	Transmission control server provides a remote transmit media response message to transmission control participant A indicating success (media transmission occurs from transmission control participant B) or a failure reason.

5.	Transmission control participant B starts sending video media over the session established to the transmission control server.

### 7.7.2	Off-network transmission control

#### 7.7.2.1	General

The procedure is for providing transmission control to MCVideo UE in an off-network case. Transmission control is performed by using transmission control information flows between the transmission control participant and the transmission control arbitrator. The transmission control arbitrator is a member MCVideo UE of the MCVideo group where the transmission rules are applied.

Off-network transmission control is based on ProSe capabilities as described in clause 7.18.

Transmission control in off-network can be performed in two ways:

-	Single arbitrator: transmission participants rely on a single participant designated as transmission arbitrator for the arbitraton of transmission requests.

-	Self arbitration: each transmsission participant arbitrates its own transmission based on its view of the topology.

Both of the approaches, as appropriate for the deployment model, can be adopted for MCVideo group using a configurable parameter (as defined in Annex A.4).

In the single arbitrator approach, one MCVideo client assumes the responsibility for arbitration of transmission requests for all group members within range. All requests for transmission are directed to the arbitrator, and the arbitrator checks the configured limits on the simultaneous transmissions, and grants or denies the request. If an MCVideo client is out of range of the current arbitrator, the MCVideo client is allowed to transmit and also become a transmission arbitrator.  If there is insufficient capacity to carry an extra transmission i.e. the configured limit for simultaneous transmissions is reached, the MCVideo client may request that an existing transmitting MCVideo client is pre-empted; the pre-emption request is sent to the transmission arbitrator.

In the self arbitration approach, each MCVideo client decides for itself whether there is sufficient capacity to carry the transmission. If it determines that there is insufficient capacity i.e. the configured limit for simultaneous transmissions is reached, and from its perspective another transmitting MCVideo client has a lower priority, the requesting MCVideo client may send an override request directly to this other transmitting MCVideo client, which will either accept the override request and give way, or deny the override request.

In both the single arbitrator approach and the self arbitration approach, if there is insufficient capacity to carry the communication i.e. the configured limit on the simultaneous transmissions is reached, the MCVideo client may report this to the MCVideo user. The MCVideo user may decide to transmit anyway, and instruct the MCVideo client to proceed with the transmission.

NOTE:	The ProSe function within the MCVideo client could determine that there is insufficient capacity to carry an MCVideo call requested by the MCVideo client, however interactions between the MCVideo client and the ProSe function are outside the scope of the present document.

Further subclauses apply to one or both of the single arbitrator approach and the self arbitration approach. Applicability is explicitly indicated in each of the relevant subclauses.

#### 7.7.2.2	Information flows for off-network transmission control

##### 7.7.2.2.1	Transmission request

Table 7.7.2.2.1-1 describes the information flow for the transmission request sent by a transmission participant to request for the transmission permission. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.1-1: Transmission request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting the transmission permission |
| Transmission priority | M | Priority of the request |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.2.2.2	Transmission granted

Table 7.7.2.2.2-1 describes the information flow for the transmission granted sent by the transmission arbitrator, to indicate that a request for transmission is granted and media may be transmitted. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.2-1: Transmission granted

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID | M | Identity of the user that has been granted transmission permission |
| Duration | M | The time for which the granted party is allowed to transmit |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| MCVideo ID of subsequent arbitrator | O | Subsequent transmission arbitrator's identity |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

##### 7.7.2.2.3	Transmission release

Table 7.7.2.2.3-1 describes the information flow for transmission release sent by the transmission participant, to indicate that the media transmission is complete and transmission permission is released. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.3-1: Transmission release

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of user releasing transmission |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

##### 7.7.2.2.4	Transmission rejected

Table 7.7.2.2.4-1 describes the information flow for transmission rejected sent by the transmission arbitrator, to indicate that a request for the transmission is rejected. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.4-1: Transmission rejected

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID of rejected party | M | Identity of user whose transmission request has been rejected |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Rejection cause | O | Indicates the cause for transmission rejection |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

##### 7.7.2.2.5	Transmission revoked

Table 7.7.2.2.5-1 describes the information flow for transmission revoked sent by the transmission arbitrator, to indicate that the transmission permission, that was earlier granted, is revoked. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.5-1: Transmission revoked

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID | M | Identity of user whose transmission permission has been revoked |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

##### 7.7.2.2.6	Transmission arbitration taken

Table 7.7.2.2.6-1 describes the information flow for transmission taken sent by the transmission participant, to indicate that the transmission participant has taken the responsibility of transmission arbitration. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.6-1: Transmission arbitration taken

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the MCVideo user taking responsibility of transmission arbitration |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |
| Permission to request the transmission | O | Indicates whether receiving parties are allowed to request the transmission or not (e.g. broadcast call). |
| Acknowledgement required | O | Indicates if acknowledgement from the transmission participant is required |

##### 7.7.2.2.7	Transmission arbitration release

Table 7.7.2.2.7-1 describes the information flow for transmission arbitration release sent by the transmission arbitrator, to indicate that the responsibility of transmission arbitration is released. This information flows is sent in unicast or broadcast.

Table 7.7.2.2.7-1: Transmission arbitration release

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | Identity of the user whose client is acting as transmission arbitrator |
| MCVideo ID | O | Identity of the user whose client is being delegated transmission arbitrator function |
| Source identifier | O | Identifies the communication, e.g. by identifying the media flow within a media multiplex, present only if media multiplexing |

#### 7.7.2.3	Initializing transmission control – single arbitrator approach

This subclause is applicable only in single arbitrator approach.

Figure 7.7.2.3-1 describes procedures for transmission participants when an MCVideo client initializes transmission control. The MCVideo client sends a transmission request to detect presence of a transmission arbitrator. If the MCVideo client does not receive a response to the transmission request, it sends a transmission arbitration taken message and becomes the transmission arbitrator. The MCVideo client may now transmit a video. This procedure applies when either there have been no recent MCVideo transmissions within the MCVideo group and therefore no arbitrator has been selected, or where there have been recent MCVideo transmissions which may still be ongoing, but the arbitrator cannot be reached, e.g. where MCVideo client that wants to transmit video has gone out of range of the arbitrator.

Pre-conditions:

1.	An off-network group communication has been established.

2.	MCVideo client 1 wishes to transmit video, and has determined that the pre-configured limit on the number of transmissions within the MCVideo group has not been reached.

Figure 7.7.2.3-1: Initializing transmission control, single arbitrator case

1.	MCVideo client 1 sends a transmission request message to the MCVideo group.

2.	MCVideo client 1 does not detect any response to the transmission request.

3a.	MCVideo client 1 sends a transmission arbitration taken message to the MCVideo group.

3b.	MCVideo user may be notified that the video can now be transmitted.

4.	MCVideo client 1 transmits video to the MCVideo group.

#### 7.7.2.3A	Initializing transmission control – self arbitration approach

This subclause is applicable only in self arbitration approach.

Figure 7.7.2.3a-1 describes procedures for transmission participants when an MCVideo client initializes transmission control.

Pre-conditions:

1.	An off-network group communication has been established.

2.	MCVideo client 1 wishes to transmit video.

Figure 7.7.2.3a-1: Initializing transmission control, self arbitration case

1.	The MCVideo client checks whether the pre-configured limit on the number of transmissions within the MCVideo group has been reached and informs the user. If the pre-configured limit on the number of transmissions within the MCVideo group has been reached, the MCVideo user may defer the transmission, or request an override as specified in subclause 7.7.2.8, or decide to continue with the transmission anyway.

NOTE:	If the MCVideo user decides to transmit even if the pre-configured limit on the number of transmissions within the MCVideo group has been reached, the MCVideo user decides to accept any consequences of interference.

2.	MCVideo client 1 sends a transmission arbitration taken message to the MCVideo group.

3.	MCVideo client 1 transmits video to the MCVideo group.

#### 7.7.2.4	Transmission permission granted

This subclause is applicable only in single arbitrator approach.

Figure 7.7.2.4-1 describes procedures for transmission participants when an MCVideo client requests for transmission permission.

The MCVideo client has detected presence of a transmission arbitrator e.g., by receiving responses to transmission arbitration control message. The MCVideo client sends a transmission request and waits for a response. The MCVideo client upon receiving a transmission granted message may transmit a video.

Pre-conditions:

1.	An off-network group communication has been established.

2.	At least one participant is currently transmitting video.

Figure 7.7.2.4-1: Requesting transmission permission

1.	MCVideo client 2 sends a transmission request message to the MCVideo group.

2.	MCVideo client 1, being the transmission arbitrator, checks if the configured limit of maximum simultaneous transmissions is already reached.

3.	If the maximum simultaneous transmissions limit is not reached, MCVideo client 1 sends a transmission granted message to the MCVideo group. Transmission granted message indicates MCVideo client 2 as the intended recipient.

4.	MCVideo user at MCVideo client 2 may be notified that the video can now be transmitted.

5.	MCVideo client 2 transmits video to the MCVideo group.

#### 7.7.2.5	Transmission permission rejected

This subclause is applicable only in single arbitrator approach.

Figure 7.7.2.5-1 describes procedures for transmission participants when an MCVideo client requests for transmission permission.

The MCVideo client has detected presence of a transmission arbitrator e.g., by receiving responses to transmission arbitration control message. The MCVideo client sends a transmission request and waits for a response.

Pre-conditions:

1.	An off-network group communication has been established.

2.	At least one participant is currently transmitting video.

Figure 7.7.2.5-1: Requesting transmission permission

1.	MCVideo client 2 sends a transmission request message to the MCVideo group.

2.	MCVideo client 1, being the transmission arbitrator, checks if the configured limit of maximum simultaneous transmissions is already reached.

3.	If the maximum simultaneous transmissions limit has reached, MCVideo client 1 sends a transmission rejected message to the MCVideo group. Transmission denied message indicates MCVideo client 2 as the intended recipient. The transmission rejected message may include a rejection cause which indicates that the configured maximum transmissions limit has been reached.

4.	MCVideo user at MCVideo client 2 may be notified that the video cannot be transmitted right now.

Following step 4, the MCVideo user at MCVideo client 2 may decide to transmit anyway, for example if the user knows the topology of the off-network MCVideo group and locations of the MCVideo group members and needs to transmit video to other local MCVideo group members despite causing a potential conflict with one or more other existing MCVideo transmissions within the MCVideo group. In this case, the MCVideo client follows the procedure in subclause 7.7.2.3.

#### 7.7.2.6	Releasing transmission permission

This subclause is applicable in both the single arbitrator and self arbitration approaches.

Figure 7.7.2.6-1 describes procedures for transmission participants when an MCVideo client releases transmission permission.

The MCVideo client has detected presence of a transmission arbitrator e.g., by receiving responses to transmission arbitration control message. The MCVideo client stops the video transmission and sends a transmission release request.

Pre-conditions:

1.	An off-network group communication has been established.

2.	MCVideo client has requested transmission permission and may have received transmission permission.

Figure 7.7.2.6-1: Requesting transmission permission

1.	If transmitting, the MCVideo client 2 stops the transmission of the video.

2.	MCVideo client 2 sends a transmission release message to the MCVideo group.

NOTE:	The transmission arbitrator does not respond to a pending (not granted or denied) transmission request if a transmission release notification is received from the MCVideo client.

#### 7.7.2.7	Transmission override

This subclause is applicable in the single arbitrator in the approach.

Figure 7.7.2.7-1 describes procedures for transmission participants when an MCVideo client authorized to override, requests for transmission permission.

The MCVideo client has detected presence of a transmission arbitrator e.g., by receiving responses to transmission arbitration control message. The MCVideo client sends a transmission request and waits for a response.

Pre-conditions:

1.	An off-network group communication has been established.

2.	Maximum simultaneous transmissions limit has been reached.

Figure 7.7.2.7-1: Transmission override

1.	MCVideo client 2 sends a transmission request message to the MCVideo group.

2.	As the configured limit of maximum simultaneous transmissions is already reached, MCVideo client 1, being the transmission arbitrator, checks the override policy.

3.	If MCVideo client 2 is authorized to override (based on e.g., transmission priority), MCVideo client 1 sends a transmission revoked message to the MCVideo group. Transmission revoked message indicates the MCVideo client from which the permission is revoked, as the intended recipient.

4.	MCVideo client 3 stops transmission of video and MCVideo user at MCVideo client 3 may be notified that the transmission permission has been revoked.

5.	MCVideo client 1 sends a transmission granted message to the MCVideo group. Transmission granted message indicates MCVideo client 2 as the intended recipient.

6.	MCVideo user at MCVideo client 2 may be notified that the video can now be transmitted.

7.	MCVideo client 2 transmits video to the MCVideo group.

#### 7.7.2.8	Transmission override (revoke self)

This subclause is applicable in the single arbitrator approach.

Editor's note: transmission override in the self arbitration approach is FFS.

Figure 7.7.2.8-1 describes procedures for transmission participants when an MCVideo client authorized to override, requests for transmission permission.

The MCVideo client has detected presence of a transmission arbitrator e.g., by receiving responses to transmission arbitration control message. The MCVideo client sends a transmission request and waits for a response.

Pre-conditions:

1.	An off-network group communication has been established.

2.	Maximum simultaneous transmissions limit has been reached.

Figure 7.7.2.8-1: Transmission override

1.	MCVideo client 2 sends a transmission request message to the MCVideo group.

2.	As the configured limit of maximum simultaneous transmissions is already reached, MCVideo client 1, being a transmission arbitrator, checks the override policy.

3.	If MCVideo client 2 is authorized to override (based on e.g., transmission priority), MCVideo client 1 may send a transmission revoked message to the MCVideo group. Transmission revoked message indicates the MCVideo client 1, the current arbitrator, whose permission is revoked.

4.	MCVideo client 1 stops transmission of video and MCVideo user at MCVideo client 1 may be notified that the transmission permission has been revoked.

5.	MCVideo client 1 sends a transmission granted message to the MCVideo group. The transmission granted message indicates MCVideo client 2 as the intended recipient and MCVideo client 2 as the subsequent transmission arbitrator.

6a.	MCVideo client 2 sends a transmission arbitration taken message to the MCVideo group.

6b.	MCVideo user at MCVideo client 2 may be notified that the video can now be transmitted.

NOTE:	Step 6a and step 6b can occur in any order.

7.	MCVideo client 1, upon receiving the transmission arbitration taken message releases transmission arbitration.

8.	MCVideo client 2 transmits video to the MCVideo group.

#### 7.7.2.9	Transmission arbitration release

##### 7.7.2.9.1	Transmission arbitration release

This subclause is applicable only in single arbitrator approach.

Figure 7.7.2.9.1-1 describes procedures for an MCVideo client to release transmission arbitration. There is no other MCVideo client to which transmission arbitration can be delegated.

Pre-conditions:

1.	An off-network group communication has been established.

2.	Only MCVideo client 1 is transmitting a video.

Figure 7.7.2.9.1-1: Transmission arbitration release without delegation

1.	MCVideo client 1 sends a transmission release message to the MCVideo group.

2.	MCVideo client 1 stops transmission of the video.

##### 7.7.2.9.2	Transmission arbitration release with delegation

This subclause is applicable only in single arbitrator approach.

Figure 7.7.2.9.2-1 describes procedures for an MCVideo client to release transmission arbitration. There are other MCVideo clients currently transmitting to which transmission arbitration can be delegated.

Pre-conditions:

1.	An off-network group communication has been established.

2.	At least one more MCVideo client is transmitting video other than the current transmission arbitrator.

Figure 7.7.2.9.2-1: Transmission arbitration release with delegation

1.	MCVideo client 1 stops video transmission but does not release transmission arbitration.

2.	MCVideo client 1 sends a transmission arbitration release message. The transmission arbitration release message indicates a MCVideo client which is currently transmitting video, MCVideo client 2, as the subsequent transmission arbitrator. The MCVideo client 1 waits for a confirmation before releasing transmission arbitration.

3.	MCVideo client 1 detects no response from MCVideo client 2.

4.	MCVideo client 1 sends another transmission arbitration release message. The transmission arbitration release message indicates another MCVideo client which is currently transmitting video, MCVideo client 3, as the subsequent transmission arbitrator. The MCVideo client 1 waits for a confirmation before releasing transmission arbitration.

5.	MCVideo client 3 sends a transmission arbitration taken message to the MCVideo group.

6.	MCVideo client 1, upon receiving the transmission arbitration taken message releases transmission arbitration.

#### 7.7.2.10	Simultaneous transmission requests

This subclause is applicable in both the single arbitrator and self arbitration approaches.

Figure 7.7.2.10-1 describes procedures for transmission participants when simultaneous transmission requests are generated when more than one MCVideo client initializes transmission control.

Figure 7.7.2.10-1 shows the expected behaviour if simultaneous transmission requests are generated in the following scenarios:

-	a single arbitrator approach  is used but there is currently no arbitrator; or

-	self arbitration is used.

Pre-conditions:

1.	An off-network group communication has been established.

2.	MCVideo client 1 has higher transmission priority than MCVideo client 2.

Figure 7.7.2.10-1: Simultaneous transmission requests

1a.	The MCVideo client 1 sends the transmission request message to the MCVideo group.

1b.	The MCVideo client 2 sends the transmission request message to the MCVideo group.

NOTE:	Step 1a and 1b happen in parallel

2.	On receiving a transmission request message, while waiting for a response to the sent transmission request message, the MCVideo client compares its transmission priority with the transmission priority indicated in the received transmission request message.

3.	On determining that it has higher transmission priority than the received transmission request message(s), and no response to the sent transmission request message is received, the MCVideo client 1 sends the transmission arbitration taken message to the MCVideo group.

4.	MCVideo user may be notified that the video can now be transmitted.

## 7.8	MCVideo service configuration

The MCVideo service shall support the procedures and related information flows as specified in subclause 10.1 of 3GPP TS 23.280 [6] with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server;

-	The MC service ID is the MCVideo ID; and

-	The MC service user profile index is the MCVideo user profile index.

## 7.9	Affiliation and de-affiliation to/from MCVideo group(s)

The MCVideo service shall support the procedures and related information flows as specified in subclause 10.8 of 3GPP TS 23.280 [6] with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server;

-	The MC service group is the MCVideo group;

-	The MC service ID is the MCVideo ID; and

-	The MC service group ID is the MCVideo group ID.

When an MCVideo user has affiliated to an MCVideo group then the MCVideo user can send and receive MCVideo related media for that MCVideo group. When an MCVideo user has de-affiliated from an MCVideo group then the MCVideo user cannot send and receive MCVideo related media to and from that MCVideo group.

## 7.9a	Functional alias management

The MCVideo service shall support the functional alias activation, de-activation, taken over, subscription and notification and functional alias to group binding procedures and related information flows as specified in subclause 10.13 of 3GPP TS 23.280 [6].

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server;

-	The MC service ID is the MCVideo ID.

## 7.10	Use of MBMS transmission (on-network)

### 7.10.1	Information flows for MBMS Transmission

#### 7.10.1.1	General

Information flows for generic MBMS procedures are defined in 3GPP TS 23.280 [6].

#### 7.10.1.2	MapGroupToBearer

Table 7.10.1.2-1 describes the information flow to associate a MCVideo group call to a MBMS bearer. It is sent from the MCVideo server to the MCVideo client.

Table 7.10.1.2-1: MapGroupToBearer

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo group ID | M | This element identifies the MCVideo group, in which the call is started. |
| Media stream identifier (video) | M | This element identifies the media stream of the SDP used for the Video group call (e.g. MBMS subchannel). |
| Media stream identifier (audio) | O | This element identifies the audio media stream of the SDP used for the Video group call (e.g. MBMS subchannel). This is used if separate codecs are used for video and audio. |
| TMGI (see NOTE) | O | The MBMS bearer identifier if the media of the MCVideo group call is not sent on the same MBMS bearer as this MapGroupToBearer message. |
| Call acknowledgement indicator | O | Indication that the MCVideo group call requires acknowledgement from receiving MCVideo clients. |
| NOTE: TMGI shall be present if this message is sent over a different MBMS bearer than the media of the group call; TMGI may be present if this message is sent over the same MBMS bearer as the media of the group call. |  |  |

#### 7.10.1.3	UnmapGroupFromBearer

Table 7.10.1.3-1 describes the information flow to disconnect a MCVideo group call from a MBMS bearer. It is sent from the MCVideo server to the MCVideo client.

Table 7.10.1.3-1: UnmapGroupFromBearer

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo group ID | M | This element identifies the MCVideo group which will no longer use the MBMS bearer. |

### 7.10.2	Use of pre-established MBMS bearers

The MCVideo service shall support the procedure for using pre-established MBMS bearers as specified 3GPP TS 23.280 [6] with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server; and

-	The MC service ID is the MCVideo ID.

The MCVideo service shall use the MCVideo-1, MCVideo-8 and MCVideo-9 reference points for this procedure.

MCVideo may use pre-established MBMS bearer for the different types of MCVideo group calls. Both pre-arranged group calls and chat group calls can use the pre-established MBMS bearer for distributing the media. The MBMS bearer can be used by any group. Depending on the capacity of the MBMS bearer, the bearer can be used to broadcast one or more group calls in parallel.

Both the media packets as well as the transmission control messages to the receiving MCVideo clients are sent on the MBMS bearer. Optionally a separate MBMS bearer could be used for the transmission control messages, due to different bearer characteristic requirements.

When using the pre-established MBMS bearer for MCVideo, the MCVideo server perform the procedure of call connect and disconnect over MBMS as defined in subclause 7.10.4 at the group communication session establishment step.

### 7.10.3	Use of dynamic MBMS bearer establishment

The MCVideo service shall support the procedure for using dynamic MBMS bearers as specified 3GPP TS 23.280 [6] with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server; and

-	The MC service ID is the MCVideo ID.

The MCVideo service shall use the MCVideo-1, MCVideo-4, MCVideo-7, MCVideo-8 and MCVideo-9 reference points for this procedure.

MCVideo may use dynamic MBMS bearer establishment for the different types of MCVideo group calls. Both pre-arranged group calls and chat group calls can use the dynamic MBMS bearer for distributing the media.

Both the media packets as well as the transmission control messages to the receiving MCVideo clients are sent on the MBMS bearer.

When using the procedures for dynamic MBMS bearer establishment for MCVideo, the MCVideo server perform the procedure of mapping group communication to MBMS bearer as defined in subclause 7.10.4 at the group communication session establishment step.

### 7.10.4	Video call connect and disconnect over MBMS

#### 7.10.4.1	General

MBMS bearer can be used for MCVideo group calls. One MBMS bearer is not permanently associated to one specific group or group call. Before sending media packets of a group call over MBMS bearer, the MCVideo server shall send the association information between group call and the MBMS bearer. The group call setup procedure indicates the media stream or media streams within one MBMS bearer that is used for the specific MCVideo group call. When the group call over the MBMS bearer is finished, this temporary association information of an MCVideo group call to specific resources on a MBMS bearer is undone. The procedure in figure 7.10.4.2.1-1 requires that the group session is setup before the call start. This eliminates the need for the receiving clients to continuously use a unicast bearer. Prior to this the MBMS bearer is activated and announced to the MCVideo clients.

#### 7.10.4.2	Procedure

##### 7.10.4.2.1	Call connect over MBMS

In figure 7.10.4.2.1-1 the MCVideo client 1 is the client that initiate an MCVideo chat group call and also the transmitting client. MCVideo client 1 may, but does not have to be in an MBMS service area. The MCVideo client 2 and MCVideo client 3 represents MCVideo clients receiving the MCVideo call over an MBMS bearer. There may be other receiving clients both over unicast bearers and over this or other MBMS bearer(s), however they are not illustrated in this figure.

The same procedure as for chat group call can also be applied for pre-arranged calls.

Pre-conditions:

-	All users participating in the MCVideo group call are already affiliated to the group.

-	All participating users have joined the group session.

Figure 7.10.4.2.1-1: Chat group call connect on MBMS bearer

1.	Activation and announcement of MBMS bearer availability.

NOTE 1:	The procedure does not include the steps for MCVideo client location reporting, or for MBMS capability information exchange.

2.	MCVideo client 1 initiate the MCVideo group call by sending an initial transmit media request over a unicast bearer to the MCVideo server (reference point MCVideo-4).

3.	The MCVideo server will send a MapGroupToBearer message over a previously activated MBMS bearer to all users that will receive the call over an MBMS bearer. The MapGroupToBearer message includes association information between the group call and MBMS bearer. The MapGroupToBearer message includes MCVideo group ID and information about the media stream identifier or media stream identifiers of the activated MBMS bearer and may include the identifier (i.e. the TMGI) of the MBMS bearer broadcasting the call. The message is sent over reference point MCVideo-9.

NOTE 2:	Step 3 can be deferred until step 5 and the MapGroupToBearer message can then be included in the transmission arbitration taken message.

4.	The MCVideo server grants the right to transmit for MCVideo client 1 and by that sends a transmit media granted message to the MCVideo client 1. This message is sent over a unicast bearer (reference point MCVideo-4).

5.	A media transmission notification message is sent from the MCVideo server to all receiving users. This message includes the MCVideo ID of the transmitting MCVideo client as well as the MCVideo group ID. The message is sent over a MBMS bearer to all users that have previously been setup to receive calls over the MBMS bearer. The message is sent over reference point MCVideo-9.

6.	The users of MCVideo clients are informed about the details of video tranmission and the video transmitter.

7.	The MCVideo clients may send a receive media request to the MCVideo server, to indicate the reception of media over the MBMS bearer.

8.	MCVideo server accepts the media receive request and sends a receive media response message to MCVideo clients over a MBMS bearer by using reference point MCVideo-9.

NOTE 3:	The receive media response can also be sent by unicast to the MCVideo clients that request to receive media over the MBMS bearer.

NOTE 4:	Step 7 and 8 may be omitted when auto-receive (i.e. forced reception mode) transmission mode is used based on configurations (e.g. for emergency group calls, imminent peril calls etc) as described in Annex A.

9.	The media is sent from MCVideo client 1 to the MCVideo server over unicast bearer and from the MCVideo server to the MCVideo client 2 and MCVideo client 3 over MBMS bearer.

NOTE 5:	Additional transmission request messages in the same call will not trigger the MapGroupToBearer message to be sent.

##### 7.10.4.2.2	Call disconnect over MBMS

Figure 7.10.4.2.2-1 shows the high level procedure where an UnmapGroupFromBearer message is sent by the MCVideo server to the MCVideo clients to indicate that the MCVideo group call is being dissociated from the MBMS bearer.

Figure 7.10.4.2.2-1: Chat group call disconnect on MBMS bearer

1.	An MCVideo group call is ongoing; the media is broadcasted over MBMS bearer to MCVideo client 2 and MCVideo client 3.

2.	MCVideo server has determined to disconnect the call over the MBMS bearer for the MCVideo clients

3.	An UnmapGroupFromBearer message is sent by the MCVideo server to MCVideo client 2, MCVideo client 3 and possibly to MCVideo client 1 (if in MBMS coverage area) on MBMS bearer(s).

NOTE 1:	The UnmapGroupFromBearer message can be sent as part of other messages.

NOTE 2:	The UnmapGroupFromBearer message and the MapGroupToBearer message can be combined, to facilitate switching bearers in one signalling step, as necessary.

### 7.10.5		Switching from MBMS bearer to unicast bearer

The MCVideo service shall support the procedure for switching from MBMS bearer to unicast bearer as specified 3GPP TS 23.280 [6] with the following clarifications:

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server; and

-	The MC service ID is the MCVideo ID.

The MCVideo service shall use the MCVideo-1, MCVideo-4, MCVideo-7, MCVideo-8 and MCVideo-9 reference points for this procedure.

## 7.11	Simultaneous session for MCVideo calls (on-network)

An MCVideo client and MCVideo server may use a simultaneous session as defined in 3GPP TS 23.280 [6] for MCVideo calls. The MCVideo client becomes involved in a simultaneous session for MCVideo calls by inviting, joining or accepting more than one MCVideo call, or affiliating to a group.

NOTE:	An MCVideo client affiliating to multiple MCVideo groups with active calls will result in the MCVideo client being invited simultaneously to multiple MCVideo calls.

The MCVideo client can also still handle multiple MCVideo calls in parallel at the same time i.e. using multiple dialogs.

The simultaneous session is established during either an originating on-demand call establishment or during pre-established session establishment or a modification of an already established pre-established session or on-demand call.

It is possible to change the prioritisation while the MCVideo client is engaged in multiple MCVideo calls. The setting of the priority can be made at MCVideo call setup or by performing a modification after the MCVideo call is established. This may result in more than one media bearer.

## 7.12	User authentication and authorization for MCVideo service

NOTE:	Flow 7.12-1 is a high level user authentication and authorization flow. 3GPP TS 33.180 [14] defines the specific user authentication and authorization architecture required by the MCVideo service in order to realize the MCVideo user authentication and authorization requirements as defined in 3GPP TS 22.280 [2].

A procedure for user authentication is illustrated in figure 7.12-1. The user authentication is performed based on the procedure specified in 3GPP TS 23.280 [6].

Figure 7.12-1: MCVideo user authentication and registration, single domain

1.	The user authentication is performed as per the general user authentication procedure specified in 3GPP TS 23.280 [6].

2.	MCVideo client performs the MCVideo service authorization for the user. Step 2 utilizes the results of step 1.

## 7.13	Support for multiple devices

An MCVideo user may be authorized to use the MCVideo service from multiple MCVideo UEs as per the procedure in subclause 7.12.

If an MCVideo server receives a service authorization request for an MCVideo user who is previously MCVideo service authorized on another MCVideo UE, then the MCVideo server shall process this service authorization request as described in subclause 7.12. In the MCVideo service authorization response to the MCVideo user, the MCVideo server shall also indicate that the MCVideo user is already MCVideo service authorized from another MCVideo UE.

## 7.14	Location information (on-network)

The MCVideo system makes use of all of the procedures for location management as specified in 3GPP TS 23.280 [6], utilising the CSC-14 reference point between the location management client and location management server and the CSC-15 reference point between the MCVideo server and location management server.

-	The MC service client is the MCVideo client;

-	The MC service server is the MCVideo server;

-	The MC service group is the MCVideo group;

-	The MC service ID is the MCVideo ID; and

-	The MC service group ID is the MCVideo group ID.

## 7.15	MCVideo resource management (on-network)

Procedures for resource management are defined in subclause 10.11 of 3GPP TS 23.280 [6].

## 7.16	MCVideo client query

### 7.16.1	General

The MCVideo client query is used by an authorized MCVideo client to retrieve the characteristics of the MCVideo clients  in an area with certain criteria. The query criteria may be a set of capabilities, or category tags, or a mix of such criterions, etc.

### 7.16.2	Information flows for MCVideo client query

#### 7.16.2.1	MCVideo client query request

Table 7.16.2.1-1 describes the information flow for the MCVideo client query request from MCVideo client to MCVideo sever.

Table 7.16.2.1-1: MCVideo client query request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting MCVideo client query |
| Location information | M | The location information where the query to be applied |
| Query criteria | O | A criteria filter for the query (e.g. category tags, video capabilities, etc.) |

#### 7.16.2.2	MCVideo client query response

Table 7.16.2.2-1 describes the information flow for the MCVideo client query response from the MCVideo server to the MCVideo client.

Table 7.16.2.2-1: MCVideo client query response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID list | O | The MCVideo clients that fulfil the query criteria |

### 7.16.3	MCVideo client query procedure

Figure 7.16.3-1 describes the procedure for MCVideo client query in on-network. This procedure is applicable to query the MCVideo clients belonging to the same MCVideo system as the requesting MCVideo user.

Pre-conditions:

1.	The video capabilities have been collected at MCVideo server during the update MCVideo capabilities information at the MCVideo server procedure in subclause 7.5.2.3.

Figure 7.16.3-1: MCVideo client query procedure

1.	MCVideo client A sends a MCVideo client query request to the MCVideo server with certain search criteria.

2.	MCVideo server checks whether the MCVideo user of MCVideo client A has the authorization to perform MCVideo client query. If success, MCVideo server retrieves the MCVideo clients that fulfils the search criteria. If MCVideo group ID list is included in the MCVideo client query request, MCVideo server retrieves the affiliated group members that fulfil the query criteria in each affiliated MCVideo group in the received MCVideo group ID list.

3.	MCVideo server returns the MCVideo client query response to the MCVideo client A with the retrieved MCVideo clients.

Editor's note:	It is FFS when the requesting MCVideo user and the MCVideo users or the affiliated groups to be requested are not within the same MCVideo system.

## 7.17	MCVideo adaptation during MCVideo communication

### 7.17.1	General

The MCVideo adaption is used by an authorized MCVideo client to automatically change the video communication parameters including codec, resolution etc. according to the network conditions changes detected through such as packet loss or packet delay.

### 7.17.2	Information flows for MCVideo adaptation

#### 7.17.2.1	MCVideo communication parameter update request

Table 7.17.2.1-1 describes the information flow for the MCVideo communication parameter update request from MCVideo client to MCVideo sever and from MCVideo server to MCVideo client.

Table 7.17.2.1-1: MCVideo communication parameter update request

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The identity of the MCVideo user requesting to update MCVideo communication parameter |
| MCVideo group ID | O (see NOTE) | The identity of the MCVideo group which the MCVideo communication parameter to be updated (Only used in group communication) |
| MCVideo ID | O (see NOTE) | The identity of the MCVideo user ID which the MCVideo communication parameter to be updated (Only used in private communication) |
| SDP | M | The new SDP including codec, resolution, frame rate and etc. for MCVideo communication |
| NOTE: At least one of these information elements shall be present |  |  |

#### 7.17.2.2	MCVideo communication parameter update response

Table 7.17.2.2-1 describes the information flow for the MCVideo communication parameter update response from the MCVideo server to the MCVideo client and from MCVideo client to MCVideo server.

Table 7.17.2.2-1: MCVideo communication parameter update response

| Information Element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure |

### 7.17.3	MCVideo adaptation procedure

#### 7.17.3.1	MCVideo adaptation during group communication procedure – transmitting client triggered

Figure 7.17.3.1-1 describes the procedure for transmitting MCVideo client triggered MCVideo communication parameters update in on-network. For simplicity, only one receiving user is shown in the figure. This procedure is applicable to only one MCVideo transmitting user during the MCVideo group call, e.g., broadcast group call.

NOTE:	The MCVideo adaptation for MCVideo group communication allows multiple transmitting users are not specified in this release.

Pre-conditions:

1.	MCVideo client 1 and MCVideo client 2 are the group communication participants, and the MCVideo client 1 is transmitting video in the MCVideo group communication.

Figure 7.17.3.1-1: Transmitting user triggered MCVideo adaptation in group communication

1.	MCVideo client 1 sends MCVideo communication parameter update request to the MCVideo server to adjust the video communication parameters due to network conditions changes detected, e.g., through packet loss or packet delay. The new parameters including codec, resolution, frame rate and etc. are included.

2.	MCVideo server checks whether the MCVideo user of MCVideo client 1 has the authorization to update the MCVideo communication parameters.

3.	When authorized, MCVideo server sends the MCVideo communication parameter update request to the MCVideo client 2.

4.	The MCVideo user of MCVideo client 2 is notified about the changes of MCVideo communication parameters.

5.	The MCVideo client 2 accepts the request, and sends the MCVideo communication parameter update response to the MCVideo server.

6.	The MCVideo server sends the MCVideo communication parameter update response to the MCVideo client 1 with the result.

7.	Then, the MCVideo group communication applies the new MCVideo communication parameters.

#### 7.17.3.2	MCVideo adaptation during group communication procedure – receiving user triggered

Figure 7.17.3.2-1 describes the procedure for MCVideo server initiated MCVideo communication parameters update in on-network. For simplicity, only one receiving user is shown in the figure. This procedure is applicable to only one MCVideo transmitting user during the MCVideo group call e.g., broadcast group call.

Pre-conditions:

1.	MCVideo cient 1 and MCVideo client 2 are the group communication participants. MCVideo client 2 is transmitting video, and MCVideo client 1 is receiving video.

Figure 7.17.3.2-1: Receiving user triggered MCVideo adaptation in group communication

1.	MCVideo client 1 sends MCVideo communication parameter update request to the MCVideo server to request the video communication parameters due to network conditions changes detected, e.g., through packet loss. The expected parameters including codec, resolution, frame rate and etc. are included.

2.	MCVideo server may receive a multitude of notifications from the receiving users. The MCVideo server performs authorization check and determines to update the MCVideo communication parameters according to the received notifications.

NOTE 1:	The determination of update the MCVIdeo communication parameters is implementation.

3.	When authorized, MCVideo server sends the MCVideo communication parameter update request to the MCVideo client 2.

4.	The MCVideo users of MCVideo client 1 and MCVideo client are notified about the changes of MCVideo communication parameters.

5.	The MCVideo client 1 and MCVideo client 2 accepts the request, and sends the MCVideo communication parameter update response to the MCVideo server.

6.	Then, the MCVideo group communication applies the new MCVideo communication parameters.

#### 7.17.3.3	MCVideo adaptation during private communication procedure – transmitting client triggered

Figure 7.17.3.3-1 describes the procedure for transmitting MCVideo client initiated MCVideo communication parameters update in on-network. This procedure is applicable to the video push and video pull.

Pre-conditions:

1.	The private communication is ongoing between MCVideo cient 1 and MCVideo client 2, and MCVideo client 1 is transmitting video to MCVideo client 2.

Figure 7.17.3.3-1: Transmitting user initiated MCVideo adaptation in private communication

1.	MCVideo client 1 sends MCVideo communication parameter update request to the MCVideo server to adjust the video communication parameters due to network conditions changes detected, e.g., through packet loss. The new parameters including codec, resolution, frame rate and etc. are included.

2.	MCVideo server checks whether the MCVideo user of MCVideo client 1 has the authorization to update the MCVideo communication parameters.

3.	When authorized, MCVideo server sends the MCVideo communication parameter update request to the MCVideo client 2.

4.	The MCVideo user of MCVideo client 2 is notified about the changes of MCVideo communication parameters.

5.	The MCVideo client 2 accepts the request, and sends the MCVideo communication parameter update response to the MCVideo server.

6.	The MCVideo server sends the MCVideo communication parameter update response to the MCVideo client 1 with the result.

7.	Then, the MCVideo communication applies the new MCVideo communication parameters.

#### 7.17.3.4	MCVideo adaptation during group communication procedure – receiving user triggered

Figure 7.17.3.4-1 describes the procedure for receiving user initiated MCVideo communication parameters update in on-network. This procedure is applicable in video push and video pull.

Pre-conditions:

1.	The private communication is ongoing between MCVideo cient 1 and MCVideo client 2, and the MCVideo client 1 is receiving video from MCVideo client 2.

Figure 7.17.3.4-1: Receiving user triggered MCVideo adaptation in private communication

1.	MCVideo client 1 sends MCVideo communication parameter update request to the MCVideo server to request the video communication parameters due to network conditions changes detected, e.g., through packet loss. The expected parameters including codec, resolution, frame rate and etc. are included.

2.	MCVideo server checks whether the MCVideo user of MCVideo client 1 has the authorization to update the MCVideo communication parameters.

3.	When authorized, MCVideo server sends the MCVideo communication parameter update request to the MCVideo client 2.

4.	The MCVideo user of MCVideo client 2 is notified about the request of MCVideo communication parameters update.

5.	The MCVideo client 2 accepts the request, and sends the MCVideo communication parameter update response to the MCVideo server.

6.	The MCVideo server sends the MCVideo communication parameter update response to the MCVideo client 1 with the result.

7.	Then, the MCVideo communication applies the new MCVideo communication parameters.

## 7.18	Use of ProSe capabilities in off-network MCVideo communications

### 7.18.1	General

When an MCVideo user using a ProSe-enabled UE wants to communicate with a specific MCVideo group or MCVideo user using ProSe capabilities, the MCVideo client enables the use of the ProSe layer procedures for public safety, as specified in 3GPP TS 23.303 [7].

For an off-network MCVideo group communication, the MCVideo client obtains configuration data such as the user info ID of the calling MCVideo user and the ProSe Group IP multicast address and ProSe Layer-2 Group ID associated to the target MCVideo group (as described in clause 8.1.3.2 in 3GPP TS 23.280 [6]), and provides it to the ProSe layer. The ProSe Layer-2 Group ID of the target MCVideo group may be used by the ProSe layer as the target group info and the discovery group ID (defined in 3GPP TS 23.303 [7]).

Prior to initiating an off-network MCVideo group communication, a group member discovery procedure may be initiated to identify whether other members of the target MCVideo group are in the proximity of the calling MCVideo user, as described in the ProSe direct discovery for public safety use procedure in 3GPP TS 23.303 [7]. The off-network MCVideo group communication using the ProSe capability is based on the one-to-many ProSe direct communication procedure for public safety use described in 3GPP TS 23.303 [7].

For an off-network one-to-one MCVideo communication, the MCVideo client obtains configuration data such as the ProSe discovery group ID and user info ID of the target MCVideo user from the "List of user(s) who can be called in MCVideo private call" in the MCVideo user profile and requests the IP address of the MCVideo UE associated with the target MCVideo user from the ProSe layer.

The MCVideo client enables the ProSe layer to determine the IP address for the communication with the target MCVideo UE by providing the ProSe discovery group ID and user info ID (as defined in 3GPP TS 23.303 [7]) associated to the target MCVideo user. This may trigger the ProSe direct discovery for public safety use procedure to identify whether the target MCVideo user is in the proximity of the calling MCVideo user. The user info ID of the target MCVideo user is used by the ProSe layer as the target info (as defined in 3GPP TS 23.303 [7]).

The ProSe layer can then provide the IP address related to the target MCVideo user to the MCVideo client to initiate the off-network one-to-one MCVideo communication based on the one-to-one ProSe direct communication procedure described in 3GPP TS 23.303 [7].

### 7.18.2	Procedures

The off-network MCVideo communication procedures using the ProSe capabilities are described within the corresponding clauses of each MCVideo capability, e.g. group communication procedures for off-network are described in clause 7.1.3 and private communication procedures for off-network are described in clause 7.2.3.

Annex A (normative):
MCVideo related configuration data

# A.1	General

This Annex provides information about the static data needed for configuration for the MCVideo service, which belongs to one of the following categories:

-	MCVideo UE configuration data (see subclause A.2);

-	MCVideo user profile configuration data (see subclause A.3);

-	MCVideo related group configuration data (see subclause A.4); and

-	MCVideo service configuration data (see subclause A.5).

For each configuration category, data is split between configuration data that is applicable to both on network and off network, configuration data that is applicable to on-network only, and configuration data that is applicable to off-network only. The configuration data in each configuration category corresponds to a single instance of the category type i.e. the MCVideo UE, MCVideo group, MCVideo user and MCVideo service configuration data refers to the information that will be stored against each MCVideo UE, MCVideo group, MCVideo user and MCVideo service. This means that the three separate tables (on-network and off-network, on-network only, off-network only) for each configuration category represent the complete set of data for each configuration data category element.

The columns in the tables have the following meanings:

-	Reference: the reference of the corresponding requirement in 3GPP TS 22.281 [3] or 3GPP TS 22.280 [2] or the corresponding subclause from either the present document or the referenced document.

-	Parameter description: A short definition of the semantics of the corresponding item of data, including denotation of the level of the parameter in the configuration hierarchy.

-	When it is not clear to which functional entities the parameter is configured, then one or more columns indicating this are provided where the following nomenclature is used:

-	"Y" to denote "Yes" i.e. the parameter denoted for the row needs to be configured to the functional entity denoted for the column.

-	"N" to denote "No" i.e. the parameter denoted for the row does not need to be configured to the functional entity denoted for the column.

Parameters within a set of configuration data have a level within a hierarchy that pertains only to that configuration data. The hierarchy of the configuration data is common across all three tables of on-network and off-network, on network only and off network only. The level of a parameter within the hierarchy of the configuration data is denoted by use of the character ">" in the parameter description field within each table, one per level. Parameters that are at the top most level within the hierarchy have no ">" character. Parameters that have one or more ">" characters are child parameters of the first parameter above them that has one less ">" character. Parent parameters are parameters that have one or more child parameters. Parent parameters act solely as a "grouping" of their child parameters and therefore do not contain an actual value themselves i.e. they are just containers for their child parameters.

Each parameter that can be configured online shall only be configured through one online reference point. Each parameter that can be configured offline shall only be configured through one offline reference point. The most recent configuration data made available to the MCVideo UE shall always overwrite previous configuration data, irrespective of whether the configuration data was provided via the online or offline mechanism.

# A.2	MCVideo UE configuration data

The general aspects of UE configuration are specified in 3GPP TS 23.280 [6]. Data in tables A.2-1 and A.2-2 have to be known by the MCVideo UE after MCVideo authorization.

Data in table A.2-1 can be configured offline using the CSC-11 reference point. Table A.2-1 contains the UE configuration required to support the use of off-network MCVideo service.

Table A.2-1: UE configuration data (on and off network)

| Reference | Parameter description |
| --- | --- |
| [R-5.4.2-002] of 3GPP TS 22.280 [2] | Maximum number of simultaneous MCVideo group calls (Nc4) |
| [R-5.4.2-003] of 3GPP TS 22.280 [2] | Maximum number of video transmissions (Nc5) in a group |

Table A.2-2: UE configuration data (on network)

| Reference | Parameter description |
| --- | --- |
| Subclause 5.2.3 of 3GPP TS 23.280 [6] | Relay service (Y/N) |
| Subclause 5.2.3 of 3GPP TS 23.280 [6] | List of allowed relayed MCVideo groups and their relay service code (as specified in 3GPP TS 23.303 [7]) (optional) (see NOTE) |
|  | > MCVideo group ID |
|  | > Relay service code (as specified in 3GPP TS 23.303 [7]) |
| NOTE: When the value of the parameter Relay service is N, this parameter and its child parameters are not needed. |  |

# A.3	MCVideo user profile configuration data

The general aspects of MC service user profile configuration data are specified in 3GPP TS 23.280 [6]. The MCVideo user profile configuration data is stored in the MCVideo user database. The MCVideo server obtains the MCVideo user profile configuration data from the MCVideo user database (MCVideo-2).

Tables A.3-1 and A.3-2 contain the MCVideo user profile configuration required to support the use of on-network MCVideo service. Tables A.3-1 and A.3-3 contain the MCVideo user profile configuration required to support the use of off-network MCVideo service. Data in table A.3-1 and table A.3-3 can be configured offline using the CSC-11 reference point.

Table A.3-1: MCVideo user profile configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo identity (MCVideo ID) | Y | Y | Y | Y |
| 3GPP TS 33.180 [14] | KMSUri for security domain of MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | Pre-selected MCVideo user profile indication (see NOTE 2) | Y | Y | Y | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo user profile index | Y | Y | Y | Y |
| Subclause 5.2.11 of 3GPP TS 23.280 [6] | MCVideo user profile name | Y | Y | Y | Y |
| [R-5.17-007], [R-6.13.4-002] of 3GPP TS 22.280 [2] | User profile status (enabled/disabled) |  | Y | Y | Y |
| [R-5.7-001] [R-6.9-003] of 3GPP TS 22.280 [2] | Authorised to create and delete aliases of an MCVideo user and its associated user profiles. |  |  | Y | Y |
| [R-5.7-002], [R-6.9-003] of 3GPP TS 22.280 [2] | Alphanumeric aliases of user | Y | Y | Y | Y |
| [R-5.1.1-005], [R-5.9-001] of 3GPP TS 22.280 [2] | Participant type of the user | Y | Y | Y | Y |
| [R-5.1.8-006], [R-5.3-002], [R-5.9-001], [R-5.16.2-001], [R-5.16.2-002] of 3GPP TS 22.280 [2] | User's Mission Critical Organization (i.e. which organization a user belongs to) | Y | Y | Y | Y |
| [R-5.2.2-003] of 3GPP TS 22.280 [2] | Authorisation to create a group-broadcast group |  |  | Y | Y |
| [R-5.2.2-003] of 3GPP TS 22.280 [2] | Authorisation to create a user-broadcast group |  |  | Y | Y |
| [R-5.6.2.4.1-002] of 3GPP TS 22.280 [2] | Authorised to activate MCVideo emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-013] of 3GPP TS 22.280 [17] | Automatically trigger a MCVideo emergency communication after initiating the MCVideo emergency alert | Y | Y | Y | Y |
| [R-5.6.2.4.1-004] [R-5.6.2.4.1-008] [R-5.6.2.4.1-012] of 3GPP TS 22.280 [2] | Group used on initiation of an MCVideo emergency group call (see NOTE 5) | Y | Y | Y | Y |
| [R-5.6.2.4.1-004], [R-5.6.2.4.1-008], [R-5.6.2.4.1-012] of 3GPP TS 22.280 [17] | Recipient for an emergency private MCVideo call (see NOTE 5) |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
| 3GPP TS 33.180 [19] | > KMSUri for security domain of MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
| [R-5.6.2.4.2-002] of 3GPP TS 22.280 [2] | Authorisation to cancel an MCVideo emergency alert | Y | Y | Y | Y |
| [R-5.1.2.1.2-004] of 3GPP TS 22.281 [3] | Authorisation to modify the video settings of the transmitted video stream of another MCVideo User | Y | Y | Y | Y |
| [R-5.1.2.1.2-006] of 3GPP TS 22.281 [3] | Authorisation to renegotiate a codec during a video transmission. | Y | Y | Y | Y |
| [R-5.1.2.1.2-007] of 3GPP TS 22.281 [3] | Authorisation to remotely control the video capabilities or parameters for a camera on an MCVideo UE | Y | Y | Y | Y |
| [R-5.1.2.2.2-001] of 3GPP TS 22.281 [3] | Authorisation to remotely control the video capabilities or parameters of a remote MCVideo UE | Y | Y | Y | Y |
| [R-5.1.2.2.2-004] of 3GPP TS 22.281 [3] | Authorisation to receive and display the capabilities of a remote MCVideo UE | Y | Y | Y | Y |
| [R-5.1.3.1.2-004] of 3GPP TS 22.281 [3] | Authorisation to remotely activate another MCVideo User's camera | Y | Y | Y | Y |
| [R-5.1.9.2.2-002] of 3GPP TS 22.281 [3] | Authorisation to push a video to another MCVideo user . | Y | Y | Y | Y |
| [R-5.1.9.2.2-003] of 3GPP TS 22.281 [3] | Authorisation to enable and to disable the automatic sending of notification to a second MCVideo User that a video is being pushed to a third MCVideo User | Y | Y | Y | Y |
| [R-5.1.9.2.2-004] of 3GPP TS 22.281 [3] | List of MCVideo users for whom to receive notifications about video being pushed to them | Y | Y | Y | Y |
|  | > MCVideo IDs |  |  |  |  |
| [R-5.1.9.2.2-005] of 3GPP TS 22.281 [3] | List of specific video categories to receive (see NOTE 3) | Y | Y | Y | Y |
|  | > Video categories |  |  |  |  |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | List of user(s) who can be called in MCVideo private call |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
|  | > User info ID | Y | N | Y | Y |
|  | > ProSe discovery group ID | Y | N | Y | Y |
|  | > Presentation priority relative to other users and groups (see NOTE 4) | Y | Y | Y | Y |
| 3GPP TS 33.180 | > KMSUri for security domain of MCVideo ID (see NOTE 1) | Y | Y | Y | Y |
| [R-6.7.3-007] of 3GPP TS 22.280 [2] | Authorised to make a private video call towards users not included in "list of user(s) who can be called in MCVideo private call" | Y | Y | Y | Y |
| [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of category tags |  |  |  |  |
|  | > Video category tag | Y | Y | Y | Y |
| [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | Authorization to query MCVideo client | Y | Y | Y | Y |
| [R-5.1.3.2.2-004] [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of category tags that authorized to query MCVideo client |  |  |  |  |
|  | > Video category tag for query | Y | Y | Y | Y |
| [R-5.1.3.2.2-004] [R-5.1.10.2-002] of 3GPP TS 22.281 [3] | List of geography areas that authorized to query MCVideo client |  |  |  |  |
|  | > Geography area to query | Y | Y | Y | Y |
| [R-5.1.1.1-015] of 3GPP TS 22.281 [3] | Authorization to perform video adaptation | Y | Y | Y | Y |
| [R-5.1.7-002] and [R-6.8.7.2-007] and [R-6.8.7.2-008] of 3GPP TS 22.280 [2] | Priority of the user (see NOTE 6) |  | Y | Y | Y |
| [R-6.12-003] of 3GPP TS 22.280 [17] | Authorised to restrict the dissemination of the location information | Y | Y | Y | Y |
| NOTE 1: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: As specified in 3GPP TS 23.280 [6], for each MCVideo user's set of MCVideo user profiles, only one MCVideo user profile shall be indicated as being the pre-selected MCVideo user profile. NOTE 3: If this list is blank then this implies that all video categories are acceptable for the MCVideo user. NOTE 4: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 5: This parameter is used for the emergency communication and also used as a target of the emergency alert request. At most one of them is configured; i.e. emergency communication will go to either a group or a user. If both are not configured the MCVideo user's currently selected group will be used. NOTE 6: The use of the parameter is left to implementation. |  |  |  |  |  |

Table A.3-2: MCVideo user profile configuration data (on network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| [R-5.1.5-001], [R-5.1.5-002], [R-5.10-001], [R-6.4.7-002], [R-6.8.1-008], [R-6.7.4-002] of 3GPP TS 22.280 [2] | List of on-network MCVideo groups for use by an MCVideo user |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | Y | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | Y | Y | Y |
| 3GPP TS 33.180 [14] | > KMSUri for security domain of group (see NOTE 4) | Y | Y | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | Y | Y | Y |
| Subclause 5.2.5 of 3GPP TS 23.280 [6] | List of groups user implicitly affiliates to after MCVideo service authorization for the user |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
| [R-6.4.2-006] of 3GPP TS 22.280 [2] | Authorisation of an MCVideo user to request a list of which MCVideo groups a user has affiliated to |  | Y | Y | Y |
| [R-6.4.6.1-002], [R-6.4.6.1-003] of 3GPP TS 22.280 [2] | Authorisation to change affiliated groups of other specified user(s) |  | Y | Y | Y |
| [R-6.4.6.2-001], [R-6.4.6.2-002] of 3GPP TS 22.280 [2] | Authorisation to recommend to specified user(s) to affiliate to specific group(s) |  | Y | Y | Y |
| [R-6.6.1-004] of 3GPP TS 22.280 [2] | Authorisation to perform regrouping | Y | Y | Y | Y |
| [R-6.7.2-001] of 3GPP TS 22.280 [2] | Presence status is available/not available to other users | Y | Y | Y | Y |
| [R-6.7.1-002], [R-6.7.2-002] of 3GPP TS 22.280 [2] | List of MCVideo users that MCVideo user is authorised to obtain presence of |  |  |  |  |
|  | > MCVideo IDs | Y | Y | Y | Y |
| [R-6.8.7.4.2-001], [R-6.8.7.4.2-002] of 3GPP TS 22.280 [2] | Authorisation of a user to cancel an emergency alert on any MCVideo UE of any user |  | Y | Y | Y |
| [R-6.13.4-001] of 3GPP TS 22.280 [2] | Authorisation for an MCVideo user to enable/disable an MCVideo user |  | Y | Y | Y |
| [R-6.13.4-003], [R-6.13.4-005], [R-6.13.4-006], [R-6.13.4-007] of 3GPP TS 22.280 [2] | Authorisation for an MCVideo user to (permanently /temporarily) enable/disable a UE |  | Y | Y | Y |
| [R-7.14-002], [R-7.14-003] of 3GPP TS 22.280 [2] | Authorization for manual switch to off-network while in on-network | Y | Y | Y | Y |
| [R-5.1.5-004] of 3GPP TS 22.280 [2] | Limitation of number of affiliations per user (Nc2) | N | Y | Y | Y |
| [R-6.4.6.1-001], [R-6.4.6.1-004] of 3GPP TS 22.280 [2] | List of MCVideo users whose selected groups are authorized to be remotely changed |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
| [R-5.2.3.2-002], [R-5.2.3.2-003] of 3GPP TS 22.281 [3] | Period after which MCVideo data on a MCVideo UE is to be deleted if no action is taken by an authorized MCVideo user | Y | Y | Y | Y |
| [R-5.2.6.2.2-004] of 3GPP TS 22.281 [3] | Maximum number of simultaneous video streams that can be received (see NOTE 3) | Y | Y | Y | Y |
| [R-5.2.6.2.2-005] of 3GPP TS 22.281 [3] | Authorisation to automatically receive video communications | Y | Y | Y | Y |
| [R-5.2.6.2.2-006] of 3GPP TS 22.281 [3] | Authorisation to automatically receive emergency video streams | Y | Y | Y | Y |
| [R-5.2.6.2.2-007] of 3GPP TS 22.281 [3] | Authorisation to automatically receive imminent peril video streams | Y | Y | Y | Y |
| [R-5.2.6.2.2-008] of 3GPP TS 22.281 [3] | List of MCVideo groups for which video can be automatically/mandatorily received |  |  |  |  |
|  | > MCVideo group IDs | Y | Y | Y | Y |
| [R-5.2.7.2-001] of 3GPP TS 22.281 [3] | Authorisation to request to override an active MCVideo transmission | Y | Y | Y | Y |
| [R-5.2.7.2-002] of 3GPP TS 22.281 [3] | Authorisation to select MCVideo transmissions that can be overridden | Y | Y | Y | Y |
| [R-5.2.7.2-004] of 3GPP TS 22.281 [3] | Authorisation to allow MCVideo private communications to override or not to override active MCVideo group communications | Y | Y | Y | Y |
| [R-5.2.8-005], [R-5.2.8-006] of 3GPP TS 22.281 [3] | Maximum length of time of a single video transmission | Y | Y | Y | Y |
| [R-6.7.3-007a] of 3GPP TS 22.280 [2] | List of user(s) from which MCVideo private calls can be received |  |  |  |  |
|  | > MCVideo ID | Y | Y | Y | Y |
|  | > Presentation priority relative to other users and groups | Y | Y | Y | Y |
| 3GPP TS 33.180 [18] | > KMSUri for security domain of MCVideo ID | Y | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [2] [R-5.9a-013] of 3GPP TS 22.280 [2] | Authorised to request association between active functional alias(es) and MCVideo ID(s) |  | Y | Y | Y |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | List of partner MCVideo systems in which this profile is valid for use during migration |  |  |  |  |
| Subclause 5.2.9 of 3GPP TS 23.280 [16] | > Identity of partner MCVideo system | Y | N | Y | Y |
| Subclause 10.1.1 of 3GPP TS 23.280 [16] | > Access information for partner MCVideo system (see NOTE 5) | Y | N | Y | Y |
| [R-6.6.4.2-002a] and [R-6.6.4.2-002b] of 3GPP TS 22.280 [2] | List of groups the client affiliates/de-affiliates when criteria is met |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
|  | >> Criteria for affiliation (see NOTE 7) | Y | Y | Y | Y |
|  | >> Criteria for de-affiliation (see NOTE 7) | Y | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y | Y | Y | Y |
| [R-6.6.4.2-002] of 3GPP TS 22.280 [2] | List of groups the client affiliates after receiving an emergency alert |  |  |  |  |
|  | > MCVideo Group ID | Y | Y | Y | Y |
|  | >> Manual de-affiliation is not allowed if the criteria for affiliation are met | Y | Y | Y | Y |
| [R-5.9a-012] of 3GPP TS 22.280 [2] | Authorised to take over a functional alias from another MCVideo user | N | Y | Y | Y |
|  | List of functional alias(es) of the MCVideo user |  |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | > Functional alias | Y | Y | Y | Y |
| [R-5.9a-018] of 3GPP TS 22.280 [2] | >> MCVideo server trigger criteria for activation (see NOTE 6) | N | Y | Y | Y |
| [R-5.9a-017], [R-5.9a-018] of 3GPP TS 22.280 [2] | >> MCVideo server trigger criteria for de-activation (see NOTE 6) | N | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [16] | >> MCVideo client trigger criteria for activation (see NOTE 6) | Y | Y | Y | Y |
| [R-5.9a-019] of 3GPP TS 22.280 [16] | >> MCVideo client trigger criteria for de-activation (see NOTE 6) | Y | Y | Y | Y |
|  | >> Manual de-activation is not allowed if the criteria are met (see NOTE 6) | Y | Y | Y | Y |
| [R-5.10-001b] of 3GPP TS 22.280 [16] | Maximum number of successful simultaneous MCVideo service authorizations for this user (see NOTE 8) | N | Y | Y | Y |
| [R-5.9a-020] of 3GPP TS 22.280 [17] | List of functional aliases to which private calls are allowed when using a certain functional alias |  |  |  |  |
|  | > Used functional alias | Y | Y | Y | Y |
|  | >> List of functional aliases which can be called |  |  |  |  |
|  | >>> Functional alias | Y | Y | Y | Y |
| [R-5.9a-021] of 3GPP TS 22.280 [17] | List of functional aliases from which private calls can be received when using a certain functional alias |  |  |  |  |
|  | > Used functional alias | N | Y | Y | Y |
|  | >> List of functional aliases from which calls can be received |  |  |  |  |
|  | >>> Functional alias | N | Y | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 3: The parameter can be set to an unlimited number of simultaneous streams received that can be received. NOTE 4: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 5: Access information for each partner MCVideo system comprises the list of information required for initial UE configuration to access an MCVideo system, as defined in table A.6-1 of 3GPP TS 23.280 [16] NOTE 6: The criteria may consist of conditions like the location of the MCVideo user, local time etc. NOTE 7: The criteria may consist of conditions such as the location of the MCVideo user or the active functional alias of the MCVideo user. NOTE 8: If configured, this value has precedence over the system level parameter "maximum number of successful simultaneous service authorisations" in table A.5-2. If not configured, the corresponding parameter from table A.5-2 shall be used. |  |  |  |  |  |

Table A.3-3: MCVideo user profile configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server | MCVideo user database |
| --- | --- | --- | --- | --- | --- |
| [R-7.2-003], [R-7.6-004] of 3GPP TS 22.280 [2] | List of off-network MCVideo groups for use by an MCVideo user |  |  |  |  |
|  | > MCVideo Group IDs | Y | N | Y | Y |
|  | > Application plane server identity information of group management server where group is defined |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
|  | > Application plane server identity information of identity management server which provides authorization for group (see NOTE 1) |  |  |  |  |
|  | >> Server URI | Y | N | Y | Y |
| 3GPP TS 33.180 [14] | > KMSUri for security domain of group (see NOTE 3) | Y | N | Y | Y |
|  | > Presentation priority of the group relative to other groups and users (see NOTE 2) | Y | N | Y | Y |
| [R-7.12-002], [R-7.12-003] of 3GPP TS 22.280 [2] | Authorization for off-network services | Y | N | Y | Y |
| Subclause 7.18.1 | User info id (as specified in 3GPP TS 23.303 [7]) | Y | N | Y | Y |
| NOTE 1: If this parameter is not configured, authorization to use the group shall be obtained from the identity management server identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. NOTE 2: The use of this parameter by the MCVideo UE is outside the scope of the present document. NOTE 3: If this parameter is absent, the KMSUri shall be that identified in the initial MC service UE configuration data (on-network) configured in table A.6-1 of 3GPP TS 23.280 [6]. |  |  |  |  |  |

# A.4	MCVideo related Group configuration data

The general aspects of group configuration are specified in 3GPP TS 23.280 [6].

Parameters specified in table A.4-1 are child parameters of the MCVideo configuration parameter specified in table A.4-1 in 3GPP TS 23.280 [6]. Parameters specified in table A.4-2 are child parameters of the MCVideo configuration parameter specified in table A.4-2 in 3GPP TS 23.280 [6]. Parameters specified in table A.4-3 are child parameters of the MCVideo configuration parameter specified in table A.4-3 in 3GPP TS 23.280 [6].

Table A.4-1: Group configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Media confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Transmission control confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | >> Group media protection security material (see NOTE) | Y | N | Y |
| [R-5.1.1.1-010] of 3GPP TS 22.281 [3] | >> List of allowed sets of video codecs and related information in order of preference |  |  |  |
| [R-5.1.1.1-010] of 3GPP TS 22.281 [3] | >>> Video codecs | Y | Y | Y |
| [R-5.1.1.1-010] of 3GPP TS 22.281 [3] | >>> List of allowed video resolution |  |  |  |
|  | >>>> Video resolutions | Y | Y | Y |
| [R-5.1.1.1-010] of 3GPP TS 22.281 [3] | >>> List of allowed video frame rates |  |  |  |
|  | >>>> Video frame rates | Y | Y | Y |
| [R-5.1.1.3.2-002] of 3GPP TS 22.281 [3] | >> Allowed video modes |  |  |  |
|  | >>> Urgent real time mode allowed | N | Y | Y |
|  | >>> Non-urgent real time mode allowed | N | Y | Y |
|  | >>> Non real time mode allowed | N | Y | Y |
| [R-5.1.1.3.2-002] of 3GPP TS 22.281 [3] | >> Active video mode (urgent real time mode, non urgent real time mode, non real time mode) | Y | Y | Y |
| [R-5.3.2.2-002] of 3GPP TS 22.281 [3] | >> Allowed maximum simultaneous MCVideo transmitting group members | N | Y | Y |
| NOTE: Security mechanisms are specified in 3GPP TS 33.180 [14]. |  |  |  |  |

Table A.4-2: Group configuration data (on network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-6.4.5-001], [R-6.4.5-003] of 3GPP TS 22.280 [2] | >> Authorisation of a user to request a list of affiliated members of a group | Y | Y | Y |
| [R-5.1.7-002], [R-6.2.2-001], [R-6.6.2.2-006], [R-6.8.7.2-003] of 3GPP TS 22.280 [2] | >> Priority of the group | N | Y | Y |

Table A.4-3: Group configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Group management server |
| --- | --- | --- | --- | --- |
| Subclause 7.7.2.1 | >> Indication of whether the UE shall use single arbitrator approach or self arbitration approach. | Y | N | Y |
| [R-5.3.2.2-003] of 3GPP TS 22.281 [3] | >> Allowed maximum number of simultaneous transmissions (N9) | Y | Y | Y |
| Subclause 7.1.3 | >> Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | >>> MCVideo group call signalling | Y | N | Y |
|  | >>> MCVideo group call media | Y | N | Y |
|  | >>> MCVideo emergency group call signalling | Y | N | Y |
|  | >>> MCVideo emergency group call media | Y | N | Y |
|  | >>> MCVideo imminent peril group call signalling | Y | N | Y |
|  | >>> MCVideo imminent peril group call media | Y | N | Y |

# A.5	MCVideo service configuration data

The general aspects of MC service configuration are specified in 3GPP TS 23.280 [6]. The MCVideo service configuration data is stored in the MCVideo server.

Tables A.5-1 and A.5-2 describe the configuration data required to support the use of on-network MCVideo service. Tables A.5-1 and A.5-3 describe the configuration data required to support the use of off-network MCVideo service. Data in table A.5-1 and table A.5-3 can be configured offline using the CSC-11 reference point.

Table A.5-1: MCVideo service configuration data (on and off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.2.2-001] of 3GPP TS 22.280 [2] | Levels of group hierarchy for group-broadcast groups (Bc1) | Y | Y | Y |
| [R-5.2.3-001] of 3GPP TS 22.280 [2] | Levels of user hierarchy for user-broadcast groups (Bc2) | Y | Y | Y |
| [R-5.7-002] of 3GPP TS 22.280 [2] | Minimum length (Nc3) of an alphanumeric identifier (i.e. alias) assigned by an MCVideo administrator. | Y | N | Y |

Table A.5-2: MCVideo service configuration data (on network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Protect confidentiality of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Protect integrity of signalling (see NOTE 1) | Y | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Use signalling protection between MCVideo servers (see NOTE 1) | N | Y | Y |
| [R-5.12-001] of 3GPP TS 22.280 [2] | Use transmission control protection between MCVideo servers (see NOTE 1) | N | Y | Y |
| [R-5.10-001a] of 3GPP TS 22.280 [2] | Maximum number of successful simultaneous service authorizations of clients from a user | N | Y | Y |
|  | List of functional alias identities |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | > Functional alias | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >> Limit number of simultaneous activations | N | Y | Y |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >> This functional alias can be taken over | N | Y | Y |
|  | >> List of users who can activate this functional alias |  |  |  |
| [R-5.9a-005] of 3GPP TS 22.280 [2] | >>> MCVideo ID | N | Y | Y |
| [R-5.9a-016] of 3GPP TS 22.280 [2] | >> Communication priority (see NOTE 2) | N | Y | Y |
| Subclause 5.15 of 3GPP TS 22.280 [2] | List of permitted GW MC service ID(s) |  |  |  |
|  | > GW MC service ID | Y | Y | Y |
| NOTE 1: Security mechanisms are specified in 3GPP TS 33.180 [14]. NOTE 2: The usage of this parameter by the MCVideo server is up to implementation. |  |  |  |  |

Table A.5-3: MCVideo service configuration data (off network)

| Reference | Parameter description | MCVideo UE | MCVideo Server | Configuration management server |
| --- | --- | --- | --- | --- |
| [R-7.6-001], [R-7.6-002], [R-7.6-003] of 3GPP TS 22.280 [2] | Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | > MCVideo private call signalling | Y | N | Y |
|  | > MCVideo private call media | Y | N | Y |
|  | > MCVideo Emergency private call signalling | Y | N | Y |
|  | > MCVideo Emergency private call media | Y | N | Y |
| Subclause 7.5.3.3 | Periodicity interval (in seconds) of Capability announcement (see NOTE) | Y | N | Y |
| NOTE: If the value of the periodicity interval is set to zero, the capability announcement is not sent. |  |  |  |  |

Annex B (informative):
Change history

| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2016-04 |  |  |  |  |  | Initial version. | 0.0.0 |
| 2016-06 |  |  |  |  |  | Implemented the following pCRs approved in SA6#11: S6-160535, S6-160536, S6-160589 Title aligned with the title in MCVideo WID. | 0.1.0 |
| 2016-08 |  |  |  |  |  | Implemented the following pCRs approved in SA6#12: S6-160800, S6-160805,S6-160881, S6-160882, S6-160931 | 0.2.0 |
| 2016-10 |  |  |  |  |  | Implemented the following pCRs approved in SA6#13: S6-161155, S6-161166, S6-161226, S6-161227, S6-161233, S6-161234, S6-161235, S6-161236, S6-161237, S6-161239, S6-161240, S6-161241, S6-161264 | 0.3.0 |
| 2016-11 |  |  |  |  |  | Implemented the following pCRs approved in SA6#14: S6-161461, S6-161464, S6-161465, S6-161468, S6-161469, S6-161473, S6-161474, S6-161476, S6-161479, S6-161481, S6-161482, S6-161483, S6-161485, S6-161497, S6-161566, S6-161567, S6-161568, S6-161569, S6-161570, S6-161574, S6-161575, S6-161585, S6-161588, S6-161599, S6-161600, S6-161601, S6-161602, S6-161603, S6-161608 | 0.4.0 |
| 2016-11 | SA#74 | SP-160877 |  |  |  | Submitted for Approval at SA#74 | 1.0.0 |
| 2016-12 | SA#74 | SP-160877 |  |  |  | MCC Editorial update for publication after TSG SA approval (SA#74) | 14.0.0 |
| 2017-03 | SA#75 | SP-170070 | 0001 | 1 | F | Alignment of definitions | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0002 | 2 | F | Alignment of group affiliation and de-affiliation requirements | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0003 | 1 | F | Alignment of bearer management | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0004 | 2 | F | Adding descriptive text for several general sections | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0005 | 1 | F | MCVideo call connect and disconnect on MBMS | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0007 |  | F | Alignment of business relationships, identities, and application of functional model to deployments | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0008 | 1 | F | Functional model update | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0009 | 1 | F | Location related description alignment | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0010 | 1 | F | Reception control at receiving participant | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0011 | 1 | F | Feedback between reception control and transmission control | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0013 | 1 | F | Activity status sharing | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0014 | 1 | F | Corrections to video push | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0017 |  | F | Obsolete editor's notes | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0018 |  | F | Response message | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0020 | 1 | F | Clarification on Editor's notes in subclause 7.3.3.2 in MCVideo | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0021 | 1 | F | Clarification on Editor's notes in MCVideo | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0022 | 1 | F | Clarification on transmission control | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0023 |  | D | Editorial corrections in MCVideo | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0024 | 2 | F | MCVideo configurations | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0027 | 1 | F | Addition of definition for MCVideo ID | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0028 | 1 | F | Correction of requirement reference for list of implicitly affiliated groups in MCVideo user profile config | 14.1.0 |
| 2017-03 | SA#75 | SP-170070 | 0029 | 3 | F | Off-network transmission arbitration | 14.1.0 |
| 2017-06 | SA#76 | SP-170391 | 0042 | 1 | F | Miscellaneous corrections to configuration | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0043 | 5 | F | Addition of identity management server address per group | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0044 |  | F | Clarifications on off-network group call answer response | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0045 | 1 | F | Clarifications on off-network group call procedure descriptions | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0046 | 1 | F | Clarifications on pre-conditions of group communication in off-network | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0048 |  | F | Remove Editor's Note on broadcast group communication in 23281 | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0049 |  | F | Remove Editor's Note on Video details in Video pull and push in 23281 | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0050 | 2 | F | Remove Editor's Note on Device ID used for Capabilities info request in 23281 | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0051 | 1 | F | Remove Editor's Note on Clarifications for periodic capability announcements in 23281 | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0052 | 1 | F | Some clarifications on off-network group call | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0060 | 1 | F | Editorial correction in chapter Release chat group call of MCVideo | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0061 | 2 | F | Inclusion of KMSUri to allow multiple security domains | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0062 |  | F | Addition of ProSe Per Packet Priority to group configuration | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0063 | 1 | F | Configuration for emergency alerts | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0066 | 1 | F | Video modes usage | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0067 | 2 | F | Communications remote control | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0069 | 3 | F | Alignment of MCVideo user profile configurations | 14.2.0 |
| 2017-06 | SA#76 | SP-170391 | 0071 |  | F | Corrections to Group configuration data for all MC services | 14.2.0 |
| 2017-06 | SA#76 | SP-170395 | 0070 | 1 | C | Resource Management in MCVideo | 15.0.0 |
| 2017-09 | SA#77 | SP-170680 | 0072 | 1 | A | CR on MCVideo capabilities information retrival and remote transmit media end | 15.1.0 |
| 2017-09 | SA#77 | SP-170680 | 0075 | 2 | A | MCVideo call connect | 15.1.0 |
| 2017-09 | SA#77 | SP-170683 | 0077 | 3 | B | Camera query | 15.1.0 |
| 2017-09 | SA#77 | SP-170683 | 0078 | 4 | B | Video capabilities sharing enhancement | 15.1.0 |
| 2017-09 | SA#77 | SP-170683 | 0079 | 1 | B | MCVideo emergency private call | 15.1.0 |
| 2018-01 | SA#78 | SP-170895 | 0080 | 3 | B | Functional model for MCVideo interconnection | 15.2.0 |
| 2018-01 | SA#78 | SP-170888 | 0083 | 1 | A | CR on clarification for MC service group ID | 15.2.0 |
| 2018-01 | SA#78 | SP-170892 | 0084 | 4 | B | Mutual adaption during MCVideo communication | 15.2.0 |
| 2018-01 | SA#78 | SP-170892 | 0085 | 3 | B | Renaming of MCVideo emergency group call cancel and correction to clause 7.1.2.5.1.3 | 15.2.0 |
| 2018-01 | SA#78 | SP-170892 | 0086 | 1 | F | Adding implicit transmit media request IE to MCVideo emergency group call request | 15.2.0 |
| 2018-01 | SA#78 | SP-170892 | 0087 | 1 | F | Renaming of MCVideo "emergency condition" | 15.2.0 |
| 2018-01 | SA#78 | SP-170888 | 0091 | 2 | A | Rejection of group call request | 15.2.0 |
| 2018-01 | SA#78 | SP-170888 | 0093 | 2 | A | MCVideo server PSI configuration | 15.2.0 |
| 2018-01 | SA#78 | SP-170892 | 0095 | 1 | D | Correction of wrong "MCPTT" words | 15.2.0 |
| 2018-01 | SA#78 | SP-170892 | 0097 | 1 | F | Renaming of "MCVideo emergency state cancel" and other corrections | 15.2.0 |
| 2018-04 | SA#79 | SP-180147 | 0099 | 1 | A | Correction of security specification references | 15.3.0 |
| 2018-04 | SA#79 | SP-180154 | 0100 | 2 | B | User profile for MCVideo migration | 15.3.0 |
| 2018-04 | SA#79 | SP-180152 | 0101 | 1 | F | Update of references to stage 1 specifications | 15.3.0 |
| 2018-04 | SA#79 | SP-180154 | 0103 |  | B | Definition of MCVideo-3 | 15.3.0 |
| 2018-06 | SA#80 | SP-180366 | 0105 |  | A | Clarification for presentation priority in MCVideo UE configuration | 15.4.0 |
| 2018-12 | SA#82 | SP-181170 | 0111 | 2 | A | MCVideo QCI update | 15.5.0 |
| 2018-12 | SA#82 | SP-181180 | 0109 |  | D | Spelling of ProSe | 16.0.0 |
| 2019-03 | SA#83 | SP-190070 | 0114 | 1 | A | Corrections to MCVideo Group join information flows and procedure | 16.1.0 |
| 2019-03 | SA#83 | SP-190073 | 0116 | 1 | F | Corrections to 'general' subclause for pre-arranged and chat group calls | 16.1.0 |
| 2019-03 | SA#83 | SP-190073 | 0117 | 1 | B | Late entry for MCVideo chat group call | 16.1.0 |
| 2019-03 | SA#83 | SP-190073 | 0118 | 1 | B | Procedure for MCVideo user leaving a group call | 16.1.0 |
| 2019-06 | SA#84 | SP-190488 | 0119 | 1 | B | Restricting incoming private communications | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0120 | 2 | B | User configuration for functional alias information query for MCVideo | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0121 | 1 | B | User requested priority in 23.281 | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0122 | 2 | B | MCVideo configuration for functional alias | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0123 | 3 | B | Criteria based automatic group affiliation and deaffiliation | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0124 | 1 | B | Add configuration data to support functional alias in MCVideo | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0125 | 3 | B | Add functional alias to MCVideo for video pull and push operations | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0126 | 3 | B | Add functional alias to MCVideo private calls | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0127 | 4 | B | Add functional alias to MCVideo group calls | 16.2.0 |
| 2019-06 | SA#84 | SP-190484 | 0129 | 2 | F | Remove the duplicated MCVideo server URI in UE configuration table | 16.2.0 |
| 2019-06 | SA#84 | SP-190489 | 0130 | 2 | F | Fix the missing parameters in the user profile configuration table | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0131 |  | B | MCVideo server limits the number of simultaneous successful service authorisations | 16.2.0 |
| 2019-06 | SA#84 | SP-190488 | 0132 |  | B | MCVideo client performs automatic activation and deactivation of functional aliases based on location | 16.2.0 |
| 2019-09 | SA#85 | SP-190731 | 0133 | 3 | F | Correct the configuration parameters for the MCVideo emergency alert procedures | 16.3.0 |
| 2019-09 | SA#85 | SP-190732 | 0136 | 1 | F | Fixing the user profile configuration data for criteria triggered functional alias activation and de-activation | 16.3.0 |
| 2019-09 | SA#85 | SP-190735 | 0134 | 1 | B | Communication priority for functional aliases | 17.0.0 |
| 2019-12 | SA#86 | SP-191113 | 0137 | 2 | B | Priority of the user | 17.1.0 |
| 2019-12 | SA#86 | SP-191114 | 0138 | 1 | F | Alignment on MCX emergency alert procedures | 17.1.0 |
| 2020-03 | SA#87-E | SP-200118 | 0139 | 3 | B | Enhancing MCVideo communication requests with application priority capabilities in on-network mode | 17.2.0 |
| 2020-03 | SA#87-E | SP-200118 | 0140 | 2 | F | Alignment of functional alias to group binding handling in MCVideo | 17.2.0 |
| 2020-03 | SA#87-E | SP-200118 | 0143 | 1 | F | Adding missing server to server information flows for group calls | 17.2.0 |
| 2020-03 | SA#87-E | SP-200118 | 0144 | 1 | F | Adding missing server to server information flows for private calls | 17.2.0 |
| 2020-07 | SA#88-E | SP-200340 | 0142 | 6 | B | Introducing a functional alias as target address for private video calls | 17.3.0 |
| 2020-09 | SA#89-E | SP-200843 | 0145 | 1 | F | Correction of private call setup procedures when using a functional alias as target address | 17.4.0 |
| 2020-09 | SA#89-E | SP-200846 | 0146 | 1 | F | Add the IE of priority in Information flows for transmission control | 17.4.0 |
| 2020-09 | SA#89-E | SP-200843 | 0147 | 1 | B | Limit the number of simultaneous logins on per user basis | 17.4.0 |
| 2020-09 | SA#89-E | SP-200846 | 0148 | 1 | F | MCVideo emergency group call clarifications | 17.4.0 |
| 2020-09 | SA#89-E | SP-200846 | 0149 | 1 | F | MCVideo emergency private call clarifications | 17.4.0 |
| 2020-09 | SA#89-E | SP-200846 | 0150 | 1 | F | Clarifications on the use of ProSe in off-network MCVideo communications | 17.4.0 |
| 2020-12 | SA#90-E | SP-201000 | 0152 |  | F | MCVideo pull and push corrections | 17.5.0 |
| 2020-12 | SA#90-E | SP-200994 | 0153 |  | C | Call restrictions for normal private calls | 17.5.0 |
| 2020-12 | SA#90-E | SP-200990 | 0155 |  | A | FA controlling role description in MCVideo | 17.5.0 |
| 2020-12 | SA#90-E | SP-200990 | 0157 |  | A | FA management procedure in MCVideo | 17.5.0 |
| 2021-06 | SA#92-E | SP-210567 | 0158 |  | F | Correction to MCVideo-5 reference point description | 17.6.0 |
| 2021-12 | SA#94-E | SP-211529 | 0159 | 1 | B | Connection authorisation configuration data | 18.0.0 |
| 2022-06 | SA#96 | SP-220476 | 0162 | 1 | B | Allow the user to restrict the dissemination of the location information – MCVideo Configuration | 18.1.0 |
| 2022-09 | SA#97-E | SP-220924 | 0163 |  | F | Private MCVideo call to a FA user | 18.2.0 |
| 2022-09 | SA#97-E | SP-220926 | 0164 |  | C | Updating MCVideo private call end request information elements | 18.2.0 |
| 2022-09 | SA#97-E | SP-220924 | 0165 | 1 | F | Auto-receive transmission mode for emergency call, imminent peril calls etc over unicast session | 18.2.0 |