| 3GPP TS 23.434 V18.2.0 (2022-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Architectural requirements	8

4.1	General	8

4.1.1	Description	8

4.1.2	Requirements	8

4.2	Deployment models	8

4.2.1	Description	8

4.2.2	Requirements	8

4.3	Location management	9

4.3.1	Description	9

4.3.2	Requirements	9

4.4	Group management	9

4.4.1	Description	9

4.4.2	Requirements	9

4.5	Configuration management	9

4.5.1	Description	9

4.5.2	Requirements	10

4.6	Key management	10

4.6.1	Description	10

4.6.2	Requirements	10

4.7	Identity management	10

4.7.1	Description	10

4.7.2	Requirements	10

4.8	Network resource management	10

4.8.1	Description	10

4.8.2	Requirements	10

5	Involved business relationships	11

6	Generic functional model for SEAL services	12

6.1	General	12

6.2	On-network functional model description	12

6.3	Off-network functional model description	15

6.4	Functional entities description	15

6.4.1	General	15

6.4.2	Application plane	15

6.4.2.1	General	15

6.4.2.2	VAL client	15

6.4.2.3	VAL server	16

6.4.2.4	SEAL client	16

6.4.2.5	SEAL server	16

6.4.2.6	VAL user database	16

6.4.3	Signalling control plane	16

6.4.3.1	SIP entities	16

6.4.3.1.1	Signalling user agent	16

6.4.3.1.2	SIP AS	16

6.4.3.1.3	SIP core	16

6.4.3.1.3.1	General	16

6.4.3.1.3.2	Local inbound / outbound proxy	17

6.4.3.1.3.3	Registrar finder	17

6.4.3.1.3.4	Registrar / application service selection	17

6.4.3.1.4	Diameter proxy	18

6.4.3.2	SIP database	18

6.4.3.2.1	General	18

6.4.3.2.2	SIP database logical functions	19

6.4.3.3	HTTP entities	19

6.4.3.3.1	HTTP client	19

6.4.3.3.2	HTTP proxy	19

6.4.3.3.3	HTTP server	20

6.4.3.4	LWP entities	20

6.4.3.4.1	LWP client	20

6.4.3.4.2	LWP proxy	20

6.4.3.4.3	LWP server	20

6.4.3.5	LWP usage	20

6.5	Reference points description	21

6.5.1	General reference point principle	21

6.5.2	Application plane	21

6.5.2.1	General	21

6.5.2.2	VAL-UU	21

6.5.2.3	VAL-PC5	21

6.5.2.4	SEAL-UU	21

6.5.2.5	SEAL-PC5	21

6.5.2.6	SEAL-C	21

6.5.2.7	SEAL-S	21

6.5.2.8	SEAL-E	22

6.5.2.9	SEAL-X	22

6.5.2.9.1	General	22

6.5.2.9.2	Reference point SEAL-X1 (between the key management server and the group management server)	22

6.5.2.9.3	Reference point SEAL-X2 (between the group management server and the location management server)	22

6.5.2.9.4	Reference point SEAL-X3 (between the group management server and the configuration management server)	22

6.5.2.10	Reference point VAL-UDB (between the VAL user database and the SEAL server)	22

6.5.3	Signalling control plane	22

6.5.3.1	General	22

6.5.3.2	Reference point SIP-1(between the signalling user agent and the SIP core)	23

6.5.3.3	Reference point SIP-2 (between the SIP core and the SIP AS)	23

6.5.3.4	Reference point SIP-3 (between the SIP core and SIP core)	23

6.5.3.5	Reference point HTTP-1 (between the HTTP client and the HTTP proxy)	23

6.5.3.6	Reference point HTTP-2 (between the HTTP proxy and the HTTP server)	24

6.5.3.7	Reference point HTTP-3 (between the HTTP proxy and HTTP proxy)	24

6.5.3.8	Reference point AAA-1 (between the SIP database and the SIP core)	24

6.5.3.9	Reference point AAA-2 (between the SIP core and Diameter proxy)	24

6.5.3.10	Reference point LWP-1 (between the LWP client and the LWP proxy)	24

6.5.3.11	Reference point LWP-2 (between the LWP proxy and the LWP server)	24

6.5.3.12	Reference point LWP-3 (between the LWP proxy and LWP proxy)	24

6.5.3.13	Reference point LWP-HTTP-2 (between the LWP proxy and the HTTP server)	24

6.5.3.14	Reference point LWP-HTTP-3 (between the LWP proxy and the HTTP proxy)	24

7	Identities	25

7.1	User identity (User ID)	25

7.2	VAL user identity (VAL user ID)	25

7.3	VAL UE identity (VAL UE ID)	25

7.4	VAL service identity (VAL service ID)	25

7.5	VAL group identity (VAL group ID)	25

7.6	VAL system identity (VAL system ID)	25

7.7	VAL Stream ID	26

8	Application of functional model to deployments	26

8.1	General	26

8.2	Deployment of SEAL server(s)	26

8.2.1	SEAL server(s) deployment in PLMN operator domain	26

8.2.2	SEAL server(s) deployment in VAL service provider domain	28

8.2.3	SEAL server(s) deployment outside of VAL service provider domain and PLMN operator domain	29

9	Location management	30

9.1	General	30

9.2	Functional model for location management	30

9.2.1	General	30

9.2.2	On-network functional model description	30

9.2.3	Off-network functional model description	31

9.2.4	Functional entities description	31

9.2.4.1	General	31

9.2.4.2	Location management client	32

9.2.4.3	Location management server	32

9.2.5	Reference points description	32

9.2.5.1	General	32

9.2.5.2	LM-UU	32

9.2.5.3	LM-PC5	32

9.2.5.4	LM-C	32

9.2.5.5	LM-S	32

9.2.5.6	LM-E	32

9.2.5.7	T8	33

9.3	Procedures and information flows for Location management (on-network)	33

9.3.1	General	33

9.3.2	Information flows for location information	33

9.3.2.0	Location reporting configuration request	33

9.3.2.1	Location reporting configuration response	33

9.3.2.2	Location information report	34

9.3.2.3	Location information request	34

9.3.2.4	Location reporting trigger	34

9.3.2.5	Location information subscription request	35

9.3.2.6	Location information subscription response	35

9.3.2.7	Location information notification	35

9.3.2.8	Location reporting configuration cancel request	36

9.3.2.9	Get UE(s) information request	36

9.3.2.10	Get UE(s) information response	36

9.3.2.11	Monitor Location Subscription Request	37

9.3.2.12	Monitor Location Subscription Response	37

9.3.2.13	Notify Location Monitoring Event	37

9.3.2.14	Location area monitoring subscription request	38

9.3.2.15	Location area monitoring subscription response	38

9.3.2.16	Location area monitoring notification	38

9.3.2.17	Location area monitoring subscription modify request	39

9.3.2.18	Location area monitoring subscription modify response	39

9.3.2.19	Location area monitoring unsubscribe request	39

9.3.2.20	Location area monitoring unsubscribe response	40

9.3.3	Event-triggered location reporting procedure	40

9.3.3.1	General	40

9.3.3.2	Fetching location reporting configuration	40

9.3.3.3	Location reporting	41

9.3.4	On-demand location reporting procedure	41

9.3.5	Client-triggered or VAL server-triggered location reporting procedure	42

9.3.6	Location reporting triggers configuration cancel	43

9.3.7	Location information subscription procedure	44

9.3.8	Event-trigger location information notification procedure	45

9.3.9	On-demand usage of location information procedure	45

9.3.10	Obtaining UE(s) information at a location	46

9.3.11	Monitoring Location Deviation	47

9.3.11.1	General	47

9.3.11.2	Monitoring Location Deviation procedure	47

9.3.12	Location area monitoring information procedure	48

9.3.12.1	Location area monitoring subscribe procedure	48

9.3.12.2	Location area monitoring subscribe modify procedure	49

9.3.12.3	Location area monitoring unsubscribe procedure	49

9.3.12.4	Location area monitoring notification procedure	50

9.4	SEAL APIs for location management	50

9.4.1	General	50

9.4.2	SS_LocationReporting API	51

9.4.2.1	General	51

9.4.2.2	Create_Trigger_Location_Reporting operation	51

9.4.2.3	Update_Trigger_Location_Reporting operation	51

9.4.2.4	Cancel_Trigger_Location_Reporting operation	51

9.4.3	SS_LocationInfoEvent API	52

9.4.3.1	General	52

9.4.3.2	Subscribe_Location_Info operation	52

9.4.3.3	Notify_Location_Info operation	52

9.4.4	SS_LocationInfoRetrieval API	52

9.4.4.1	General	52

9.4.4.2	Obtain_Location_Info operation	52

9.4.5	SS_LocationAreaInfoRetrieval API	53

9.4.5.1	General	53

9.4.5.2	Obtain_UEs_Info operation	53

9.4.6	SS_LocationMonitoring API	53

9.4.6.1	General	53

9.4.6.2	Subscribe_Location_Monitoring	53

9.4.6.3	Notify_Location_Monitoring_Events	53

9.4.7	SS_LocationAreaMonitoring API	54

9.4.7.1	General	54

9.4.7.2	Subscribe_Location_Area_Monitoring	54

9.4.7.3	Notify_Location_Area_Monitoring_Events	54

9.4.7.4	Update_Location_Area_Monitoring_Subscribe	54

9.4.7.5	Unsubscribe_Location_Area_Monitoring	54

9.5	Procedures and information flows for Location management (Off-network)	55

9.5.1	General	55

9.5.2	Information flows for off network location management	55

9.5.2.1	Off-network location reporting trigger configuration	55

9.5.2.2	Off-network location reporting trigger configuration response	55

9.5.2.3	Off-network location management ack	55

9.5.2.4	Off-network location report	56

9.5.2.5	Off-network location reporting trigger cancel	56

9.5.2.6	Off-network location reporting trigger cancel response	56

9.5.2.7	Off-network location request	56

9.5.2.8	Off-network location response	56

9.5.3	Event-triggered location reporting procedure	57

9.5.3.1	Location reporting trigger configuration	57

9.5.3.2	Location reporting	57

9.5.3.3	Location reporting trigger cancel	58

9.5.4	On-demand location reporting procedure	59

10	Group management	60

10.1	General	60

10.2	Functional model for group management	60

10.2.1	General	60

10.2.2	On-network functional model description	60

10.2.3	Off-network functional model description	60

10.2.4	Functional entities description	61

10.2.4.1	General	61

10.2.4.2	Group management client	61

10.2.4.3	Group management server	61

10.2.5	Reference points description	61

10.2.5.1	General	61

10.2.5.2	GM-UU	62

10.2.5.3	GM-PC5	62

10.2.5.4	GM-C	62

10.2.5.5	GM-S	62

10.2.5.6	GM-E	62

10.2.5.7	N33	62

10.3	Procedures and information flows for group management	62

10.3.1	General	62

10.3.2	Information flows for group management	63

10.3.2.1	Group creation request	63

10.3.2.2	Group creation response	63

10.3.2.3	Group creation notification	63

10.3.2.4	Group information query request	64

10.3.2.5	Group information query response	64

10.3.2.6	Group membership update request	64

10.3.2.7	Group membership update response	65

10.3.2.8	Group membership notification	65

10.3.2.9	Group deletion request	65

10.3.2.10	Group deletion response	66

10.3.2.11	Group deletion notification	66

10.3.2.12	Group information request	66

10.3.2.13	Group information response	66

10.3.2.14	Group information subscribe request	67

10.3.2.15	Group information subscribe response	67

10.3.2.16	Group information notify request	67

10.3.2.17	Group information notify response	67

10.3.2.18	Store group configuration request	68

10.3.2.19	Store group configuration response	68

10.3.2.20	Get group configuration request	68

10.3.2.21	Get group configuration response	68

10.3.2.22	Subscribe group configuration request	69

10.3.2.23	Subscribe group configuration response	69

10.3.2.24	Notify group configuration request	69

10.3.2.25	Notify group configuration response	70

10.3.2.26	Configure VAL group request	70

10.3.2.27	Configure VAL group response	70

10.3.2.28	Group announcement	70

10.3.2.29	Group registration request	71

10.3.2.30	Group registration response	72

10.3.2.31	Identity list notification	72

10.3.2.32	Group de-registration request	72

10.3.2.33	Group de-registration response	72

10.3.2.34	Location-based group creation request	73

10.3.2.35	Location-based group creation response	73

10.3.2.36	Group list fetch request	73

10.3.2.37	Group list fetch response	73

10.3.2.38	Temporary group formation request	74

10.3.2.39	Temporary group formation response	74

10.3.2.40	Temporary group formation notify	74

10.3.2.41	Temporary group formation notification	74

10.3.2.42	Temporary group formation notification response	75

10.3.3	Group creation	75

10.3.4	Group information query	76

10.3.4.1	General	76

10.3.4.2	Procedure	76

10.3.5	Group membership	76

10.3.5.1	Group membership notification	76

10.3.5.2	Group membership update by authorized user/UE/VAL server	77

10.3.6	Group configuration management	78

10.3.6.1	Store group configurations at the group management server	78

10.3.6.2	Retrieve group configurations	79

10.3.6.3	Subscription and notification for group configuration data	80

10.3.6.4	Structure of group configuration data	81

10.3.7	Location-based group creation	81

10.3.8	Group announcement and join	82

10.3.8.1	General	82

10.3.8.2	Procedure	82

10.3.9	Group member leave	84

10.3.9.1	General	84

10.3.9.2	Procedure	84

10.3.10	Temporary groups	85

10.3.10.1	Temporary group formation within a VAL system	85

10.3.11	Group List Fetch	86

10.3.12	Location-based group update	87

10.3.13	Group deletion	87

10.4	SEAL APIs for group management	88

10.4.1	General	88

10.4.2	SS_GroupManagement API	89

10.4.2.1	General	89

10.4.2.2	Query_Group_Info operation	89

10.4.2.3	Update_Group_Info operation	89

10.4.2.4	Create_LocationBasedGroup_Info operation	89

10.4.3	Void	90

10.4.3.1	Void	90

10.4.3.2	Void	90

10.4.4	Void	90

10.4.4.1	Void	90

10.4.4.2	Void	90

10.4.5	SS_Group_Management_Event API	90

10.4.5.1	General	90

10.4.5.2	Subscribe_ Group_Info_Modification operation	90

10.4.5.3	Notify_Group_Info_Modification operation	90

10.4.5.4	Notify_Group_Creation operation	90

10.4.5.5	Notify_TempGroupFormation operation	91

11	Configuration management	91

11.1	General	91

11.2	Functional model for configuration management	91

11.2.1	General	91

11.2.2	On-network functional model description	91

11.2.3	Off-network functional model description	92

11.2.4	Functional entities description	92

11.2.4.1	General	92

11.2.4.2	Configuration management client	92

11.2.4.3	Configuration management server	92

11.2.5	Reference points description	93

11.2.5.1	General	93

11.2.5.2	CM-UU	93

11.2.5.3	CM-PC5	93

11.2.5.4	CM-C	93

11.2.5.5	CM-S	93

11.2.5.6	CM-E	93

11.2.5.7	Reference point CM-VAL-UDB (between the configuration management server and the VAL user database)	93

11.3	Procedures and information flows for configuration management	94

11.3.1	General	94

11.3.2	Information flows	94

11.3.2.1	Get VAL UE configuration request	94

11.3.2.2	Get VAL UE configuration response	94

11.3.2.3	Get VAL user profile request	94

11.3.2.4	Get VAL user profile response	95

11.3.2.5	Notification for VAL user profile data update	95

11.3.2.6	Get updated VAL user profile data request	95

11.3.2.7	Get updated VAL user profile data response	95

11.3.2.8	Update VAL user profile data request	96

11.3.2.9	Update VAL user profile data response	96

11.3.2.10	Updated user profile subscription request	96

11.3.2.11	Updated user profile subscription response	96

11.3.2.12	Updated user profile notification	97

11.3.2.13	Get VAL service request	97

11.3.2.14	Get VAL service response	97

11.3.3	VAL UE configuration data	98

11.3.3.1	General	98

11.3.3.2	Procedures	98

11.3.3.3	Structure of VAL UE configuration data	98

11.3.4	VAL user profile data	98

11.3.4.1	General	98

11.3.4.2	Obtaining the VAL user profile(s) from the network	98

11.3.4.2.1	Obtaining the VAL user profile(s) in primary VAL system	98

11.3.4.2.2	VAL user receiving VAL service from a partner VAL system	99

11.3.4.3	VAL user receives updated VAL user profile data from the network	101

11.3.4.4	VAL user updates VAL user profile data to the network	101

11.3.4.5	Updated user profile subscription procedure	102

11.3.5	VAL service data	103

11.3.5.1	General	103

11.3.5.2	Procedures	103

11.4	SEAL APIs for configuration management	104

11.4.1	General	104

11.4.2	SS_UserProfileRetrieval API	104

11.4.2.1	General	104

11.4.2.2	Obtain_User_Profile operation	104

11.4.3	SS_UserProfileEvent API	104

11.4.3.1	General	104

11.4.3.2	Subscribe_User_Profile_Update operation	104

11.4.3.3	Notify_User_Profile_Update operation	105

11.4.4	SS_VALServiceData API	105

11.4.4.1	General	105

11.4.4.2	Obtain_VAL_Service_Data operation	105

12	Identity management	105

12.1	General	105

12.2	Functional model for identity management	105

12.2.1	General	105

12.2.2	On-network functional model description	106

12.2.3	Off-network functional model description	106

12.2.4	Functional entities description	106

12.2.4.1	General	106

12.2.4.2	Identity management client	107

12.2.4.3	Identity management server	107

12.2.5	Reference points description	107

12.2.5.1	General	107

12.2.5.2	IM-UU	107

12.2.5.3	IM-PC5	107

12.2.5.4	IM-C	107

12.2.5.5	IM-S	107

12.2.5.6	IM-E	107

12.3	Procedures and information flows for identity management	107

12.3.1	General	107

12.3.2	Information flows	108

12.3.3	General user authentication and authorization for VAL services	108

12.3.3.1	General	108

12.3.3.2	Primary VAL system	108

12.3.3.3	Interconnection partner VAL system	109

12.4	SEAL APIs for identity management	109

12.4.1	General	109

12.4.2	Void	109

12.4.2.1	Void	109

12.4.2.2	Void	109

13	Key management	109

13.1	General	109

13.2	Functional model for key management	109

13.2.1	General	109

13.2.2	On-network functional model description	109

13.2.3	Off-network functional model description	110

13.2.4	Functional entities description	110

13.2.4.1	General	110

13.2.4.2	Key management client	110

13.2.4.3	Key management server	111

13.2.5	Reference points description	111

13.2.5.1	General	111

13.2.5.2	KM-UU	111

13.2.5.3	KM-PC5	111

13.2.5.4	KM-C	111

13.2.5.5	KM-S	111

13.2.5.6	KM-E	111

13.2.5.7	SEAL-X1	112

13.3	Procedures and information flows for key management	112

13.4	SEAL APIs for key management	112

13.4.1	General	112

13.4.2	Void	112

13.4.2.1	Void	112

13.4.2.2	Void	112

14	Network resource management	112

14.1	General	112

14.2	Functional model for network resource management	112

14.2.1	General	112

14.2.2	On-network functional model description	112

14.2.2.1	Generic on-network functional model for network resource management	112

14.2.2.2	On-network functional model for network resource management for TSN	113

14.2.2.3	On-network functional model for network resource management for 5G TSC	114

14.2.3	Off-network functional model description	115

14.2.4	Functional entities description	115

14.2.4.1	General	115

14.2.4.2	Network resource management client	115

14.2.4.3	Network resource management server	116

14.2.5	Reference points description	116

14.2.5.1	General	116

14.2.5.2	NRM-UU	116

14.2.5.3	NRM-PC5	116

14.2.5.4	NRM-C	116

14.2.5.5	NRM-S	116

14.2.5.6	NRM-E	116

14.2.5.7	MB2-C	116

14.2.5.8	xMB-C	116

14.2.5.9	Rx	117

14.2.5.10	N5	117

14.2.5.11	N33	117

14.2.5.12	Nmb13	117

14.2.5.13	Nmb10	117

14.2.5.14	N6mb	117

14.2.5.15	Nmb8	117

14.2.5.16	N6	117

14.3	Procedures and information flows for network resource management	117

14.3.1	General	117

14.3.2	Information flows	117

14.3.2.1	Network resource adaptation request	117

14.3.2.2	Network resource adaptation response	118

14.3.2.3	MBMS bearer announcement	118

14.3.2.4	MBMS listening status report	119

14.3.2.5	MBMS suspension reporting instruction	119

14.3.2.6	Resource request	120

14.3.2.7	Resource response	120

14.3.2.8	Resource modification request	120

14.3.2.9	Resource modification response	120

14.3.2.10	MBMS bearers request	121

14.3.2.11	MBMS bearers response	121

14.3.2.12	User plane delivery mode	121

14.3.2.13	end-to-end QoS management request	122

14.3.2.14	end-to-end QoS management response	122

14.3.2.15	QoS downgrade indication	123

14.3.2.16	Application QoS change notification	123

14.3.2.17	Monitoring Events Subscription Request	123

14.3.2.18	Monitoring Events Subscription Response	123

14.3.2.19	Monitoring Events Notification message	124

14.3.2.20	Unicast QoS monitoring subscription request	124

14.3.2.21	Unicast QoS monitoring subscription response	127

14.3.2.22	Unicast QoS monitoring notification	127

14.3.2.23	TSC stream availability discovery request	127

14.3.2.24	TSC stream availability discovery response	127

14.3.2.25	TSC stream creation request	128

14.3.2.26	TSC stream creation response	128

14.3.2.27	TSC stream deletion request	128

14.3.2.28	TSC stream deletion response	128

14.3.2.29	TSN bridge information report	129

14.3.2.30	TSN bridge information confirmation	129

14.3.2.31	TSN bridge configuration request	129

14.3.2.32	TSN bridge configuration response	129

14.3.2.33	Unicast QoS monitoring data request	129

14.3.2.34	Unicast QoS monitoring data response	130

14.3.2.35	Application connectivity request	130

14.3.2.36	Application connectivity response	130

14.3.2.37	Application connectivity notification	131

14.3.2.38	Unicast QoS monitoring subscription update request	131

14.3.2.39	Unicast QoS monitoring subscription update response	131

14.3.3	Unicast resource management	131

14.3.3.1	General	131

14.3.3.2	Unicast resource management with SIP core	132

14.3.3.2.1	Request for unicast resources at VAL service communication establishment	132

14.3.3.2.1.1	General	132

14.3.3.2.1.2	Procedure	132

14.3.3.2.2	Request for modification of unicast resources	133

14.3.3.2.2.1	General	133

14.3.3.2.2.2	Procedure	133

14.3.3.3	Unicast resource management without SIP core	134

14.3.3.3.1	Network resource adaptation	134

14.3.3.3.1.1	General	134

14.3.3.3.1.2	Procedure	134

14.3.3.3.2	Request for unicast resources at VAL service communication establishment	135

14.3.3.3.2.1	General	135

14.3.3.3.2.2	Procedure	135

14.3.3.3.3	Request for modification of unicast resources	136

14.3.3.3.3.1	General	136

14.3.3.3.3.2	Procedure	136

14.3.3.4	Unicast QoS monitoring	137

14.3.3.4.1	Unicast QoS monitoring subscription procedure	137

14.3.3.4.1.1	General	137

14.3.3.4.1.2	Procedure	137

14.3.3.4.2	Unicast QoS monitoring notification procedure	138

14.3.3.4.2.1	General	138

14.3.3.4.2.2	Procedure	138

14.3.3.4.3	Unicast QoS monitoring subscription termination procedure	138

14.3.3.4.3.1	General	138

14.3.3.4.3.2	Procedure	139

14.3.3.4.4	Unicast QoS monitoring data retrieval procedure	139

14.3.3.4.4.1	General	139

14.3.3.4.4.2	Procedure	139

14.3.3.4.5	Unicast QoS monitoring subscription update procedure	140

14.3.3.4.5.1	General	140

14.3.3.4.5.2	Procedure	140

14.3.4	Multicast resource management for EPS	141

14.3.4.1	General	141

14.3.4.2	Use of pre-established MBMS bearers	141

14.3.4.2.1	General	141

14.3.4.2.2	Procedure	142

14.3.4.3	Use of dynamic MBMS bearer establishment	143

14.3.4.3.1	General	143

14.3.4.3.2	Procedure	143

14.3.4.4	MBMS bearer announcement over MBMS bearer	145

14.3.4.4.1	General	145

14.3.4.4.2	Procedure	145

14.3.4.5	MBMS bearer quality detection	147

14.3.4.5.1	General	147

14.3.4.5.2	Procedure	147

14.3.4.6	Service continuity in MBMS scenarios	148

14.3.4.6.1	General	148

14.3.4.6.2	Service continuity when moving from one MBSFN to another	148

14.3.4.7	MBMS suspension notification	151

14.3.4.7.1	General	151

14.3.4.7.2	Procedure	151

14.3.4.8	MBMS bearer event notification	152

14.3.4.8.1	General	152

14.3.4.8.2	Procedure	152

14.3.4.9	Switching between MBMS bearer and unicast bearer	154

14.3.4.9.1	General	154

14.3.4.9.2	Procedure	154

14.3.4A	Multicast resource management for EPS	155

14.3.4A.1	General	155

14.3.4A.2	MBS session creation and MBS session announcement	156

14.3.4A.2.1	General	156

14.3.4A.2.2	Procedure for pre-created MBS session and MBS session announcement	156

14.3.4A.2.3	Procedure for dynamic MBS sessions	158

14.3.4A.3	MBS resources update	160

14.3.4A.3.1	General	160

14.3.4A.3.2	Procedure for updating MBS resources without dynamic PCC rule	160

14.3.4A.4	MBS resource deletion	162

14.3.4A.4.1	General	162

14.3.4A.4.2	Procedure	162

14.3.4A.5	Request to activate / de-activate multicast MBS sessions	163

14.3.4A.5.1	General	163

14.3.4A.5.2	Multicast MBS session activation procedure	163

14.3.4A.5.3	Multicast MBS session de-activation procedure	164

14.3.4A.6	VAL service group media transmissions over 5G MBS sessions	165

14.3.4A.6.1	General	165

14.3.4A.6.2	Procedure	166

14.3.4A.7	Aplication level control signalling over 5G MBS sessions	168

14.3.4A.7.1	Description	168

14.3.4A.7.2	Procedure	168

14.3.5	QoS/resource management for network-assisted UE-to-UE communications	169

14.3.5.1	General	169

14.3.5.2	QoS/resource management capability initiation in network assisted UE-to-UE communications	170

14.3.5.2.1	Procedure	170

14.3.5.3	Procedure for coordinated QoS provisioning operation in network assisted UE-to-UE communications	170

14.3.5.3.1	Procedure	171

14.3.6	Event Monitoring	172

14.3.6.1	General	172

14.3.6.2	Monitoring Events Subscription Procedure	172

14.3.6.2.1	General	172

14.3.6.2.2	Procedure	172

14.3.6.3	Monitoring Events Notification Procedure	173

14.3.6.3.1	General	173

14.3.6.3.2	Procedure	173

14.3.7	5G TSC resource management procedures	174

14.3.7.1	General	174

14.3.7.2	TSC stream availability discovery procedure	174

14.3.7.3	TSC stream creation procedure	174

14.3.7.4	TSC stream deletion procedure	175

14.3.8	TSN resource management procedures	176

14.3.8.1	General	176

14.3.8.2	5GS TSN Bridge information reporting	176

14.3.8.3	5GS TSN Bridge configuration procedure	177

14.3.9	Establishing communication with application service requirements	177

14.3.9.1	General	177

14.3.9.2	Procedures	178

14.3.9.2.1	Procedure for establishing communication with application service requirements	178

14.4	SEAL APIs for network resource management	179

14.4.1	General	179

14.4.2	SS_NetworkResourceAdaptation API	180

14.4.2.1	General	180

14.4.2.2	Reserve_Network_Resource operation	180

14.4.2.3	Request_Unicast_Resource	180

14.4.2.4	Update_Unicast_Resource	180

14.4.2.5	Request_Multicast_Resource	180

14.4.2.6	Notify_UP_Delivery_Mode	181

14.4.2.7	TSC_Stream_ Availability_Discovery	181

14.4.2.8	TSC_Stream_Creation	181

14.4.2.9	TSC_Stream_Deletion	181

14.4.3	SS_EventsMonitoring API	181

14.4.3.1	Subscribe_Monitoring_Events	181

14.4.3.2	Notify_Monitoring_Events	182

14.4.4	SS_NetworkResourceMonitoring API	182

14.4.4.1	General	182

14.4.4.2	Subscribe_Unicast_QoS_Monitoring operation	182

14.4.4.3	Notify_Unicast_QoS_Monitoring operation	182

14.4.4.4	Unsubscribe_Unicast_QoS_Monitoring operation	182

14.4.4.5	Obtain_Unicast_QoS_Monitoring_Data operation	183

14.4.4.6	Update_Unicast_QoS_Monitoring_Subscription operation	183

15	Service-based interface representation of the functional model for SEAL services	184

15.1	General	184

15.2	Functional model representation	184

15.3	Service-based interfaces	184

16	Network slice capability enablement	185

16.1	General	185

16.2	Functional model	185

16.2.1	General	185

16.2.2	Functional model description	185

16.2.3	Functional entities description	186

16.2.3.1	General	186

16.2.3.2	Network slice capability enablement server	186

16.2.3.3	Network slice capability enablement client	186

16.2.4	Reference points description	186

16.2.4.1	General	186

16.2.4.2	NSCE-UU	187

16.2.4.3	NSCE-C	187

16.2.4.4	NSCE-S	187

16.2.4.5	N33	187

16.3	Procedures and information flows for network slice capability enablement	187

16.3.1	General	187

16.3.2	Network slice adaptation for VAL application	187

16.3.2.1	General	187

16.3.2.2	Information flows	187

16.3.2.2.1	Network slice adaptation request	187

16.3.2.2.2	Network slice adaptation response	188

16.3.2.2.3	Network slice adaptation trigger	188

16.3.2.2.4	Network slice adaptation notification	188

16.3.2.3	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application	188

16.3.2.4	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application	189

16.4	SEAL APIs for network slice capability enablement	190

16.4.1	General	190

16.4.2	SS_NetworkSliceAdaptation API	191

16.4.2.1	General	191

16.4.2.2	Network_Slice_Adaptation	191

17	Notification Management	191

17.1	General	191

17.2	Functional model	191

17.2.1	General	191

17.2.2	Functional model description	191

17.2.3	Functional entities description	192

17.2.3.1	General	192

17.2.3.2	Notification Management client	192

17.2.3.3	Notification Management server	192

17.2.4	Reference points description	193

17.2.4.1	General	193

17.2.4.2	NM-UU	193

17.2.4.3	NM-C	193

17.2.4.4	NM-S	193

17.3	Procedures and information flows for notification management	193

17.3.1	General	193

17.3.2	Information flows for notification management	193

17.3.2.1	Create notification channel request	193

17.3.2.2	Create notification channel response	193

17.3.2.3	Open notification channel	194

17.3.2.4	Notification message	194

17.3.2.5	Delete notification channel request	194

17.3.2.6	Delete notification channel response	195

17.3.3	Procedure for creating notification channel to receive notifications	195

17.3.4	Procedure for deleting notification channel	196

Annex A (informative): SEAL integration with 3GPP network exposure systems	197

Annex B (informative): SEAL functional model mapping with Common functional architecture (CFA)	200

Annex C (normative): Protocol realizations of LWP in the signalling control plane	201

C.1	General	201

C.2	Usage of CoAP as LWP	201

Annex D (informative): Change history	202

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

This document specifies a functional architecture for service enabler architecture layer (SEAL) over 3GPP networks to support vertical applications (e.g. V2X applications). This functional architecture will include common application plane and signalling plane entities. A set of common services (e.g. group management, configuration management, location management) specified in this document can be shared across vertical applications.

The SEAL functional architecture takes into consideration the common capabilities to support mission critical and other vertical applications.

# 1	Scope

The present document specifies the functional architecture for service enabler architecture layer (SEAL) and the procedures, information flows and APIs for each service within SEAL in order to support vertical applications over the 3GPP system. The present document is applicable to vertical applications using E-UTRAN or NR access based on the EPC or 5GS architecture defined in 3GPP TS 23.401 [9], 3GPP TS 23.246 [17], 3GPP TS 23.468 [16], 3GPP TS 23.501 [10], and 3GPP TS 23.247 [39]. To ensure efficient use and deployment of vertical applications over 3GPP systems this specification for SEAL services includes the group management, configuration management, location management, identity management, key management and network resource management.

NOTE: In the present document, the multicast services offered by SEAL are only applicable for EPS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[37]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

[38]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[39]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

VAL user: An authorized user, who can use a VAL UE to participate in one or more VAL services.

VAL user ID: A generic name for the user ID of a VAL user within a specific VAL service.

VAL UE: A UE that can be used to participate in one or more VAL services.

VAL client: An entity that provides the client side functionalities corresponding to the vertical applications.

SEAL client: An entity that provides the client side functionalities corresponding to the specific SEAL service.

VAL service: A generic name for any service offered by the VAL service provider to their VAL users.

SEAL service: A generic name for a common service (e.g. group management, configuration management, location management) that can be utilized by multiple vertical applications.

SEAL provider: Provider of SEAL service(s).

VAL server: A generic name for the server application function of a specific VAL service.

SEAL server: An entity that provides the server side functionalities corresponding to the specific SEAL service.

VAL system: The collection of applications, services, and enabling capabilities required to support a VAL service.

Primary VAL system: VAL system where the VAL user profiles of a VAL user are defined.

Partner VAL system: A VAL system that has a business relationship with the primary VAL system such that service can be offered to primary VAL system users.

VAL group: A defined set of VAL UEs or VAL users configured for specific purpose in a VAL service.

NOTE:	The set could be of either VAL UEs or VAL users depending on the specific VAL service.

VAL group home system: The VAL system where the VAL group is defined.

VAL group member: A VAL service user, whose VAL user ID is listed in a particular VAL group.

VAL stream: A time sensitive communication stream is used to transport a time sensitive data flow, and is defined by a stream specification (which identifies a source and destination of the data flow) and a traffic specification (which defines the characteristics of the data flow). VAL stream is identified by a VAL stream ID.

Vertical: See vertical domain.

Vertical application: An application catering to a specific vertical.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 22.104 [2] apply:

Vertical domain

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

5GVN	5G Virtual Network

CAPIF	Common API Framework for northbound APIs

CNC	Centralized Network Configuration

CRUDN	Create, Retrieve, Update, Delete and Notify

EPC	Evolved Packet Core

GPSI	Generic Public Subscription Identifier

LWP	Light-weight Protocol

NEF	Network Exposure Function

NR	New Radio

PCC	Policy and Charging Control

SCEF	Service Capability Exposure Function

SEAL	Service Enabler Architecture Layer for Verticals

TSC	Time Sensitive Communication

TSN	Time Sensitive Networking

VAL	Vertical Application Layer

# 4	Architectural requirements

## 4.1	General

### 4.1.1	Description

This subclause specifies the general requirements for SEAL.

### 4.1.2	Requirements

[AR-4.1.2-a] The SEAL shall support applications from one or more verticals.

[AR-4.1.2-b] The SEAL shall support multiple applications from the same vertical.

[AR-4.1.2-c] The SEAL shall offer SEAL services as APIs to the vertical applications.

[AR-4.1.2-d] The SEAL shall support notification mechanism for SEAL service events.

[AR-4.1.2-e] The API interactions between the vertical application server(s) and SEAL server(s) shall conform to CAPIF as specified in 3GPP TS 23.222 [8].

[AR-4.1.2-f] The SEAL server(s) shall provide a service API compliant with CAPIF as specified in 3GPP TS 23.222 [8].

## 4.2	Deployment models

### 4.2.1	Description

This subclause specifies the requirements for various deployment models.

### 4.2.2	Requirements

[AR-4.2.2-a] The SEAL shall support deployments in which SEAL services are deployed only within PLMN network.

[AR-4.2.2-b] The SEAL shall support deployments in which SEAL services are deployed only outside of PLMN network.

[AR-4.2.2-c] The SEAL shall support deployments in which SEAL services are deployed both within and outside the PLMN domain at the same time.

[AR-4.2.2-d] The SEAL shall support SEAL capabilities for centralized deployment of vertical applications.

[AR-4.2.2-e] The SEAL shall support SEAL capabilities for distributed deployment of vertical applications.

## 4.3	Location management

### 4.3.1	Description

This subclause specifies the requirements for location management service.

### 4.3.2	Requirements

[AR-4.3.2-a] The SEAL shall enable sharing location data between client and server for vertical applications usage.

[AR-4.3.2-b] The SEAL shall support different granularity of location data, as required by the vertical application.

[AR-4.3.2-c] The SEAL shall support requests for on-demand location reporting.

[AR-4.3.2-d] The SEAL shall support client location reporting based on triggers.

[AR-4.3.2-e] The SEAL shall enable vertical applications to receive updates to the location information.

[AR-4.3.2-f] The SEAL shall enable sharing the network location information obtained from the 3GPP network systems to the vertical applications.

[AR-4.3.2-g] The SEAL shall provide a mechanism to enable vertical applications to obtain a list of UE(s), and the location information of each UE, in the proximity to a designated/requested location.

## 4.4	Group management

### 4.4.1	Description

This subclause specifies the requirements for group management service.

### 4.4.2	Requirements

[AR-4.4.2-a] The SEAL shall enable group management operations (e.g. CRUDN) by the authorized users or VAL server.

[AR-4.4.2-b] The SEAL shall enable creation of group to be used by one or more vertical applications within the same VAL system.

[AR-4.4.2-c] The SEAL shall enable two or more groups to be merged (temporarily or permanently) into a single group by the authorized users or VAL server wherein all the group members of the constituent groups are designated as members of the merged group.

## 4.5	Configuration management

### 4.5.1	Description

This subclause specifies the requirements for configuration management service.

### 4.5.2	Requirements

[AR-4.5.2-a] The SEAL shall enable configuring service specific configuration data applicable to vertical applications.

[AR-4.5.2-b] The SEAL shall support configuring data applicable to different vertical applications.

## 4.6	Key management

### 4.6.1	Description

This subclause specifies the requirements for key management service.

### 4.6.2	Requirements

[AR-4.6.2-a] The SEAL shall support secure distribution of security related information (e.g. encryption keys).

[AR-4.6.2-b] The SEAL shall support all communications in SEAL ecosystem to be secured.

## 4.7	Identity management

### 4.7.1	Description

This subclause specifies the requirements for identity management service.

### 4.7.2	Requirements

[AR-4.7.2-a] The SEAL shall enable the access to SEAL services from the vertical application layer entities to be authorized.

Editor's Note: The relationship between identity management service and authorization is FFS.

## 4.8	Network resource management

### 4.8.1	Description

This subclause specifies the requirements for network resource management service.

### 4.8.2	Requirements

[AR-4.8.2-a] The SEAL shall enable support for unicast bearer establishment and modification to support service KPIs for VAL communications.

[AR-4.8.2-b] The SEAL shall enable support for multicast bearer establishment and modification to support service KPIs for VAL communications.

[AR-4.8.2-c] The SEAL shall support announcement of multicast bearers to the UEs.

[AR-4.8.2-d] The SEAL shall support switching of bearers between unicast and multicast.

[AR-4.8.2-e] The SEAL shall support multicast bearer quality detection.

[AR-4.8.2-f] The SEAL shall enable support for unicast PDU session establishment and modification to support service KPIs for VAL communications.

[AR-4.8.2-g] The SEAL shall enable support for MBS session (multicast or broadcast type) establishment and modification to support service KPIs for VAL communications.

[AR-4.8.2-h] The SEAL shall support announcement of MBS session (multicast or broadcast type) to the UEs.

[AR-4.8.2-i] The SEAL shall support switching of between unicast PDU session and MBS session (multicast or broadcast type).

[AR-4.8.2-j] The SEAL shall support MBS session (multicast or broadcast type) quality detection.

# 5	Involved business relationships

Figure 5-1 shows the business relationships that exist and that are needed to support a single VAL user.

Figure 5-1: Business relationships for VAL services

The VAL user belongs to a VAL service provider based on a VAL service agreement between the VAL user and the VAL service provider. The VAL service provider can have VAL service agreements with several VAL users. The VAL user can have VAL service agreements with several VAL service providers.

The VAL service provider and the home PLMN operator can be part of the same organization, in which case the business relationship between the two is internal to a single organization.

The VAL service provider can have SEAL provider arrangements with multiple SEAL providers and the SEAL provider can have PLMN operator service arrangements with multiple home PLMN operators. The SEAL provider and the VAL service provider or the home PLMN operator can be part of the same organization, in which case the business relationship between the two is internal to a single organization.

The home PLMN operator can have PLMN operator service arrangements with multiple VAL service providers and the VAL service provider can have PLMN operator service arrangements with multiple home PLMN operators. As part of the PLMN operator service arrangement between the VAL service provider and the home PLMN operator, PLMN subscription arrangements can be provided which allows the VAL UEs to register with home PLMN operator network.

The home PLMN operator can have PLMN roaming agreements with multiple visited PLMN operators and the visited PLMN operator can have PLMN roaming agreements with multiple home PLMN operators.

# 6	Generic functional model for SEAL services

## 6.1	General

The functional model for SEAL is organized into generic SEAL service functional model and specific SEAL service functional models. The generic SEAL service functional model will be used as the reference model for the specific SEAL service functional models.

The following SEAL services are supported towards the vertical application layer:

-	Location management;

-	Group management;

-	Configuration management;

-	Identity management;

-	Key management; and

-	Network resource management.

The generic functional model for the SEAL is organized into generic functional entities to describe a functional architecture which addresses the application layer support aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

## 6.2	On-network functional model description

Figure 6.2-1 illustrates the generic on-network functional model for SEAL.

Figure 6.2-1: Generic on-network functional model

In the vertical application layer, the VAL client communicates with the VAL server over VAL-UU reference point. VAL-UU supports both unicast and multicast delivery modes.

NOTE 1:	The VAL-UU reference point is out of scope of the present document.

The SEAL functional entities on the UE and the server are grouped into SEAL client(s) and SEAL server(s) respectively. The SEAL consists of a common set of services (e.g. group management, location management) and reference points. The SEAL offers its services to the vertical application layer (VAL).

NOTE 2:	The functionalities and reference points of the vertical application layer are out of scope of the present document.

NOTE 3:	The vertical application layer may further consist of vertical application enabler layer functionalities (specified by 3GPP) and application specific functionalities, which is out of scope of the present document.

The SEAL client(s) communicates with the SEAL server(s) over the SEAL-UU reference points. SEAL-UU supports both unicast and multicast delivery modes. The SEAL client(s) provides the service enabler layer support functions to the VAL client(s) over SEAL-C reference points. The VAL server(s) communicate with the SEAL server(s) over the SEAL-S reference points. The SEAL server(s) may communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system.

Editor's Note:	SEAL-UU support for multicast delivery is FFS.

The specific SEAL client(s) and the SEAL server(s) along with their specific SEAL-UU reference points and the specific network interfaces of 3GPP network system used are described in the respective on-network functional model for each SEAL service.

Figure 6.2-2 illustrates the functional model for interconnection between SEAL servers.

Figure 6.2-2: Interconnection between SEAL servers

To support distributed SEAL server deployments, the SEAL server interacts with another SEAL server for the same SEAL service over SEAL-E reference point.

Figure 6.2-3 illustrates the functional model for inter-service communication between SEAL servers.

Figure 6.2-3: Inter-service communication between SEAL servers

The SEAL server interacts with another SEAL server for inter-service communication over SEAL-X reference point.

Figure 6.2-4 illustrates the functional model for communication between SEAL server and VAL user database.

Figure 6.2-4: Communication between SEAL server and VAL user database

The SEAL server interacts with the VAL user database for storing and retrieving user profile over VAL-UDB reference point.

Figure 6.2-5 shows the functional model for the signalling control plane.

Figure 6.2-5: Functional model for signalling control plane

NOTE:	The Light-weight Protocol (LWP) functional entities and reference points are a generic representation of protocol entities and reference points for use in constrained environments. Realizations of LWP by means of a particular transport protocol are defined in the annex of this specification. Realizations of LWP by means of transport protocols is not limited to those defined in the annex of this specification.

## 6.3	Off-network functional model description

Figure 6.3-1 illustrates the generic off-network functional model for SEAL.

Figure 6.3-1: Generic off-network functional model

In the vertical application layer, the VAL client of UE1 communicates with VAL client of UE2 over VAL-PC5 reference point. A SEAL client of UE1 interacts with the corresponding SEAL client of UE2 over SEAL-PC5 reference points. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

NOTE:	The VAL-PC5 reference point is out of scope of the present document.

Editor's Note:	The functionalities of reference points between the SEAL clients of two UEs over SEAL-PC5 reference point is FFS.

The specific SEAL client(s) along with their specific SEAL-PC5 reference points are described in the respective off-network functional model for each SEAL service.

## 6.4	Functional entities description

### 6.4.1	General

Each subclause is a description of a functional entity corresponding to SEAL and does not imply a physical entity.

### 6.4.2	Application plane

#### 6.4.2.1	General

Entities within the application plane of a VAL system provide application control and media specific functions to support one or more VAL services.

#### 6.4.2.2	VAL client

The VAL client provides the client side functionalities corresponding to the vertical applications (e.g. V2X client). The VAL client supports interactions with the SEAL client(s).

NOTE:	The details of the VAL client is specific to the vertical and out of scope of the present document.

#### 6.4.2.3	VAL server

The VAL server provides the server side functionalities corresponding to the vertical applications (e.g. V2X application servers). The VAL server acts as CAPIF's API invoker as specified in 3GPP TS 23.222 [8].

NOTE:	The details of the VAL server is specific to the vertical and out of scope of the present document.

#### 6.4.2.4	SEAL client

The SEAL client provides the client side functionalities corresponding to the specific SEAL service. The SEAL client(s) supports interactions with the VAL client(s). The SEAL client also supports interactions with the corresponding SEAL client between the two UEs.

NOTE:	It is up to each SEAL client to support the appropriate signalling plane entities.

#### 6.4.2.5	SEAL server

The SEAL server provides the server side functionalities corresponding to the specific SEAL service. The SEAL server supports interactions with the VAL server(s). The SEAL server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The SEAL server also supports interactions with the corresponding SEAL server in distributed SEAL deployments.

NOTE:	It is up to each SEAL server to support the appropriate signalling plane entities.

#### 6.4.2.6	VAL user database

This functional entity contains information of the user profile associated with a VAL service that is served by the VAL service provider at the application plane.

Each VAL service may have a corresponding user database e.g. MCPTT user database as defined in 3GPP TS 23.379 [3], MCVideo user database as defined in 3GPP TS 23.281 [5] and MCData user database as defined in 3GPP TS 23.282 [6].

NOTE:	It is up to each SEAL server to support the appropriate signalling plane entities.

### 6.4.3	Signalling control plane

#### 6.4.3.1	SIP entities

##### 6.4.3.1.1	Signalling user agent

This functional entity acts as the SIP user agent (both client and server) for all SIP transactions.

##### 6.4.3.1.2	SIP AS

The SIP AS functional entity supports the following functions on behalf of the VAL service:

-	influencing and impacting the SIP session; and

-	supporting event subscription and event notification.

NOTE:	In the IM CN subsystem, this is provided by the Application Server as defined in 3GPP TS 23.002 [14].

##### 6.4.3.1.3	SIP core

###### 6.4.3.1.3.1	General

The SIP core contains a number of sub-entities responsible for registration, service selection and routing in the signalling control plane.

The SIP core shall be either:

1.	compliant with 3GPP TS 23.228 [15], i.e. the SIP core is a 3GPP IP multimedia core network subsystem; or

2.	a SIP core, which internally need not comply with the architecture of 3GPP TS 23.228 [15], but with the reference points that are defined in subclause 6.5.3 (if exposed), compliant to the reference points defined in 3GPP TS 23.002 [14].

The data related to the functions of the SIP core, e.g. for data for application service selection, the identity of the serving registrar or authentication related information may be provided by the PLMN operator responsible for the bearer plane. In this case, the SIP database that is the source of the data may be part of the HSS. Alternatively, this data may be provided by the VAL service provider. In this case, the source of the data may be the VAL service provider's SIP database.

###### 6.4.3.1.3.2	Local inbound / outbound proxy

The local inbound / outbound proxy functional entity acts as both an inbound proxy and an outbound proxy for all SIP transactions. This functional entity can provide the following functions:

-	NAT traversal;

-	Resource control;

-	Route/forward requests and responses to the user agents;

-	SIP signalling security; and

-	Depending on the PLMN operator policy, discovery and address resolution, including E.164 numbers.

NOTE:	In the IM CN subsystem, this functional entity is provided by the P-CSCF as defined in 3GPP TS 23.228 [15].

###### 6.4.3.1.3.3	Registrar finder

The registrar finder functional entity is responsible for:

a)	Identifying the serving registrar / application service selection functional entity. The serving registrar / application service selection functional entity is identified using information provided either by the PLMN operator's own SIP database or the VAL service provider's SIP database, and optionally using the PLMN operator's internal information e.g. network topology, registrar availability.

1)	Registrar finder and registrar in the VAL service provider domain: registrar finder in the VAL service provider's domain uses the information from the VAL service provider's SIP database to identify the serving registrar in the VAL service provider domain.

2)	Registrar finder and registrar in the PLMN operator domain: registrar finder uses information from PLMN operator's SIP database to identify the serving registrar in the PLMN operator domain.

3)	Registrar finder in PLMN operator domain and registrar in VAL service provider domain: registrar finder uses information from the VAL service provider's SIP database to identify the serving registrar in the VAL service provider domain.

NOTE 1:	The need for the registrar finder is deployment specific e.g. a deployment that has only one registrar does not need the registrar finder and the related SIP database information.

b)	Providing discovery and address resolution, including E.164 numbers.

NOTE 2:	In the IM CN subsystem, this is provided by the I-CSCF as defined in 3GPP TS 23.228 [15].

###### 6.4.3.1.3.4	Registrar / application service selection

The registrar / application service selection functional entity provides the following functions:

-	Registrar function (with integral provision of a location server) and also acts as an inbound proxy (with access to the integral location server), and outbound proxy for all SIP transactions where application service selection is required. It registers the user and maintains the association of the location and identity of the user in a location service. It provides notifications of the registration states.

-	Supports authentication for identities provided within SIP signalling. Both the registrar (with integral location server) and authentication functions are supported by access either to the public network's own SIP database or the VAL service provider's SIP database.

-	Can provide the application service selection for all SIP transactions, possibly based on application service selection information stored by either the public network's own SIP database or the VAL service provider's SIP database.

-	Performs SIP signalling security.

NOTE:	In the IM CN subsystem, this is provided by the S-CSCF as defined in 3GPP TS 23.228 [15].

##### 6.4.3.1.4	Diameter proxy

This functional entity acts as a proxy agent for Diameter messaging as specified in IETF RFC 6733 [24].

The Diameter proxy, when used on the AAA-2 interface, is collocated with the migration management server.

Other instances of the Diameter proxy may also be present in the SIP core / IMS.

NOTE:	The number of instances of the Diameter proxy is deployment specific.

#### 6.4.3.2	SIP database

##### 6.4.3.2.1	General

The SIP database contains information concerning the SIP subscriptions and corresponding identity and authentication information required by the SIP core, and such information as application service selection.

In deployment scenarios where the PLMN operator provides the SIP core, this database is provided by the HSS.

In deployment scenarios where the VAL service provider provides the SIP core, the SIP database may be provided by the VAL service provider.

Access to the data residing in the SIP database is restricted to the SIP core entities that are specifically serving the subscriber/user whose data are stored, i.e. registrars and registrar finders can access SIP databases only when they are part of the same trust domain for the data being provided.

NOTE:	The SIP database can be in a different network than the registrar finder since the trust domain for the criteria for registrar selection can be different than the trust domain for the signalling plane user identities.

The SIP database is responsible for storing the following user related information:

-	signalling plane user identities: Numbering and addressing information;

-	signalling plane security information: SIP core access control information for authentication and authorization;

-	VAL UE Location information at inter-system level: the SIP database supports the user registration, and stores inter-system location information, etc.; and

-	signalling plane subscription profile (including initial filter criteria).

The SIP database also generates signalling plane security information for mutual authentication, communication integrity check and ciphering.

Based on this information, the SIP database is also responsible to support the call control and session management entities of the SIP core.

The SIP database consists of the following functionalities:

-	support for control functions of the SIP core such as the Registrar and Registrar finder. This is needed to enable subscriber usage of the SIP core services. This functionality is independent of the access network used to access the SIP core; and

-	authentication functionality required by the SIP core to authenticate the VAL UE.

##### 6.4.3.2.2	SIP database logical functions

The SIP database provides the following logical functions:

a)	mobility management;

-	provides the UE mobility through the SIP core.

b)	registrar assignment support;

-	provides to the registrar finder the required capabilities for VAL services based on VAL service provider requirements on a per-user basis, (e.g. whether a particular registrar within the PLMN operator's network (e.g. a registrar reserved for VAL service use or a registrar in a secure location) or a registrar within the VAL service provider network is assigned.

c)	call and/or session establishment support;

-	provides the call and/or session establishment procedures in the SIP core. For terminating traffic, it provides information on which registrar currently hosts the user.

d)	user security information generation;

-	provides generation of user authentication, integrity and ciphering data for the SIP core.

e)	signalling plane security support;

-	provides authentication procedures to access VAL services by storing the generated data for authentication, integrity and ciphering at the signalling plane and by providing these data to the appropriate registrar.

f)	user identification handling;

-	provides the appropriate relations among all the identifiers uniquely determining the signalling plane identities in the SIP core e.g. IMS public identities.

g)	access authorisation; and

-	provides authorisation of the user for mobile access when requested by the registrar e.g. by checking that the user is allowed to roam to that visited network.

h)	service authorisation support.

-	provides basic authorisation for terminating call/session establishment and service invocation. The SIP database may update the registrar with filter criteria to trigger the VAL server(s).

#### 6.4.3.3	HTTP entities

##### 6.4.3.3.1	HTTP client

This functional entity acts as the client for all hypertext transactions.

##### 6.4.3.3.2	HTTP proxy

This functional entity acts as a proxy for hypertext transactions between the HTTP client and one or more HTTP servers. The HTTP proxy terminates a TLS session on HTTP-1 with the HTTP client of the VAL UE allowing the HTTP client to establish a single TLS session for hypertext transactions with multiple HTTP servers that are reachable by the HTTP proxy.

The HTTP proxy terminates the HTTP-3 reference point that lies between different HTTP proxies. It may provide a topology hiding function from HTTP entities outside the trust domain of the VAL system.

The HTTP proxy shall be in the same trust domain as the HTTP clients and HTTP servers that are located within a VAL service provider's network. There can be multiple instances of an HTTP proxy e.g. one per trust domain.

NOTE:	The number of instances of the HTTP proxy is deployment specific.

##### 6.4.3.3.3	HTTP server

This functional entity acts as the HTTP server for all hypertext transactions.

#### 6.4.3.4	LWP entities

##### 6.4.3.4.1	LWP client

This functional entity acts as the light-weight protocol client for all transactions of the SEAL client executing in a constrained UE. A SEAL client executing in an unconstrained UE may choose to use the LWP client if it is available.

##### 6.4.3.4.2	LWP proxy

This functional entity acts as a proxy for transactions between the LWP client and one or more LWP servers. The LWP proxy typically terminates a secure transport protocol (e.g. DTLS, TLS or secure WebSocket) session on LWP-1 reference point with the LWP client of the VAL UE allowing the LWP client to establish a single secure session for transactions with multiple LWP servers that are reachable by the LWP proxy.

The LWP proxy can act as a cross-protocol LWP-HTTP proxy to enable LWP clients to access resources on HTTP servers via the LWP-HTTP-2 reference point.

The LWP proxy terminates LWP-3 reference point that lies between different LWP proxies. It may provide a topology hiding function from LWP entities outside the trust domain of the VAL system.

The LWP proxy can also terminate LWP-HTTP-3 reference point for interworking with another HTTP proxy. In this role it provides cross-protocol mapping and may provide a topology hiding function from HTTP entities outside the trust domain of the VAL system.

The LWP proxy shall be in the same trust domain as the LWP clients and LWP servers that are located within a VAL service provider's network. There can be multiple instances of a LWP proxy e.g. one per trust domain.

##### 6.4.3.4.3	LWP server

This functional entity acts as the LWP server for all LWP transactions of the SEAL server.

NOTE:	A SEAL client can act as LWP server for certain transactions as required by the SEAL service.

#### 6.4.3.5	LWP usage

LWP is a generic representation of a light-weight protocol for use in constrained environments. Realizations of the light-weight protocol (LWP) functional entities and reference points to a particular protocol are defined in the annexes of this specification.

LWP is a representation of a protocol to be used by the SEAL service enablers on their respective SEAL-UU reference points when the SEAL client is executing in a constrained UE. In this case the SEAL client should use the LWP-1 reference point with the LWP proxy and should use either the LWP-2 or the LWP-HTTP-2 reference point for transport and routing of the related signalling with the SEAL server.

Editor's note: Which procedures of a SEAL service enabler are not necessary to be supported for a constrained UE is FFS.

A SEAL client executing in a non-constrained UE may choose to use the LWP-1 reference point with the LWP proxy and may use either the LWP-2 or the LWP-HTTP-2 reference point for transport and routing of the related signalling with the SEAL server.

LWP may be used for interactions between SEAL servers on their respective SEAL-E reference points. For this usage the SEAL-E reference point shall use the LWP-1 and either the LWP-2 or the LWP-3 reference point depending on the trust relationship between the interacting SEAL servers.

## 6.5	Reference points description

### 6.5.1	General reference point principle

The protocols on any reference point that is exposed for VAL service interoperability with other SIP core or other IMS entities in other systems shall be compatible with the protocols defined for the corresponding reference point defined in 3GPP TS 23.002 [14].

### 6.5.2	Application plane

#### 6.5.2.1	General

The reference points for the generic functional model for SEAL are described in the following subclauses.

#### 6.5.2.2	VAL-UU

The interactions related to vertical application layer support functions between VAL client and VAL server are supported by VAL-UU reference point. This reference point is an instance of Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

NOTE:	The details of VAL-UU reference point is out of scope of the present document.

#### 6.5.2.3	VAL-PC5

The interactions related to vertical application layer support functions between the VAL clients of two UEs are supported by VAL-PC5 reference point. This reference point is an instance of PC5 reference point as described in 3GPP TS 23.303 [12].

NOTE:	The details of VAL-PC5 reference point is out of scope of the present document.

#### 6.5.2.4	SEAL-UU

The interactions between a SEAL client and the corresponding SEAL server are generically referred to as SEAL-UU reference point. The specific SEAL service reference point corresponding to SEAL-UU is specified in the specific SEAL service functional model.

#### 6.5.2.5	SEAL-PC5

The interactions between the SEAL clients of two VAL UEs are generically referred to as SEAL-PC5 reference point. The specific SEAL service reference point corresponding to SEAL-PC5 is specified in the specific SEAL service functional model.

#### 6.5.2.6	SEAL-C

The interactions between the VAL client(s) and the SEAL client(s) within a VAL UE are generically referred to as SEAL-C reference point. The specific SEAL service reference point corresponding to SEAL-C is specified in the specific SEAL service functional model.

#### 6.5.2.7	SEAL-S

The interactions between the VAL server and the SEAL server are generically referred to as SEAL-S reference point. The specific SEAL service reference point corresponding to SEAL-S is specified in the specific SEAL service functional model.

#### 6.5.2.8	SEAL-E

The interactions between the SEAL servers of the same type are generically referred to as SEAL-E reference point. The specific SEAL service reference point corresponding to SEAL-E is specified in the specific SEAL service functional model.

#### 6.5.2.9	SEAL-X

##### 6.5.2.9.1	General

The interactions between the SEAL servers of different type are generically referred to as SEAL-X reference point. The specific SEAL server interactions corresponding to SEAL-X are described in the following subclauses.

##### 6.5.2.9.2	Reference point SEAL-X1 (between the key management server and the group management server)

The SEAL-X1 reference point, which exists between the key management server and the group management server, provides a means for the key management server to provide security related information (e.g. encryption keys) to the group management server.

The SEAL-X1 reference point shall use the HTTP-1 and HTTP-2 reference points and may use the HTTP-3 reference point for transport and routing of security related information to the group management server.

NOTE:	SEAL-X1 is specified in subclause 5.1.1.1 of 3GPP TS 33.434 [29].

##### 6.5.2.9.3	Reference point SEAL-X2 (between the group management server and the location management server)

The SEAL-X2 reference point enables the group management server to interact with the location management server.

The SEAL-X2 reference point supports:

-	the group management server to create a location-based group with the help from the location management server.

##### 6.5.2.9.4	Reference point SEAL-X3 (between the group management server and the configuration management server)

The SEAL-X3 reference point enables the group management server to interact with the configuration management server.

The SEAL-X3 reference point supports:

-	the group management server to retrieve VAL service data from the configuration management server.

#### 6.5.2.10	Reference point VAL-UDB (between the VAL user database and the SEAL server)

The VAL-UDB reference point, which exists between the VAL user database and the SEAL server, is used for:

-	storing the user profile data in the specific VAL user database; and

-	obtaining the user profile from the specific VAL user database for further configuration in the UE.

NOTE:	The details of the VAL-UDB reference point is out of scope of the present document.

### 6.5.3	Signalling control plane

#### 6.5.3.1	General

The reference points for the SIP and HTTP signalling are described in the following subclauses.

#### 6.5.3.2	Reference point SIP-1(between the signalling user agent and the SIP core)

The SIP-1 reference point, which exists between the signalling user agent and the SIP core for establishing a session in support of VAL service, shall use the Gm reference point as defined in 3GPP TS 23.002 [14] (with necessary enhancements to support VAL service requirements and profiled to meet the minimum requirements for support of VAL service). The SIP-1 reference point fulfils the requirements of the GC1 reference point specified in 3GPP TS 23.468 [16]. The SIP-1 reference point is used for:

-	SIP registration;

-	authentication and security to the service layer;

-	event subscription and event notification;

-	communication of the TMGI for multicast operation;

-	overload control;

-	session management; and

-	media negotiation.

#### 6.5.3.3	Reference point SIP-2 (between the SIP core and the SIP AS)

The SIP-2 reference point, which exists between the SIP core and the SIP AS for establishing a session in support of VAL service, shall use the ISC and Ma reference points as defined in 3GPP TS 23.002 [14]. The SIP-2 reference point is used for:

-	notification to the VAL service server(s) of SIP registration by the VAL UE;

-	authentication and security to the service layer;

-	event subscription and event notification;

-	communication of the TMGI for multicast operation;

-	session management; and

-	media negotiation.

#### 6.5.3.4	Reference point SIP-3 (between the SIP core and SIP core)

The SIP-3 reference point, which exists between one SIP core and another SIP core for establishing a session in support of VAL service, shall use the Mm and ICi reference points as defined in 3GPP TS 23.002 [14]. The SIP-3 reference point is used for:

-	event subscription and event notification;

-	session management; and

-	media negotiation.

Editor's note:	it is FFS whether changes are needed to SIP-3 when used between servers in different trust domains.

#### 6.5.3.5	Reference point HTTP-1 (between the HTTP client and the HTTP proxy)

The HTTP-1 reference point exists between the HTTP client and the HTTP proxy. Between the VAL UE and the HTTP proxy, the HTTP-1 reference point shall use the Ut reference point as defined in 3GPP TS 23.002 [14] (with necessary enhancements to support specific VAL service requirements). The HTTP-1 reference point is based on HTTP (which may be secured using e.g. SSL, TLS).

#### 6.5.3.6	Reference point HTTP-2 (between the HTTP proxy and the HTTP server)

The HTTP-2 reference point, which exists between the HTTP proxy and the HTTP server, is based on HTTP (which may be secured using e.g. SSL, TLS).

#### 6.5.3.7	Reference point HTTP-3 (between the HTTP proxy and HTTP proxy)

The HTTP-3 reference point, which exists between the HTTP proxy and another HTTP proxy in a different network, is based on HTTP (which may be secured using e.g. SSL, TLS).

Editor's note:	it is FFS whether changes are needed to HTTP-3 when used between servers in different trust domains.

#### 6.5.3.8	Reference point AAA-1 (between the SIP database and the SIP core)

The AAA-1 reference point, which exists between the SIP database and the SIP core, is used by the SIP core to retrieve signalling plane data from the SIP database. The AAA-1 reference point utilises the Cx reference point as defined in 3GPP TS 23.002 [14].

In some deployment scenarios the registrar and SIP database are located in the VAL service provider's network while the registrar finder is in the PLMN operator's network and the AAA-1 reference point is an inter-network interface.

#### 6.5.3.9	Reference point AAA-2 (between the SIP core and Diameter proxy)

The AAA-2 reference point, which exists between the SIP core / IMS and Diameter proxy for SIP registration during migration, shall use the Cx reference point as defined in 3GPP TS 23.002 [14]. The AAA-2 reference point is used for:

-	authentication and security to the service layer for migration;

#### 6.5.3.10	Reference point LWP-1 (between the LWP client and the LWP proxy)

The LWP-1 reference point exists between the LWP client and the LWP proxy.

#### 6.5.3.11	Reference point LWP-2 (between the LWP proxy and the LWP server)

The LWP-2 reference point exists between the LWP proxy and the LWP server.

#### 6.5.3.12	Reference point LWP-3 (between the LWP proxy and LWP proxy)

The LWP-3 reference point exists between the LWP proxy and another LWP proxy in a different network.

#### 6.5.3.13	Reference point LWP-HTTP-2 (between the LWP proxy and the HTTP server)

The LWP-HTTP-2 reference point exists between the LWP proxy and the HTTP server. HTTP-2 and LWP-HTTP-2 reference points are equivalent.

#### 6.5.3.14	Reference point LWP-HTTP-3 (between the LWP proxy and the HTTP proxy)

The LWP-HTTP-3 reference point exists between the LWP proxy and another HTTP proxy in a different network. HTTP-3 and LWP-HTTP-3 reference points are equivalent.

# 7	Identities

## 7.1	User identity (User ID)

The VAL user presents the user identity to the identity management server during a user authentication transaction, to provide the identity management client a means for VAL service authentication. In general, since identity management is a common SEAL service, it uses a set of credentials (e.g. biometrics, secureID, username/password) that may not necessarily be tied to a single VAL service. The user credentials uniquely identifies the VAL user to the identity management server.

NOTE:	The specific security and authentication mechanisms required in order to use the user ID is specified in 3GPP TS 33.434 [29].

Editor's note: The naming and definition of the identities in subclause 7 may require further study (e.g. renaming user identity to VAL user identity, and renaming VAL user identity to VAL service user identity).

## 7.2	VAL user identity (VAL user ID)

The VAL user ID is a unique identifier within the VAL service that represents the VAL user. For example, the VAL user ID may be a URI. The VAL user ID is used for authentication and authorization purposes for providing the VAL service towards the VAL user via the VAL UE. The VAL user ID also indicates the VAL service provider with whom the VAL user has a VAL service agreement. The VAL user may have VAL service agreement with several VAL service providers and thus will have obtained unique VAL user ID per VAL service provider. The VAL user ID can be used to access any SEAL service.

Based on the service agreement, VAL user ID may be mapped to a VAL UE ID.

## 7.3	VAL UE identity (VAL UE ID)

The VAL UE ID is a unique identifier within the VAL service that represents the VAL UE. For example, the VAL UE ID for V2X service is mapped to the StationID as specified in ETSI TS 102 894-2 [25]. The VAL UE ID is used to address the VAL UE in order to send VAL messages or to access SEAL services.

Based on the service agreement, GPSI (as specified in 3GPP TS 23.003 [38]) may be used as VAL UE identity.

## 7.4	VAL service identity (VAL service ID)

The VAL service ID is a unique identifier that represents the VAL service. A VAL server provides a list of VAL services towards the VAL users or VAL UE. Each VAL service is uniquely identified by a VAL service ID, which is an identifier of the VAL application providing that VAL service. The VAL service ID can be used for policy mapping, QoS handling for VAL communication and VAL message distribution. For example, an identifier of a V2X service, e.g. ITS-AID or PSID specified in ETSI TS 102 965 [26] and ISO TS 17419 [27], can be used as a V2X service ID.

## 7.5	VAL group identity (VAL group ID)

The VAL group ID is a unique identifier within the VAL service that represents a set of VAL users or VAL UE according to the VAL service. The set of VAL users may belong to the same or different VAL service providers. It indicates the VAL application server where the group is defined.

## 7.6	VAL system identity (VAL system ID)

The VAL system ID is a globally unique identifier representing a VAL system.

## 7.7	VAL Stream ID

A VAL Stream ID is an identity used by the VAL server to identify a VAL stream.

# 8	Application of functional model to deployments

## 8.1	General

This clause describes deployments of the functional model specified in clause 6.

## 8.2	Deployment of SEAL server(s)

The SEAL server(s) may be deployed either in the PLMN operator domain or deployed in the VAL service provider domain. The SEAL server(s) connects with the 3GPP network system in one or more PLMN operator domain. The SEAL server(s) may be supporting multiple VAL servers.

### 8.2.1	SEAL server(s) deployment in PLMN operator domain

Figure 8.2.1-1 illustrates deployment of the SEAL server(s) in a single PLMN operator domain and the VAL server(s) in the VAL service provider domain.

Figure 8.2.1-1: SEAL server(s) deployed in a single PLMN operator domain

Figure 8.2.1-2 illustrates the deployment of SEAL server(s) in multiple PLMN operator domain and provides SEAL services to the VAL server(s) deployed in the VAL service provider domain. SEAL servers deployed in multiple PLMN operator domain are not interconnected.

Figure 8.2.1-2: SEAL server(s) deployed in multiple PLMN operator domain without interconnection between SEAL servers

Figure 8.2.1-3 illustrates the deployment of SEAL servers in multiple PLMN operator domain and provides SEAL services to the VAL server(s) deployed in the VAL service provider domain. SEAL servers deployed in multiple PLMN operator domain are interconnected.

Figure 8.2.1-3: SEAL server(s) deployed in multiple PLMN operator domain with interconnection between SEAL servers

Figure 8.2.1-4 illustrates the deployment of SEAL servers in a single PLMN operator domain and provides SEAL services to the VAL server(s) deployed in the VAL service provider domain. SEAL servers deployed in a single PLMN operator domain are interconnected.

Figure 8.2.1-4: SEAL server(s) deployed in a single PLMN operator domain with interconnection between SEAL servers

### 8.2.2	SEAL server(s) deployment in VAL service provider domain

Figure 8.2.2-1 illustrates deployment of the SEAL server(s) and the VAL server(s) in VAL service provider domain.

Figure 8.2.2-1: Deployment of SEAL server(s) with connections to 3GPP network system in a single PLMN operator domain

Figure 8.2.2-2 illustrates deployment of the SEAL server(s) which connects to the 3GPP network system in multiple PLMN operator domain.

Figure 8.2.2-2: Deployment of SEAL server(s) with connections to 3GPP network system in multiple PLMN operator domains

Figure 8.2.2-3 illustrates the deployment of multiple SEAL servers in the VAL service provider domain where SEAL server 1 and SEAL server 2 connect with 3GPP network system of PLMN operator domain 1 and PLMN operator domain 2 respectively. The SEAL servers interconnect via SEAL-E and support the VAL service provider domain applications for the VAL UEs connected via both the PLMN operator domains.

Figure 8.2.2-3: Distributed deployment of SEAL servers in VAL service provider domain

### 8.2.3	SEAL server(s) deployment outside of VAL service provider domain and PLMN operator domain

Figure 8.2.3-1 illustrates deployment of the SEAL server(s) outside of both the VAL service provider domain and PLMN operator domain i.e. in SEAL provider domain.

Figure 8.2.3-1: Deployment of SEAL server(s) outside of VAL service domain and PLMN operator domain

# 9	Location management

## 9.1	General

The location management is a SEAL service that offers the location management related capabilities to one or more vertical applications.

## 9.2	Functional model for location management

### 9.2.1	General

The functional model for the location management is based on the generic functional model specified in clause 6. It is organized into functional entities to describe a functional architecture which addresses the support for location management aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

### 9.2.2	On-network functional model description

Figure 9.2.2-1 illustrates the generic on-network functional model for location management.

Figure 9.2.2-1: On-network functional model for location management

The location management client communicates with the location management server over the LM-UU reference point. The location management client provides the support for location management functions to the VAL client(s) over LM-C reference point. The VAL server(s) communicate with the location management server over the LM-S reference point.

The location management server communicates with the SCEF via T8 reference point to obtain location information from the underlying 3GPP network system. The location management server obtains location information from the NEF via N33 reference point by mechanism defined in clause 5.2.6.2 of 3GPP TS 23.502 [11].

NOTE:	Location information from LCS of 4G system is not exposed by SCEF.

Editor's note: Use of Le interface for acquiring location information provided by PLMN is FFS.

### 9.2.3	Off-network functional model description

Figure 9.2.3-1 illustrates the off-network functional model for location management.

Figure 9.2.3-1: Off-network functional model for location management

The location management client of the UE1 communicates with the location management client of the UE2 over the LM-PC5 reference point.

### 9.2.4	Functional entities description

#### 9.2.4.1	General

The functional entities for location management SEAL service are described in the following subclauses.

#### 9.2.4.2	Location management client

The location management client functional entity acts as the application client for location management functions. It interacts with the location management server. The location management client also supports interactions with the corresponding location management client between the two UEs.

#### 9.2.4.3	Location management server

The location management server is a functional entity that receives and stores user location information and provides user location information to the vertical application server. The location management server may also acquire location information provided by PLMN operator via T8 reference point. The location management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The location management server also supports interactions with the corresponding location management server in distributed SEAL deployments.

NOTE:	The accuracy of location information acquired from 4G system via T8 reference point is not higher than at cell level (ECGI) for E-UTRAN.

### 9.2.5	Reference points description

#### 9.2.5.1	General

The reference points for the functional model for location management are described in the following subclauses.

#### 9.2.5.2	LM-UU

The interactions related to location management functions between the location management client and the location management server are supported by LM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

LM-UU reference point provides a means for the location management server to receive location information report from the location management client. The LM-UU reference point shall use SIP-1 and SIP-2 reference points for subscription/notification related signalling. And for transport and routing of location management related signalling LM-UU reference point uses the HTTP-1 and HTTP-2 signalling control plane reference points.

#### 9.2.5.3	LM-PC5

The interactions related to location management functions between the location management clients located in different VAL UEs are supported by LM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

#### 9.2.5.4	LM-C

The interactions related to location management functions between the VAL client(s) and the location management client within a VAL UE are supported by LM-C reference point.

#### 9.2.5.5	LM-S

The interactions related to location management functions between the VAL server(s) and the location management server are supported by LM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

LM-S reference point is used by the VAL server to request and receive location information from location management server. The LM-S reference point shall use SIP-1 and SIP-2 reference points for subscription/notification related signalling. And for transport and routing of location management related signalling LM-S reference point uses the HTTP-1 and HTTP-2 signalling control plane reference points.

#### 9.2.5.6	LM-E

The interactions related to location management functions between the location management servers in a distributed deployment are supported by LM-E reference point.

Editor's Note:	The functions enabled over LM-E reference point is FFS.

#### 9.2.5.7	T8

The reference point T8 supports the interactions between the location management server and the SCEF and is specified in 3GPP TS 23.682 [13]. The functions related to location management of T8 are supported by the location management server.

## 9.3	Procedures and information flows for Location management (on-network)

### 9.3.1	General

Location information of VAL service user shall be provided by the location management client to the location management server. The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided.

Editor's note:	The security and privacy aspects related to VAL server obtaining UE's location information are to be addressed by SA3.

### 9.3.2	Information flows for location information

#### 9.3.2.0	Location reporting configuration request

Table 9.3.2.0-1 describes the information flow from the location management client to the location management server for requesting the location reporting configuration.

Table 9.3.2.0-1: Location reporting configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or identity of the VAL UE. |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested. |

#### 9.3.2.1	Location reporting configuration response

Table 9.3.2.1-1 describes the information flow from the location management server to the location management client for the location reporting configuration. This information flow may be sent individually addressed or group addressed on unicast or multicast.

Table 9.3.2.1-1: Location reporting configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user or VAL group to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | O (see NOTE) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE) | Identifies when the location management client will send the location report |
| Minimum time between consecutive reports | O (see NOTE) | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| NOTE: If none of the information element is present, this represents a cancellation for location reporting. |  |  |

#### 9.3.2.2	Location information report

Table 9.3.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting or from the location management server to the requesting location management client or VAL server to report location information.

Table 9.3.2.2-1: Location information report

| Information element | Status | Description |
| --- | --- | --- |
| Set of identities | M | Set of identities of the reporting VAL users or VAL UEs |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information | M | Location information |
| Timestamp | O | Timestamp of the location report |

#### 9.3.2.3	Location information request

Table 9.3.2.3-1 describes the information flow from the VAL server to the location management server and from the location management server to the location management client for requesting an immediate location information report.

Table 9.3.2.3-1: Location information request

| Information element | Status | Description |
| --- | --- | --- |
| Identity list | M | List of VAL users or VAL UEs whose location information is requested |
| VAL service ID | O | Identity of the VAL service for which the location information is requested. |

Editor's Note: It's FFS whether and how the LMS need to identify the VAL service when the VAL UE ID is used for location request.

Editor's Note: It's FFS the security aspects for LM-Uu and LM-S in relation to the VAL service ID that need to be coordinated with SA3.

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE |
| Minimum time between consecutive reports | O see (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

#### 9.3.2.5	Location information subscription request

Table 9.3.2.5-1 describes the information flow from the VAL server or location management client to the location management server for location information subscription request.

Table 9.3.2.5-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Identities list | M | List of VAL users or VAL UEs whose location information is requested. |
| VAL service ID | O | Identity of the VAL service for which the location information is subscribed. |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |

#### 9.3.2.6	Location information subscription response

Table 9.3.2.6-1 describes the information flow from the location management server to the VAL server or location management client for location information subscription response.

Table 9.3.2.6-1: Location information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL user or VAL UE |
| Subscription status | M | It indicates the subscription result |

#### 9.3.2.7	Location information notification

Table 9.3.2.7-1 describes the information flow from the location management server to the VAL server or the location management client.

Table 9.3.2.7-1: Location information notification

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of the VAL users or VAL UEs whose location information needs to be notified |
| Identity (see NOTE 2) | O | Identity of the VAL user or VAL UE subscribed to location of another VAL user or VAL UE (see NOTE 1) |
| Subscription ID (see NOTE 2) | O | Subscription identity related to VAL server subscription with Location management server for location information notification. |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information | M | Location information |
| Timestamp | O | Timestamp of the location report |
| NOTE 1: This is only used for location management server sends location information notification to the VAL user or VAL UE who has subscribed the location. NOTE 2: Either Identity or Subscription ID shall be included. |  |  |

#### 9.3.2.8	Location reporting configuration cancel request

Table 9.3.2.8-1 describes the location reporting configuration cancel request information flow from the location management client or VAL server to the location management server.

Table 9.3.2.8-1: Location reporting configuration cancel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting configuration is requested to be canceled. |

#### 9.3.2.9	Get UE(s) information request

Table 9.3.2.9-1 describes the information flow for a VAL server to get UE(s) information at the LM server.

Table 9.3.2.9-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| Location information | M | Location information around which the UE(s) information is requested. |
| Application defined proximity range information | M | Description of the range information over which the UE(s) information is required. |

#### 9.3.2.10	Get UE(s) information response

Table 9.3.2.10-1 describes the information flow for a LM server to respond with UE(s) information to the VAL server.

Table 9.3.2.10-1: Get UE(s) information response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAE server in response to subscription request indicating success or failure |
| List of UEs information (see NOTE) | O | The information of the UEs which were detected in the application defined proximity range. The list can be empty. |
| >UE ID | M | The identifier of UE |
| >Location information | M | Location information of UE within the application defined proximity range |
| NOTE: This IE shall be included when the Result indicates success. |  |  |

#### 9.3.2.11	Monitor Location Subscription Request

Table 9.3.2.11-1 describes the information flow from the VAL server to the LM server for monitoring the location deviation of the VAL UE(s).

Table 9.3.2.11-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identifier of the VAL users or VAL UE whose location monitoring is requested to be monitored in a given location. |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE's location adherence. |
| Notify Interval | M | Periodic time interval in which the LM server needs to notify the VAL UE's location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL UE's location. |
| Timeout | O | A timeout period when subscription response is not received. |

#### 9.3.2.12	Monitor Location Subscription Response

Table 9.3.2.12-1 describes the information flow from LM server to the VAL server for Monitor Location Subscription Response.

Table 9.3.2.12-1: Monitor Location Subscription Response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |

#### 9.3.2.13	Notify Location Monitoring Event

Table 9.3.2.13-1 describes the information flow from LM server to the VAL server for notification of location monitoring events.

Table 9.3.2.13-1: Notify Location Monitoring Event message

| Information element | Status | Description |
| --- | --- | --- |
| Event | M | Information of the event to be reported. The event shall be one of the following: "Notify_Mismatch_Location" – When the location information of the VAL UE, from the location management client and the core network do not match. "Notify_Absence" – When the VAL UE's current location is deviating from the VAL server's area of interest information. "Notify_Presence" – When the VAL UE's current location is within the VAL server's area of interest information. |
| Identity | M | Identifier of the VAL UE whose location information is reported. |
| Location | M | Current location of the VAL UE. |

Editor's note: It is FFS how the VAL Server's area of interest information is related with the Area of Interest within 3GPP system defined by SA2.

#### 9.3.2.14	Location area monitoring subscription request

Table 9.3.2.14-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription request.

Table 9.3.2.14-1: Location information monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a reference UE along with the application defined proximity range from the reference UE. |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled) |

#### 9.3.2.15	Location area monitoring subscription response

Table 9.3.2.15-1 describes the information flow from the location management server to the VAL server for location area monitoring subscription response.

Table 9.3.2.15-1: Location area monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Subscription identity | O (see NOTE) | If subscription is successful, identifies the subscription |
| NOTE: This IE shall be present if subscription status is set to success. |  |  |

#### 9.3.2.16	Location area monitoring notification

Table 9.3.2.16-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.16-1: Location area monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UEs currently present (see NOTE) | O | List of the identities of all VAL UEs who are currently present in the given location area |
| UEs moved in/out (see NOTE) | O | List of UEs either moved in to the location area or moved out of the location area |
| >> UEs moved in | O | List of the identities of the VAL UEs who moved in to the given location area since previous notification. |
| >> UEs moved out | O | List of the identities of the VAL UEs who moved out of the given location area since previous notification. |
| Triggering event | O | Identity of the event that triggered the sending of the notification |
| NOTE: Any one of these shall be present. |  |  |

#### 9.3.2.17	Location area monitoring subscription modify request

Table 9.3.2.17-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription modify request.

Table 9.3.2.17-1: Location information monitoring subscription modify request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Subscription identity | M | Identifies the subscription |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a reference UE along with the application defined proximity range from the reference UE. |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled) |

#### 9.3.2.18	Location area monitoring subscription modify response

Table 9.3.2.18-1 describes the information flow from the location management server to the VAL server for location area monitoring subscription modify response.

Table 9.3.2.18-1: Location area monitoring subscription modify response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription modification status | M | It indicates the request result |

#### 9.3.2.19	Location area monitoring unsubscribe request

Table 9.3.2.19-1 describes the information flow from the VAL server to the location management server for location area monitoring unsubscribe request.

Table 9.3.2.19-1: Location information monitoring unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Subscription identity | M | Identifies the subscription |

#### 9.3.2.20	Location area monitoring unsubscribe response

Table 9.3.2.20-1 describes the information flow from the location management server to the VAL server for location area monitoring unsubscribe response.

Table 9.3.2.20-1: Location area monitoring unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the request result |

### 9.3.3	Event-triggered location reporting procedure

#### 9.3.3.1	General

The location management server provides location reporting configuration to the location management clients, indicating what information the location management server expects and what events will trigger the sending of this information to the location management server. The decision to report location information can be triggered at the location management client by different conditions, e.g., the reception of the location reporting configuration, initial registration, distance travelled, elapsed time, cell change, MBMS SAI change, MBMS session change, leaving a specific MBMS bearer service area, tracking area change, PLMN change, call initiation, or other types of events such as emergency. The location report can include information described as ECGI, MBMS SAIs, geographic coordinates and other location information.

#### 9.3.3.2	Fetching location reporting configuration

Figure 9.3.3.2-1 illustrates the procedure for fetching location reporting configuration.

Pre-condition:

-	If multicast delivery mode is used, the MBMS bearer being used is activated by the location management server.

-	The location management client is aware that the location reporting configuration is available at the location management server.

Figure 9.3.3.2-1: Fetching location reporting configuration procedure

1.	The location management client sends location reporting configuration request message to the location management server.

2.	The location management server sends location reporting configuration message to the location management client(s) containing the initial location reporting event triggers configuration (or a subsequent update) , e.g. minimum time between consecutive reports, SAI changes, or ECGI changes for reporting the location of the VAL UE. This message can be sent over a unicast bearer to a specific location management client or as a group message over an MBMS bearer to update the location reporting configuration for multiple location management clients at the same time.

NOTE 1:	The location reporting configuration information can be made part of the user profile, in which case the sending of the message is not necessary.

NOTE 2:	Different location management clients may be given different location reporting criteria.

3.	The location management client stores or updates the location reporting event triggers configuration. A location reporting event occurs, triggering step 3.

#### 9.3.3.3	Location reporting

Figure 9.3.3.3-1 illustrates the procedure for location reporting.

Figure 9.3.3.3-1: Location reporting procedure

1.	The location management client sends a location information report to the location management server, containing location information identified by the location management server and available to the location management client.

2.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

### 9.3.4	On-demand location reporting procedure

The location management server can request UE location information at any time by sending a location information request to the location management client, which may trigger location management client to immediately send the location report.

Figure 9.3.4-1: On-demand location information reporting procedure

1.	Based on configurations such as periodical location information timer, or location information request from other entities (e.g., another location management client, VAL server), location management server initiates the immediately request location information from the location management client.

2.	The location management server sends a location information request to the location management client.

3.	VAL user or VAL UE is notified and asked about the permission to share its location. VAL user can accept or deny the request

4.	The location management client immediately responds to the location management server with a report containing location information identified by the location management server and available to the location management client.

5.	Upon receiving the report, the location management server updates location of the reporting location management client. If the location management server does not have location information of the reporting location management client before, then just stores the reporting location information for that location management client.

### 9.3.5	Client-triggered or VAL server-triggered location reporting procedure

Figure 9.3.5-1 illustrates the high level procedure of client-triggered or VAL server-triggered location reporting.

Figure 9.3.5-1: Client-triggered location reporting procedure

1.	Location management client 2 (authorized VAL user or VAL UE) or VAL server sends a location reporting trigger to the location management server to activate a location reporting procedure for obtaining the location information of location management client 1.

NOTE:	Step 1 can be performed when Location management client 2 or VAL server require to update the location reporting trigger.

2.	Location management server checks whether location management client 2 or VAL server is authorized to send a location reporting trigger. Depending on the information specified by the location reporting trigger, location management server initiates an on-demand location reporting procedure or an event-triggered location reporting procedure for the location of location management client 1.

3.	Once the location information of the location management client 1 is available in the location management server by the on-demand location reporting procedure, a location information report is sent to the location management client 2 or VAL server.

### 9.3.6	Location reporting triggers configuration cancel

Figure 9.3.6-1 illustrates the procedure used for cancelling the location reporting triggers configuration at the target Location management client.

Pre-conditions:

1.	The location management server has subscribed the location management client 2 location with the location reporting event triggers.

2.	If multicast delivery mode is used, the MBMS bearer being used is activated by the location management server.

Figure 9.3.6-1: Location reporting triggers configuration cancel

1.	The location management client 1 (authorized VAL user or VAL UE) or VAL server sends a location reporting configuration cancel request to the location management server (1a). The location management server sends the location reporting configuration cancel request to the location management client 2 to stop receiving the UE location information (1b). This message can be sent via unicast or multicast.

NOTE:	Step 1b can be initiated without step 1a.

2.	The location management client invalidates the location reporting triggers configuration and no longer reports its location to the location management server.

3.	The location management client 2 sends the location reporting configuration cancel response to the location management server (3a) as an acknowledgement. The location management server sends the location reporting configuration cancel response to the location management client 1 (3b) as an acknowledgement.

### 9.3.7	Location information subscription procedure

Figure 9.3.7-1 illustrates the high level procedure of location information subscription request. The same procedure can be applied for location management client and other entities that would like to subscribe to VAL user or VAL UE location information. This procedure is also used for intiating tracking a UE's location.

Figure 9.3.7-1: Location information subscription request procedure

1.	The VAL server sends a location information subscription request to the location management server to subscribe location information of one or more VAL users or VAL UEs. The request may include an indication for supplementary location information.

2.	The location management server shall check if the VAL server is authorized to initiate the location information subscription request. Further, the location management server may initiate location reporting configuration with the location management client of the UE for immediate reporting.

3.	The location management server may optionally subscribe for UE location information from 3GPP core network for the UE. If the indication for supplementary location information is included in step 1, then UE location information is obtained from the 3GPP core network.

4.	The location management server determines the UE location information of the UE as received in steps 3 and 4.

5.	The location management server replies with a location information subscription response indicating the subscription status and if immediate reporting was requested, the location information of the VAL UE(s).

### 9.3.8	Event-trigger location information notification procedure

Figure 9.3.8-1 illustrates the high level procedure of event-trigger usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE. This procedure is also used for obtaining latest UE's location for tracking purpose.

Figure 9.3.8-1: Event-trigger usage of location information procedure

1.	The location management server receives the latest location information of the UE as per the location report procedure described in clause 9.3.3.3.

2.	The location management server may optionally receive the location information of the UE from 3GPP core network. If the indication for supplementary location information is included in the subscription, then UE location information is obtained from the 3GPP core network.

3.	Based on the configurations, e.g., subscription, periodical location information timer, location management server is triggered to report the latest user location information to VAL server. The location management server determines the location information of UE as received in steps 1 and 2, including the supplementary location information (if indicated).

4.	The location management server sends the location information report including the latest location information of one or more VAL users or VAL UEs to the VAL server or to the location management client that has previously configured.

5.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 5 can be skipped if not needed.

### 9.3.9	On-demand usage of location information procedure

The VAL server can request UE location information at any time by sending a location information request to the location management server, which may trigger location management server to immediately send the location report.

Figure 9.3.9-1 illustrates the high level procedure of on demand usage of location information. The same procedure can be applied for location management client and other entities that would like to subscribe to location information of VAL user or VAL UE.

Figure 9.3.9-1: On-demand usage of location information procedure

1.	VAL server sends a location information request to the location management server.

2.	The location management server acquires the latest location of the UEs being requested, by triggering an on-demand location report procedure as described in subclause 9.3.4, or from PLMN operator.

3.	Then, location management server immediately sends the location information report including the latest location information acquired of one or more VAL users or VAL UEs.

4.	VAL server may further share this location information to a group or to another VAL user or VAL UE.

NOTE:	For other entities, the step 3 can be skipped if not needed.

### 9.3.10	Obtaining UE(s) information at a location

Figure 9.3.10-1 describes the procedure for obtaining UE(s) information at a location.

Pre-condition:

-	The VAL server has a jurisdiction over a geographical area for which the location management server is configured to operate.

-	The UE(s) in the geographical area have provided its location information to the location management server.

Figure 9.3.10-1: Obtaining UE(s) information at a location

1.	The VAL server sends get UE information request to the location management server. The request contains a location information and application defined proximity range.

2.	The location management server determines the UE(s) whose location are within the application defined proximity range of the location information provided in step 1.

3.	The location management server sends get UE information response to the VAL server with a list of UE(s) and its corresponding location information as determined in step 1.

### 9.3.11	Monitoring Location Deviation

#### 9.3.11.1	General

The VAL server requests the Location Management Server to monitor the location of the VAL UE in relation to an area of interest. The LMS fetches the VAL UE's location information periodically from 3GPP core network as specified in 3GPP TS 23.502 [11] and also, using the Location Information procedures specified in clause 9.3.7 and clause 9.3.10. With the periodic location information of the UE from 3GPP core network and SEAL procedures, the LMS server evaluates the current location of the VAL UE in relation to the area of interest configured by the VAL server. If subscribed, the VAL server is notified by the LMS server when the VAL UE relationship (e.g. inside or outside) to the area of interest changes along with current location information of the VAL UE.

#### 9.3.11.2	Monitoring Location Deviation procedure

Figure 9.3.11.2-1 describes the procedure for monitoring the VAL UE's location in a given area of interest.

Pre-condition:

-	The LMS server authorized to consume the 3GPP core network service (Monitoring events as specified in 3GPP TS 23.502 [11]).

Figure 9.3.11.2-1: Monitoring VAL UE's location at a given location

1.	The VAL server sends Monitor Location Subscription Request to LMS server including VAL UE Identifier, predetermined area of interest information, notification interval and notification URI where the VAL server intends to receive the notifications from LMS server regarding VAL UE's presence in a given area.

-	"Area of interest" is the location information, which the VAL server wishes to monitor the VAL UE's location adherence. This parameter can include an area of interest information and other relevant parameters.

-	"Notify_Interval" represents the periodic interval in which the LMS server needs to notify VAL UE's location information to the VAL server. When the VAL UE moves away from the " Area of interest", then the LMS server ignores the "Notify_Interval" and sends the location notification to the VAL server immediately.

2.	LMS processes the Area of interest information in the request, and then subscribes to UE location monitoring as specified in 3GPP TS 23.502 [11] with appropriate parameters mapping. Based on the subscription, the LMS receives the VAL UE location information periodically from the 3GPP core network.

Editor's note: How the LMS's processing of Area of Interest information and the subscription of UE location monitoring based on Area of Interest are related with procedures in 3GPP TS 23.502 and 3GPP TS 23.256 are FFS.

3.	LMS shall use the Location information procedures as specified in clause 9.3.7 and clause 9.3.10, to periodically obtain the VAL UE location information. Based on the geographic information from the VAL server, the LMS server may determine to additionally include the positioning methods in SEAL LMS procedures to obtain location information.

4.	LMS server, after successful subscription according to steps 2 and 3, sends Monitor Location Subscription response, indicating that the LMS server accepts VAL server's request and will monitor the location of the VAL UE to verify if the VAL UE is in the area of interest.

5a and 5b. LMS server processes the location information received from SEAL Location Information procedures and the core network, and validates the information. If the location information is matching, then the LMS shall check if the VAL UE's current location is within the area of interest received in step 1. The LMS will continue with step 6, step 7 and step 8 as applicable.

6.	If the location information received from Location management client and the core network do not match, then the LMS server shall consider the VAL UE as outside from its specified area of interest and shall notify ("Notify Mismatch Location" message) the VAL server of the same, including VAL UE ID and the location information from LMS and the core network in the notification message.

7.	If the VAL UE's current location is from Location management client and the core network matches, and not in the area of interest received from VAL server in Monitor Location Subscription Request message, then the LMS considers the VAL UE as outside from its specified area of interest and shall notify the VAL server that the VAL UE's current location is outside of area of interest and VAL UE ID in "Notify Absence" message.

8.	When the VAL UE's current location is in area of interest, then the LMS shall notify ("Notify Presence" message) the VAL server periodically, according to the "Notify_Interval" value in "Monitor Location Subscription Request" message, indicating the VAL server that the VAL UE is within the area of interest, along with VAL UE's current location information.

### 9.3.12	Location area monitoring information procedure

#### 9.3.12.1	Location area monitoring subscribe procedure

Figure 9.3.12.1-1 illustrates the high level procedure of location area monitoring subscription request. The same procedure can be applied for location management client and other SEAL servers that would like to subscribe to the list of UEs moving in or moving out of the specific location area. The subscribe request can be for a reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.12.1-1: Location area monitoring subscription procedure

1.	The VAL server sends a location area monitoring subscription request to the location management server to subscribe to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.14-1. The location information criteria may include the geographic location information where the UEs moving in or moving out to be monitored, or it may include reference UE information where in the UEs moving in or moving out of given application defined proximity range from the reference UE (target UE) to be monitored. The reference UE information may include VAL UE ID.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription request.

3.	The location management server replies with a location area monitoring subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.15-1.

#### 9.3.12.2	Location area monitoring subscribe modify procedure

Figure 9.3.12.2-1 illustrates the high level procedure of location area monitoring subscribe modify request. The same procedure can be applied for location management client and other SEAL servers that would like to modify the subscription to the list of UEs moving in or moving out of the specific location area. The subscribe modify request can be for a reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.12.2-1: Location area monitoring subscription modify procedure

1.	The VAL server sends a location area monitoring subscription modify request to the location management server to modify the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.17-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription modification request.

3.	The location management server replies with a location area monitoring subscription modify response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.18-1.

#### 9.3.12.3	Location area monitoring unsubscribe procedure

Figure 9.3.12.3-1 illustrates the high level procedure of location area monitoring unsubscribe request. The same procedure can be applied for location management client and other SEAL servers that would like to unsubscribe to the list of UEs moving in or moving out of the specific location area. The unsubscribe request can be for a reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.12.3-1: Location area monitoring unsubscribe procedure

1.	The VAL server sends a location area monitoring unsubscrbe request to the location management server to unsubscribe to the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.19-1.

2.	The location management server replies with a location area monitoring unsubscrbe response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.20-1.

#### 9.3.12.4	Location area monitoring notification procedure

Figure 9.3.12.4-1 illustrates the high level procedure of location area monitoring notification. The same procedure can be applied for location management client and other SEAL servers who have subscribe to the list of the UEs moving in or moving out of the specific location area.

Figure 9.3.12.4-1: Location are monitoring notification procedure

1.	One of the events occurs at the location management server as specified in the subscribe request. The location management server identifies the UEs which are moved into the area or moved out of the area based on their location data and time stamp of the location. The LMS may report the list of all UEs in the given location or UEs moved in and moved out.

2.	The location management server sends a location area monitoring notification to the VAL server. In the notification message, the location management server includes the information as specified in Table 9.3.2.16-1.

## 9.4	SEAL APIs for location management

### 9.4.1	General

Table 9.4.1-1 illustrates the SEAL APIs for location management.

Table 9.4.1-1: List of SEAL APIs for location management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_LocationReporting | Create_Trigger_Location_Reporting Update_Trigger_Location_Reporting Cancel_Trigger_Location_Reporting | VAL server | Request /Response |
| SS_LocationInfoEvent | Subscribe_Location_Info | VAL server | Subscribe/Notify |
|  | Notifiy_Location_Info | VAL server |  |
| SS_LocationInfoRetrieval | Obtain_Location_Info | VAL server | Request /Response |
| SS_LocationAreaInfoRetrieval | Obtain_UEs_Info | VAL server | Request/Response |
| SS_LocationMonitoring | Subscribe_Location_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Location_Monitoring_Events | VAL server |  |

### 9.4.2	SS_LocationReporting API

#### 9.4.2.1	General

API description: This API enables the VAL server to trigger reporting of location information to the location management server over LM-S.

#### 9.4.2.2	Create_Trigger_Location_Reporting operation

API operation name: Create_Trigger_Location_Reporting

Description: Creates the trigger to report location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.4

Outputs: Refer subclause 9.3.2.4

See subclause 9.3.5 for the details of usage of this API operation.

#### 9.4.2.3	Update_Trigger_Location_Reporting operation

API operation name: Update_Trigger_Location_Reporting

Description: Updates the trigger to report location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.4

Outputs: Refer subclause 9.3.2.4

See subclause 9.3.5 for the details of usage of this API operation.

#### 9.4.2.4	Cancel_Trigger_Location_Reporting operation

API operation name: Cancel_Trigger_Location_Reporting

Description: Cancels the trigger to report location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.8

Outputs: Refer subclause 9.3.2.8

See subclause 9.3.6 for the details of usage of this API operation.

### 9.4.3	SS_LocationInfoEvent API

#### 9.4.3.1	General

API description: This API enables the VAL server to subscribe and receive the UEs location information from the location management server over LM-S.

#### 9.4.3.2	Subscribe_Location_Info operation

API operation name: Subscribe_Location_Info

Description: Subscription to the location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.5

Outputs: Refer subclause 9.3.2.6

See subclause 9.3.7 for the details of usage of this API operation.

#### 9.4.3.3	Notify_Location_Info operation

API operation name: Notify_Location_Info

Description: Location information notification to the existing subscription.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.7

Outputs: Refer subclause 9.3.2.7

See subclause 9.3.8 for the details of usage of this API operation.

### 9.4.4	SS_LocationInfoRetrieval API

#### 9.4.4.1	General

API description: This API enables the VAL server to obtain UEs location information from the location management server over LM-S.

#### 9.4.4.2	Obtain_Location_Info operation

API operation name: Obtain_Location_Info

Description: Request UEs location information.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.3

Outputs: Refer subclause 9.3.2.2

See subclause 9.3.9 for the details of usage of this API operation.

### 9.4.5	SS_LocationAreaInfoRetrieval API

#### 9.4.5.1	General

API description: This API enables the VAL server to obtain UE(s) information in an application defined proximity range of a location from the location management server over LM-S.

#### 9.4.5.2	Obtain_UEs_Info operation

API operation name: Obtain_UEs_Info

Description: Request UEs information in an application defined proximity range of a location.

Known Consumers: VAL server.

Inputs: Refer subclause 9.3.2.9

Outputs: Refer subclause 9.3.2.10

See subclause 9.3.10 for the details of usage of this API operation.

### 9.4.6	SS_LocationMonitoring API

#### 9.4.6.1	General

API description: This API enables the VAL server to monitor the VAL UE's location in a given area of interest, from the location management server over LM-S.

#### 9.4.6.2	Subscribe_Location_Monitoring

API operation name: Subscribe_Location_Monitoring

Description: Subscription to location monitoring.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.11

Outputs: 9.3.2.12.

See subclause 9.3.11.2 for the details of usage of this API operation.

#### 9.4.6.3	Notify_Location_Monitoring_Events

API operation name: Notify_Location_Monitoring_Events

Description: Notifying the VAL server with the location monitoring events related to VAL UE.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.13

Outputs: None.

See subclause 9.3.11.2 for the details of usage of this API operation.

### 9.4.7	SS_LocationAreaMonitoring API

#### 9.4.7.1	General

API description: This API enables the VAL server to monitor the list of UEs moving in or moving out of the specific location area.

#### 9.4.7.2	Subscribe_Location_Area_Monitoring

API operation name: Subscribe_Location_Area_Monitoring

Description: Subscription to location area monitoring.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.14

Outputs: 9.3.2.15.

See subclause 9.3.12.1 for the details of usage of this API operation.

#### 9.4.7.3	Notify_Location_Area_Monitoring_Events

API operation name: Notify_Location_Area_Monitoring_Events

Description: Notifying the VAL server with the list of UEs moved in or moved out of the specific location area.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.16

Outputs: None.

See subclause 9.3.12.4 for the details of usage of this API operation.

#### 9.4.7.4	Update_Location_Area_Monitoring_Subscribe

API operation name: Update_Location_Area_Monitoring_Subscribe

Description: Updates subscription to location area monitoring.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.17

Outputs: See subclause 9.3.2.18

See subclause 9.3.12.2 for the details of usage of this API operation.

#### 9.4.7.5	Unsubscribe_Location_Area_Monitoring

API operation name: Unsubscribe_Location_Area_Monitoring

Description: Unsubscribing from location area monitoring.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.19

Outputs: See subclause 9.3.2.20

See subclause 9.3.12.3 for the details of usage of this API operation.

## 9.5	Procedures and information flows for Location management (Off-network)

### 9.5.1	General

Location information of VAL service user shall be provided by the location management client of one UE to the location management client of another UE. The location information reporting triggers are based on the location reporting configuration. Different type of location information can be provided e.g. retrieved from non-3GPP source.

NOTE:	VAL clients sharing location information directly at vertical enabler layer is outside the scope of this specification.

Editor's note:	Adding architecture requirement(s) for off-network location management is FFS.

Editor's note:	Charging aspects for off-network location management is FFS.

### 9.5.2	Information flows for off network location management

#### 9.5.2.1	Off-network location reporting trigger configuration

Table 9.5.2.1-1 describes the information flow from the location management client-1 to the location management client-2 for the off-network location reporting configuration.

Table 9.5.2.1-1: Off-network location reporting trigger configuration

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user to which the location reporting configuration is targeted or identity of the VAL UE. |
| Requested location information | M | Identifies what location information is requested |
| List of triggering criteria(s) | M | One or more triggering criteria that identifies when the location management client will send the location report. Each triggering criteria is identified by trigger-id. |
| Minimum time between consecutive reports | O | Defaults to 0 if absent otherwise indicates the time interval between consecutive reports |
| Life Time of the configuration | O | Time till when location report configurations are valid. |

#### 9.5.2.2	Off-network location reporting trigger configuration response

Table 9.5.2.2-1 describes the information flow from the location management client-2 to the location management client-1 for the off-network location reporting configuration response. The Off-network location reporting trigger configuration response acts as an acknowledgement to the location management client-1.

Table 9.5.2.2-1: Off-network location reporting trigger configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation |
| Cause | O | Provides reason for the failure. |

#### 9.5.2.3	Off-network location management ack

The Off-network location management ack message is sent from the message receiver location management client-2 to message originator location management client-1.

#### 9.5.2.4	Off-network location report

Table 9.5.2.4-1 describes the information flow from the location management client-2 to the location management client-1 for the off-network location report.

Table 9.5.2.4-1: Off-network location report

| Information element | Status | Description |
| --- | --- | --- |
| Triggering event | M | Identity of the event that triggered the sending of the report |
| Location Information | M | Location information shared by VAL client e.g. retrieved from non-3GPP source |
| Acknowledgement Required | O | If present, indicate the recipient of the message to acknowledge the message. |

#### 9.5.2.5	Off-network location reporting trigger cancel

Table 9.5.2.5-1 describes the information flow from the location management client-1 to the location management client-2 for the off-network location reporting trigger cancel.

Table 9.5.2.5-1: Off-network location reporting trigger cancels

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user to which the location reporting trigger cancel is targeted or identity of the VAL UE. |

#### 9.5.2.6	Off-network location reporting trigger cancel response

Table 9.5.2.6-1 describes the information flow from the location management client-2 to the location management client-1 for the off-network location reporting cancel response. The Off-network location reporting trigger cancel response acts as an acknowledgement to the location management client-1.

Table 9.5.2.6-1: Off-network location reporting trigger cancel response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation |

#### 9.5.2.7	Off-network location request

Table 9.5.2.7-1 describes the information flow from the location management client-1 to the location management client-2 for the off-network location request.

Table 9.5.2.7-1: Off-network location request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the VAL user to which the location request is targeted or identity of the VAL UE. |
| Requested location information | M | Identifies what location information is requested |

#### 9.5.2.8	Off-network location response

Table 9.5.2.8-1 describes the information flow from the location management client-2 to the location management client-1 for the off-network location response. The Off-network location response acts as an acknowledgement to the location management client-1.

Table 9.5.2.8-1: Off-network location response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure for the operation |
| Location Information | M | Location information shared by VAL client e.g. retrieved from non-3GPP source |

### 9.5.3	Event-triggered location reporting procedure

#### 9.5.3.1	Location reporting trigger configuration

Figure 9.5.3.1-1 illustrates the procedure for configuring location reporting triggers from the location management client-1 residing in UE-1 to the location management client-2 residing in UE-2.

Pre-condition:

-	The UE-1 and UE-2 are within PC5 communication range of each other, and aware of Layer-2 ID of each other.

-	The VAL service user in UE-1 is authorized to configure location reporting trigger to the UE-2.

-	The VAL service user in UE-1 requests to configure location reporting triggers to the UE-2.

Figure 9.5.3.1-1: Location reporting trigger configuration

1.	The location management client-1 in UE-1 sends off network location reporting trigger configuration message to the location management client-2 in UE-2 containing the initial location reporting event triggers configuration (or a subsequent update) for reporting the location of the VAL UE. The message includes information elements as specified in Table 9.5.2.1-1.

2.	The location management client-2 stores the location reporting configuration, and sends off network location reporting trigger configuration response to the location management client-1. The message includes information elements as specified in Table 9.5.2.2-1.

3.	Upon receiving the off network location reporting trigger configuration response message, the location management client-1 sends off-network location management ack messages. The message includes information elements as specified in clause 9.5.2.3.

#### 9.5.3.2	Location reporting

Figure 9.5.3.2-1 illustrates the procedure for sending off-network location report from the location management client-2 residing in UE-2 to the location management client-1 residing in UE-1.

Pre-condition:

-	The UE-1 and UE-2 are within PC5 communication range of each other, and aware of Layer-2 ID of each other.

-	The location management client-1 has previously configured off-network location reporting triggers to the location management client-2 as specified in clause 9.5.3.1.

Figure 9.5.3.2-1: Location reporting

1.	The location management client-2 is monitoring the location reporting triggers and one of the event is triggered.

2.	The location management client-2 sends the off-network location report message. The message includes information elements as specified in Table 9.5.2.4-1.

3.	Upon receiving the off network location report message, the location management client-1 sends the off-network location management ack message if requested in the received message. The message includes information elements as specified in clause 9.5.2.3.

#### 9.5.3.3	Location reporting trigger cancel

Figure 9.5.3.3-1 illustrates the procedure for sending off-network location reporting trigger cancel from the location management client-1 residing in UE-1 to the location management client-2 residing in UE-2.

Pre-condition:

-	The UE-1 and UE-2 are within PC5 communication range of each other, and aware of Layer-2 ID of each other.

-	The location management client-1 has previously configured location reporting triggers to the location management client-2 as specified in clause 9.5.3.1.

Figure 9.5.3.3-1: Location reporting trigger cancel

1.	The location management client-1 in UE-1 sends off network location reporting trigger cancel message to the location management client-2 in UE-2 to cancel the location reporting trigger configuration. The message includes information elements as specified in Table 9.5.2.5-1.

2.	The location management client-2 clears the location reporting configuration, and sends off network location reporting trigger cancel response to the location management client-1. The message includes information elements as specified in Table 9.5.2.6-1.

3.	Upon receiving the off network location reporting trigger configuration response message, the location management client-1 sends off-network location management ack message. The message includes information elements as specified in clause 9.5.2.3.

### 9.5.4	On-demand location reporting procedure

Figure 9.5.4-1 illustrates the procedure for on-demand location report from the location management client-1 residing in UE-1 to the location management client-2 residing in UE-2.

Pre-condition:

-	The UE-1 and UE-2 are within PC5 communication range of each other, and aware of Layer-2 ID of each other.

-	The VAL service user in UE-1 is authorized to request location report from the UE-2.

-	The VAL service user in UE-1 requests immediate location reporting to the UE-2.

Figure 9.5.4-1: Location reporting trigger cancel

1.	Based on configurations such as periodical location information timer the location management client-1 initiates the immediately request location information from the location management client-2. The location management client sends an off-network location request to the location management client-2. The message includes information elements as specified in Table 9.5.2.7-1.

2.	VAL user or VAL UE is notified and asked about the permission to share its location. VAL user can accept or deny the request

3.	The location management client-2 immediately responds to the location management client-1. If permission is received from the VAL user, the location management client-2 includes a report containing location information identified by the location management client-1 and available to the location management client-2. The message includes information elements as specified in Table 9.5.2.8-1.

4.	Upon receiving the off network location reporting trigger configuration response message, the location management client-1 sends off-network location management ack message. The message includes information elements as specified in clause 9.5.2.3

# 10	Group management

## 10.1	General

The group management is a SEAL service that offers the group management related capabilities to one or more vertical applications.

## 10.2	Functional model for group management

### 10.2.1	General

The functional model for the group management is based on the generic functional model specified in clause 6. It is organized into functional entities to describe a functional architecture which addresses the support for group management aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

### 10.2.2	On-network functional model description

Figure 10.2.2-1 illustrates the generic on-network functional model for group management.

Figure 10.2.2-1: On-network functional model for group management

The group management client communicates with the group management server over the GM-UU reference point. The group management client provides the support for group management functions to the VAL client(s) over GM-C reference point. The VAL server(s) communicate with the group management server over the GM-S reference point.

The group management server interacts with the NEF of the underlying 3GPP network system via N33 reference point to perform group management procedures for 5G Virtual Network (5GVN) groups.

### 10.2.3	Off-network functional model description

Figure 10.2.3-1 illustrates the off-network functional model for group management.

Figure 10.2.3-1: Off-network functional model for group management

The group management client of the UE1 communicates with the group management client of the UE2 over the GM-PC5 reference point.

### 10.2.4	Functional entities description

#### 10.2.4.1	General

The functional entities for group management SEAL service are described in the following subclauses.

#### 10.2.4.2	Group management client

The group management client functional entity acts as the application client for management of groups. A VAL system maintains groups corresponding to one or more vertical applications. The group management client interacts with the group management server. The group management client also supports interactions with the corresponding group management client between the two UEs.

The group management client functional entity is supported by the signalling user agent and HTTP client functional entities of the signalling control plane.

#### 10.2.4.3	Group management server

The group management server functional entity provides for management of groups supported within the vertical application layer. The group management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The group management server also supports interactions with the corresponding group management server in distributed SEAL deployments.

The group management server functional entity is supported by the SIP AS and HTTP server functional entities of the signalling control plane.

All the group management clients supporting users belonging to a single group are required to use the same group management server for that group. A group management client supporting a user involved in multiple groups can have relationships with multiple group management servers.

### 10.2.5	Reference points description

#### 10.2.5.1	General

The reference points for the functional model for group management are described in the following subclauses.

#### 10.2.5.2	GM-UU

The interactions related to group management functions between the group management client and the group management server are supported by GM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

GM-UU reference point is used for VAL service signalling for VAL service data management of the VAL service. The GM-UU reference point supports:

-	Configuration of group related data at the group management client by the group management server; and

-	Configuration of group related data at the group management server by the group management client.

The GM-UU reference point uses the HTTP-1/HTTP-2 reference points for transport and routing of group management related signalling. The GM-UU reference point uses the SIP-1/SIP-2 reference points for subscription/notification related signalling.

#### 10.2.5.3	GM-PC5

The interactions related to group management functions between the group management clients located in different VAL UEs are supported by GM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

#### 10.2.5.4	GM-C

The interactions related to group management functions between the VAL client(s) and the group management client within a VAL UE are supported by GM-C reference point.

#### 10.2.5.5	GM-S

The interactions related to group management functions between the VAL server(s) and the group management server are supported by GM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

GM-S reference point supports the VAL server to obtain group information corresponding to the VAL service. The GM-S reference point uses HTTP-1/HTTP-2 reference points for transport and routing of group management related signalling. The GM-S reference point uses SIP-2 reference point for subscription/notification related signalling.

#### 10.2.5.6	GM-E

The interactions related to group management functions between the group management servers in a distributed deployment are supported by GM-E reference point.

Editor's Note:	The functions enabled over GM-E reference point is FFS.

#### 10.2.5.7	N33

For group management procedures pertaining to a 5GVN group the group management server interacts with the NEF of the underlying 3GPP network using the dynamic 5G Virtual Network group management procedures exposed by the NEF via the N33 reference point, as specified in TS 23.501 [10] and in TS 23.502 [11].

## 10.3	Procedures and information flows for group management

### 10.3.1	General

Group management procedures apply to on-network VAL service only.

Group creation provides a dedicated VAL group to individual VAL users to enable the required communication for one or multiple VAL services. This includes the normal group creation by administrators or by authorized user/UE. The group management server supports the external group identifier within the VAL group document to enable the SEAL servers to consume the NEF services for the member UEs of the VAL group.

NOTE:	If an authorized VAL user/UE wants to participate in a new group created by the authorized VAL user/UE, then the authorized VAL user/UE needs to have been included in the new group as a member.

### 10.3.2	Information flows for group management

#### 10.3.2.1	Group creation request

Table 10.3.2.1-1 describes the information flow group creation request from the group management client to the group management server.

Table 10.3.2.1-1: Group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| Identity list | M | List of VAL user IDs or VAL UE IDs that are part of the group to be created corresponding to the list of the configured services |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.2	Group creation response

Table 10.3.2.2-1 describes the information flow group creation response from the group management server to the group management client.

Table 10.3.2.2-1: Group creation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | VAL group ID of the group |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of VAL group ID information element has no meaning. |  |  |

#### 10.3.2.3	Group creation notification

Table 10.3.2.3-1 describes the information flow group creation notification from the group management server to the VAL server(s) and the group management clients.

NOTE:	When group is configured for multiple VAL services, the group creation notification message is sent from the group management server to the VAL servers configured for the group.

Table 10.3.2.3-1: Group creation notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID that was created based on the VAL user ID list and the VAL services enabled on them |
| Identity list | M | List of VAL user IDs or VAL UE IDs that are part of the created group |
| VAL service specific information (see NOTE) | O | Placeholder for VAL service specific information |
| NOTE: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.4	Group information query request

Table 10.3.2.4-1 describes the information group information query request from group management client to group management server.

Table 10.3.2.4-1: Group information query request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | The identity of the VAL user or VAL UE performing the query. |
| VAL group ID | M | The identity of the VAL group to be queried. |
| Query type | M | It indicates the query type, i.e., membership information. |

#### 10.3.2.5	Group information query response

Table 10.3.2.5-1 describes the information flow group information query response from group management server to group management client.

Table 10.3.2.5-1: Group information query response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | The identity of the VAL group to be queried. |
| Query type | M (see NOTE) | It indicates the query type, e.g. membership information. |
| Query result | M (see NOTE) | The group information retrieved from the group management server based on the query type, i.e., a list of group members. |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the values of other information elements have no meaning.. |

#### 10.3.2.6	Group membership update request

Table 10.3.2.6-1 describes the information flow group membership update request from the group management client to the group management server.

Table 10.3.2.6-1: Group membership update request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| VAL group ID | M | Identity of the VAL group |
| Identity | M | List of identities of the VAL users and VAL UEs affected by this operation |
| Operations | M | Add to or delete from the group |
| VAL service specific information (see NOTE) | O | Placeholder for VAL service specific information |
| NOTE: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.7	Group membership update response

Table 10.3.2.7-1 describes the information flow group membership update response from the group management server to the group management client.

Table 10.3.2.7-1: Group membership update response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Result | M | Indicates the success or failure for the operation |

#### 10.3.2.8	Group membership notification

Table 10.3.2.8-1 describes the information flow group membership notification from the group management server to the VAL server.

Table 10.3.2.8-1: Group membership notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Identity | M | List of identities of the VAL users and VAL UEs affected by this operation |
| Operations | M | Add to or delete from the group |
| VAL service specific information (see NOTE) | O | Placeholder for VAL service specific information |
| NOTE: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

Table 10.3.2.8-2 describes the information flow group membership notification from the group management server to the group management client.

Table 10.3.2.8-2: Group membership notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Operations | M | Add to or delete from the group |

#### 10.3.2.9	Group deletion request

Table 10.3.2.9-1 describes the information flow group deletion request from the group management client to the group management server.

Table 10.3.2.9-1: Group deletion request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| VAL group ID | M | VAL group ID of the group to delete |

#### 10.3.2.10	Group deletion response

Table 10.3.2.10-1 describes the information flow group deletion response from the group management server to the group management client.

Table 10.3.2.10-1: Group deletion response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group requested to be deleted |
| Result | M | Indicates success (group no longer exists), or failure (group deletion did not occur, e.g. authorization failure). |

#### 10.3.2.11	Group deletion notification

Table 10.3.2.11-1 describes the information flow group deletion notification from the group management server to the VAL server, and from the group management server to the group management clients for VAL users which are members of the group.

Table 10.3.2.11-1: Group deletion notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID has been deleted. |

#### 10.3.2.12	Group information request

Table 10.3.2.12-1 describes the information flow group information request from the group management server in the partner VAL system of the VAL group to the group management server in the primary VAL system of the VAL group.

Table 10.3.2.12-1: Group information request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management server performing the request. |
| VAL group ID | M | VAL group ID of the group |

#### 10.3.2.13	Group information response

Table 10.3.2.13-1 describes the information flow group information response from the group management server in the primary VAL system of the VAL group to the group management server in the partner VAL system of the VAL group.

Table 10.3.2.13-1: Group information response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration information | O (see NOTE 1) | Configuration information for the VAL group |
| Failure reason | O (see NOTE 2) | Indicates reason for failure to provide VAL group configuration information |
| NOTE 1: Shall be present if the request can be fulfilled by the group management server in the primary VAL system of the VAL group. NOTE 2: Shall be present if the request cannot be fulfilled by the group management server in the primary VAL system of the VAL group. |  |  |

#### 10.3.2.14	Group information subscribe request

Table 10.3.2.14-1 describes the information flow group information subscribe request from the group management server in the partner VAL system of the VAL group to the group management server in the primary VAL system of the VAL group.

Table 10.3.2.14-1: Group information subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management server performing the request. |
| VAL group ID | M | VAL group ID of the group |

#### 10.3.2.15	Group information subscribe response

Table 10.3.2.15-1 describes the information flow group information subscribe response from the group management server in the primary VAL system of the VAL group to the group management server in the partner VAL system of the VAL group.

Table 10.3.2.15-1: Group information subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates success or failure of the subscribe request |

#### 10.3.2.16	Group information notify request

Table 10.3.2.16-1 describes the information flow group information notify request from the group management server in the primary VAL system of the VAL group to the group management server in the partner VAL system of the VAL group.

Table 10.3.2.16-1: Group information notify request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration information | M | Configuration information for the VAL group |

#### 10.3.2.17	Group information notify response

Table 10.3.2.17-1 describes the information flow group information notify response from the group management server in the partner VAL system of the VAL group to the group management server in the primary VAL system of the VAL group.

Table 10.3.2.17-1: Group information notify response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates success or failure of the notification request |

#### 10.3.2.18	Store group configuration request

Table 10.3.2.18-1 describes the information flow store group configuration request from the group management client to the group management server.

Table 10.3.2.18-1: Store group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M | VAL group configuration data |

#### 10.3.2.19	Store group configuration response

Table 10.3.2.19-1 describes the information flow store group configuration response from the group management server to the group management client.

Table 10.3.2.19-1: Store group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

#### 10.3.2.20	Get group configuration request

Table 10.3.2.20-1 describes the information flow get group configuration request from the group management client to the group management server.

Table 10.3.2.20-1: Get group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group information reference | M | Reference to configuration data for the VAL group |
| VAL services requested (see NOTE) | O | Service(s) for which group configuration is requested |
| NOTE: If 'VAL services requested' is not present, group configuration is requested for all services defined for the VAL group |  |  |

#### 10.3.2.21	Get group configuration response

Table 10.3.2.21-1 describes the information flow get configuration response from the group management server to the group management client.

Table 10.3.2.21: Get group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| VAL group configuration data | M (see NOTE) | VAL group configuration data |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of the VAL group configuration data information element has no meaning. |  |  |

#### 10.3.2.22	Subscribe group configuration request

Table 10.3.2.22-1 describes the information flow subscribe group configuration request from the group management client to the group management server.

Table 10.3.2.22-1: Subscribe group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL services requested (see NOTE) | O | Service(s) for which group configuration is requested |
| Minimum time between consecutive notifications | O | Defaults to 0 if not provided, otherwise indicates the minimum time interval between consecutive notifications |
| NOTE: If 'VAL services requested' is not present, group configuration is requested for all services defined for the VAL group |  |  |

#### 10.3.2.23	Subscribe group configuration response

Table 10.3.2.23-1 describes the information flow subscribe group configuration response from the group management server to the group management client.

Table 10.3.2.23-1: Subscribe group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

#### 10.3.2.24	Notify group configuration request

Table 10.3.2.24-1 describes the information flow notify group configuration request from the group management server to the group management client.

Table 10.3.2.24-1: Notify group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management server performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL group information reference (see NOTE) | O | Reference to information stored relating to the VAL group |
| Group related key material (see NOTE) | O | Key material for use with the VAL group |
| NOTE: At least one of these information elements shall be present. |  |  |

#### 10.3.2.25	Notify group configuration response

Table 10.3.2.25-1 describes the information flow notify group configuration response from the group management client to the group management server.

Table 10.3.2.25-1: Notify group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

#### 10.3.2.26	Configure VAL group request

Table 10.3.2.26-1 describes the information flow for configure VAL group request from a VAL server to a group management server.

Table 10.3.2.26-1: Configure VAL group request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL group ID | M | The group ID used for the VAL group. |
| VAL group description (see NOTE 2) | M | Information related to the VAL group e.g. group definition including communication type, policy, group size, group leader. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| Geo ID list (see NOTE 1) | O | List of geographical areas to be addressed by the group. |
| Identity list (see NOTE 1) | O | List of VAL UE IDs who are invited to be member of the group. |
| Identity list subscription | O | Indicates interest to receive notifications of newly registered or de-registered VAL UE IDs. |
| NOTE 1: At least one of these IEs shall be present. NOTE 2: Group communication type may indicate 5G LAN-Type communication using either Ethernet or IP (IPv4 and/or IPv6) transport. |  |  |

#### 10.3.2.27	Configure VAL group response

Table 10.3.2.27-1 describes the information flow for configure VAL group response from group management server to a VAL server.

Table 10.3.2.27-1: Configure VAL group response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the configure VAL group operation |
| Subscription result | O (see NOTE) | Indicates whether subscription to receive notifications of newly registered or de-registered VAL UE IDs is successful or not |
| NOTE: Shall be present only if there is a subscription in the configure VAL group request and successful. |  |  |

#### 10.3.2.28	Group announcement

Table 10.3.2.28-1 describes the information flow for a group management server to announce a VAL group to the group management clients.

Table 10.3.2.28-1: Group announcement

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | The group ID used for the VAL group. |
| VAL group description (see NOTE 3) | M | Information related to the VAL group e.g. group definition including communication type and connection parameters, policy, group size, group leader. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| Geo ID list (see NOTE 1) | O | List of geographical areas to be addressed by the group. |
| Identity list (see NOTE 1, NOTE 2) | O | List of VAL UE IDs who are invited to be member of the group. |
| NOTE 1: At least one of these IEs shall be present. NOTE 2: This element is not present if it results in privacy concerns. NOTE 3: Group communication type may indicate 5G LAN-Type communication using either Ethernet or IP (IPv4 and/or IPv6) transport. Group connection parameters may include DNN and S-NSSAI. |  |  |

#### 10.3.2.29	Group registration request

Table 10.3.2.29-1 describes the information flow for a group management client to register to a VAL group in response to a group announcement from the group management server. Message filters for throttling messages and notifications is included in this request e.g. for limiting message exchange on a constrained UE, for receiving only improtant group communication.

Table 10.3.2.29-1: Group registration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE registering to the VAL group. |
| VAL Group ID | M | The group ID to be registered by the VAL UE for the VAL group. |
| Identity list subscription | M | Indicates interest to receive notifications of newly registered or de-registered VAL UE IDs |
| Message filters | O | Group message communication will be sent to the VAL UE after applying message filters as described in Table 10.3.2.29-2. |

Table 10.3.2.29-2: Message filters

| Information element | Status | Description |
| --- | --- | --- |
| Identities | O | List of VAL UE identities whose messages will be send to the VAL UE being registered |
| Max number of messages (see NOTE 1) | O | Total number of messages allowed to be sent to the VAL UE in a given time frame as defined by the VAL service. |
| Time frame | O | Time frame associated to Max number of messages IE |
| Message Types (see NOTE 2) | O | List of message types which will be sent to the VAL UE being registered (e.g. high priority messages, or group configuration messages, etc.). |
| NOTE 1: Specifying value for max number of messages is outside the scope of SEAL and up to the implementation of a particular Vertical requirement. NOTE 2: Message types are as decided by the specific Vertical. |  |  |

Editor's note:	Handling of updates to the "message filters" over the life of the group communication is FFS.

#### 10.3.2.30	Group registration response

Table 10.3.2.30-1 describes the information flow for a group management server to respond for a group registration request from the group management client.

Table 10.3.2.30-1: Group registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the VAL server in response to VAL group registration request indicating success or failure |
| Subscription result | M | Indicates whether subscription to receive notifications of newly registered or de-registered VAL UE IDs is successful or not |

#### 10.3.2.31	Identity list notification

Table 10.3.2.31-1 describes the information flow identity list notification from the group management server to the group management client.

Table 10.3.2.31-1: Identity list notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Identity list | M | List of VAL UE IDs who are newly registered or de-registered members of the group |

Table 10.3.2.31-2 describes the information flow identity list notification from the group management server to the VAL server.

Table 10.3.2.31-2: Identity list notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | Identity of the VAL group |
| Identity list | M | List of VAL UE IDs who are newly registered or de-registered members of the group |
| Message filters list | O | List of Message filters per VAL UE ID who are newly registered. |

#### 10.3.2.32	Group de-registration request

Table 10.3.2.32-1 describes the information flow for a group management client to de-register to a VAL group.

Table 10.3.2.32-1: Group de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identity of the VAL UE de-registering to the VAL group. |
| VAL Group ID | M | The group ID to be de-registered by the VAL UE for the VAL group. |

#### 10.3.2.33	Group de-registration response

Table 10.3.2.33-1 describes the information flow for a group management server to respond for a group de-registration request from the group management client.

Table 10.3.2.33-1: Group de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the group management server in response to VAL group de-registration request indicating success or failure |

#### 10.3.2.34	Location-based group creation request

Table 10.3.2.34-1 describes the information flow location-based group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.34-1: location-based group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| Location criteria | M | Criteria to combine Users or UEs in a location. |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

#### 10.3.2.35	Location-based group creation response

Table 10.3.2.35-1 describes the information flow location-based group creation response from the group management server to the group management client or VAL server.

Table 10.3.2.35-1: Location-based group creation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M (see NOTE) | VAL group ID of the group |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of VAL group ID information element has no meaning. |  |  |

#### 10.3.2.36	Group list fetch request

Table 10.3.2.36-1 describes the information flow for Group list fetch request from the group management client to the group management server.

Table 10.3.2.36-1: Group list fetch request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | The identity of the VAL UE or VAL User performing the operation. |
| Period | O | Indicates to the group management server to provide list of the groups created between specified period. |

#### 10.3.2.37	Group list fetch response

Table 10.3.2.37-1 describes the information flow for Group list fetch response from the group management server to the group management client.

Table 10.3.2.37-1: Get group list fetch response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the group management server in response to group list fetch request indicating success or failure |
| VAL group IDs | M | The list of VAL groups for which the VAL User is a member |

#### 10.3.2.38	Temporary group formation request

Table 10.3.2.38-1 describes the information flow for the temporary group formation request from the group management client to the group management server.

Table 10.3.2.38-1: Temporary group formation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of VAL group IDs to be combined |
| VAL service list (see NOTE) | O | A subset of the common VAL service(s) to be applied for the temporary group |
| NOTE: If this information element is not present, all the VAL service(s) that are common to the groups being combined will be applicable for the temporary group. |  |  |

#### 10.3.2.39	Temporary group formation response

Table 10.3.2.39-1 describes the information flow for the temporary group formation response from the group management server to the group management client.

Table 10.3.2.39-1: Temporary group formation response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | O (see NOTE) | VAL group ID of the temporary group |
| VAL service list | O | List of VAL services whose service communications are to be enabled on this temporary group |
| Result | M | Indicates the success or failure of temporary group formation. |
| NOTE: Shall be present if the Result information element indicates that the temporary group formation operation is successful. Otherwise VAL group ID shall not be present. |  |  |

#### 10.3.2.40	Temporary group formation notify

Table 10.3.2.40-1 describes the information flow temporary group formation notify from the group management server to the VAL server.

Table 10.3.2.40-1: Temporary group formation notify

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| VAL group ID | M | VAL group ID of the temporary group |
| VAL service list | O | List of VAL services whose service communications are to be enabled on this temporary group. |

#### 10.3.2.41	Temporary group formation notification

Table 10.3.2.41-1 describes the information flow temporary group formation notification from the group management server to the group management client.

Table 10.3.2.41-1: Temporary group formation notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID list | M | List of constituent VAL group IDs |
| VAL group ID | M | VAL group ID of the temporary group |

#### 10.3.2.42	Temporary group formation notification response

Table 10.3.2.42-1 describes the information flow temporary group formation notification response from the group management client to the group management server.

Table 10.3.2.42-1: Temporary group formation notification response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the temporary group |

### 10.3.3	Group creation

Figure 10.3.3-1 below illustrates the group creation operations by authorized VAL user/UE/administrator to create a group. It applies to the scenario of normal group creation by a VAL administrator or by authorized user/UE.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is aware of the users' identities which will be combined to form the VAL group.

Figure 10.3.3-1: Group creation

1.	The group management client of the authorized VAL user/UE/administrator requests group create operation to the group management server. The identities of the users or UEs being combined and the information of the VAL services that are enabled on the group shall be included in this message.

2.	During the group creation, the group management server creates and stores the information of the group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). The external group identifier, identifying the member UEs of the VAL group at the 3GPP core network, is stored in the newly created VAL group's configuration information.

NOTE:	The exact policies are out of scope of the present document.

3.	The group management server may conditionally notify the VAL server regarding the group creation with the information of the group members.

4.	The VAL group members of the VAL group are notified about the newly created VAL group configuration data.

5.	The group management server provides a group creation response to the group management client of the administrator/authorized VAL user/UE.

### 10.3.4	Group information query

#### 10.3.4.1	General

A VAL user/UE can request the membership list on an VAL group regardless the user or UE's group membership.

#### 10.3.4.2	Procedure

Figure 10.3.4.2-1 below illustrates the group information query on a VAL group.

Figure 10.3.4.2-1: Group information query

1.	The group management client of the VAL user/UE requests the group information on the VAL group from the group management server by sending a group information query request. The query type is included.

2.	The group management server checks whether the VAL user/UE is authorized to perform the query. If authorized, then the group management server retrieves the requested group information based on the query type.

3.	The group management server sends a group information query response including the retrieved group information to the group management client.

### 10.3.5	Group membership

#### 10.3.5.1	Group membership notification

Figure 10.3.5.1-1 illustrates the group membership notification operations to the VAL server(s) and group management clients upon the group membership change at group management server.

Pre-conditions:

1.	VAL group is created on the group management server.

Figure 10.3.5.1-1: group membership notification

1.	The membership of a specific VAL group is changed at group management server.

2.	The group management server notifies the VAL server(s) regarding the group membership change with the information of the updated group members.

3.	The group management server updates the group management clients of the VAL users/UEs who have been added to or removed from the group.

4.	The group management client requests to retrieve the relevant group configurations from group management server, if the user or UE is added to the group. If the user or UE is deleted from the group, the locally stored group configurations in the VAL UE may be removed.

#### 10.3.5.2	Group membership update by authorized user/UE/VAL server

Figure 10.3.5.2-1 below illustrates the group membership update operations by an authorized user/UE/administrator/VAL server to change the membership of a VAL group (e.g. to add or delete group members).

Pre-conditions:

1.	The group management server and VAL server serve the same VAL system;

2.	The initiator of this operation is aware of the current group membership of the VAL group;

3.	The authorized user/UE/administrator/VAL server is aware of the users' identities which will be added to or deleted from the VAL group.

Figure 10.3.5.2-1: Group membership update by authorized user/UE/VAL server

1.	The group management client of the authorized user/UE/administrator or VAL server requests group membership update operation to the group management server.

2.	The group management server updates the group membership information. The group management server may perform the check on policies e.g. the maximum limit of the total number of VAL group members.

NOTE 1:	The exact policies are out of scope of the present document.

3.	The group management server notifies the VAL server(s) regarding the group membership change with the information of the updated group members.

NOTE 2:	Step 3 does not happen when the VAL server is requesting group membership update operation.

4.	The group members that are added to or deleted from the group by this operation are notified about the group membership change. This step may be followed by retrieving group configurations.

5.	The group management server provides a group membership response to the group management client of the authorized user/UE/administrator or the VAL server.

### 10.3.6	Group configuration management

#### 10.3.6.1	Store group configurations at the group management server

The procedure for store group configurations at the group management server is described in figure 10.3.6.1-1.

Pre-conditions:

-	The group management server may have some pre-configuration data which can be used for online group configuration validation;

Figure 10.3.6.1-1: Store group configurations at group management server

1.	The group configurations are received by the group management client of an authorized user/UE or VAL server.

2.	The received group configurations are sent to the group management server for storage using a store group configuration request.

3.	The group management server may validate the group configurations before storage.

4.	The group management server stores the group configurations.

5.	The group management server provides a store group configuration response indicating success or failure. If any validation or storage fails, the group management server provides a failure indication in the store group configuration response.

#### 10.3.6.2	Retrieve group configurations

The procedure for retrieve group configurations at the group management client or the VAL server is described in figure 10.3.6.2-1. This procedure can be used following service authorisation when the configuration management client has received the list of groups and the group management client needs to obtain the group configurations, or following a notification from the group management server that new group configuration information is available.

Pre-conditions:

-	The group management server has received configuration data for groups, and has stored this configuration data;

-	The VAL UE has registered for service and the group management client or the VAL server needs to download group configuration data applicable to the user/UE.

Figure 10.3.6.2-1: Retrieve group configurations

1.	The group management client or the VAL server requests the group configuration data.

2.	The group management server provides the group configuration data to the client or the VAL user.

3.	The group management client or the VAL server stores the group configuration information.

#### 10.3.6.3	Subscription and notification for group configuration data

The procedure for subscription for group configuration data as described in figure 10.3.6.3-1 is used by the group management client to indicate to the group management server that it wishes to receive updates of group configuration data for groups for which it is authorized.

Pre-conditions:

-	The group management server has some group configurations stored.

Figure 10.3.6.3-1: Subscription for group configurations

1.	The group management client subscribes to the group configuration information stored at the group management server using the subscribe group configuration request.

2.	The group management server provides a subscribe group configuration response to the group management client indicating success or failure of the request.

The procedure for notification of group configuration data as described in figure 10.3.6.3-2 is used by the group management server to inform the group management client that new group configuration data is available. It can also be used by the group management server to provide new group related key material to the group management client.

Pre-conditions:

-	The group management client has subscribed to the group configuration information

-	The group management server has received and stored new group configuration information, or the group management server has generated and stored new key material, or both of these have occurred.

Figure 10.3.6.3-2: Notification of group configurations

1.	The group management server provides the notification to the group management client, who previously subscribed for the group configuration information. Optionally, the notify group configuration request may contain group related key material for the group management client.

2.	The group management client provides a notify group configuration response to the group management server.

3.	If the group management server had provided group related key material to the group management client, the group management client stores the key material.

If the group management server has notified the group management client about new group configuration information through this procedure, the group management client may then follow the procedure described in subclause 10.3.6.2 in order to retrieve that group configuration information.

#### 10.3.6.4	Structure of group configuration data

The group configuration data contains group configuration data common to all VAL services and group configuration data specific to each VAL service.

NOTE:	For a VAL service, the VAL group configuration data is listed in the corresponding VAL service specification and is outside the scope of the present document.

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information.

3.	The location management server composes the list of users or UEs within the requested location.

4.	The group management server receives the composed list of users or UEs from the location management server. The location management server may include VAL service IDs associated with the list of users or UEs.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group's configuration information. If VAL service ID list was received in step 1 as part of group formation criteria and not in step 4 for the composed list of users or UEs, the group management server shall retrieve the VAL service data from the configuration management server as described in clause 11.3.2.13 and form the group with relevant users or UEs.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

### 10.3.8	Group announcement and join

#### 10.3.8.1	General

This subclause describes the procedures for establishing group communication from the group management server to the group management clients.

#### 10.3.8.2	Procedure

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL clients belong to the same VAL system.

2.	The VAL server is aware of the users' identities and is authorized to form a VAL group.

Figure 10.3.8.2-1: Procedure for establishing VAL group communication between the group management server and group management client1.

1.	The VAL server determines group information and the identity list to which the group announcement shall be sent. The decision can be based on the list of authorized UEs and other criteria (e.g. user consent, service, or vehicle driving profile).

2.	The VAL server configures VAL group for Uu communication defined by VAL Group ID for one or more VAL services with list of VAL Service ID with the group management server.

3.	The group management server creates an empty group based on the information provided in the Configure VAL group request. The group management server stores the mapping between the VAL group ID and the external Group Id in the VAL group document, along with a list of GPSIs corresponding to the identity list provided by the VAL server. The group management server also determines whether the group is for 5G LAN-Type communication and whether Ethernet or IP (IPv4 and/or IPv6) transport shall be used for the 5G LAN-Type communication.

4.	If 5G LAN-Type communication is to be used, the group management server creates a 5GVN group in the 5GS via N33 using the create group procedure specified in 3GPP TS 23.501 [10] clause 5.29.2 and 3GPP TS 23.502 [11] clause 4.15.6. The group management server creates the 5GVN group data and the 5GVN group membership data defined in 3GPP TS 23.502 [11] clause 4.15.6.3b to be configured in the 5GS. To create the 5GVN group data the group management server uses the 5G LAN-Type communication type information provided by the VAL server to set the PDU session type (Ethernet or IP) and maps the VAL service IDs to Application descriptors. To create the 5GVN group membership data the group management server maps the VAL group ID to the External Group ID and makes a list of GPSIs corresponding to the identity list provided by the VAL server.

NOTE 1:	This step is skipped for the case that a 5G LAN-Type communication is not being used.

NOTE 2:	The PDU session type, DNN, S-NSSAI provided within 5GVN group data cannot be modified in the 5GS after the create procedure.

NOTE 3:	The 5GS supports only a 1:1 mapping between DNN/S-NSSAI combination and 5GVN group.

NOTE 4:	The group management server maintains a mapping between DNN and S-NSSAI of the 5GVN group and the VAL server requester identity based on operator policy. How such mapping is configured is implementation specific and out of the scope of this specification.

5.	The group management server announces the VAL group to the group management clients. For a 5GVN group the announcement includes the communication type (IP or Ethernet), DNN, and S-NSSAI corresponding to the 5GVN group.

6.	The group management client registers to VAL group communication using the VAL Group ID.

7.	The group management server records the users who have registered to be the members of the group.

8.	The group management server sends a VAL group registration response to the group management client.

9.	The group management server sends a configure VAL group response to the VAL server.

NOTE 5:	Step 9 may occur any time after step 5.

10.	The group management server sends identity list notification about the newly registered users to the other members of the group and VAL server, whose subscription to receive notifications of newly registered VAL UE IDs is successful in step 8 and step 9 respectively.

11.	The group management client may inform VAL client about the updated identity list.

### 10.3.9	Group member leave

#### 10.3.9.1	General

This subclause describes the procedures for group member to leave the group by de-registering.

#### 10.3.9.2	Procedure

Pre-conditions:

1.	Group is previously defined on the group management server including the list of registered users and each member of the group and VAL server is aware of it.

Figure 10.3.9.2-1: Procedure for group member leaving the group.

1.	The VAL client determines to de-register member from the group and group management client is aware of it.

2.	The group management client initiates the group de-registration request towards the group management server.

3.	The group management server checks the authorization of group de-registration request and updates the group member list.

4.	The group management server sends a group de-registration response to the group management client.

5.	The group management server sends identity list notification about the leaving registered user to the other members of the group and the VAL server, whose subscription to receive notifications of de-registered VAL UE IDs is successful in step 7 and step 8 of the procedure in clause 10.3.8.2 respectively.

### 10.3.10	Temporary groups

#### 10.3.10.1	Temporary group formation within a VAL system

Figure 10.3.10.1-1 below illustrates the temporary group formation within a VAL system.

Temporary groups are formed by combining two or more groups. The temporary group formation is applicable only for groups configured with at least one common VAL service. The temporary group formation shall be rejected if any of the requested VAL services are not common to all the constituent VAL service groups.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The group management client has retrieved the group configurations of the groups.

3.	The VAL server has subscribed to receive group management event notifications.

Figure 10.3.10.1-1: Temporary group formation within a VAL system

1.	The group management client of the VAL user requests temporary group formation operation to the group management server, which is the group management server of one of the groups to be regrouped. The identities of the groups being combined shall be included in this message. The group management client may indicate the security level required for the temporary group. The group management client may indicate the priority level required for the temporary group.

2.	The group management server checks whether temporary group formation operation is performed by an authorized VAL user, based on group policy. The group management server checks whether group1 or group2 is a temporary group. If group 1 or group2 is a temporary group, then the group regrouping will be rejected, otherwise the group regrouping can proceed.

3.	The group management server creates and stores the information of the temporary group, including the temporary VAL group ID, the VAL group ID of the groups being combined, the priority level of the temporary group and the security level of the temporary group. If the authorized VAL user does not specify the security level and the priority level, the group management server shall set the lowest security level and the highest priority of the constituent groups. If VAL service types of the groups being combined are not identical, group management server determines the overlapping part and stores the VAL service list for the temporary group.

4.	The group management server notifies the VAL server regarding the temporary group creation with the information of the constituent groups, i.e. temporary VAL group ID, group1's VAL group ID and group2's VAL group ID. If VAL service list is included, VAL server stores it and provides VAL service types accordingly.

5.	The group management server notifies the VAL group members of the constituent VAL groups by sending temporary group formation notification messages.

6.	The VAL group members of the constituent VAL groups send individual temporary group formation notification response messages.

7.	The group management server provides a temporary group formation response to the group management client of the authorized VAL user. If VAL service list is included, group management client stores it and initiates VAL service types accordingly.

8.	The VAL group members of the constituent VAL groups individually request group configuration data from the group management server for the temporary group. The group configuration data includes security, priority, and other parameters.

### 10.3.11	Group List Fetch

Figure 10.3.11-1 illustrates the group list fetch operations to fetch list of groups by the group management client.

Pre-conditions:

1)	List of groups to which a VAL UE/ VAL User belongs to is known to the Group management server for each of the VAL UE/ VAL User.

2)	VAL user has not received group announcement message as it was offline previously.

Figure 10.3.11-1: Group list fetch

1)	The VAL client requests group management client to provide the list of groups in which the VAL UE or VAL User is a member.

2)	The group management client initiates the group list fetch request towards the Group management server. The information elements described in clause 10.3.2.36 are included in the group list fetch request.

3)	The group management server checks the authorization of group list fetch request and if authorized, sends the group list fetch response containing list of groups in which the VAL user is member. The information elements described in clause 10.3.2.37 are included in the group list fetch response.

4)	The group management client notifies the list of groups to the VAL client.

### 10.3.12	Location-based group update

Figure 10.3.12-1 below illustrates the location-based group update.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The location based group has been created as specified in clause 10.3.7.

3.	The group management server has subscribed to monitor UEs moving in or out of the fixed location area as specified in clause 9.3.12.

Figure 10.3.12-1: Location-based group update

1.	The group management server receives location area monitoring notification from location management server as specified in clause 9.3.12.4.

2.	The group management server updates the group members and sends notification as specified in clause 10.3.5.1.

### 10.3.13	Group deletion

Figure 10.3.13-1 below illustrates the group deletion operation by authorized VAL user/UE/administrator to delete a group. It applies to group deletion by a VAL administrator or by authorized user/UE.

Pre-conditions:

1.	The group management client, group management server, VAL server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is aware of the group identity which needs to be deleted.

Figure 10.3.13-1: Group deletion

1.	The group management client of the authorized VAL user/UE/administrator requests group deletion operation to the group management server. The identity of the group to be deleted shall be included in this message.

2.	The group management server authorizes the request and if authorized, deletes the group.

3.	The group management server notifies the VAL server regarding the group deletion.

4.	The group members of the VAL group are notified about the deleted VAL group.

5.	The group management server provides a group deletion response to the group management client of the administrator/authorized VAL user/UE.

## 10.4	SEAL APIs for group management

### 10.4.1	General

Table 10.4.1-1 illustrates the SEAL APIs for group management.

Table 10.4.1-1: List of SEAL APIs for group management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_GroupManagement | Query_Group_Info | VAL server | Request /Response |
|  | Create_LocationBasedGroup_Info | VAL server | Request/Response |
|  | Update_Group_Info | VAL server | Request /Response |
| SS_GroupManagementEvent | Subscribe_ Group_Info_Modification | VAL server | Subscribe/Notify |
|  | Notify_Group_Info_Modification | VAL server |  |
|  | Notify_Group_Creation | VAL server |  |
|  | Notify_TempGroupFormation | VAL server |  |

### 10.4.2	SS_GroupManagement API

#### 10.4.2.1	General

API description: This API enables the VAL server to communicate with the group management server for querying group information, obtaining stored group configuration, modify the group membership and configuration information on the group management server over GM-S.

#### 10.4.2.2	Query_Group_Info operation

API operation name: Query_Group_Info

Description: Query group information and obtaining group configuration information.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.4, 10.3.2.20

Outputs: See subclause 10.3.2.5, 10.3.2.21

See subclause 10.3.4 and 10.3.6.2 for the details of usage of this API operation.

#### 10.4.2.3	Update_Group_Info operation

API operation name: Update_Group_Info

Description: Storing group membership and configuration information.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.6, 10.3.2.18

Outputs: See subclause 10.3.2.7, 10.3.2.19

See subclause 10.3.6.5 and 10.3.6.1 for the details of usage of this API operation.

#### 10.4.2.4	Create_LocationBasedGroup_Info operation

API operation name: Create_LocationBasedGroup_Info

Description: Create location-based group

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.34

Outputs: See subclause 10.3.2.35

See subclause 10.3.7 for the details of usage of this API operation.

### 10.4.3	Void

#### 10.4.3.1	Void

#### 10.4.3.2	Void

### 10.4.4	Void

#### 10.4.4.1	Void

#### 10.4.4.2	Void

### 10.4.5	SS_Group_Management_Event API

#### 10.4.5.1	General

API description: This API enables the VAL server to communicate with the group management server to subscribe and receive subsequent notification events over GM-S.

#### 10.4.5.2	Subscribe_ Group_Info_Modification operation

API operation name: Subscribe_ Group_Info_Modification

Description: Subscribing to changes to group membership and configuration information.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.14

Outputs: See subclause 10.3.2.15

See subclause 10.3.6.3 for the details of usage of this API operation.

#### 10.4.5.3	Notify_Group_Info_Modification operation

API operation name: Notify_Group_Info_Modification

Description: Notification for changes to group membership and configuration information.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.8

Outputs: See subclause 10.3.2.8

See subclause 10.3.5.1 and 10.3.5.2 for the details of usage of this API operation.

#### 10.4.5.4	Notify_Group_Creation operation

API operation name: Notify_Group_Creation

Description: Notification for new group creation.

Known Consumers: VAL server.

Inputs: See subclause 10.3.2.3

Outputs: See subclause 10.3.2.3

See subclause 10.3.3 for the details of usage of this API operation.

#### 10.4.5.5	Notify_TempGroupFormation operation

API operation name: Notify_TempGroupFormation

Description: Notification of the new temporary group formed at the group management server.

Known Consumers: VAL server.

Inputs: See clause 10.3.2.40

Outputs: None

See subclause 10.3.10.1 for the details of usage of this API operation.

# 11	Configuration management

## 11.1	General

The configuration management is a SEAL service that offers the configuration management related capabilities to one or more vertical applications.

## 11.2	Functional model for configuration management

### 11.2.1	General

The functional model for the configuration management is based on the generic functional model specified in clause 6. It is organized into functional entities to describe a functional architecture which addresses the support for configuration management aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

### 11.2.2	On-network functional model description

Figure 11.2.2-1 illustrates the generic on-network functional model for configuration management.

Figure 11.2.2-1: On-network functional model for configuration management

The configuration management client communicates with the configuration management server over the CM-UU reference point. The configuration management client provides the support for configuration management functions to the VAL client(s) over CM-C reference point. The VAL server(s) communicate with the configuration management server over the CM-S reference point. The configuration management server communicates with the VAL user database over the CM-VAL-UDB reference point.

### 11.2.3	Off-network functional model description

Figure 11.2.3-1 illustrates the off-network functional model for configuration management.

Figure 11.2.3-1: Off-network functional model for configuration management

The configuration management client of the UE1 communicates with the configuration management client of the UE2 over the CM-PC5 reference point.

### 11.2.4	Functional entities description

#### 11.2.4.1	General

The functional entities for configuration management SEAL service are described in the following subclauses.

#### 11.2.4.2	Configuration management client

The configuration management client functional entity acts as the application client for configuration related transactions. The configuration management client interacts with the configuration management server and provides and receives configuration data. The configuration management client also supports interactions with the corresponding configuration management client between the two UEs.

The configuration management client functional entity is supported by the signalling user agent and HTTP client functional entities of the signalling control plane.

#### 11.2.4.3	Configuration management server

The configuration management server is a functional entity used to configure one or more vertical applications with 3GPP system related vertical applications provisioning information and configure data on the configuration management client. The configuration management server manages vertical application configuration supported within the vertical's service provider. The configuration management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The configuration management server also supports interactions with the corresponding configuration management server in distributed SEAL deployments.

The configuration management server functional entity is supported by the SIP AS and HTTP server functional entities of the signalling control plane.

### 11.2.5	Reference points description

#### 11.2.5.1	General

The reference points for the functional model for configuration management are described in the following subclauses.

#### 11.2.5.2	CM-UU

The interactions related to configuration management functions between the configuration management client and the configuration management server are supported by CM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

The CM-UU reference point provides the configuration information required for VAL services and supports:

-	configuration of the VAL UE by the VAL service; and

-	configuration of the VAL application with the VAL service related information e.g. policy information by the VAL UE.

The CM-UU reference point shall use the HTTP-1/HTTP-2 reference points for transport and routing of configuration management related signalling. The CM-UU reference point shall use SIP-1/SIP-2 reference points for subscription/notification related signalling.

#### 11.2.5.3	CM-PC5

The interactions related to configuration management functions between the configuration management clients located in different VAL UEs are supported by CM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

#### 11.2.5.4	CM-C

The interactions related to configuration management functions between the VAL client(s) and the configuration management client within a VAL UE are supported by CM-C reference point.

#### 11.2.5.5	CM-S

The interactions related to configuration management functions between the VAL server(s) and the configuration management server are supported by CM-S reference point. The CM-S reference point supports VAL server to obtain the VAL service related vertical applications provisioning information. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

The CM-S reference point shall use HTTP-1/ HTTP-2 reference points for transport and routing of configuration management related signalling. The CM-S reference point shall use SIP-2 reference point for subscription/notification related signalling.

#### 11.2.5.6	CM-E

The interactions related to configuration management functions between the configuration management servers in a distributed deployment are supported by CM-E reference point.

Editor's Note:	The functions enabled over CM-E reference point is FFS.

#### 11.2.5.7	Reference point CM-VAL-UDB (between the configuration management server and the VAL user database)

The CM-VAL-UDB reference point is an instance of VAL-UDB reference point, which exists between the VAL user database and the configuration management server, is used for:

-	the configuration management server to store the user profile data in the specific VAL user database; and

-	the configuration management server to obtain the user profile from the specific VAL user database for further configuration in the VAL UE.

## 11.3	Procedures and information flows for configuration management

### 11.3.1	General

The procedures related to the configuration management are described in the following subclauses.

### 11.3.2	Information flows

#### 11.3.2.1	Get VAL UE configuration request

Table 11.3.2.1-1 describes the information flow get VAL UE configuration request from the configuration management client to the configuration management server.

Table 11.3.2.1-1: Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | Identify of the VAL UE requesting the configuration information. |
| VAL service ID | O (see NOTE 1) | Identify of the VAL service for which the configuration information is requested. |
| VAL UE Information (see NOTE 2) | O | Additional UE related information required to identify the configuration data (e.g. device type, device vendor, etc) |
| NOTE 1: If the VAL service ID information element is not present, then the default service is service. NOTE 2: The VAL service provider can configure the VAL UE with different configuration data based on this IE. |  |  |

#### 11.3.2.2	Get VAL UE configuration response

Table 11.3.2.2-1 describes the information flow get VAL UE configuration response from the configuration management server to the configuration management client.

Table 11.3.2.2-1: Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of getting the configuration information |
| VAL UE configuration data | O (see NOTE) | The VAL UE configuration data as specified in the corresponding VAL service specification and outside the scope of the present document |
| NOTE: If the Result information element indicates failure then VAL UE configuration data information element is not included. |  |  |

#### 11.3.2.3	Get VAL user profile request

Table 11.3.2.3-1 describes the information flow get VAL user profile request from the configuration management client to the configuration management server.

Table 11.3.2.3-1: Get VAL user profile request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the configuration management client performing the request. |
| Identity | M | The VAL user ID of the VAL user or VAL UE ID. |

#### 11.3.2.4	Get VAL user profile response

Table 11.3.2.4-1 describes the information flow get VAL user profile response from the configuration management server to the configuration management client.

Table 11.3.2.4-1: Get VAL user profile response

| Information element | Status | Description |
| --- | --- | --- |
| VAL user profile data | M (see NOTE) | One or more VAL user profiles associated with the VAL user ID or VAL UE ID provided in the associated get VAL user profile request. |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of VAL user profile data information element has no meaning. |  |  |

#### 11.3.2.5	Notification for VAL user profile data update

Table 11.3.2.5-1 describes the information flow notification for VAL user profile data update from the configuration management server to the configuration management client.

Table 11.3.2.5-1: Notification for VAL user profile data update

| Information element | Status | Description |
| --- | --- | --- |
| Pointer to modified VAL user profile data. | M | Pointer to the modified VAL user profile data. |

#### 11.3.2.6	Get updated VAL user profile data request

Table 11.3.2.6-1 describes the information flow get updated VAL user profile data request from the configuration management client to the configuration management server.

Table 11.3.2.6-1: Get updated VAL user profile data request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | The VAL user ID of the originating VAL user or VAL UE ID. |
| Pointer to modified VAL user profile data. | M | Pointer to the modified VAL user profile data. |

#### 11.3.2.7	Get updated VAL user profile data response

Table 11.3.2.7-1 describes the information flow get updated VAL user profile data response from the configuration management server to the configuration management client.

Table 11.3.2.7-1: Get updated VAL user profile data response

| Information element | Status | Description |
| --- | --- | --- |
| Updated VAL user profile data | M (see NOTE) | VAL user profile data that has been modified. |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of Updated VAL user profile data information element has no meaning. |  |  |

#### 11.3.2.8	Update VAL user profile data request

Table 11.3.2.8-1 describes the information flow update VAL user profile data request from the configuration management client to the configuration management server.

Table 11.3.2.8-1: Update VAL user profile data request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | The VAL user ID of the originating VAL user or VAL UE ID. |
| Updated VAL user profile data | M | The contents of the user profile data to be updated. |

#### 11.3.2.9	Update VAL user profile data response

Table 11.3.2.9-1 describes the information flow update VAL user profile data response from the configuration management server to the configuration management client.

Table 11.3.2.9-1: Update VAL user profile data response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure |

#### 11.3.2.10	Updated user profile subscription request

Table 11.3.2.10-1 describes the information flow from the VAL server to the configuration management server for updated user profile subscription request.

Table 11.3.2.10-1: Updated user profile subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Identities list | M | List of VAL users or VAL UEs whose updates on user profile is requested. |
| Time between consecutive user profile updates | M | It indicates the interval time between consecutive user profile updates |

#### 11.3.2.11	Updated user profile subscription response

Table 11.3.2.11-1 describes the information flow from the configuration management server to the VAL server for updated user profile subscription response.

Table 11.3.2.11-1: Updated user profile subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M (see NOTE) | It indicates the subscription result |
| Result | M | Indicates the success or failure for the operation |
| NOTE: If the Result information element indicates failure then the value of the Subscription status information element has no meaning. |  |  |

#### 11.3.2.12	Updated user profile notification

Table 11.3.2.12-1 describes the information flow updated user profile notification from the configuration management server to the VAL server.

Table 11.3.2.12-1: Notify updated user profile event

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | M | List of VAL users or VAL UEs whose user profile is modified. |
| Updated user profile data | M | User profile data that has been modified. |

#### 11.3.2.13	Get VAL service request

Table 11.3.2.13-1 describes the information flow get VAL service request from the group management server to the configuration management server.

Table 11.3.2.13-1: Get VAL service request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the entity performing the request. |
| Identity list | O (see NOTE) | The VAL user IDs or VAL UE IDs. |
| VAL service ID list | O (see NOTE) | The requested VAL service IDs for the identity list. |
| NOTE: At least one of Identity list or VAL service ID list shall be included. With both Identity list and VAL service ID in the request, the response only contains common VAL services (as requested) for the requested VAL users or VAL Ues. |  |  |

#### 11.3.2.14	Get VAL service response

Table 11.3.2.14-1 describes the information flow get VAL service response from the configuration management server to the group management server.

Table 11.3.2.14-1: Get VAL service response

| Information element | Status | Description |
| --- | --- | --- |
| A list of VAL service data | O (see NOTE 1) | One or more VAL service data associated with the VAL user ID or VAL UE ID. |
| > Identity | M | The VAL user ID or VAL UE ID. |
| > VAL service IDs | M | The supported VAL service IDs. |
| > VAL service specific information | O (see NOTE 2) | Placeholder for VAL service specific information. |
| Result | M | Indicates the success or failure for the operation. |
| NOTE 1: It is valid when the Result information element indicates success. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

### 11.3.3	VAL UE configuration data

#### 11.3.3.1	General

The VAL UE configuration data has to be known by the VAL UE before it can use the VAL service.

#### 11.3.3.2	Procedures

The procedure for VAL UE obtaining the VAL UE related configuration data is illustrated in figure 11.3.3.2-1.

Pre-conditions:

-	The VAL UE has the secure access to the configuration management server.

Figure 11.3.3.2-1: VAL UE obtains the configuration data

1.	The configuration management client sends a get VAL UE configuration request to the configuration management server for obtaining VAL UE configuration data.

2.	The configuration management server sends get VAL UE configuration response to the configuration management client. This message carries the VAL UE configuration data.

#### 11.3.3.3	Structure of VAL UE configuration data

NOTE:	For a VAL service, the VAL UE configuration data is listed in the corresponding VAL service specification and outside the scope of the present document.

### 11.3.4	VAL user profile data

#### 11.3.4.1	General

The VAL user profile procedures are described in the following subclauses.

#### 11.3.4.2	Obtaining the VAL user profile(s) from the network

##### 11.3.4.2.1	Obtaining the VAL user profile(s) in primary VAL system

The procedure for the VAL user or VAL server obtaining VAL user profiles in the primary VAL system of that VAL user is illustrated in figure 11.3.4.2.1-1.

Pre-conditions:

-	The VAL user has performed user authentication in the identity management server.

-	The VAL UE or VAL server has secure access to the configuration management server.

Figure 11.3.4.2.1-1: VAL user obtains the VAL user profile(s) from the network

1.	The configuration management client or VAL server sends a get VAL user profile request message to the configuration management server, which includes the VAL user ID or VAL UE ID.

2.	The configuration management server obtains the VAL user profile information.

3.	The configuration management server sends get VAL user profile response message to the configuration management client or VAL server. When a download is necessary, this message includes all VAL user profiles that are associated with the VAL user ID or VAL UE ID.

##### 11.3.4.2.2	VAL user receiving VAL service from a partner VAL system

Figure 11.3.4.2.2-1 below illustrates mechanism for the configuration management client to retrieve the VAL user profile for the migrating VAL user from the partner VAL system.

NOTE:	Any proxy servers at the edges of the primary and partner VAL systems which are used to hide the topology of the VAL systems from external entities are not shown in this procedure.

Preconditions

-	The VAL user is permitted to migrate to the partner VAL system, and the relevant authorizations are configured in both the primary and partner VAL systems of that VAL user

-	The VAL user has performed VAL user authentication in the partner VAL system, and has received the necessary credentials to retrieve configuration information and to request service authorization.

-	The VAL UE has been provided with addressing information to allow the configuration management client in the VAL UE to access the configuration management server in the partner VAL system.

Figure 11.3.4.2.2-1:	Retrieval of user profile in partner VAL system

1.	The configuration management client in the VAL UE of the migrating VAL user requests the VAL user profile for migration from the configuration management server in the partner VAL system.

2.	The configuration management server in the partner VAL system requests the VAL user profile from the configuration management server in the primary VAL system of the VAL user.

3.	The configuration management server in the primary VAL system of the VAL user retrieves the VAL user profile from the VAL user database in that primary VAL system. The identification of the partner VAL system to which the VAL user is attempting to migrate is used to determine which VAL user profile is retrieved for that VAL user for migration to that partner VAL system.

4.	The configuration management server in the primary VAL system provides the VAL user profile to the configuration management server in the partner VAL system of the VAL user, optionally requesting validation of the modified VAL user profile.

5.	The partner VAL system of the VAL user modifies the VAL user profile according to local configuration information and stores the modified VAL user profile in the VAL user database in the partner VAL system.

6.	If the primary VAL system requested validation of the VAL user profile in step 4, the configuration management server in the partner VAL system of the migrating VAL user may send the modified VAL user profile to the configuration management server of the primary VAL system of the VAL user to allow the primary VAL system of the VAL user to validate the modified VAL user profile.

7.	The primary VAL system of the migrated VAL user validates the modified VAL service profile of the migrated VAL user.

8.	The primary VAL system of the migrated VAL user responds to the partner VAL system with the results of the validation process.

9.	The configuration management server in the partner VAL system provides the VAL user profile to the configuration management client of the migrating VAL user,

NOTE:	Step 9 is not followed if the validation process fails.

#### 11.3.4.3	VAL user receives updated VAL user profile data from the network

The procedure for VAL user obtaining updated VAL user profile data that is initiated by the network is illustrated in figure 11.3.4.3-1.

Pre-conditions:

-	The VAL user has performed user authentication in identity management server.

-	The VAL UE has secure access to the configuration management server.

-	The VAL UE has already obtained one or more VAL user profiles.

-	The configuration management server has access to the VAL user profile(s) associated with the VAL user ID of the VAL user or VAL UE ID.

Figure 11.3.4.3-1: VAL user receives updated VAL user profile data from the network

1.	The configuration management server obtains updated VAL user profile data.

2.	The configuration management server sends a notification for VAL user profile data update to the configuration management client.

3.	The configuration management client sends get updated VAL user profile data request to the configuration management server, which includes the VAL user ID or VAL UE ID.

4.	The configuration management server sends get updated VAL user profile data response to the configuration management client which includes the updated VAL user profile data requested in step 3.

NOTE:	The updated VAL user profile data could be for a specific VAL user profile, a specific parameter in an VAL user profile, a set of VAL user profiles, or all the VAL user profiles for the VAL user ID or VAL UE ID. VAL user profile data is defined per VAL service.

#### 11.3.4.4	VAL user updates VAL user profile data to the network

The procedure for VAL user updating the VAL user profile data is illustrated in figure 11.3.4.4-1.

Pre-conditions:

-	The VAL user has performed user authentication in identity management server.

-	The VAL UE has secure access to the configuration management server.

-	The VAL UE has already obtained one or more VAL user profiles.

Figure 11.3.4.4-1: VAL user updates VAL user profile data to the network

1.	The configuration management client is triggered (e.g. by user interaction operation) to update the VAL user profile data on the configuration management server.

2.	The configuration management client sends update VAL user profile data request to the configuration management server, which includes the VAL user profile data to be updated.

3.	The configuration management server stores the received VAL user profile data.

4.	The configuration management server sends update VAL user profile data response to the configuration management client to confirm the VAL user profile data update is complete.

NOTE:	The updated VAL user profile data could be for a specific VAL user profile, a specific parameter in an VAL user profile, a set of VAL user profiles, or all the VAL user profiles for the VAL user ID and VAL UE ID. VAL user profile data is defined per VAL service.

#### 11.3.4.5	Updated user profile subscription procedure

Figure 11.3.4.5-1 illustrates the high level procedure for obtaining updated user profile data based on subscription request.

Figure 11.3.4.5-1: Updated user profile subscription procedure

1.	Configuration management client or VAL server sends an updated user profile subscription request to the configuration management server to subscribe any updates to user profile of one or more VAL users or VAL UEs.

2.	The configuration management server shall check if the configuration management client or VAL server is authorized to initiate the updated user profile subscription request.

3.	The configuration management server replies with a updated user profile subscription response indicating the subscription status.

Figure 11.3.4.5-2 illustrates the high level procedure of updated user profile notification event.

Figure 11.3.4.5-2: Updated user profile notification

1.	The configuration management server sends the updated user profile notification including the changes to user profile of one or more VAL users or VAL UEs to the configuration management client or VAL server.

NOTE:	Updated user profile notification is based on the subscription.

### 11.3.5	VAL service data

#### 11.3.5.1	General

The VAL service data related procedures are described in the following subclauses.

#### 11.3.5.2	Procedures

The procedure for consumer (e.g. group management server) obtaining the VAL service data is illustrated in figure 11.3.5.2-1.

Pre-conditions:

-	The consumer has the secure access to the configuration management server.

-	The configuration management server has VAL service data maintained for VAL UE or user.

Figure 11.3.5.2-1: Obtains the VAL service data for VAL UE(s) or VAL user(s)

1.	The consumer (e.g. group management server) sends a get VAL service request to the configuration management server for obtaining VAL service data. The request includes a list of VAL UE IDs or VAL user IDs and/or a VAL service ID list as service data retrieval filters.

2.	The configuration server retrieves the VAL service data for the requested VAL UEs or users or VAL services and sends get VAL service response to the consumer (e.g. group management server) with the discovered list of VAL service data.

## 11.4	SEAL APIs for configuration management

### 11.4.1	General

Table 11.4.1-1 illustrates the SEAL APIs for configuration management.

Table 11.4.1-1: List of SEAL APIs for configuration management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_UserProfileRetrieval | Obtain_User_Profile | VAL server | Request /Response |
| SS_UserProfileEvent | Subscribe_User_Profile_Update | VAL server | Subscribe/Notify |
|  | Notify_User_Profile_Update | VAL server |  |
| SS_VALServiceData | Obtain_VAL_Service_Data | group management server | Request /Response |

### 11.4.2	SS_UserProfileRetrieval API

#### 11.4.2.1	General

API description: This API enables the VAL server to communicate with the configuration management server for obtaining user profile over CM-S.

#### 11.4.2.2	Obtain_User_Profile operation

API operation name: Obtain_User_Profile

Description: Obtaining user profile.

Known Consumers: VAL server.

Inputs: See subclause 11.3.2.3

Outputs: See subclause 11.3.2.4

See subclause 11.3.4.2 for the details of usage of this API operation.

### 11.4.3	SS_UserProfileEvent API

#### 11.4.3.1	General

API description: This API enables the VAL server to communicate with the configuration management server for obtaining updated user profile over CM-S.

#### 11.4.3.2	Subscribe_User_Profile_Update operation

API operation name: Subscribe_User_Profile_Update

Description: Subscribing to changes to user profile.

Known Consumers: VAL server.

Inputs: See subclause 11.3.2.10

Outputs: See subclause 11.3.2.11

See subclause 11.3.4.5 for the details of usage of this API operation.

#### 11.4.3.3	Notify_User_Profile_Update operation

API operation name: Notify_User_Profile_Update

Description: Notification for changes to user profile.

Known Consumers: VAL server.

Inputs: See subclause 11.3.2.12

Outputs: See subclause 11.3.2.12

See subclause 11.3.4.5 for the details of usage of this API operation.

### 11.4.4	SS_VALServiceData API

#### 11.4.4.1	General

API description: This API enables the group management server to communicate with the configuration management server for obtaining VAL service data over SEAL-X.

#### 11.4.4.2	Obtain_VAL_Service_Data operation

API operation name: Obtain_VAL_Service_Data

Description: Obtaining VAL service data.

Known Consumers: group management server.

Inputs: See subclause 11.3.2.13

Outputs: See subclause 11.3.2.14

See subclause 11.3.5.2 for the details of usage of this API operation.

# 12	Identity management

## 12.1	General

The identity management is a SEAL service that offers the identity management related capabilities to one or more vertical applications.

## 12.2	Functional model for identity management

### 12.2.1	General

The functional model for the identity management is based on the generic functional model specified in clause 6. It is organized into functional entities to describe a functional architecture which addresses the support for identity management aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

### 12.2.2	On-network functional model description

Figure 12.2.2-1 illustrates the generic on-network functional model for identity management.

Figure 12.2.2-1: On-network functional model for identity management

The identity management client communicates with the identity management server over the IM-UU reference point. The identity management client provides the support for identity management functions to the VAL client(s) over IM-C reference point. The VAL server(s) communicate with the identity management server over the IM-S reference point.

Editor's Note:	The role of VAL-UU in the context of identity management is FFS.

### 12.2.3	Off-network functional model description

Figure 12.2.3-1 illustrates the off-network functional model for identity management.

Figure 12.2.3-1: Off-network functional model for identity management

The identity management client of the UE1 communicates with the identity management client of the UE2 over the IM-PC5 reference point.

### 12.2.4	Functional entities description

#### 12.2.4.1	General

The functional entities for identity management SEAL service are described in the following subclauses.

#### 12.2.4.2	Identity management client

The identity management client functional entity acts as the application client for vertical applications layer user identity related transactions. The identity management client interacts with the identity management server. The identity management client also supports interactions with the corresponding identity management client between the two UEs.

#### 12.2.4.3	Identity management server

The identity management server is a functional entity that authenticates the vertical application layer user identity. The authentication is performed by verifying the credentials provided by the vertical applications' user. The identity management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The identity management server also supports interactions with the corresponding identity management server in distributed SEAL deployments.

### 12.2.5	Reference points description

#### 12.2.5.1	General

The reference points for the functional model for identity management are described in the following subclauses.

#### 12.2.5.2	IM-UU

The interactions related to identity management functions between the identity management client and the identity management server are supported by IM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

#### 12.2.5.3	IM-PC5

The interactions related to identity management functions between the identity management clients located in different VAL UEs are supported by IM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

#### 12.2.5.4	IM-C

The interactions related to identity management functions between the VAL client(s) and the identity management client within a VAL UE are supported by IM-C reference point.

#### 12.2.5.5	IM-S

The interactions related to identity management functions between the VAL server(s) and the identity management server are supported by IM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

#### 12.2.5.6	IM-E

The interactions related to identity management functions between the identity management servers in a distributed deployment are supported by IM-E reference point.

Editor's Note:	The functions enabled over IM-E reference point is FFS.

## 12.3	Procedures and information flows for identity management

### 12.3.1	General

The procedures related to the identity management are described in the following subclauses.

### 12.3.2	Information flows

NOTE:	The procedure for identity management is specified in subclause 5.2.3 and 5.2.4 of 3GPP TS 33.434 [29].

### 12.3.3	General user authentication and authorization for VAL services

#### 12.3.3.1	General

The high level user authentication and authorization procedure is described in the following subclause.

#### 12.3.3.2	Primary VAL system

Figure 12.3.3.2-1 is a high level user authentication and authorization flow.

NOTE:	The specific user authentication and authorization architecture required by the VAL services in order to realize the VAL user authentication and authorization is specified in subclauses 5.2.3, 5.2.4 and 5.2.5 of 3GPP TS 33.434 [29].

The user authentication process shown in figure 12.3.3.2-1 may take place in some scenarios as a separate step independently from a SIP registration phase, for example if the SIP core is outside the domain of the VAL server.

Editor's note: The procedure described in this subclause as shown in Figure 12.3.3.2-1 may require further study.

A procedure for user authentication is illustrated in figure 12.3.3.2-1. Other alternatives may be possible, such as authenticating the user within the SIP registration phase.

Figure 12.3.3.2-1: VAL user authentication and registration with Primary VAL system, single domain

1.	In this step the identity management client begins the user authorization procedure. The VAL user supplies the user credentials (e.g. biometrics, secureID, username/password) for verification with the identity management server. This step may occur before or after step 3. In a VAL system with multiple VAL services, a single user authentication as in step 1 can be used for multiple VAL service authorizations for the user.

2.	The signalling user agent establishes a secure connection to the SIP core for the purpose of SIP level authentication and registration.

3.	The signalling user agent completes the SIP level registration with the SIP core (and an optional third-party registration with the VAL service server(s)).

NOTE 1:	The VAL client(s) perform the corresponding VAL service authorization for the user by utilizing the result of this procedure.

NOTE 2:	Steps 2 and 3 are not required to be performed if the VAL service does not use SIP.

#### 12.3.3.3	Interconnection partner VAL system

Where communications with a partner VAL system using interconnection are required, user authorization takes place in the serving VAL system of the VAL service user, using the VAL user service authorization procedure specified in subclauses 5.2.5 and 5.2.6 of 3GPP TS 33.434 [29].

## 12.4	SEAL APIs for identity management

### 12.4.1	General

There are no APIs defined for SEAL Identity Management.

### 12.4.2	Void

#### 12.4.2.1	Void

#### 12.4.2.2	Void

# 13	Key management

## 13.1	General

The key management is a SEAL service that offers the key management related capabilities to one or more vertical applications.

## 13.2	Functional model for key management

### 13.2.1	General

The functional model for the key management is based on the generic functional model specified in clause 6. It is organized into functional entities to describe a functional architecture which addresses the support for key management aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

### 13.2.2	On-network functional model description

Figure 13.2.2-1 illustrates the generic on-network functional model for key management.

Figure 13.2.2-1: On-network functional model for key management

The key management client communicates with the key management server over the KM-UU reference point. The key management client provides the support for key management functions to the VAL client(s) over KM-C reference point. The VAL server(s) communicate with the key management server over the KM-S reference point.

### 13.2.3	Off-network functional model description

Figure 13.2.3-1 illustrates the off-network functional model for key management.

Figure 13.2.3-1: Off-network functional model for key management

The key management client of the UE1 communicates with the key management client of the UE2 over the KM-PC5 reference point.

### 13.2.4	Functional entities description

#### 13.2.4.1	General

The functional entities for key management SEAL service are described in the following subclauses.

#### 13.2.4.2	Key management client

The key management functional entity acts as the application client for key management functions. It interacts with the key management server. The key management client also supports interactions with the corresponding key management client between the two UEs.

NOTE:	The functionality of the key management client is specified in subclause 5.3 of 3GPP TS 33. 434 [29].

#### 13.2.4.3	Key management server

The key management server is a functional entity that stores and provides security related information (e.g. encryption keys) to the key management client, group management server and vertical application server to achieve the security goals of confidentiality and integrity of media and signalling. The key management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The key management server also supports interactions with the corresponding key management server in distributed SEAL deployments.

NOTE:	The functionality of the key management server is specified in subclause 5.3 of 3GPP TS 33.434 [29].

### 13.2.5	Reference points description

#### 13.2.5.1	General

The reference points for the functional model for key management are described in the following subclauses.

#### 13.2.5.2	KM-UU

The interactions related to key management functions between the key management client and the key management server are supported by KM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

KM-UU reference point provides a means for the key management server to provide security related information (e.g. encryption keys) to the key management client. The KM-UU reference point shall use the HTTP-1 and HTTP-2 signalling control plane reference points for transport and routing of security related information to the key management client.

NOTE:	KM-UU reference point is specified in subclause 5.1.1.4 of 3GPP TS 33.434 [29].

#### 13.2.5.3	KM-PC5

The interactions related to key management functions between the key management clients located in different VAL UEs are supported by KM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

#### 13.2.5.4	KM-C

The interactions related to key management functions between the VAL client(s) and the key management client within a VAL UE are supported by KM-C reference point.

#### 13.2.5.5	KM-S

The interactions related to key management functions between the VAL server(s) and the key management server are supported by KM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

KM-S reference point provides a means for the key management server to provide security related information (e.g. encryption keys) to the VAL server. The KM-S reference point shall use the HTTP-1 and HTTP-2 signalling control plane reference points for transport and routing of security related information to the VAL server.

NOTE:	KM-S is specified in subclause 5.1.1.4 of 3GPP TS 33.434 [29].

#### 13.2.5.6	KM-E

The interactions related to key management functions between the key management servers in a distributed deployment are supported by KM-E reference point.

Editor's Note:	The functions enabled over KM-E reference point is FFS.

#### 13.2.5.7	SEAL-X1

NOTE:	SEAL-X1 reference point between the key management server and the group management server is described in subclause 6.5.9.2.

## 13.3	Procedures and information flows for key management

NOTE:	The procedure for key management is specified in subclause 5.3 of 3GPP TS 33.434 [29].

## 13.4	SEAL APIs for key management

### 13.4.1	General

The SEAL APIs for Key Management are specified in subclauses 5.7.1 and 7.6.1 of 3GPP TS 29.549 [30].

### 13.4.2	Void

#### 13.4.2.1	Void

#### 13.4.2.2	Void

# 14	Network resource management

## 14.1	General

The network resource management is a SEAL service that offers the network resource management (e.g. unicast and multicast network resources) and monitoring related capabilities to one or more vertical applications.

## 14.2	Functional model for network resource management

### 14.2.1	General

The functional model for the network resource management is based on the generic functional model specified in clause 6. It is organized into functional entities to describe a functional architecture which addresses the support for network resource management aspects for vertical applications. The on-network and off-network functional model is specified in this clause.

### 14.2.2	On-network functional model description

#### 14.2.2.1	Generic on-network functional model for network resource management

Figure 14.2.2-1 illustrates the generic on-network functional model for network resource management.

Figure 14.2.2.1-1: On-network functional model for network resource management

The network resource management client communicates with the network resource management server over the NRM-UU reference point. The network resource management client provides the support for network resource management functions to the VAL client(s) over NRM-C reference point. The VAL server(s) communicate with the network resource management server over the NRM-S reference point.

The network resource management server obtains and controls the multicast/broadcast resource from the underlaying 3GPP network system by communicating with the BM-SC via MB2-C and xMB-C reference points towards LTE eMBMS and with the 5G MBS control plane functions via Nmb13, Nmb10 or N33 towards the 5GS. The network resource management server communicates with the PCRF via Rx reference point or communicates with the PCF via N5 reference point to control the unicast resources from the underlying 3GPP network system. The network resource management server communicates with the SCEF via T8 reference point or communicates with the NEF via N33 reference point to perform event monitoring procedures from the underlying 3GPP network system. The network resource management server interacts with NEF via N33 to obtain QoS monitoring information from the 5GS.

#### 14.2.2.2	On-network functional model for network resource management for TSN

The architecture for integration of the 5G with TSN [36] is depicted in Figure 14.2.2.2-1. The SEAL Network Resource Management (NRM) server acts as a TSN AF (defined in [10] in clause 5.28.1). TSN CNC (defined in [35]) via the NRM-S reference point configures the TSN flows in the 5GS. In this case the NRM-S supports the IEEE 802.1Qcc management protocol. As a TSN AF the SEAL NRM server interacts with the 5GS PCF over the N5 reference point to configure the 5G QoS and TSCAI parameters in the 5GS.

Figure 14.2.2.2-1: On-network functional model for network resource management for TSN

NOTE: Whether DS-TT and UE are combined or are separate is up to implementation.

Acting as the TSN AF the NRM server collects 5GS TSN Bridge information by interaction with the 5GS via the N5 reference point, as described in in TS 23.502 [11] Annex F.1.

NRM server triggers via N5 the AF request procedure as described in 3GPP TS 23.502 [11] Annex F.2.

#### 14.2.2.3	On-network functional model for network resource management for 5G TSC

5G TSC refers to time sensitive communication service offered within the 5G system (i.e. without integration with a TSN system) by the 5GS for the UEs connected to the 5GS. The architecture for the 5G TSC is depicted in Figure 14.2.2.3-1. The SEAL NRM server acts as an AF towards the 5G Core Network and performs coordination of QoS flows to fulfill the end-to-end QoS requirements for the UEs involved in the TSC communication. It combines the roles of TSCTSF and TSC CNC (similar to the TSN CNC in the TSN integration case), which means that it controls the allocation of resources of TSC communication within the boundaries of the 5G domain.

Upon request from a VAL server via the NRM-S reference point it configures the TSC end-to-end QoS flows in the 5GS. In line with other SEAL service enablers the SEAL NRM server provides a RESTful interface on the NRM-S reference point. As a TSCTSF the SEAL NRM server interacts with the 5GS PCF over the N5 reference point to configure the 5G QoS and TSCAI parameters in the 5GS.

Figure 14.2.2.3-1: On-network functional model for network resource management for 5G TSC

### 14.2.3	Off-network functional model description

Figure 14.2.3-1 illustrates the off-network functional model for network resource management.

Figure 14.2.3-1: Off-network functional model for network resource management

The network resource management client of the UE1 communicates with the network resource management client of the UE2 over the NRM-PC5 reference point.

Editor's note:	Whether off-network support is required for network resource management is FFS.

### 14.2.4	Functional entities description

#### 14.2.4.1	General

The functional entities for network resource management SEAL service are described in the following subclauses.

#### 14.2.4.2	Network resource management client

The network resource management client functional entity acts as the application client for the management of network resources. The network resource management client interacts with the network resource management server.

#### 14.2.4.3	Network resource management server

The network resource management server functional entity provides for management of 3GPP system network resources (e.g. unicast, multicast) and monitoring events to support the VAL applications. The network resource management server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8]. The network resource management server also supports interactions with the corresponding network resource management server in distributed SEAL deployments. The NRM server's role may be assumed by the VAL server in some deployments, in which case, the VAL server performs the procedures for network resource management of the NRM server.

### 14.2.5	Reference points description

#### 14.2.5.1	General

The reference points for the functional model for network resource management are described in the following subclauses.

#### 14.2.5.2	NRM-UU

The interactions related to network resource management functions between the network resource management client and the network resource management server are supported by NRM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

#### 14.2.5.3	NRM-PC5

The interactions related to network resource management functions between the network resource management clients located in different VAL UEs are supported by the NRM-PC5 reference point. This reference point utilizes PC5 reference point as described in 3GPP TS 23.303 [12].

Editor's note:	Whether NRM-PC5 reference point is required for network resource management is FFS.

#### 14.2.5.4	NRM-C

The interactions related to network resource management functions between the VAL client(s) and the network resource management client within a VAL UE are supported by the NRM-C reference point.

Editor's note:	Whether NRM-C reference point is required for network resource management is FFS.

#### 14.2.5.5	NRM-S

The interactions related to network resource management functions between the VAL server(s) and the network resource management server are supported by the NRM-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

#### 14.2.5.6	NRM-E

The interactions related to network resource management functions between the network resource management servers in a distributed deployment are supported by NRM-E reference point.

Editor's Note:	The functions enabled over NRM-E reference point is FFS.

#### 14.2.5.7	MB2-C

The reference point MB2-C supports the control plane interactions between the network resource management server and the BM-SC and is specified in 3GPP TS 29.468 [22].

#### 14.2.5.8	xMB-C

The reference point xMB-C supports the control plane interactions between the network resource management server and the BM-SC and is specified in 3GPP TS 26.348 [20].

#### 14.2.5.9	Rx

The reference point Rx supports the interactions between the network resource management server and the PCRF and is specified in 3GPP TS 29.214 [21].

#### 14.2.5.10	N5

The reference point N5 supports the interactions between the network resource management server and the PCF and is specified in 3GPP TS 23.501 [10].

#### 14.2.5.11	N33

The reference point N33 supports the interactions between the network resource management server and the NEF and is specified in 3GPP TS 23.501 [10].

#### 14.2.5.12	Nmb13

The reference point Nmb13 supports the interactions between the network resource management server and the MB-SMF is specified in 3GPP TS 23.247 [39].

#### 14.2.5.13	Nmb10

The reference point Nmb10 supports the interactions between the network resource management server and the MBSF is specified in 3GPP TS 23.247 [39].

#### 14.2.5.14	N6mb

The reference point N6mb supports the interactions between the VAL server and the MB-UPF is specified in 3GPP TS 23.247 [39].

#### 14.2.5.15	Nmb8

The reference point Nmb8 supports the interactions between the VAL server and the MBSTF is specified in 3GPP TS 23.247 [39].

#### 14.2.5.16	N6

The reference point N6 supports the interactions between the VAL server and the UPF is specified in 3GPP TS 23.501 [10].

## 14.3	Procedures and information flows for network resource management

### 14.3.1	General

The procedures related to the network resource management are described in the following subclauses.

### 14.3.2	Information flows

#### 14.3.2.1	Network resource adaptation request

Table 14.3.2.1-1 describes the information flow network resource adaptation request from the VAL server to the NRM server.

Table 14.3.2.1-1: Network resource adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE) | List consisting of one or more VAL UE IDs for whom the network resource adaptation occurs. |
| VAL group ID | O (see NOTE) | The VAL group ID for whom the network resource adaptation occurs. |
| Resource adaptation requirement | M | The resource adaptation requirement corresponds to the VAL service QoS requirements as applied for a UE or group of UEs (E.g. bandwidth, resource). |
| NOTE: Either of the information elements should be present. |  |  |

#### 14.3.2.2	Network resource adaptation response

Table 14.3.2.2-1 describes the information flow network resource adaptation response from the NRM server to the VAL server.

Table 14.3.2.2-1: Network resource adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network resource adaptation with the underlying network. The response can also include an updated value for some of the parameters included in the network resource adaptation request (e.g. negotiation of resource offering) |

#### 14.3.2.3	MBMS bearer announcement

Table 14.3.2.3-1 describes the information flow MBMS bearer announcement from the NRM server to the NRM client.

Table 14.3.2.3-1: MBMS bearer announcement

| Information element | Status | Description |
| --- | --- | --- |
| TMGI | M | TMGI information |
| Alternative TMGI | O | A list of additional alternative TMGI may be included and used in roaming scenarios. |
| QCI | O | QCI information used by the ProSe UE-Network Relay to determine the ProSe Per-Packet Priority value to be applied for the multicast packets relayed to Remote UE over PC5 |
| List of service area identifier | M | A list of service area identifier for the applicable MBMS broadcast area. |
| Frequency | O | Identification of frequency if multi carrier support is provided |
| SDP information | M | SDP with media and application control information applicable to groups that can use this bearer (e.g. codec, protocol id, FEC information) |
| Monitoring state | O | The monitoring state is used to control if the client is actively monitoring the MBMS bearer quality or not. |
| Announcement acknowledgment | O | Indicate if the NRM server requires an acknowledgement of the MBMS bearer announcement. |
| Unicast status | O | An indication that the listening status of the unicast bearer is requested. |
| ROHC information | O | Indicate the usage of ROHC and provide the parameters of the ROHC channel to signal to the ROHC decoder. |
| NOTE: When MBMS bearer announcement is done on a MBMS bearer all attributes above are optional except the TMGI. |  |  |

#### 14.3.2.4	MBMS listening status report

Table 14.3.2.4-1 describes the information flow for the MBMS listening status report from NRM client to NRM server.

Table 14.3.2.4-1: MBMS listening status report

| Information element | Status | Description |
| --- | --- | --- |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE who wants to report the MBMS listening status. |
| TMGI(s) | M | TMGI(s) information. |
| MBMS listening status(s) | M | The MBMS listening status per TMGI. |
| MBMS reception quality level | O | The reception quality level per TMGI (see NOTE) |
| Unicast listening status | O | The unicast listening status. |
| NOTE: The set of quality levels helps service continuity in MBMS scenarios. A reception quality level may help to make an efficient switching decision to another bearer. How these levels are used is implementation specific. |  |  |

#### 14.3.2.5	MBMS suspension reporting instruction

Table 14.3.2.5-1 describes the information flow for the MBMS suspension reporting instruction from NRM server to NRM client in a unicast bearer for MBMS suspension reporting.

Table 14.3.2.5-1: MBMS suspension reporting instruction (unicast)

| Information element | Status | Description |
| --- | --- | --- |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE. |
| Suspension reporting | M | Enables or disable the suspension reporting for a specific NRM client |

Table 14.3.2.5-2 describes the information flow for the MBMS suspension reporting instruction from NRM server to NRM client in a multicast bearer for MBMS suspension reporting.

Table 14.3.2.5-2: MBMS suspension reporting instruction (multicast)

| Information element | Status | Description |
| --- | --- | --- |
| Suspension reporting client subset | M | Contains a uniquely defined subset of NRM clients that shall report MBMS suspension |

#### 14.3.2.6	Resource request

Table 14.3.2.6-1 describes the information flow for the resource request from VAL server to NRM server for unicast resources.

Table 14.3.2.6-1: Resource request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE. |
| VAL service requirement information (see NOTE) | O | VAL service requirements for unicast resource (e.g. VAL service ID, Bitrate) |
| NOTE: When this information element is not included, the NRM server considers default VAL service requirement for the unicast resources. |  |  |

#### 14.3.2.7	Resource response

Table 14.3.2.7-1 describes the information flow for the resource response from NRM server to VAL server for unicast resources.

Table 14.3.2.6-1: Resource response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource request operation. |

#### 14.3.2.8	Resource modification request

Table 14.3.2.8-1 describes the information flow for the resource modification request from VAL server to NRM server for unicast resources.

Table 14.3.2.8-1: Resource modification request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL user ID or VAL UE ID | M | The identity of the VAL user or VAL UE. |
| VAL service requirement information | M | VAL service requirements for unicast resource (e.g. VAL sevice ID, Bitrate) |

#### 14.3.2.9	Resource modification response

Table 14.3.2.9-1 describes the information flow for the resource modification response from NRM server to VAL server for unicast resources.

Table 14.3.2.9-1: Resource modification response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the resource modification operation. |

#### 14.3.2.10	MBMS bearers request

Table 14.3.2.10-1 describes the information flow for the MBMS bearers request from VAL server to NRM server.

Table 14.3.2.10-1: MBMS bearers request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL group ID | M | The identity of the group that the MBMS bearer is requested for. |
| Service announcement mode | M | Indicates whether the request is sent by NRM server or by the VAL server |
| QoS | M | Indicates the requested QoS for the bearer |
| Broadcast area | O | Indicate the area where the MBMS bearer is requested for |
| Endpoint information | M | Information of the endpoint of the VAL server to which the user plane notifications have to be sent. |
| Local MBMS information (see NOTE) | O |  |
| > MB2-U information | O | IP address, UDP port number of the MB2-U interface |
| > xMB-U information | O | IP address, UDP port number of the xMB-U interface |
| > M1 interface information | O | M1 interface information for local MBMS |
| Local MBMS activation indication (see NOTE) | O | Indicates whether to request the NRM server to use Local MBMS information. |
| NOTE: The VAL server may provide either the Local MBMS information or the Local MBMS activation indication. This IE is present when the local MBMS is required for VAL services like V2X service. |  |  |

#### 14.3.2.11	MBMS bearers response

Table 14.3.2.11-1 describes the information flow for the MBMS bearers response from NRM server to VAL server.

Table 14.3.2.11-1: MBMS bearers response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the MBMS bearers request operation. |
| TMGI | O (see NOTE 1) | TMGI information. |
| User plane address | M (see NOTE 2) | BM-SC user plane IP address and port |
| Service description | O (see NOTE 2) | Indicates MBMS bearer related configuration information as defined in 3GPP TS 26.346 [28] (e.g. radio frequency and MBMS Service Area Identities) |
| NOTE 1: TMGI may not be required if the service announcement mode indicates that the request is sent by the NRM server. NOTE 2: If the Result Information element indicates failure then the values of the other information elements have no meaning. |  |  |

#### 14.3.2.12	User plane delivery mode

Table 14.3.2.12-1 describes the information flow for the user plane delivery mode from NRM server to VAL server.

Table 14.3.2.12-1: User plane delivery mode

| Information element | Status | Description |
| --- | --- | --- |
| Delivery mode | M | Indicates whether to deliver the user data to the UE(s) via unicast mode or multicast mode |
| MBMS media stream identifier | M | Indicates the MBMS media stream to be used to deliver the media currently over unicast, or the MBMS media stream currently being used. |
| Unicast media stream identifier(s) | M | Indicates the unicast media stream to be used to deliver the media currently over multicast, or the unicast to be stopped and switched to multicast. |

#### 14.3.2.13	end-to-end QoS management request

Table 14.3.2.13-1 describes the information flow end-to-end QoS management request from the NRM client to the NRM server.

Table 14.3.2.13-1: end-to-end QoS management request

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UEs | M | List of VAL UEs for whom the end-to-end QoS management occurs |
| >VAL UE/user ID | M | Identity of the VAL UE |
| > IP address | M | IP address of the VAL UE |
| VAL service ID | O | The VAL service identity for whom the end-to-end QoS management occurs. |
| End-to-end QoS requirements | O | The application QoS requirements / KPIs (latency, error rate, ..) for the end-to-end session. This may optionally include information which will support the NRM server to identify the per session QoS requirements (e.g. a flag indicating the use of HD video for assisting the end-to-end session, a video resolution/encoding required for the HD video). |
| Service area | O | The area where the QoS management request applies. This can be geographical area, or topological area. |
| Time validity | O | The time of validity of the requirement. |

#### 14.3.2.14	end-to-end QoS management response

Table 14.3.2.14-1 describes the information flow end-to-end QoS management response from the NRM server to the NRM client.

Table 14.3.2.14-1: end-to-end QoS management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the end-to-end QoS management request. |
| QoS report configuration | O | The configuration of the NRM client's report triggering by NRM server (e.g. setting thresholds for reporting a QoS downgrade / notifications based on channel loss great than threshold value) |

#### 14.3.2.15	QoS downgrade indication

Table 14.3.2.15-1 describes the information flow QoS downgrade indication from the NRM client to the NRM server.

Table 14.3.2.15-1: QoS downgrade indication

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID | M | The identifier of the VAL UE which indicates the QoS downgrade. |
| QoS downgrade report | M | The report including the expected or actual QoS / QoE parameters which were downgraded (i.e. latency, throughput, reliability, jitter). This report may be configured by the end-to-end QoS management response message. |

#### 14.3.2.16	Application QoS change notification

Table 14.3.2.16-1 describes the information flow Application QoS change notification from the NRM server to the NRM clients (which are involved at the end-to-end session).

Table 14.3.2.16 -1: Application QoS change notification

| Information element | Status | Description |
| --- | --- | --- |
| NRM server ID | M | The identifier of the NRM server which provides the notification. |
| Requested QoS parameters | M | The updated requested QoS parameters for the end-to-end session (NRM server to source VAL UE or NRM server to target VAL UE), based on the QoS change on one or both links involved in the network-assisted end-to-end communication. |

#### 14.3.2.17	Monitoring Events Subscription Request

Table 14.3.2.17-1 describes the information flow from the VAL server to the NRM server for monitoring events subscription request.

Table 14.3.2.17-1: Monitoring Events Subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identities list | O (see NOTE 1) | List of VAL users or VAL UEs whose events monitoring is requested. |
| VAL group ID | O (see NOTE 1) | VAL group ID of the VAL UE group of target UEs. |
| VAL service ID | O | VAL service ID. |
| Monitoring profile ID | O (see NOTE 2) | The monitoring profile ID, which identifies a list of monitoring and/or analytics events. |
| Validity conditions | O | The temporal and/or spatial conditions applied for the events to be considered as valid. |
| Event Details | O (see NOTE 2) | List of monitoring and/or analytics events that the VAL server is interested in. |
| NOTE 1: For identifying the target UE(s), either a list of VAL users/UEs or a group of VAL UEs shall be provided. NOTE 2: Either Event Details or Monitoring profile ID is present. |  |  |

#### 14.3.2.18	Monitoring Events Subscription Response

Table 14.3.2.18-1 describes the information flow from the NRM server to the VAL server for Monitoring Events Subscription response.

Table 14.3.2.18-1: Monitoring Events Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Monitoring profile ID | O | It indicates the monitoring profile identifier. It is present when Event Details are provided in the Monitoring Events Subscription request. |

#### 14.3.2.19	Monitoring Events Notification message

Table 14.3.2.19-1 describes the information flow from the NRM server to the VAL server on notification of monitoring events.

Table 14.3.2.19-1: Monitoring Events Notification

| Information element | Status | Description |
| --- | --- | --- |
| EventDetails |  | List of events related to VAL UE(s). |
| > identity | M | VAL UE for which the events are related to. |
| > events | M | List of Monitoring and Analytics events related to the VAL UE. |
| Timestamp | O | The timestamp for the monitoring and analytics events |

#### 14.3.2.20	Unicast QoS monitoring subscription request

Table 14.3.2.20-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription request.

Table 14.3.2.20-1: Unicast QoS monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE 1 NOTE 4) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE 1, NOTE 4) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE 1, NOTE 4) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | O | It describes the requirements on the QoS measurements. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates a current time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| Reporting requirements | O | It describes the requirements for QoS monitoring reporting. |
| > Frequency of reporting | O | It indicates the requested frequency of reporting. The reporting frequency may be periodic or event triggered (i.e. threshold reached). If absent in the request, default event triggered reporting is used. |
| > Reporting threshold | O | If the Frequency of reporting is event triggered (threshold reached), the reporting threshold corresponding to the measurement index shall be provided. |
| > Threshold reaching direction | O | It indicates the reaching direction (i.e. ascending, descending or crossed) for the reporting threshold. It shall be provided if reporting threshold is present. |
| > Reporting periodicity | O | If the Frequency of reporting is periodic, the reporting periodicity shall be provided. |
| > Termination of reporting | O | It indicates when the reporting of QoS monitoring shall stop. It can be event triggered (i.e. either number of reports reached or threshold reached) or when reporting duration expires or when explicit termination is requested. If absent in the request, it defaults to explicit termination. Once the termination condition is met, the monitoring subscription ceases to exist. |
| > Max number of reports | O (NOTE 3) | It indicates the maximum number of reports, it shall be provided when Termination of reporting is set to event triggered (number of reports reached). |
| > Termination threshold | O (see NOTE 2, NOTE 3) | It indicates the reporting termination threshold corresponding to the measurement index. Each measurement index has only one termination threshold. |
| > Termination condition for termination threshold | O (see NOTE 2) | It indicates threshold-based termination condition (e.g. all reached or any of them reached) if multiple measurement indexes are provided. |
| > Reporting duration | O (NOTE 3) | It indicates the reporting duration, it shall be provided when Termination of reporting is set to duration expiration. |
| NOTE 1: Only one of these information elements shall be present. NOTE 2: It shall be provided when Termination of reporting is set to event triggered (threshold reached). NOTE 3: Only one of these information elements may be present. NOTE 4: When used by the Update_Unicast_QoS_Monitoring_Subscription operation, the same information element with identical content shall be present as in the associated Subscribe_Unicast_QoS_Monitoring operation. |  |  |

#### 14.3.2.21	Unicast QoS monitoring subscription response

Table 14.3.2.21-1 describes the information flow from the network resource management server to the VAL server for unicast QoS montoring subscription response.

Table 14.3.2.21-1: Unicast QoS monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |

#### 14.3.2.22	Unicast QoS monitoring notification

Table 14.3.2.22-1 describes the information flow for unicast QoS monitoring notification from the network resource management server to the VAL server.

Table 14.3.2.22-1: Notify unicast QoS monitoring event

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UE IDs | O (see NOTE) | List of VAL UEs whose QoS monitoring data is provided. This list is either the list provided in the subscription request or a subset of that list. It can contain a single UE. |
| VAL group ID | O (see NOTE) | The group ID used for the VAL group for which QoS monitoring data is provided. This group ID shall be the same VAL group ID as the VAL group ID provided in the subscription request. |
| List of VAL stream IDs | O (see NOTE) | List of VAL stream IDs whose QoS monitoring data is provided. This list is either the list provided in the subscription request or a subset of that list. It can contain a single stream ID. |
| QoS monitoring data | M | QoS monitoring data is an aggregate of QoS measurements data obtained from the 5GS. |
| NOTE: Only one of these information elements shall be present. |  |  |

#### 14.3.2.23	TSC stream availability discovery request

Table 14.3.2.23-1 describes the information flow TSC stream availability discovery request from the VAL server to the NRM server.

Table 14.3.2.23-1: TSC stream availability discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports (e.g. as defined in IEEE 802.1CB [37]). |

#### 14.3.2.24	TSC stream availability discovery response

Table 14.3.2.24-1 describes the information flow TSC stream availability discovery response from the NRM server to the VAL server.

Table 14.3.2.24-1: TSC stream availability discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the TSC stream availability discovery with the underlying network. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports (e.g. as defined in IEEE 802.1CB [37]). |
| List of traffic specifications | M | The traffic classes supported by the DS-TTs and available end-to-end MaxLatency value per traffic class. The VAL server should not request lower latency than the available end-to-end latency. |

#### 14.3.2.25	TSC stream creation request

Table 14.3.2.25-1 describes the information flow TSC stream creation request from the VAL server to the NRM server.

Table 14.3.2.25-1: TSC stream creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination DS-TT ports (e.g. as defined in IEEE 802.1CB [37]). |
| Traffic specification | M | It includes MaxLatency, MaxFrameInterval, MaxFrameSize, MaxIntervalFrames, etc. (e.g. as described in IEEE 802.1Qcc [IEEE8021Qcc] in clause 46.2). |

#### 14.3.2.26	TSC stream creation response

Table 14.3.2.26-1 describes the information flow TSC stream creation response from the NRM server to the VAL server.

Table 14.3.2.26-1: TSC stream creation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the TSC stream creation. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.27	TSC stream deletion request

Table 14.3.2.27-1 describes the information flow TSC stream deletion request from the VAL server to the NRM server.

Table 14.3.2.27-1: TSC stream deletion request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.28	TSC stream deletion response

Table 14.3.2.28-1 describes the information flow TSC stream deletion response from the NRM server to the VAL server.

Table 14.3.2.28-1: TSC stream deletion response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network TSC stream deletion. Even in case of failure, the stream is deemed unusable. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.29	TSN bridge information report

The 5GS bridge management information is defined in TS 23.501 [10], clause 5.28.1 (5GS bridge management).

#### 14.3.2.30	TSN bridge information confirmation

The 5GS bridge management information is defined in TS 23.501 [10], clause 5.28.1 (5GS bridge management).

#### 14.3.2.31	TSN bridge configuration request

The configuration information of 5GS Bridge is defined in TS 23.501 [10], clause 5.28.2 (The configuration information of 5GS Bridge).

#### 14.3.2.32	TSN bridge configuration response

The configuration information of 5GS Bridge is defined in TS 23.501 [10], clause 5.28.2 (The configuration information of 5GS Bridge).

#### 14.3.2.33	Unicast QoS monitoring data request

Table 14.3.2.33-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring data request.

Table 14.3.2.33-1: Unicast QoS monitoring data request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| List of VAL UE IDs | O (see NOTE) | List of VAL UEs whose QoS monitoring data is requested. |
| VAL group ID | O (see NOTE) | The group ID used for the VAL group for which QoS monitoring data is requested. |
| List of VAL stream IDs | O (see NOTE) | List of VAL streams for which QoS monitoring data is requested. |
| Measurement requirements | M | It describes the requirements on the QoS measurement data to be provided. |
| > type of data | M | At least one of the following measurement index shall be provided. |
| >> packet delay | O | UL/DL/RT packet delay. For VAL stream, the packet delay means measurement from a source UE to a target UE. |
| >> packet loss rate | O | Average packet loss rate. |
| >> data rate | O | Average data rate and/or maximum data rate. |
| >> traffic volume | O | Average traffic volume for UL and/or DL |
| > measurement time period | O | The measurement time period indicates either a current or past time period. If absent, current time and 5 minutes duration are used as default setting. |
| > aggregation granularity window | O | Indicates how long the QoS monitoring is performed to compute the average value and gather the maximum value. If absent, 1 minute is used as default setting. |
| NOTE: Only one of these information elements shall be present. |  |  |

#### 14.3.2.34	Unicast QoS monitoring data response

Table 14.3.2.34-1 describes the information flow from the VAL server to the network resource management server for unicast QoS monitoring data response.

Table 14.3.2.34-1: Unicast QoS monitoring data response

| Information element | Status | Description |
| --- | --- | --- |
| List of VAL UE IDs | O (see NOTE 1) | List of VAL UEs whose QoS monitoring data is provided. This list is either the list provided in the request or a subset of that list. It can contain a single UE. |
| VAL group ID | O (see NOTE 1) | The group ID used for the VAL group for which QoS monitoring data is provided. This group ID shall be the same VAL group ID as the VAL group ID provided in the request. |
| List of VAL stream IDs | O (see NOTE 1) | List of VAL stream IDs whose QoS monitoring data is provided. This list is either the list provided in the request or a subset of that list. It can contain a single stream ID. |
| Result | M | Indicates the success or failure for the operation |
| QoS monitoring data | M (see NOTE 2) | QoS monitoring data is an aggregate of QoS measurements data obtained from the 5GS. |
| Cause | O | Provides reason for the failure (e.g. when data for the requested time period is not available). |
| NOTE 1: Only one of these information elements shall be present. NOTE 2: If the Result information element indicates failure then the value of the QoS monitoring data information element has no meaning. |  |  |

#### 14.3.2.35	Application connectivity request

Table 14.3.2.35-1 describes the information flow for application connectivity request from a NRM client to the NRM server.

Table 14.3.2.35-1: Application connectivity request

| Information element | Status | Description |
| --- | --- | --- |
| Source UE ID | M | Identifier of the requestor UE |
| VAL service ID | M | Identifier of the VAL service |
| List of destination UE IDs | M | Identifiers of the destination UEs |
| Application service requirements | M | Details of service requirements |
| >Packet size (see NOTE) | O | Size of the packet to be transmitted. |
| >Packet transmission interval (see NOTE) | O | Intervals at which the packet is to be transmitted |
| >Packet E2E latency (see NOTE) | O | E2E Latency for packet transmission |
| >Packet error KPI (see NOTE) | O | The KPIs related to packet error (e.g. allowed packet loss rate, packet loss amount, packet error rate) |
| >Bitrate (see NOTE) | O | The bitrate required |
| NOTE: At least one of information elements of the application service requirements shall be included. |  |  |

#### 14.3.2.36	Application connectivity response

Table 14.3.2.36-1 describes the information flow for application connectivity response from the NRM server to the NRM client.

Table 14.3.2.36-1: Application connectivity response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the NRM server in response to request indicating success or failure for accepting the request. |
| Cause | O | The cause of failure, if the result indicates failure. |

#### 14.3.2.37	Application connectivity notification

Table 14.3.2.37-1 describes the information flow for application connectivity notification from the NRM server to the NRM client.

Table 14.3.2.37-1: Application connectivity notification

| Information element | Status | Description |
| --- | --- | --- |
| Session information | M | Session information for the established connectivity for application communication |
| VAL service ID | M | Identifier of the VAL service |
| List of accepted UE IDs (see NOTE) | O | The list of UEs which are accepted for the connectivity established for the application service communication |
| NOTE: The List of accepted UE IDs IE shall be included when the application connectivity notification is sent to the initiator of the application connectivity request. |  |  |

#### 14.3.2.38	Unicast QoS monitoring subscription update request

The information flow from the VAL server to the network resource management server for unicast QoS monitoring subscription update request has the same content as unicast QoS monitoring subscription request specified in clause 14.3.2.20.

#### 14.3.2.39	Unicast QoS monitoring subscription update response

Table 14.3.2.39-1 describes the information flow from the network resource management server to the VAL server for unicast QoS montoring subscription update response.

Table 14.3.2.39-1: Unicast QoS monitoring subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Update status | M | It indicates the subscription update result |

### 14.3.3	Unicast resource management

#### 14.3.3.1	General

The following subclauses specify the procedures for unicast resource management for vertical application layer. The NRM server sets up bearers and may need to modify the bearers for an already established VAL service communication.

Characteristics that may need to be modified include:

-	activation and deactivation of the bearer;

-	modification of the QoS characteristics of the bearer (e.g. bearer priority adjustment); and

-	modification of GBR due to application requirement

NOTE:	A VAL service communication can consist of both unicast and multicast bearers which can all need modification due to the same event.

Vertical application layer specific pre-requisites and resultant behaviour by functional entities in performing these procedures are specified in the respective vertical application layer TS (e.g. for V2X application layer, see 3GPP TS 23.286 [7]).

Unicast resource management is supported with PCC interactions with SIP core and PCC interactions with NRM server. The PCC procedures for EPS are specified in 3GPP TS 23.203 [18] and the PCC procedures for 5GS are specified in 3GPP TS 23.503 [19].

#### 14.3.3.2	Unicast resource management with SIP core

##### 14.3.3.2.1	Request for unicast resources at VAL service communication establishment

###### 14.3.3.2.1.1	General

The procedure defined in this subclause specifies how network resources are requested at VAL service communication establishment. If concurrent sessions are used the NRM server may utilize the capability of resource sharing specified for underlying network policy and charging functions. The request for resources includes application type, bandwidth, priority, application identifier and resource sharing information.

###### 14.3.3.2.1.2	Procedure

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 14.3.3.2.1.2-1 are the signalling procedures for the requesting resource at session establishment.

Pre-condition:

-	The VAL client has requested VAL service communication with the VAL server.

Figure 14.3.3.2.1.2-1: Resource request at VAL service communication establishment

1.	The VAL server sends request for resources to the NRM server.

2.	The NRM server evaluates the need for network resources and use of resource sharing.

3.	The NRM server sends a session progress request containing request for resources.

4.	PCC procedures are initiated from SIP core local inbound/outbound proxy.

5.	The SIP core local inbound / outbound proxy sends a OK message to the NRM server.

6.	The NRM server sends a resource response to the VAL server.

7.	The VAL service communication is established, and resources have been allocated.

##### 14.3.3.2.2	Request for modification of unicast resources

###### 14.3.3.2.2.1	General

To modify unicast bearers, the NRM server shall send a resource modification request containing the parameters to be modified for the UE.

Possible scenarios when this procedure may be used are:

-	Modify the allocation and retention priority for unicast resources;

-	Release and resume resources in-between VAL service communications; or

-	Release and resume resources when a UE is able to receive the VAL service communications over multicast transmission

###### 14.3.3.2.2.2	Procedure

Procedures in figure 14.3.3.2.2.2-1 are the signalling procedures for the modification of a unicast:

Pre-condition:

-	A VAL service communication is already in progress;

Figure 14.3.3.2.2.2-1: Bearer modification request

1.	The VAL server sends a resource modification request to the NRM server.

2a.	The NRM server decides to modify the parameters of a unicast bearer.

2b.	If the media bearer modification is not required, the NRM server sends a resource modification response to the VAL server.

3.	The NRM server sends a session update which includes a resource modification request containing the modified parameters of the unicast bearer.

4.	PCC procedures are initiated from SIP core local inbound/outbound proxy.

5.	The SIP core local inbound / outbound proxy sends a OK message to the NRM server.

6.	The NRM server sends a resource modification response to the VAL server.

7.	The VAL service communication continues with the modified unicast resources.

NOTE 1:	If the VAL service communication is transferred to multicast transmission, the unicast resources could be temporarily be released.

NOTE 2:	If multiple VAL service communication streams are sent to the UE, additional bearer resources could be required during an established VAL service communication. Pre-allocation of additional bearer resources already at VAL service communication establishment could be useful.

#### 14.3.3.3	Unicast resource management without SIP core

##### 14.3.3.3.1	Network resource adaptation

###### 14.3.3.3.1.1	General

This subclause describes the procedure for network resource adaptation using PCC procedures. This procedure satisfies the requirements for requesting unicast resources and modification to already allocated unicast resources to VAL communications.

###### 14.3.3.3.1.2	Procedure

Figure 14.3.3.3.1.2-1 illustrates the procedure for the network resource adaptation.

Figure 14.3.3.3.1.2-1: Network resource adaptation

1.	The VAL server sends a network resource adaptation request to the NRM server for one or more users belonging to one or more VAL services, and may comprise one or more VAL UEs that will have updated resource requirement. This requirement may be in the form of exact resources /resource pools to be used or indication of bandwidth increase/decrease for the corresponding VAL UEs or set of VAL UEs.

2.	The NRM server processes the request and applies / enforces the resource adaptation per VAL UE.

3.	The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status) and initiates the PCC procedures for each VAL UE.

4.	The NRM server provides a network resource adaptation response to the VAL server, providing information on the fulfilment of the network resource adaptation request. This will include information either per VAL UE or per set of VAL UEs, as indicated by the request of the VAL server in step 1.

##### 14.3.3.3.2	Request for unicast resources at VAL service communication establishment

###### 14.3.3.3.2.1	General

The procedure defined in this subclause specifies how network resources are requested at VAL service communication establishment. If concurrent sessions are used the NRM server may utilize the capability of resource sharing specified for underlying network policy and charging functions. The request for resources includes application type, bandwidth, priority, application identifier and resource sharing information.

###### 14.3.3.3.2.2	Procedure

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 14.3.3.3.2.2-1 are the signalling procedures for the requesting resource at session establishment.

Pre-condition:

-	The VAL client has requested VAL service communication with the VAL server.

Figure 14.3.3.3.2.2-1: Resource request at VAL service communication establishment

1.	The VAL server sends request for resources to the NRM server.

2.	The NRM server evaluates the need for network resources and use of resource sharing.

3.	The NRM server retrieves UE IP address by using event monitoring capability for PDU session status (or PDN connectivity status), and then PCC procedures are initiated from NRM server.

4.	The NRM server sends a resource response to the VAL server.

5.	The VAL service communication is established, and resources have been allocated.

##### 14.3.3.3.3	Request for modification of unicast resources

###### 14.3.3.3.3.1	General

To modify unicast resources, the NRM server shall send a resource modification request containing the parameters to be modified for the UE.

Possible scenarios when this procedure may be used are:

-	Modify the allocation and retention priority for unicast resources;

-	Release and resume resources in-between VAL service communications; or

-	Release and resume resources when a UE is able to receive the VAL service communications over multicast transmission

###### 14.3.3.3.3.2	Procedure

Procedures in figure 14.3.3.3.3.2-1 are the signalling procedures for the modification of a unicast:

Pre-condition:

-	A VAL service communication is already in progress;

Figure 14.3.3.3.3.2-1: Bearer modification request

1.	The VAL server sends a resource modification request to the NRM server.

2a.	The NRM server decides to modify the parameters of a unicast bearer.

2b.	If the media bearer modification is not required, the NRM server sends a resource modification response to the VAL server.

3.	PCC procedures are initiated from NRM server.

4.	The NRM server sends a resource modification response to the VAL server.

5.	The VAL service communication continues with the modified unicast resources.

NOTE 1:	If the VAL service communication is transferred to multicast transmission, the unicast resources could be temporarily be released.

NOTE 2:	If multiple VAL service communication streams are sent to the UE, additional bearer resources could be required during an established VAL service communication. Pre-allocation of additional bearer resources already at VAL service communication establishment could be useful.

#### 14.3.3.4	Unicast QoS monitoring

##### 14.3.3.4.1	Unicast QoS monitoring subscription procedure

###### 14.3.3.4.1.1	General

This subclause describes the high level procedure for unicast QoS monitoring subscription. This procedure satisfies the requirements for monitoring of unicast QoS for already allocated unicast resources to VAL communications.

###### 14.3.3.4.1.2	Procedure

Figure 14.3.3.4.1.2-1 illustrates the high level procedure for unicast QoS monitoring subscription.

Pre-conditions:

-	The VAL UE has an established connection in the 5GS

Figure 14.3.3.4.1.2-1: Unicast QoS monitoring subscription

1.	The VAL server sends a unicast QoS monitoring subscription request to the NRM server either in conjunction with a request for unicast network resources requiring QoS or when the unicast QoS connection is already established. The NRM server checks if the VAL server is authorized to initiate the unicast QoS monitoring request for the requested target VAL UEs, VAL group ID, or VAL stream IDs.

NOTE:	It is left for stage 3 to decide whether to combine the QoS monitoring subscription request with the request for unicast resources.

2.	The NRM server interacts with the NEF to establish relevant QoS monitoring subscriptions. The NRM server uses the NEF procedures for the AFsessionWithQoS described in clause 5.2.6.9 of 3GPP TS 23.502 [11] and the NEF procedures for the AnalyticsExposure described in clause 5.2.6.16 of 3GPP TS 23.502 [11] and in particular the UE Communication Analytics described in clause 6.7.3 of 3GPP TS 23.288 [34] and DN Performance Analytics described in clause 6.14 of 3GPP TS 23.288 [34]. Based on the input received from the VAL server in step 1, the NRM server determines the relevant NEF subscription procedures and the parameters for these subscriptions, such as the QoS parameters to be measured (e.g. packet delay, data rate, traffic volume), the frequency of reporting etc. For the frequency of reporting which can be event triggered, periodic, or when the PDU Session is released, the NRM server determines the following:

a)	if the reporting frequency is event triggered:

i)	the corresponding reporting threshold to each QoS parameter;

ii)	minimum waiting time between subsequent reports;

b)	if the reporting frequency is periodic, the reporting period.

3.	The NRM server responds with a unicast QoS monitoring subscription response indicating the subscription status.

##### 14.3.3.4.2	Unicast QoS monitoring notification procedure

###### 14.3.3.4.2.1	General

This subclause describes the high level procedure for unicast QoS monitoring notification.

###### 14.3.3.4.2.2	Procedure

Figure 14.3.3.4.2.2-1 illustrates the high level procedure of unicast QoS monitoring notification event.

Pre-conditions:

-	The VAL server has an active unicast QoS monitoring subscription with the NRM server

Figure 14.3.3.4.2.2-1: Unicast QoS monitoring notification procedure

1.	The NRM server receives QoS monitoring data by means of notifications provided by the NEF. The NRM server coordinates and combines the information from the NEF notifications and determines whether to send a notification to the VAL server based on the VAL server subscription's frequency of reporting. For a VAL group or a list of VAL UEs, the NRM server aggregates QoS monitoring data for each UE belonging to the group or the list; for a VAL stream, the NRM server aggregates the QoS monitoring data for the stream. The NRM server stores the QoS monitoring data as needed for later retrieval.

2.	The NRM server sends a unicast QoS monitoring notification including the measured QoS data to the VAL server. If the reporting termination criteria is met (e.g. number of reports reached, threshold reached), NRM server shall also terminate the subscription.

##### 14.3.3.4.3	Unicast QoS monitoring subscription termination procedure

###### 14.3.3.4.3.1	General

This subclause describes the high level procedure for unicast QoS monitoring subscription termination.

###### 14.3.3.4.3.2	Procedure

Figure 14.3.3.4.3.2-1 illustrates the high level procedure of unicast QoS monitoring subscription termination.

Pre-conditions:

-	The VAL server has an active unicast QoS monitoring subscription with the NRM server

Figure 14.3.3.4.3.2-1: Unicast QoS monitoring subscription termination procedure

1.	When the VAL server decides to terminate a unicast QoS monitoring subscription, it sends a QoS monitoring unsubscribe request to the NRM server.

2.	The NRM server interacts with the NEF to terminate the related QoS monitoring subscriptions.

3.	The NRM server sends a QoS monitoring unsubscribe response to the VAL server.

##### 14.3.3.4.4	Unicast QoS monitoring data retrieval procedure

###### 14.3.3.4.4.1	General

This subclause describes the high level procedure for unicast QoS monitoring data retrieval.

###### 14.3.3.4.4.2	Procedure

Figure 14.3.3.4.4.2-1 illustrates the high level procedure of unicast QoS monitoring data retrieval.

Figure 14.3.3.4.4.2-1: Retrieval of unicast QoS monitoring data

1.	The VAL server sends a unicast QoS monitoring data request to the NRM server specifying the details of the requested data and the time period of interest. The NRM server checks if the VAL server is authorized to initiate the unicast QoS monitoring data request for the requested target VAL UEs, VAL group ID, or VAL stream IDs.

2.	The NRM server determines if it has the requested data stored internally or whether it needs to interact with the NEF to fetch the data using the AnalyticsExposure described in clause 5.2.6.16 of 3GPP TS 23.502 [11] and in particular the UE Communication Analytics described in clause 6.7.3 of 3GPP TS 23.288 [34] and DN Performance Analytics described in clause 6.14 of 3GPP TS 23.288 [34]. The NRM server collects and processes the collected data to match the measurement data requirement provided in the VAL server request.

3.	The NRM server responds with a unicast QoS monitoring data response with the requested data or with a failure indication, and optionally the cause of the failure, in case the requested data is not available.

##### 14.3.3.4.5	Unicast QoS monitoring subscription update procedure

###### 14.3.3.4.5.1	General

This subclause describes the high-level procedure to update unicast QoS monitoring subscription. This procedure updates the parameters to be monitored of an already existing unicast QoS monitoring subscription.

###### 14.3.3.4.5.2	Procedure

Figure 14.3.3.4.5.2-1 illustrates the high level procedure for unicast QoS monitoring subscription update.

Pre-conditions:

-	The VAL UE has an established connection in the 5GS

-	Unicast QoS monitoring subscription has successfully been invoked established for the VAL UE

Figure 14.3.3.4.5.2-1: Unicast QoS monitoring subscription update

1.	The VAL server sends a unicast QoS monitoring subscription update request to the NRM server. It shall refer to an existing subscription, identified by the list of VAL UEs, VAL UE group or VAL stream IDs included into the message.

2.	The NRM server interacts with the NEF to update the relevant QoS monitoring subscription parameters and reporting frequency as specified in step 2 of clause 14.3.3.4.1.2 of this document.

3.	The NRM server responds with a unicast QoS monitoring subscription update response indicating the subscription status.

### 14.3.4	Multicast resource management for EPS

#### 14.3.4.1	General

The VAL server utilizes the NRM server for multicast resource management.

To activate the multicast bearers in the EPS, the NRM server shall use the Activate MBMS Bearer procedure specified in 3GPP TS 23.468 [16] with the NRM server performing the GCS AS function.

To deactivate the multicast bearers in the EPS, the NRM server shall use the Deactivate MBMS Bearer procedure specified in 3GPP TS 23.468 [16] with the NRM server performing the GCS AS function.

To modify multicast bearers in the EPS, the NRM server shall use the Modify MBMS Bearer procedure specified in 3GPP TS 23.468 [16] with the NRM server performing the GCS AS function.

Editor's note:	To support other modes of MBMS is FFS.

#### 14.3.4.2	Use of pre-established MBMS bearers

##### 14.3.4.2.1	General

In this scenario, upon triggered by VAL server, the NRM server pre-establishes MBMS bearer(s) in certain pre-configured areas before the initiation of the VAL service group communication session. When a user originates a request for a VAL service group communication session for one of these areas, the pre-established MBMS bearer(s) is used for the DL VAL service communication.

The following steps need to be performed prior to the start of the VAL service group communication session over pre-established MBMS bearer:

-	Pre-establish MBMS bearer(s)

-	Announce the pre-established MBMS bearer to the NRM clients

When these preparation steps have been done the VAL service group communication session using MBMS bearer can start.

The vertical application level communications are sent on the MBMS bearer. Optionally a separate MBMS bearer could be used for the application level control messages, due to different bearer characteristic requirements.

##### 14.3.4.2.2	Procedure

The procedure figure 14.3.4.2.2-1 shows only one of the receiving VAL clients using an MBMS bearer. There might also be VAL clients in the same VAL service group communication session that receive the communication on unicast bearers.

Figure 14.3.4.2.2-1: Use of pre-established MBMS bearers

1.	The VAL server sends a MBMS bearers request to the NRM server including service description(s) for which the MBMS bearers are requested.

2a.	The NRM server determines to activate MBMS bearer. The activation of the MBMS bearer in EPS is done on the MB2-C reference point and according to 3GPP TS 23.468 [16]. This bearer will be used for the VAL service communication. If local MBMS is requested in step 1, the NRM server uses the local MBMS information provided by VAL server in step 1 or the local MBMS information configured locally in the NRM server to activate the MBMS bearers. The NRM server performs local MBMS procedures in line with the procedure of L.MBMS based MBMS data delivery defined in 3GPP TS 23.285 [31].

2b.	Optionally, the NRM server may also activate an MBMS bearer dedicated for application level control signalling. The activation of the MBMS bearer is done on MB2-C reference point and according to 3GPP TS 23.468 [16]. If local MBMS is requested in step 1, the NRM server uses the local MBMS information provided by VAL server in step 1 or the local MBMS information configured locally in the NRM server to activate the MBMS bearers. The NRM server performs local MBMS procedures in line with the procedure of L.MBMS based MBMS data delivery defined in 3GPP TS 23.285 [31].

NOTE 1:	The procedure to determine the activation of MBMS bearers is implementation specific.

3a.	The NRM server passes the MBMS bearer info for the service description associated with the pre-established MBMS bearer to the NRM client. The NRM client obtains the TMGI, identifying the MBMS bearer, from the service description.

3b.	The NRM server may pass the MBMS bearer info for the service description associated with the application control MBMS bearer to the NRM client. The NRM client obtains the TMGI, identifying the MBMS bearer, from the service description.

NOTE 2:	Step 3a and step 3b can be done in one MBMS bearer announcement message.

4.	The NRM client stores the information associated with the TMGI(s). The NRM service client uses the TMGI and other MBMS bearer related information to activate the monitoring of the MBMS bearer by the VAL UE. The NRM client shares the MBMS bearer related information with the VAL client.

5.	The NRM client that enters or is in the service area of at least one announced TMGI indicates to the NRM server that the VAL UE is able to receive VAL service communication over MBMS, whereby the NRM server may decide to use the MBMS bearer instead of unicast bearer for VAL service communication sessions based on available information at the NRM server including the MBMS listening status report as described in clause 14.3.4.5.

NOTE 3:	Step 5 is optional for the VAL UE on subsequent MBMS bearer announcements.

6.	The NRM server provides a MBMS bearers response to the VAL server.

7.	A VAL service group communication session is established.

8.	As the VAL server transmits the VAL service communication over the MBMS bearer, the VAL service communication packets are detected and delivered to the VAL client.

#### 14.3.4.3	Use of dynamic MBMS bearer establishment

##### 14.3.4.3.1	General

In this scenario, the VAL server uses a unicast bearer for communication with the UE on the DL at the start of the group communication session. When the VAL server triggers to use an MBMS bearer in EPS for the DL VAL service communication, the NRM server decides to establish an MBMS bearer in EPS using the procedures defined in 3GPP TS 23.468 [16]. The NRM server provides MBMS service description information associated with MBMS bearer(s), obtained from the BM-SC, to the UE. The UE starts using the MBMS bearer(s) to receive DL VAL service and stops using the unicast bearer for the DL VAL service communication.

##### 14.3.4.3.2	Procedure

Figure 14.3.4.3.2-1 illustrates the use of dynamic MBMS bearer establishment.

Figure 14.3.4.3.2-1: Use of dynamic MBMS bearer establishment

1.	A VAL service group communication session is established.

2.	The downlink data is sent by unicast delivery.

3.	The VAL server sends MBMS bearers request to the NRM server.

4.	The NRM server establishes the MBMS bearer(s) for the VAL service group communication session according to the procedures defined in 3GPP TS 23.468 [16]. Service description associated with the MBMS bearer(s) is returned from the BM-SC. If local MBMS is requested in step 3, the NRM server uses the local MBMS information provided by VAL server in step 3 or the local MBMS information configured locally in the NRM server to activate the MBMS bearers. The NRM server performs local MBMS procedures in line with the procedure of L.MBMS based MBMS data delivery defined in 3GPP TS 23.285 [31].

5.	The NRM server provides service description information associated with the MBMS bearer to the UE. The VAL UE obtains the TMGI from the announcement message. This message may be sent on an application level control signalling bearer.

6.	The VAL UE starts monitoring data over MBMS associated with the TMGI, while in the service area associated with the TMGI.

7.	The VAL UE detects that it is able to receive data over MBMS associated with the TMGI.

8.	The NRM client notifies the NRM server the MBMS listening status associated to the monitored TMGI, (e.g. that it is successfully receiving the TMGI). The NRM client may also notify the MBMS reception quality level of the TMGI. The NRM server may decide to use the MBMS bearer instead of unicast bearer for VAL service communication sessions based on available information at the NRM server including the MBMS listening status report as described in clause 14.3.4.5.

9.	The NRM server provides an MBMS bearer response to the VAL server with the dynamic MBMS bearer(s) information. The VAL server stops sending VAL service communication data over unicast way to the VAL client.

NOTE:	The MBMS reception quality level may be used by the NRM server to make an efficient decision to switch again to a unicast transmission or to take measures to prepare such a switch (e.g. when the quality level indicates that the reception quality of the MBMS bearer is decreasing or reaching an insufficient quality level for the reception of VAL services).

10.	A VAL service group communication session via dynamic MBMS bearer(s) is established.

11.	The VAL server sends the downlink VAL service communication for the VAL service group communication session over the MBMS.

#### 14.3.4.4	MBMS bearer announcement over MBMS bearer

##### 14.3.4.4.1	General

The MBMS announcement may be done on either a unicast bearer or a MBMS bearer. Using a unicast bearer for MBMS bearer announcement provides an interactive way of doing announcement. The NRM server will send the MBMS bearer announcement message to the NRM client regardless if there is an MBMS bearer active or the VAL client can receive the data on the MBMS bearer with sufficient quality. The benefit of the existing procedure is that it gives a secure way to inform the NRM client about the MBMS bearer and how to retrieve the data on the MBMS bearer.

When there is more than one MBMS bearer active in the same service area for VAL service, there are not the same reasons to use unicast bearer for additional MBMS bearer announcement. Instead a MBMS bearer for application level control signalling can be used to announce additional MBMS bearers.

The MBMS bearer announcement messages are sent on an MBMS bearer used for application control messages. This bearer will have a different QoS setting compared to an MBMS bearer used for VAL service communication, since application signalling messages are more sensitive to packet loss.

##### 14.3.4.4.2	Procedure

Figure 14.3.4.4.2-1 illustrates a procedure that enables the NRM server to announce a new MBMS bearer.

Pre-conditions:

1.	An MBMS bearer used for VAL service application control messages must have been pre-established and announced to the NRM client.

2.	Additional MBMS bearer information may have already been announced to the NRM client.

Figure 14.3.4.4.2-1: MBMS bearer announcement over an MBMS bearer used for application control messages

1.	The NRM client monitors an MBMS bearer that is used for VAL service application signalling messages, such as bearer announcement messages.

2.	The NRM server activates a new MBMS bearer.

3.	The NRM server announces the MBMS bearer to the NRM client. The bearer may have just been activated or may have already been running for some time. The step may be repeated as needed.

4.	The NRM server sends a MBMS bearer announcement on the MBMS bearer used for VAL application control messages. The MBMS bearer announcement contains the identity of the MBMS bearer (i.e. the TMGI) and may optionally include additional information about the newly announced bearer. Required and optional MBMS bearer announcement details may have already been provided. In this case the MBMS bearer identity could be used as a key for such MBMS bearer details.

5.	The NRM client start to monitor the newly announced MBMS bearer.

6.	If requested by the NRM server, the NRM client sends an acknowledgement of the MBMS bearer to the NRM server.

7.	The NRM server de-announce the MBMS bearer.

8.	The NRM server sends a MBMS bearer de-announcement message that contains the identity of the MBMS bearer.

9.	The NRM client stops monitoring the de-announced MBMS bearer.

The same procedure can also be used to modify existing MBMS bearer announcement information. Example of such modification could be addition of UDP ports or modification of codec in the SDP.

#### 14.3.4.5	MBMS bearer quality detection

##### 14.3.4.5.1	General

The NRM client and NRM server use this procedure to report and take action on the MBMS bearer quality towards VAL service communications. A NRM client monitors an MBMS bearer to enable receiving VAL service communication. Based on the received quality (e.g. radio level quality, transport level quality), the NRM client needs to inform the NRM server that the VAL UE is able to receive the VAL service communication on the MBMS bearer with sufficient quality or not able to receive the VAL service communication on the MBMS bearer with sufficient quality. Furthermore, based on the received quality, the NRM client may notify the NRM server at which MBMS reception quality level it has received the VAL service communication on the MBMS bearer.

The issue can be more complex since the NRM client needs to estimate the quality of the bearer even in the scenario when there are no data currently transmitted on the MBMS bearer. The reason for this is that an NRM client that has entered an area with significantly degraded MBMS quality, might not even notice that a VAL service communication is ongoing, meanwhile the NRM server still assumes that the VAL UE can receive the VAL service communication being broadcasted.

To estimate the MBMS bearer quality, for example as an equivalent BLER (Block Error Rate), when no data is sent is implementation specific. This estimation can be dependent on for example the modulation and coding scheme (MCS) and measurements from the reference signals from the eNB(s). Other metrics (e.g. RTP packet loss) may be used to estimate the MBMS bearer quality.

Based on the MBMS bearer quality reported from the NRM clients, the NRM server may decide to use the MBMS bearer for a group communication if a certain number of NRM clients located in the MBMS service area are able to receive the VAL service communication. And if a NRM client is not able to receive the VAL service communication on the MBMS bearer, the NRM server may decide to switch the user plane deliver mode for the NRM client from MBMS bearer to unicast bearer.

##### 14.3.4.5.2	Procedure

The NRM client shall indicate the ability of the NRM client to receive the MBMS bearer.

Pre-conditions:

1.	There is an MBMS bearer activated and the MBMS bearer information is announced to the NRM client

2.	The NRM client is located in the MBMS broadcasting area

3.	The VAL UE monitors SIB-13 (or SIB-20) and (SC-)MCCH to receive the modulation and coding scheme

4.	The VAL UE monitors the cell specific reference signal and when MBSFN transmission is used, the MBSFN specific reference signals

Figure 14.3.4.5.2-1: MBMS bearer quality detection

1.	The NRM client determines that the MBMS bearer quality shall be reported to the NRM server. The NRM client may determine the MBMS bearer quality by using the BLER of the received data. When no data is received, the quality estimation can consider the reference signals and the modulation and coding scheme (MCS). The UE may also use predictive methods to estimate the expected MBMS bearer quality (e.g. speed and direction) to proactively inform the NRM server of an expected loss of the MBMS bearer quality. The NRM client may also map the determined MBMS bearer quality to a MBMS reception quality level. The MBMS reception quality level indicates at which specific MBMS bearer quality level the VAL service communication has been received.

Editor's note:	The set of MBMS reception quality levels and the mapping of the determined MBMS bearer quality to those levels are FFS.

NOTE 1:	When MBSFN transmission is used, the MBSFN reference signal needs to be used and when SC-PTM is used the cell specific reference signal needs to be used. With the measured reference signal, the reference signal received quality (RSRQ) can be calculated.

2.	If the MBMS bearer quality reaches a certain threshold, the NRM client sends an MBMS listening status report. The threshold is used to define the MBMS listening status, which indicates if the MBMS bearer quality has been acceptable or not to receive a specific VAL service communication. If the MBMS bearer quality is mapped to a different MBMS reception quality level, the NRM client may send an MBMS listening status report including the MBMS reception quality level. Based on the MBMS listening status, if MBMS reception quality level is received, then the NRM server may efficiently decide to switch to another bearer (e.g., MBMS bearer or unicast bearer) or to take measures to prepare such a switch and further notify the VAL server.

NOTE 2:	Prior sending the MBMS listening status report, it could be beneficial to also include information for different alternatives e.g. another MBMS bearer might have better quality and could be a better option than a transfer of the communication to unicast.

NOTE 3:	The threshold used to indicate MBMS bearer quality depends on VAL service type (i.e. V2X, MCPTT, MCVideo or MCData) and the metrics used. The metrics used and the associated thresholds are out of scope of this specification.

3.	The NRM server may send additional proposal for measurements e.g. information about neighbouring MBMS bearers. This message may be an MBMS bearer announcement message.

4.	The NRM server may send user plane delivery mode to VAL server based on the MBMS listening status to preserve the service continuity as described in clause 14.3.4.6 and clause 14.3.4.9.

#### 14.3.4.6	Service continuity in MBMS scenarios

##### 14.3.4.6.1	General

This subclause specifies service continuity scenarios when MBMS bearers are used. There are different solutions for different scenarios.

##### 14.3.4.6.2	Service continuity when moving from one MBSFN to another

The service continuity solution described in this subclause is suitable in the scenario when multiple MBMS bearers are used with the purpose to cover a larger area. In VAL communications several VAL service communication streams may be multiplexed in one MBMS bearer. Furthermore, one VAL service communication stream may be sent on more than one MBMS bearer if the receiving users are distributed over more than one MBMS service area. A VAL UE that is interested in receiving a VAL service communication stream that is broadcasted in both MBMS bearers is a candidate for this service continuity procedure.

Figure 14.3.4.6.2-1 illustrates a deployment scenario that provides service continuity between two MBSFN areas. Two different MBMS bearers are activated (TMGI 1 and TMGI 2), the activation of the bearers is done in the two MBSFN areas (MBSFN 1 and MBSFN 2). The MBSFN areas 1 and 2 are partially overlapping, meaning that some transmitting cells belong to both MBSFN area 1 and MBSFN area 2.

Figure 14.3.4.6.2-1: Two MBMS bearer using overlapping MBSFN areas

Figure 14.3.4.6.2-2 illustrates the procedure:

Figure 14.3.4.6.2-2: Service continuity when moving from one MBSFN to another

1.	The VAL UE is located in MBSFN 1 and can listen to TMGI 1. No additional MBMS bearers that the NRM client is interested in are active in the current cell.

2a.	The NRM client notifies the NRM server that the VAL UE is successfully receiving the VAL service communication over TMGI 1. The NRM client may also notify the MBMS reception quality level of TMGI 1.

2b.	The NRM server notifies a user plane delivery mode to the VAL server.

NOTE:	The MBMS reception quality level may be used by the NRM server to make an efficient decision to switch to another MBMS bearer or to a unicast bearer, or to take measures to prepare such a switch (e.g. when the quality level indicates that the reception quality of the MBMS bearer is decreasing or reaching an insufficient quality level for the reception of VAL services).

3.	The VAL UE moves into a new cell in which both TMGI 1 and TMGI 2 are active. This cell is part of both MBSFN area 1 and MBSFN area 2, and broadcast the same service on both TMGIs.

4.	The NRM client sends a location information report to the NRM server. For that, the UE uses the SAI information found in the system information block (SIB) transmitted by the radio cells.

5.	The NRM server sends to the NRM client a MBMS bearer announcement with information related to TMGI 2 (if the NRM server had not done it before). Hence, the NRM client knows that TMGI 2 transmits the same VAL service communication.

6a.	The NRM client notifies the NRM server that it is successfully receiving TMGI 1 and TMGI 2. The NRM client may also notify the MBMS reception quality level per TMGI.

6b.	The NRM server notifies a user plane delivery mode to the VAL server.

7.	The VAL UE may receive the VAL service communication over both MBMS bearers, i.e. TMGI 1 and TMGI 2. The VAL UE may also verify that it is the same content sent on both bearers. The duplicated packets may also be used to perform error corrections.

8.	The VAL UE moves into a new cell in MBSFN area 2, where only TMGI 2 is active.

9a.	The NRM client notifies the NRM server that the VAL UE is successfully receiving the VAL service communication over TMGI 2. The NRM client may also notify the MBMS reception quality level of TMGI 2.

9b.	The NRM server notifies a user plane delivery mode to the VAL server.

10.	The VAL UE receives the VAL service communication only over TMGI 2.

This service continuity procedure mitigates the risk of packet loss that may occur if the VAL UE would request to transfer the VAL service communication stream to a unicast bearer when moving into the new area and then back to a multicast bearer when the UE can listen to TMGI 2. However, it is still required that the NRM client sends a location report (and MBMS listening report), as described in steps 4-6 above. To send the location report and the MBMS listening report by the NRM client to the NRM server a unicast bearer is needed. The location report from the NRM client is required, since the NRM server must know that the VAL UE has entered a new area and can only listen to MBMS bearer active in that area. If this is not done the VAL server might send a VAL service communication stream that the VAL UE is required to listen to on the MBMS bearer 1, since the NRM server still assumes that the VAL:UE is located in the MBSFN area 1.

The solution can be improved as illustrated in figure 14.3.4.6.2-3. In this case two different MBMS bearers are activated (TMGI 1 and TMGI 2), these MBMS bearers are used only for VAL service communication. An application level signalling bearer is activated (TMGI 9), in both MBSFN areas. This bearer is used for application level signalling messages that are sent on the MBMS bearer TMGI 9. By using an application level signalling bearer (e.g. TMGI 9) the VAL UEs can receive application control messages for all VAL service communication going on in the areas of both TMGI 1 and TMGI 2. A VAL UE that is located in the area of TMGI 2 and is interested in a VAL service group transmission (e.g. V2X) only going on in TMGI 1, can with the information received in TMGI 9 initiate a unicast bearer and request to receive that specific VAL service communication over a unicast instead. Without the information received over TMGI 9 the NRM client must immediately report that the VAL UE has left the broadcast area that the NRM server assumes that the VAL UE is located in. With the use of TMGI 9 there is no immediate need for the NRM client to inform the NRM server of a location change.

Figure 14.3.4.6.2-3: Two MBMS bearer using overlapping MBSFN areas with a separate application signalling bearer

The procedural steps in this scenario will be the same as described above in this subclause. However, in this scenario the NRM client is not required to initiate a unicast bearer to send location report (or MBMS listening report). The VAL UE may move between the two MBMS bearers (TMGI 1 and TMGI 2) without the need to report an area change. A condition for this to work is that there is an application level signalling bearer (TMGI 9) activated in the full area (i.e. the area of both TMGI 1 and TMGI 2). The TMGI 9 will broadcast all application control messages for all VAL service communications ongoing in both areas. If the VAL UE is in coverage of one of the two MBMS bearers that does not transmit the VAL service communication of interest the VAL UE can report to the NRM server that it is not able to listen to the VAL service communication over the MBMS bearer, which triggers the NRM server to switch to a unicast bearer instead.

#### 14.3.4.7	MBMS suspension notification

##### 14.3.4.7.1	General

In this procedure the NRM client is requested by the NRM server to send a MBMS suspension report. This request for MBMS suspension report can be included in the MBMS bearer announcement and the NRM server may choose to only send this request for MBMS suspension report to a subset of all NRM clients.

##### 14.3.4.7.2	Procedure

Figure 14.3.4.7.2.-1 illustrates a procedure in which the NRM client notifies the NRM server about an MBMS suspension decision in RAN.

The NRM server can decide on a subset of all VAL UEs in the MBMS broadcast area that shall report on MBMS bearer suspension. When the NRM server makes the decision of the VAL UE subset, consideration shall be taken to the location of the VAL UEs, since VAL UEs' location is dynamically changed. This means that the MBMS suspension reporting instruction may need to be updated regularly based on the VAL UEs mobility.

Pre-condition:

-	It is assumed that there is at least one active MBMS bearer

Figure 14.3.4.7.2-1:	MBMS suspension notification

1.	The NRM server sends an MBMS suspension reporting instruction to the NRM client.

NOTE:	This message may be included in the MBMS bearer announcement message and may be sent both on a unicast bearer and a multicast bearer.

2.	RAN decides to suspend the MBMS bearer, according to existing procedures in 3GPP TS 36.300 [23].

3.	An MBMS suspension indication is sent in the MSI (MCH Scheduling Information), according to existing procedures in 3GPP TS 36.300 [23].

4.	The NRM client detect the MBMS suspension and sends an MBMS suspension report.

5.	Based on the MBMS suspsension report received, the NRM server determines whether to switch to a new bearer (unicast or MBMS). If NRM server determines to switch to unicast bearer, then the NRM server sends the user plane delivery mode message to VAL server , and the VAL server sends the downlink data over the new bearer.

The NRM client that is not instructed to send an MBMS suspension report shall still detect the MBMS suspension indication from RAN (step 3). A NRM client shall in this case not send other types of report (e.g. MBMS listening reports).

The same procedure can be applied at MBMS resumption or other MBMS events that may be detected by the NRM client.

#### 14.3.4.8	MBMS bearer event notification

##### 14.3.4.8.1	General

The NRM server is an instantiation of a GCS AS. For the NRM server to know the status of the MBMS bearer, and thus know the network's ability to deliver the VAL service, it is required that the network provides MBMS bearer event notifications to the NRM server. The different events notified to the NRM server include the MBMS bearer start result (e.g. when the first cell successfully allocated MBMS resources), including information if any cells fail to allocate MBMS resources to a specific MBMS bearer, the current status of the MBMS bearer, MBMS bearer suspension/resume or overload scenarios.

Editor's note:	The procedure defined in this sub clause requires an enhancement to GCSE and RAN and is therefore subject to implementation in EPC and RAN.

##### 14.3.4.8.2	Procedure

The procedure in figure 14.3.4.8.2-1 shows notification information flows from NRM server to BM-SC.

Figure 14.3.4.8.2-1: MBMS bearer event notification

1.	The NRM server activates an MBMS bearer. The activation of the MBMS bearer is done on the MB2-C reference point and according to 3GPP TS 23.468 [16].

2.	The BMSC will respond to the activation with an Activate MBMS bearer response message, according to 3GPP TS 23.468 [16].

3.	The EPC and RAN will initiate the MBMS session start procedure according to 3GPP TS 23.246 [17]. This procedure is outside the scope of this specification.

4a.	At the first indication of a successful MBMS session start procedure, the BM-SC sends a MBMS bearer event notification, indicating that the MBMS bearer is ready to use.

4b.	The NRM server notifies user plane delivery mode to the VAL server.

5.	The VAL server starts to use the MBMS bearer according to the MBMS procedures in this specification.

6.	An event from RAN related to the MBMS session is received by the BM-SC.

7a.	The BM-SC notifies the NRM server of certain MBMS related events including references to affected MBMS services areas or list of cells. Example of such events may be radio resources not available, overload, MBMS suspension.

7b.	The NRM server notifies user plane delivery mode to the VAL server.

8.	The NRM server may decide, based on the received events, to switch to unicast transmission for relevant VAL UEs.

NOTE:	Steps 6-8 should be seen as example events from the network that may occur and possible actions taken by the NRM server. These steps may be done at any time and repeatedly during the life time of an MBMS bearer.

#### 14.3.4.9	Switching between MBMS bearer and unicast bearer

##### 14.3.4.9.1	General

The NRM server monitors the bearers used for VAL service communications and decides to switch between MBMS and unicast bearers.

##### 14.3.4.9.2	Procedure

Figure 14.3.4.9.2-1 shows the procedure for service continuity when a UE is about to move out of MBMS coverage or getting into good MBMS coverage by switching between MBMS bearer and unicast bearer.

Pre-condition:

-	It is assumed that a bearer (unicast or MBMS) has been activated by the VAL server for downlink delivery.

Figure 14.3.4.9.2-1: Switching between MBMS delivery and unicast delivery

1.	The VAL UE detects changing MBMS bearer condition (good or bad MBMS coverage) for the corresponding MBMS service. The method to detect is implementation specific.

2.	The NRM client notifies the NRM server about the MBMS bearer condition for the corresponding MBMS service by sending the MBMS listening status report.

NOTE 1:	To efficiently notify the NRM server, e.g., when the NRM client detects that the reception quality of the MBMS bearer is decreasing or reaching an insufficient quality level for the reception of VAL services, the NRM client proactively may send to the NRM server a MBMS listening status report including the MBMS reception quality level.

3.	The NRM server makes the decision to switch between MBMS delivery and unicast delivery based on available information at the NRM server including the MBMS listening status report as described in clause 14.3.4.5. The NRM server notifies a user plane delivery mode to the VAL server.

4.	The VAL server sends the downlink data over the new bearer (unicast or MBMS) to the VAL client as per step 3.

NOTE 2:	The new bearer (unicast or MBMS) may be set up on demand after step 3 or before.

5.	During the switching, the VAL client simultaneously receives downlink data through both bearers (unicast bearer and MBMS bearer). If there is no downlink data to the VAL client, this step can be skipped.

6.	The VAL client ceases to receive the downlink data through previous bearer but continues receiving data through new bearer.

### 14.3.4A	Multicast resource management for EPS

#### 14.3.4A.1	General

This subclause defines information flows and procedures for 5G MBS usage that applies to VAL services. 5G MBS session can be used by any VAL service for any VAL service group.

The main purpose of using 5G Multicast-Broadcast Service (MBS) by verticals is to provide efficient downlink delivery of user traffic in VAL service group communications or in a certain area.

Multicast and broadcast services in 5G for vertical applicationsrely on the creation and establishment of MBS sessions to deliver user data in downlink. Shared and individual delivery from the VAL server to multiple VAL users is supported either as point-to-point or point-to-multipoint over the radio. The MBS session which is consist of one or multiple QoS flows for different service requirements can either be broadcast or multicast type.

NOTE 1:	Support of MBS and specific session types is an implementation choice.

Within this arrangement, the VAL server decides whether to create broadcast or multicast MBS sessions to be associated with certain VAL service groups or area. The 5GC adaptively decides whether to deliver the MBS traffic from the MB-UPF in the form of shared delivery or individual delivery, where the latter is applicable to multicast MBS sessions. The NG-RAN decides to utilize point-to-point or point-to-multipoint delivery methods applicable for shared delivery only. MBS provides reliability enhancements and minimizes loss of information, e.g., due to mobility and handover.

MBS group scheduling mechanism enables simultaneous reception of MBS and unicast user traffic by the VAL UEs. The UEs can receive broadcast MBS sessions irrespective of their RRC state (i.e., connected, inactive or idle) and multicast sessions only in RRC-CONNECTED state.

The following capabilities (non-exhaustive list) provided by MBS could be used by NRM server:

-	MBS session creation;

-	MBS session update;

-	MBS session release;

-	MBS session ID allocation;

-	Dynamic PCC control for MBS session.

The first phase to utilize MBS sessions for VAL media transmission is to reserve the network resource by creating a MBS sessions. The MBS session creation is initiated by the VAL server towards the NRM server, and the NRM server further interacts with the 5GS to complete the whole process. The UE’s capabilities and service related information e.g., UE’s MBS capabilities, location, MBS listening status report sent by group members are considered when deciding to create or use MBS sessions. During the interaction with NRM server, the necessary information related to the requested session is determined, e.g., MBS session mode (either a broadcast or a multicast session) and the required service profile. This interaction between the NRM server and the 5GS depends on the configuration option under consideration, i.e., whether the NRM server is in trusted domain (limited operations), and whether the session creation is done with or without a dynamic PCC rule.

NOTE 1:	It is implementation specific whether the VAL server decides to use multicast or broadcast MBS sessions.

NOTE 2:	It is implementation specific whether the VAL decides to create (one or multiple) MBS sessions for VAL media for VAL group communications associated to a certain VAL group or create (one or multiple) dynamic MBS sessions once the need has emerged, e.g., dynamic MBS sessions to be associated for an ad hoc group.

NOTE 3:	It is implementation specific whether an MBS session is associated to one or multiple VAL service groups, and whether it is re-assigned to other VAL service groups.

NOTE 4:	How the NRM server uses the UE’s capabilities and service related information in order to create and use the MBS session is implementation specific.

The information elements describing the MBS session under consideration is then sent to the NRM clients via MBS session announcement, where the latter need to react according to the announced session mode.

If eMBMS and 5G MBS co-exist for VAL services, the NRM service server may decide to trigger the establishment of an eMBMS bearer to deliver the VAL media associated to the VAL service group communications, if the target VAL service group(s) consists of members with MBMS capable RAT. As a result, the NRM server subsequently needs to send an eMBMS bearer announcement towards the clients camping on LTE.

NOTE 5:	It is implementation specific whether the NRM server triggers an eMBMS bearer or a unicast bearer to serve VAL clients camping on LTE.

Editor’s note:	The information flows for the subclauses are FFS.

#### 14.3.4A.2	MBS session creation and MBS session announcement

##### 14.3.4A.2.1	General

The procedures in this clause describe how MBS session creation and MBS session announcement can be used for the transmission of VAL service group communication data over either broadcast or multicast MBS sessions. The MBS session can either be created with or without dynamic PCC rule, where the latter requires less interaction done by the NRM server towards the 5GC (either directly or via NEF).

##### 14.3.4A.2.2	Procedure for pre-created MBS session and MBS session announcement

Pre-conditions:

-	The NRM server has decided to use an MBS session for VAL service group communications associated to a certain VAL servive group based on transport only mode.

-	The NRM server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	NRM clients 1 to n are attached to the 5GS, registered and belong to the same VAL service group X.

-	The NRM server is aware whether to request the creation of the MBS service server with or without dynamic PCC rule.

Figure 14.3.4A.1.2-1: Use of pre-created MBS session.

1.	The VAL server sends a multicast/broadcast resource request towards the NRM server including the VAL server identity, service description(s), multicast resource type (e.g,. multicast type or broadcast type), service announcement mode (i.e., service announcement is sent by NRM or the VAL itself), Endpoint of the VAL server.

2.	The NRM server initiates an MBS session creation procedure towards the 5GC as described in 3GPP TS 23.247 [39]. The procedure starts once the NRM server initiates a TMGI allocation request (either directly to MB-SMF or indirectly via NEF). Upon the reception of the TMGI allocation response, the NRM server sends an MBS session creation request, including further information related to the MBS session, e.g., MBS session ID, MBS session mode and the QoS requirements if dynamic PCC rule is not considered. However, if dynamic PCC rule is considered, the NRM server defines these requirements at a later step, namely it sends an MBS authorization/policy create request towards PCF (either directly or to the NEF) indicating the QoS requirements.

NOTE 1:	In case of LTE eMBMS and 5G MBS co-existence, the NRM server may trigger the establishment of eMBMS bearers as described in clause 14.3.4 (or it may establish a unicast bearer) based on the RAT capabilities supported by the VAL service group members in the VAL service group X. If MBSF and BM-SC are co-located, TMGI used by 4G eMBMS can be the same as the MBS session ID.

NOTE 2:	For the case of multi carrier support for broadcast MBS sessions, the NRM server may indicate the frequencies within a broadcast MBS service area by providing the MBS frequency selection area ID(s) (MBS FSA ID(s)) to the MB-SMF or indirectly via NEF.

3.	The NRM server provides the NRM clients of the VAL service group X with the information related to the created MBS session via the MBS session announcement. The MBS session announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type) and SDP information related to the MBS session under consideration.

NOTE 3:	The NRM server may send an MBS session announcement at an earlier step during the MBS session creation procedure towards the NRM clients once the VAL service group associated to the MBS session is known.

Optionally, the NRM server includes the information elements related to the established eMBMS bearer, as indicated in table 7.3.2.2-1. The NRM clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

4.	NRM clients store and process the received MBS session information.

5.	NRM clients may provide an MBS session announcement acknowledgment to the NRM server to indicate the reception of the corresponding MBS session announcement.

6.	Based on the MBS session mode (either multicast or broadcast), the following actions take place;

6a.	For multicast MBS sessions, NRM clients initiate a UE session join request towards the 5GC using the information provided via the MBS session announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved, if the session is in an active state. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [39]. The NRM clients sends a UE session join notification towards the server. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server; or

6b.	For broadcast MBS sessions, if the NRM client is accessing over 5G, the session is established as part of the session creation procedures as described in 3GPP TS 23.247 [39], and the network resources are reserved both in 5GC and NG-RAN. The NRM clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server.

NOTE 4:		It is implementation specific whether the MBS session reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signals, such as the reference signals from the NG-RAN node(s), or packet loss.

7.	The NRM clients provide a listening status notification related to the announced session (multicast or broadcast session) in the form of an MBS listening status report.

8.	The NRM server provides a multicast/broadcast resource response to the VAL server.

9.	An VAL service group communication setup takes place and uses the pre-created MBS session for this group communication packet DL delivery.

##### 14.3.4A.2.3	Procedure for dynamic MBS sessions

In this scenario, the VAL service group communication is already taken place and a unicast PDU session is utilized for  DL transmission. When the NRM server decides to use an MBS session for the transmission under consideration, the NRM server interacts with 5GC to reserve the necessary network resources.

NOTE 1:	The NRM server logic for determining when to create a dynamic MBS session is implementation specific.

The procedure in figure 14.3.4A.2.3-1 shows one NRM client receiving the DL media. There might also be NRM clients in the same VAL service group communication session that receive the communication on a PDU session.

Pre-conditions:

-	NRM client is attached to the 5GS, registered and affiliated to a certain VAL service group X.

-	The NRM server is aware whether to request the creation of the MBS session with or without dynamic PCC rule.

-	The NRM server has performed MB-SMF discovery and selection either directly or indirectly via NEF/MBSF, unless the corresponding information is locally configured.

-	No MBS session exists, or the existing multicast MBS session fails to satisfy the QoS requirements.

Figure 14.3.4A.2.3-1: Use of dynamic MBS session.

1.	An VAL service group communication session is established and the DL media packet is delivered to the VAL client via unicast.

2.	The VAL server sends a multicast/broadcast resource request towards the NRM server including the VAL server identity, service description(s), multicast resource type (e.g,. multicast type or broadcast type), service announcement mode (i.e., service announcement is sent by NRM or the VAL itself), Endpoint of the VAL server.

3.	The NRM server decides to create an MBS session. The MBS session creation procedure takes place as described in clause 14.3.4A.1.

NOTE 2:	In case of LTE eMBMS and 5G MBS co-existence, the NRM server may trigger the establishment of eMBMS bearers as described in 3GPP TS 23.280 [3] (or it may establish a unicast bearer) based on the RAT capabilities supported by the affiliated members in the VAL service group X. If MBSF and BM-SC are co-located, TMGI used by 4G eMBMS can be the same as the MBS session ID.

NOTE 3:	For the case of multi carrier support for broadcast MBS sessions, the NRM server may indicate the frequencies within a broadcast MBS service area by providing the MBS frequency selection area ID(s) (MBS FSA ID(s)) to the MB-SMF or indirectly via NEF.

4.	The NRM server provides the NRM client with the information related to the created MBS session via an MBS session announcement. As described in table 7.3.2.2-1, the session announcement includes information such as the MBS session ID, MBS session mode (broadcast or multicast service type), and SDP information related to the MBS session.

Optionally, the NRM server includes the information elements related to the established eMBMS bearer once the NRM server has determined the need, as indicated in table 7.3.2.2-1. The NRM clients which camp on LTE will subsequently react to the information elements related to the eMBMS bearer as described in 3GPP TS 23.280 [3].

5.	The NRM client stores the MBS session ID and other associated information.

6.	The NRM client may send an MBS session announcement ack back to the NRM server.

7.	Based on the MBS session mode (either multicast or broadcast), the following actions take place:

7a.	For multicast MBS sessions, NRM client initiates a UE session join request towards the 5GC using the information provided via the MBS session announcement. Hence, upon the first successful UE session join request, the multicast is then established, and the radio resources are reserved, if the session is in active state. The established session can either be in active or inactive state as indicated in 3GPP TS 23.247 [39]. The NRM client sends a UE session join notification towards the server. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server; or

7b.	For broadcast MBS sessions, if the NRM client is accessing over 5G, the session is established as part of the session creation procedures as described in 3GPP TS 23.247 [39], and the network resources are reserved both in 5GC and NG-RAN. The NRM clients start monitoring the reception quality of the broadcast MBS session. If indicated in the MBS session announcement information, NRM clients report the monitoring state (i.e. the reception quality of the MBS session) back to the NRM server.

NOTE 4:		It is implementation specific whether the MBS session reception quality level is determined per MBS session, per media stream or per MBS QoS flow level via e.g., measurements of radio level signaling such as the reference signals from the NG-RAN node(s), packet loss.

8.	The NRM clients provide a listening status notification related to the announced session (multicast or broadcast session) in the form of an MBS listening status report.

9.	The NRM server provides a multicast/broadcast resource response to the VAL server.

10.	An VAL service group communication via dynamic MBS session is established. The VAL server sends the downlink packet for the VAL service group communication session over the MBS session.

#### 14.3.4A.3	MBS resources update

##### 14.3.4A.3.1	General

The VAL server can create one or several MBS sessions via the NRM server, based on certain service requirements, a certain service area, or the activity status of multicast MBS sessions. However, during the life cycle of the MBS sessions, the VAL server may need to trigger the update of the sessions via the NRM server to meet emerging needs, including the service requirements, service area related parameters.

Editor's note: The MBS resource update procedure with dynamic PCC is FFS.

##### 14.3.4A.3.2	Procedure for updating MBS resources without dynamic PCC rule

The procedure shown in figure 14.3.4A.3.2-1 presents an MBS session update procedure triggered by the VAL server via the NRM server (either directly interacting with the MB-SMF, or indirectly with NEF/MBSF). Within the update request, either the service requirements, MBS service area, activity status of multicast MBS session, or all three are done, as indicated in 3GPP TS 23.247 [39].

Pre-conditions:

-	The NRM clients 1 to n are attached to the 5GS, registered and belong to the same active VAL service group.

-	The NRM server has obtained the required information related to the MB-SMF, either locally configured or during initial session configuration.

-	The MBS session is created with certain service requirements and optionally with a certain broadcast/multicast service area. The MBS session is announced to be associated with the VAL service group for group communication purposes.

Figure 14.3.4A.3.2-1: MBS session update without dynamic PCC

1.	An MBS session is established as described in in 3GPP TS 23.247 [39] (either a multicast or a broadcast session), and associated with a certain active VAL service group for group communication purposes. In the case of a multicast MBS session, the NRM clients have already joined the session.

2.	The VAL server invoke the multicast resource update request to the NRM server once the need has emerged to modify some aspects for the given MBS session under consideration. The updated parameters are included, e.g., service requirements, service area or both. In case of multicast MBS sessions, the VAL server may as well include the status (active or inactive) of the multicast MBS session to be set.

3.	The NRM server performs an MBS session update procedure with the 5GC as described in 3GPP 23.247 [39]. Based on the needed requirements, the corresponding MBS session is accordingly modified at the 5GS. The update may lead to QoS Flow(s) addition, modification, or removal. The NRM server receives an MBS session update response once the requested modifications are performed, and the indicated MBS session is updated accordingly.

4.	The NRM server may initiate a session announcement towards the NRM clients associated with the ongoing session in order to announce the updated information, if required, e.g., the updated service area or SDP information.

5.	The NRM server sends an MapGroupToSessionStream over the configured MBS session providing the required information to receive the media related to the established VAL service group communication.

6.	The NRM clients process the received information over the MapGroupToSessionStream in order to receive the associated VAL media over the specific MBS session stream.

7.	The NRM server returns the multicast resource update resonse to the VAL server.

8.	VAL client 1 sends media to the VAL server over unicast to be distributed for the established group communication.

9.	The VAL server distributes the VAL media to the VAL clients 2 to n over the indicated streams.

#### 14.3.4A.4	MBS resource deletion

##### 14.3.4A.4.1	General

The VAL server can decide to release a certain MBS session once it is no longer further utilized for the associated VAL service group communication, e.g., the VAL service group is no longer active, the VAL media transmission is over and no further VAL media to be delivered, group communication is terminated. The MBS session deletion procedure leads to releasing the network resources associated to that MBS session.

NOTE:	It is up to implementation of VALserver to decide whether to release the MBS session or re-use it for subsequent group operations.

To delete the MBS session, the VAL server invokes the multicast/broadcast resource release service of NRM server which further triggers the NRM server to send an MBS session deletion request to the 5GS providing the corresponding MBS session ID. The MBS session deletion request is sent to the MB-SMF (directly or via NEF/MBSF) when PCC is not used. However, if dynamic PCC rule is utilized, a policy authorization deletion request is initially sent to the PCF. Further details of the MBS session deletion are provided in 3GPP TS 23.247 [39].

NRM server further informs the NRM client with the MBS session de-announcement, so that the VAL UE stops monitoring the broadcast MBS session or leaves the multicast MBS session. This procedure is applied for both broadcast MBS session and multicast MBS session.

##### 14.3.4A.4.2	Procedure

The procedure in figure 14.3.4A.4.2-1 describes the MBS session deletion aspects for group communication.

Pre-conditions:

-	NRM clients 1 to n are attached to the 5GS, registered and affiliated to the same active VAL service group.

-	An MBS session is configured to address the corresponding VAL service group with certain service requirements and optionally with a certain broadcast/multicast service area. The session is announced and established for group communication purposes for the VAL service group.

Figure 14.3.4A.4.2-1: MBS session deletion procedure.

1.	The VAL server decides to delete the MBS session for the associated VAL group communication, either multicast or broadcast session.

2.	The VAL server invokes the multicast/broadcast resource release service of the NRM server by sending the multicast/broadcast resource release request.

3.	Upon receiving the multicast/broadcast resource release request, the NRM server sends an MBS session de-announcement message with the MBS session ID towards the NRM client(s). Upon receiving the MBS session de-announcement message, either 4a or 4b is performed.

4a.	If the MBS session identified by MBS session ID is a broadcast MBS session, the UE(s) stops monitoring the broadcast MBS session and removes the broadcast MBS session related information.

4b.	If the MBS session identified by MBS session ID is a multicast MBS session, the joined UE(s) initiate an MBS session leave procedure to leave the indicated MBS session in order to release the respective network resources, as defined in 3GPP TS 23.247 [39].

5. Subsequently, the NRM clients may send an MBS session de-announcement acknowledgement message to the NRM server indicating the status of MBS session.

6.	The NRM server initiates the MBS session deletion procedure with the 5GC (either directly or through NEF/MBSF) in order to stop using the configured MBS session and release the corresponding network resources. The NRM server indicates within the MBS session release request the corresponding MBS session ID. The MBS session deletion procedure can either be with or without a dynamic PCC rule, as indicated in 3GPP TS 23.247 [39].

7.	The NRM server returns the multicast/broadcast resource release response to the VAL server indicating the result.

#### 14.3.4A.5	Request to activate / de-activate multicast MBS sessions

##### 14.3.4A.5.1	General

In case of multicast MBS sessions, the members affiliated to a certain VAL service group need to initiate a UE session join request towards the 5GC in order to receive the VAL media sent via the associated MBS session. The UE session join request enables the reservation of NG-RAN resources for the members of the VAL group. However, it is not necessary that the VAL media is delivered over the whole time the multicast MBS session is associated to the group under consideration. Therefore, the VAL server is able to efficiently utilize and control the reservation of radio resources based on the availability of VAL data to be delivered via the activation and de-activation procedure with NRM server. This presents more flexibility and efficient use of resources different from LTE.

The most suitable scenario to activate/de-activate a certain multicast MBS session is based on whether there is a VAL group communication, taking place over that associated session to the VAL service group. In this manner, the VAL server can activate the associated multicast session once a VAL group communication takes place, then deactivate it once the VAL group communication is over. Whether the multicast session is activated (i.e., in an active state), or de-activated (in an inactive state), the VAL group is associated to the multicast session and its members are within a UE session join.

The activation or de-activation request is initiated by the VAL server towards the NRM server which further interacts either directly with the MB-SMF or indirectly with NEF/MBSF.

NOTE:	The activation of de-activation procedure may also be triggered by MB-SMF based on receiving notification from MB-UPF based on the availability of VAL data to be transmitted.

##### 14.3.4A.5.2	Multicast MBS session activation procedure

The procedure shown in figure 14.3.4A.5.2-1 presents the multicast MBS session activation procedure initiated by the VAL server.

Pre-conditions:

-	NRM clients are attached to the 5GS, registered and affiliated to the same VAL service group X.

-	The NRM server has directly performed (or via NEF/MBSF) an MB-SMF discovery and selection, unless the corresponding information is locally configured.

-	The multicast MBS session for NRM group communications associated to VAL service group X.

-	The MBS session is created and announced to address VAL group communication related to the associated VAL service group X with certain service requirements and optionally with a certain service area.

Figure 14.3.4A.5.2-1: Multicast MBS session activation procedure.

1.	The multicast MBS session is established as the first UE session join request, which is initiated by the first VAL UE towards 5GC, is granted. At this stage, the multicast MBS session is established with an inactive state.

2.	The VAL server decides to activate the multicast MBS session as VAL data is needed to be transmitted over the session to the VAL service group X, as a VAL group communication is to take place over the associated MBS session.

3.	The VAL server invokes the multicast resource activation request provided by NRM server, the MBS session ID is included.

4.	Upon receiving the request in step 3, the NRM server sends an MBS session activation request towards the 5GC, either directly to the MB-SMF or via NEF/MBSF, indicating the TMGI to be activated.

5.	The 5GC changes the session status to "active" and finds the list of joined VAL UEs associated with the session and activates the NG- RAN resources for VAL data delivery.

6.	The 5GC may send an MBS session activation response to the NRM server indicating that the requested multicast MBS session has been activated.

7.	The NRM server returns the multicast resource activation response to the VAL server.

##### 14.3.4A.5.3	Multicast MBS session de-activation procedure

The procedure shown in figure 14.3.4A.5.3-1 presents the multicast MBS session activation procedure initiated by the VAL server.

Pre-conditions:

-	NRM clients are attached to the 5GS, registered and affiliated to the same VAL service group X.

-	The NRM server has directly performed (or via NEF/MBSF) an MB-SMF discovery and selection, unless the corresponding information is locally configured.

-	A multicast MBS session is created and announced to address the corresponding VAL service group with certain service requirements and optionally with a certain multicast service area.

-	The VAL UE have already joined the multicast MBS session and are able to receive the VAL data over the associated MBS session.

Figure 14.3.4A.5.3-1: Multicast MBS session deactivation procedure.

1.	The group communication associated with VAL service group X takes place, and the corresponding VAL data is delivered over the associated multicast MBS session, hence the MBS session has an active state.

2.	The VAL server decides to deactivate the multicast MBS session, as no further VAL data to be delivered to the associated group, or the VAL group communication is over, and no further VAL media is to be delivered.

3.	The VAL server invokes the multicast resource deactivation request provided by NRM server, the MBS session ID is included.

4.	Upon receiving the request in step 3, the NRM server sends an MBS session deactivation request towards the 5GC, either directly to the MB-SMF or via NEF/MBSF, indicating the TMGI to be deactivated.

4.	The 5GC changes the session state to "inactive" and deactivates the radio resources associated with the joined VAL UEs.

5.	The 5GC may send an MBS deactivation response to the server indicating that the requested multicast MBS session has been inactivated.

7.	The NRM server may return the multicast resource deactivation response to the VAL server.

#### 14.3.4A.6	VAL service group media transmissions over 5G MBS sessions

##### 14.3.4A.6.1	General

The VAL server can decide to configure an MBS session per VAL service group to transmit the media related to the corresponding VAL group communications. Such group communications can comprise different service requirements. For that, multicast and broadcast MBS sessions need to be configured with multiple MBS QoS flows to address different service requirements, e.g., different required QoS, provided by the NRM server. For instance, application-level control messages or media associated to a group communication can comprise different QoS requirements. Also, different type of group communications can comprise different QoS requirements, e.g., emergency group communication should be handled with a higher priority than normal group communication.

The configuration of multiple MBS QoS flows to address different service requirements is associated to the assignment of different streams (e.g., different ports) within an MBS session.

The established multicast MBS session can either be in active or inactive state, where the former indicates the activation of radio resources hence transmitting the VAL media to the associated VAL service group, and the latter indicates their deactivation as no VAL media is being transmitted. The VAL server may initiate the activation of multicast MBS sessions once the VAL service group is established and active, as well as once the VAL media is available for transmission. For this purpose, the VAL server sends a multicast MBS session activation request towards the NRM indicating the MBS session ID to be activated, and the NRM server further interacts with the 5GS to complete the MBS session activation.

Similar to the use of eMBMS, the NRM server shall provide the associated information between a specific group communication and the stream to be used within an MBS session to the UE. This information could be sent in advance in an MBS session announcement or could be provided on demand in an additional signalling message for the MBS session, e.g., MapGroupToSessionStream (similar to the MapGroupToBearer in eMBMS).

##### 14.3.4A.6.2	Procedure

The procedure in figure 14.3.4A.6.2-1 describes how media related to a specific group communication can be distributed over a configured MBS session which consist of multiple QoS flows, i.e. addressing different service requirements.

Pre-conditions:

-	VAL UE 1 to n are attached to the 5GS, registered and belong to the same VAL service group X.

-	The VAL server has decided to use an MBS session for VAL service group communications associated to VAL service group X.

-	VAL UE 2 to n are within the MBS service area where the MBS session is configured.

Figure 14.3.4A.6.2-1: VAL service group media transmission over MBS sessions

1.	The VAL server creates a multicast or a broadcast MBS session targeting group communications associated to VAL service group X via the NRM server, as being specified in clause 14.3.4A.2. Therefore, the VAL server can provide default service requirements to be addressed by the MBS session.

The MBS session is announced by NRM server and received by NRM client 2 to n, which are within the MBS service area. The NRM server has identified that VAL UE 2 to n can receive media over the MBS sessions, e.g. based on a notification from NRM clients indicating the successful join of the multicast MBS session or a monitoring report of the broadcast MBS session (similar to the listening status report used for MBMS).

2.	A new VAL group communication is established for the VAL service group X consisting of a specific required service requirements, e.g. a VAL service emergency group communication. The group communication setup can be done over unicast.

2a. For broadcast MBS sessions, the session is established as described in 3GPP TS 23.247 [39].

2b. For multicast MBS session, the session is established upon the acceptance of the first UE session join request initiated from the VAL UE towards the 5GS, as described in 3GPP TS 23.247 [39]. The multicast session can then have either an active or an inactive state.

3. 	The VAL server may initiate the multicast MBS activation towards the NRM as described in clause 14.3.4A.5 in order to activate the multicast MBS session in case the session has an inactive state.

4.	Considering that the established group communication requires a specific QoS, e.g. an VALemergency group communication which requires higher priority (i.e. better ARP), the VAL server initiates an MBS session update to the NRM to provide the new required service requirements, if not done during the MBS session creation in step 1, as described in clause 14.3.4A.2. The MBS session should then be updated and an additional QoS flow may be configured.

5.	The NRM server sends a MapGroupToSessionStream to NRM clients 2 to n over the configured MBS session providing the required stream information to receive the media related to the specific established VAL group communication within the MBS session.

6.	NRM clients process the MapGroupToSessionStream information to receive the related media over the specific MBS session stream.

7.	VAL client 1 sends media to the VAL server over unicast to be distributed for the established group communication.

8.	The VAL server distributes the media to VAL clients 2 to n over the indicated stream within the established MBS session.

9.	The VAL server may initiate the multicast MBS session deactivation towards the NRM as described in clause 14.3.4A.5, in order to deactivate the multicast MBS session.

10.	The NRM server may further trigger the UE to leave multicast MBS session.

#### 14.3.4A.7	Aplication level control signalling over 5G MBS sessions

##### 14.3.4A.7.1	Description

The VAL server may use an 5G MBS session for application level control signalling. An 5G MBS session for application level control signalling is typically used for the purposes beyond the benefit for using 5G for resource efficiency, e.g. for improved MC service performance (KPIs), handling of high load scenarios.

Similar to the usage of eMBMS, both broadcast and multicast 5MBS session for application level control signalling may be used to transmit the following messages, for example:

-	MBS session announcement for media sessions

-	Group application paging

-	Group dynamic data (e.g. status of the group)

5G MBS session for application level control signalling is created in a service area that is larger than the estimated service for media MBS session. The service area for the media sessions is mainly based on counting of group members in each defined service area. The MBS session for application level control signalling is also created with a QoS that is better than MBS media session since the packet loss requirements are much stricter.

The NRM client shall not send responses to group-addressed application level control signalling unless instructed or configured to respond.

Editor’s note:	The definition of MBS session announcement is FFS.

##### 14.3.4A.7.2	Procedure

The procedure in figure 14.3.4A.7.2-1 shows only one of the receiving VAL UE using a 5G MBS session.

Figure 14.3.4A.7.2-1: Use of 5G MBS for application-level control signalling

1.	The VAL server determines to create MBS session for application-level control signalling, the VAL server initiated the 5G MBS session establishment via the NRM is done according to clause 14.3.4A.2.

2.	The NRM server passes the 5G MBS session info for the service description associated with the 5G MBS session to the NRM client. The NRM client obtains the MBS session ID, from the service description.

NOTE:	For 5G MBS and 4G MBMS co-existence, the MBMS bearers activation and MBS session announcement are performed as specified in the procedure for pre-created MBS session and session announcement.

3.	The NRM client stores the information associated with the MBS session ID. The NRM client uses the MBS session ID and other 5G MBS session related information to enable monitoring of the 5G MBS session by the VAL UE.

4.	Steps 4 to 6 defined in clause 14.3.4A.2 are performed.

5.	The VAL server transmits MC application control messages over the MBS session.

### 14.3.5	QoS/resource management for network-assisted UE-to-UE communications

#### 14.3.5.1	General

This feature provides the SEAL NRM support for coordinated QoS/resource management for network assisted UE-to-UE communications. Such capability may be required for guaranteeing end-to-end QoS fulfilment (primarily for meeting end-to-end latency requirements) in network assisted UE to UE communications and may accommodate various vertical-specific application services, e.g.:

-	Network-assisted Command and Control (C2) communications in UASAPP [TS 23.255], where the UAV controller navigates its UAV over the 5GS;

-	Teleoperated Driving (ToD) in eV2XAPP [TS 23.286], where the a V2X UE acting as server may remotely control a further V2X UE over the 5GS;

-	Network-assisted Device-to-Device communications in Factory of the Future (FF) use cases, such as control-to-control communications.

#### 14.3.5.2	QoS/resource management capability initiation in network assisted UE-to-UE communications

This procedure provides a mechanism for initiating the capability at the NRM server for managing the end-to-end application QoS requirement fulfilment for a network-assisted VAL UE to VAL UE session (comprising a PDU session for each of the constituent links, e.g. VAL UE 1 to PLMN, and PLMN to VAL UE 2). The request may come from NRM client of either of the VAL UEs within the service and will trigger the end-to-end QoS/resource management by the NRM server. The triggering the end-to-end QoS management request can be initiated by the VAL application at the VAL UE, and the conditions may depend on the requirements of the VAL service, e.g. for UAS such triggering may be needed when a UAV is in-flight, or for V2X such trigger may be initiated when a controlled VAL UE enters an urban area.

##### 14.3.5.2.1	Procedure

Figure 14.3.5.2.1-1 illustrates the procedure where the NRM server is initiating the end-to-end QoS/resource management capability for network-assisted UE-to-UE communications.

Pre-conditions:

1.	The NRM client is connected to the NRM server.

2.	The VAL UEs involved in the end-to-end session (VAL UE 1 and VAL UE 2) are connected to one or more PLMNs and have ongoing PDU sessions.

3.	NRM server has used the "Setting up an AF session with required QoS procedure" (clause 4.15.6.6 of TS 23.502).

Figure 14.3.5.2.1-1 end-to-end QoS management request / response

1.	The NRM client 1 (of VAL UE 1) sends to the NRM server an end-to-end QoS management request for managing the QoS for the end-to-end application session.

2.	The NRM server configures the application QoS parameters by decomposing the end-to-end QoS requirements (VAL UE 1 to VAL UE 2) to application QoS parameters for each individual session (e.g. network session for VAL UE 1 -and network session for VAL UE 2) which are part of the end-to-end application session.

3.	The NRM server sends to the NRM client 1 an end-to-end QoS management response with a positive or negative acknowledgement of the request.

4.	The NRM server may also send a notification to NRM client 2 (of VAL UE 2) to inform about the end-to-end QoS management initiation by the NRM server.

#### 14.3.5.3	Procedure for coordinated QoS provisioning operation in network assisted UE-to-UE communications

This procedure provides a mechanism for ensuring the end-to-end application QoS requirement fulfilment for the application service (which is between two or more VAL UEs), considering that the QoS of one of the links may downgrade. It is assumed that the application session is ongoing, and both the source and target VAL UEs are connected to 3GPP network (the same or different). The communication between the VAL UEs is assumed to be in-direct / network-assisted; hence two PDU sessions are established respectively (one per VAL UE).

##### 14.3.5.3.1	Procedure

Figure 14.3.5.3.1-1 illustrates the procedure where the NRM server supports the coordinated QoS provisioning for network-assisted UE-to-UE communications.

Pre-conditions:

1.	NRM server has activated the end-to-end QoS/resource management capability, as described in 14.3.5.2.1

2.	NRM server, acting as AF, has registered to receive QoS monitoring event notifications from 5GC and notifications from VAL UEs (from both UEs), as specified in TS 23.501.

Figure 14.3.5.3.1-1: NRM-assisted coordinated QoS provisioning for C2 communication

1a.	A QoS downgrade trigger event is sent from the NRM client of the VAL UE 1 to the NRM server, denoting an application QoS degradation (experienced or expected) e.g. based on the experienced packet delay or packet loss for the Uu link (e.g. packet loss great than threshold value). The conditions for triggering the QoS downgrade indication from the NRM client is based on the threshold that may be provided in advance by the NRM server (at the end-to-end QoS management response by the NRM server in 14.3.5.2.1).

1b.	Alternatively, the NRM server receives a trigger event from the 5GC (SMF/NEF), denoting a QoS downgrade notification for the VAL UE 1 session. (described in clause 5.7.2.4.1b of 3GPP TS 23.501).

2.	The NRM server evaluates the fulfilment/non-fulfilment of the end-to-end QoS based on the trigger event. NRM server may retrieve additional information based on subscription to support its evaluation. This could be from the 5GC (NEF Monitoring Events as in 23.502, QoS sustainability analytics as in TS 23.288 [34]) or SEAL LMS (on demand location reporting for one or both VAL UEs 1 and 2).

Then, the NRM server, determines an action, which is the QoS parameter adaptation of one or both links (QoS profile downgrade for the link receive QoS notification control, and QoS upgrade for the link which can be upgraded).

3.	The NRM/SEAL server, acting as AF, sends to the 5GC (to SMF via NEF or to PCF via N5) a request for a change of the QoS profile mapped to the one or both network sessions (for VAL UE 1 and UE 2) or the update of the PCC rules to apply the new traffic policy (as specified in 3GPP TS 23.502 in clause 4.15.6.6a: AF session with required QoS update procedure).

4.	The NRM server sends an application QoS change notification to the affected NRM clients, to inform on the adaptation of the QoS requirements for the individual session.

### 14.3.6	Event Monitoring

#### 14.3.6.1	General

The VAL server utilizes the NRM server for monitoring the events related to its VAL UEs and receive the event reports. The NRM server shall subscribe to multiple core network services to fetch all the required events related to the multiple VAL UEs served by the VAL server and report the same to the VAL server with the event details.

To monitor and report the events related to the VAL UE from the 3GPP core network, the NRM server shall use the Monitoring Events procedures as specified in 3GPP TS 23.502 [11].

To monitor and report the analytics events related to the VAL UE, the NRM server shall use the procedures specified in 3GPP TS 23.288 [34].

#### 14.3.6.2	Monitoring Events Subscription Procedure

##### 14.3.6.2.1	General

The VAL server subscribes to the NRM server to monitor the events related to VAL UE(s). Based on the VAL server request, the NRM server consumes the relevant core network services to receive the events related to the VAL UE(s). The related procedure is illustrated in the next clause.

##### 14.3.6.2.2	Procedure

The procedure for VAL server subscribing to the NRM server, to monitor the VAL UE(s) related events is described in figure 14.3.6.2.2-1.

Pre-conditions:

-	The NRM server is authorized to consume the core network services (Monitoring events as specified in 3GPP TS 23.502 [11] and Analytics services as specified in 3GPP TS 23.288 [34]);

Figure 14.3.6.2.2-1: Monitoring Events Subscription Procedure

1.	The VAL server sends Monitoring Events Subscription request to the NRM server, requesting the NRM server to monitor the events related to the VAL UE(s) as per the subscription request, and shall include the information related to the events that the VAL server is interested in.

2.	The NRM server shall check if the VAL server is authorized to initiate the Monitoring Events Subscription request and if authorized, shall respond with Monitoring Events Subscription Response message, indicating the successful subscription status along with subscription information to the VAL server. The VAL service ID may be used by the NRM server to derive event specific information in 3GPP core network services (e.g. QoS requirement in analytics event subscription), based on e.g. local configuration. The NRM server maps the VAL group ID (if received) to the External Group ID known to the 3GPP core network.

NOTE:	The mapping between Monitoring profile ID and event details in the NRM server can be pre-configured and/or dynamically built based on VAL server request with explicitly sent event details, which is implementation specific.

3.	Based on the events of interest information in the subscription request message, if applicable, the NRM server shall subscribe to the UE monitoring events (like, LOSS_OF_CONNECTIVITY, COMMUNICATION_FAILURE etc.) for the set of UEs (VAL UEs) in the subscription request, as specified in 3GPP TS 23.502 [11].

4.	Based on the events of interest information in the subscription request message, if applicable, the NRM server shall subscribe to the UE analytics events (like ABNORMAL_BEHAVIOUR etc.) for the set of UEs (VAL UEs) in the subscription request, as specified in 3GPP TS 23.288 [34].

#### 14.3.6.3	Monitoring Events Notification Procedure

##### 14.3.6.3.1	General

The NRM server receives the events related to VAL UE(s) from the 3GPP core network. The NRM server reports the monitoring events information to the VAL server.

##### 14.3.6.3.2	Procedure

The procedure for NRM server notifying the VAL server with VAL UE(s) related events is described in figure 14.3.6.3.2-1.

Pre-conditions:

-	The VAL server has subscribed with NRM server using Monitoring Events Subscription Procedure as specified in clause 14.3.6.2;

Figure 14.3.6.3.2-1: Monitoring Events Notification Procedure

1.	If applicable, the NRM server receives the VAL UE related monitoring event notifications from the 3GPP core network as specified in 3GPP TS 23.502 [11].

2.	If applicable, the NRM server receives the VAL UE related Analytics event notifications from the 3GPP core network as specified in 3GPP TS 23.288 [34].

3.	The NRM server notifies the VAL server about the events related to the VAL UE in Notify Monitorng Events message. If multiple events are to be notified, then the NRM server may aggregate the notifications and send to the VAL server.

### 14.3.7	5G TSC resource management procedures

#### 14.3.7.1	General

The procedures related to the 5G TSC network resource management are described in the following subclauses.

#### 14.3.7.2	TSC stream availability discovery procedure

The TSC stream availability discovery procedure is used by the VAL server to discover the availability of resources for TSC communication for the given stream specification (i.e., between the target UEs) prior to creating the stream.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session and DS-TTs are connected to the 5GS TSC bridge. The traffic classes are configured on each DS-TT.

2.	The NRM server has collected the 5GS TSC bridge management and port management information. The latter is related to the Ethernet ports located in the DS-TTs including bridge delay per DS-TT Ethernet port pair per traffic class.

3.	NRM server has calculated the bridge delay for each port pair, i.e. composed of (ingress DS-TT Ethernet port, egress DS-TT Ethernet port) including the UE-DS-TT residence time, packet delay budget (PDB) and propagation delay for both UL from sender UE and DL to receiver UE.

Figure 14.3.7.2-1: TSC stream availability discovery procedure

1.	The NRM server receives a request from a VAL server on NRM-S reference point to discover the connectivity and available QoS characteristics between DS-TTs identified by the stream specification.

2.	The NRM server validates the connectivity between the DS-TTs connected in the same 5GS TSC bridge based on the collected 5GS TSC bridge management and port management information, identifies the traffic classes supported by the DS-TTs and determines the end-to-end latency (including the UE-DS-TT residence times, PDBs and propagation delay).

3.	NRM server responds to the VAL server with the stream specification and a list of traffic specifications with the available end-to-end latency and the traffic classes supported by the DS-TTs.

#### 14.3.7.3	TSC stream creation procedure

This procedure allows the VAL server to create a TSC stream. The TSC stream creation procedure enables the VAL server to establish TSC connectivity with the required QoS between the UEs connected to the 5GS after the stream discovery procedure.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session for its DS-TT port MAC address.

2.	Connectivity between the DS-TTs has been validated by the TSC stream availability discovery procedure.

3.	NRM server maintains mapping from the traffic class to TSC QoS.

Figure 14.3.7.3-1: TSC stream creation procedure

1.	NRM server receives a TSC stream creation request from a VAL server to create a TSC stream identified by a VAL Stream ID, between DS-TT ports in the stream specification and for the traffic class in the traffic specification.

2.	NRM server calculates the schedule for the VAL Stream ID based on the information collected earlier from the 5GS via N5. It provides per-stream filtering and policing parameters (e.g. as defined in IEEE 802.1Q [IEEE8021Q]) used to derive the TSC QoS information and related flow information. NRM server also provides the forwarding rule (e.g. as defined in IEEE 802.1Q [IEEE8021Q]) used to identify the DS-TT MAC address of the corresponding PDU session. Based on the 5GS bridge delay information it determines the TSC QoS information and TSC Assistance information for the stream.

3.	As a TSCTSF, the NRM server triggers via N5 the Npcf_policy_Authorization_Create service operation as described in 3GPP TS 23.502 [11] for the TSC stream for both UL QoS flow (sender UE to UPF/bridge) and DL QoS flow (UPF/bridge to receiver UE). The Policy Authorization request includes the DS-TT port MAC address, TSC QoS information, TSC Assistance Information (3GPP TS 23.501 [1], cl.5.27.2.3), flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters. The QoS flow will be assigned for the PDU session for the source MAC address for the UL direction and for the PDU session for the destination MAC address for the DL direction. This information is delivered to the DS-TT by the 5GS.

Editor's note: Using gate control parameters for hold and forward buffering is FFS.

4.	NRM server sends TSC stream creation response to the VAL server with the result of TSC stream creation for the VAL Stream ID.

#### 14.3.7.4	TSC stream deletion procedure

This procedure allows the VAL server to delete a TSC stream.

Pre-conditions:

1.	The TSC stream is configured in the 5GS and the DS-TTs.

Figure 14.3.7.4-1: TSC stream deletion procedure

1.	NRM server receives a request from VAL server to delete a TSC stream for with a VAL Stream ID.

2.	NRM server identifies the MAC addresses of the DS-TTs involved in the stream based on the stored information for the VAL Stream ID.

3.	As a TSCTSF, the NRM server triggers via Nxx the Npcf_PolicyAuthorization_Delete service operation defined in 3GPP TS 23.502 [11] for MAC addresses referred to by the VAL Stream ID. NRM server uses the procedure to delete both UL QoS flow (sender UE to UPF/bridge) and DL QoS flows (UPF/bridge to receiver UE) from the PDU sessions of the UEs referred to by the VAL Stream ID.

4.	NRM server sends TSC stream deletion response to the VAL server with the result of TSC stream deletion for the VAL Stream ID.

### 14.3.8	TSN resource management procedures

#### 14.3.8.1	General

The procedures related to the TSN network resource management are described in the following subclauses.

#### 14.3.8.2	5GS TSN Bridge information reporting

Pre-conditions:

1.	There is already an established session between the TSN CNC and the NRM-S acting as TSN-AF.

Figure 14.3.8.2-1: TSN Bridge information reporting procedure

1.	Acting as the TSN AF the NRM server collects 5GS TSN Bridge information by interaction with the 5GS via the N5 reference point, as described in in TS 23.502 [11] Annex F.1. The NRM server stores the binding relationship between 5GS Bridge ID, MAC address of the DS-TT Ethernet port and also updates 5GS bridge delay as defined in clause 5.27.5 of TS 23.501 [10]. The NRM server retrieves txPropagationDelay and Traffic Class table from DS-TT and it also retrieves txPropagationDelay and Traffic Class table from NW-TT.

2.	Whenever there is a new or updated bridge information the NRM server interacts with the TSN CNC and reports the TSN Bridge information to register a new TSN Bridge or update an existing TSN Bridge. The TSN CNC stores the TSN Bridge information and confirms to the NRM server.

#### 14.3.8.3	5GS TSN Bridge configuration procedure

Pre-conditions:

1.	The TSN CNC has stored the 5GS TSN Bridge information received from the NRM server acting as TSN AF.

2.	The NRM server acting as TSN AF has stored the 5GS TSN Bridge information collected from the 5GS, as described in clause 14.3.8.2.

Figure 14.3.8.3-1: TSN Bridge configuration procedure

1.	The NRM server receives from the TSN CNC per-stream filtering, policing parameters and related flow information according to IEEE 802.1Q [36] and it uses them to derive TSN QoS information and related flow information. The TSN AF uses this information to identify the DS-TT MAC address of the corresponding PDU session.

2.	NRM server triggers via N5 the AF request procedure as described in 3GPP TS 23.502 [11] Annex F.2. The AF request includes the DS-TT port MAC address, TSC QoS information, TSC Assistance Information, flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters.

3.	NRM server responds with a TSN Bridge configuration.

### 14.3.9	Establishing communication with application service requirements

#### 14.3.9.1	General

The NRM client and the NRM server (acting as an AS) are involved in the exchange and analysis of the desired service requirements (e.g. packet size, packet transmission interval, reliability, packet error rate) for the E2E communication amongst the Vertical UEs. The NRM server triggers the establishment of direct service connectivity via Uu based on the information provided by the UEs and static configuration information available to the NRM server prior to the UE interaction. Note that service connectivity among VAL clients is established over the Uu, without device-to-device direct radio connectivity (e.g. PC5) requirement.

#### 14.3.9.2	Procedures

##### 14.3.9.2.1	Procedure for establishing communication with application service requirements

The procedure for establishing Uu service communication with application service requirements is as illustrated in figure 14.3.9.2.1-1.

Pre-conditions:

-	NRM client 1 and NRM client 2 are provided configuration information for the VAL clients served e.g. connectivity requirements, which destination UEs to connect to over Uu, etc.

-	The NRM client 1 and NRM client 2 are configured with the information of the NRM server and have connectivity enabled to communicate with the NRM server. The information is provided via pre-configuration.

-	The NRM server is configured with policies and information of the UEs to determine authorization of the UEs requesting connectivity via Uu.

-	The VAL clients associated with NRM client 1 and NRM client 2 have triggered the establishment of connectivity.

Figure 14.3.9.2.1-1: Establishing communication with application service requirements

1a.	The NRM client 1 sends the application connectivity request (source identity and IP address, destination identities, service requirements) to the NRM Server. The service requirement from the source includes packet size, packet transmission interval, packet E2E latency, allowed packet loss rate/packet loss amount/packet error rate, etc. The destination may be multiple UEs (devices). The identity of source and destination may be the application user identity or the MAC address.

1b.	The NRM server determines whether the UE of NRM client 1 is authorized to connect to the destination UEs for direct service communications via Uu. If UE of NRM client 1 is authorized to connect to the destination UEs, then a response is provided to the NRM client 1 indicating acceptance of the request.

2a.	The NRM client 2 sends the application connectivity request (destination identity and IP address, source identity, service requirements) to the NRM server. The service requirements from the destination includes the service requirements as described in step 1a.

2b.	The NRM server determines whether the UE of NRM client 2 is authorized to connect to the destination UEs for direct service communications via Uu. If UE of NRM client 2 is authorized to connect to the destination UEs, then a response is provided to the NRM client 2 indicating acceptance of the request.

3.	Based on the service requirements received in step 1 and step 2, the NRM server determines the parameters and patterns for direct service connectivity between the UEs via Uu and also the transport requirements, i.e., QoS requirements for the 3GPP system (e.g. 5GS). This step may also include retrieving the direct link status of the UEs (e.g. PDU Session Status, UE reachability). If the NRM server determines that direct service connectivity via Uu is not authorized or not possible with the given connectivity requirements, it skips step 4 and proceeds to steps 5 and 6, informing each NRM client accordingly.

NRM server will process E2E connectivity establishment between NRM client 1 and NRM client 2 only after it receives the request from NRM client 2. There can be several NRM clients (destinations) which will perform step 2 and NRM server will process their E2E connectivity with NRM client 1 (source) as and when the requests are received by the NRM server.

4.	The NRM server triggers 3GPP system to establish Uu connectivity between the UE of NRM client 1 and UE of NRM client 2 with required QoS as specified in 3GPP TS 23.501 [10].

5.	The NRM server sends the application connectivity notification (connectivity/session information) to NRM client 1 indicating successful establishment of the connectivity. The connectivity/session information may contain the accepted destination identities.

6.	The NRM server sends the application connectivity notification (connectivity/session information) to NRM client 2 indicating successful establishment of the connectivity.

## 14.4	SEAL APIs for network resource management

### 14.4.1	General

Table 14.4.1-1 illustrates the SEAL APIs for configuration management.

Table 14.4.1-1: List of SEAL APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkResourceAdaptation | Reserve_Network_Resource | VAL server | Request /Response |
|  | Request_Unicast_Resource | VAL server | Request /Response |
|  | Update_Unicast_Resource | VAL server | Request /Response |
|  | Request_Multicast_Resource | VAL server | Request /Response |
|  | Notify_UP_Delivery_Mode | VAL server | Subscribe/Notify |
|  | TSC_Stream_Availability_Discovery | VAL server | Request /Response |
|  | TSC_Stream_Creation | VAL server | Request /Response |
|  | TSC_Stream_Deletion | VAL server | Request /Response |
| SS_EventsMonitoring | Subscribe_Monitoring_Events | VAL server | Subscribe/Notify |
|  | Notify_Monitoring_Events | VAL server |  |
| SS_NetworkResourceMonitoring | Subscribe_Unicast_QoS_Monitoring | VAL server | Subscribe/Notify |
|  | Notify_Unicast_QoS_Monitoring | VAL server |  |
|  | Unsubscribe_Unicast_QoS_Monitoring | VAL server |  |
|  | Obtain_Unicast_QoS_Monitoring_Data | VAL server | Request /Response |
|  | Update_Unicast_QoS_Monitoring_Subscription | VAL server |  |

### 14.4.2	SS_NetworkResourceAdaptation API

#### 14.4.2.1	General

API description: This API enables the VAL server to communicate with the network resource management server for network resource adaptation and VAL UE monitoring over NRM-S.

#### 14.4.2.2	Reserve_Network_Resource operation

API operation name: Reserve_Network_Resource

Description: Requesting for network resource adaptation.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.1

Outputs: See subclause 14.3.2.2

See subclause 14.3.3 for the details of usage of this API operation.

#### 14.4.2.3	Request_Unicast_Resource

API operation name: Request_Unicast_Resource

Description: Requesting unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.6

Outputs: See subclause 14.3.2.7

See subclause 14.3.3 for the details of usage of this API operation.

#### 14.4.2.4	Update_Unicast_Resource

API operation name: Update_Unicast_Resource

Description: Updating unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.8

Outputs: See subclause 14.3.2.9

See subclause 14.3.3 for the details of usage of this API operation.

#### 14.4.2.5	Request_Multicast_Resource

API operation name: Request_Multicast_Resource

Description: Requesting multicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.10

Outputs: See subclause 14.3.2.11

See subclause 14.3.4 for the details of usage of this API operation.

#### 14.4.2.6	Notify_UP_Delivery_Mode

API operation name: Notify_UP_Delivery_Mode

Description: Notifying the user plane delivery mode.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.12

Outputs: None.

See subclause 14.3.4 for the details of usage of this API operation.

#### 14.4.2.7	TSC_Stream_ Availability_Discovery

API operation name: TSC_Stream_ Availability_Discovery

Description: Requesting to discover the connectivity and available QoS characteristics between DS-TTs.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.23

Outputs: See subclause 14.3.2.24

See subclause 14.3.7.2 for the details of usage of this API operation.

#### 14.4.2.8	TSC_Stream_Creation

API operation name: TSC_Stream_Creation

Description: Requesting the NRM to create a TSC stream.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.25

Outputs: See subclause 14.3.2.26

See subclause 14.3.7.3 for the details of usage of this API operation.

#### 14.4.2.9	TSC_Stream_Deletion

API operation name: TSC_Stream_Deletion

Description: Requesting the NRM to delete the TSC stream.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.27

Outputs: See subclause 14.3.2.28

See subclause 14.3.7.4 for the details of usage of this API operation.

### 14.4.3	SS_EventsMonitoring API

#### 14.4.3.1	Subscribe_Monitoring_Events

API operation name: Subscribe_Monitoring_Events

Description: Subscription to monitoring events.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.17

Outputs: 14.3.2.18.

See subclause 14.3.6.2 for the details of usage of this API operation.

#### 14.4.3.2	Notify_Monitoring_Events

API operation name: Notify_Monitoring_Events

Description: Notifying the VAL server with monitoring events related to VAL UE(s).

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.19

Outputs: None.

See subclause 14.3.6.3 for the details of usage of this API operation.

### 14.4.4	SS_NetworkResourceMonitoring API

#### 14.4.4.1	General

API description: This API enables the VAL server to monitor a network resource and to retrieve monitoring data.

#### 14.4.4.2	Subscribe_Unicast_QoS_Monitoring operation

API operation name: Subscribe_Unicast_QoS_Monitoring

Description: Subscribing to QoS monitoring of a unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.20

Outputs: See subclause 14.3.2.21

See subclause 14.3.3.4.1 for the details of usage of this API operation.

#### 14.4.4.3	Notify_Unicast_QoS_Monitoring operation

API operation name: Notify_Unicast_QoS_Monitoring

Description: Notification of latest QoS data of a monitored unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.22

Outputs: See subclause 14.3.2.22

See subclause 14.3.3.4.2 for the details of usage of this API operation.

#### 14.4.4.4	Unsubscribe_Unicast_QoS_Monitoring operation

API operation name: Unsubscribe_Unicast_QoS_Monitoring

Description: Unsubscribing from QoS monitoring of a unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.20

Outputs: See subclause 14.3.2.21

See subclause 14.3.3.4.3 for the details of usage of this API operation.

#### 14.4.4.5	Obtain_Unicast_QoS_Monitoring_Data operation

API operation name: Obtain_Unicast_QoS_Monitoring_Data

Description: Request QoS monitoring data of a unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.33

Outputs: See subclause 14.3.2.34

See subclause 14.3.3.4.4 for the details of usage of this API operation.

#### 14.4.4.6	Update_Unicast_QoS_Monitoring_Subscription operation

API operation name: Update_Unicast_QoS_Monitoring_Subscription

Description: Updates the QoS monitoring subscription of a unicast resource.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.38

Outputs: See subclause 14.3.2.39

See subclause 14.3.3.4.5 for the details of usage of this API operation.

# 15	Service-based interface representation of the functional model for SEAL services

## 15.1	General

The functional models for SEAL services is represented using functional entities and reference points between the functional entities as specified in subclause 6. The vertical applications consume the SEAL services in the form of APIs. Each SEAL service offers these APIs on a service-based interface to all its consumer entities.

## 15.2	Functional model representation

Figure 15.2-1 illustrates the service-based interface representation of the functional model for SEAL services.

Figure 15.2-1: SEAL generic functional model representation using service-based interfaces

The SEAL function(s) exhibit the service-based interfaces which are used for providing and consuming SEAL services. The service APIs are specified for each SEAL function enabled over the service-based interface. The service-based interfaces of specific SEAL services are specified in this document. All the interactions with SEAL are governed based on the reference point interactions of the functional models specified in subclause 6. VAL function represents the functionalities of the VAL server.

NOTE:	The service-based interface Sval for the VAL function is out of scope of the present document.

The service APIs offered by the SEAL function(s) are published and discovered on the CAPIF core function as specified in 3GPP TS 23.222 [8].

Editor's note:	Refinement of the SEAL services in service-based interface representation is FFS.

## 15.3	Service-based interfaces

Table 15.3-1 specifies the service-based interfaces supported by SEAL.

Table 15.3-1: Service-based interfaces supported by SEAL

| Service-based interface | Application functionEntity | Mapping server entity | APIs offered |
| --- | --- | --- | --- |
| Slm | Location management function | Location Management Server | Specified in subclause 9.4 |
| Sgm | Group management function | Group management server | Specified in subclause 10.4 |
| Scm | Configuration management function | Configuration management server | Specified in subclause 11.4 |
| Sim | Identity management function | Identity management server | Specified in subclause 12.4 |
| Skm | Key management function | Key management server | Specified in subclause 13.4 |
| Snrm | Network resource management function | Network resource management server | Specified in subclause 14.4 |
| Snsce | Network slice capability enablement function | Network slice capability enablement server | Specified in subclause 16.4 |
| Cccf | CAPIF core function | Not applicable | Specified in subclause 10 of 3GPP TS 23.222 [8] |

# 16	Network slice capability enablement

## 16.1	General

The network slice capability enablement is a SEAL service that offers network slice capability enablement capabilities, such as support for vertical application to slice re-mapping (which can be defined as the mapping of the UEs running a vertical application to different slice), to one or more vertical applications. In particular, network slice capability enablement uses a network-based mechanism to apply the slice re-mapping based on the network slice capability enablement server configuration, where the network slice capability enablement server acting as AF influences the URSP rules for the application traffic per UE by providing a guidance on the route selection parameters (including the S-NSSAI and DNN mapping).

NOTE:	The network slice capability enablement server, acts as an AF within the trusted domain (MNO-owned, or trusted third party).

## 16.2	Functional model

### 16.2.1	General

The functional model for the network slice capability enablement is based on the generic functional model specified in clause 6.2. It is organized into functional entities to describe a functional architecture which addresses the support for network slice capability enablement aspects for vertical applications. Since the slicing is a feature which considers the Uu interfaces, only the on-network functional model is specified in this clause.

### 16.2.2	Functional model description

Figure 16.2.2-1 illustrates the generic functional model for network slice capability enablement.

Figure 16.2.2-1: Functional model for network slice capability enablement

The network slice capability enablement client communicates with the network slice capability enablement server over the NSCE-UU reference point. The network slice capability enablement client provides the support for network slice capability enablement functions to the VAL client(s) over NSCE-C reference point. The VAL server(s) communicates with the network slice capability enablement server over the NSCE-S reference point. It is assumed that the network slice capability enablement server is deployed at the 5G system domain. The network slice capability enablement server, acting as AF, may communicate with the 5G Core Network functions via NEF (N33) reference point (for interactions with PCF).

### 16.2.3	Functional entities description

#### 16.2.3.1	General

The functional entities for network slice capability enablement SEAL service are described in the following subclauses.

#### 16.2.3.2	Network slice capability enablement server

The network slice capability enablement server functional entity provides the enablement of the network slicing aspects to support the VAL applications. Such enablement supports the mapping or migration of one or more vertical applications to one or more network slices (from a set of network slices, as provided by the 3GPP network system) as described in procedure in clauses 16.3.2.3 and 16.3.2.4. Such adaptation assumes that the UE is subscribed to more than one slice and is done via providing a guidance to update the URSP rules at the 5GS (denoted in clause 16.3.3 as network-based mechanism).

#### 16.2.3.3	Network slice capability enablement client

The network slice capability enablement client functional entity acts as the application client for the slice enablement. The network slice capability enablement client interacts with the network slice capability enablement server to trigger a network slice adaptation due to an application requirement change. This trigger may be due to an application QoS requirement change, a service operation change. The NSCE client may receive a network slice / DNN re-mapping notification from the NSCE server. The NSCE client may optionally notify the VAL client on the network slice / DNN re-mapping.

### 16.2.4	Reference points description

#### 16.2.4.1	General

The reference points for the functional model for network slice capability enablement are described in the following subclauses.

#### 16.2.4.2	NSCE-UU

The interactions related to network slice capability enablement functions between the network slice capability enablement server and the network slice capability enablement client are supported by NSCE-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.501 [10].

#### 16.2.4.3	NSCE-C

The interactions related to network slice capability enablement functions between the VAL client(s) and the network slice capability enablement client within a VAL UE are supported by the NSCE-C reference point. The NSCE client may receive an application requirement change over NSCE-C. Further, the NSCE client may provide a notification on the network slice adaptation upon successful adaptation of the slice to application mapping.

#### 16.2.4.4	NSCE-S

The interactions related to network slice capability enablement functions between the VAL server(s) and the network slice capability enablement server are supported by the NSCE-S reference point. This reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

#### 16.2.4.5	N33

The reference point N33 supports the interactions between the network slice capability enablement server and the NEF and is specified in 3GPP TS 23.501 [10]. N33 is used for the network-based mechanism for slice re-mapping, where NSCE server acting as AF influences the URSP rules for the application traffic per UE by providing a guidance on the route selection parameters (including the S-NSSAI and DNN mapping), as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4.

## 16.3	Procedures and information flows for network slice capability enablement

### 16.3.1	General

### 16.3.2	Network slice adaptation for VAL application

#### 16.3.2.1	General

This subclause describes the procedure for network slice adaptation at the Network Slice Capability Enablement (NSCE) server, based on a request from a VAL server to adapt the network slice for the VAL application. This request is handled between the NSCE server and the NSCE client per each VAL UE of the VAL application.

#### 16.3.2.2	Information flows

##### 16.3.2.2.1	Network slice adaptation request

Table 16.3.2.2.1-1 describes the information flow network slice adaptation request from the VAL server to the NSCE server.

Table 16.3.2.2.1-1: Network slice adaptation request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation may corresponds to. |
| List of VAL UE IDs | M | List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds |
| Requested slice information | O | Indication of the new slice which is requested. The slice information can be either the S-NSSAI or the ENSI (as specified in TS 33.501, for AF outside 3GPP MNO domain). |
| Requested DNN | O | Indication of the new DNN which is requested. |

##### 16.3.2.2.2	Network slice adaptation response

Table 16.3.2.2.2-1 describes the information flow network slice adaptation response from the NSCE server to the VAL server.

Table 16.3.2.2.2-1: Network slice adaptation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation with the underlying network. |

##### 16.3.2.2.3	Network slice adaptation trigger

Table 16.3.2.2.3-1 describes the information flow Network slice adaptation trigger from the NSCE client to the NSCE server.

Table 16.3.2.2.3-1: Network slice adaptation trigger

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation trigger applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice configuration may corresponds to. |
| Requested S-NSSAI | M | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |

##### 16.3.2.2.4	Network slice adaptation notification

Table 16.3.2.2.4-1 describes the information flow network slice adaptation notification from the NSCE server to the NSCE client and optionally to the VAL client.

Table 16.3.2.2.4-1: Network slice adaptation notification

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network slice adaptation. |

#### 16.3.2.3	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.3-1 illustrates the VAL server-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Figure 16.3.2.3-1: Network slice adaptation for VAL application

1.	The VAL server sends a network slice adaptation request to the NSCE server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN).

2.	The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

3.	The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE:	NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

4.  Upon successful adaptation of the route selection parameters, the NSCE server provides a network slice adaptation response to the VAL server, providing information on the fulfilment of the network slice adaptation request per VAL application.

#### 16.3.2.4	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application

Figure 16.3.2.4-1 illustrates the VAL UE-triggered and network-based procedure where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Pre-condition:

-	The NSCE client has connected to the NSCE server;

Figure 16.3.2.4-1: Network slice adaptation for VAL application

1.	The VAL client provides a new application requirement to the NSCE client, indicating a new service profile for the VAL application. This may be in the form of a change at the application QoS requirements, service operation change, or other application-related parameters.

2.	The NSCE client sends a network slice adaptation trigger to the NSCE server for the VAL application. This trigger may be in the form of exact requested network slice (and optionally DNN) for the VAL UE of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN).

NOTE 1：How the requested network slice is known by the NSCE client is out of scope of this release.

3.	The NSCE server processes the request and triggers the network slice configuration per VAL UE within the VAL Application.

NOTE 2：Whether and how the NSCE server triggers the network slice adaptation for all the VAL UEs within the VAL Application is out of scope of this release.

4.	The NSCE server acting as AF provides the updated S-NSSAI and DNN per VAL UE. In particular, NSCE server sends this information to the PCF via NEF as part of the AF-driven guidance for URSP determination to 5G system (as specified in TS23.502 clause 4.15.6.10, TS 23.503 clause 6.6.2.2, TS 23.548 clause 6.2.4). This guidance may update the route selection parameters to indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic.

NOTE 3:	NSCE server provides the updated S-NSSAI/DNN as a suggestion/guidance to PCF; however it is up to PCF to decide whether to perform the slice/DNN re-mapping

5. The NSCE server may send a notification to the NSCE client indicating success or failure.

## 16.4	SEAL APIs for network slice capability enablement

### 16.4.1	General

Editor's note: The definition of SEAL APIs for NSCE-UU is FFS

Table 16.4.1-1 illustrates the SEAL APIs for network slice capability enablement.

Table 16.4.1-1: List of SEAL APIs for network slice capability enablement

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkSliceAdaptation | Network_slice_adaptation | VAL server | Request /Response |

### 16.4.2	SS_NetworkSliceAdaptation API

#### 16.4.2.1	General

API description: This API enables the VAL server to communicate with the network slice capability enablement server for network slice adaptation over NSCE-S.

#### 16.4.2.2	Network_Slice_Adaptation

API operation name: Network_Slice_Adaptation

Description: Requesting for network slice adaptation.

Known Consumers: VAL server.

Inputs: See subclause 16.3.2.2.1

Outputs: See subclause 16.3.2.2.2

See subclause 16.3.2.3 for the details of usage of this API operation.

# 17	Notification Management

## 17.1	General

The notification management is a SEAL service that offers the notification functionality to one or more verticals. This service enables VAL clients to subscribe and receive notifications from the VAL servers and thereby offloading the complexity of delivery and reception of notifications to the enabler layer. It provides common notification delivery service to vertical applications.

## 17.2	Functional model

### 17.2.1	General

The functional model for the notification management is based on the generic functional model specified in clause 6.2. It is organized into functional entities to describe a functional architecture which addresses the notification management aspects required for vertical applications. Since the notification management is a feature which considers the Uu interfaces, only the on-network functional model is specified in this clause.

### 17.2.2	Functional model description

Figure 17.2.2-1 illustrates the generic functional model for notification management.

Figure 17.2.2-1: Functional model for notification management

The notification management client communicates with the notification management server over the NM-UU reference point. The notification management client provides the support for notification management functions to the VAL client(s) over NM-C reference point. The VAL server(s) communicates with the notification management server over the NM-S reference point for delivering the notification messages which is targeted for the VAL client(s). Notification management server sends these notification messages to the notification management client either through NM-UU interface for direct delivery (e.g. Long-polling, WebSocket) or through the OEM PUSH server for indirect delivery (e.g. FCM, APNS, OMA PUSH) which is implementation specific and outside the scope of this specification.

NOTE:	Notification messages from PUSH notification server to the PUSH notification client are delivered either through 3GPP network system or through any non-3GPP system, which is implementation specific.

### 17.2.3	Functional entities description

#### 17.2.3.1	General

The functional entities for notification management service are described in the following subclauses.

#### 17.2.3.2	Notification Management client

The notification management client functional entity acts as the application client for notification management aspects. It interacts with the notification management server. It handles the notification messages received from the notification management server and deliver it to the corresponding VAL clients residing on the VAL UE.

The notification management client functional entity is supported by the HTTP client functional entities of the signalling control plane.

#### 17.2.3.3	Notification Management server

The notification management server is a functional entity that handles the notification management aspects by interacting with the notification management client and the VAL servers. The notification management server receives the notification messages from the vertical application layer and delivers it to the notification management client.

Editor's note: Notification management server acting as CAPIF’s API exposing function as specified in 3GPP TS 23.222 [8] is FFS.

### 17.2.4	Reference points description

#### 17.2.4.1	General

The reference points for the functional model for notification management are described in the following subclauses.

#### 17.2.4.2	NM-UU

The interactions related to notification management functions between the notification management client and the notification management server are supported by NM-UU reference point. This reference point utilizes Uu reference point as described in 3GPP TS 23.401 [9] and 3GPP TS 23.501 [10].

#### 17.2.4.3	NM-C

The interactions related to notification management functions between the VAL client(s) and the notification management client within a VAL UE are supported by NM-C reference point.

#### 17.2.4.4	NM-S

The interactions related to notification management functions between the VAL server(s) and the notification management server are supported by NM-S reference point.

Editor's note: NM-S reference point using CAPIF-2 reference point as specified in 3GPP TS 23.222 [8] is FFS.

## 17.3	Procedures and information flows for notification management

### 17.3.1	General

This sub-clause describes the procedure and information flows for notification management service. The notification management procedures apply to on-network VAL service only.

### 17.3.2	Information flows for notification management

#### 17.3.2.1	Create notification channel request

Table 17.3.2.1-1 describes the information flow create notification channel request from the notification management client to the notification management server.

Table 17.3.2.1-1: Create notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the requesting notification management client |
| Channel type | M | Indicates PULL or PUSH method to be used for delivering the notification messages |
| PUSH channel details (see NOTE) | O | Carries the details of the type of PUSH delivery and its associated data |
| Validity Duration | M | How long the notification channel will last (i.e. channel lifetime) as requested by the notification management client. |
| NOTE : This IE shall be present of the channel type is PUSH |  |  |

#### 17.3.2.2	Create notification channel response

Table 17.3.2.2-1 describes the information flow create notification channel response from the notification management server to the notification management client.

Table 17.3.2.2-1: Create notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Notification URL | O | The URL to receive the notification message if a PULL method is requested. For some PUSH method implementation (such as WebSockets) this URL is used to start the PUSH notification service from the notification management server |
| Callback URL | O | The URL to be shared to the VAL server by the VAL client for sending the notifications to the notification management server |
| Validity duration | O | How long the notification channel will last (i.e. channel lifetime) as granted by the notification management server. |
| Channel ID | M | Unique identifier for the created notification channel |

#### 17.3.2.3	Open notification channel

Table 17.3.2.3-1 describes the information flow open notification channel from the notification management client to the notification management server.

Table 17.3.2.3-1: Open notification channel

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Notification URL | M | The URL to receive the notification message and it is same as received in the Create notification channel response |
| Channel ID | M | Identifies the notification channel and the value shall be same as returned in Create notification channel response |

#### 17.3.2.4	Notification message

Table 17.3.2.4-1 describes the information flow notification message from VAL server to the notification management server and from notification management server to the notification management client.

Table 17.3.2.4-1: Notification Message

| Information element | Status | Description |
| --- | --- | --- |
| VAL Application ID | M | Identity of the VAL application residing in the VAL UE this notification message is targeted |
| VAL Service ID | M | Identity of the VAL service for which this notification is generated |
| Notification data | M | This information element carries the details of the notification data such as type of the event and the data corresponding to the event |

#### 17.3.2.5	Delete notification channel request

Table 17.3.2.5-1 describes the information flow delete notification channel request from notification management client to the notification management server.

Table 17.3.2.5-1: Delete notification channel request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Channel ID | M | Identifies the notification channel to be deleted |
| Request type | M | De-register or delete channel |

#### 17.3.2.6	Delete notification channel response

Table 17.3.2.6-1 describes the information flow delete notification channel response from notification management server to the notification management client.

Table 17.3.2.6-1: Delete notification channel response

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting notification management client |
| Result | M | Indicates if the deletion of the channel is success or failure or if the VAL server de-registered from channel sucessfully |

### 17.3.3	Procedure for creating notification channel to receive notifications

Figure 17.3.3-1 below illustrates the procedure for receiving notifications from the notification management server by the notification management client.

Pre-conditions:

1.	The notification management client does not have any notification channel open with the notification management server.

Figure 17.3.3-1: Receiving notifications from notification management server

1.	The VAL client requests the notification management client to subscribe to and receive notifications from the VAL server.

2.	The notification management client creates notification channels (i.e. endpoint URLs), if not created already, to be used by the VAL server to send notification messages. To create notification channel, the notification management client sends a create notification channel request to the notification management server. The desired validity duration for the channels to be used and the notification channel type (PUSH or PULL) are included in the request. The notification management client decides which type of the notification channel to be used based on the UE capability.

3.	The notification management server authenticates the notification management client and authorizes its request.

4.	The notification management server sends the notification management client the Create notification channel response with the endpoint URLs that will be used by the VAL server to send the notification messages and the notification management client to receive the notification messages. The notification management server also includes what is the valid duration for these endpoint URLs to be used and the channel ID to uniquely identify the channel created in the response.

5.	If the notification type is PULL method, the notification management client sends the Open notification channel to the notification management server to start receiving the notification message. For certain PUSH method notification type (such as WebSockets) the notification management client requests the notification management server to start the PUSH notification service with its specific protocol that is outside the scope of this specification.

NOTE 1:	This step is not required if the notification client wants to receive the notification messages via PUSH notification server.

6.	The notification management client responds to the VAL client the status of its request as in step 1 to receive notifications from the VAL server. It also includes the callback URL which needs to be used by the VAL client while subscribing to the VAL server for receiving notifications.

7.	The VAL client subscribes to the VAL server for receiving notifications. This request includes the subscription data and the callback URL returned from the notification management server in step 4.

NOTE 2:	This step is outside the scope of this specification.

8.	The VAL server generates the notification message for the subscription request from the VAL client.

9.	The VAL server sends the notification message to the notification management server containing the VAL UE or VAL user ID, VAL service ID, VAL application ID and the notification data.

10.	If the delivery method is PULL, the notification management server sends the notification message to the notification management client over the opened notification channel.

NOTE 3:	If the delivery method is PUSH via PUSH notification server, the notification management server sends the notification message to the OEM Push server (not shown in the figure and is outside the scope of this specification) to deliver to the notification management client.

11.	The notification management client delivers the received notification message to the appropriate VAL client based on the details as received in the notification message from the notification management server.

Editor's note: How to support different types of PULL and PUSH mechanisms for delivery of notification data is FFS.

### 17.3.4	Procedure for deleting notification channel

Figure 17.3.4-1 below illustrates the procedure for deleting notification channel by the notification management client.

Pre-conditions:

1.	The notification management client has notification channel open with the notification management server.

Figure 17.3.4-1: Deletion of notification channel by the notification management client1.	The VAL client decides to stop receiving notifications from the VAL server and deletes the subscription with the VAL server.

NOTE:	Step 1 is outside the scope of 3GPP but triggers step 2.

2.	The VAL client requests the notification management client to stop receiving the notifications.

3.	If the same notification channel is being used for other VAL clients in the UE, the notification management client sends the delete notification channel request with request type as de-register otherwise the notification management client sends the delete notification channel request with request type as delete to the notification management server. The delete notification channel request carries the channel ID of the notification channel.

4.	The notification management server acknowledges the request and sends the delete notification channel response to the notification management client.

5.	The notification management client indicates to the VAL client about the status (de-registered or channel deleted or failure) of its request to stop receiving the notification.

Annex A (informative):
SEAL integration with 3GPP network exposure systems

NOTE:	Not all possible SEAL integration with 3GPP network exposure systems are illustrated in this subclause.

Figure A-1 illustrates the service-based interface representation of the functional model for SEAL services integration with 5GC network exposure system.

Figure A-1: SEAL integration with 5GC network exposure system

The details of NEF and its role in exposing network capabilities of 5GS to 3rd party applications are specified in 3GPP TS 23.501 [10] and the details of NEF service operations are specified in 3GPP TS 23.502 [11].

Figure A-2 illustrates the service-based interface representation of the functional model for SEAL services integration with EPC network exposure system.

Figure A-2: SEAL integration with EPC network exposure system

The details of SCEF and its role in exposing network capabilities of EPS to 3rd party applications are specified in 3GPP TS 23.682 [13].

Annex B (informative):
SEAL functional model mapping with Common functional architecture (CFA)

The table B-1 shows the mapping between the SEAL functional model and the Common functional architecture (CFA). The details of CFA functional entities and reference points are specified in 3GPP TS 23.280 [4].

Table B-1: SEAL functional model mapping with CFA

| SEAL service | Aspects | SEAL | CFA |
| --- | --- | --- | --- |
| Location management | Functional entity | Location management client | Location management client |
|  |  | Location management server | Location management server |
|  | Reference points | LM-UU | CSC-14 |
|  |  | LM-S | CSC-15 |
|  |  | LM-C | Not defined |
|  |  | LM-E | Not defined |
|  |  | LM-PC5 | Not defined |
| Group management | Functional entity | Group management client | Group management client |
|  |  | Group management server | Group management server |
|  | Reference points | GM-UU | CSC-2 |
|  |  | GM-S | CSC-3 |
|  |  | GM-C | Not defined |
|  |  | GM-E | CSC-16 |
|  |  | GM-PC5 | CSC-12 |
| Configuration management | Functional entity | Configuration management client | Configuration management client |
|  |  | Configuration management server | Configuration management server |
|  | Reference points | CM-UU | CSC-4 |
|  |  | CM-S | CSC-5 |
|  |  | CM-C | Not defined |
|  |  | CM-E | CSC-17 |
|  |  | CM-PC5 | CSC-11 |
| Identity management | Functional entity | Identity management client | Identity management client |
|  |  | Identity management server | Identity management server |
|  | Reference points | IM-UU | CSC-1 |
|  |  | IM-S | Not defined |
|  |  | IM-C | Not defined |
|  |  | IM-E | Not defined |
|  |  | IM-PC5 | Not defined |
| Key management | Functional entity | Key management client | Key management client |
|  |  | Key management server | Key management server |
|  | Reference points | KM-UU | CSC-8 |
|  |  | KM-S | CSC-9 |
|  |  | KM-PC5 | Not defined |
| Network resource management | Functional entity | Network resource management client | Not defined (see NOTE) |
|  |  | Network resource management server | Not defined (see NOTE) |
|  | Reference points | NRM-UU | Not defined (see NOTE) |
|  |  | NRM-S | Not defined |
|  |  | NRM-C | Not defined |
|  |  | NRM-E | Not defined |
|  |  | NRM-PC5 | Not defined |
| NOTE: Defined in the application layer for Mission Critical service (e.g. MCPTT). |  |  |  |

Annex C (normative):
Protocol realizations of LWP in the signalling control plane

# C.1	General

This annex specifies protocol realizations of the light-weight protocol in the signalling control plane.

# C.2	Usage of CoAP as LWP

This clause specifies how the CoAP protocol shall be used to realize the generic light-weight protocol in the signalling control plane.

The Constrained Application Protocol (CoAP) is a light-weight protocol defined by IETF in RFC 7252 [32] and designed specifically for application layer communication for constrained devices. CoAP provides a request/response interaction model between application endpoints, supports built-in discovery of services and resources, and includes key concepts of the Web such as URIs and Internet media types. CoAP is designed to easily interface with HTTP for integration with the Web while meeting specialized requirements such as multicast support, very low overhead, and simplicity for constrained environments. RFC 7252 [32] specifies bindings to UDP and DTLS. IETF RFC 8323 [33] specifies bindings to TCP, WebSocket and TLS.

Figure C.2-1 illustrates the functional model for the LWP signalling control plane when CoAP is used as the LWP.

Figure C.2-1: Functional model for LWP signalling control plane when CoAP is used as LWP

When CoAP is used to realize the generic light-weight protocol defined in clause 6.2, then,

1.	CoAP client is a realization of the LWP client

2.	CoAP proxy is a realization of the LWP proxy, with the following clarifications:

a.	CoAP proxy shall be able to terminate a DTLS, TLS or secure WebSocket session on LWP-1 reference point;

b.	CoAP proxy shall be able to act as a cross-protocol CoAP-HTTP proxy to support LWP-HTTP-2 and LWP-HTTP-3 reference points;

3.	CoAP server is a realization of the LWP server

4.	CoAP supports the interactions over LWP-1, LWP-2 and LWP-3 reference points

5.	The usage of CoAP by the SEAL service enablers shall follow the rules set out in clause 6.4.3.5.

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-01 | SA6#28 |  |  |  |  | TS skeleton | 0.0.0 |
| 2019-01 | SA6#28 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-190283, S6-190284, S6-190285, S6-190301, S6-190210, S6-190286, S6-190272, S6-190287, S6-190295, S6-190215, S6-190296, S6-190297 | 0.1.0 |
| 2019-03 | SA6#29 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-190446, S6-190447, S6-190448, S6-190509, S6-190526, S6-190515, S6-190452, S6-190453, S6-190510, S6-190511, S6-190456, S6-190457, S6-190458 | 0.2.0 |
| 2019-03 | SA#83 | SP-190063 |  |  |  | Presentation for information at SA#83 | 1.0.0 |
| 2019-04 | SA6#30 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-190661, S6-190663, S6-190848, S6-190746, S6-190747, S6-190748, S6-190749, S6-190750, S6-190872 | 1.1.0 |
| 2019-05 | SA6#31 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-191003, S6-191115, S6-191005, S6-191116, S6-191117, S6-191007, S6-191189, S6-191212, S6-191121, S6-191012, S6-191229, S6-191191, S6-191124, S6-191013, S6-191192, S6-191193 | 1.2.0 |
| 2019-05 | SA#84 | SP-190473 |  |  |  | Presentation for Approval at SA#84 | 2.0.0 |
| 2019-06 | SA#84 | SP-190473 |  |  |  | MCC Editorial update for publication after TSG SA approval (SA#84) | 16.0.0 |
| 2019-09 | SA#85 | SP-190733 | 0001 | 2 | F | Architecture requirements group management | 16.1.0 |
| 2019-09 | SA#85 | SP-190733 | 0002 | 3 | F | Group announcement and join | 16.1.0 |
| 2019-09 | SA#85 | SP-190733 | 0003 | 6 | F | Corrections to network resource management procedures | 16.1.0 |
| 2019-09 | SA#85 | SP-190733 | 0004 |  | F | N5 reference point description | 16.1.0 |
| 2019-09 | SA#85 | SP-190733 | 0006 | 1 | F | Change of service-based interface representation of the functional model for SEAL | 16.1.0 |
| 2019-09 | SA#85 | SP-190733 | 0007 |  | F | Remove EN on bearer type identification | 16.1.0 |
| 2019-09 | SA#85 | SP-190733 | 0008 |  | F | Remove EN on granularity of decision of NRM server | 16.1.0 |
| 2019-12 | SA#86 | SP-191111 | 0009 | 1 | F | Corrections to naming and other fixes | 16.2.0 |
| 2019-12 | SA#86 | SP-191111 | 0010 | 2 | F | Result element missing | 16.2.0 |
| 2019-12 | SA#86 | SP-191111 | 0011 | 1 | F | Anonymous requests | 16.2.0 |
| 2019-12 | SA#86 | SP-191111 | 0012 | 1 | F | No multicast resource management in 5GS | 16.2.0 |
| 2019-12 | SA#86 | SP-191111 | 0013 | 2 | F | Mention of SA3 responsibility in a published TS is not relevant. | 16.2.0 |
| 2019-12 | SA#86 | SP-191111 | 0014 | 2 | F | SEAL APIs corrections | 16.2.0 |
| 2019-12 | SA#86 | SP-191111 | 0015 | 1 | F | Update to location configuration procedure | 16.2.0 |
| 2020-03 | SA#87-E | SP-200114 | 0016 |  | F | Complete SS_NetworkResourceAdaptation API | 16.3.0 |
| 2020-03 | SA#87-E | SP-200114 | 0017 |  | F | Correct dynamic MBMS bearer establishment | 16.3.0 |
| 2020-03 | SA#87-E | SP-200114 | 0019 |  | F | MBMS procedures alignment | 16.3.0 |
| 2020-07 | SA#88-E | SP-200338 | 0021 | 1 | F | Align the Group Management API operation name with CT3 | 16.4.0 |
| 2020-07 | SA#88-E | SP-200338 | 0022 | 2 | F | Clarification and correction on media direction mode | 16.4.0 |
| 2020-09 | SA#89-E | SP-200841 | 0023 | 2 | F | Correct location trigger configuration | 16.5.0 |
| 2020-09 | SA#89-E | SP-200841 | 0024 | 1 | F | Clarifications on MBMS listening status uage | 16.5.0 |
| 2020-09 | SA#89-E | SP-200841 | 0025 | 2 | F | Correct SEAL location API operations | 16.5.0 |
| 2020-09 | SA#89-E | SP-200841 | 0026 | 1 | F | Correction to NRM unicast procedures | 16.5.0 |
| 2020-12 | SA#90-E | SP-200991 | 0029 | 1 | F | Clarification on group join notification | 16.6.0 |
| 2020-12 | SA#90-E | SP-200991 | 0030 | 1 | F | Resolution of ENs on security aspects | 16.6.0 |
| 2020-12 | SA#90-E | SP-200991 | 0033 | 1 | F | Clarifications for T8 interface | 16.6.0 |
| 2020-12 | SA#90-E | SP-200991 | 0036 | 1 | F | Correction to location notification to VAL server | 16.6.0 |
| 2020-12 | SA#90-E | SP-200998 | 0027 | 3 | B | Tracking UE and obtaining dynamic UE information | 17.0.0 |
| 2020-12 | SA#90-E | SP-200998 | 0031 | 2 | B | Support local MBMS | 17.0.0 |
| 2020-12 | SA#90-E | SP-200999 | 0032 | 2 | B | Network slice adaptation for VAL applications | 17.0.0 |
| 2020-12 | SA#90-E | SP-200999 | 0034 | 1 | F | Clarifications for T8 interface | 17.0.0 |
| 2020-12 | SA#90-E | SP-200999 | 0035 | 1 | B | Temporary Groups formation | 17.0.0 |
| 2021-04 | SA#91-E | SP-210181 | 0038 | 3 | B | Service identification in location management procedures | 17.1.0 |
| 2021-04 | SA#91-E | SP-210181 | 0039 | 2 | B | Group management support for 5G-VN groups | 17.1.0 |
| 2021-04 | SA#91-E | SP-210181 | 0040 | 2 | B | Add location criteria to group creation request | 17.1.0 |
| 2021-04 | SA#91-E | SP-210181 | 0042 | 1 | B | Update to LMS server APIs | 17.1.0 |
| 2021-04 | SA#91-E | SP-210181 | 0044 |  | B | Enhancement of information flows to add VAL service specific information | 17.1.0 |
| 2021-04 | SA#91-E | SP-210181 | 0045 | 1 | B | Service identification in location management procedures | 17.1.0 |
| 2021-06 | SA#92-E | SP-210484 | 0037 | 3 | B | Network Slice Capability Management functional model | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0043 | 3 | B | Off-network location management | 17.2.0 |
| 2021-06 | SA#92-E | SP-210485 | 0047 | 1 | B | Location report timestamp support | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0049 | 2 | F | Update to GMS APIs | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0051 | 2 | B | SEAL support for CoAP | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0052 | 1 | B | Resolve EN for group management | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0053 | 1 | B | Coordinated QoS/resource management for network-assisted UE-to-UE communications | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0054 | 1 | B | Group list fetch procedure | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0055 | 1 | B | Group management enhancements | 17.2.0 |
| 2021-06 | SA#92-E | SP-210485 | 0056 | 4 | B | SEAL Location Deviation Service | 17.2.0 |
| 2021-06 | SA#92-E | SP-210485 | 0057 | 1 | B | SEAL Event Monitoring Service | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0058 | 1 | B | Requirements for Location management service | 17.2.0 |
| 2021-06 | SA#92-E | SP-210485 | 0059 |  | B | Supplementary location information to verticals | 17.2.0 |
| 2021-06 | SA#92-E | SP-210485 | 0060 | 2 | B | add VAL UE Information to configuration management procedure | 17.2.0 |
| 2021-06 | SA#92-E | SP-210483 | 0062 | 1 | A | Complete resource reservation with PCC procedure | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0063 | 1 | F | QoS Monitoring support | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0064 | 2 | B | Unified support for TSC/TSN services | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0065 | 2 | B | SEAL enable 5G CN capabilities for SEAL groups | 17.2.0 |
| 2021-06 | SA#92-E | SP-210485 | 0066 | 2 | B | SEAL Location Deviation Service Information flows and APIs | 17.2.0 |
| 2021-06 | SA#92-E | SP-210484 | 0067 | 1 | B | API and information flow description for Temporary groups | 17.2.0 |
| 2021-06 | SA#92-E | SP-210483 | 0069 | 1 | A | Fixing the descriptions of IEs in Information flows for location information | 17.2.0 |
| 2021-09 | na | na | 0043 | 3 | B | Editorial re-application of CR0043r3 "Off-network location management" | 17.3.0 |
| 2021-09 | SA#93-E | SP-210964 | 0075 | 2 | C | Improved Event Monitoring Service | 17.3.0 |
| 2021-09 | SA#93-E | SP-210964 | 0076 | 2 | B | Utilize NEF location service for SEAL LM | 17.3.0 |
| 2021-09 | SA#93-E | SP-210964 | 0078 | 5 | B | Updates to Location based Group | 17.3.0 |
| 2021-09 | SA#93-E | SP-210964 | 0079 | 2 | F | Support for TSC services procedures | 17.3.0 |
| 2021-09 | SA#93-E | SP-210964 | 0080 | 1 | F | Support for TSN services procedures | 17.3.0 |
| 2021-09 | SA#93-E | SP-210964 | 0081 | 1 | B | Unicast QoS monitoring data retrieval | 17.3.0 |
| 2021-12 | SA#94-E | SP-211525 | 0082 | 1 | F | Corrections to network slice adaptation | 17.4.0 |
| 2021-12 | SA#94-E | SP-211521 | 0083 | 1 | F | Group Deletion procedure | 17.4.0 |
| 2021-12 | SA#94-E | SP-211525 | 0084 | 1 | F | Replace the NSCM with NSCE to align the terminologies | 17.4.0 |
| 2021-12 | SA#94-E | SP-211525 | 0086 | 1 | F | Removal of PCP from TSC stream discovery | 17.4.0 |
| 2021-12 | SA#94-E | SP-211525 | 0087 |  | F | Add missing location area monitoring API | 17.4.0 |
| 2022-03 | SA#95-E | SP-220101 | 0087 |  | F | Clarify the VAL UE ID | 17.5.0 |
| 2022-03 | SA#95-E | SP-220101 | 0092 | 1 | F | Correct QoS monitoring service | 17.5.0 |
| 2022-03 | SA#95-E | SP-220101 | 0093 |  | F | Correct TSC stream availability discovery | 17.5.0 |
| 2022-03 | SA#95-E | SP-220102 | 0091 | 1 | B | Complete location retrieval in an area | 18.0.0 |
| 2022-06 | SA#96 | SP-220473 | 0097 | 2 | A | Minor essential corrections to TS 23.434 | 18.1.0 |
| 2022-06 | SA#96 | SP-220473 | 0101 |  | A | Minor corrections on network resource management for 5G TSC | 18.1.0 |
| 2022-06 | SA#96 | SP-220473 | 0103 | 2 | A | QoS monitoring clarification | 18.1.0 |
| 2022-09 | SA#97-E | SP-220925 | 0104 | 3 | B | SEAL Notification Management service – Functional Model | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0105 | 3 | B | SEAL Notification Management service – Information Flows and Procedures | 18.2.0 |
| 2022-09 | SA#97-E | SP-220920 | 0108 | 1 | A | EN resolution for network slice adaptation request | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0110 | 1 | B | Establishing communication with service requirements | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0111 | 2 | B | Information flows and procedures to maintain notification channel | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0112 | 1 | B | Update_Unicast_QoS_Monitoring_Subscription operation in the SS_NetworkResourceMonitoring API | 18.2.0 |
| 2022-09 | SA#97-E | SP-220920 | 0115 | 2 | A | Correction on location management information flow | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0116 |  | B | Update the scope and reference to support 5MBS | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0117 | 2 | B | Update the requirement to support 5MBS | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0118 | 1 | B | Update the NRM functional model to support 5MBS | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0119 | 1 | B | MBS session creation and MBS session announcement | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0120 | 2 | B | Updating MBS resources for group communications | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0121 | 1 | B | MBS session deletion | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0122 |  | B | Activate or de-activate multicast MBS sessions | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0123 |  | B | Group media transmissions over 5G MBS sessions | 18.2.0 |
| 2022-09 | SA#97-E | SP-220925 | 0124 |  | B | Application level control signalling over 5G MBS sessions | 18.2.0 |