| 3GPP TS 23.435 V0.2.0 (2022-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Procedures for Network Slice Capability Exposure for Application Layer Enablement Service (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview	8

4.1	Slice API configuration and translation	8

4.x	<Functionality>	8

5	Business models and relationships for NSCALE	8

6	Architectural requirements	8

6.1	General Description	8

6.2	General requirements	8

6.3	Network sliceLifecycle management requirements	9

6.x	<Architectural requirements>	9

7	Application architecture for NSCALE	9

8	Identities and commonly used values	9

8.1	General	9

8.2	NSCE server ID	9

8.3	NSCE client ID	9

8.4	Slice coverage area	9

8.5	NSCE service area	9

9	Procedures and information flows	10

9.1	General	10

9.2	VAL server registration	10

9.2.1	General	10

9.2.2	Procedure	10

9.2.2.1	VAL server registration	10

9.2.2.2	VAL server de-registration	10

9.2.3	Information flows	10

9.2.3.1	General	10

9.2.3.2	VAL server registration request	10

9.2.3.3	VAL server registration response	11

9.2.3.4	Network slice capability de-registration request	11

9.2.3.5	Network slice capability de-registration response	12

9.2.4	APIs	12

9.2.4.1	General	12

9.2.4.2	Ensce_VALRegistration_Request operation	12

9.2.4.3	Ensce_VALRegistration_Deregister operation	12

9.3	Slice API configuration and translation procedures	13

9.3.1	General	13

9.3.2	Procedure on slice API configuration	13

9.3.2.1	Initial Configuration	13

9.3.2.2	VAL server-initiated Configuration Update	14

9.3.3	Procedure on slice API translation	14

9.3.4	Information flows	15

9.3.4.1	General	15

9.3.4.2	VAL application requirement request	15

9.3.4.3	VAL application requirement response	16

9.3.4.4	Slice API information notify	16

9.3.4.5	VAL server-initiated trigger event	16

9.3.4.6	slice API invocation request	17

9.3.4.7	slice API invocation response	17

9.3.5	APIs	17

9.3.5.1	SS_VAL_Application_requirement API	18

9.3.5.1.1	General	18

9.3.5.1.2	VAL_Application_requirement	18

9.3.5.2	SS_Slice_API_notification API	18

9.3.5.1.1	General	18

9.3.5.1.2	Slice_API_information_notify	18

9.3.5.1	SS_Slice_API_invocation API	18

9.3.5.1.1	General	18

9.3.5.1.2	Slice_API_invocation	18

9.4	Automatic application layer network slice management	19

9.4.1	General	19

9.4.2	Procedure	19

9.4.3	Information flows	20

9.4.3.1	General	20

9.4.3.2	Automatic application layer network slice lifecycle management subscribe request	20

9.4.3.3	Automatic application layer network slice lifecycle management response	21

9.4.3.4	Automatic application layer network slice lifecycle management notify	21

9.4.3.5	QoE metrics subscribe	21

9.4.3.6	QoE metrics response	22

9.4.3.7	QoE metrics notification	22

9.4.3.8	Network slice LCM recommendation request	22

9.4.3.9	Network slice LCM recommendation response	23

9.4.4	APIs	23

9.4.4.1	General	23

9.4.4.2	Ensce_AutoNSLCM_Subscribe operation	23

9.4.4.3	Ensce_AutoNSLCM_Notify operation	23

9.4.4.4	Eval_QoEMetrics_Subscribe operation	24

9.4.4.5	Eval_QoEMetrics_Notify operation	24

9.4.4.6	Eval_NSLCMRecommendation _Request operation	24

9.x	<procedure name>	24

9.x.1	General	24

9.x.2	Procedure	24

9.x.3	Information flows	24

9.x.4	APIs (if applicable)	24

Annex A (informative): Deployment models	25

Annex B (informative): Change history	26

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

Network slice capability enablement(NSCE) is a service that enabling the network slice related capabilities towards 3rd party on the basis of R17 SEAL. The NSCE service providing additional functionality and expose slice capabilities based on SA5 MnS services and SA2 service (e.g. NEF APIs, NWDAF APIs, NSACF APIs). This technical specification provides procedures for enabling NSCE service over 3GPP networks while the architecture is defined in TS 23.434[x].

1	Scope

The present document specifies the procedures and information flows necessary for Network Slice Capability Exposure for Application Layer Enablement on the basis of R17 SEAL.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

…

3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	Overview

Clauses under this clause shall document the functionalities of the Network Slice Capability Exposure for Application Layer Enablement.

4.1	Slice API configuration and translation

This functionality is provided to the vertical application specific layer and configures the exposure of APIs in a slice-tailored manner. It is assumed that the VAL server is not initially aware of the all the API exposure capabilities and information which will be needed for the given slice based on the SLA, and NSCE plays a vital role in configuring and translating the slice API based on the per slice requirements to service APIs.

4.x	<Functionality>

Add a copy of this clause for each  functionality, adding a title and description.

5	Business models and relationships for NSCALE

Clauses under this clause shall document the business relationships for NSCALE.

6	Architectural requirements

Clauses under this clause shall document the architectural requirements for the architecture.

6.1	General Description

The following clauses specify the requirements for network slice capability enablement services for application layers.

6.2	General requirements

[AR-6.2-1] The NSCE architecture shall support the NSCE server to communicate with the VAL server with one or more applications.

[AR-6.2-2] The NSCE architecture shall support the NSCE server to communicate with one or more VAL servers.

[AR-6.2-3] The application enablement layer shall support the interactions with 3GPP network management system to consume network slice management service provided by MNO.

[AR-6.2-4] The APIs interactions between the vertical application server(s) and NSCE server(s) shall conform to CAPIF as specified in 3GPP TS 23.222 [3].

6.3	Network slice Lifecycle management requirements

[AR-6.3-1] The NSCE server shall enable lifecycle management capability exposure of network slice by authorized VAL server.

[AR-6.3-2] The NSCE architecture shall enable lifecycle management capability exposure of network slice communication service by authorized VAL server.

[AR-6.3-3] The NSCE server shall support the interactions with 3GPP network management system to consume network slice management service provided by MNO.

[AR-6.3-4] The NSCE server shall support the end-to-end network slice performance and analytics monitoring capability exposure to authorized VAL server.

6.x	<Architectural requirements>

Add a copy of this clause for each architectural requirements, adding a title and description.

7	Application architecture for NSCALE

Add a reference to TS 23.434.

8	Identities and commonly used values

8.1	General

The common identities for SEAL refer to TS 23.434[2]. The following clauses list the additional identities and commonly used values for Network Slice Capability Enablement.

8.2	NSCE server ID

The NSCE server ID uniquely identifies the Network Slice Capability enablement server.

8.3	NSCE client ID

The Network Slice Capability enablement client ID identifies a particular NSCE client.

8.4	Slice coverage area

The slice coverage area is the area where the network slice is available in the whole PLMN or in one or more Tracking Areas of the PLMN. The slice coverage area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

8.5	NSCE service area

The NSCE service area is the area where the Network Slice Capability Enablement server owner provides its services. It is equal to the collection of coverage area of slices it can enable.

The NSCE service area can be expressed as a Topological Service Area (e.g. a list of TA), a Geographical Service Area (e.g. geographical coordinates) or both.

NOTE:	The NSCE server service area shall not smaller than the collection of slice(s) coverage area(s) the NSCE server can enable.

9	Procedures and information flows

Subclause of this clause shall describe the procedures for the application architecture for enabling Edge Application.

9.1	General

9.2	VAL server registration

9.2.1	General

The NSCE layer supports the exposure of network slice related capabilities to the VAL. To access network slice related enablement capabilities, e.g. monitoring of network slice performance, the VAL server should register on the NSCE layer.

9.2.2	Procedure

9.2.2.1	VAL server registration

For registration of the VAL server to be a recognized user of the NSCE server, the VAL server triggers registration procedure which is defined as onboarding the API invoker in TS 23.222[3] clause 8.1.3. In such procedure, the NSCE server could play the role of CAPIF core function.

9.2.2.2	VAL server de-registration

For de-registration of the VAL server, the VAL server triggers de-registration procedure which is defined as offboarding the API invoker in TS 23.222[3] clause 8.2.3.

9.2.3	Information flows

9.2.3.1	General

The following information flows are specified for service provisioning:

-	VAL server registration request and response; and

-	VAL server de-registration request and response.

9.2.3.2	VAL server registration request

Table 9.2.3.2-1 describes information elements for the VAL server registration request from the VAL server to the NSCE server.

Table 9.2.3.2-1: VAL server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration information | M | The information of the VAL server |
| >VAL server ID | M | The identifier of the VAL server |
| >VAL Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the VAL. |
| >Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| >Proposed expiration time | O | Proposed expiration time for the registration |
| APIs for enrollment | O | Indicates interested NSCE services. |

9.2.3.3	VAL server registration response

The information elements specified in the Table 9.2.3.3-1-1 is used for the VAL server registration response sent from the NSCE server to the VAL server.

Table 9.2.3.3-1: VAL server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Registration status | M | The result of Registration request |
| >Successful response (NOTE1) | O | Indicates that the VAL server registration request was successful. |
| > >Registration ID | M | Identifier of the registration. |
| > >Expiration time | O | Indicates the expiration time of the registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| >Failure response (NOTE1) | O | Indicates that the VAL server registration request failed. |
| Registration information | O (NOTE 2) | Information allow the VAL server to be authenticated and to obtain authorization for service APIs |
| Service API information | O (NOTE 3) | NSCE service API name, service API type, communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, and data format. |
| Reason | O (NOTE 4) | Indicates the cause of VAL server registration request failure |
| NOTE 1: One of these IEs shall be present in the message. NOTE 2: Information element shall be present when registration status is successful. NOTE 3: Information element may be present when registration status is successful. NOTE 4: Information element shall be present when registration status is failure. |  |  |

9.2.3.4	Network slice capability de-registration request

Table 9.2.3.4-1 describes information elements for the network slice capability de-registration request from the VAL server to the NSCE server.

Table 9.2.3.4-1: Network slice capability de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the registration. |
| Security credentials | M | Security credentials of the EAS |
| Reason | O | Indicate the reason of de-registration |

9.2.3.5	Network slice capability de-registration response

The information elements specified in the Table 9.2.3.5-1-1 is used for the Network slice capability de-registration response sent from the NSCE server to the VAL server.

Table 9.2.3.5-1: Network slice capability de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Network slice capability de-registration request was successful. |
| Failure response (NOTE) | O | Indicates that the Network slice capability de-registration request failed. |
| > Cause | O | Indicates the cause of Network slice capability de-registration request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

9.2.4	APIs

9.2.4.1	General

Table 9.2.4.1-1 illustrates the API for VAL server registration.

Table 9.2.4.1-1: Ensce_VALRegistration API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_VALRegistration | Request | Request/Response | VAL |
|  | Deregister |  |  |

9.2.4.2	Ensce_VALRegistration_Request operation

API operation name: Ensce_VALRegistration_Request

Description: The consumer requests to register the VAL server on the NSCE server.

Inputs: See clause 9.2.3.2.

Outputs: See clause 9.2.3.3.

See clause 9.2.2.1 for details of usage of this operation.

9.2.4.3	Ensce_VALRegistration_Deregister operation

API operation name: Ensce_VALRegistration_Deregister

Description: The consumer requests to deregister the VAL server from the NSCE server.

Inputs: See clause 9.2.3.4.

Outputs: See clause 9.2.3.5.

See clause 9.2.2.2 for details of usage of this operation.

9.3	Slice API configuration and translation procedures

9.3.1	General

This functionality is a service related to the translation of the service API as invoked by the end applications to slice APIs based on the API configuration and application to slice mapping. Slice APIs can be defined as customized/tailored sets of service APIs (which can be either NEF northbound APIs or OAM provided APIs or enabler layer/SEAL provided APIs) and can be mapped to particular slice instances. The slice APIs can be a bundled or combined API comprising of different types of APIs, which will be used to expose the telco (5GS/SEAL)-provided services as needed by the applications of the slice customer. Each slice API may be configured per network slice instance.

9.3.2	Procedure on slice API configuration

In this procedure, the VAL server initially provides an application requirement to enabler server including the service KPIs and the subscribed/preferred slices. Then, the slice enabler configures the mapping of the VAL application to a slice API which is a combination/bundling of northbound APIs (from both management and control plane). In particular, a slice API consists of telco-provided/platform dependent service APIs (e.g., NEF, OAM, SEAL, etc), and provides an abstraction/simplification on top of them. The procedure also covers the scenario where a trigger event occurs (e.g., QoS degradation, slice load) and the mapping configuration or the slice API configuration needs to change. In this scenario, the slice enabler updates the configuration of the API and provides a notification to the VAL server.

These two procedures for the initial configuration and the configuration update are covered in 9.3.2.1 and 9.3.2.2 accordingly.

9.3.2.1	Initial Configuration

Figure 9.3.2.1-1 illustrates the flow for the initial slice API configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability enablement services.

Figure 9.3.2.1-1: Initial Slice API configuration

1.	The VAL server sends a VAL application requirement request to the network slice capability enablement server.

2.	The network slice capability enablement server maps the VAL application requirement to a slice API which includes a list of APIs which are needed to be consumed as part of this service capability exposure.

The network slice capability enablement server may also store the mapping of the slice API to the service API list and per service API information (e.g. data encoding, transport technology, API protocol and versions)

3.	The network slice capability enablement server registers to consume the corresponding APIs from the 5GS (NEF and OAM) and SEAL service producers. The network slice capability enablement server registers to the following:

- to consume NEF monitoring events as specified in TS 29.522 clause 5 e.g., related to network monitoring, slice status, analytics exposure,

- to consume PM services and KPI monitoring from OAM

- to consume SEAL services based on TS 23.434

4.	The network slice capability enablement server sends a VAL application requirement response to notify on the result of the request and indicate whether configuration of the slice API can be possible or not.

5.	The network slice capability enablement server sends the slice API information notify to the VAL server.

9.3.2.2	VAL server-initiated Configuration Update

Figure 9.3.2.2-1 illustrates the flow for the slice API configuration updated based on a trigger event.

Figure 9.3.2.2-1: Slice API configuration update

1.	A trigger event is captured by the VAL server side (application server relocation to different EDN/DN, UE mobility to different EDN, application change of behaviour).

2.	The network slice capability enablement server processes the trigger event and checks the feasibility of such change and updates the mapping of service APIs to the slice APIs. One criterion for the update of the mapping is, if possible, to avoid changing the slice API configuration, which can be achieved by the re-mapping of the underlying service APIs

3.	The network slice capability enablement server updates the subscription/registration to the underlying 5GS and SEAL service producers, if an update on the service APIs (e.g., NEF APIs, SEAL APIs, OAM provided APIs) is needed.

4.	The network slice capability enablement server optionally sends a slice API information update notification to the VAL server.

9.3.3	Procedure on slice API translation

This procedure follows the 9.3.2 and aims to describe how the slice API invocation request is translated to service API invocations after the slice API configuration mapping. In this procedure, the network slice capability enablement server initially receives a slice API invocation request from the vertical application. Then, the network slice capability enablement server based on the slice API request fetches the service APIs to be invoked based on the slice API configuration and performs invocation requests to the corresponding service API providers.

Figure 9.3.3-1 illustrates the flow for the slice API translation based on the initial configuration.

Pre-conditions:

1.	The VAL server has registered to receive network slice capability enablement services.

2.	The slice API mapping to the VAL server has been performed based on 9.3.2.1 step 2 and the slice API information is provided to the VAL server based on 9.3.2.1 step 5.

Figure 9.3.3-1: Slice API translation

1.	The VAL server sends a slice API invocation request to the network slice capability enablement server

2.	The network slice capability enablement server checks that the user is authenticated and authorized to perform the slice API invocation and maps the requested slice API to a service APIs. If CAPIF is used, the network slice capability enablement server acts as AEF, and the authorization is obtained by CCF.

3.	The network slice capability enablement server generates a trigger for service API invocation requests to all the service APIs within the slice API.

4.	The network slice capability enablement server performs the corresponding service API invocation procedures based on CAPIF or via performing requests to the corresponding service API providers, which are mapped to the slice API.	If CAPIF is used, the requests are sent to the corresponding AEFs of the API provider's domain, and the authorization is obtained by CCF.

5.	The network slice capability enablement server sends to the VAL server a slice API invocation response, based on the result of the service API invocation response(s) of step 4.

9.3.4	Information flows

9.3.4.1	General

The following information flows are specified for slice API translation and configuration based on 9.3.2 and 9.3.3.

9.3.4.2	VAL application requirement request

Table 9.3.4.2-1 describes information elements for the VAL application requirement request from the VAL server to the NSCE server.

This request provides the service requirements / KPIs, the capability exposure requirements and a preferred/subscribed slice identification (e.g., S-NSSAI or ENSI).

Table 9.3.4.2-1: VAL application requirement request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Service requirements | M | The VAL application requirements |
| > VAL server KPIs | O | KPIs including application QoS requirements (latency, error rates, throughput, jitter,..) |
| > Slice ID (NOTE) | M | S-NSSAI or ENSI for which the request applies |
| > NSI ID (NOTE) | M | If the request applies for a certain NSI, the identifier of the NSI shall be present |
| > Service Profile | O | If known by the VAL server, the Service Profile can be provided to the NSCE server. |
| Area of Interest | O | The geographical or service area for which the requirement request applies |
| Time validity | O | The time validity of the request |
| NOTE: One of this shall be present |  |  |

9.3.4.3	VAL application requirement response

Table 9.3.4.3-1 describes information elements for the VAL application requirement response from the NSCE server to the VAL server.

Table 9.3.4.3-1: VAL application requirement response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the application requirements request. |

9.3.4.4	Slice API information notify

Table 9.3.4.4-1 describes information elements for the Slice API information notify from the NSCE server to the VAL server.

Table 9.3.4.4-1: Slice API information notify

| Information element | Status | Description |
| --- | --- | --- |
| Slice API info | M | The information for the configured slice API |
| > API details | M | The slice API name, type and identifier |
| > API version | O | The version of the slice API |
| > Communication method | O | The communication type (e.g. request/response, subscribe/notify) |
| List of service API information | O | The service API information which are mapped to the slice API. |

9.3.4.5	VAL server-initiated trigger event

Table 9.3.4.5-1 describes information elements for the Trigger Event from the VAL server to the NSCE server.

Table 9.3.4.5-1: VAL server-initiated trigger event

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server |
| Trigger Event Details | M | The trigger event details can be included, providing the cause for the need slice API configuration adaptation. Such trigger event can be: - the UE mobility to a different service area (outside the original area of interest), - application server migration to different edge/cloud platform, - service API unavailability, - VAL application QoS requirements change |

9.3.4.6	slice API invocation request

Table 9.3.4.6-1 describes information elements for the slice API invocation request from the VAL server to the NSCE server.

Table 9.3.4.6-1: slice API invocation request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Authorization information | O | The authorization information obtained before initiating the slice API invocation request |
| Slice API identification | M | The identification information of the slice API for which invocation is requested. The slice API identification is part of the specific slice API invocation request. |

9.3.4.7	slice API invocation response

Table 9.3.4.7-1 describes information elements for the slice API invocation response from the NSCE server to the VAL server.

Table 9.3.4.7-1: slice API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of slice API invocation. |

9.3.5	APIs

Table 9.3.5-1 illustrates the NSCALE APIs for the slice API configuration and translation feature.

Table 9.3.5-1: List of APIs for slice API configuration and translation feature

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_VAL_Application_requirement | VAL_Application_requirement | VAL server | Request /Response |
| SS_Slice_API_notification | Slice_API_information_notify | VAL server | Subscribe/Notify |
| SS_Slice_API_invocation | Slice_API_invocation | VAL server | Request /Response |

9.3.5.1	SS_VAL_Application_requirement API

9.3.5.1.1	General

API description: This API enables the VAL server to communicate with the network slice capability enablement server for providing the slice related application requirement over NSCE-S.

9.3.5.1.2	VAL_Application_requirement

API operation name: VAL_Application_requirement

Description: Providing for VAL_Application_requirement to the NSCE server.

Known Consumers: VAL server.

Inputs: See subclause 9.3.4.2

Outputs: See subclause 9.3.4.3

9.3.5.2	SS_Slice_API_notification API

9.3.5.1.1	General

API description: This API enables the network slice capability enablement server to communicate with the VAL server for notifying the slice API information over NSCE-S.

9.3.5.1.2	Slice_API_information_notify

API operation name: Slice_API_information_notify

Description: Notifying the slice API information

Known Consumers: VAL server.

Inputs: -

Outputs: See subclause 9.3.4.4

9.3.5.1	SS_Slice_API_invocation API

9.3.5.1.1	General

API description: This API enables the VAL server to communicate with the network slice capability enablement server for invoking the slice API over NSCE-S.

9.3.5.1.2	Slice_API_invocation

API operation name: SS_Slice_API_invocation

Description: Requesting slice API invocation from NSCE server.

Known Consumers: VAL server.

Inputs: See subclause 9.3.4.6

Outputs: See subclause 9.3.4.7

9.4	Automatic application layer network slice management

9.4.1	General

When NSCE receives a request for automatic application layer network slice management(Auto_NS_LCM) from VAL server, the NSCE server performs the service operations including subscribing the event which may trigger the automatic network slice lifecycle management, making the network slice lifecycle management recommendation/decision, triggering the network slice management operations, notifying the consumer about the network slice information.

9.4.2	Procedure

Figure 9.4.2-1 illustrates a procedure of automatic application layer network slice lifecycle management based on network slice related data and QoE collected from application layer.

Pre-conditions:

1.	The NSCE server has authenticated and authorized to the capabilities to collect current network slice from 5GS.

2.	The NSCE server has authenticated and authorized to the capabilities trigger the network slice LCM operations.

# Figure 9.4.2-1: Automatic application layer network slice lifecycle management

1.	The VAL server sends the automatic application layer network slice lifecycle management(Auto-NS-LCM) subscribe request to NSCE server,  with network slice requirements (e.g. delay, throughput, load, the maximum number of users supported, etc.). The request can indicate whether the notification is needed before performing the auto-NS-LCM. The request can also indicate the trigger conditions, such as by providing the monitored parameters and the corresponding thresholds as described in clause 9.4.3.2.

2.	After receiving the request, the NSCE server checks that the user is authenticated and authorized to perform the corresponding auto-NS-LCM operations, and filters the unauthorized requests, if any.

3.	Once authenticated and authorized, the NSCE server sends the auto-NS-LCM response to the VAL server.

4.	According to network slice requirements and/or the trigger conditions, NSCE server triggers the provision of network slice status and QoE metrics.

4a.	The network slice status could be collected through subscribing or requesting to 5GS. For example, to monitor the slice load, it could subscribe to/request the relevant service(s), such as AnalyticsExposure defined in TS 23.502 [4] clause 5.2.6.16, provisioning data report exposure for NSI in clause 5 of TS 28.532 [8]. If the trigger conditions are not indicated in the subscription, the NSCE server can help to configure an appropriate trigger condition, such as report period or thresholds.

4b.	Also, the NSCE could get the information of QoE metrics from the application layer domain by QoE metric subscribe as described in clause 9.4.3.5.

5.	Once the trigger condition or the combination of trigger conditions is met, based on requirements and network slice capabilities with updated network slice status and QoE metrics collected in step 4, such as changing in QoE, changing in network slice status etc., the NSCE determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi/AllocateNsi/DeallocateNsi request as specified in TS 28.531 [5].

6.	Optionally, if it is indicated in the request to notify the VAL server/consumer before performing the auto-NS-LCM, the NSCE sends the network slice LCM recommendation(s) with network slice information to VAL server, to see whether takes the recommendation(s) or not.

7.	Optionally, the VAL server sends the response to NSCE server.

8.	Based on decision made by VAL server or NSCE server, the NSCE server performs the corresponding operation(s).

9. According to the corresponding operation(s) result, the NSCE server sends the response to the VAL server.

9.4.3	Information flows

9.4.3.1	General

The following information flows are specified for Auto_NS_LCM:

-	Auto_NS_LCM subscribe, response and notification;

-	QoE metrics subscribe, response and notification;

-	Network slice LCM recommendation request and response;

9.4.3.2	Automatic application layer network slice lifecycle management subscribe request

Table 9.4.3.2-1 describes information elements for the Automatic application layer network slice lifecycle management subscribe request from the VAL server to the NSCE server.

Table 9.4.3.2-1: Auto-NS-LCM Subscribe

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Network slice requirements | M | The requirement of network requirements or updated requirements when the network slice was created. |
| List of S-NSSAI(s) | M | Identifier of the interested network slice |
| Auto-NS-LCM notification indication | M | Indicates whether to notify the VAL server/consumer about the auto-NS-LCM |
| >Auto-NS-LCM notification address (NOTE) | M | The address (e.g. URL) of the consumer that can receive the auto-NS-LCM notification |
| Trigger condition | O | Indicates the monitored parameters and the corresponding thresholds which could trigger the Auto-NS-LCM |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| NOTE: When the auto-NS-LCM notification indication is needed, this information element should be provided. |  |  |

9.4.3.3	Automatic application layer network slice lifecycle management response

The information elements specified in the Table 9.4.3.3-1 is used for the Automatic application layer network slice lifecycle management response sent from the NSCE server to the VAL server.

Table 9.4.3.3-1: Auto-NS-LCM Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Auto-NS-LCM request was successful. |
| Failure response (NOTE) | O | Indicates that the Auto-NS-LCM request failed. |
| > Cause | O | Indicates the cause of Auto-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

9.4.3.4	Automatic application layer network slice lifecycle management notify

The information elements specified in the Table 9.4.3.4-1 is used for the Automatic application layer network slice lifecycle management notify sent from the NSCE server to the VAL server.

Table 9.4.3.4-1: Auto-NS-LCM Notify

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the Auto-NS-LCM request was successful. |
| >network slice information | M | Network slice information (i.e. NEST) with network slice identifier(i.e. S-NSSAI) |
| Failure response (NOTE) | O | Indicates that the Auto-NS-LCM request failed. |
| > Cause | O | Indicates the cause of Auto-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

9.4.3.5	QoE metrics subscribe

Table 9.4.3.5-1 describes information elements for the QoE metrics subscribe from the NSCE server to the VAL server.

Table 9.4.3.5-1: QoE metrics Subscribe

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Event Filter | M | The associated filter on a network slice to be notified |
| >Application indicator | O | Indicator of the interested application (i.e. App ID) |
| >QoE type indicator | O | QoE metric type including latency, throughput, jitter, etc. |
| Event Reporting information | M | Information indicates whether the notification is threshold based or the notification is periodical and optionally the Immediate reporting flag |
| >Threshold | O | Threshold of QoE metrics |
| >Reporting period | O | Indicating the metrics reporting period |
| >Immediate reporting flag | O | Indicating the request needs immediate reporting or not |
| Proposed expiration time | O | Proposed expiration time for the subscribe |

9.4.3.6	QoE metrics response

Table 9.4.3.6-1 describes the information elements for the QoE metrics response from the VAL server to the NSCE server.

Table 9.4.3.6-1: QoE metrics Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the QoE metrics request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| >QoE metrics report | O | List of result values for the observed or computed QoE metrics value if the immediate reporting is needed. |
| Failure response (NOTE) | O | Indicates that the QoE metrics request failed. |
| > Cause | O | Indicates the cause of QoE metrics request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

9.4.3.7	QoE metrics notification

Table 9.4.3.7-1 describes the information elements for the QoE metrics notification from the VAL server to the NSCE server.

Table 9.4.3.7-1: QoE metrics Notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Indicates that the QoE metrics request was successful. |
| QoE metrics report | M | List of result values for the observed or computed QoE metrics value. |

9.4.3.8	Network slice LCM recommendation request

Table 9.4.3.8-1 describes information elements for the Network slice LCM recommendation request from the NSCE server to the VAL server.

Table 9.4.3.8-1: Network slice LCM recommendation Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Network slice LCM recommendation | M | Recommended network slice lifecycle management operation |
| >Network slice identifier(i.e. S-NSSAI) | M | Unique identifier of network slice (i.e. S-NSSAI) |
| >Recommend network slice LCM action | M | Recommend network slice LCM action(i.e. modifying the configuration, allocating a network slice) |
| >Network slice information | O | Network slice information if the action is taken(i.e. NEST) |

9.4.3.9	Network slice LCM recommendation response

Table 9.4.3.9-1 describes the information elements for the Network slice LCM recommendation response from the VAL server to the NSCE server.

Table 9.4.3.9-1: Network slice LCM recommendation Response

| Information element | Status | Description |
| --- | --- | --- |
| Recommendation accepted flag | M | Indicates that the Network slice LCM recommendation request was accepted or not. |

9.4.4	APIs

9.4.4.1	General

Table 9.4.4.1-1 illustrates the API for Automatic application layer network slice management.

Table 9.4.4.1-1: API for Automatic application layer network slice management

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_AutoNSLCM | Subscribe | Subscribe/Response | VAL server |
|  | Notify | Notify | VAL server |
| Eval_QoEMetrics | Subscribe | Subscribe/Response | NSCE |
|  | Notify | Notify | NSCE |
| Eval_NSLCMRecommendation | Subscribe | Subscribe/Response | NSCE |

9.4.4.2	Ensce_AutoNSLCM_Subscribe operation

API operation name: Ensce_AutoNSLCM_Request

Description: The consumer subscribes for Auto_NS_LCM.

Inputs: See clause 9.4.3.2.

Outputs: See clause 9.4.3.3.

See clause 9.4.2 for details of usage of this operation.

9.4.4.3	Ensce_AutoNSLCM_Notify operation

API operation name: Ensce_AutoNSLCM_Notify

Description: The consumer is notified with result of Auto_NS_LCM.

Inputs: See clause 9.4.3.4.

Outputs: None

See clause 9.4.2 for details of usage of this operation.

9.4.4.4	Eval_QoEMetrics_Subscribe operation

API operation name: Ensce_AutoNSLC_Request

Description: The consumer subscribes for the QoE metrics.

Inputs: See clause 9.4.3.5.

Outputs: See clause 9.4.3.6.

See clause 9.4.2 for details of usage of this operation.

9.4.4.5	Eval_QoEMetrics_Notify operation

API operation name: Eval_QoEMetrics_Notify

Description: The consumer is notified with the QoE metrics.

Inputs: See clause 9.4.3.7.

Outputs: None

See clause 9.4.2 for details of usage of this operation.

9.4.4.6	Eval_NSLCMRecommendation _Request operation

API operation name: Eval_NSLCMRecommendation _Request

Description: The consumer request for whether to take the recommendation of the Auto_NS_LCM.

Inputs: See clause 9.4.3.8.

Outputs: See clause 9.4.3.9.

See clause 9.4.2 for details of usage of this operation.

9.x	<procedure name>

Add a copy of this clause for each procedure, adding a title, a general description, information flows and the procedure. Include an APIs clause if APIs are needed for the procedure.

9.x.1	General

9.x.2	Procedure

9.x.3	Information flows

9.x.4	APIs (if applicable)

This clause may be skipped if the procedure under 10.x does not require specification of APIs.

Annex A (informative): Deployment models

Informative annexes may appear in both Technical Specifications and Technical Reports. Use style "Heading 8" for use in TSs.

Informative annexes shall not contain requirements for the implementation of the Technical Specification.

Annex B (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-06 | SA6#49-bis-e |  |  |  |  | TS skeleton | 0.0.0 |
| 2022-07 | SA6#49-bis-e |  |  |  |  | TS Skeleton agreed in SA6#44: S6-221640, Implemented pCRs approved in SA6#49-bis-e: S6-221641, S6-221642, S6-221643, S6-221822, S6-221818. | 0.1.0 |
| 2022-09 | SA6#50-e |  |  |  |  | Implemented pCRs approved in SA6#49-bis: S6-222146, S6-222197, S6-222407 Editorial changes by the rapporteur. | 0.2.0 |