| 3GPP TR 23.700-01 V0.3.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application enablement for satellite access enabled 5G services; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Key issues	8

4.1	Key issue #1: Usage of satellite access characteristics for the application enablement	8

4.1.1	Description	8

4.1.2	Open issues	8

4.2	Key issue #2: Edge computing on satellite	8

4.2.1	Description	8

4.2.2	Open issues	9

4.3	Key issue #3: Satellite access with discontinuous coverage	9

4.3.1	Description	9

4.3.2	Open issues	10

4.4	Key issue #4: Satellite access support for MC services	10

4.4.1	Description	10

4.4.2	Open issues	10

4.5	Key issue #5: Edge on board NGSO Satellite	11

4.5.1	Description	11

4.5.2	Open issues	11

4.6	Key issue #6: Support of UE-Satellite-UE communication for IMS services via Satellite access	11

4.6.1	Description	11

4.6.2	Open issues	12

4.7	Key issue #7: Usage of S&F events information for the application enablement	12

4.7.1	Description	12

4.7.2	Open issues	13

5	Architecture requirements	13

5.1	General architecture requirements	13

5.x	<Mission Critical service x> architecture requirements	13

5.y	<Application Enabler y> architecture requirements	13

6	Architecture for satellite access enabled 3GPP services and application enablers	14

6.x	Option#x: <Mission Critical service x>	14

6.x.1	Application architecture	14

6.x.2	Functional Elements	14

6.x.3	Reference Points	14

6.y	Option#x: <Application Enabler y>	14

6.y.1	Application enablement architecture	14

6.y.2	Functional Elements	14

6.y.3	Reference Points	14

7	Solutions	14

7.0	Mapping of solutions to key issues	14

7.1	Solutions for Mission Critical	15

7.1.x	Solution #x: <title>	15

7.1.x.1	Solution description	15

7.1.x.2	Architecture Impacts	15

7.1.x.4	Solution evaluation	15

7.2	Solutions for Application Enablers	15

7.2.1	Solution #1: Satellite edge computing	15

7.2.1.1	Solution description	15

7.2.1.1.1	General	15

7.2.1.1.2	Further consideration for service provisioning	16

7.2.1.1.3	EAS discovery for EAS on board satellite	17

7.2.1.1.4	Service provisioning  for EES on board satellite	17

7.2.1.2	Architecture Impacts	17

7.2.1.3	Corresponding APIs	17

7.2.1.3.1 	EAS Profile, EES Profile and AC Profile for discovery and service provisioning	17

7.2.1.4	Solution evaluation	18

7.2.2	Solution #2: Enhancement for on board EES(s) and service provisioning	19

7.2.2.1	Solution description	19

7.2.2.1.1	General	19

7.2.2.2	Architecture Impacts	21

7.2.2.3	Corresponding APIs	21

7.2.2.4	Solution evaluation	21

7.2.3	Solution #3: Support discontinuous coverage for satellite access	21

7.2.3.1	Solution description	21

7.2.3.1.1	Procedure of SEAL Server generating Satellite coverage availability and unavailability period information	22

7.2.3.2	Architecture Impacts	23

7.2.3.3	Corresponding APIs	23

7.2.3.4	Solution evaluation	23

8	Deployment scenarios	23

8.1	General	23

8.x	Deployment model #x: <Title>	24

9	Business Relationships	24

10	Overall evaluation	24

10.1	Architecture evaluations for Mission Critical	24

10.2	Key issue evaluations for Mission Critical	24

10.3	Architecture evaluations for Application Enablers	24

10.4	Key issue evaluations for Application Enablers	24

11	Conclusions	24

11.1	General conclusions	24

11.2	Conclusions for Mission Critical	24

11.2.x	Conclusions of key issue #x	24

11.3	Conclusions for Application Enablers	24

11.3.y	Conclusions of key issue #y	24

Annex A (informative): Change history	26

Annex B (informative):	Information related to Satellite access support for MC Service	26

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report capturing the study on application enablement for Satellite access enabled 5G Services over 3GPP networks. The aspects of the study include identifying architecture requirements, supporting architecture for satellite access enabled 3GPP services and application enablers (either by defining new functional model or by enhancing existing functional model), and corresponding solutions.

The study is based on the requirements as defined in 3GPP TS 22.261 [r22261] and related use cases (for e.g. in 3GPP TR 22.822 [r22822] and 3GPP TR 22.865 [r22865]). The study is dependent on 3GPP TS 23.501 [r23501] (5GC architecture for satellite access for NR), 3GPP TS 23.502 [r23502] (procedures for NR satellite access), 3GPP TS 23.503 [r23503] (PCF for Satellite backhaul), 3GPP TS 23.401 [r23401] (EPC architecture for satellite access for IoT), 3GPP TS 23.682 [r23682] (EPC architecture enhancements to facilitate communications with packet data networks and applications).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TR 22.822: "Study on using satellite access in 5G; Stage 1".

[4]	3GPP TR 22.865: "Study on satellite access - Phase 3".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[7]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[8]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[9]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications"

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Key issues

## 4.1	Key issue #1: Usage of satellite access characteristics for the application enablement

### 4.1.1	Description

3GPP SA1 has specified the service requirements for 5G for satellite access in TS 22.261[22.261] which including supporting 5G system with satellite access, suitable QoS parameters for traffic over a satellite backhaul, etc. And 3GPP SA2 has also specified the architecture and solutions in Rel-17 and Rel-18 to support these services requirements identified in SA1.

Service enablers defined by SA6 should also considering to take advantage of the output from SA1 and SA2 to ensure that application enablers support the satellite access, and should utilize the satellite access characteristics (e.g. QoS parameters for traffic over a satellite access) to optimise application layer enablement behaviour to provide a better service experience.

### 4.1.2	Open issues

To support the satellite access in application enablers, the following aspects need to be studied:

-	What are the satellite access characteristics that can be utilized by the application enablers?

-	Whether and how to use the satellite access characteristics (e.g., QoS parameters for traffic over a satellite access) to optimise the application enablement behaviour for the existing application enablers.

## 4.2	Key issue #2: Edge computing on satellite

### 4.2.1	Description

UPF and Edge in the backhaul part of a 5GS (i.e. on board a GEO satellite) facilitates reduction of latency and faster service provisioning to end users. TS 23.558 (SA6) defines Architectures for enabling Edge applications including the procedures for EAS discovery. Although TS 23.558 specification is created to cover the most generic EDGEAPP cases– it would be beneficial to study whether the placement of Edge on board satellite requires some enhancements of application enablers. The motivation for this is to explore whether EDGEAPP enhancements could reduce latency in cases when Edge has been placed on board GEO satellite. Besides, the interest is to investigate whether EDGEAPP enhancements could reduce data load in the feeder link (the link between 5GC on the ground and GEO satellites).

NOTE:	The focus of this Key Issue is limited to aspects of EDGEAPP in TS23.558.

### 4.2.2	Open issues

Deployment of Edge computing on board GEO satellites might impact the latency of application control and data information exchange. To assess the impacts of using Edge on board GEO satellites it would be benefitial to study the following aspects with the aim to reduce latency and data exchange over satellite feeder link:

1)	Investigate different deployment options for EDGEAPP when EASs are deployed on board GEO satellites?

2)	When/how could EAS discovery by the EEC in EDGEAPP be optimized?

3)	Whether and how the service continuity procedures are impacted while the UE is only connected with NTN?

## 4.3	Key issue #3: Satellite access with discontinuous coverage

### 4.3.1	Description

In a network with satellite access, a UE may experience a situation of discontinuous coverage, due to e.g., a sparse NGSO satellite constellation deployment. An illustration of a discontinuous coverage pattern for a given UE location and a single example LEO satellite at a nominal 600 km altitude is represented in Figure 4.x.2-1 (reproduced from R2-2107453). The figure shows the flyovers where the satellite passes above 30 degrees elevation as seen from the UE location. The rise time of the satellite is plotted on the x axis against the visibility duration on the y axis. The results do not consider link budget aspects, only geometrical visibility above the given elevation angle. In the provided example, the satellite is visible from the UE location twice on most days, and occasionally three times, and the interval between passes varies between ~9 to 13.3 hours, with a mean of 11.9 hours. The median duration of the visibility window lasts around 220 seconds (3.6 minutes), with 90% of flyovers longer than 110 seconds.

This sort of discontinuous coverage pattern between the UE and a satellite can be predicted ahead of time for periods of days or even a few weeks with good accuracy (e.g. time errors of a few tens of seconds in estimating the starting time of the satellite pass when considering prediction windows of up to a few tens of days) by means of orbit propagation models and satellite ephemeris information (see R2-2206115 for prediction estimation errors using TLE and SGP4 propagator).

Figure 4.3.1-1: Flyovers and duration of visibility (≥ 30 degrees elevation) for a single LEO-600 km satellite (source: R2-2107453)

From a network perspective, support to cope with the discontinuous coverage nature of the service link in sparse constellations has been specified in Rel-17 and Rel-18. This support includes optional enhancements within the radio access network (e.g., satellite ephemeris parameters are broadcast in SIBs for satellite pass predictions for IoT NTN access) and within the core network (e.g., mobility management and power saving optimization, coverage availability information provisioning, paging, overload control). Coverage availability information provisioning to the UE and to the MME/AF entities have been also been identified, though no specific protocol or format have been defined in the specifications. All these enhancements help the UE and the network to gracefully operate under a discontinuous coverage pattern.

However, regardless of these enhancements at the network level, the discontinuous coverage nature of the service link results into an intermittent connectivity pattern between the UE and the Application Server (AS)/Application Function (AF) that has also a direct impact on the behaviour of the application.

Supplying information on the discontinuous coverage pattern or related services to the application layer will help applications to design themselves for handling discontinuity accordingly. For instance, if the discontinuous /intermittent connectivity pattern can be predicted in advance and exposed to the application layer in the network side, the application layer could use this information to properly schedule data transfers between the UE and the AS/AF (e.g. some information flows can be given precedence during short connectivity intervals, bulky data exchanges can be deferred and planned during longer connectivity intervals, notifications on expected data delivery times can be provided, etc.).

Once the discontinuous coverage patterns provided by different satellite operators are retrieved by an application enabler on the network side, specific information exposure policies can be enforced. For example, a certain vertical application may be allowed to access such discontinuous coverage patterns in specific times of the day and under specific circumstances, for example when the UE is located in a certain geographical area.

It is therefore necessary to study if some support to handle discontinuous coverage at application layer should be introduced in existing or a new application enabler.

### 4.3.2	Open issues

To support the discontinuous satellite coverage in application enabler, the following aspects need to be studied:

-	Whether and how the information on discontinuous satellite coverage needs to be exposed to the application layer (e.g. semantics, service-based interfaces, etc.) on the AS/AF side.

-	Whether and how the application enabler enforces policies (e.g., based on the time of the day, the UE location, etc.) on the discontinuous coverage information exposure towards the vertical applications.

## 4.4	Key issue #4: Satellite access support for MC services

### 4.4.1	Description

To ensure reliable MC services with larger coverage area, MC systems can benefit from satellite access, especially when the connectivity provided by the terrestrial networks faces limitations, e.g., bad coverage at remote areas. Hence, MC service UEs can be able to connect to MC system via satellite access to ensure service continuity.

There are different satellite systems, based on altitude, roundtrip time and coverage such as; Low Earth Orbit (LEO), Medium Earth Orbit (MEO), and Geostationary Equatorial Orbit (GEO). Among these different systems, LEO offers low latency, and large area capacity density.

In this study, it is worth understanding the deployment scenarios to utilize satellite access for MC services, and understanding how the MC KPIs (e.g., latency requirement) can be met when MC service users are connecting to the MC system via satellite access.

Location reporting aspects for MC service clients is to be considered and understand whether location reports provided by MC service users over satellite access may or may not be different from MC service users over terrestrial access.

### 4.4.2	Open issues

There are several aspects that need to be considered and studied to support satellite access for MC services, including but not limited to:

1.	Identify the different deployment scenarios for MC services using satellite access to result in a better MC service user experience.

## 4.5	Key issue #5: Edge on board NGSO Satellite

### 4.5.1	Description

NGSO satellites are satellites moving with respect to the earth surface and they can be deployed in MEO (8,000 km – 20,000 km ) or LEO (400 km – 2000 km) orbits. This will reduce the one way latency. To utilize these reduced latencies UPFs as well as gNBs can be deployed on the NGSO.

### 4.5.2	Open issues

This key issue studies how to deploy EAS(s) and EES(s) on NGSO satellite and whether and how the corresponding discovery, service provisioning and service continuity are impacted.​

-	How the EES(s) are placed on board the Satellite.

-	How the EAS(s) are placed on board the Satellite.

-	Whether and how the discovery and the service provisioning are impacted.​

-	Whether and how the service continuity procedures are impacted while the UE is only connected with NTN.

-	Whether and how the EAS can be relocated while the UE can be connected with NTN/TN.

## 4.6	Key issue #6: Support of UE-Satellite-UE communication for IMS services via Satellite access

### 4.6.1	Description

It has been described in the TS 22.261[6] that a 5G system with satellite access shall support UE-Satellite-UE communication regardless of whether the feeder link is available or not, and 5G system also be able to support different types of UE-Satellite-UE communication (e.g. voice, messaging, broadband, unicast, multicast, broadcast).

According to TR 23.700-29[x], the UE-satellite-UE communication refers to the communication between UEs under the coverage of one or more serving satellites without the user plane traffic going through the ground network.

The serving satellites may be the different types (e.g. LEO, MEO, and GEO) and the feeder link may be direct from the serving satellite to a ground gateway or through ISL (Inter-Satellite Link) to another satellite with a link to a ground gateway, etc.

Figure 4.6.1-1 is the example of scenarios for UEs-satellite-UEs communications for IMS services over LEO constellations without ISL.

Figure 4.6.1-1: An example of UEs-SAT-UEs communication on LEO satellite without ISL

In this study, it is worth understanding the deployment scenarios to utilize satellite access and the procedure and function impacts on the application enabler for the IMS services.

### 4.6.2	Open issues

From the application enabler’s perspective, the key issue will focus on how to support the IMS service via satellite access in the application enabled layer, it is proposed to study the following items:

-	Investigate different deployment options for the application enabler of IMS (i.e. eMMTEL App) to utilize satellite access.

## 4.7	Key issue #7: Usage of S&F events information for the application enablement

### 4.7.1	Description

3GPP SA1 has specified the service requirements for 5G for satellite access in TS 22.261[22.261] as following which includes supporting Store and Forward (S&F) Satellite Operation. And 3GPP SA2 is studying the exposure of S&F events information to support these services requirements identified in SA1.

-	Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data retention period.

-	Subject to operator’s policies, a 5G system with satellite access supporting S&F Satellite operation shall be able to allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data storage quota.

-	A 5G system with satellite access shall be able to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g. estimated delivery time to the authorised UE).

SA2 is studying the following S&F events information:

-	Registration in S&F Mode

-	Feeder Link Available

-	Expected Delivery/Delay Time

Editor’s note:	S&F events information is dependent on SA2.

Services and application enablers defined by SA6 should also consider to take advantage of the output from SA1 and SA2 related to utilizing the S&F events information. When a UE registers to the network in S&F mode and is available to send/receive data on specific occasions only (e.g. when satellites fly-over and later connect to ground station), AF can leverage S&F events information. For e.g. if the UE is registered in S&F mode, the AF may want to adjust the frequency, size of data, message acknowledgement handling, ack timers etc. Another e.g. is AF can schedule delivery based on Feeder-Link availability etc.

Hence, it is desirable to study impacts to application enablers, while supporting S&F Satellite operation and S&F events information is shared to the AF.

### 4.7.2	Open issues

To support utilization of S&F events information by application enablers, the following aspects need to be studied:

-	What are the S&F events information that can be utilized by the application enablers?

-	How to use the S&F events information to optimise the application enablement behaviour?

-	Whether and how the S&F Satellite operation information can be exposed to the 3rd party/VAL server.

# 5	Architecture requirements

## 5.1	General architecture requirements

Editor's Note:	This clause will describe general architectural requirements.

## 5.x	<Mission Critical service x> architecture requirements

Editor's Note:	This clause will describe the architectural requirements for Mission Critical service.

## 5.y	<Application Enabler y> architecture requirements

Editor's Note:	This clause will describe the architectural requirements for Application Enablers.

# 6	Architecture for satellite access enabled 3GPP services and application enablers

## 6.x	Option#x: <Mission Critical service x>

### 6.x.1	Application architecture

Editor's Note:	This clause will illustrate the high level architecture and possible architecture enhancements for supporting Satellite access enabled Mission Critical services.

### 6.x.2	Functional Elements

Editor's Note:	The functional elements corresponding to the architecture will be presented in this clause.

### 6.x.3	Reference Points

Editor's Note:	The reference points corresponding to the architecture will be presented in this clause.

## 6.y	Option#x: <Application Enabler y>

### 6.y.1	Application enablement architecture

Editor's Note:	This clause will illustrate the high level architecture and possible architecture enhancements for supporting Satellite access enabled 3GPP Services.

### 6.y.2	Functional Elements

Editor’s Note:	The functional elements corresponding to the architecture will be presented in this clause.

### 6.y.3	Reference Points

Editor’s Note:	The reference points corresponding to the architecture will be presented in this clause.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Mission Critical Services |  |  |  |  |  |  |
| Sol #MC1 |  |  |  |  |  |  |
| Sol #MC2 |  |  |  |  |  |  |
| Application Enablers |  |  |  |  |  |  |
| Sol #AE1 |  | x |  |  | x |  |
| Sol #AE2 |  |  |  |  | x |  |
| Sol #AE3 |  |  | x |  |  |  |

## 7.1	Solutions for Mission Critical

### 7.1.x	Solution #x: <title>

Editor's Note:	Provide a suitable title for the solution.

#### 7.1.x.1	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

This solution maps to KI# a [and KI# b]. This solution …

#### 7.1.x.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts (if any) of the solution and possible new SA6 capabilities and interfaces.

#### 7.1.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.2	Solutions for Application Enablers

### 7.2.1	Solution #1: Satellite edge computing

#### 7.2.1.1	Solution description

Editor's Note:	This clause will describe the solution. Each solution should clearly describe which of the key issues it covers and how.

##### 7.2.1.1.1	General

Figure 7.2.1.1.1-1: satellite EDN deployment

In this deployment option, ECS is deployed on ground. The UE can be on the ground/sea or in the air (e.g. drone). The  UPF to access satellite EDN is deployed on satellite, EAS and EES are deployed in one or more satellites. RAN (e.g. gNodeB) can either be deployed on ground/sea (e.g. in a ship) and connected to satellite UPF or be deployed on regenerative satellite (see Annex A and key issue #1 in 3GPP TR 23.700-29 [TR2370029]). The 5GS control plane functions (e.g. AMF, SMF) are deployed on the ground, which is not depicted in the figure for simplicity. EASs and their registered EES are deployed on one or more satellites which constitutes an EDN and these satellites’ coverage areas may correspond to an EDN service area. UPF can be deployed on ground to access the ECS, the EEC (in UE) can reach the ECS by EDGE-4 (on ground or via space).

If there are multiple EESs in a satellite EDN, EEC may trigger EAS discovery towards each EES which increases delay due to EDGE-1 interactions. It is recommended to deploy a single EES per EDN to reduce complexity in satellite.

NOTE:	The UPF and RAN deployment are described above for the sake of clarity which is related to EDN on-board satellite (e.g. EDGE-4 path), see also TS 23.501 [23501] clause 5.43.2 and TR 23.700-29 [TR2370029]).

During initial service discovery, the EEC (in UE) contacts the ECS via EDGE-4 (on ground or via space) to find an appropriate EES, then an appropriate EAS instance is selected during EAS discovery via EDGE-1 interaction. Finally, AC (in UE) communicates with the selected EAS.

Editor's Note:	Other deployment options and associated latency aspects are FFS.

##### 7.2.1.1.2	Further consideration for service provisioning

An external server responsible for providing satellite availability information for each satellite based on real-time satellite orbit information (e.g. eccentricity, inclination, true anomaly) may exist (e.g. see TS 23.501 [TS23501], Annex Q). During service provisioning, the EEC can provide needed satellite capability information (e.g. satellite frequency bands) to the ECS and the ECS will be responsible to monitor UE location and UE moving prediction, and query the external server(s) to get a list of satellites (each identified by satellite ID) that can service the list of waypoints of the UE, then determine a suitable EES with a matching satellite ID to serve the UE with longest service time in service provisioning. Once the ECS identifies the suitable EES, it responds/notifies the EEC with the EES information. In addition, the corresponding ECS selected satellite ID is sent to the EEC.

##### 7.2.1.1.3	EAS discovery for EAS on board satellite

Unlike EAS discovery procedures EAS(s) on the ground, the EAS discovery for EAS(s) on board the satellite could be mobile depending on the satellite they are deployed on: GEO, MEO, or LEO. For GEO satellites, since the satellites are stationary relative to the Earth's surface, the EAS is also static. However, for MEO and LEO satellites, the EAS(s) are mobile as the satellites are moving relative to the Earth's surface. In this case, if the trajectory of the EAS is not considered for the discovery, frequent interruption, ACR, and/or service degradation may be experienced when there is a mismatch between the movement of the UE and the movement of the EAS on board satellite. This can be alleviated by considering the trajectory of the EAS on board satellite during the discovery. This solution is based on the assumption that the actual trajectory calculation for the satellite is performed by the satellite service provider and/or the operator. This trajectory calculation can be based on tlr-based mean orbital ephemeris, instantaneous/osculating ephemeris or any other means as per the satellite provider and/or operator. It is further assumed, the satellite provider and/or operator can also recommend a trajectory pattern that closely matches the planned route of the UE given the planned or expected route of a UE. The EEL does not need to deal with the actual trajectory calculation and only deals with trajectory ID that is provided by the satellite service provider and/or operator. This can be accomplished by adding mobility information and trajectory ID for the EAS in its profile.

- EAS mobility: to indicate the mobile EASs (EAS on board MEO or LEO satellite) and non-mobile EAS (EAS on board GEO satellite and EAS on-ground)

- Trajectory ID: to indicate the trajectory of the EAS on board MEO and LEO satellites.

NOTE 1: It is up to the satellite service provider or operator to associate the Trajectory ID with the actual trajectory of the satellite. The trajectory and position of the satellite can be calculed based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris.

NOTE 2:	The generation of the trajectory ID for the EAS(s) is done by the operator and/or the satellite service provider.

##### 7.2.1.1.4	Service provisioning  for EES on board satellite

Similar to EAS on board a MEO or LEO satellite, the EES will also be mobile when the EES is also on board a MEO or LEO satellite. Therefore considering the trajectory of the EES is important for efficient service service provisioning for selecting EDN matched with movement of the AC hosting UE during the AC’s operation schedule.

- EES mobility: to indicate the mobile EESs (EES on board MEO or LEO satellite) and non-mobile EES (EES on board GEO satellite and EES on ground).

- Trajectory ID: to indicate the trajectory of the EES on board MEO and LEO satellites.

NOTE 3:	It is up to the satellite service provider or operator to associate the Trajectory ID with the actual trajectory of the satellite. The trajectory and position of the satellite can be calculed based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris.

NOTE 4:	The generation and assignment of the trajectory ID for the EES(s) is done by the operator, the satellite service provider.

#### 7.2.1.2	Architecture Impacts

This solution has no architecture impacts to EDGEAPP. Under the existing EDGEAPP architecture, the service provisioning procedure is enhanced to support different cases in service continuity.

#### 7.2.1.3	Corresponding APIs

##### 7.2.1.3.1 	EAS Profile, EES Profile and AC Profile for discovery and service provisioning

According to clause 7.2.1.1.x EAS discovery for EAS onboard satellite and clause 7.2.1.1.x Service provisioning for EES on board satellite, the following change is needed in TS 23.558.

- TS 23.558 Clause 8.2.4 EAS Profile EAS mobility on board LEO or MEO Satellite

TS 23.558 Table 8.2.4-1: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X, UAV, application enabler) |
| EAS description | O | Human-readable description of the EAS |
| … | … | … |
| Dynamic service area indication | O | Indicates if the service area is dynamic or not. The service area is dynamic in case of EAS on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| Trajectory ID (NOTE 1) | O | For mobile EAS, indicates an identifier for that relates to trajectory of the EAS on board a MEO and LEO satellites. |
| … NOTE 1: The assignment of the trajectory ID for EAS(s) can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculed for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |

- TS 23.558 Clause 8.2.6 EES Profile EES and EAS mobility on board LEO or MEO Satellite

TS 23.558 Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| … | … | … |
| Dynamic service area indication | O | Indicates if the service area is dynamic or not. The service area is dynamic in case of EES on board a MEO or LEO satellite and not dynamic in case of a GEO satellite. |
| Trajectory ID (NOTE 2) | O | For mobile EES, indicates the trajectory of the EES on board MEO and LEO satellites. |
| … NOTE 2: The assignment of the trajectory ID for the EES can be done by the operator and/or the satellite service provider. The trajectory and position of the satellite can be calculed for example based on instantaneous/osculating ephemeris versus using TLE-based mean orbital ephemeris. |  |  |

Editor’s Note: Whether and how the Trajectory ID is defined is FFS.

Editor’s Note: Expected trajectory ID in the AC Profile is FFS.

#### 7.2.1.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

### 7.2.2	Solution #2: Enhancement for on board EES(s) and service provisioning

#### 7.2.2.1	Solution description

This solution proposes to address the following open issues in KI#5,

-	How the EES(s) are placed on board the Satellite.

-	Whether and how the service provisioning is impacted.​

It is proposed to add the Satellite assistant information in the EES profile to indicating the EES(s) are placed on board the satellite, so that, the ECS could do the service provisioning based on those Satellite assistant information.

##### 7.2.2.1.1	General

When EES is deployed in the satellite, it is proposed to add the Satellite assistant information in the EES Profile to indicating the EES is deployed on-board. The Satellite assistant information could be the satellite ephemeris information(e.g., time slot and spatial location). So that the ECS could obtain the EES’s Satellite assistant information during the EES registration.

Table 7.2.2.1.1-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EDN information | O | EDN information where the EES resides. |
| > DNN | M | Data network name to identify the EDN. |
| > DNAI(s) | O | DNAI(s) associated with the EDN. |
| EASIDs | M | List of EASIDs registered or expected to be registered with the EES. |
| >Allowed MNO information | O | Information of the allowed operator as described in EAS profile clause 8.2.4, Only subscribers from these operators can consume the EES services. |
| List of EAS bundle information | O | List of EAS bundles per EASID to which the EAS belongs and related bundling requirements. |
| > Bundle ID (NOTE 2) | O | A bundle ID as described in clause 7.2.10. |
| > List of EASIDs (NOTE 2) | O | List of EASIDs associated with the EAS bundle. |
| > Bundle type | M | Type of the EAS bundle as described in clause 7.2.10 |
| > EAS bundle requirements | O | Requirements associated with the EAS bundle as described in clause 8.2.10. |
| Instantiable EAS information | O | The EAS instantiation status per EASID (e.g. instantiated, instantiable but not be instantiated yet). |
| > Instantiation criteria (NOTE 1) | O | The criteria upon which EAS can be instantiated (e.g. based on specific date and time). |
| EEC registration configuration | M | Indicates whether the EEC is required to register on the EES to use edge services or not. |
| ECSP ID | O | The identifier of the ECSP that provides the EES. |
| EES Topological Service Area | O | The EES serves UEs that are connected to the Core Network from one of the cells included in this service area. EECs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES DNAI(s) | O | DNAI(s) associated with the EES. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN, where the EES resides. |
| EES Service continuity support | O | Indicates if the EES supports service continuity or not. This IE indicates which ACR scenarios are supported by the EES, also indicates the EES ability (e.g. EAS bundle information) of handling bundled EAS ACR. |
| Satellite assistant information | O | Assistant information indicating the EES is on-board, and could be used to calculate the satellite's position and movement. It could be the statistic satellite ephemeris information (e.g.,signal quality metrics and orbital elements) |
| NOTE 1: "Instantiation criteria" IE shall be present only when the value of "Instantiable EAS information" IE is "instantiable but not be instantiated yet". NOTE 2: At least one of the IEs shall be present if EAS bundle information is provided. |  |  |

After that, when UE requests the service provisioning, the following changes may apply.

Pre-condition:

- The ECS support the availability information calculation.

Figure 7.2.2.1.1-1: service provisioning

-	In step2, the ECS calculates the EES availability information based on the Satellite assistant information, and determines the EES based on the EES availability information so that the EES is accessible to the UE. The EES availability information indicates when and how the EES is available for the requesting EEC. It could be a sequence of time durations for each grid point where each time duration includes an indication of coverage availability or unavailability as discussed in Annex Q of TS 23.501.

-	In step3, if the EES meet the requirement so that the on-board EES is selected by the ECS, the ECS should send the EES availability information to the EEC. The EEC can select one or more such EES(s) to perform EAS discovery based on EES availability information.

#### 7.2.2.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts (if any) of the solution and possible new SA6 capabilities and interfaces.

#### 7.2.2.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

#### 7.2.2.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

### 7.2.3	Solution #3: Support discontinuous coverage for satellite access

#### 7.2.3.1	Solution description

This solution addresses the KI#3: Satellite access with discontinuous coverage.

This solution shows how the application enabler obtaining the satellite related information (e.g., ephemeris) from the satellites, retrieving unavailability period for UE with satellite access then exposing to the VAL UE and the VAL server.

This solution is based on the following assumptions and principles:

-	This solution main serves for IoT services and take the SEAL as the application enabler.

-	The SEAL server has obtained the ephemeris for Satellites.

-	The SEAL server is aware of VAL UE’s location.

In this solution, it’s assumed the SEAL server has obtained the ephemeris and it could generate the satellite coverage availability information which can be indicated to the UE whether or not coverage is available for a specific satellite RAT Type for a particular location and time. The example of coverage availability information can refer to the Annex Q of TS 23.501[2]. And based on clause 5.4.13.2 of TS 23.501[2], the coverage availability information can be provided to a UE by an external server. That means the SEAL server could provision UE the coverage availability information if UE is using the satellite access.

From the ephemeris and Satellite coverage availability information, the SEAL server could retrieve the unavailability period (e.g., duration and start time) based on UE’s location and then provide to the VAL UE and expose to the VAL server to indicate them to consider the unavailability period when the VAL UE is using satellite access.

The procedure of 7.2.3.1.1 specifies the SEAL server generates or formats the Satellite coverage availability information based on ephemeris and retrieve the unavailability period (e.g., duration and start time) based on UE’s location and expose it to the VAL UE and VAL server to guild their following actions.

##### 7.2.3.1.1	Procedure of SEAL Server generating Satellite coverage availability and unavailability period information

Pre-condition:

-	SEAL Client has registered on SEAL server.

-	The VAL server asks SEAL Server to provide VAL UE’s information (e.g. location data) periodically.

-	The SEAL server is aware of the VAL UE’s location information.

Figure 7.2.3.1.1-1: Procedure of SEAL Server generating Satellite coverage availability and unavailability period information

0. 	Assume the SEAL server has obtained the ephemeris for Satellites.

1.	The SEAL server may generate the satellite coverage availability information based on the obtained the ephemeris for Satellites and the VAL UE’s location data or shape them in a unified format in case it receives the satellite coverage availability information directly from different 3rd parties (e.g., different Satellite companies). The example of satellite coverage availability information can refer to the Annex Q of TS 23.501[2].

2.	As the satellite coverage availability information could indicate to the UE whether the coverage is available for a particular location and time, the SEAL server configures the satellite coverage availability information to the VAL UE/ SEAL Client via satellite coverage availability information configuration request.

3.	The SEAL Client acknowledges the SEAL server if it supports the satellite access.

4.	Based on the ephemeris for satellites and Satellite coverage availability information, the SEAL server also could retrieve the unavailability period (e.g., unavailability period duration and start time for the unavailability period) to guild UE when it could trigger the UL service flow, in case the UE wants to get the unavailability period information directly.

5.	The SEAL server sends the unavailability period information to the VAL UE/SEAL Client via unavailability period information configuration request.

6.	The SEAL Client acknowledges the SEAL server if it supports the satellite access. And the SEAL Client will operate considering the unavailability period information (e.g. not send UL messages during the unavailability period).

7.	If the VAL server asks to provide VAL UE’s information (e.g. location data) periodically, the SEAL server may notify the VAL server the unavailability period information of VAL UE which could indicate VAL server how to operate to minimize the service impact (e.g., extended data buffering, pending downlink data transferring).

#### 7.2.3.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts (if any) of the solution and possible new SA6 capabilities and interfaces.

#### 7.2.3.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

#### 7.2.3.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# 8	Deployment scenarios

## 8.1	General

Editor's Note:	This clause will provide a general description of the deployment scenarios.

## 8.x	Deployment model #x: <Title>

Editor's Note:	Provide a description of the deployment scenarios including applicability to Mission Critical or Application Enablers.

# 9	Business Relationships

Editor's Note:	Provide a description of the involved business relationships including applicability to Mission Critical or Application Enablers.

# 10	Overall evaluation

## 10.1	Architecture evaluations for Mission Critical

Editor's Note:	This clause will provide evaluation of different architectures for Mission Critical.

## 10.2	Key issue evaluations for Mission Critical

Editor's Note:	This clause will provide evaluation of different solutions for Mission Critical.

## 10.3	Architecture evaluations for Application Enablers

Editor's Note:	This clause will provide evaluation of different architectures for Application Enablers.

## 10.4	Key issue evaluations for Application Enablers

Editor's Note:	This clause will provide evaluation of different solutions for Application Enablers.

# 11	Conclusions

## 11.1	General conclusions

Editor's note:	This clause will provide general conclusions for the study.

## 11.2	Conclusions for Mission Critical

### 11.2.x	Conclusions of key issue #x

Editor's Note:	This clause will provide conclusions for the specific key issue.

## 11.3	Conclusions for Application Enablers

### 11.3.y	Conclusions of key issue #y

Editor's Note:	This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-11 | SA6#58 | S6-233765 |  |  |  | Proposed skeleton for FS_5GSAT_Ph3_APP (23.700-01) | 0.0.0 |
| 2024-01 |  |  |  |  |  | After FS_5GSAT_Ph3_APP was approved at SA#102 Plenary (December 2023) all pCRs approved at SA6#58 are implemented in TR 23.700-01 V0.1.0 | 0.1.0 |
| 2024-01 |  | S6-233944 |  |  |  | FS_5GSAT_Ph3_APP_Scope | 0.1.0 |
| 2024-01 |  | S6-234109 |  |  |  | New KI on Application enabled for services over satellite access | 0.1.0 |
| 2024-01 |  | S6-234090 |  |  |  | New KI on Enabling re-discovery and re-allocation of EASs on-board satellites | 0.1.0 |
| 2024-01 |  |  |  |  |  | References [2]-[10] added. | 0.1.0 |
| 2024-03 | SA#59 | S6-240350 |  |  |  | FS_5GSAT_Ph3_App_editorials | 0.2.0 |
| 2024-03 | SA#59 | S6-240740 |  |  |  | FS_5GSAT_Ph3_App_update-KI#1 | 0.2.0 |
| 2024-03 | SA#59 | S6-240733 |  |  |  | New KI on satellite access with discontinuous coverage | 0.2.0 |
| 2024-03 | SA#59 | S6-240604 |  |  |  | pCR on KI for satellite backhaul support for MC services | 0.2.0 |
| 2024-03 | SA#59 | S6-240741 |  |  |  | New Key Issue on Edge on board NGSO Satellite | 0.2.0 |
| 2024-03 | SA#59 | S6-240742 |  |  |  | Satellite edge computing | 0.2.0 |
| 2024-04 | SA#60 | S6-241591 |  |  |  | KI update with service continuity | 0.3.0 |
| 2024-04 | SA#60 | S6-241646 |  |  |  | New KI: Support of UE-Satellite-UE communication for IMS services via Satellite access | 0.3.0 |
| 2024-04 | SA#60 | S6-241620 |  |  |  | New KI: Usage of S&F events information for the application enablement | 0.3.0 |
| 2024-04 | SA#60 | S6-241655 |  |  |  | Update Sol#AE1 (KI#5): Satellite Edge computing | 0.3.0 |
| 2024-04 | SA#60 | S6-241595 |  |  |  | Update Sol#AE1 (KI#5, KI#2): Satellite Edge computing | 0.3.0 |
| 2024-04 | SA#60 | S6-241419 |  |  |  | New Sol for KI#5: Enhancement for on board EES(s) and service provisioning | 0.3.0 |
| 2024-04 | SA#60 | S6-241596 |  |  |  | New solution for KI#3 to support discontinuous coverage for satellite access | 0.3.0 |
| 2024-04 | SA#60 | S6-241658 |  |  |  | Annex-B: Information related to Satellite access support for MC Service | 0.3.0 |

Annex B (informative):	Information related to Satellite access support for MC Service

As per the current 3GPP specifications 3GPP TS 22.261 and 3GPP TS 23.501, terrestrial communicationis possible via satellite access, where there is a connectivity between the UE and the satellite. This means that 5G services are possible over both Terrestrial Network (TN) and Non-Terrestrial Network (NTN). In NTN, Satellite is placed into Low-Earth Orbit (LEO) typically at an altitude between 300 km to 2000 km, Medium-Earth Orbit (MEO) typically at an altitude between 8000 to 20000 km, or Geostationary satellite Earth Orbit (GEO) at 35786 km altitude. Utilizing satellite access is beneficial to increase coverage and availability of the services. However, there is significant propagation delay associated to connectivity via satellite as compared to connecting via terrestrial network.

The propagation delay via satellite associated with different orbit is as below as discussed in 3GPP TS 22.261:

Table B-1: Satellite propogation delays

|  | UE to serving satellite propagation delay [ms] [NOTE 1] |  | UE to ground max propagation delay [ms] [NOTE 2] |
| --- | --- | --- | --- |
|  | Min | Max |  |
| LEO | 3 | 15 | 30 |
| MEO | 27 | 43 | 90 |
| GEO | 120 | 140 | 280 |
| NOTE 1: The serving satellite provides the satellite radio link to the UE NOTE 2: delay between UE and ground station via satellite link; inter satellite links are not considered |  |  |  |

Satellite systems of relatively low altitude, e.g., LEO, can offer relatively lower latency, which can be the most beneficial choice latency sensitive services such as MC services.

Furthermore, unlike GEO, beams of non-GEO satellites (LEO and MEO) are moving with respect to the ground. Therefore, it is necessary to have a constellation of several non-GEO satellites associated with the MC service under consideration to ensure proper service continuity over the non-terrestrial networks. This results in having the same cell being covered by different beams, i.e., different non-GEO satellites over time.

Following KPIs are requirements for MC service as specified in 3GPP TS 22.179:

Table B-2: Mission Critical KPIs

| KPI 1 | MCPTT Access time | < 300 ms |
| --- | --- | --- |
| KPI 2 | End-to-end MCPTT Access time | < 1000 ms |
| KPI 3 | Mouth-to-ear latency | < 300 ms |
| KPI 4a | Maximum Late call entry time (without application layer encryption) | < 150 ms |
| KPI 4b | Maximum Late call entry time (with application layer encryption) | < 350 ms |

MC services are not restricted only to the ones defined in this clause and such services can also have priority treatment, if defined via operator's policy and/or local regulation. Priority treatment for MC Services require appropriate ARP and 5QI (plus 5G QoS characteristics) setting for QoS Flows according to the operator's policy. As per 3GPP TS 23.501, the following 5QIs are proposed for catering mission critical KPIs:

Table B-3: 5QIs catering Mission Critical KPIs

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 69 | Non-GBR | 5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 8 | Non-GBR | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive |
| 65 | GBR | 7 | 75 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 67 | GBR | 15 | 100 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 4 | GBR | 50 | 300 ms | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 70 | Non-GBR | 55 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |
| 10 | Non-GBR | 90 | 1100ms | 10-6 | N/A | N/A | Video (Buffered Streaming) TCP-based (e.g. www, e-mail, chat, ftp, p2p file sharing, progressive video, etc.) and any service that can be used over satellite access type with these characteristics |

Considering MCPTT Access time and Mouth-to-ear latency KPI requirements, which are less than 300 ms, the consumption of MC Services has impacts when MC Service users are connected via Satellite due to significant propagation delay.